3GPP TSG RAN WG1 #113	                                         	           R1-2305265

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.11.1

Source:	Apple Inc.

Title:	On performance evaluation for low power wake-up signal

Document for:	Discussion/Decision

# Introduction

In Rel-18, a study item was approved for low-power wake-up signal and receiver (LP WUS/WUR) for NR (WID in RP-222644 [1]), and it includes the following objectives.

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In this contribution, we provide some power saving evaluation results for LP WUS based on the agreed power modeling.

# Power saving evaluation

We evaluate the power saving gain for idle/inactive UEs based on the following assumptions:

Table 1 Assumptions for power saving evaluation for idle/inactive UEs

| Parameter |  | Value | Note |
| --- | --- | --- | --- |
| Bandwidth |  | 20 MHz |  |
| SCS |  | 30 kHz |  |
| iDRX cycle |  | 1.28 s |  |
| (Per UE paging rate per iDRX cycle, number of UEs per paging group, number of UEs per LP-WUS group) |  | (0.1%, 100, 1) (1%, 10, 1) (1%, 10, 5) (1%, 10, 10) | These cases roughly correspond to group paging rate of 10% for legacy operation. |
| MR ultra-deep sleep state | Relative power | 0.015 |  |
|  | Transition energy (unit x ms) | 15000, 40000 |  |
|  | Ramp up time | 400, 800 ms |  |
| LP WUS/WUR | Relative power for ON state | 0.1, 1, 4, 10 |  |
|  | Relative power for OFF state | 0.001 |  |
|  | Transition energy (unit x ms) | 10ms for ON power >= 1 | No transition energy assumed for ON power < 1 |
|  | Duty cycle for WUS | 1.28 s |  |
|  | WUR ON duration per cycle | 10 ms |  |
|  | FAR | 1% |  |
| Paging procedure |  | Once a WUS is received, the UE wakes up the main radio and monitors PO as in legacy procedure. |  |
| Number of SSBs before PO after MR wakes up |  | 3 (Low SINR assumed) | No additional SSBs assumed compared to Rel-17 PEI study. |
| RRM measurement |  | Intra-freq and inter-freq RRM measurement are assumed in the cycles when MR is on due to LP-WUS. In addition, cases are considered where RRM measurement is assumed to be perform by MR at least once every 5/10/20 cycles. |  |
| Baseline |  | Rel-17 PEI with 8 subgroups | The difference between the cases with subgrouping and without subgroups is quite small. |

The power saving gain relative to the baseline is provided in Figure 1 for transition energy of 15000 and 40000, assuming that RRM measurements by MR are only done when MR is on due to LP-WUS. This implies that LP WUR may need to perform some RRM measurements. As we can observe from the figure, the power saving gain is very sensitive to (1) the MR transition energy and (2) the probability of LP-WUS paging per cycle. The reason is that the MR power consumption in ultra-deep sleep state and MR transition energy are the dominating part in the total power consumption, and it highly depends on how often the MR wakes up. Significant power saving is observed when the probability of MR waking up is low, as the MR stays in ultra-deep sleep state most of the time and rarely wakes up (less overhead from transition energy).

Another important observation from Figure 1 is that the power saving gain is not very sensitive to the power consumption of the LP WUR in these cases. This is because in these cases, the total power consumption is largely dominated by the MR power consumption in ultra-deep sleep state and MR transition energy, while the power consumption of the LP WUR is only a small percentage.

Observation 1: For idle/inactive UEs, the power saving gain of LP WUS/WUR highly depends on MR transition energy and the probability of LP-WUS waking up the MR.

Observation 2: For idle/inactive UEs, the power saving gain of LP WUS/WUR is not very sensitive to the power consumption of LP WUR, as long as the power consumption of LP WUR is sufficiently lower (e.g. one order of magnitude lower) than the MR and the ON duration is relatively short.

- MR transition energy = 15000	     		(b) MR transition energy = 40000

Figure 1 UE power saving gain for different LP WUR power consumption

The power consumption also highly depends on how RRM measurement procedures. In Figure 1, it is assumed that RRM measurement is performed by MR only when it is woken up by LP-WUS. However, this may not sufficient if the UE is moving. Here we assume a simple relaxation factor for RRM measurement, meaning that MR needs to perform RRM measurement at least once every X cycles, where X=5, 10, or 20. The power saving results are provided in Figure 2. For MR transition energy of 15000, the gain starts to diminish when RRM measurement needs to be done at least once every 5 cycles. For MR transition energy of 40000, the gain is already negative when RRM measurement needs to be done at least once every 10 cycles.

- MR transition energy = 15000                                (b) MR transition energy = 40000

Figure 2 Power saving gain with different RRM measurement assumptions

Therefore, in the design, it is critical to consider the mechanisms that can relax and/or offload RRM measurement at the MR.

Observation 3: For idle/inactive UEs, the frequency of RRM measurements performed by the MR needs to be sufficiently low to achieve power saving gain. For MR transition energy of 15000, the RRM should not be performed more frequently than once every 5 cycles. For MR transition energy of 40000, the RRM should not be performed more frequent than once every [20] cycles.

In terms of latency, Table summarizes the latency for different cases. The assumption here is that the MR needs 3 SSBs for (re-)sync purpose, same as what is needed for PO decoding for low SINR case in legacy operation. Therefore, the additional latency for using LP-WUR is the ramp-up time for the MR.

|  | Baseline: R17 PEI | LP-WUR 400ms ramp-up time | LP-WUR 800ms ramp-up time |
| --- | --- | --- | --- |
| Average latency (ms) | 690 | 1090 | 1490 |

Observation 4: For idle/inactive UEs, the additional latency from LP-WUR is dominated by the MR ramp-up time.

If latency is a concern, the periodicity of LP-WUS can be reduced, and the paging monitoring after receiving LP-WUS can be enhanced so that the UE does not need to wait for the legacy PO.

# Conclusion

In this contribution, we have presented some evaluation results on power saving gain for LP WUS/WUR, and have the following observations and proposals:

Observation 1: For idle/inactive UEs, the power saving gain of LP WUS/WUR highly depends on MR transition energy and the probability of LP-WUS waking up the MR.

Observation 2: For idle/inactive UEs, the power saving gain of LP WUS/WUR is not very sensitive to the power consumption of LP WUR, as long as the power consumption of LP WUR is sufficiently lower (e.g. one order of magnitude lower) than the MR and the ON duration is relatively short.

Observation 3: For idle/inactive UEs, the frequency of RRM measurements performed by the MR needs to be sufficiently low to achieve power saving gain. For MR transition energy of 15000, the RRM should not be performed more frequently than once every 5 cycles. For MR transition energy of 40000, the RRM should not be performed more frequent than once every [20] cycles.

Observation 4: For idle/inactive UEs, the additional latency from LP-WUR is dominated by the MR ramp-up time.

# Reference

[1] RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, RAN#97e, September 2021.