3GPP TSG RAN WG1 #113			R1-2305304

Incheon, Korea, May 22nd – May 26th, 2023

Title:	Draft reply LS on RS supported for group-based reporting

Response to:	R1-2304311(R4-2306394)

Release:	Rel-18

Work Item:	NR_FR2_multiRX_DL-Core

Source:	LG Electronics [RAN1]

To:	RAN4

Cc:	RAN2

Contact Person:

Name:

E-mail Address:

Attachments:	none

1. Overall Description:

RAN1 would like to thank RAN4 for the question on RS supported for group-based reporting. Answers from RAN1 are given below:

Question 1: RAN4 understands that two resource sets consisting of SSB + SSB or CSI-RS + CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report. However, RAN4 is not clear whether the two resource sets configured for Rel-17 group-based L1-RSRP measurement and report can contain a mix of SSB and CSI-RS. Can RAN1 clarify whether a mix of SSB and CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report?

Answer: There is no restriction regarding two resource sets consisting of SSB + CSI-RS for Rel-17 group-based L1-RSRP measurement and report from RAN1 perspective.

Question 2: When CSI-RS resources are configured as resource set for Rel-17 group-based L1-RSRP measurement and report, can the CSI-RS resource with repetition ON be configured for Rel-17 group-based L1-RSRP report?

Answer: Yes. CSI-RS resource set for channel measurement for group-based L1-RSRP measurement and report can be configured with repetition ON. If the higher layer parameter repetition is set to ‘on’ for a CSI-RS resource set of two resource sets for Rel-17 group-based L1-RSRP measurement and report, CRI for the CSI-RS Resource set for channel measurement is not reported and one highest L1-RSRP value among the resources of the CSI-RS resource set for channel measurement is reported regardless of the configured number of beams to report. For example, if repetition is set to ‘on’ for resource set #1 and ‘off’ for resource set #2 with four beam pairs to be reported, the beam report shall include Resource set indicator, one highest L1-RSRP value for the resource set #1, four CRIs for resource set #2, and four L1-RSRP values for resource set #2. If repetition is set to ‘on’ for both resource sets in above example, beam report for the two resource sets shall include Resource set indicator, one highest L1-RSRP value for resource set #1, and one highest L1-RSRP value for resource set #2.

Question 3: Does RAN1 specification define UE reporting behaviour for the case when NW configures UE with Rel-17 group-based L1-RSRP measurement and report, and UE did not find any pair of beams for report (e.g., the measured L1-RSRP of at least one of the beams of any beam pair is lower than a certain value, e.g., -140dBm)?

Answer: As legacy beam report, the number of reported beam pairs is determined by gNB configuration. If gNB configures UE to report N beam pairs, UE should report N beam pairs in any case.

2. Actions:

To RAN1 group.

ACTION: 	RAN1 respectfully requests RAN4 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #114		              21 – 25 August 2023			Toulouse, France

TSG RAN WG1 Meeting #114-bis	              9 – 13 October 2023			Xiamen, China