3GPP TSG RAN WG1 #113				R1-2305375

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.2

Source: 	Qualcomm Incorporated

Title: 	UE features for dedicated spectrum less than 5MHz

Document for:	Discussion/Decision

- Introduction

In this contribution, we discuss our initial views on UE features for Rel18 NR dedicated spectrum less than 5MHz.

- UE features for dedicated spectrum less than 5MHz

Proposal: Introduce the following new FGs/rows for dedicated spectrum less than 5MHz:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_ FR1_lessthan_5MHz_BW | 51-1 | Transmission bandwidth of 20RBs in 5MHz channel bandwidth | 1. Subcarrier spacing of 1.25kHz/15kHz for PRACH and 15kHz for other channels/signals 2. BWP=25RBs 3. SSB bandwidth of 20RBs, same as that of transmission bandwidth of 25RBs in 5MHz channel bandwidth 4. Tx bandwidth within 20RBs, same as the SSB transmission bandwidth of 20RBs 5. Disabled PUCCH hopping in msg4 6. CSI-RS/TRS transmission bandwidth of 20RBs 7. 3-FDMed PRACH occasions with 1.25kHz subcarrier spacing | 6-1 | Yes |  |  | Per FSPC | FDD only | FR1 only |  |  | Optional with capability signalling |
| 51. NR_ FR1_lessthan_5MHz_BW | 51-2 | Transmission bandwidth of 15RBs in 3MHz channel bandwidth | 1. Subcarrier spacing of 1.25kHz/15kHz for PRACH and 15kHz for other channels/signals 2. BWP=15RBs 3. SSB bandwidth of 12RBs, with PBCH transmission bandwidth same as that of PSS/SSS 4. Tx bandwidth within 15RBs, confining the SSB transmission bandwidth of 12RBs 5. Disabled PUCCH hopping in msg4 | 6-1 | Yes |  |  | Per FSPC | FDD only | FR1 only |  |  | Optional with capability signalling |
| 51. NR_ FR1_lessthan_5MHz_BW | 51-3 | Transmission bandwidth of 12RBs in 3MHz channel bandwidth | 1. Subcarrier spacing of 1.25kHz/15kHz for PRACH and 15kHz for other channels/signals 2. BWP=15RBs 3. SSB bandwidth of 12RBs, with PBCH transmission bandwidth same as that of PSS/SSS 4. Tx bandwidth within 12RBs, same as the SSB transmission bandwidth of 12RBs 5. Disabled PUCCH hopping in msg4 6. CSI-RS/TRS transmission bandwidth of 12RBs | 6-1 | Yes |  |  | Per FSPC | FDD only | FR1 only |  |  | Optional with capability signalling |

# References

[1] TS38.822, “User Equipment (UE) feature list”

Table 4.1-1: Layer-1 feature list

| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for Pcell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  | n/a | n/a | n/a | n/a | This feature should be mandatory without capability signalling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP | Mandatory without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |