3GPP TSG RAN WG1 #113		R1-2305474

Incheon, Korea, May 22nd – May 26th, 2023

Source:	OPPO

Title:	Discussion on RAN2 LS for using 1-symbol PRS in PDC

Agenda Item:	5

Document for:	Discussion and Decision

# Introduction

RAN2 asked RAN1 the following question in the LS R1-2304328 (R2-2304510) [1], in the observation that 1-symbol PRS is introduced into positioning framework while its fate is unclear in PRS-based PDC feature.

RAN2 would also like to ask the following question to RAN1: Question: Do we need changes to DL PRS configuration used for RTT-based Propagation Delay Compensation?

Although RAN1 already agreed to make changes to RRC signaling of PDC [2], this contribution provides our further viewpoints.

# Discussion

In RAN1 Rel-18 TEI discussion, it was agreed [2] to add in TS 38.211 one PRS symbol per PRS pattern, i.e. LPRS=1, for all comb sizes KcombPRS∈2,4,6,12. If nothing more is provided in RAN1 specifications, this 1-symbol PRS, which was originally targeted to positioning framework, would automatically propagate to PRS-based PDC framework, due to the following TS38.213 text.

Text in 38.214 section 9.1 (specifically for PDC): - dl-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211]. All the DL PRS resources within the resource set have the same value of dl-PRS-NumSymbols.

Consequently, a direct answer of YES to RAN2 LS is simply straightforward or even redundant, given that RAN1 agreed in [2] to add 1-symbol PRS to the RRC IE “NR-DL-PRS-PDC-ResourceSet”.

However, although the 1-symbol PRS is sorted out nicely in RAN1 and RAN2 specifications, there can be an issue on RAN4 side. This is because the original RAN1 LS [4] sent to other RAN WGs, as copied below, mentions NR positioning only but not PDC.

RAN1 LS text in [3]: In RAN1#112 meeting TEI agenda, RAN1 has made the following agreement for NR positioning - Introduce 1-symbol PRS with legacy comb sizes. - UE expects the suitable expected RSTD windows provided by LMF such that peak ambiguity is addressed. Otherwise no measurement accuracy requirements are expected to be met. - Not to define RAN4 RRM requirement, including core/performance in Rel-18

The issue is that: there can be a conflict between the claimed benefits of 1-symbol PRS and the RAN4 RRM performance requirement applicability to the 1-symbol PRS. To be more specific, in current RAN4 specification TS 38.133 section 10.1.40 (specifically for PRS-based PDC), the UE Rx-Tx time difference measurement accuracy requirement tables list the accuracy requirements against a number of parameters, one of which is “PRS resource repetition” that is formulated as TrepPRS×LPRS/KcombPRS, where TrepPRS corresponds to the PRS repetition factor in time domain. RAN4 currently does not seem to consider this “PRS resource repetition” to be less than a constant, say γ (γ equals to either 1 or 4 in those RAN4 tables). In such a case, TrepPRS×LPRSKcombPRS≥γ≥1 leads to TrepPRS≥γ∙KcombPRS/LPRS. When LPRS=1, TrepPRS≥γ∙KcombPRS≥2γ≥2. That is to say, in order to fit the existing RAN4 performance requirement framework or to maintain a comparable “PRS resource repetition” against an existing <LPRS, KcombPRS> pair for LPRS≥2, unless γ≤0.5 appears in 38.133 which is not the case for now, 1-symbol PRS pattern likely needs to be repeated in time domain, which unfortunately neutralizes the claimed benefits from using 1-symbol PRS, such as PRS overhead, PRS capacity and UE power saving. Therefore, if a common understanding is reached between RAN1 and RAN2 that 1-symbol PRS is applicable to PDC as well and this common understanding is also taken by RAN4, RAN1 should update with RAN4 that the RAN1 guidance in [4], such as “no RRM requirements”, is also applicable to the PDC case; otherwise, RAN4 may have to take extra efforts to work on new RRM requirements for PDC because the existing RAN4 RRM requirements may not be compatible to 1-symbol PRS unless 1-symbol PRS is required to be repeated.

Proposal: In case 1-symbol PRS is applicable to PDC across RAN1 and RAN2, RAN1 informs RAN4 via LS not to define RRM requirements for 1-symbol PRS in PDC.

# Conclusions

This contribution concludes with the following proposal:

Proposal: In case 1-symbol PRS is applicable to PDC across RAN1 and RAN2, RAN1 informs RAN4 via LS not to define RRM requirements for 1-symbol PRS in PDC.

# References

- R1-2304328 (R2-2304510), Reply LS on 1-symbol PRS, RAN2

- R1-2304238,  Consolidated higher layers parameter list for Rel-18 NR, Ericsson

- R1-2304279,  (Rel-18 TEI CR) Introduction of additional PRS configurations [1symbol_PRS], Ericsson

- R1-2302201, LS on 1-symbol PRS, RAN1