3GPP TSG RAN WG1 #113			     R1-2305608

Incheon, Korea, May 22nd – May 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on spatial and power domain enhancements for NW energy savings

Agenda Item:	9.7.1

Document for: 	Discussion and Decision

- Introduction

At RAN1#112bis-e meeting, following agreements on techniques of spatial and power domain are achieved [1]:

Agreement Define necessary enhancements to support both types of spatial adaptation cases (as defined in RAN1#112) in Rel-18. - Note: This does not imply explicit definition in specifications for adaptation types. - Note: This does not imply explicit specification changes are made for both cases Agreement Support configurability of NZP CSI-RS resource(s) for channel measurement within one resource setting corresponding to more than one spatial adaptation patterns with at least one of the following - A1-1-revised: a resource set with multiple resources is configured within a resource setting, where each resource is associated with only one spatial adaptation pattern - A1-2-revised: For a resource configured in a resource set within a resource setting, the resource can be associated with more than one spatial adaptation patterns - One or more resources can be configured in the resource set for channel measurement. Agreement At least support A2-2, i.e. one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern. - FFS: impact on CSI processing requirement Agreement For a CSI report config with L sub-configuration(s), support a framework that enables a UE to report N CSI(s) in one reporting instance where the N CSI(s) are associated with N sub-configuration(s) from L (where 1≤N≤L ) and each CSI corresponds to one sub-configuration. - For discussion purpose, N=1 refers to single-CSI while N>1 refers to multi-CSI. - For Semi-persistent/Aperiodic CSI reporting, support gNB trigger/indicate/activate report of N≤L CSIs where N>=1 - The maximum value of N and L are subject to UE capability - Further study how to address/minimize additional UE complexity The following bullet was not agreed due to objection from Apple and vivo - For Periodic CSI reporting, at least the case of N=L is supported where N>=1 Conclusion New CSI-RS resource (RE mapping) pattern is not introduced for R18 network energy savings purpose. - Note: CSI-RS resource (RE mapping) pattern above refers to a row in TS 38.211 Table 7.4.1.5.3-1 determining CSI-RS locations within a slot. Agreement For power domain adaptation, for CSI(s) reporting, support configuration of more than one power offset values for PDSCH relative to CSI-RS - FFS: impact on CSI processing requirement - FFS: details on configuration/indication of the power offset values - FFS: whether/how to additionally consider the case where CSI-RS power is changed Agreement For CSI feedback with CSI overhead/report payload reduction, further study whether/how to report a common value and/or a differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns, at least for the following o CRI o RI o PMI o CQI o FFS: L1-RSRP o Other (new) report quantity, if any Further study whether/how it is feasible/possible for the UE to skip the evaluations of some sub-configurations/adaptation patterns to reduce the burden at the UE Agreement For CSI report configuration, if L>1 in a CSI report configuration, at least the following can be included for each sub-configuration for Type 1 SD adaptation  N1, N2 for single-panel and N1, N2, Ng for multi-panel o FFS: details on explicit indication or implicit derivation  Port subset indication when A1-2 is used (if A1-2 is supported) o FFS: details on explicit indication or implicit derivation  FFS: rank restriction  FFS: codebook subset restriction  FFS: supported codebook types for PMI, e.g., Type-I or Type-II  FFS: report quantity  FFS: reportFreqConfiguration  FFS: Group identity of NZP CSI-RS resource(s) in a resource set for channel measurement when A1-1 is used For CSI report configuration for type 2 SD adaptation, further study under which cases sub-configurations may or may not be needed including sub-configuration content Agreement For power domain adaptation, support the following configuration(s) for CSI-RS resource configuration,  A1-2-power: one or more resources can be configured in a resource set within a resource setting and each resource can be associated with one or more power offset values  FFS: A1-1-power: a resource set with multiple resources is configured within a resource setting, where resources can have different power offset values  FFS: Details of how the different power offset values(s) are configured/indicated. Working Assumption Al-1-revised and A1-2-revised are supported • FFS: Which Type of SD adaptation A1-1-revised and A1-2-revised are applicable for Agreement For R18 NES, only legacy port configuration values (N1, N2) or (Ng, N1, N2) are supported. • FFS: Whether/what restriction for A1-1-revised and A-1-2-revised w.r.t number of ports Agreement For Semi-persistent/Aperiodic CSI reporting with 1≤N≤L, study what enhancements to the current DCI and MAC-CE mechanisms are needed for gNB triggering/indication/activation of the N CSI(s) in a reporting instance, where the N CSI(s) are associated with N sub-configuration(s) from L in a report config. Conclusion From RAN1 perspective, there is no action needed for the LS R1-2302288 from SA5 this time.

In this contribution, we share our views on spatial and power domain techniques for network energy saving.

- CSI resource

Following agreement related to association between CSI resource and spatial pattern is achieved at RAN1#112bis-e meeting.

Agreement Support configurability of NZP CSI-RS resource(s) for channel measurement within one resource setting corresponding to more than one spatial adaptation patterns with at least one of the following - A1-1-revised: a resource set with multiple resources is configured within a resource setting, where each resource is associated with only one spatial adaptation pattern - A1-2-revised: For a resource configured in a resource set within a resource setting, the resource can be associated with more than one spatial adaptation patterns - One or more resources can be configured in the resource set for channel measurement. Working Assumption Al-1-revised and A1-2-revised are supported • FFS: Which Type of SD adaptation A1-1-revised and A1-2-revised are applicable for

Firstly, it is better to confirm the working assumptions above as both alternatives should be supported to ensure the flexibility of spatial adaptation.

Secondly, for both alternatives of A1-1-revised and A1-2-revised above, the supportive on Type 1 and Type 2 adaptation is showed in Table 1 as follow. For A1-1-revised with each resource associated with only one spatial adaptation pattern, it can support both Type 1 and Type 2 adaptation. One CSI-RS resource is associated with one Type 1 or Type 2 pattern. Multiple CSI-RS resources are associated with multiple Type 1 or Type 2 patterns. The multiple CSI-RS resources could be in one or multiple CSI-RS resource set. For A1-2-revised with each resource associated with more than one spatial adaptation patterns, it can only support Type 1 adaptation but cannot support Type 2 adaptation. To support Type 1 adaptation, one CSI-RS resource can be associated with multiple Type 1 patterns. As one CSI-RS resource can just support one Type 2 pattern, A1-2-revised cannot support Type 2 adaptation with only one resource.

In real operation, it is not necessary for one gNB to support both Type 1 and Type 2 spatial adaptation at the same time. But from specification point of view, it is suggested that RAN1 should strive a common configuration design to support both Type 1 and Type 2 adaptations.

Table 1: Supportive of adaptation types of two resource association alternatives

Proposal 1:

- Confirm the working assumption to support both of Al-1-revised and A1-2-revised.

Proposal 2:

- RAN1 should strive a common configuration design to support both Type 1 and Type 2 adaptation.

- CSI report configuration

Following agreement related to CSI report configuration is achieved at RAN1#112bis-e meeting.

Agreement For a CSI report config with L sub-configuration(s), support a framework that enables a UE to report N CSI(s) in one reporting instance where the N CSI(s) are associated with N sub-configuration(s) from L (where 1≤N≤L ) and each CSI corresponds to one sub-configuration. - For discussion purpose, N=1 refers to single-CSI while N>1 refers to multi-CSI. - For Semi-persistent/Aperiodic CSI reporting, support gNB trigger/indicate/activate report of N≤L CSIs where N>=1 - The maximum value of N and L are subject to UE capability - Further study how to address/minimize additional UE complexity The following bullet was not agreed due to objection from Apple and vivo - For Periodic CSI reporting, at least the case of N=L is supported where N>=1

From our consideration, reporting N CSI(s) from L sub-configurations in one reporting instance can reduce the gNB configuration complexity. The feature should be supported not only for semi-persistent and aperiodic CSI reporting but also for periodic CSI reporting as periodic CSI reporting reduces the configuration attempts from gNB and always be supported in 3GPP from LTE to NR.

Furthermore, N<L is necessary in the realistic operation even for periodic CSI reporting. For example, if 2/4/8 port CSIs are configured by periodic CSI reporting, with operating with 8-port now, gNB would like to know whether it can switch to 4-port, then it could trigger CSIs of both 4-port and 8-port. In this case, 2-port CSI may be useless for the gNB. If it enables such flexibility to report N < L, the feedback overhead can be reduced.

Proposal 3:

- Periodic CSI reporting to support reporting N CSI(s) from L sub-configurations in one reporting instance should be supported.

  - Support N<=L to enable the configuration flexibility.

- Overhead reduction

Following agreement related to reporting with overhead reduction is achieved at RAN1#112bis-e meeting.

Agreement For CSI feedback with CSI overhead/report payload reduction, further study whether/how to report a common value and/or a differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns, at least for the following o CRI o RI o PMI o CQI o FFS: L1-RSRP o Other (new) report quantity, if any Further study whether/how it is feasible/possible for the UE to skip the evaluations of some sub-configurations/adaptation patterns to reduce the burden at the UE

It is already agreed that multiple CSI(s) will be reported in one reporting instance. Then we can explore the room of reporting payload reduction as some of the reporting contents between CSIs are the same or similar. In above agreement of last RAN1 meeting, three CSI overhead reduction approaches will be studied: common value, differential value, and joint coded value. In this section, we will provide our views on these approaches.

Table 2 shows the reporting contents of CSI quantities are the same or different across CSIs with spatial or power adaptation. In the table, the power adaptation is regarded as Type 3 adaptation. For Type 1 adaptation, CRI is the same for CSIs with different port numbers, then reporting one CRI across these CSIs is reasonable. For Type 3 adaptation, CRI, RI, and PMI may be the same for CSIs with different power levels, then only report one CRI, RI, PMI across these CSIs is reasonable. Furthermore, once the UE is configured to report common value, it should be indicated on which adaptation pattern/sub-configuration the UE derives the common value.

Table 2: contents of CSI quantities are same or different across CSIs with spatial/power adaptation

Proposal 4:

- The reporting of common value across same CSI quantity of different sub-configurations/adaptation should be supported.

  - Once the UE is configured to report common value, it should be indicated on which adaptation pattern/sub-configuration the UE derives the common value.

The approach of reporting the differential value can also reduce the CSI reporting payload. It can be applied for CQI reporting as the CQI gap between different adaptation pattern are not large and reporting differential value can save bits than reporting the whole value. In legacy CQI reporting, differential subband CQI is already supported. For R18 network energy saving, reporting differential wideband CQI between different sub-configurations/adaptation patterns can be supported. Furthermore, it needs an ordering approach for the reported CSIs in one reporting instance. According to the ordering, one of the CSIs containing whole wideband CQI and other CSIs contains differential wideband CQI. Furthermore, as there is one or two TBs with one or two wideband CQIs and number of TBs between adaptation pattern may be different, it needs to specific a rule on which TB of the reference CSI is used for calculating the differential wideband CQI.

Proposal 5:

- The reporting of differential value across same CSI quantity of different sub-configurations/adaptation should be applied only for wideband CQI.

  - An ordering approach is needed to indicate which CSI containing whole wideband CQI and which CSIs contains differential wideband CQI.

  - As number of TBs of different adaptation patterns may be different, it needs to specific a rule on which TB of the reference CSI is used for calculating the differential wideband CQI.

The approach of reporting the joint coded value can be applied for RI reporting. In Table 3, an illustration example of joint coded RI for Type 1 spatial adaptation is showed. In this example, gNB triggers CSIs for both 8-port and 4-port CSI-RSs. If independent RIs without overhead reduction is applied, 5 bits for RI reporting are needed with 3 bits for RI of 8 port CSI-RS and 2 bits for RI of 4 port CSI-RS. If joint coded RI is applied, only 3 bits are needed.

The joint coded RI can save the payload bits by excluding useless and low probability codepoints. In the example of Table 3, if the RI of 8-port CSI-RSs is rank=1, then there is no possibility of rank>1 for 4 port CSI-RS. These cases are useless codepoints. If rank=4 for 8-port CSI-RS, the probability of rank=4 for 4 port CSI-RS is very low. These cases are low probability codepoints. By joint coded RI, these useless and low probability codepoints can be excluded. When more CSIs are reported in one reporting instance, the more payload bits can be saved by using joint coded RI as more useless and low probability codepoints can be excluded.

Table 3: Example of joint coded RI for Type 1 spatial adaptation

Proposal 6:

- The joint coded value can be applied for RI reporting by excluding useless and low probability codepoints. For example, in case of reporting both 8-port and 4-port CSIs for Type 1 spatial adaptation,

  - The useless codepoints are like this: rank=1 for 8-port CSI-RS, and rank = 2/3/4 for 4 port CSI-RS

  - The low probability codepoints are like this: rank=4 for 8-port CSI-RS, and rank = 4 for 4 port CSI-RS

- Beam management enhancement

For measurement and data reception at UE side, a TCI state is configured with one or two associated RS resource(s) as QCL reference. UE could reuse some of the largescale properties, such as delay spread, doppler spread and average gain, of QCL reference for measurement and data reception. But with the dynamic spatial and power adaptation, the largescale property between the QCL reference (RS configured in TCI state) and QCL target (RS/PDCCH/PDSCH) may be changed then invalid. In case of invalidation of TCI, a fast TCI switch mechanism corresponding to dynamic spatial and power adaptation, should be investigated and supported.

Proposal 7:

- Configured TCI may be invalid due to dynamic spatial and power adaptation. An enhanced TCI switch mechanism corresponding to dynamic adaptation should be supported.

- Conclusion

In this contribution, we present the following proposals of spatial and power domain techniques for network energy saving.

Proposal 1:

- Confirm the working assumption to support both of Al-1-revised and A1-2-revised.

Proposal 2:

- RAN1 should strive a common configuration design to support both Type 1 and Type 2 adaptation.

Proposal 3:

- The periodic CSI reporting to support reporting N CSI(s) from L sub-configurations in one reporting instance should be supported.

  - Support N<=L to enable the configuration flexibility.

Proposal 4:

- The reporting of common value across same CSI quantity of different sub-configurations/adaptation should be supported.

  - Once the UE is configured to report common value, it should be indicated on which adaptation pattern/sub-configuration the UE derives the common value.

Proposal 5:

- The reporting of differential value across same CSI quantity of different sub-configurations/adaptation should be applied only for wideband CQI.

  - An ordering approach is needed to indicate which CSI containing whole wideband CQI and which CSIs contains differential wideband CQI.

  - As number of TBs of different adaptation patterns may be different, it needs to specific a rule on which TB of the reference CSI is used for calculating the differential wideband CQI.

Proposal 6:

- The joint coded value can be applied for RI reporting by excluding useless and low probability codepoints. For example, in case of reporting both 8-port and 4-port CSIs for Type 1 spatial adaptation,

  - The useless codepoints are like this: rank=1 for 8-port CSI-RS, and rank = 2/3/4 for 4 port CSI-RS

  - The low probability codepoints are like this: rank=4 for 8-port CSI-RS, and rank = 4 for 4 port CSI-RS

Proposal 7:

- Configured TCI may be invalid due to dynamic spatial and power adaptation. An enhanced TCI switch mechanism corresponding to dynamic adaptation should be supported.

References

- 3GPP, RAN1#112bis-e meeting, Chairman’s notes