3GPP TSG RAN WG1 #113			R1-2305625

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.12

Source: 	NTT DOCOMO, INC.

Title: 	Discussion on UE features for dedicated spectrum less than 5MHz

Document for:	Discussion and Decision

- Introduction

This contribution discusses the Rel-18 UE features for dedicated spectrum less than 5MHz.

- UE features for operating in spectrum allocations from approximately 3 MHz up to below 5 MHz

As stated in the WID [1], this WI aims to identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz, such as around 3 MHz BW @ n100 in Europe for FRMCS, 3MHz BW @ n8/n26 in the US for smart grid, and 3MHz BW @ n28 in Europe for PPDR. According to the WID, 5MHz CBW is assumed at least for FRMCS and optional 3MHz CBW can be also assumed for all use cases. Therefore, following two cases should be considered in this WI as shown in Fig.1.

- Case 1: 3 MHz CBW, transmission BW is less than or equal to CBW

- Case 2: 5 MHz CBW, transmission BW is less than CBW considering the coexistence with GSM-R

Case 1	Case 2

Figure 1.  Channel bandwidths considered in this WI

RAN1 got following reply LS from RAN plenary [2]:

RAN Plenary has discussed the possible transmission bandwidth options for 3 MHz and 5 MHz channel bandwidths for the spectrum allocations on the bands of interest in this work item, and concluded the following: - For the 3MHz channel bandwidth in band n100 (max channel utilization 15 PRBs as already agreed in RAN1/RAN4): - PBCH transmission bandwidth is 12 PRBs - CORESET#0 transmission bandwidth is to be decided by RAN1 - RAN1 is requested to consider whether the above also applies for other bands with 3MHz channel bandwidth, or whether the PBCH transmission bandwidth is 15 PRBs for such bands. - For the 5MHz channel bandwidth: - PBCH transmission bandwidth is 20 PRBs - CORESET#0 transmission bandwidth is to be decided by RAN1 - Other details (including sync raster details) are to be progressed in the WGs.

According to the above reply and RAN1 discussion/agreements so far, it has not been decided whether a unified PBCH BW is applied to every target band in this WI or not. Also, the only one UE feature which seems stable now is the support of punctured PBCH with 12 PRBs for 3MHz CBW in band n100 based on the reply from RAN plenary and following working assumption.

Working Assumption For transmission bandwidth[s] of <5MHz, for PBCH, in the case[s] that available PRBs for PBCH transmission are less than 20PRB, - PBCH based on RB-level puncturing (i.e., PBCH encoding is based on 20PRB. The encoded bits and DMRS are mapped to 20PRBs based on legacy SSB structure, and those PRBs that fall outside of available PRBs for PBCH transmission are punctured) - Note: No other optimization is needed

Therefore, following proposal is made. Details can be discussed once RAN1 makes further progress.

Proposal 1:

- Introduce FG for the punctured PBCH with 12 PRBs for 3MHz CBW [in band n100]

  - FFS whether this FG is applicable only for band n100 or any other target bands with 3MHz CBW in this WI

  - FFS whether to introduce additional components

  - FFS reporting type

  - Optional with capability signaling. For UE supporting 3MHz CBW [in band n100], UE must indicate this FG is supported.

References

[1] RP-230186, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Anterix, T-Mobile USA.

[2] R1-2302275, Reply LS on transmission bandwidths for NR on dedicated spectrum less than 5 MHz, RAN, Nokia

Appendix: List of RAN1 agreements

Coordination with RAN4

Agreement

In an LS to RAN4, in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW.

Agreement

RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as any input from RAN1 for RAN4’s answer to this question.

Agreement

Before getting RAN4 responses, RAN1 assume maximum transmission bandwidth, 15RBs or 16RBs for 3 MHz channel BW for evaluation and analysis.

Note: include agreement into the LS

Agreement

Before getting RAN4 responses, RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis.

Note: it does not mean indication signaling is needed.

Note: include this agreement into the LS.

Agreement

Including following 2 questions into the LS

Question 1: RAN1’s understanding is that in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW

Question 2: RAN1 have discussed aspects related to synch raster in the spectrum of interest. RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as if RAN4 needs any input from RAN1.

Decision: From Nov 17th session, the draft LS to RAN4 R1-2212898 is endorsed in principle with the modified questions as agreed above and all agreements and conclusions made in RAN1#111. Final LS is approved in R1-2212919.

Coordination with RAN

Agreement

- For transmission BWs for 3MHz and 5MHz channel BW, send an LS to RAN plenary for operators input for the following and RAN plenary guidance,

  - For 5MHz channel BW, whether to allow/support transmission BW(s) for physical channels of approximate 3 MHz up to below 5 MHz. What is the recommended transmission BW(s) to consider?

  - For 3MHz channel BW, whether to allow/support transmission BW(s) for physical channels of approximate 3 MHz. What is the recommended transmission BW(s) to consider?

- No intention to change the WID scope and TU

R1-2302157	DRAFT LS to RAN on transmission bandwidths for NR on dedicated spectrum less than 5 MHz	Moderator (Nokia)

Decision: Draft LS in R1-2302157 is endorsed in principle. Final LS is approved in R1-2302186.

PBCH

Agreement

For transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs.

- FFS which PRBs are used and how to use the PRBs

- Note: PRBs for PSS/SSS are not punctured.

Agreement

Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered,

- Opt.1: Power boosting

- Opt.2: Multiple PBCH receptions

- Opt.3: PBCH remapping

- Opt.4: PBCH payload reduction

- Opt.5: PBCH rate matching around the punctured PRBs

- Opt.6: no enhancement specified

Working Assumption

For transmission bandwidth[s] of <5MHz, for PBCH, in the case[s] that available PRBs for PBCH transmission are less than 20PRB,

  - PBCH based on RB-level puncturing (i.e., PBCH encoding is based on 20PRB. The encoded bits and DMRS are mapped to 20PRBs based on legacy SSB structure, and those PRBs that fall outside of available PRBs for PBCH transmission are punctured)

  - Note: No other optimization is needed

PDCCH

Agreement

For CORESET#0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, following options are for study,

- Opt.1: Existing configuration table for 15kHz SCS, 5MHz minimum channel BW (i.e., table 13-1 in TS38.213) is reused for configuration

- Opt.2: A new CORESET#0 configuration table is to be introduced for the configuration.

Agreement

Study whether and how to recover PDCCH detection performance of CORESET#0 for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered,

- Opt.1: Power boosting

- Opt.2: Non-interleaved CCE-to-REG mapping

- Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum

- Opt.4: New aggregation level(s) for fit in the spectrum

- Opt.5: PDCCH rate matching

- Opt.6.: no enhancement specified

CSI-RS

Conclusion

For transmission bandwidths of <5MHz for 3MHz channel bandwidth, for CSI-RS other than for RRM measurements, no enhancements are needed.

FFS: CSI-RS for RRM

PUCCH

Conclusion

No enhancements are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth,

- FFS: the necessity for PUCCH FH disabling.

PRACH

Conclusion

No enhancements are required for PRACH to operate NR on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth.

- Note: PRACH formats and configurations not fitting into the transmission BW are not applicable

Agreement

Short PRACH formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS are supported for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth.