3GPP TSG RAN WG1 #113		R1-2305691

Incheon, South Korea, May 22nd – May 26thth, 2023

Agenda Item:	9.2.1

Source:	Indian Institute of Tech (M), IIT Kanpur

Title:	Discussions on General Aspects of AI/ML Framework

Document for:	Discussion

# Introduction

This document includes discussions on the general aspects of the AI/ML framework for Release 18. Specifically, we focus on the different stages involved in the integration of AI/ML algorithms and their life cycle management. We also offer our views on terminology related to functionality and model identification.

In this section, we reproduce the relevant agreements from prior meetings.

RAN1 #112 bis-e [1]

Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. Conclusion From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. - When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. Agreement - Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities. - Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models. Working Assumption The definition of ‘AI/ML model transfer’ is revised (marked in red) as follows: AI/ML model transfer; Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.. Working Assumption Model selection; The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation.

RAN1 #112 [2]

Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

RAN1 #111 [3]

Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Functionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs Working Assumption Terminology; Description. Model identification; A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.. Terminology; Description. Functionality identification; A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality. Note: whether and how to indicate Functionality will be discussed separately. Working Assumption Terminology; Description. Model update; Process of updating the model parameters and/or model structure of a model. Model parameter update; Process of updating the model parameters of a model.

RAN1 #110 bis-e [4]

Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models FFS: other mechanisms Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

RAN1 #110 [5]

Agreement Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability

# Discussion

  - AI/ML Model Management

AI/ML models for a particular feature can be one-sided (either deployed at the UE or NW) or two-sided (one part is deployed at the NW and the other at the UE). One-sided models are trained and deployed independently. Two-sided models are trained jointly but deployed independently.

In this section, we present our understanding of the different stages of the AI/ML general framework (Block Diagram in Fig 1).

The initial significant stage of developing AI/ML algorithms is Model Training (Fig 1 stages 1a, 1b, 1c). Collection of data that encompasses various conditions (for example: channel scenarios, SNRs etc) is an integral part of training.

Figure 1: Block diagram representation of different stages of AI/ML General Framework

Trained models are stored for future deployment and the model training statistics could be used for model management purposes (Fig 1 stage 2). Based on applicable conditions, a model is selected for activation (Fig 1 stages 3 and 4). The activated models are then deployed (Fig 1 stage 5). Deployment could involve model transfer, if training occurred elsewhere. Deployed models are available for inference.

In the case of one-sided models, stages 1 through 5 result in the model being available for real-time inference (Fig 1 stage 6) at either the NW or the UE. In the case of two-sided models, the model will be available at both the NW and UE. Inference output could be shared with model management for the purpose of model monitoring (Fig 1 stage 7).

Observation 1: Model performance can be monitored either on demand from NW/UE or as part of general life cycle management.

In our view, model monitoring involves the following steps:

- Request for monitoring from NW/UE (Fig 1 stage 8a)

- Selection of model(s) for monitoring (Fig 1 stage 8b, 8c, 8d)

- Collection of Monitoring Data (Fig 1 stage 9)

- Inference of the monitored models using monitoring data (Fig 1 stage 10, 11).

The inference output is analysed using the appropriate KPIs at the model management level and a decision on model update, switching, activation/deactivation or fallback is made (Fig 1 stage 12). The revised model is then deployed for real-time inference (Fig 1 stage 13).

Based on the above description, we put forward the following proposals:

Proposal 1: Model management can be initiated by either the NW or the UE for a particular model. Upon request from the NW or UE, Model Management will select the model(s) for monitoring and collect the appropriate data.

Proposal 2: Model monitoring can be conducted with new data (which we refer to as monitoring data). Unlike inference data, monitoring data is one in which the ground truth is known. It is our view that additional testing with monitoring data can augment the analysis of inference results and KPIs.

2.2 Functionality Identification and Model Identification

In this section we offer our observations on the definitions of Functionality and Model Identification.

Observation 2: Functionality Identification for one-sided models (deployed at the UE) requires only successful reception of UE capability report at the NW. Functionality Identification for two-sided models requires both UE and NW capabilities.

Functionality identification for one sided models (deployed at the UE):

- Step 1: UE reports conditions through the UE capability report. At this point, NW accepts these reports without any consideration of its own capabilities or interests.

- Step 2: NW configures functionalities based on conditions reported in step 1 and NW interests.

Functionality identification is complete after step 1.

Functionality identification for two sided models:

- Step 1: UE reports conditions through the UE capability report.

- Step 2: NW configures functionalities based on conditions reported in step 1 AND also its own capabilities and interests.

Functionality identification is complete after BOTH steps 1 and 2.

We note that AI/ML functionality identification is essential for both one-sided models (deployed at the UE) and the UE-part of two-sided models. In contrast, model identification is required only for the UE-part of two-sided models. It is our view that for one-sided models (deployed at the UE), model identification is unnecessary at the NW level.

Observation 3: For the UE-part of two-sided AI/ML models, after functionality identification is complete, the NW performs model identification of one or more model(s) for the given AI/ML functionality.

Observation 4: It has been agreed that models are identified by model ID at the NW. The UE then indicates supported AI/ML models. We believe this to be true only for the UE-part of two-sided models.

Proposal 3: In the event that the NW doesn’t initiate the model identification process, the UE can request the NW for model identification.

Proposal 4: In addition to the NW identifying the models (Observation 4), we believe that for the UE-part of two-sided models, it is also possible for the UE to directly report the Model IDs along with the conditions (under which a functionality could use an AI/ML based approach) in the capability report.

Then the NW can identify supported models based on its own capabilities. The NW then signals this to the UE. We note that Model IDs for one-sided models (deployed at the UE) need not be available at the NW.

  - Functionality/Model-ID-based Life Cycle Management

In this section we describe our views and understanding of the Functionality and Model-ID based Life Cycle Management Processes. We are specifically focussed on UE side models (One-sided models deployed at the UE and UE-side of two-sided models).

    - Functionality-based Life Cycle Management

Our understanding of the Functionality-based Life Cycle Management is given in Fig 2.

Observation 5: Functionality based LCM starts after all the applicable AI/ML functionalities have been identified.

Once a functionality has been identified and configured, that functionality can be supported by one or more AI/ML models – each with a model ID.

Observation 6: Model-ID-based LCM is also a part of functionality-based LCM.

The key part of Model-ID based LCM is model monitoring which results in decisions on model activation/deactivation/switching/fallback. Model activation/deactivation/switching results in an update in the set of applicable models for a given functionality. Fallback results in one or more functionality/functionalities not being supported by AI/ML approaches.

Proposal 5: There should be a distinction between AI/ML functionalities and Non-AI/ML functionalities.

It is our view that the term fallback refers to the act of moving from an AI/ML algorithm to a Non-AI/ML algorithm for a particular functionality. On the other hand, switching refers to the act of moving from one AI/ML model to another for a particular functionality.

Figure 2 Block diagram representation of functionality based life cycle management

    - Model-ID-based Life Cycle Management

It has been agreed that Model-ID-based LCM, models are identified at the NW, and the NW/UE may select, activate, deactivate or switch between individual AI/ML models via the Model ID. With this context, we present our understanding of Model-ID-based LCM below and in Fig 3.

Figure 3 Block diagram representation of model ID-based life cycle management

Observation 7: In Model-ID-based LCM, the first step is the selection of model(s) for monitoring. This can be triggered by a request from the NW or UE. The(se) model(s) could either be in a deployed or undeployed state.

Observation 8: The second step of Model-ID-based LCM is monitoring-data collection and inference. As described in Sec 2.1, it is our view that model monitoring (inference) should be done using a dedicated dataset in which the ground truth values are known.

Observation 9: The terms update, activation, deactivation and switch apply only at the model level.

Inference output and statistics resulting from the monitoring process are analysed using the appropriate KPIs to make a decision on the next steps of the Model-ID-based LCM viz., update, activation, deactivation, switch, or fallback. We reiterate that update, activation deactivation and switch are strictly at the model level. Fallback indicates the act of moving from an AI/ML approach to a non-AI/ML approach for the given functionality.

Observation 10:  In Model-ID-based LCM, the model selection (and/or switching) would require monitoring one or more applicable models.

Observation 11: For one-sided models (deployed at the UE), the Model-ID-based LCM can be triggered by the UE/NW and the model activation, deactivation, fallback decision can be taken by UE/NW based on the monitoring results.

# Conclusion

In this contribution, we have expanded upon our previous contributions [6] and previous agreements to provide a discussion on the general aspects of the AI/ML framework for Rel 18. The discussion has resulted in the following observations and proposals.

Observation 1: Model performance can be monitored either on demand from NW/UE or as part of general life cycle management.

Observation 2: Functionality Identification for one-sided models (deployed at the UE) requires only successful reception of UE capability report at the NW. Functionality Identification for two-sided models requires both UE and NW capabilities.

Observation 3: For the UE-part of two-sided AI/ML models, after functionality identification is complete, the NW performs model identification of one or more model(s) for the given AI/ML functionality.

Observation 4: It has been agreed that models are identified by model ID at the NW. The UE then indicates supported AI/ML models. We believe this to be true only for the UE-part of two-sided models.

Observation 5: Functionality based LCM starts after all the applicable AI/ML functionalities have been identified.

Observation 6: Model-ID-based LCM is also a part of functionality based LCM.

Observation 7: In Model-ID-based LCM, the first step is the selection of model(s) for monitoring. This can be triggered by a request from the NW or UE. The(se) model(s) could either be in a deployed or undeployed state.

Observation 8: The second step of Model-ID-based LCM is monitoring-data collection and inference. As described in Sec 2.1, it is our view that model monitoring (inference) should be done using a dedicated dataset in which the ground truth values are known.

Observation 9: The terms update, activation deactivation and switch apply only at the model level.

Observation 10:  In Model-ID-based LCM, the model selection (and/or switching) would require monitoring one or more applicable models.

Observation 11: For one-sided models (deployed at the UE), the Model-ID-based LCM can be triggered by the UE/NW and the model activation, deactivation, fallback decision can be taken by UE/NW based on the monitoring results.

Proposal 1: Model management can be initiated by either the NW or the UE for a particular model. Upon request from the NW or UE, Model Management will select the model(s) for monitoring and collect the appropriate data.

Proposal 2: Model monitoring can be conducted with new data (which we refer to as monitoring data). Unlike inference data, monitoring data is one in which the ground truth is known. It is our view that additional testing with monitoring data can augment the analysis of inference results and KPIs.

Proposal 3: In the event that the NW doesn’t initiate the model identification process, the UE can request the NW for model identification.

Proposal 4: In addition to the NW identifying the models (Observation 4), we believe that for the UE-part of two-sided models, it is also possible for the UE to directly report the Model IDs along with the conditions (under which a functionality could use an AI/ML based approach) in the capability report.

Proposal 5: There should be a distinction between AI/ML functionalities and Non-AI/ML functionalities.

# References

[1] Chair’s Notes, RAN1 #112 bis-e, April 2023

[2] Chair’s Notes, RAN1 #112, February-March 2023

[3] Chair’s Notes, RAN1 #111, November 2022

[4] Chair’s Notes, RAN1 #110 bis-e, October 2022

[5] Chair’s Notes, RAN1 #110, August 2022

[6] R1-2301664 Identifying Procedures for General Aspects of AIML Frameworks, IIT Madras, CEWiT, IIT Kanpur, RAN1 #112 Feb-March 2023