3GPP TSG RAN WG1 #113	R1-2305720

Incheon (KR), 22nd – 26th May, 2023

Agenda item:		9.16.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for Network Controlled Repeaters

Document for:		Discussion and Decision

# Introduction

RAN1 started discussion Rel-18 UE features for selected topics in RAN1#112bis-e [1], one of the work items discussed in that meeting was Network Controlled Repeaters, with the summary captured in [2]. In this contribution we discuss the open questions on the NCR WI related UE features.

# Discussion

FG43-1 components

- Support of fixed beam for C-link/backhaul link

- Support of TDMed UL transmission of C-link and backhaul link

- Support of ON-OFF operation for NCR-Fwd based on access link beam indication

- Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link

- [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link

- Support of beam correspondence of the DL/UL of the access link at NCR-Fwd

Proposal 1: On component 5 of FG43-1, remove the square brackets on “Support of “

FGs 43-2, 43-3, 43-4

| 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one |
| --- | --- | --- | --- |
| 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values |
| 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one |

It would be attractive to make both periodic and aperiodic beam indication components of FG 43-1, but as it is risky to have two alternatives in the basic component, it may nevertheless be better to limit the basic capability to periodic beam reporting.

Proposal 2: Make periodic beam indication for access link (FG 43-2) a component of 43-1

The component 2 should be understood as the time for the UE to process the DCI. In Rel-15 RAN1 defined two capabilities for PUSCH scheduling, which could be considered an upper bound for beam switching as in addition to the DCI modulation, decoding and parsing it also includes the PUSCH processing before the transmission can start.

| SCS | PUSCH timing capability 1 [symbols] | PUSCH timing capability 2 [symbols] |
| --- | --- | --- |
| 15 kHz | 10 | 5 |
| 30 kHz | 23 | 5.5 |
| 60 kHz | 23 | 11 (applicable for FR1 only) |
| 120 kHz | 36 | n/a |
| 480 kHz | 144 | n/a |
| 960 kHz | 288 | n/a |

Proposal 3: Adopt the PUSCH timing capability 1 and 2 as defined in TS38.214 as the basis for SCS-specific supported slot offset values for component 2 of aperiodic beam indication FG 43-3

# Conclusion

In this contribution we presented our views on the remaining open items of the Network Controlled Repeaters WI UE features , and we have made the following proposals:

Proposal 1: On component 5 of FG43-1, remove the square brackets on “Support of “

Proposal 2: Make periodic beam indication for access link (FG 43-2) a component of 43-1

Proposal 3: Adopt the PUSCH timing capability 1 and 2 as defined in TS38.214 as the basis for SCS-specific supported slot offset values for component 2 of aperiodic beam indication FG 43-3

# References

- R1-2304282, “Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e,” Moderators (AT&T, NTT DOCOMO, INC.), Apr. 2023

- R1-2304216, “Session Notes of AI 9.17.4”, Ad-Hoc Chair (AT&T)

# Annex: 	 Post-RAN1#112bis NCR FGs

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4a | Beam index updates for semi-persistent beam indication | 1.Support of [temporary] override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1, 43-4 | N/A | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Yes | FFS: whether to merge this FG with 43-4 | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |