3GPP TSG RAN WG1 #113	R1-2305726

Incheon (KR), 22nd – 26th May, 2023

Agenda item:		9.16.12

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on UE features for NR support for dedicated spectrum less than 5MHz for FR1

Document for:		Discussion and Decision

# Introduction

RAN1 started discussion Rel-18 UE features for selected topics in RAN1#112bis-e [1]. In RAN1#113 an extended set of topics will be included, where rapporteur’s input on UE features for each WI will be presented for discussion as well. Hence, in this contribution we provide our initial views on UE features for NR support for dedicated spectrum less than 5MHz for FR1 WI [2].

# Discussion

The following new FGs can be identified:

| Feature group | Components |
| --- | --- |
| Support for 3 MHz channel BW | 1. Reception of [12 or 15] RB PBCH. 2. [Reception of 12 or 15 RB CORESET]. 3. [Disabling of PUCCH Frequency hopping] |
| [Support for reduced PRB allocation for 5 MHz channel BW] | [Support for CORESET0 of [XX] PRBs] |

1st FG, Component 1: RAN agreement [3] for band n100 is 12 PRBs, but it could still be 15 PRBs for some other bands; subject to discussions in RAN1#113 AI 9.13

1st FG, components 2 and 3, as well as the complete 2nd FG being subject to further discussions in AI 9.13 in RAN1#113 before it is possible to make the final decision on them.

The FGs should be defined per band, as there are no band combinations due to no CA support with sub-5 MHz bands as per the RAN guidance in [4].

# Conclusion

In this contribution the following two FGs are proposed:

Proposal 1: Adopt the following FGs for NR support for dedicated spectrum less than 5MHz for FR1

| Feature group | Components |
| --- | --- |
| Support for 3 MHz channel BW | 1. Reception of [12 or 15] RB PBCH. 2. [Reception of 12 or 15 RB CORESET]. 3. [Disabling of PUCCH Frequency hopping] |
| [Support for reduced PRB allocation for 5 MHz channel BW] | [Support for CORESET0 of [XX] PRBs] |

Proposal 2: Make all the FGs of the NR support for dedicated spectrum less than 5MHz for FR1 WI “per band”

# References

- R1-2304282, “Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e,” Moderators (AT&T, NTT DOCOMO, INC.), Apr. 2023

- RP-230186, “Revised WID: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, Anterix, T-Mobile USA.

- RP-230780, “Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz”, RAN (Nokia)

- RP-230781, “Response to LS from RAN4 on spectrum less than 5 MHz”, RAN (Nokia)