3GPP TSG RAN WG1 #113	R1-2305789

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.2.2.1

Source: 	ETRI

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion

Introduction

This contribution presents ETRI’s views on the evaluation of AI/ML for CSI feedback enhancement use case for the AI/ML for NR Air Interface study [1].

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: o CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] o Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] o Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] …

Discussion

Evaluation methodologies for AI/ML-based CSI prediction sub use case

In last RAN #111 meeting, time domain CSI prediction is selected as sub-use case for AI/ML CSI enhancement [2]. In this section, we discuss simulation results for the CSI prediction sub-use case to evaluate performance of AI/ML models.

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer futher till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

CSI prediction represents that an AI/ML model inferences T number of future CSI information (RAW channel matrix or eigenvectors) based on K number of previous CSI information. The structure of CSI prediction described in the following Figure 1.

Figure 1. The structure of CSI prediction

In the case of UE sided CSI prediction which is selected as sub-use case, both eigenvectors and RAW channel matrix can be considered as input of AI/ML model. Because, the UE directly measures channel matrix by receiving CSI-RS signals from gNB. In view of eigenvectors as input of AI/ML model, eigenvectors may have smaller dimensions and data size compared to raw channel matrix as input of AI/ML model. Thus, computational complexity of AI/ML model could be decreased. However, due to loss of information during making eigenvectors from RAW channel matrix, performance of AI/ML model could be degraded. At this time, it is important to check the degree of performance improvement in AI/ML model compared to the baseline. Therefore, RAW channel matrix is used as the AI/ML model input in our simulations.

Evaluations on AI/ML-based CSI compression

In this section, we provide initial evaluation results on CSI compression sub use case based on SLS (System level simulations).

Evaluation assumption

For evaluation of AI/ML based CSI compression sub use case, the wireless channel data generated using the SLS for AI/ML based CSI compression sub use case. The parameters used for the SLS are provided in Table 1.

Table 1. Parameters for the SLS

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Scenario | Dense Urban (Macro only) |
| BWP | 48 RBs |
| Subcarrier spacing | 15 kHz |
| Subband/PRG size | 4 RBs |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx Power | 44dBm for 20MHz |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| Slot/non-slot | 14 OFDM symbol slot |
| SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth | 20 MHz |
| Number of layers | 1 |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| Channel estimation | Ideal |

Autoencoder based AI model for CSI compression

For AI/ML based CSI compression sub use case, AI model at UE (Encoder) gets wireless channel information as the input and generates compressed feedback information as the output, and AI model at gNB (Decoder) gets compressed feedback information as the input and generates the original wireless channel information as the output.

Autoencoder architecture is deployed for the evaluation of CSI compression. The architecture of Autoencoder is in Figure 2.

Figure 2. Architecture of the Autoencoder

We set eigenvectors of subbands as input of the encoder and output of the decoder in the evaluation. The number of latent variables is M, and Q bits are used to quantize each latent variable, i.e., the size of CSI feedback information is MQ bits.

The Encoder and Decoder in Autoencoder architecture are deployed using neural networks and design choices of the neural networks can be made with considerations of performance and complexity. We use Transformer-based neural networks for both the encoder and decoder. Figure 3 shows the Transformer-based model for AI/ML based CSI compression.

Figure 3. Transformer-based model for AI/ML based CSI compression

In order to train the AI Model, training samples are collected by the SLS. The parameters of AI Model and training are in Table 2.

Table 2. Parameters of AI Models

| Parameter | Value |
| --- | --- |
| Total number of samples | 57e4 |
| Portion of validation samples | 0.1 |
| Batch size | 512 |
| Total number of epoches | 512 |
| Learning algorithm | Adam |
| Learning rate | 0.0003 |
| Loss function | NMSE and SGCS |

Summary of evaluation results of AI/ML-based CSI compression

According to the initial template discussed at the last meeting, the initial evaluation results are summarized as follows.

Table 3. Evaluation results for CSI compression on 1-on-1 joint training (Intermediate KPI)

| Assumptions |  | ETRI#1 |
| --- | --- | --- |
| CSI generation part | AI/ML model backbone | Transformer |
|  | Pre-processing | SVD |
|  | Post-processing | N/A |
|  | FLOPs/M for model | 179 |
|  | FLOPs/M for pre/post processing |  |
|  | Number of parameters/M | 7.15 |
|  | Storage /Mbytes |  |
| CSI reconstruction part | AI/ML model backbone | Transformer |
|  | Pre-processing | N/A |
|  | Post-processing | N/A |
|  | FLOPs/M for model | 179 |
|  | FLOPs/M for pre/post processing |  |
|  | Number of parameters/M | 7.15 |
|  | Storage /Mbytes |  |
| Common description | Input type | Eigenvector |
|  | Output type | Eigenvector |
|  | Quantization /dequantization method | Case 2-1 (Fixed SQ) |
|  | Rank/layer adaptation settings for rank>1 | N/A |
| Dataset description | Train/k | 570 |
|  | Test/k | 1 |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) | Float32 |
|  | Overhead reduction compared to Float32 if high resolution quantization of ground-truth CSI is applied | N/A |
| [Other assumptions/settings agreed to be reported] |  | N/A |
| Benchmark |  | R16 eTypeII |
| Benchmark assumptions, e.g., CSI overhead calculation method (Optional) |  |  |
| SGCS of benchmark, layer 1, rank=1/2 (α=1) or rank=4 (α=2) | X: <=80/α bits | 0.717 |
|  | Y: 100/α bits-140/α bits | 0.79 |
|  | Z: >=230/α bits | 0.831 |
| SGCS-Absolute value(gain% over benchmark), layer 1, rank=1/2 (α=1) or rank=4 (α=2) | X: <=80/α bits | 0.746 (4.04%) |
|  | Y: 100/α bits-140/α bits | 0.805 (1.9%) |
|  | Z: >=230/α bits | 0.86 (3.6%) |

Observation 1: In the CSI compression sub-use case, with a Transformer based Autoencoder, there are 1.9 ~ 4% improvements in terms of SGCS compared to the baseline (R16 eTypeII) for payload sizes X/Y/Z.

Evaluations on AI/ML-based CSI prediction

In this section, we provide basic evaluation results on the CSI prediction sub-use case based on a dataset derived from our SLS platform.

Evaluation assumption and dataset generation

For the evaluation of AI/ML based CSI prediction sub-use case, wireless channel datasets are generated by the SLS platform. The detailed parameters used for evaluation are described as following Table 4

Table 4. Parameters for channel generation

| Parameter | Value |
| --- | --- |
| Scenario | Urban Macro |
| Carrier Frequency | 4GHz |
| Inter-BS distance | 200m |
| Channel Model | NR Uma |
| Subcarrier spacing | 15kHz |
| Antennas at gNB | 32 ports (8,8,2,1,1,2,8) |
| Antennas at UE | 4 ports (1,2,2,1,1,2,2) |
| UE distribution | 100% outdoor |
| UE velocity | 30 km/h |

Details of generated datasets in preliminary simulation are shown in the below Table 5. We generate channel sample of 570 UEs during 500ms with CSI-RS interval 5ms. We utilize 90% of samples as training datasets and other 10% samples are used as test datasets

Table 5. Description of datasets

| Parameter | Value |
| --- | --- |
| CSI-RS periodicity | 5ms |
| Simulation time | 500ms |
| Number of UEs | 570 |
| Number of RB | 52 |
| Number of subblock | 12 |
| Number of samples | 53,500 |
| Training/Test sets split | 90% / 10% |

AI/ML model for CSI prediction

In our evaluation, two AI/ML models for CSI prediction are simulated. First AI/ML model for CSI prediction utilizes Conv-LSTM[3] as the backbone. Conv-LSTM is modified basic LSTM structure by including convolution operation. Specifically, in the internal operation of LSTM cell, convolution layer is used instead of fully connected layer in order to better capture spatial information. The detailed structure of AI/ML model based on Conv-LSTM is shown in below Figure 4. This AI/ML model for prediction has 178.71M parameters and 1481.87M FLOPs of prediction

Figure 4. Structure of AI/ML model based on Conv-LSTM

And, second AI/ML model for CSI prediction utilizes MLP-Mixer[4] as the backbone. As the name implies, the MLP-Mixer is simply composed of MLP (multi-layer perceptrons). The detailed structure of AI/ML model based on MLP-Mixer is shown in below Figure 5. Original channel images(inputs) are separated into patches and patches are entered them in a sequential order. In mixer layers, MLPs are repeatedly applied across either spatial locations or feature channels. This AI/ML model for prediction has 8.77M parameters and 738.87M FLOPs of prediction

Figure 5. Structure of AI/ML model based on MLP-mixer

The parameters of AI/ML model and training are in Table 6. In the input dimension, K means the number of the observed channel matrix. And, in the output dimension, T means the number of the predicted channel matrix. For simplicity, we perform simulations considering only one Rx antenna.

Table 6. Parameters of AI/ML model

| Parameter | Value |
| --- | --- |
| Backbone | MLP-Mixer |
| Input / Output type | RAW channel matrix |
| Input dimension | K x 2 x 32 x 52 |
| Output dimension | T x 2 x 32 x 52 |
| Batch size | 128 |
| Number of epochs | 100 |
| Optimizer | Adam |
| Initial learning rate | 0.001 |
| Loss function | MSE |

Evaluation results on CSI prediction

In this chapter, we show evaluation results on CSI prediction with sample and hold baseline which stores the nearest historical CSI. We evaluate intermediate KPIs such as SGCS. Firstly, we simulate 5 historical CSI information as the input of AI/ML model to predict 3 future CSIs. The results show that the AI/ML based prediction method outperforms the baseline. And, it can be confirmed that the performance of AI/ML model is superior in the results of distant times. In other words, the performance of the baseline rapidly degrades over time.

Table 7. Evaluation result on CSI prediction with 5 historical CSI

|  | 5ms CSI |  | 10ms CSI |  | 15ms CSI |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | AI/ML model | Baseline | AI/ML model | Baseline | AI/ML model | Baseline |
| SGCS (Gain %) | 0.8785 (10.43%) | 0.7956 | 0.8006 (8.75%) | 0.7362 | 0.7524 (7.62%) | 0.6991 |

Observation 2: AI/ML based CSI prediction improves performance compared to the baseline.

Evaluation results changing the number of historical CSI

The next results show the performance of AI/ML based CSI prediction while changing the number of CSI information used as input of AI/ML model. According to the results, if more CSI inputs are given, the performance of AI/ML prediction model slightly increases.

Table 8. Evaluation result on AI/ML based CSI prediction with various number of historical CSI

|  | 5ms CSI |  | 10ms CSI |  | 15ms CSI |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | 5 samples | 8 samples | 5 samples | 8 samples | 5 samples | 8 samples |
| SGCS (Gain %) | 0.8785 (10.43%) | 0.8922 (12.32%) | 0.8006 (8.75%) | 0.8142 (11.38%) | 0.7524 (7.62%) | 0.7645 (10.28%) |

Observation 3: AI/ML based CSI prediction improves performance as the number of historical input CSIs increase.

Evaluation results using eigenvectors as input of model

The next results show the performance of AI/ML based CSI prediction using eigenvectors as input of AI/ML model instead of RAW channel matrix. As shown in the results, AI/ML based prediction method outperforms the baseline. However, the degree of improvement of AI/ML model as input of eigenvectors is slightly reduced compared to the AI/ML model as input of the RAW channel matrix.

Table 9. Evaluation result on CSI prediction using eigenvectors as input

|  | 5ms CSI |  | 10ms CSI |  | 15ms CSI |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type | AI/ML model | Baseline | AI/ML model | Baseline | AI/ML model | Baseline |
| SGCS (Gain %) | 0.8284 (2.24%) | 0.8103 | 0.6812 (3.91%) | 0.6556 | 0.5688 (3.39%) | 0.5502 |

Observation 4: Performance improvement of AI/ML based CSI prediction as input of eigenvectors is slightly reduced compared to the AI/ML model as input of RAW channel matrix.

Summary on evaluation results of AI/ML-based CSI prediction

In this chapter, according to the template without generalization discussed at the last meeting, the simulation results described in the previous sections are summarized as follows.

Table 10. Evaluation results for CSI prediction without model generalization/scalability

|  |  | ETRI (Section 3.2.3) | ETRI (Section 3.2.5) |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone | Conv-LSTM | Conv-LSTM |
|  | [Pre-processing] | Min-Max | Min-Max |
|  | [Post-processing] | Min-Max | Min-Max |
|  | FLOPs/M | 1,481.87M | 341.97M |
|  | FLOPs/M for pre/post processing |  |  |
|  | Parameters/M | 178.71M | 178.71M |
|  | [Storage /Mbytes] | - | - |
|  | Input type | RAW channel matrix | Eigenvector |
|  | Output type | RAW channel matrix | Eigenvector |
| Assumption | UE speed | 30 Km/h | 30 Km/h |
|  | CSI feedback periodicity | 5ms | 5ms |
|  | Observation window (number/distance) | 5 / 5ms | 5 / 5ms |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) | 3 / 5ms / 5ms | 3 / 5ms / 5ms |
|  | Whether/how to adopt spatial consistency | - | - |
|  | Codebook type for CSI report |  |  |
| Dataset size | Train/k | 48K | 48K |
|  | Test/k | 5K | 5K |
| Benchmark 1 |  | Nearest historical CSI w/o prediction | Nearest historical CSI w/o prediction |
| SGCS of benchmark 1 (1,…N, N is number of prediction instances) |  | 0.7956, 0.7362, 0.6991 | 0.8103, 0.6556, 0.5502 |
| SGCS-Absolute value/gain% over benchmark 1 (1,…N, N is number of prediction instances) |  | 0.8785 / 10.43%, 0.8006 / 8.75%, 0.7524 / 7.62% | 0.8284 / 2.24% 0.6812 / 3.91% 0.5688 / 3.39% |

Generalization performances of AI/ML-based CSI prediction

In the last RAN1 meeting, FL summarized potential issues related to the generalization of CSI prediction as follows [5].

- Drawing observations - The current plan is to treat the follow aspects: - CSI prediction - Basic performance gain over benchmark#1 and benchmark#2 and trends (performance trend for different observation window settings, prediction window settings, UE speeds, etc.): KPIs include intermediate KPI and eventual KPI. - Generalization (UMa/UMi/InH, carrier frequency, UE speeds).

Generalization performance over UE speeds

In this chapter, we provide some evaluation results on generalization performances over various UE speeds. The AI/ML model is trained using a training dataset of a certain UE speed and this model is tested to the dataset of different speeds. According to the simulation results, changes in the speed of UE significantly degrade the performance of AI/ML model.

Table 11. Evaluation result on CSI prediction with various UE speeds

| SGCS | Testing (UE speed) |  |  |  |
| --- | --- | --- | --- | --- |
| Training (UE speed) | 30km/h | 60km/h | 90km/h | 120km/h |
| 30km/h | 0.8841 | 0.7364 | 0.6330 | 0.4807 |
| 60km/h | 0.7061 | 0.7987 | 0.6411 | 0.5100 |
| 90km/h | 0.6334 | 0.6418 | 0.7219 | 0.5090 |
| 120km/h | 0.4921 | 0.5278 | 0.5126 | 0.7273 |

To overcome this performance degradation, we trained AI/ML model with mixed datasets which equally combined different UE speeds. First mixed dataset consists of samples of UE speed 30km/h, 60km/h and 120km/h. Second mixed dataset consists of samples of UE speed 30km/h, 60km/h, 90km/h. The results show that the performance degradation is alleviated compared to the result of AI/ML model trained on a certain speed.

Table 12. Evaluation result on CSI prediction with various UE speeds using the mixed dataset

| SGCS | Testing (UE speed) |  |  |
| --- | --- | --- | --- |
| Training | 30km/h | 60km/h | 120km/h |
| Mixed datasets #1 | 0.7661 | 0.6030 | 0.7127 |
| Training | 30km/h | 60km/h | 90km/h |
| Mixed datasets #2 | 0.6874 | 0.7578 | 0.7102 |

Observation 5: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of UE speeds.

Observation 6: Mixed datasets of different UE speeds can mitigate performance degradation compared to datasets of single UE speed.

Proposal 1: Further evaluate the AI/ML based CSI prediction over various UE speeds to overcome performance degradation in untrained UE speed.

Generalization performance over Carrier Frequencies

In this chapter, we provide some evaluation results on generalization performances over various carrier frequencies. The AI/ML model is trained using a training dataset of a certain UE speed and this model is tested to the dataset of different carrier frequencies. According to the simulation results, changes in carrier frequency significantly degrade the performance of AI/ML model.

Table 13. Evaluation result on CSI prediction with various carrier frequencies

| NMSE [dB] | Testing (Carrier Freq.) |  |  |
| --- | --- | --- | --- |
| Training (Carrier Freq.) | 2GHz | 3GHz | 4GHz |
| 2GHz | -7.611 | -0.827 | -0.026 |
| 3GHz | -0.394 | -5.998 | 2.942 |
| 4GHz | -1.497 | 1.612 | -4.587 |

To overcome this performance degradation, we trained AI/ML model with a mixed dataset which equally combined different carrier frequencies (2GHz, 3GHz, 4GHz). The results show that the performance degradation is alleviated compared to the result of AI/ML model trained on a certain speed.

Table 14. Evaluation result on CSI prediction with various carrier frequencies using the mixed dataset

| NMSE [dB] | Testing (Carrier Freq.) |  |  |
| --- | --- | --- | --- |
| Training (Carrier Freq.) | 2GHz | 3GHz | 4GHz |
| Mixed datasets | -6.583 | -4.860 | -2.143 |

Observation 7: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of carrier frequencies.

Observation 8: Mixed datasets of different carrier frequencies can mitigate performance degradation compared to datasets of single carrier frequency.

Proposal 2: Further evaluate the AI/ML based CSI prediction over various carrier frequencies to overcome performance degradation in untrained carrier frequency.

Summary on generalization results of AI/ML-based CSI prediction

In this chapter, according to the template with model generalization discussed at the last meeting, the simulation results described in the previous sections are summarized as follows.

Table 15. Evaluation results for CSI prediction with model generalization over UE speeds

|  |  | ETRI #1 | ETRI #2 | ETRI #3 | ETRI #4 |
| --- | --- | --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone | Conv-LSTM | Conv-LSTM | Conv-LSTM | MLP-Mixer |
|  | [Pre-processing] | Min-Max | Min-Max | Min-Max | Min-Max |
|  | [Post-processing] | Min-Max | Min-Max | Min-Max | Min-Max |
|  | FLOPs/M for model | 1,481.87M | 1,481.87M | 1,481.87M | 1,481.87M |
|  | FLOPs/M for pre/post processing | - | - | - | - |
|  | Parameters/M | 178.71M | 178.71M | 178.71M | 178.71M |
|  | [Storage /Mbytes] | - | - |  |  |
|  | Input type | RAW channel matrix | RAW channel matrix | RAW channel matrix | RAW channel matrix |
|  | Output type | RAW channel matrix | RAW channel matrix | RAW channel matrix | RAW channel matrix |
| Assumption | CSI feedback periodicity | 5ms | 5ms | 5ms | 5ms |
|  | Observation window (number/distance) | 5 / 5ms | 5 / 5ms | 5 / 5ms | 5 / 5ms |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) | 3 / 5ms / 5ms | 3 / 5ms / 5ms | 3 / 5ms / 5ms | 3 / 5ms / 5ms |
|  | Whether/how to adopt spatial consistency | - | - |  |  |
| Generalization Case 1 | Train (setting#A, size/k) | 30km/h, 48K | 60km/h, 48K | 90km/h, 48K | 120km/h, 48K |
|  | Test (setting#A, size/k) | 30km/h, 5K | 60km/h, 5K | 90km/h, 5K | 120km/h, 5K |
|  | SGCS (1,…N, N is number of prediction instances) | 0.8841, 0.8153, 0.7685 | 0.7987, 0.7541, 0.7420 | 0.7219, 0.7223, 0.7187 | 0.7273, 0.7259, 0.7240 |
| Generalization Case 2 | Train (setting#A, size/k) | 60km/h, 48K 90km/h, 48K 120km/h, 48K | 30km/h, 48K 60km/h, 48K 90km/h, 48K | 30km/h, 48K 60km/h, 48K 120km/h, 48K | 30km/h, 48K 60km/h, 48K 90km/h, 48K |
|  | Test (setting#A, size/k) | 30km/h, 5K | 60km/h, 5K | 90km/h, 5K | 120km/h, 5K |
|  | SGCS (1,…N, N is number of prediction instances) | 0.7364, 0.6722, 0.6446 0.6330, 0.6252, 0.6282 0.4807, 0.4885, 0.5230 | 0.7061, 0.6549, 0.6346 0.6411, 0.6377, 0.6397 0.5100, 0.5190, 0.5493 | 0.6334, 0.6118, 0.6017 0.6418, 0.6158, 0.6121 0.5090, 0.5176, 0.5481 | 0.4921, 0.5613, 0.5449 0.5278, 0.4825, 0.5418 0.5126, 0.5081, 0.5250 |
| Generalization Case 3 | Train (setting#A, size/k) | 30km/h+60km/h+90km/h, 16K+16K+16K 30km/h+60km/h+120km/h, 16K+16K+16K | 30km/h+60km/h+90km/h, 16K+16K+16K 30km/h+60km/h+120km/h, 16K+16K+16K | 30km/h+60km/h+90km/h, 16K+16K+16K | 30km/h+60km/h+120km/h, 16K+16K+16K |
|  | Test (setting#A, size/k) | 30km/h, 5K | 60km/h, 5K | 90km/h, 5K | 120km/h, 5K |
|  | SGCS (1,…N, N is number of prediction instances) | 0.7661, 0.7363, 0.6921 0.6874, 0.5862, 0.5648 | 0.6030, 0.6368, 0.5870 0.7578, 0.7159, 0.7013 | 0.7102, 0.6900, 0.6909 | 0.7127, 0.7037, 0.7021 |

Table 16. Evaluation results for CSI prediction with model generalization over carrier frequencies

|  |  | ETRI #1 | ETRI #2 | ETRI #3 |
| --- | --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone | MLP-Mixer | MLP-Mixer | MLP-Mixer |
|  | [Pre-processing] | Min-Max | Min-Max | Min-Max |
|  | [Post-processing] | Min-Max | Min-Max | Min-Max |
|  | FLOPs/M for model | 738.87M | 738.87M | 738.87M |
|  | FLOPs/M for pre/post processing | - | - | - |
|  | Parameters/M | 8.77M | 8.77M | 8.77M |
|  | [Storage /Mbytes] | - | - |  |
|  | Input type | RAW channel matrix | RAW channel matrix | RAW channel matrix |
|  | Output type | RAW channel matrix | RAW channel matrix | RAW channel matrix |
| Assumption | UE speed | 20 Km/h | 20 Km/h | 20 Km/h |
|  | CSI feedback periodicity | 5ms | 5ms | 5ms |
|  | Observation window (number/distance) | 5 / 5ms | 5 / 5ms | 5 / 5ms |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) | 3 / 5ms / 5ms | 3 / 5ms / 5ms | 3 / 5ms / 5ms |
|  | Whether/how to adopt spatial consistency | - | - | - |
| Generalization Case 1 | Train (setting#A, size/k) | 2GHz, 45K | 3GHz, 45K | 4GHz, 45K |
|  | Test (setting#A, size/k) | 2GHz, 5K | 3GHz, 5K | 4GHz, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | -7.611, -7.012, -5.418 | -5.998, -5.070, -4.188 | -4.587, -3.607, -3.253 |
| Generalization Case 2 | Train (setting#A, size/k) | 3GHz, 45K 4GHz, 45K | 2GHz, 45K 4GHz, 45K | 2GHz, 45K 3GHz, 45K |
|  | Test (setting#A, size/k) | 2GHz, 5K | 3GHz, 5K | 4GHz, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | -1.497, 0.050, 0.577 -0.394, 0.199, 0.931 | -0.827, 1.028, 1.969 1.612, 2.396, 2.584 | -0.026, 2.460, 1.867 2.942, 3.523, 4.163 |
| Generalization Case 3 | Train (setting#A, size/k) | 2GHz+3GHz+4GHz, 15K+15K+15K | 2GHz+3GHzh+4GHz, 15K+15K+15K | 2GHz+3GHzh+4GHz, 15K+15K+15K |
|  | Test (setting#A, size/k) | 2GHz, 5K | 3GHz, 5K | 4GHz, 5K |
|  | NMSE (1,…N, N is number of prediction instances) | -6.583, -6.457, -5.477 | -4.860, -4.052, -3.325 | -2.413, -1.559, -0.819 |

Conclusion

In this contribution, ETRI’s views on the evaluation on AI/ML for CSI feedback enhancement were shown and the following proposals and observations were made:

Proposal 1: Further evaluate the AI/ML based CSI prediction over various UE speeds to overcome performance degradation in untrained UE speed.

Proposal 2: Further evaluate the AI/ML based CSI prediction over various carrier frequencies to overcome performance degradation in untrained carrier frequency.

Observation 1: In the CSI compression sub-use case, with a Transformer based Autoencoder, there are 1.9 ~ 4% improvements in terms of SGCS compared to the baseline (R16 eTypeII) for payload sizes X/Y/Z.

Observation 2: AI/ML based CSI prediction improves performance compared to the baseline.

Observation 3: AI/ML based CSI prediction improves performance as the number of historical input CSIs increase.

Observation 4: Performance improvement of AI/ML based CSI prediction as input of eigenvectors is slightly reduced compared to the AI/ML model as input of RAW channel matrix.

Observation 5: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of UE speeds.

Observation 6: Mixed datasets of different UE speeds can mitigate performance degradation compared to datasets of single UE speed.

Observation 7: For AI/ML based CSI prediction, the performance reduction occurs significantly depending on changes of carrier frequencies.

Observation 8: Mixed datasets of different carrier frequencies can mitigate performance degradation compared to datasets of single carrier frequency.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- 3GPP RAN1, Chairman’s Notre, 3GPP RAN WG1 #111, November 2022

- Xingjian Shi, Zhourong Chen, Hao Wang and Dit-Yan Yeung, “Convolutional LSTM Network: A Machine Learning Approach for Precipitation Nowcasting”, international Conference on Neural Information Processing Systems (NIPS), 2015.

- Ilya Tolstikhin et al., “MLP-Mixer: An all-MLP Architecture for Vision”, Arxiv, 2021

- R1-2304247, Summary#7 for [112bis-e-R18-AI/ML-02], Moderator (Huawei)