3GPP TSG RAN WG1#113	        	R1-2305813

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:					    9.16.4

Source:	Sony

Title:	Discussion on UE features for NR NCR

Document for:	Discussion and decision

# Introduction

During RAN1#112bis-e meeting, RAN1 reached the following agreements [1], [2]:

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| Features | Index | Feature group | Components | Prereq | SL | gNB | Consequence | Type | FDD/TDD | FR1/FR2 | Combo | Note | Mandatory/optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Basic NCR support [for C-link and backhaul link] | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link [3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication] [4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link] [5. Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link] 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd [7. Support of backhaul link beam determination based on predefined rule] | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 3 candidate values: FFS] [Component 4 candidate values: FFS] [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1. Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1. Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1. Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4a | Beam index updates for semi-persistent beam indication | 1. Support of [temporary] override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1, 43-4 | N/A | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Yes | FFS: whether to merge this FG with 43-4 | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-7 | Priority flag | Priority flag for periodic/semi-persistent indication | 43-1, one of 43-2 or 43-4 | N/A | Yes | Priority flag is not supported | Per NCR-MT | No | No | Yes | FFS: whether to split this FG into two separate FGs for periodic and semi-persistent indication or whether to merge with FGs 43-2 and 43-4, respectively | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-1a8 | Adaptive beam for NCR backhaul link[/C-link] | 1. [Support adaptive beam for NCR C-link] 2. Support [explicit] adaptive beam for NCR backhaul link Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} FFS: relation between 43-1a and existing UE features for beam operation | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-8a | Explicit beam indication for NCR backhaul link | Support explicit adaptive beam for NCR backhaul link | 43-8 | N/A | Yes | Explicit beam indication for NCR backhaul link is not supported | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |

This contribution discusses some open issues on UE features for NR NCR.

# Discussion

## On candidate values for component 5 of feature 43-1

Component 5: [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link.

Note: [Component 5 candidate values: FFS]

Feature 43-1 provides basic NCR support and consists of six components. However, there is one unsolved issue related to the fifth component:

We do not fully understand what is meant by "candidate values." In our view, one can amend component five as follows:

- Regarding component 5 of feature 43-1, amend the text as follows: "Support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link." That is, drop the brackets. No candidate values are needed: the backhaul and access links follow the timing of the C-link.

## On which one of the periodic, aperiodic, or semi-persistent beam indications should be mandatory

RAN1#112bis-e agreed tu sppurt features 43-2, 43-3, and 43-4, which relate to periodic beam indication for access link, aperiodic beam indication for access link, and semi-periodic beam indication for access link, respectively. Moreover, at least one of these features will be merged with feature 43-1, which defines the essential components of NCR support. The question is which one(s) that should be merged.

We first observe that an NCR providing coverage extension to a host gNB will at least need to forward the SS/PBCH blocks of the host gNB. These SS/PBCH blocks, of course, have a periodic structure.

- An NCR must forward the host cell's SS/PBCH blocks, which possess a periodic structure.

Turning to the functionality provided by features 43-2, 43-3, and 43-4, we note that feature 43-2 defines sets of periodic resources [3]. Suitably configured, these can be superimposed with the SS/PBCH resources of the host cell, thereby enabling the forwarding of SS/PBCH transmissions by the NCR, as desired. Feature 43-3, which defines sets of periodic resources (de)activated by a MAC-CE command [3], would also work. However, this feature has been designed for other use cases wherein switching NCR forwarding on and off is essential [4]. Finally, the aperiodic beam indications of feature 43-4 require unnecessarily large signaling overhead since a DCI format 2_8 is required to schedule each forwarded SS/PBCH block, or group of SS/PBCH blocks [3]. Again, this feature has been designed with other use cases in mind, i.e., signals scheduled dynamically based on each UE's traffic needs. Thus, while feature 43-2 is essential for enabling efficient initial access via an NCR, features 43-3 and 43-4 are also needed to handle other forwarding use cases effectively.

- Preferably, merge all of 43-2, 43-3, and 43-4 into the primary feature 43-1. If only one is merged, select 43-2 to ensure initial access in the NCR-covered area with low signaling overhead.

## On RRC configuration override for semi-periodic beam indications

Component: 1. Support of [temporary] override of the RRC configuration of the beam index(es) in semi-persistent beam indication.

Note: FFS: whether to merge this FG with 43-4.

Another open issue concerns component 2 of feature 43-4a, "Beam index updates for semi-persistent beam indication."

Agreement: For semi-persistent beam indication:

- Alt-0:

  - RRC configures Y list of forwarding resource, the yth list is consist of Zy  (1≤Zy≤Zmax) forwarding resources, and each forwarding resource is defined by {beam index, time resource}.

    - The periodicity and reference SCS is configured as part of the RRC signaling for each list of forwarding resource.

  - MAC-CE activate/de-activate one of Y list, and all the Zy forwarding resources in this list are selected.

  - MAC-CE can optionally provide update for Zy beam index.

- Note: The value of Zmax (1≤Z≤Zmax) is 128, where Zmax refers to the maximum beams indicated in one indication.

This "override of the RRC configuration of the beam index(es)" seems motivated by the third sub-bullet from the following RAN1#112 agreement (highlighted in green):

According to this interpretation, a MAC-CE command's override of the beam indices is permanent. In our view, a temporary update would require an expiration parameter specifying the maximum temporal validity of the newly updated parameters or some other explicit mechanism. However, this is not the case. Of course, a new MAC-CE command overrides the beam indices with yet newer values, but this does not change the situation. Or one should call all MAC-CE commands temporary.

- The MAC-CE command used to update the RRC configuration of the beam indices has a permanent effect. At least, the NCR-MT believes so at the time of application of the MAC-CE command.

- Remove the bracketed text, i.e., "[temporary]." The MAC-CE command has a permanent effect.

Regarding the note, the RAN1#112 agreement clearly states that the beam-index overriding capability via MAC-CE is optional. Therefore, one should see it as a feature separate from 43-4.

- In view of RAN1#112 agreements, feature 43-4a should be optional and separate from feature 43-4.

# Conclusions

We made the following observations and proposals:

- An NCR must forward the host cell's SS/PBCH blocks, which possess a periodic structure.

- The MAC-CE command used to update the RRC configuration of the beam indices has a permanent effect. At least, the NCR-MT believes so at the time of application of the MAC-CE command.

- Regarding component 5 of feature 43-1, amend the text as follows: "Support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link." That is, drop the brackets. No candidate values are needed: the backhaul and access links follow the timing of the C-link.

- Preferably, merge all of 43-2, 43-3, and 43-4 into the primary feature 43-1. If only one is merged, select 43-2 to ensure initial access in the NCR-covered area with low signaling overhead.

- Remove the bracketed text, i.e., "[temporary]." The MAC-CE command has a permanent effect.

- In view of RAN1#112 agreements, feature 43-4a should be optional and separate from feature 43-4.

# References

- 3GPP R1-22xxxxx, “Draft report of 3GPP TSG RAN WG1 #112bis-e v0.1.0,” Chairman.

- 3GPP R1-2303652, "Summary of UE features for NR NCR," Moderator (AT&T.)

- 3GPP R1-2304197, "Introduction of network-controlled repeaters," CR to TS 38.213, Samsung.

- 3GPP TR 38.867, "Study on NR network-controlled repeaters."