3GPP TSG-RAN WG1 Meeting #113			R1-2305936

Incheon, Korea, 22 – 26 May, 2023

Title:	Draft Reply LS on cell DTX/DRX activation/deactivation

Response to:	R2-2304568/ R1-2304334

Release:	Rel-18

Work Item:	Netw_Energy_NR-Core

Source:	Huawei [RAN1]

To:	3GPP RAN2

Cc:	3GPP RAN3

Contact Person:

Name: 	Yi Wang

E-mail Address: 	wangyi6@huawei.com

Attachments:			N/A

# 1. Overall Description:

RAN1 thanks RAN2 for the LS on Cell DTX/DRX activation/deactivation, and would like to provide the following answers to the questions from RAN2

- Regarding “the feasibility and reliability of using dedicated and group common L1 signalling for Cell DTX/DRX activation and deactivation”

RAN1 answer

- It is feasible to introduce L1 signaling for the (de)activation of cell DTX/DRX.

- Reliability is not an issue for L1 signaling.

- Regarding “Once L1 signalling for activation and deactivation of Cell DTX/DRX for a single configuration is decided in RAN1 please inform us about the decision and design details”

RAN1 would like to provide the following information on the design details of L1 signaling:

- L1 signaling is PDCCH based signaling

RAN1 also observes some potential L1 signalling design that needs RAN2 early consideration, including:

- Whether a new RNTI is needed or not.

- Whether a new search space is needed or not.

- Whether L1 signaling should be supported or not for Cell DTX/DRX multiple configurations patterns

- Whether Timer or validity duration based activation/deactivation of cell DTX/DRX is needed or not

Meanwhile, RAN1 has achieved the following agreements from RAN1#112bis-e that may have RAN2 impact as well

Agreement

From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions.

- Periodic/Semi-persistent CSI report

- Periodic/Semi-persistent SRS

  - FFS: SRS for positioning

- FFS:

  - HARQ feedback for SPS PDSCH

- FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX

- FFS Whether the listed signals/channels can be configurable by gNB

- FFS: Whether the same or different UE behavior is applicable with or without C-DRX

- FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

# 2. Actions:

To 3GPP RAN2

ACTION: RAN1 respectfully asks RAN2 to take above response into account in the future work. In particular in regards the L1 signaling design that needs early RAN2 consideration, RAN1 respectfully suggests that RAN2 should proceed without delay on their related work, based on the above information.

# 3. Date of Next TSG WG RAN1 Meetings:

- TSG-RAN WG1 Meeting #114							21 August – 25 August 2023							Toulouse