3GPP TSG RAN WG1 #113		                                   					       												                          R1-2306162

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.16.4

Agenda Item:	9.16.4

Document for: 	Endorsement

### UE features for NR NCR

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd 7.Support periodic beam indication for access link 8. Priority flag for periodic indication | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: Component 2 candidate values: - 15 kHz: {[0,] 1 [, 2,3,4]} - 30 kHz: {[0,]1 [, 2,3,4]} - 60 kHz: {[0,]1, 2 [ ,3,4]} - 120 kHz: {[0,]1, 2 [,,3,4]} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3.Support of [temporary] MAC CE override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-4a | Beam index updates for semi-persistent beam indication | 1.Support of [temporary] override of the RRC configuration of the beam index(es) in semi-persistent beam indication | 43-1, 43-4 | N/A | Yes | The semi-persistent beam indication cannot provide updates for the beam index(es). | Per NCR-MT | No | No | Yes | FFS: whether to merge this FG with 43-4 | Optional with capability signaling |

R1-2304509	Discussion on UE feature for NCR	vivo

R1-2304631	Discussions on UE features for NR NCR	Huawei, HiSilicon

R1-2304777	Discussion on UE features for NR NCR	Fujitsu

R1-2304800	UE features for NCR	Intel Corporation

R1-2304890	Discussion on UE features for NR NCR	xiaomi

R1-2304942	Summary of UE features for NR NCR	Moderator (AT&T)

R1-2305118	Discussion on UE feature of NR NCR	CMCC

R1-2305277	On UE features for Rel-18 NR network-controlled repeaters	Apple

R1-2305369	UE features on NR NCR	Qualcomm Incorporated

Withdrawn

R1-2305395	Discussion on UE features for NR NCR	LG Electronics

R1-2305543	UE features for NR NCR	Samsung

R1-2305555	Discussion on the UE feature for NCR	ZTE

R1-2305620	Discussion on UE features for NR NCR	NTT DOCOMO, INC.

R1-2305717	UE features for NCR	Ericsson

R1-2305720	On UE features for Network Controlled Repeaters	Nokia, Nokia Shanghai Bell

R1-2305804	Discussion on NCR features	ETRI

R1-2305813	Discussion on UE features for NR NCR	Sony