3GPP TSG-RAN WG1 Meeting #113	R1-2306306

Incheon, Korea, May 22 – 26, 2023

Agenda Item:	9.17

Source:	Ericsson

Title:	Editor’s summary on draft CR 38.211 for NR_pos_enh2-Core

Document for:	Discussion, Decision

# 1	Introduction

This document is intended to facilitate the review process of the draft CR 38.211 for NR_pos_enh2-Core.

# 2	Discussion – first round

Please provide your comments on the latest version of the draft CR on 38.211 available in this folder.

| Company | Comment |
| --- | --- |
| vivo | Comment 1: remove 8.4.5.1.1 general part Considering SL PRS resource refers to a time-frequency resource within a slot and will be common for different UEs and different sequences, the relation between SL PRS resource and sequence is unclear. So we prefer to remove the content at least. Comment 2: mod 12 should be added based on following WA Working assumption - For SL PRS sequence generation, the parameter is defined as below: - is provided by higher layers to a Tx UE - Details on higher layers, including consideration of Tx UE’s own higher layer, are up to RAN2 - The higher layer parameter is provided to an Rx UE via LPP/SLPP. - FFS: If (pre-)configured for a resource pool and use of SL PRS for sensing is supported, is based on 12 LSB bits CRC of PSCCH associated with the SL PRS - Otherwise (i.e., if not provided by higher layers), is based on 12 LSB bits CRC of PSCCH associated with the SL PRS - nID,seqSL-PRS∈0,1,…,4095 is the sidelink PRS sequence ID, given by the higher-layer parameter XXX if configured, otherwise nID,seqSL-PRS=NIDX mod 212 where the quantity NIDX equals the decimal representation of the CRC for the sidelink control information mapped to the PSCCH associated with the SL-PRS according to NIDXnID,seqSL-PRS=(i=0L-1pi∙2L-1-i ) with p and L given by clause 7.3.2 in [4, TS 38.212].. Comment 3: Adding following part in 8.4.5.1.3 where - the comb size KcombSL-PRS - the resource-element offset koffsetSL-PRS∈0,1,…,KcombSL-PRS-1 - the starting symbol lstartSL-PRS and number of symbols LPRS The reference point for k is subcarrier 0 in common resource block 0.. |
| Huawei, HiSilicon | #1. Clause 8.4.1.5/8.4.1.5.x currently in 17.4.0 is for CSI-RS, so SL-PRS should start from 8.4.1.6/8.4.1.6.x. #2. Clause affected on the cover sheet should be filled. #3. No hyphen for ‘SL-PRS’ so better to have ‘SL PRS’ to align with other specs but ok to have SL-PRS in the formulas. #4. ‘given by the higher-layer parameter XXX if configured’ should be FFS because the agreement states ‘is provided by higher layers to a Tx UE’ which also includes Tx UE’s own higher layer without high layer parameter. #5. ‘Mapping to physical resources’ seems better to be changed to ‘Mapping to physical resources in a sidelink PRS resource’ because SL-PRS resources can be multiplexed within a slot. #6: in clause 8.4.1.5.1, ‘used for SL-PRS transmission’ under 8.4.1.5.1 should be deleted because Rx UE or UEs performing sensing in sheme2 will also take the SL PRS mapped in the slot. Moreover, there is no need to have the following characteristics in TS 38.211, which is anyway defined in TS 38.214. The symbols are explained in the clause 8.4.1.5.3. A SL-PRS resource refers to a time-frequency resource within a slot koffsetSL-PRSKcombSL-PRS lstartSL-PRSLPRS. #7: in clause 8.4.1.5.3, the description of parameters can be updated below. The sequence shall be multiplied with the amplitude scaling factor βSL-PRS and mapped to resources elements k,lp,μ according to ak,lp,μ=βSL-PRS rm m=0, 1, … k=mKcombSL-PRS+koffsetSL-PRS+k' mod KcombSL-PRS l=lstartSL-PRS, lstartSL-PRS+1, …, lstartSL-PRS+LSL-PRS-1 where - the antenna port p=6000 - KcombSL-PRS is the comb size provided by the higher layer parameter XXX - lstartSL-PRS is the starting symbol, provided by the higher layer parameter XXX - LSL-PRS is the number of symbols provided by the higher layer parameter - the resource-element offset koffsetSL-PRS∈0,1,…,KcombSL-PRS-1 is obtained from the higher-layer parameter XXX - the quantity k' is given by 8.4.1.5.3-1. The reference point for k=0 is subcarrier 0 in common resource block 0. Table 8.4.1.5.3-1: The frequency offset k' as a function of l-lstartSL-PRS. KcombSL-PRS; Symbol number within the sidelink PRS resource l-lstartSL-PRS. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11. 1; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0; 0. 2; 0; 0; 0; 1; 0; 1; 0; 1; 0; 1; 0; 1. 4; 0; 2; 1; 3; 0; 2; 1; 3; 0; 2; 1; 3. 6; 0; 3; 1; 4; 2; 5; 0; 3; 1; 4; 2; 5.. #8: in clause 8.2.4, antenna ports starting from 6000 for SL-PRS should be added |
| Intel | Comment 1 Section number should be 8.4.1.6. Comment 2 On SL PRS sequence ID, “given by the higher-layer parameter XXX if configured” does not seem fully aligned with the RAN1 agreement since the details of how the sequence ID is obtained via higher layers is left up to RAN2. Further, we agree with vivo’s suggestion on capturing that the 12 LSB bits are used from the decimal representation of the PSCCH CRC. Thus, we suggest the following modification to address both issues: nID,seqSL-PRS∈0,1,…,4095 is the sidelink PRS sequence ID that can be provided by higher layers, otherwise, if not provided by higher layers, nID,seqSL-PRS=NIDX mod 212 where the quantity NIDX equals the decimal representation of the CRC for the sidelink control information mapped to the PSCCH associated with the SL-PRS according to NIDXnID,seqSL-PRS=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in [4, TS 38.212]. Comment 3 On mapping to physical resources, we think the subclause title is accurate as is. Also, for this subclause, we prefer the modifications suggested by vivo over those from Huawei. While addition of the antenna port number may be fine, the other details may be too premature. For example, the details like comb offset sequence depend on lengths of SL PRS that are supported which is still an open issue in RAN1. For instance, for a dedicated resource pool, the maximum number of SL PRS transmission can be equal to 9, rather than 12, if we assume 1 symbol for PSCCH AGC, 2 symbols for PSCCH, 1 symbol for SL PRS AGC and 1 symbol for guard. Then, for some of the parameters it is still unclear if separate higher layer parameters are to be used for both Tx and Rx UEs – this may depend on how the SL PRS resource ID is determined. Based on RAN1 agreements so far, at least some of these parameters (e.g., starting symbol, RE-offset) associated with a SL PRS resource may be determined by a Tx/Rx UE based on one or more of: higher layers, information in PSCCH/SCI, or selection by Tx UE, etc. Thus, we are not sure it would be appropriate to list all these parameters as being obtained via individual higher layer parameters. Comment 4 We also suggest replacing “SL-PRS” with “SL PRS” to align terminology across specifications and other RS, like “DL PRS”. Comment 5 On SL PRS resource definition, we think it is okay to say “used for SL PRS transmission” since that is the fundamental purpose of the resource and even for a receiving UE, the resource is of relevance only in the context of potential SL PRS transmission in there. We are fine with removing the characteristics from 38.211 and let these be captured in 38.214. |
| Qualcomm | Comment 1 We agree with Intel’s wording for nID,seqSL-PRS , it avoids issues about Rx/Tx UE and the details of how the value is provided by higher layers, which will be decided by RAN2. Comment 2 Some of the mapping details are being proposed instead as restrictions in 214, for example the mapping on contiguous symbols and not mapping to symbols with PSSCH DMRS. We propose to capture those instead in 211 as part of the mapping procedure to make it clear that these are not error cases, but that PSSCH DMRS and PSCCH are boundaries for SL PRS. Otherwise, the currently proposed text in 214 could be interpreted as PSSCH DMRS or SL PRS puncturing the other, which is not the case. |
| OPPO | - I would be better to use LSL-PRS to align the naming rule as other parameters. 2. How to determine nID,seqSL-PRS have not been completely decided in RAN1 and RAN2 yet, the working assumption may be changed up to how to address the FFS and RAN2’s decision on the details of higher layers, e.g., whether to introduce a RRC parameter or not. Given that we propose to remove the following for the time being: - nID,seqSL-PRS∈0,1,…,4095 is the sidelink PRS sequence IDnID,seqSL-PRS=i=0L-1pi∙2L-1-ipL. Working assumption - For SL PRS sequence generation, the parameter is defined as below: - is provided by higher layers to a Tx UE - Details on higher layers, including consideration of Tx UE’s own higher layer, are up to RAN2 - The higher layer parameter is provided to an Rx UE via LPP/SLPP. - FFS: If (pre-)configured for a resource pool and use of SL PRS for sensing is supported, is based on 12 LSB bits CRC of PSCCH associated with the SL PRS - Otherwise (i.e., if not provided by higher layers), is based on 12 LSB bits CRC of PSCCH associated with the SL PRS 3. SL PRS is mapped to a SL-PRS resource, this can also address QC’s comments 2, as according to 214 it is clear that SL-PRS resource should not include symbols with PSCCH, PSSCH DMRS and PSSCH in shared resource pool. - the comb size KcombSL-PRS of a SL-PRS resource - the comb offset koffsetSL-PRS of a SL-PRS resource - the starting symbol lstartSL-PRS of a SL-PRS resource and number of symbols LSL-PRS of a SL-PRS resource |
| ZTE | Comment 1: SL PRS should be added in section 8.1.2: ### 8.1.2 Overview of physical signals A sidelink physical signal corresponds to a set of resource elements used by the physical layer but does not carry information originating from higher layers. The following sidelink physical signals are defined: - Demodulation reference signals, DM-RS - Channel-state information reference signal, CSI-RS - Phase-tracking reference signals, PT-RS - Sidelink primary synchronization signal, S-PSS - Sidelink secondary synchronization signal, S-SSS - Positioning reference signal, PRS Comment 2: SL PRS port should be added in section 8.2.4: ### 8.2.4 Antenna ports An antenna port is defined in clause 4.4.1. The following antenna ports are defined for the sidelink: - Antenna ports starting with 1000 for PSSCH - Antenna ports starting with 2000 for PSCCH - Antenna ports starting with 3000 for CSI-RS - Antenna ports starting with 4000 for S-SS/PSBCH - Antenna ports starting with 5000 for PSFCH - Antenna ports starting with 6000 for PRS Comment 3: Agree with Huawei and Intel, the section number should be 8.4.1.6 Comment 4: regarding SL PRS sequence ID, we also think details for higher layer parameter is still under discussion and it’s better to put it in [given by the higher-layer parameter XXX if configured] brackets for now. Comment 5: The parameter LPRS is already used in clause 7.4.1.7.3 for DL PRS. Suggest to replace it by LSL-PRS. |
| LGE | Comment 1 For Section 8.4.1.5.1, we agree with companies to remove the characteristics of SL PRS resource. Huawei’s suggested text is fine. Comment 2 For Section 8.4.1.5.2, we suggest to replace the wording " the sidelink PRS sequence ID, given by the higher-layer parameter XXX if configured" by " the sidelink PRS sequence ID if configured by the higher layer," according to the agreement. We agree that further details are FFS in RAN2 discussion. Comment 3 For Section 8.4.1.5.3, we are fine with Huawei’s text except resource element offset part, which is not yet finalized and requires further discussion. |

Editor’s response:

- The section numbering has been corrected

- The text in 8.4.1.6.1 has been significantly shortened.

- mod 12 has been added to the expression of n_ID,seq^SL-PRS

- The text regarding SL PRS sequence ID and higher layers has been updated. According to my understanding of the chairman’s notes it reflects the current working assumption but will of course be updated to match any agreements made at RAN1#114

- Some editorial corrections as discussed above has been done.

With these updates v2 of the draft CR is stable enough to be endorsed in principle. Further updates can be handled after RAN1#114.

The endorsed draft CR is available in R1-2306307.