3GPP TSG-RAN WG1 Meeting #114	R1-2306607

Toulouse, France, August 21st - 25th, 2023

Agenda Item:	9.8.1

Source:	Ericsson

Title:	Capacity Enhancements for XR

Document for:	Discussion, Decision

# 1	Introduction

In RAN plenary 98-e, the Rel-18 WI on eXtended Reality (XR) was agreed [1] and was further revised in RAN#99 [2], with the following objectives:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2). Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

Among the above objectives, RAN1 is tasked to carry out the normative work for the enhancements defined by the following two objectives:

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1, RAN2);

The discussion during the last meetings resulted in a set of agreements for designing the features associated to the above objectives. In this contribution, we discuss our view on the remaining design issues for specifying these enhancements for configured grant (CG) based transmission.

# 2	Discussion

## Multiple transmission occasions per CG period

We discuss the remaining design issues corresponding to the following WID objective in this section:

- Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

### General

The design of multi-PUSCH CG is almost completed. For the core design, the main remaining issue is related to repetition that is addressed in next section. However, for completeness and proper specifications of the feature, it is important to conclude on a common understanding regarding any multi-PUSCH CG parameter/procedure that lacks a specific agreement during this WI. Our understanding of the common understanding is that the legacy setting is applied for any multi-PUSCH CG parameter/procedure that lacks a specific agreement during this WI. Therefore, we propose the following:

Endorse the following proposed conclusion:

  - Proposed conclusion: For completeness of the core design of multi-PUSCH CG and ease of specifications, the common understanding is that the legacy parameters/procedures are applied for any parameters/procedures that lack a specific agreement/conclusion during this WI for multi-PUSCH CG design, unless an inconsistency is identified.

    - Note: Support of repetition for multi-PUSCH CG should be discussed and concluded separately.

### Multi-PUSCH CG Repetition

With respect to TDRA design, the following agreement was made during the last meeting which implies that a single SLIV is determined from TDRA table and used for the consecutive CG PUSCHs in a period.

Agreement

For time domain resource allocation for multi-PUSCH CGs, support

- For TDRA determination (based on NR-U framework)

  - For Type-1, follow the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1 of TS 38.214.

    - Note: To determine the configuration of TDRA, PUSCH repetition type A is assumed according to description in 6.1.2.3 in 38.214 for Type-1.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

  - For Type-2, the TDRA table is determined by the TDRA table associated with activation DCI, as defined in Clause 6.1.2.1 of TS 38.214.

    - Note: The DCI format for activation DCI with pusch-RepTypeA is applicable.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

- N is configured by higher layers

- A single SLIV is determined from TDRA.

  - The SLIV used for 1st PUSCH per CG period.

- The PUSCH is used in each of N consecutive slots per CG period

- Note: N is configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. N configuration is independent from cgRetransmissionTimer configuration.

- To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration:

  - For the first PUSCH in the period, follow the legacy procedures.

  - For remaining PUSCHs in the period

    - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.

The remaining issue is whether/how to support repetition.

Our view in previous meetings as the proponent of Alt-C2 for TDRA design of multi-PUSCH CG was not to support repetition. However, with the above agreement, we believe that whether repetition is supported or not, depends on the configuration of SLIV and corresponding TDRA. In case of other TDRA tables, support of repetition depends on the configuration of repetition factor independently or as part of SLIV configuration, as per legacy procedures. In other words, if the legacy procedures support repetition for a SLIV used for construction of a multi-PUSCH CG configuration, the best would be to allow that for multi-PUSCH CG; otherwise not. For example, if the corresponding TDRA is multiPUSCH and a row with single SLIV is used for multi-PUSCH CG resource allocation, the repetition is not supported. In this way, the specification impact would be minimum and the choice of applying repetition or not, would be up to implementation and deployment scenarios instead of being restricted by design.

Legacy procedures with respect to NR-U configured grant (i.e., when cg-nrofSlots and cg-nrofPUSCH-InSlot are configured) with repetition are captured in clause 6.1.2.3 and 6.1.2.3.1. Reusing NR-U procedures with support of repetition for multi-PUSCH CG requires only time domain resource allocation specifications with an update similarly to the highlighted text in blue in these clauses as shown in Table 1, and the multi-PUSCH CG corresponding RRC parameter (i.e., N in corresponding agreements) specifications. In this case, specifications of remaining procedures such as restriction of repetition to a CG period, conditions on terminating a repetition by dynamic scheduling of a an overlapping PUSCH with a same HP ID, determination of corresponding redundancy version, associations to first or second SRS resource set, etc. would remain the same as in legacy. See Figure 1, Figure 2 and Figure 3 for illustrative examples comparing the multi-PUSCH CG with the legacy CG.

Based on the above discussion, we make the following observation and proposal:

Support of repetition for multi-PUSCHs CG, would minimize the specification impact by reusing legacy procedures for NR-U CG and would result in a design with a choice to enable/disable repetition whichever better suited for operation.

Repetition is supported for Type-1 and Type-2 multi-PUSCH CG, reusing the existing procedures for NR-U CG.

Figure 1: Case 1 illustrates an example of Rel-15 CG with one TO per period of duration 4 slots. Case 1-A and Case 1-B show the PUSCHs in respective TOs that carry TB1 and TB2 without and with repetition Type-A assuming repetition factor=2, respectively.

Figure 2: Case 2 illustrates an example of Rel-16 NR-U CG with 4 TOs per period of duration 4 slots, using cg-nrofSlots=2 and cg-nrofPUSCH-InSlot=2. Case 2-A and Case 2-B show the PUSCHs in respective TOs that carry TB1 and TB2 without and with repetition assuming repetition factor =2, respectively.

Figure 3: Case 3 illustrates an example of Rel-18 Multi-PUSCH CG with 3 TOs per period of duration 4 slots, using N=3. Case 3-A and Case 3-B show the PUSCHs in respective TOs that carry TB1 and TB2 without and with repetition assuming repetition factor =2, respectively, mimicking NR-U behavior.

Table 1: Legacy procedures descriptions with respect to NR-U configured grant (i.e., when cg-nrofSlots and cg-nrofPUSCH-InSlot are configured) with repetition

#### 6.1.2.3 Resource allocation for uplink transmission with configured grant When PUSCH resource allocation is semi-statically configured by higher layer parameter configuredGrantConfig in BWP-UplinkDedicated information element, and the PUSCH transmission corresponding to a configured grant, the following higher layer parameters are applied in the transmission: - For Type 1 PUSCH transmissions with a configured grant, the following parameters are given in configuredGrantConfig unless mentioned otherwise: - For the determination of the PUSCH repetition type, if the higher layer parameter pusch-RepTypeIndicator in rrc-ConfiguredUplinkGrant is configured and set to 'pusch-RepTypeB', PUSCH repetition type B is applied; otherwise, PUSCH repetition type A is applied; - For PUSCH repetition type A, the selection of the time domain resource allocation table follows the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1. - For PUSCH repetition type B, the selection of the time domain resource allocation table is as follows: - If pusch-RepTypeIndicatorDCI-0-1 in pusch-Config is configured and set to 'pusch-RepTypeB', pusch-TimeDomainAllocationListDCI-0-1 in pusch-Config is used; - Otherwise, pusch-TimeDomainAllocationListDCI-0-2 in pusch-Config is used. - It is not expected that pusch-RepTypeIndicator in rrc-ConfiguredUplinkGrant is configured with 'pusch-RepTypeB' when none of pusch-RepTypeIndicatorDCI-0-1 and pusch-RepTypeIndicatorDCI-0-2 in pusch-Config is set to 'pusch-RepTypeB'. - The higher layer parameter timeDomainAllocation value m provides a row index m+1 pointing to the determined time domain resource allocation table, where the start symbol and length are determined following the procedure defined in Clause 6.1.2.1; - Frequency domain resource allocation is determined by the N LSB bits in the higher layer parameter frequencyDomainAllocation, forming a bit sequence f17, …, f1, f0, where f0 is the LSB, according to the procedure in Clause 6.1.2.2 and N is determined as the size of frequency domain resource assignment field in DCI format 0_1 for a given resource allocation type indicated by resourceAllocation, except if useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, in which case uplink type 2 resource allocation is used wherein the UE interprets the LSB bits in the higher layer parameter frequencyDomainAllocation as for the frequency domain resource assignment field of DCI 0_1 according to the procedure in Clause 6.1.2.2.3; - The IMCS is provided by higher layer parameter mcsAndTBS; - Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Clause 7.3.1.1.2 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively; When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, precoding information and number of layers (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook') associated with the first and second SRS resource set is provided by precodingAndNumberOfLayers and precodingAndNumberOfLayers2, respectively, and SRS resource indicators associated with the first and second SRS resource sets are provided by srs-ResourceIndicator and srs-ResourceIndicator2, respectively. When both srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 are configured with two SRS resource sets, the two SRS resource sets configured by srs-ResourceSetToAddModList is used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. - If two SRS resource sets with usage set to 'codebook' or 'noncodebook' are configured in srs-ResourceSetToAddModList, the two SRS resource sets are used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. - otherwise, the two SRS resource sets with usage set to 'codebook' or 'noncodebook' configured in srs-ResourceSetToAddModListDCI-0-2 are used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. - When frequency hopping is enabled, the frequency offset between two frequency hops can be configured by higher layer parameter frequencyHoppingOffset. - For Type 2 PUSCH transmissions with a configured grant: the resource allocation follows the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI. - The PUSCH repetition type and the time domain resource allocation table are determined by the PUSCH repetition type and the time domain resource allocation table associated with the UL grant received on the DCI, respectively, as defined in Clause 6.1.2.1. The value of Koffset, if configured, is applied when determining the first transmission opportunity. For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberOfRepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. For PUSCH transmissions with a Type 2 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, the SRS resource set association to (nominal) repetitions follows MappingPattern in ConfiguredGrantConfig as defined in Clause 6.1.2.1 for PUSCH scheduled by DCI format 0_1 and 0_2. For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets with usage set to 'codebook' or 'noncodebook' are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if p0-PUSCH-Alpha2 is provided, the SRS resource set association to (nominal) repetitions is determined as follows. When K = 2, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively. - When K > 2 and cyclicMapping in ConfiguredGrantConfig is enabled, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. - When K > 2 and sequentialMapping in ConfiguredGrantConfig is enabled, first SRS resource set is applied to the first and second (nominal) repetitions, and the second SRS resource set is applied to the third and fourth (nominal) repetitions, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if configuredGrantConfig contains only one pathlossReferenceIndex, p0-PUSCH-Alpha, powerControlLoopToUse, srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook'), PUSCH repetitions are associated only with the first SRS resource set. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and the UE is not provided p0-PUSCH-Alpha2 and powerControlLoopToUse2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes an SRS resource set indicator field, the UE expects the value of the SRS resource set indicator field to be set to '00', and PUSCH repetitions are associated only with the first SRS resource set. The UE shall not transmit anything on the resources configured by configuredGrantConfig if the higher layers did not deliver a transport block to transmit on the resources allocated for uplink transmission without grant. A set of allowed periodicities P are defined in [12, TS 38.331]. The higher layer parameter cg-nrofSlots, provides the number of consecutive slots allocated within a configured grant period. The higher layer parameter cg-nrofPUSCH-InSlot provides the number of consecutive PUSCH allocations within a slot, where the first PUSCH allocation follows the higher layer parameter timeDomainAllocation for Type 1 PUSCH transmission or the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI for Type 2 PUSCH transmissions, and the remaining PUSCH allocations have the same length and PUSCH mapping type, and are appended following the previous allocations without any gaps. The same combination of start symbol and length and PUSCH mapping type repeats over the consecutively allocated slots. For operation with shared spectrum channel access, and when the higher layer parameter semiStaticChannelAccessConfigUE is not configured, where a UE is performing uplink transmission with configured grants in contiguous OFDM symbols on all resource blocks of an RB set, for the first such UL transmission the UE determines a duration of a cyclic prefix extension Text to be applied for transmission according to [4, TS 38.211] where the index for Δi [4, TS 38.211] is chosen randomly from a set of values configured by higher layers according to the following rule: - If the first such UL transmission is within a channel occupancy initiated by the gNB (defined in Clause 4 of [16, TS 37.213]), the set of values is determined by cg-StartingFullBW-InsideCOT; - otherwise, the set of values is determined by cg-StartingFullBW-OutsideCOT. For operation with shared spectrum channel access, and when the higher layer parameter semiStaticChannelAccessConfigUE is not configured, where a UE is performing uplink transmission with configured grants in contiguous OFDM symbols on fewer than all resource blocks of an RB set, for the first such UL transmission the UE determines a duration of a cyclic prefix extension Text to be applied for transmission according to [4, TS 38.211] according to the following rule: - If the first such UL transmission is within a channel occupancy initiated by the gNB (defined in Clause 4 of [16, TS 37.213]), the index for Δi [4, TS 38.211] is equal to cg-StartingPartialBW-InsideCOT; - otherwise, the index for Δi [4, TS 38.211] is equal to cg-StartingPartialBW-OutsideCOT. ##### 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant. The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(((n-mod(n, N))/N)-1,4)+1)th value in the configured RV sequence, where N=1. If a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at - the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1}, - any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3}, - any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8. When the transmission occasions are associated with the first and second SRS resource sets, if the parameter repK-RV is provided in the configuredGrantConfig, for the nth transmission occasion among all transmission occasions that are associated with the SRS resource set of the first transmission occasion, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence, and for the nth transmission occasion among all transmission occasions that are not associated with the SRS resource set of the first transmission occasion, it is associated with (mod(n-1,4)+1)th value in the adjusted RV sequence and the adjustment is based on additional shifting operation on the configured RV sequence, where the shifting operation is defined as rvi+ rvsmod 4 where rvi is the ith RV value (i=1, 2, 3, 4) in the configured RV sequence and rvs is configured by the higher layer parameter sequenceOffsetforRV in configuredGrantConfig. When the transmission occasions are associated with the first and second SRS resource sets, if a configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at - the first transmission occasion associated with RV = 0 corresponding to the first or second SRS resource set of the K repetitions if the configured RV sequence is {0,2,3,1}, - any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3}, - any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8. After the initial transmission of a transport block, later transmission occasions among the K repetitions associated with any RV value and associated to any of the first or second SRS resource set can be used for transmitting the transport block. For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1 or 0_2, whichever is reached first. In addition, the UE shall terminate the repetition of a transport block in a PUSCH transmission if the UE receives a DCI format 0_1 with DFI flag provided and set to '1', and if in this DCI the UE detects ACK for the HARQ process corresponding to that transport block. The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available for the PUSCH transmission in a slot is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion. For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, - For unpaired spectrum: - If AvailableSlotCounting is enabled, the UE shall repeat the TB across the N∙K slots determined for the PUSCH transmission applying the same symbol allocation in each slot. - A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - Otherwise, the UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - For paired spectrum and SUL band: - The UE shall repeat the TB across the N∙K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots and cg-nrofPUSCH-InSlot, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. - If AvailableSlotCounting is enabled, and in case of reduced capability half-duplex UE, the UE shall repeat the TB across the N∙K slots applying the same symbol allocation in each slot. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot does not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 9, Clause 11.1, Clause 11.2A, Clause 15 and Clause 17.2 of [6, TS 38.213].

### 2.1.2	Other design topics

- In the following we share our views about the other design topics in addition to the core design topics.

#### 2.1.2.1	Retransmission of multiple CG PUSCHs

There is a restriction on scheduling retransmissions by a multi-PUSCHs scheduling DCI when a configured grant is used for original transmission as reflected in the following agreement and corresponding description in the specification.

Agreement (RAN1#108-e) - When re-transmission of DL SPS is indicated by DCI format 1_1, the PDCCH indicates a TDRA row index including only one SLIV. - When re-transmission of UL CG is indicated by DCI format 0_1, the PDCCH indicates a TDRA row index including only one SLIV.

38.214, Clause 6.1.2.1 If a UE is configured with extendedK2 in pusch-TimeDomainAllocationListForMultiPUSCH in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, and the UE is indicated re-transmission of PUSCH by DCI format 0_1, where the PUSCH is correspond to a configured grant Type 1 or Type 2, the UE does not expect that the number of indicated SLIVs in the row of the pusch-TimeDomainAllocationListForMultiPUSCH by the DCI is more than one.

Although, the above restriction was not necessary in Rel-17 and the issue under discussion could be handled by implementation, it was considered for simplicity. Regardless of the Rel-17 outcome, this restriction is not motivated for Rel-18 with the introduction of multi-PUSCHs CG. When the initial transmission of multiple TBs by multi-PUSCHs CG is failed and all/some of the TBs require retransmissions, the usage of multi-PUSCHs DCI for retransmission is reasonable, and should be allowed by specifications. Whether it is used or not, depends on the scenario, but disallowing the usage by specification is not justified. 

The retransmission with CS-RNTI for TBs which were initially transmitted over CG will work similar to retransmissions with C-RNTI (where transmissions are dynamically scheduled). However, there can be few differences and may need slight implementation adjustments.

- In the case with C-RNTI, the scheduled multi-PUSCH allocations can contain both initial and retransmissions depending on whether the corresponding NDI bit is toggled or not, respectively. However, for retransmissions with CS-RNTI, the scheduled transmissions will have only retransmission allocations. The initial transmissions cannot be allocated as they are performed over CG.

- Given that the initial transmission cannot be performed with CS-RNTI, hence the corresponding NDI bit can be configured or understood as following

  - Set NDI bit to “1” indicating retransmission for corresponding HARQ process,

  - Set NDI bit to “0” indicating no retransmission for corresponding HARQ process.

The overall retransmission procedure with CS-RNTI is simple and does not require major changes. Therefore, we propose to relax this restriction in Rel-18.

Scheduling re-transmission of multiple TBs for corresponding initial transmission of the TBs by configured grant is supported for DCI format 0_1 scrambled with CS-RNTI.

  - NDI bit to “1” indicates retransmission for corresponding HARQ process.

#### 2.1.2.2	CBG retransmission for multiple CG PUSCHs

Support of CBG based retransmission was discussed during the study item and investigated but did not reach a consensus. In our view, this is not a necessary enhancement and can be down prioritized.

Support of CBG based transmission for multi-PUSCH CG is down prioritized.

#### 2.1.2.3	One TB over multiple PUSCHs

- One TB over multiple slots improves coverage and does not bring capacity improvements. Currently, retransmission of partial TB is not possible, and if some PUSCH fails, then retransmission will require retransmission of all PUSCHs (whole TB) which degrades the capacity.

Down prioritize one TB over multiple slots.

## Indication of unused CG transmission occasions

We discuss the remaining design issues corresponding to the following WID objective:

- Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1, RAN2);

### 2.2.1	Design for UTO-UCI and indicated TOs associations

Different design options to determine the relationship between the indicated UTO-UCI and the corresponding TOs were discussed during the last meeting. Eventually, the following design candidates were prioritized for further down-selection at this meeting:

Agreement:

For a CG configuration with UTO-UCI indication enabled, to determine the indicated CG PUSCH by a UTO-UCI indication, consider the following options for further down-selection:

Option A-1a:

  - Configure the RRC parameter UTO_period.

    - FFS range value of UTO_period

      - Alt-1: values in time unit (e.g., XR traffic periodicity)

      - Alt-2: one or multiple of CG periodicity given by integer values (n=1, 2, ..)

  - The starting time of the first period of UTO periodicity starts at the same as starting time of the first period of the CG configuration and ends after UTO_period. The next UTO period(s) are followed after the first UTO period.

  - A transmitted CG PUSCH that is confined within a UTO period, carries UTO-UCI that is applicable to the CG PUSCH TOs within the UTO period.

Option A-2a:

  - Configure the RRC parameter UTO_period.

    - FFS range value of UTO_period

      - Alt-1: values in time unit (e.g., XR traffic periodicity)

      - Alt -2: one or multiple of CG periodicity given by integer values (n=1, 2, ..)

- Configure the RRC parameter UTO_offset.

  - FFS range value of UTO_offset

- The starting time of the first period of UTO periodicity starts at the same as starting time of the first period of the CG configuration and ends after UTO_period. The next UTO period(s) are followed after the first UTO period.

- A transmitted CG PUSCH that is confined within a UTO period, carries UTO-UCI that is applicable to the CG PUSCH TOs within the UTO period and after UTO_offset from the end of the transmitted CG PUSCH.

Option B-a:

- Configure the RRC parameter UTO_period.

  - FFS range value of UTO_period

    - Alt-1: values in time unit (e.g., XR traffic periodicity)

    - Alt -2: one or multiple of CG periodicity given by integer value (n=1, 2, ..)

- UTO_offset is the offset value.

  - Alt-1: UTO_Offset is provided by configuration.

    - FFS range value of UTO_offset

  - Alt-2: UTO_Offset = 0

- A transmitted CG PUSCH carries UTO-UCI that is applicable to the valid CG PUSCH TOs that are confined within UTO_period starting with UTO_offset from the end of the transmitted CG PUSCH.

Option B-b2:

- Configure the RRC parameter Nu (Nu is the size of bit-map)

  - FFS range value of Nu

- UTO_offset is the offset value.

  - Alt-1: UTO_Offset is provided by configuration.

    - FFS range value of UTO_offset

  - Alt-2: UTO_Offset = 0

- A transmitted CG PUSCH, carries UTO-UCI that is applicable to the Nu consecutive and valid CG PUSCH TOs, starting with UTO_offset from the end of the transmitted CG PUSCH.

FFS on whether/how to extend to multiple CG configurations

In the following, first we explain our views on properties of a proper and useful design. We continue then by assessing the above design options to motivate the preferred design solution from our perspective.

#### Considerations for a useful design

The NW may provide configured grant resources to facilitate fast transmission of XR data traffic at the UE. To accommodate large and variable size of XR traffic and/or potential uncertainty in arrival time at the UE, large and/or frequent configured grant resources may be allocated that will be underutilized. This can be done using legacy configured grants and/or Rel-18 multi-PUSCH configured grant.

As stated in the WID, the feature enables transmission of the information as uplink control information (UCI) to indicate the unused transmission occasion (UTO) of the configured grant. Figure 4 shows an illustrative example of the feature. For convenience, we refer to this information as “UTO-UCI” in the discussion below.

Figure 4: Illustration of configured grant PUSCH with indicating UTO enabled

This feature was motivated to enable a mechanism to inform the gNB in advance regarding the configured resources that will not be used by the UE for transmission. This information can be beneficial at the gNB to improve system resource utilization and reduce complexity by skipping blind detection when applicable. Therefore, there are two main reasons for UTO-UCI to be beneficial for gNB:

- Repurpose the unused CG PUSCH resources

- Reduce blind detection of CG PUSCHs

Repurposing feasibility depends on many factors such as system loads, procedures for resource management and constraints in implementation. If repurposing is feasible, such re-use of un-used resources will have a positive impact on overall performance.

The second reason is very beneficial to gNB since the hypotheses testing is a costly implementation, and it is not limited only when CG resource allocation is overprovisioned.

However, both reasons are justified when the information received by UTO-UCI can be applied by the gNB. If the information is delivered too late, the feature will be useless to be enabled.

The UTO-UCI feature can be regarded useful if the information received by UTO-UCI can be applied by the gNB for useful functionalities such as repurposing CG PUSCH resources or reducing blind detection of CG PUSCHs.

Generally, repurposing resources requires more time than skipping blind detection, however the required time is gNB implementation specific and not to be specified. Therefore, the gNB can guide the UE by providing useful candidates via configuration that the UE can choose from and convey via UTO-UCI such that the information becomes useful at gNB. The configuration determines the TOs that the UTO-UCI is applicable to.

Repurposing resources requires more time than skipping blind detection, however the required time is gNB implementation specific and not to be specified. The gNB can guide the UE by providing useful configurations needed for UTO-UCI reporting.

Another aspect that is worthwhile to highlight is that we have observed during the discussions in previous meetings that there is a perception that CG configuration periodicity is the time interval that the UTO-UCI is applicable to. See for example “[in one CG period]” in the agreement from RAN1#112bis in Appendix.

It is important to note, that for configuration of resources (in this case, CG resources) many parameters are involved. Availability of resources at least depends on the spectrum being FDD or TDD (and if TDD what configuration), the load and distribution of users in the systems as well as the services, the scheduling algorithms, the link quality, and among them, the information about XR traffic. These information are deciding factor on how the gNB configures a UE with CG configurations including the associated periodicity. The assumption that the gNB can always configure CG configuration such that it is optimized for serving XR traffic, reflects misunderstanding of NW operation.

Therefore, the periodicity of CG configuration, irrespective of whether legacy CG configuration or multi-PUSCH CG configuration is used, should not be coupled with the periodicity that a UTO-UCI is applicable to.

Using the CG configuration and other available information available at NW (such as for information obtained based on BSR or XR awareness, or implementation restrictions at gNB or the capabilities at UE) the gNB can determine a periodicity that a UTO-UCI is applicable to. This periodicity can be the same or different from CG periodicity.

Within this period, the NW determines the TOs that would be affected by the UTO-UCI which consequently can be used for bitmap indication.

In configuration of resources, e.g., CG resources, different parameters are involved. The assumption that the gNB always configures CG configuration such that it is optimized for serving XR traffic, reflects misunderstanding of NW operation.

The periodicity of CG configuration, irrespective of whether legacy CG configuration or multi-PUSCH CG configuration is used, should not be coupled with the periodicity that a UTO-UCI is applicable to, where the latter can be the same or different from the CG periodicity.

#### Comparison of design options and recommendation

Different design options were discussed last meeting under two main categories:

- Option A: Sliding UTO-UCI indicator within a fixed window

- Option B: UTO-UCI indicator for an upcoming sliding window

For each category, different options were discussed and finally 4 options were remained for selection:

- Option A

  - Option A-1a and Option A-2a

- Option B

  - Option B-a and B-b2

With respect to Option-A, both Option A-1a and A-2a provide the same properties in terms of ability of indicating unused TOs. However, Option A-2a eliminates reporting the past/current TOs within a fixed window and appears as an enhancement of Option A-1a where UTO-UCI reports about the past/current/next TOs within a fixed window. However, this enhancement comes at the cost of additional complexity at both UE and gNB without justifying performance gain or additional capability in terms of indicating unused TOs.

Consequences of adopting Option A-2a are summarized below:

- For transmission of every CG PUSCH based on Option A-2a, the size of UTO-UCI changes and the UE has to adjust accordingly the codebook size as well as rate-matching properties to multiplex the UTO-UCI in the corresponding CG PUSCH.

- For detection of every CG PUSCH based on Option A-2a, the gNB has to assume different sizes of UTO-UCI and adjust accordingly the receiver parameters to retrieve the multiplexed UTO-UCI from a CG PUSCH.

- The gain in overhead reduction in Option A-2a as compared to Option A-1a depends on the size of window and number of associated TOs. For practical use cases, the gain is not substantial as the number of affected TOs would be limited, to justify the implementation complexities mentioned above.

Therefore, we are not convinced that Option A-2a would be qualified as a proper design choice.

Both Option A-1a and option A-2a provide the same properties regarding the ability of indicating unused TOs. The UCI overhead reduction in Option A-2a comes at the cost of additional complexity at both UE and gNB without justifying performance gain or additional capability in terms of indicating unused TOs

For a CG configuration with UTO-UCI indication enabled, the candidate Option A-2a to determine the indicated CG PUSCH by a UTO-UCI indication is not supported.

For comparing the remaining options, consider the discussion for consideration of a useful design and using the examples presented in the following to strengthen the arguments in our assessment:

First, we observe the TOs that are used for data transmissions based on incoming traffic for the given configurations (See Figure 5).

- In the illustrative example in Figure 5, a multi-PUSCH CG with periodicity of 5 slots (or 5ms assuming SCS=15 kHz) with a TO in every UL slot is assumed. The CG configuration is examined for two TDD configuration with patterns of DDDUU and DDDDU. In this figure, distribution of XR traffic is shown together the timing of the incoming data.

Next, we study that when the CG configuration is enabled with UTO indication, how options A-1a, B-a and B-b2 perform (See Figure 6 to Figure 11).

- We compare Option A-1a, Option B-a and Option B-b2 for TDD configuration DDDUU in Figure 6, Figure 7 and Figure 8 respectively. These options for TTD configuration DDDDU are compared in Figure 9, Figure 10 and Figure 11, respectively to provide another perspective. In these examples different UTO periodicity values (UTO_period =15/10 slots or 15/10 ms) for Option A-1a and Option B-a, and different number of valid TOs (Nu=6/4 and Nu=3/2 for TDD configurations DDDUU and DDDDU, respectively) are used. The value of UTO_Offset =0 is assumed when applicable for simplicity

Based on studying the content of UTO-UCI indication in different cases shown in Figure 6 to Figure 11, we can observe the following:

- Observation 1) Option A-1a can compromise the usefulness of enabling UTO indication.

  - As shown in Figure 6 and Figure 9, the feature may not be able to indicate the unused TOs for one of multiple period . This is a main drawback of Option A-1a. On contrary, Option B-a/B-b2 due to relying on sliding window, is capable of indication of the unused TO occasions when occurs. This property is the main advantage of  Option B-a/B-b2.

- Observation 2) There is a perception that Option B-a/B-b2 requires UE to perform prediction of incoming data as opposed to Option A-1a and hence it is claimed Option A-1a is practical/realistic. In our view, this perception is not founded.

  - Whether the UE provides UTO information based on availability of the data with or without prediction/estimation of the incoming data is totally a UE implementation capability.

  - Regardless of the option, if the UE is uncertain to use a TO for transmission, it can always indicate “0” or “not unused” for that TO with the possibility of overriding that information later if needed.

  - If the perception is that providing information on usability of TOs for upcoming TOs is complicated, option A-1a is not immune from that since at the start of each UTO period the UE has to provide information about all the TOs in that period, including the latest ones in the period. It is not clear why for this period when a UE is uncertain about the status of a TO, the UE can indicate “not unused”, but in case of Option B-a/B-b2 similar operation would be complicated.

- Observation 3) From simplicity point of view regarding the UTO-UCI size, Option A-1a and option B-b2 benefit from the same UTO-UCI size as opposed to Option B-a that may experience different UTO-UCI sizes.

  - We emphasize that although Option B-a may experience different UTO-UCI sizes, the additional complexity due to different UTO-UCI sizes for Option B-a is still justified, as compared to Option A-1a and Option A-2a. The reason is that the investment in such additional complexity for Option B-a is rewarding because of the improved performance and capability in reporting unused TOs that makes the feature useful as opposed to Option A-1a and Option A-2a.

- Observation 4) From performance point of view, we observe that Option B-a and Option B-a2 are comparable and both superior to Option A-1a (and Option A-2a).

Based on the above Observations, we summarize our assessment as the following:

- Option B-a and Option B-b2 are both superior as compared to Option A-1a that compromises the usefulness of enabling the UTO indication for a configured grant.

- Option B-b2 provides slightly lower complexity due to ensuring a fixed size of UTO-UCI as compared to Option B-a.

Therefore, we propose to adopt either option B-a or Option B-b2 with slight preference towards Option B-b2.

Summary of assessments of Options A-1a/B-a/B-b2:

  - Option B-a and Option B-b2 are both superior as compared to Option A-1a that compromises the usefulness of enabling the UTO indication for a configured grant.

  - The perception that providing information on usability of TOs for upcoming TOs is complicated for Option B-a/B-b2 as compared to option A-1a is not founded, since for Option A-1a, the UE provides  information about all the TOs in that period at the start of each UTO period, including the latest ones in the period.

  - Option B-b2 provides slightly lower complexity due to ensuring a fixed size of UTO-UCI as compared to Option B-a.

For a CG configuration with UTO-UCI indication enabled, one of the candidates Option B-a or Option B-b2 is supported with slight preference towards Option B-b2.

#### Remaining design issues UTO period or offset

With respect to UTO-period in the candidate options both alternatives Alt-1 and Alt-2 work based on the discussion in the consideration of a useful design section and the examples presented above for assessment of the options. However, a choice based on Alt-2, is a preferred choice from our point of view since it avoids unnecessarily complications. The reason is that if Alt-1 is adopted, additional adjustments should be considered by design or by gNB to apply configurations that avoid cases such as a UTO period ends or starts within the duration of a CG PUSCH TO. With Alt-2 such issues do not exist due to its simplicity where the UTO periodicity follows one of multiple of CG periodicity.

Similarly, with respect to UTO-offset in the candidate options both alternatives Alt-1 and Alt-2 work as well. However, if Alt-1 is adopted, additional adjustments should be considered by design or configuration to avoid cases such as a UTO period starts within the duration of a CG PUSCH TO and/or consequently ends within a CG PUSCH duration. Alt-2 makes the design simpler and still ensures that the main benefits of the features are obtained.

Therefore, we prefer Alt-2 for both UTO_period and UTO_offset if these parameters are needed as part of the design.

With respect to UTO_period and UTO_offset, if applicable, if Alt-1 is adopted additional adjustments should be considered by design or by gNB to allow/apply configurations not resulting a UTO period starts/ends within the duration of a CG PUSCH TO. Selecting Alt-2 results in a simpler design while maintaining the benefits of the feature.

With respect to UTO_period and/or UTO_offset, adopt Alt-2 if applicable for the selected design option for UTO-UCI indication.

Figure 5: Illustrative example of a multi-PUSCH CG with periodicity of 5 slots (or 5ms assuming SCS=15 kHz) with a TO in every UL slot is assumed. The CG configuration is examined for two TDD configuration with patterns of DDDUU and DDDDU, respectively. In this figure, distribution of XR traffic is shown together the timing of the incoming data.

Figure 6: Illustrative example of Option A-1a for UTO-UCI indication using the scenario in Figure 5 for TDD configuration DDDUU. The top and bottom figures correspond to UTO_period of 15 and 10 slots, respectively.

Figure 7: Illustrative example of Option B-a for UTO-UCI indication using the scenario in Figure 5 for TDD configuration DDDUU. The top and bottom figures correspond to UTO_period of 15 and 10 slots, respectively. UTO_offset = 0 is assumed. Note that using ‘x’ for a UTO-UCI bit is that further status of incoming traffic is not illustrated due to space limitation for illustration. ‘x’ represents 0 or 1.

Figure 8: Illustrative example of Option B-b2 for UTO-UCI indication using the scenario in Figure 5 for TDD configuration DDDUU. The top and bottom figures correspond to Nu = 6 and 4 TOs, respectively. UTO_offset = 0 is assumed. Note that using ‘x’ for a UTO-UCI bit is that further status of incoming traffic is not illustrated due to space limitation for illustration. ‘x’ represents 0 or 1.

Figure 9: Illustrative example of Option A-1a for UTO-UCI indication using the scenario in Figure 5 for TDD configuration DDDDU. The top and bottom figures correspond to UTO_period of 15 and 10 slots, respectively.

Figure 10: Illustrative example of Option B-a for UTO-UCI indication using the scenario in Figure 5 for TDD configuration DDDDU. The top and bottom figures correspond to UTO_period of 15 and 10 slots, respectively. UTO_offset = 0 is assumed. Note that using ‘x’ for a UTO-UCI bit is that further status of incoming traffic is not illustrated due to space limitation for illustration. ‘x’ represents 0 or 1

Figure 11: Illustrative example of Option B-b2 for UTO-UCI indication using the scenario in Figure 5 for TDD configuration DDDDU. The top and bottom figures correspond to Nu = 6 and 4 TOs, respectively. UTO_offset = 0 is assumed. Note that using ‘x’ for a UTO-UCI bit is that further status of incoming traffic is not illustrated due to space limitation for illustration. ‘x’ represents 0 or 1

### 2.2.2	Encoding and multiplexing of UTO-UCI

With respect to the encoding and multiplexing of UTO-UCI few issues are remained to be resolve.

The following agreements were made during the last meeting. Considering the agreement below, there are two remaining issues that need to be addressed. Additionally, based on the discussion during the last meeting, another related issue was raised regarding the UTO-UCI when the associated CG PUSCH is cancelled.

Agreement

The UCI that provides information about unused CG PUSCH transmission occasions is defined as a “new UCI” (i.e. Alt. 1 of previous agreement).

Agreement

- With respect to PHY two-level priority, for a configured grant PUSCH configuration, the “UTO-UCI” has the same priority level as the configured grant PUSCH.

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

Agreement

The existing CG-UCI encoding and multiplexing procedures are reused for encoding the “UTO-UCI” in a configured grant PUSCH in absence or presence of other UCIs being multiplexed in the PUSCH, by applying the following adjustments:

- The “UTO-UCI” is used instead of CG-UCI in the corresponding procedures for encoding of CG-UCI and/or HARQ-ACK, whichever is present.

- For determining the beta-offset,

  - Beta offset is configured for the “UTO-UCI”

    - If UTO-UCI and HARQ-ACK is not jointly encoded, the beta offset for the “UTO-UCI” is used in the procedures instead of CG-UCI beta offset

    - If UTO-UCI and HARQ-ACK is jointly encoded, HARQ-ACK beta offset is used in the procedures instead of CG-UCI beta offset

- FFS on sequence generation order between UTO-UCI and HARQ-ACK

- FFS on dropping rule between UTO-UCI and HARQ-ACK when joint encoding is not configured

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

In the following we share our views with respect to these three issues.

Issue 1) Sequence generation order between UTO-UCI and HARQ-ACK to form a UCI to multiplex in corresponding CG PUSCH

Issue 2) Dropping rule between UTO-UCI and HARQ-ACK to form a UCI to multiplex in corresponding CG PUSCH

Issue 3) UTO-UCI handling in case of cancellation of corresponding CG PUSCH due to e.g., collision

For these issues, our view in general is that UTO-UCI is not a critical information from overall operation point of view, there is no strong motivation to include additional rules in design and complicate related procedures.

There is no strong motivation for additional rules in design to ensure transmission of UTO-UCI in case its transmission may be dropped.

For example, regarding Issue 2, it has been argued that enabling/disabling joint coding of UTO-UCI and HARQ-ACK should be supported similarly to CG-UCI and HARQ-ACK by configuration of higher layer parameter as it was agreed to reuse the encoding and multiplexing procedures of CG-UCI.

- Firstly, the context of the agreement for reusing CG-UCI encoding/multiplexing procedures is that when UTO-UCI is to be encoded with/without other UCI types and multiplexed on PUSCH, it is agreed then to reuse related procedures for CG-UCI. This is different from enabling or disabling encoding and multiplexing.

- Secondly, the discussion for NR-U was initiated based on concerns that errors in determining the number of HARQ-ACK bits at the UE would lead to potential decoding failure of the CG-PUSCH. It was agreed that if HARQ ACK feedback is scheduled on a configured grant resource, configured grant transmission is skipped on that resource. However, this issue and such errors exists for legacy Rel-15 CG. It is inconsistent way of operation/design to not to support this functionality for legacy Rel-15 CG if it is critical, but support it for UTO-UCI in CG PUSCH.

Therefore, it is not clear why joint encoding should be disabled to motivate a selection/dropping rule. Moreover, by gNB implementation, enough resources can be allocated to avoid any need for ordering rules in case of insufficient resources to include both UTO-UCI and HARQ-ACK in a CG PUSCH. Moreover, if a CG PUSCH including UTO-UCI is dropped due to collision or overlapping with higher priority transmissions, the corresponding UTO-UCI should be dropped and there is no need to introduce new multiplexing rules.

Therefore, we respect to the above issues we propose the following:

For sequence generation order between UTO-UCI and HARQ-ACK to form a UCI to multiplex in the corresponding CG PUSCH, additional specification impact is not needed. Reusing CG-UCI framework based on previous agreements implies that the UCI is ordered by first UTO-UCI, and then HARQ-ACK.

No Need to enable with an RRC (similar/same as cg-UCI-Multiplexing) for joint encoding of UTO-UCI and HARQ-ACK multiplexing.

When a CG PUSCH is cancelled/dropped due to e.g. collision with higher priority transmission, the corresponding UTO-UCI, if present, is dropped with the CG PUSCH.

Another aspect that was brought up at the previous meeting was related to multiplexing the UTO-UCI size of maximum 2-bits as explained in the following.

- Issue 4) 1-2 bits UTO-UCI

Another aspect that was brought up at the previous meeting was related to multiplexing the UTO-UCI size of maximum 2-bits. Since the previous agreements regarding encoding and multiplexing of UTO-UCI rely on reusing CG-UCI related procedures, this issue should be addressed explicitly for UTO-UCI that as opposed to CG-UCI can include maximum 2-bits. For this case, we support the proposal discussed during the last meeting and recommend adopting the following for completeness of the feature design:

Support puncturing based UCI multiplexing for a UCI including UTO-UCI when the size of UCI is equal to or smaller than 2 bits.

  - Note that the 1-2 bits UCI includes UTO-UCI, but may or may not include HARQ-ACK.

### 2.2.3	Other design topics

On addition to the core design, few design topics were discussed in the previous meeting. In the following we share our view on some these topics.

#### 2.2.3.1	Timeline requirements

There are procedures with involvement of configured grant that the corresponding behaviors are specified based on the assumption that the UE may use the configured grant for transmission.  By introducing “unused configured grant PUSCH transmission occasion”, this assumption is not applicable for the configured grant PUSCHs indicated to be unused.  Hence, the specified behaviors need to be revisited to determine whether the assumption of unused CG PUSCH impacts the specified procedures.

In the following, we list few examples with respect to timeline constraint due to configured grant:

- Timeline constraints due to configured grant

  - Per existing specifications, if scheduling a PUSCH overlaps with a configured grant PUSCH, in additional to the PUSCH processing timeline, an additional timeline should be met as shown in Figure 12.

    - If the overlapping configured grant PUSCH is indicated as unused, the additional timeline is not applicable.

  - Per existing specifications, If DL transmission is scheduled in the flexible symbols with configured grant PUSCH, additional timeline is specified to cancel PUSCH and receive DL transmission.

    - If the configured grant PUSCH in flexible symbol is indicated as unused, the additional timeline is not applicable.

Figure 12: Illustration of timeline constraint for scheduling a PUSCH overlapping with a configured grant PUSCH

Therefore, we propose:

Revisit the existing timeline constraints due to configured grant PUSCH to ensure the corresponding constraints are not applicable when a configured grant PUSCH transmission occasion is indicated unused.

#### 2.2.3.3	Applicability to multiple CG configurations

It was discussed whether the UCI indication for unused CG PUSCH resources could be applicable to multiple CG configurations. The proponents explained in the previous meeting that multiple CG configurations may share the same logical channel or data buffer. Hence, in case lack of incoming traffic, no transmission would occur on any of the resources of CG configurations.

We understand the motivations by proponents to enable this capability. However, similarly to the previous discussion on overriding, the solution should not complicate the basic feature design. Therefore, discussions are needed to better understand whether it is beneficial to extend the feature for multiple CG configurations.

Therefore, we propose the following:

Whether to support capability of indication of unused CG PUSCH TOs for multiple CG configurations, study at least the following:

  - whether multiple CG configuration belong the same or different cells

  - whether the key design choices regarding e.g., content and timing of UCI, complicates support of multiple CG configurations.

  - Whether the UCI is carried by all CG PUSCHs associated to all the CG configurations or a sub-set of them.

  - FFS on other conditions.

# 3	Higher layer parameters

## 3.1	Higher layer parameters for multi-PUSCH CG

For this feature, the following RRC parameter is needed:

- nrofTOs_InCGperiod:  number of TOs per period (N in agreement)

Adopt the following RRC parameter for multi-PUSCH CG

  - nrofTOs_InCGperiod:  number of TOs per period (N in agreement)

## 3.2	Higher layer parameters for UTO-UCI indication

For this feature, the following RRC parameters are needed:

- UTO_indication: Enable/disable UTO indication

- UTO_period:  If Option A-1a or Option B-a is supported

- nrof_UTO_UCI:  Maximum number of UTO-UCI bits

- Beta-offset_UTO_UCI: Beta-offset for UTO-UCI

Adopt the following RRC parameters for UTO-UCI indication

  - UTO_indication: Enable/disable UTO indication

  - UTO_period:  If Option A-1a or Option B-a is supported

  - nrof_UTO_UCI:  Maximum number of UTO-UCI bits

  - Beta-offset_UTO_UCI: Beta-offset for UTO-UCI

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Support of repetition for multi-PUSCHs CG, would minimize the specification impact by reusing legacy procedures for NR-U CG and would result in a design with a choice to enable/disable repetition whichever better suited for operation.

Observation 2	The UTO-UCI feature can be regarded useful if the information received by UTO-UCI can be applied by the gNB for useful functionalities such as repurposing CG PUSCH resources or reducing blind detection of CG PUSCHs.

Observation 3	Repurposing resources requires more time than skipping blind detection, however the required time is gNB implementation specific and not to be specified. The gNB can guide the UE by providing useful configurations needed for UTO-UCI reporting.

Observation 4	In configuration of resources, e.g., CG resources, different parameters are involved. The assumption that the gNB always configures CG configuration such that it is optimized for serving XR traffic, reflects misunderstanding of NW operation.

Observation 5	The periodicity of CG configuration, irrespective of whether legacy CG configuration or multi-PUSCH CG configuration is used, should not be coupled with the periodicity that a UTO-UCI is applicable to, where the latter can be the same or different from the CG periodicity.

Observation 6	Both Option A-1a and option A-2a provide the same properties regarding the ability of indicating unused TOs. The UCI overhead reduction in Option A-2a comes at the cost of additional complexity at both UE and gNB without justifying performance gain or additional capability in terms of indicating unused TOs

Observation 7	Summary of assessments of Options A-1a/B-a/B-b2:

	Option B-a and Option B-b2 are both superior as compared to Option A-1a that compromises the usefulness of enabling the UTO indication for a configured grant.

	The perception that providing information on usability of TOs for upcoming TOs is complicated for Option B-a/B-b2 as compared to option A-1a is not founded, since for Option A-1a, the UE provides  information about all the TOs in that period at the start of each UTO period, including the latest ones in the period.

	Option B-b2 provides slightly lower complexity due to ensuring a fixed size of UTO-UCI as compared to Option B-a.

Observation 8	With respect to UTO_period and UTO_offset, if applicable, if Alt-1 is adopted additional adjustments should be considered by design or by gNB to allow/apply configurations not resulting a UTO period starts/ends within the duration of a CG PUSCH TO. Selecting Alt-2 results in a simpler design while maintaining the benefits of the feature.

Observation 9	There is no strong motivation for additional rules in design to ensure transmission of UTO-UCI in case its transmission may be dropped.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse the following proposed conclusion:

	Proposed conclusion: For completeness of the core design of multi-PUSCH CG and ease of specifications, the common understanding is that the legacy parameters/procedures are applied for any parameters/procedures that lack a specific agreement/conclusion during this WI for multi-PUSCH CG design, unless an inconsistency is identified.

	Note: Support of repetition for multi-PUSCH CG should be discussed and concluded separately.

Proposal 2	Repetition is supported for Type-1 and Type-2 multi-PUSCH CG, reusing the existing procedures for NR-U CG.

In the following we share our views about the other design topics in addition to the core design topics.

Proposal 3	Scheduling re-transmission of multiple TBs for corresponding initial transmission of the TBs by configured grant is supported for DCI format 0_1 scrambled with CS-RNTI.

	NDI bit to “1” indicates retransmission for corresponding HARQ process.

Proposal 4	Support of CBG based transmission for multi-PUSCH CG is down prioritized.

One TB over multiple slots improves coverage and does not bring capacity improvements. Currently, retransmission of partial TB is not possible, and if some PUSCH fails, then retransmission will require retransmission of all PUSCHs (whole TB) which degrades the capacity.

Proposal 5	Down prioritize one TB over multiple slots.

Proposal 6	For a CG configuration with UTO-UCI indication enabled, the candidate Option A-2a to determine the indicated CG PUSCH by a UTO-UCI indication is not supported.

Proposal 7	For a CG configuration with UTO-UCI indication enabled, one of the candidates Option B-a or Option B-b2 is supported with slight preference towards Option B-b2.

Proposal 8	With respect to UTO_period and/or UTO_offset, adopt Alt-2 if applicable for the selected design option for UTO-UCI indication.

Proposal 9	For sequence generation order between UTO-UCI and HARQ-ACK to form a UCI to multiplex in the corresponding CG PUSCH, additional specification impact is not needed. Reusing CG-UCI framework based on previous agreements implies that the UCI is ordered by first UTO-UCI, and then HARQ-ACK.

Proposal 10	No Need to enable with an RRC (similar/same as cg-UCI-Multiplexing) for joint encoding of UTO-UCI and HARQ-ACK multiplexing.

Proposal 11	When a CG PUSCH is cancelled/dropped due to e.g. collision with higher priority transmission, the corresponding UTO-UCI, if present, is dropped with the CG PUSCH.

Issue 4) 1-2 bits UTO-UCI

Proposal 12	Support puncturing based UCI multiplexing for a UCI including UTO-UCI when the size of UCI is equal to or smaller than 2 bits.

	Note that the 1-2 bits UCI includes UTO-UCI, but may or may not include HARQ-ACK.

Proposal 13	Revisit the existing timeline constraints due to configured grant PUSCH to ensure the corresponding constraints are not applicable when a configured grant PUSCH transmission occasion is indicated unused.

Proposal 14	Whether to support capability of indication of unused CG PUSCH TOs for multiple CG configurations, study at least the following:

	whether multiple CG configuration belong the same or different cells

	whether the key design choices regarding e.g., content and timing of UCI, complicates support of multiple CG configurations.

	Whether the UCI is carried by all CG PUSCHs associated to all the CG configurations or a sub-set of them.

	FFS on other conditions.

Proposal 15	Adopt the following RRC parameter for multi-PUSCH CG

	nrofTOs_InCGperiod:  number of TOs per period (N in agreement)

Proposal 16	Adopt the following RRC parameters for UTO-UCI indication

	UTO_indication: Enable/disable UTO indication

	UTO_period:  If Option A-1a or Option B-a is supported

	nrof_UTO_UCI:  Maximum number of UTO-UCI bits

	Beta-offset_UTO_UCI: Beta-offset for UTO-UCI

# References

RP-223502		New WID on XR Enhancements for NR; Nokia, Qualcomm

RP-230786		Updated WID on XR Enhancements for NR; Nokia, Qualcomm

# Appendix

## RAN1#112 agreements and conclusions

### The 1st objective

Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

TDRA design:

Agreement

For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study:

- Alt-A: TDRA determination based on repetition framework.

  - Alt-A1: Follow the time domain resource mapping of Type A repetition

    - N configured by higher layers or indicated by activation DCI

    - Single SLIV is determined from TDRA

    - The same SLIV in N PUSCH in consecutive slots per CG period

      - FFS for non-consecutive slots

    - FFS details, including related RRC parameters

  - Alt-A2: Follow the time domain resource mapping of Type B repetition

    - N configured by higher layers or indicated by activation DCI

    - Single SLIV is determined from TDRA

      - The SLIV used for 1st PUSCH per CG period.

    - N consecutive nominal PUSCHs with same duration per CG period

  - Note: N is not necessarily the repetition factor.

FFS details, including related RRC parameters

- Alt-B: TDRA determination based on NR-U framework

    - N and M configured by higher layers

    - Single SLIV is determined from TDRA.

      - The SLIV used for 1st PUSCH per CG period.

    - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period

    - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration.

    - FFS details, including related RRC parameters

- Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs

  - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs

    - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16

    - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

      - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots.

    - FFS details, including related RRC parameters

  - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs

    - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17

    - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

      - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots.

    - FFS details, including related RRC parameters

HARQ ID design:

Conclusion

RAN1 discusses to decide how to determine the HARQ process ID of CG PUSCHs of a multi-PUSCHs CG.

Agreement

For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives:

- Alt. 1:  The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration.

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period.

  - Alt 1-1; X = 1

  - Alt 1-2: X is the number of configured PUSCHs in a period

  - Alt 1-3: X is provided by RRC configuration.

  - FFS details

- Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

  - FFS details

- Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured.

  - FFS on potential enhancements different from previous alternatives

  - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period.

    - FFS details

  - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period.

    - FFS details

- Alt. 4:  The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured.

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

  - FFS on potential enhancements different from previous alternatives

- Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

  - FFS details

- Alt 6: FFS other solutions

MCS/FDRA, other  design parameters:

Agreement

For the PUSCHs parameters in a multi-PUSCHs CG configuration, the configuration/indication parameters except MCS and FDRA of CG PUSCHs in a multi-PUSCHs CG configuration are the same

- FFS: For MCS and FDRA, study further to decide whether/how to be different.

- FFS: Applicability to type-1 and type-2

- Note: TDRA and HP ID are not in this scope of the above statement.

### The 2nd objective:

Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1, RAN2);

What information UTO-UCI contains:

Agreement

For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI:

- Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused”

  - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain.

    - Applicable numbers can be determined from information obtained from configuration.

    - FFS details

  - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

- Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain)

  - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

  - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

- FFS whether/how the unused TO(s) can be associated to multiple CG configuration.

- Other options are not precluded. Proponent companies to provide details.

When UTO-UCI is sent:

Agreement

For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI:

- Option 1: A transmitted CG PUSCH, includes the UCI.

  - FFS details

- Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC.

  - FFS details

- Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion.

  - FFS details

    - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods

- Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s).

  - FFS details

    - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods.

Other options are not precluded. Proponent companies to provide details.

How UTO-UCI is sent:

Agreement

The physical channel that carries the UCI that provides information about unused CG PUSCH transmission occasions is CG PUSCH.

Agreement

Encoding and multiplexing for “the UCI that provides information about unused CG PUSCH transmission occasions” in a CG PUSCH applies encoding and multiplexing procedures for CG-UCI as baseline.

- FFS on details

Agreement

Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision

- Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI.

  - FFS on details

- Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI.

  - FFS on details

- Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI.

  - FFS on details

## RAN1#112bis-e agreements and conclusions

### The 1st objective

Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

TDRA design:

Agreement:

For TDRA design for multi-CG PUSCH, prioritize Alt-A1, Alt-B, and Alt-C2 for further downscoping and/or modification from corresponding agreement in RAN1#112.

- FFS: How to address TDD configuration issue

MCS design:

Agreement:

For CG PUSCHs in a multi-PUSCHs CG configuration, MCS of the CG PUSCHs in the CG configuration are the same between different PUSCH occasions

FDRA design:

Agreement:

For CG PUSCHs in a multi-PUSCHs CG configuration, FDRA of the CG PUSCHs in the CG configuration are the same between different PUSCH occasions

HARQ ID design:

Agreement:

From RAN1 perspective, for determination of HARQ process Ids associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH:

- The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable

  - HARQ Process ID = [floor(X*(CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

  - HARQ Process ID = [floor(X*(CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

    - FFS whether in formulas above X is outside or inside floor operation, i.e.

      - HARQ Process ID = [X*floor( (CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes

      - HARQ Process ID = [X*floor((CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2

  - (Working Assumption) The HARQ process ID of the remaining configured/valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by Y with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

    - FFS whether X=1 or X= the number of configured PUSCHs in the CG period

    - FFS whether Y =1 or a value larger than 1, e.g. Y=2.

      - FFS: If Y>1, Y is determined based on RRC

    - FFS whether Offset 1= 0 or can be a non-zero value.

      - FFS: If offset1 is non-zero, how offset1 is determined (i.e., based on RRC)

    - FFS whether Offset 2= 0 or can be a non-zero value.

      - FFS: If offset2 is non-zero, how offset2 is determined (i.e., based on RRC or dynamically)

- Note1: The equations will be updated accordingly when FFSs are clarified, e.g., if X=1, remove X; if Y=1, remove Y; if non-zero offset1 or Offset 2 is not supported, remove offset 1 or Offset 2.

- Note2: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

### The 2nd objective:

Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1, RAN2);

What information UTO-UCI contains:

Agreement

For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the indicated “unused” CG PUSCH TO(s), if any, by the UCI in a CG PUSCH for a CG configuration

- can be consecutive or non-consecutive CG PUSCH TO(s) in time domain [in one CG period]

- FFS whether/how the unused TO(s) can be associated to multiple CG configuration.

Note: FFSs and further details in corresponding agreement in RAN1#112 for the selected option are remained for further discussion

Note: Above corresponds to Option 2 (w.r.t. agreement in RAN1#112)

Agreement

The UTO-UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”.

- FFS: Details including time duration/range

Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

When UTO-UCI is sent:

Agreement

- Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112)

  - FFS details

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

How UTO-UCI is sent:

Agreement

The UCI that provides information about unused CG PUSCH transmission occasions is defined as a “new UCI” (i.e. Alt. 1 of previous agreement).

Agreement

- With respect to PHY two-level priority, for a configured grant PUSCH configuration, the “UTO-UCI” has the same priority level as the configured grant PUSCH.

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

Agreement

The existing CG-UCI encoding and multiplexing procedures are reused for encoding the “UTO-UCI” in a configured grant PUSCH in absence or presence of other UCIs being multiplexed in the PUSCH, by applying the following adjustments:

- The “UTO-UCI” is used instead of CG-UCI in the corresponding procedures for encoding of CG-UCI and/or HARQ-ACK, whichever is present.

- For determining the beta-offset,

  - Beta offset is configured for the “UTO-UCI”

    - If UTO-UCI and HARQ-ACK is not jointly encoded, the beta offset for the “UTO-UCI” is used in the procedures instead of CG-UCI beta offset

    - If UTO-UCI and HARQ-ACK is jointly encoded, HARQ-ACK beta offset is used in the procedures instead of CG-UCI beta offset

- FFS on sequence generation order between UTO-UCI and HARQ-ACK

- FFS on dropping rule between UTO-UCI and HARQ-ACK when joint encoding is not configured

- Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

## RAN1#113 agreements and conclusions

### The 1st objective

Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

TDRA design:

Working Assumption

For time domain resource allocation for multi-PUSCH CGs, support

- For TDRA determination (based on NR-U framework)

  - For Type-1, follow the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1 of TS 38.214.

    - Note: To determine the configuration of TDRA, PUSCH repetition type A is assumed according to description in 6.1.2.3 in 38.214 for Type-1.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

  - For Type-2, the TDRA table is determined by the TDRA table associated with activation DCI, as defined in Clause 6.1.2.1 of TS 38.214.

    - Note: The DCI format for activation DCI with pusch-RepTypeA is applicable.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

- N is configured by higher layers

- A single SLIV is determined from TDRA.

  - The SLIV used for 1st PUSCH per CG period.

- The PUSCH is used in each of N consecutive slots per CG period

- Note: N is configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. N configuration is independent from cgRetransmissionTimer configuration.

- To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration:

  - For the first PUSCH in the period, follow the legacy procedures.

  - For remaining PUSCHs in the period

    - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.

- FFS: Whether/How to further enhance for operation on TDD

Agreement

For time domain resource allocation for multi-PUSCH CGs, support

- For TDRA determination (based on NR-U framework)

  - For Type-1, follow the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1 of TS 38.214.

    - Note: To determine the configuration of TDRA, PUSCH repetition type A is assumed according to description in 6.1.2.3 in 38.214 for Type-1.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

  - For Type-2, the TDRA table is determined by the TDRA table associated with activation DCI, as defined in Clause 6.1.2.1 of TS 38.214.

    - Note: The DCI format for activation DCI with pusch-RepTypeA is applicable.

      - It is still an open issue whether repetition is supported. If it is decided repetition is not supported, it implies the corresponding repetition factor for is one.

- N is configured by higher layers

- A single SLIV is determined from TDRA.

  - The SLIV used for 1st PUSCH per CG period.

- The PUSCH is used in each of N consecutive slots per CG period

- Note: N is configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. N configuration is independent from cgRetransmissionTimer configuration.

- To determine corresponding slots for CG PUSCHs in a period of a multi-PUSCH CG configuration:

  - For the first PUSCH in the period, follow the legacy procedures.

  - For remaining PUSCHs in the period

    - ForType-1 and Type-2, reuse the corresponding procedures for NR-U by applying the RRC parameters N, instead of cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively.

HARQ ID design:

Agreement:

With respect to the agreement on HARQ process ID determination for multi-PUSCH Cg in RAN1#112bis-e, support the following:

- Y=1

- Offset 1=0 (i.e., remove Offset 1)

- Offset 2=0 (i.e., remove Offset 2)

Agreement

For determination of HARQ process Ids associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH:

- X is outside the floor operation

- X= the number of configured PUSCHs in the CG period

Send an LS to RAN2 to inform this agreement. LS is endorsed in R1-230XXXX.

Agreement

The following working assumption is confirmed

(Working Assumption) The HARQ process ID of the remaining configured/valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

Agreement

From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH:

- The HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable

  - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes

  - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

    - X= the number of configured PUSCHs in the CG period

- The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable.

- Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

Send an LS to RAN2 to convey the above RAN1 agreement. Final LS is in R1-2306233.

### The 2nd objective:

Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1, RAN2);

General

Agreement

- When a CG PUSCH occasion is indicated as “unused”, the UE is not allowed to transmit CG PUSCH on that CG PUSCH occasion.

- For any other CG PUSCH occasion that is NOT indicated as “unused”, the UE is allowed to transmit or not to transmit CG PUSCH on that CG PUSCH occasion as per legacy specification.

  - No RAN1 specification impact

Agreement

- A CG PUSCH occasion indicated as “unused” earlier, is not allowed to be indicated as “NOT unused later”.

- A CG PUSCH occasion indicated as “NOT unused” earlier, can be indicated as “unused” later.

  - FFS: Whether there is specification impact

Agreement:

The UTO-UCI indication for a CG configuration is applicable to only valid CG PUSCH TOs, if any.

- Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB. Otherwise, it is valid.

Agreement

Indication of UTO-UCI by CG PUSCHs associated to a CG configuration, is enabled by configuration of an RRC parameter.

- FFS on whether/how to extend to multiple CG configurations

What information UTO-UCI contains:

Agreement:

For a CG configuration with UTO-UCI indication enabled, to determine the indicated CG PUSCH by a UTO-UCI indication, consider the following options for further down-selection:

Option A-1a:

  - Configure the RRC parameter UTO_period.

    - FFS range value of UTO_period

      - Alt-1: values in time unit (e.g., XR traffic periodicity)

      - Alt-2: one or multiple of CG periodicity given by integer values (n=1, 2, ..)

  - The starting time of the first period of UTO periodicity starts at the same as starting time of the first period of the CG configuration and ends after UTO_period. The next UTO period(s) are followed after the first UTO period.

  - A transmitted CG PUSCH that is confined within a UTO period, carries UTO-UCI that is applicable to the CG PUSCH TOs within the UTO period.

Option A-2a:

  - Configure the RRC parameter UTO_period.

    - FFS range value of UTO_period

      - Alt-1: values in time unit (e.g., XR traffic periodicity)

      - Alt -2: one or multiple of CG periodicity given by integer values (n=1, 2, ..)

- Configure the RRC parameter UTO_offset.

  - FFS range value of UTO_offset

- The starting time of the first period of UTO periodicity starts at the same as starting time of the first period of the CG configuration and ends after UTO_period. The next UTO period(s) are followed after the first UTO period.

- A transmitted CG PUSCH that is confined within a UTO period, carries UTO-UCI that is applicable to the CG PUSCH TOs within the UTO period and after UTO_offset from the end of the transmitted CG PUSCH.

Option B-a:

- Configure the RRC parameter UTO_period.

  - FFS range value of UTO_period

    - Alt-1: values in time unit (e.g., XR traffic periodicity)

    - Alt -2: one or multiple of CG periodicity given by integer value (n=1, 2, ..)

- UTO_offset is the offset value.

  - Alt-1: UTO_Offset is provided by configuration.

    - FFS range value of UTO_offset

  - Alt-2: UTO_Offset = 0

- A transmitted CG PUSCH carries UTO-UCI that is applicable to the valid CG PUSCH TOs that are confined within UTO_period starting with UTO_offset from the end of the transmitted CG PUSCH.

Option B-b2:

- Configure the RRC parameter Nu (Nu is the size of bit-map)

  - FFS range value of Nu

- UTO_offset is the offset value.

  - Alt-1: UTO_Offset is provided by configuration.

    - FFS range value of UTO_offset

  - Alt-2: UTO_Offset = 0

- A transmitted CG PUSCH, carries UTO-UCI that is applicable to the Nu consecutive and valid CG PUSCH TOs, starting with UTO_offset from the end of the transmitted CG PUSCH.

FFS on whether/how to extend to multiple CG configurations