3GPP TSG RAN WG1 #114                                                      R1-2306649

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.5.1.1

Source:	Spreadtrum Communications

Title:	Discussion on SL positioning reference signal

Document for:	Discussion and decision

Introduction

The WID on sidelink positioning is approved in [1].

The specific objectives of this work item are: - Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we share our views on SL positioning reference signal for sidelink positioning.

SL-PRS Frequency and time-domain Pattern

In RAN1#112, the follow agreements on SL-PRS frequency and time-domain pattern had been achieved [2].

Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering.

For SL PRS in shared or dedicated resource pools, it was agreed that at least comb sizes (N) 2, 4 are supported and Comb size 6 is supported at least in dedicated resource pool.  In our view, for shared resource pools, one slot can only be used to transmit SL PRS, so Comb size 6 can also be applied to shared resource pools. In last meeting, it was also agreed that Comb size 1 is supported at least in shared resource pool. In our view, Comb size 1 can also be supported in dedicated resource pool.

Proposal 1: Comb size 6 should be supported in shared resource pool.

Proposal 2: Comb size 1 should be supported in dedicated resource pool.

For SL PRS patterns with full staggering, whether (M,N)=(6,6) is supported had been discussed in last meeting. We support that Comb size 6 should be supported in both shared resource pool and dedicated resource pool. For shared resource pools, one slot can only be used to transmit SL PRS, it is feasible for an SL PRS resource to occupy 6 symbols in one slot. Therefore, in our view, (M,N)=(6,6) should be supported in both shared resource pool and dedicated resource pool.

Proposal 3: (M,N)=(6,6) should be supported in both shared resource pool and dedicated resource pool.

The bandwidth of SL PRS

In RAN1#113, the following agreement on the bandwidth of SL PRS had been achieved [3].

Agreement For a dedicated resource pool, at least the case where SL PRS bandwidth is same as resource pool bandwidth is supported.

In NR, the parameter structure of PRS is a three-tier structure of positioning frequency layer, PRS resource set and PRS resource, that is, one layer can contain multiple resource sets and one resource set can contain multiple resources. In Uu positioning, the configuration of the bandwidth of PRS is per positioning frequency layer. Similarly, the bandwidth of PRS can be configured at the resource pool level or can be fixed to the bandwidth of dedicated resource pool for SL-PRS.

Proposal 4: The bandwidth of PRS can be configured at the resource pool level.

Power control

In RAN1#112b, the following agreement on power control had been achieved [4].

Agreement For the SL PRS open-loop power control, a UE can be configured to use DL pathloss (between TX UE and gNB) only, SL pathloss (between TX UE and RX UE) only, or both DL pathloss and SL pathloss. - The same principle as for PSSCH power control is applied for deciding which (i.e., SL, DL, or SL and DL) pathloss to use. - FFS: SL pathloss reference for open-loop power control for SL PRS. Agreement For a dedicated SL PRS resource pool, options for SL pathloss reference for OLPC for SL PRS are (to be down-selected from): - Option 1: SL PRS as pathloss reference - Option 2: PSCCH DMRS as pathloss reference - Option 3: Both Options 1 and 2 - FFS: Selection between Option 1 and Option 2, including (pre-)configuration.

For a dedicated SL PRS resource pool, one slot only contains PSCCH and SL PRS.  If only PSCCH DMRS is used as pathloss reference, considering the limited number of DMRS resources, pathloss estimation may be inaccurate. Therefore, both SL PRS and PSCCH DMRS can be use as pathloss reference in dedicated resource pool.

Proposal 5: Both SL PRS and PSCCH DMRS can be use as pathloss reference in dedicated resource pool.

For a shared resource pool, there are following two case.

- Case 1: One slot only has SL PRS transmission, no data transmission.

- Case 2: There is both SL PRS transmission and data transmission in one slot.

For case 1, considering that the bandwidth of SL PRS and PSSCH is consistent and in the same slot, SL PRS transmit power is the same as that for PSSCH and PSSCH DMRS is used as pathloss reference.

For case 2, one slot only contains PSCCH, PSSCH for 2-stage SCI and SL PRS.  If only DMRS is used as pathloss reference, considering the limited number of DMRS resources, pathloss estimation may be inaccurate. In this case, SL PRS can be used as pathloss reference.

Proposal 6: Both SL PRS and PSCCH DMRS can be use as pathloss reference in shared resource pool.

AGC & Tx-Rx turnaround time considerations for SL-PRS

In RAN1#113, the follow agreement on frequency and time domain pattern of a SL-PRS resource within a slot [3].

Agreement In a dedicated resource pool, a SL PRS resource is immediately preceded by an AGC symbol unless RAN1 explicitly agrees that an AGC symbol is not included for specific cases (if any). Agreement - In a dedicated resource pool, a SL PRS resource is immediately followed by a gap symbol at least: - if the gap symbol corresponds to the last SL symbol of a slot. - Note: the gap can be used at least for Tx/Rx switching - FFS: when TDM of multiple SL PRS resources within a slot is enabled in the dedicated resource pool - FFS: Other cases. - FFS: for SL PRS resource in a shared resource pool.

In dedicated resource pool, for Comb-based multiplexing of SL PRS from different UEs in a slot, multiple SL PRS resources may occupy the same symbols. In this case, multiple SL PRS resources need to correspond to the same AGC symbol, as shown in Figure 1.

Figure 2: Comb-based multiplexing of SL PRS from different UEs in a slot

For TDM-based multiplexing of SL PRS from different UEs in a slot, different SL PRS resources occupy different symbols. In last meeting, it was agreed that a SL PRS resource is immediately preceded by an AGC symbol unless RAN1 explicitly agrees that an AGC symbol is not included for specific cases (if any). Therefore, in this case, different SL PRS resources need to correspond to different AGC symbol, as shown in Figure 3.

Figure 3: TDM-based multiplexing of SL PRS from different UEs in a slot

In addition, we think that UE can only receive or send SL PRS in the same slot, and cannot perform both SL PRS reception and SL PRS transmission in the same slot. Therefore, setting one symbol per slot (i.e., the last symbol in one slot) for TX-RX or RX-TX turnaround is sufficient, as shown in Figure 2 and Figure 3.

Proposal 7: For Comb-based multiplexing of SL PRS from different UEs in a slot, multiple SL PRS resources can correspond to the same AGC symbol.

Proposal 8: For TDM-based multiplexing of SL PRS from different UEs in a slot, different SL PRS resources can correspond to different AGC symbol.

Conclusion

In this contribution, we provided initial views on SL positioning reference signal. In summary, we have following proposals and observations:

Proposal 1: Comb size 6 should be supported in shared resource pool.

Proposal 2: Comb size 1 should be supported in dedicated resource pool.

Proposal 3: (M,N)=(6,6) should be supported in both shared resource pool and dedicated resource pool.

Proposal 4: The bandwidth of PRS can be configured at the resource pool level.

Proposal 5: Both SL PRS and PSCCH DMRS can be use as pathloss reference in dedicated resource pool.

Proposal 6: Both SL PRS and PSCCH DMRS can be use as pathloss reference in shared resource pool.

Proposal 7: For Comb-based multiplexing of SL PRS from different UEs in a slot, multiple SL PRS resources need to correspond to the same AGC symbol.

Proposal 8: For TDM-based multiplexing of SL PRS from different UEs in a slot, different SL PRS resources occupy can correspond to different AGC symbol or the same AGC symbol.

Reference

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation.

- 3GPP RAN1#112 Chairman’s Notes, February 27th – March 3rd, 2023.

- 3GPP RAN1#113 Chairman’s Notes, May 22nd – May 26th, 2023.

- 3GPP RAN1#112b-e Chairman’s Notes, April 17th – April 26th, 2023.