3GPP TSG RAN WG1 #114                                                                                             R1-2306708

Toulouse, France, August 21st - August 25th, 2023

Source:	vivo

Title:	Discussions on RAN4 LS on UE features for NR ATG

Agenda Item:	5

Document for:	Discussion and Decision

# Introduction

In this contribution, views on UE features for NR ATG are provided based on the RAN4 LS [1].

# Discussions

In the LS from RAN4, following information has been provided on whether some enhancements defined for NTN can be applied to NR ATG band wherein a large guard period is needed to mitigate interference from DL to UL.

In ATG deployment scenario, the distance between ATG UE and BS may achieve 250km, a large guard period is required to prevent interference from DL to UL. In RAN4#107 meeting, RAN4 is discussing possible solutions to mitigate the guard period impact. RAN4 agreed that TDD pattern (30D4S6U) together with the features ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ can be considered as one of possible solutions to mitigate the guard period impact for Rel-18 ATG scenario. UE features 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ had been introduced by RAN1 in Rel-17 NTN. Recall the discussion of RAN1 Rel-17 NTN, RAN1 also explicitly agreed that such two UE capabilities can be used for ATG scenario. However, due to no ATG operating bands at that time, the applicable scenario of these two features didn’t include ATG scenario. In Rel-18 ATG WI, RAN4 has already defined ATG operating bands in TR 38.876, and it will also be specified in TS 38.101-1 and 38.104. Considering all the information above, RAN4 would like to respectfully ask RAN1 to check that if the UE features 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ can be used for Rel-18 NR ATG.

UE features 26-5 ‘Increasing the number of HARQ processes’ and 26-10 ‘K1 range extension’ are defined as following for NTN in NR Rel-17.

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Parent IE in TS 38.331 | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | max-HARQ-ProcessNumber-r17 | BandNR | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-10 | K1 range extension | 1. Support of extended K1 value range of (0..31) for unpaired spectrum |  | k1-RangeExtension-r17 | BandNR | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |

Both features of increasing the number of HARQ processes and extending K1 range can be applied to ATG which was actually not precluded in Rel-17 discussions. However, for both 26-5 and 26-10, they’re only applicable to NTN bands and HAPS operation band. For ATG band introduced in [2] for Rel-18, it will reuse the TN band and not supported in Rel-17. Therefore, a separate set of Rel-18 UE feature groups to be supported for ATG is needed, similar to 26-5 and 26-10.

According to above, we have following proposal on how to reply to RAN4 on the LS being discussed.

Proposal 1:

- Reply to RAN4 that it’s feasible to support both ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ for ATG band, and 2 new corresponding Rel-18 UE feature groups specific for ATG band should be defined.

# Conclusion

In this contribution, we discuss the RAN2 LS on UE features for NR ATG. According to the discussions, we have following proposal.

Proposal 1:

- Reply to RAN4 that it’s feasible to support both ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ for ATG band, and 2 new corresponding Rel-18 UE feature groups specific for ATG band should be defined.

# References

- R2-2309797, LS on UE features for NR ATG, 3GPP TSG RAN WG4#107, May 22nd - 26th, 2023.

- TR 38.876 V0.4.0, Study on Air-to-ground network for NR (Release 18), May, 2023.