3GPP TSG RAN WG1 #114		                                                     R1-2307086

Toulouse, France, Aug 21st – Aug 25th, 2023

Source:	CATT, GOHIGH

Title:	Remaining issues on channel access mechanism for sidelink on unlicensed spectrum

Agenda Item:	9.4.1.1

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new work item NR sidelink evolution was approved and further revision was updated in [1]. The objective of this work item is to satisfy identified requirements of commercial sidelink applications, including increased sidelink data rate and support of new carrier frequencies for sidelink. One of the main objectives is to study and specify sidelink operation on unlicensed spectrum.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, remaining issues related to channel access mechanism for sidelink on unlicensed spectrum are discussed.

Channel access mechanism

In NR sidelink, four sidelink physical channels are defined, including PSCCH, PSSCH, PSFCH and PSBCH. According to the WID [1], these physical channels shall be reused as the baseline in SL-U. However, due to the requirements such as regulation limitation, some changes of sidelink physical channel structures may be needed in SL-U. The detailed design of physical channel structures can be found in our contribution [2].

In this section, the following channel access procedures for dynamic channel occupancy are discussed:

- Channel access priority class (CAPC) for SL

- Contention window adjustment procedures

- Channel access procedures for S-SSB and PSFCH transmission

- CP extension (CPE)

- Remaining issues on Type 2 channel access procedures

Channel access priority class (CAPC) for SL

In RAN1 #110b-e meeting, the following agreement was achieved regarding CAPC for SL [3]:

Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms..

In RAN1 #112b-e meeting, the applicability of NOTE 1 can be confirmed by the following agreement [4]:

Agreement A higher layer parameter “absenceOfAnyOtherTechnology” is supported in Rel-18 for SL transmissions in unlicensed bands (e.g., by level of regulation). - This is per carrier (pre-)configuration - This parameter “absenceOfAnyOtherTechnology” is not expected to be provided if the SL-U carrier is overlapped with either the LTE-LAA or the NR-U carrier.

Therefore, the only remaining issue that need to be discussed is whether mp=1 can be used with p=1 for such as S-SSB and PSFCH transmission. In RAN1#112 meeting, the following agreements was achieved regarding CAPC level for S-SSB and PSFCH transmissions [5].

Agreement The CAPC level that should be used for S-SSB transmissions: - Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission Agreement The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission

Although it was agreed to always use p=1 for S-SSB and PSFCH transmissions, the duration of Type 1 channel access is still highly depending on the value of mp. That is, the smaller the value of mp adopts, the easier the Type 1 channel access is to succeed. Therefore, for S-SSB and PSFCH transmission whose transmission occasion is determined, the minimum value of mp, i.e., mp=1, can be used with p=1.

Proposal 1: Regarding CAPC for SL, mp=1 can be additionally used with p=1 for such as S-SSB and PSFCH transmission.

Contention window adjustment procedures

In RAN1#112 meeting, the following agreement was achieved regarding the definition of “SL reference duration” [5]:

Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

Regarding the detailed contention window adjustment procedures, the following agreements were achieved for unicast with SL-HARQ feedback enabled, groupcast option 2 and SL transmissions without explicit HARQ-ACK feedback during RAN1#112b-e [4] and RAN1#113 meeting [6]:

Agreement #112b-e The ACK/NACK HARQ-ACK feedback corresponding to the PSSCH for SL unicast in the reference duration for the latest SL channel occupancy for which ACK/NACK HARQ-ACK feedback is available is used as follows: - If ‘ACK’ is received, for every priority class p∈1,2,3,4, CWp=CWmin,p ; otherwise CWp is increased to the next allowed value. - Note: this is not applied to the case that reference duration includes multiple PSSCHs with ACK/NACK HARQ-ACK enabled, if that case is supported. Agreement #112b-e The ACK/NACK HARQ-ACK feedback corresponding to the PSSCH for SL groupcast option 2 in the reference duration for the latest SL channel occupancy for which ACK/NACK HARQ-ACK feedback is available is used according to Option 2 when the ratio in Option 1 is not (pre-)configured; otherwise Option 1. - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’ - FFS: how to calculate the ratio - Note: the (pre-)configuration ratio values of 100% is a valid candidate - Option 2: If at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for every priority class p∈1,2,3,4 CWp=CWmin,p; otherwise CWp is increased. Agreement #113 For the (pre-)configurable ratio of received SL HARQ-ACK feedbacks in determining the CWp value for the case of ACK/NACK HARQ-ACK feedback corresponding to the PSSCH for SL groupcast option 2 in the reference duration for the latest SL channel occupancy for which ACK/NACK HARQ-ACK feedback is available, the ratio is calculated by M/P, where M is the number of received ‘ACK’ feedbacks and P is the number of expected HARQ-ACK feedback to be received (equal to the number of members in a group -1). - When the calculated ratio is equal to or above the (pre-)configured ratio, CWp is reset to CWmin,p for every priority class p∈1,2,3,4, otherwise increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. Agreement #113 If UE performs SL transmission using Type 1 channel access procedures associated with the channel access priority class p on a channel and the SL transmission is not associated with explicit HARQ-ACK feedback by the corresponding UE(s), the following is adopted for the CW adjustment. - For every priority class p∈1,2,3,4, use the latest CWp used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. - If the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit, CWp is updated for every priority class p∈1,2,3,4 to the next higher allowed value. - FFS: whether this only applies to a resource pool without PSFCH configuration - FFS: value of X

According to the agreements, the following issues still need to be concluded:

- Whether to support another ending timing of “SL reference duration”

- Whether groupcast option 1 (NACK-only) can be supported for SL-U

- Remaining issue on CW adjustment when SL-HARQ feedback is disabled

- CW adjustment for PSFCH and S-SSB transmission

Issue 1: Whether to support another ending timing of “SL reference duration”

In the last meeting, the following agreement was achieved in order to define the concept of SL transmission burst [6]:

Agreement - A sidelink transmission burst is defined as a set of SL transmissions from a UE without any gaps greater than 16μs. Transmissions from a UE separated by a gap of more than 16μs are considered as separate sidelink transmission bursts. A UE can transmit SL transmission(s) after a gap of up to 16µs within a sidelink transmission burst without sensing the corresponding channel(s) for availability.

According to current agreement, the ending timing of SL reference duration is defined as the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted. While in NR-U, another ending timing is also supported for UL/DL transmission burst. Considering that SL transmission burst is also supported in SL-U, the ending time of the first transmission burst that contains PSSCH(s) with ACK/NACK HARQ-ACK enabled is transmitted should also be considered as another ending time of SL reference duration.

Proposal 2: The ending time of the first transmission burst that contains PSSCH(s) with ACK/NACK HARQ-ACK enabled is transmitted, should also be considered as another ending time of SL reference duration.

Issue 2: Whether groupcast option 1 (NACK-only) can be supported for SL-U

For sidelink transmissions on unlicensed spectrum, transmission loss may occur due to the competition with other systems. If HARQ NACK-based feedback is applied, the Tx UE cannot distinguish whether the Rx UE successfully receives the data or fails to send NACK. In case that the NACK was lost or delayed due to channel access failure, the Tx UE may assume that the data was successfully received and will no longer send retransmission(s), which will lead to reliability decrease. This ACK/DTX ambiguity issue has been discussed in the last meeting, but the majority do not prefer resolving this kind of ambiguity issue. Therefore, in order to guarantee the performance of sidelink transmissions on unlicensed spectrum, it is proposed not to support NACK-based feedback in SL-U.

Proposal 3: Groupcast option 1, i.e., NACK-based HARQ feedback, is not supported for SL-U.

Issue 3: Remaining issue on CW adjustment when SL-HARQ feedback is disabled

For the case when all transmissions within the latest SL reference duration have SL-HARQ feedback disabled, initial agreement was achieved during the last meeting. Some remaining issues, including the applicable scenario and the value of X, should also be further discussed.

In NR SL, even PSFCH resources are configured for a resource pool, HARQ feedback can also be disabled by the SCI indication. Therefore, this procedure should be applied no matter whether PSFCH is configured for a resource pool.

Regarding the value of X, selecting a certain value may be too restrictive. Similar operation can be adopted as the case when CWmax,p has been consecutively used K times for generation of Ninit. In order to avoid frequent reset of CWp, X can be selected by UE from the set of values {8, 9, …,15} for each priority class p∈1,2,3,4.

Proposal 4: If the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit, CWp is updated for every priority class p∈1,2,3,4 to the next higher allowed value. X is selected by UE from the set of values {8, 9, …,15} for each priority class p∈1,2,3,4.

Issue 4: CW adjustment for PSFCH and S-SSB transmission

Considering that PSFCH and S-SSB transmissions may also require Type 1 channel access before the transmissions occur, CW adjustment procedures should be defined.

For PSFCH and S-SSB transmissions, there is no associated HARQ-ACK feedback that can be considered for CW adjustment. If the latest CWp used for any SL transmissions on the channel is used, unfairness may be caused by different demodulation performance of each kind of transmissions. That is, even if CWp was increased after evaluating the channel occupancy situation for a PSCCH/PSSCH transmission, it cannot represent that a PSFCH transmission is also required to monitor the channel for such a long time. Therefore, only the latest CWp used for the same kind of transmissions on the channel should be used for CW adjustment for PSFCH and S-SSB transmissions.

Proposal 5: If a UE transmits PSFCH transmission using Type 1 channel access procedures associated with the channel access priority class p on a channel, the following is adopted for the CW adjustment:

- Use the latest CWp used for any PSFCH transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p.

- If the corresponding channel access priority class p has not been used for any PSFCH transmission on the channel, CWp=CWmin,p  is used.

Proposal 6: If a UE transmits S-SSB transmission using Type 1 channel access procedures associated with the channel access priority class p on a channel, the following is adopted for the CW adjustment:

- Use the latest CWp used for any S-SSB transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p.

- If the corresponding channel access priority class p has not been used for any PSFCH transmission on the channel, CWp=CWmin,p  is used.

Channel access procedure for S-SSB and PSFCH transmission

In RAN1 #111 meeting, the following agreement was achieved regarding channel access mechanism for S-SSB and PSFCH[8]:

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

Although it was agreed that Type 2A channel access is applicable for S-SSB transmissions under the condition which is similar to the condition adopted in NR-U for transmissions with discovery burst, there are still some issues that need to be discussed on this topic.

The first issue is whether/how to define observation period. This came from the consideration of duty cycle ambiguity issue, that is, how to determine the time interval when evaluating the duty cycle restriction. If the time interval is defined as S-SSB period, i.e., 160ms, S-SSB transmission(s) will always occur periodically within the time interval. In this case, the duty cycle can be calculated as (S-SSB transmission(s) within a S-SSB period) divide (S-SSB period). If the time interval is defined as another determined value (e.g., 10ms), the calculation of duty cycle may be impacted by the distribution of S-SSB occasions. When the S-SSB occasions are concentrated within a certain time interval, the duty cycle requirement may not be met in that time interval, but can be met in another time interval. Therefore, in order to simplify the calculation of duty cycle, it is proposed to define the time interval of duty cycle as S-SSB period.

Proposal 7: When Type 2A channel access procedure is applied to S-SSB transmission as discovery burst in NR-U, the total S-SSB transmission duration per UE is at most 1ms and the duty cycle is at most 1/20 with the time interval of duty cycle defined as S-SSB period, i.e., 160ms.

Another issue is about channel access procedure for PSFCH transmission. In NR sidelink, if PSFCH resource is configured in the resource pool, the Rx UE can transmit feedback information on the next available PSFCH resource after receiving data. PSFCH resources are always configured periodically, with the period of 1, 2, or 4. Before each PSFCH resource, there is a guard symbol, whose length is based on the subcarrier spacing. Therefore, Type 2 channel access procedures can be performed before PSFCH transmissions within a COT based on the length of guard symbol.

Proposal 8: Within a shared channel occupancy, Type 2 channel access procedures are applicable to PSFCH transmission in SL-U.

When there is no available COT that can be used for PSFCH transmission, PSFCH may also be transmitted as discovery burst in NR-U. The time interval for evaluating the duty cycle should be the same as S-SSB transmission, i.e., 160ms. Whether the duty cycle of 1/20 can be satisfied is based on the PSFCH period and the frequency of PSFCH transmission.

Proposal 9: Type 2A channel access procedure is applicable for PSFCH transmissions for a UE without a shared channel occupancy, with the total PSFCH transmission duration per UE is at most 1ms and the duty cycle is at most 1/20 with the time interval of duty cycle defined as S-SSB period, i.e., 160ms.

The last issue is whether to introduce short control signaling transmission (SCSt) in SL-U. This issue has been discussed since the first RAN1 meeting of Rel-18. However, due to different ideas of SCSt, the group hasn’t reached a consensus on this issue. The most controversial point is the difference between transmission with discovery burst (using Type 2A channel access) and short control signaling transmission.

Type 2A channel access procedures, which is defined in NR-U, is mainly applicable to the transmission(s) within a COT by sensing the channel to be idle for at least a sensing interval [9]. Short control signaling is adopted in European regulation [10] as well as Chinese regulation [11].  SCSt is defined as transmissions used by the equipment to send management and control signals without sensing the channel for the presence of other signals. That is to say, even if the channel is sensed to be busy, SCSt can also be adopted.

Things are also different regarding the limitations of Type 2A channel access and SCSt. For discovery burst transmitted by a gNB, the transmission(s) duration is restricted to be at most 1ms, and the duty cycle is at most 1/20. For SCSt, the following limitations are applied in European regulation:

- within an observation period of 50 ms, the number of SCS transmissions by the equipment shall be equal to or less than 50; and

- the total duration of the equipment's SCS transmissions shall be less than 2 500 µs within said observation period.

While in China, the following limitations are applied:

- within an observation period of 50 ms, the number of SCS transmissions by the equipment shall be equal to or less than 50; and

- the total duration of the equipment's SCS transmissions shall be less than 2 500 µs within said observation period, or the duty cycle shall be no more than 1/10.

It can be observed that the definition and limitations are both different between Type 2A channel access and SCSt. Therefore, these channel access procedures should be discussed separately for S-SSB and PSFCH transmission.

Observation 1: The definition and limitations are both different between Type 2A channel access and SCSt.

Proposal 10: Type 2A channel access procedures and SCSt should be discussed separately for S-SSB and PSFCH transmission.

Considering that both S-SSB and PSFCH transmission can be identified as some kind of management and control information, short control signaling can also be used for S-SSB and PSFCH transmissions. Furthermore, since different countries may have different regulation requirements, the supportive of SCS transmission shall meet each country’s regulation.

Proposal 11: UE may perform S-SSB or PSFCH transmission as SCSt without sensing the channel when the transmission meets the regulation for SCSt in each country.

CP extension (CPE)

In RAN1#113 meeting, the following agreements were achieved regarding CPE for SL-U [6]:

Agreement - A set of all candidate CPE starting positions for SL transmission in FR1 unlicensed spectrum is pre-defined in TS38.211 as followed. - For 15kHz SCS, the set contains values {Tsym_1+16μs, Tsym_1+25μs, Tsym_1+34μs, Tsym_1+43μs, Tsym_1+52μs, Tsym_1+61μs, Tsym_0} - For 30kHz SCS, the set of values for CPE window of one-symbol length is {Tsym_1+16μs, Tsym_1+25μs, Tsym_0} - For 30kHz SCS, the set of values for CPE window of two-symbol length is {Tsym_2+16μs, Tsym_2+25μs, Tsym_2+34μs, Tsym_2+43μs, Tsym_2+52μs, Tsym_2+61μs, Tsym_0} - For 60kHz SCS, the set of values for CPE window of one-symbol length is {Tsym_1+16μs, Tsym_0} - For 60kHz SCS, the set of values for CPE window of two-symbol length is {Tsym_2+16μs, Tsym_2+25μs, Tsym_0} - Tsym_0 is the starting position of the next AGC symbol - Note: when the CPE starting position is Tsym_0, it means that the CPE length is 0 - Tsym_1 is the starting position of the first symbol just before the next AGC symbol - Tsym_2 is the starting position of the second symbol just before the next AGC symbol Agreement When UE performs Type 1 channel access to initiate a COT for PSCCH/PSSCH transmission: - Scheme 1: The UE selects the (pre-)configured default CPE starting position. - Scheme 2: A CPE starting position is randomly selected among one or multiple CPE starting candidate positions (pre-)configured per priority of the PSCCH/PSSCH transmission - The mapping one or multiple CPE starting positions per priority can be up to (pre-)configuration. - FFS: whether the priority should be the L1 priority or CAPC (to be down-selected in RAN1#114) - For partial and full RB set resource allocations - If a resource reservation is transmitted or resource reservations is detected for the slot and the RB set(s) of the intended PSCCH/PSSCH transmission, Scheme 1 is applied; otherwise, Scheme 2 is applied - FFS: other conditions to determine whether to use scheme 1 or scheme 2 - FFS: further enhancements for the full RB set case Agreement A set of one or more candidate CPE starting position(s) that can be used for PSCCH/PSSCH transmission within a COT (for the case of sharing a COT) and outside a COT (for the case of initiating a COT) is separately (pre-)configured per resource pool based on the pre-defined set of all candidate CPE starting positions. - Note: for the case of sharing a COT, the CPE occurs after LBT gap for type 2A/2B/2C - FFS whether a subset of candidate CPE starting position(s) that can be used for PSCCH/PSSCH transmission within a COT is indicated by SCI carrying COT sharing information - FFS whether default starting position is included in each set Agreement When UE performs Type 2 channel access to start transmitting within a shared COT (to be further studied and down-selected in RAN1#114): - Alt. 1: Use the method for using CPE for the case when UE performs Type 1 channel access to initiate a COT for PSCCH/PSSCH transmission - Alt. 2: Use only the (pre-)configured default CPE starting position - Alt. 3: use CPE to make the gap smaller or equal 16us - Alt. 4: others

According to the agreements, the following issues need to be further considered:

- CPE starting position for PSCCH/PSSCH

- Candidate CPE starting position sets

- CPE starting position for PSFCH

- CPE starting position for S-SSB

Issue 1: CPE starting position for PSCCH/PSSCH

In the last meeting, companies devoted a full discussion regarding the CPE starting position for PSCCH/PSSCH transmission, especially for the case of initiating a COT. The only remaining issue that needs to be determined to finalize the discussion of this case is whether the priority that one or multiple CPE starting position mapping to should be the L1 priority or CAPC level.

According to the following RAN2 agreement [14], CAPC level is mapped to PQI, which is used as a reference to PC5 QoS characteristics.

Agreements on SL CAPC mapping table: 1: Mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. 2: Mapping PQI 59/61 to CAPC priority class 3. 3: Mapping PQI 25 to CAPC priority class 2. 4: Mapping PQI 24/26/60 to CAPC priority class 1.

The CAPC level directly impacts the channel access procedures and the channel occupancy time that can be shared. The intention of setting lower mp value and lower CWp is to give the opportunity for the transmission with higher CAPC level to access the channel more easily. Similar intention can be considered when selecting the CPE starting position. For example, transmission with a lower CAPC level may select a later CPE starting position after Type 1 channel access is succeed.

Proposal 12: For the case of initiating a COT, one or multiple CPE starting candidate positions are (pre-)configured per CAPC level of the PSCCH/PSSCH transmission.

For transmission within COT, four alternatives were provided based on the agreement. Alt.1 is to use the same method as the case of initiating a COT, i.e., different CPE stating position may be used when reservation information was not transmitted or detected. This method is suitable for initiating a COT, since competition may be happened among different RATs. But for COT sharing case, only sidelink transmissions are assumed to be performed, and thus NR SL design, where FDMed or full-overlapped transmissions are allowed, should be respected as far as possible. In this case, introducing inter-UE blocking issue by supporting multiple CPE staring positions is not preferred. Furthermore, since only Type 2 channel access procedures are required to perform when a UE can utilize a COT shared by a COT initiating UE, the transmission resources within a COT may be filled by different transmissions. If multiple CPE starting positions are adopted, critical inter-UE blocking issue may be caused among the transmissions which could have been transmitted at the same time. Therefore, when multiple CPE starting positions are (pre-)configured for PSCCH/PSSCH, for transmissions within COT, a (pre-)configured default CPE starting position is selected by the UE.

Proposal 13: When UE performs Type 2 channel access to start transmitting within a shared COT:

- Alt. 2: Use only the (pre-)configured default CPE starting position

Issue 2: Candidate CPE starting position sets

In the last meeting, it was agreed that two separate sets of candidate CPE starting position(s) for PSCCH/PSSCH should be (pre-)configured for transmission within COT and transmission outside COT, and the only remaining issue is how to consider the default CPE starting position.

As analyzed before, only the (pre-)configured default CPE starting position should be used for the case of COT sharing. The candidate value can be Tsym_1+16μs and Tsym_1+25μs, in order to leave the space for the UE to perform Type 2A/2B/2C channel access procedures. For the case of initiating a COT, mapping between multiple CPE starting positions and CAPC level should be defined. Regarding the default CPE starting position, the same value should be used as the COT sharing case, in order to not block other transmissions within a COT.

Proposal 14: CPE starting candidate position(s) sets are (pre-)configured per resource pool as follows:

- A default CPE starting position is (pre-)configured and the candidate value can be {Tsym_1+16μs,  Tsym_1+25μs}.

- A set of one or multiple candidate CPE starting positions are (pre-)configured per CAPC level of the PSCCH/PSSCH transmission based on the pre-defined set of all candidate CPE starting positions.

Issue 3: CPE starting position for PSFCH

According to the design in NR SL, PSFCH symbols are determined by resource pool configuration. That is, PSFCH transmission occasions are determined in each resource pool. Therefore, the CPE starting position of PSFCH should also be (pre-)configured in each RP. Considering that there is also a guard symbol before PSFCH symbol, CPE starting position can be within the guard symbol, i.e., within the symbol just before the next PSFCH symbol.

Proposal 15: The CPE starting position for PSFCH should be (pre-)configured in each RP within the symbol just before the next PSFCH symbol.

Issue 4: CPE starting position for S-SSB

In NR SL, S-SSB resource set within each S-SSB period is pre-configured. Each UE who is triggered to transmit S-SSB should perform S-SSB transmission on the determined resource within a S-SSB resource set. This principle should be reused for S-SSB transmission in SL-U. Each UE who has S-SSB to transmit should not be blocked by other UE’s CPE. That is, a single CPE starting position should be adopted for S-SSB transmission and the determined starting position should be (pre-)configured within the symbol just before the next AGC symbol.

Proposal 16: A single CPE starting position is adopted for S-SSB transmission. The CPE starting position should be (pre-)configured within the symbol just before the next AGC symbol.

Remaining issues on Type 2 channel access procedures

In RAN1 #110 meeting, the following agreement was achieved regarding Type 2 channel access procedures [13]:

Agreement - Type 2A/2B/2C SL channel access procedures - Type 2A channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≥ 25μs in a shared channel occupancy - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2A is used also for the case of short control signalling transmission - Type 2B channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE at least when the gap is 16μs in a shared channel occupancy - FFS the case when the gap is between 16 and 25us - FFS any other transmission by a UE (e.g., other than COT sharing) - Type 2C channel access procedure is applicable to the following case: - Transmission(s) by a UE following transmission(s) by another UE for a gap ≤ 16μs in a shared channel occupancy and the duration of the corresponding transmission is at most 584us. - FFS any other transmission by a UE (e.g., other than COT sharing) - FFS whether Type 2C is used also for the case of short control signalling transmission - FFS under which conditions (other than the gap) UEs can apply the Type 2A/2B/2C SL channel access procedures - FFS under which conditions Type 2B or Type 2C is applied in case of a gap of 16 μs

One remaining issue is under which conditions Type 2B or Type 2C should be applied in case of a gap of 16 μs. The following FL proposal was discussed in RAN1 #112b-e meeting but failed to achieve an agreement duo to concerns of the inter-UE blocking issue when Type 2B or Type 2C is performed based on UE implementation.

Proposal 2-1 (I): - When the gap between the transmission(s) by a UE following transmission(s) by another UE is 16μs and the duration of the corresponding transmission is at most 584µs in a shared channel occupancy, it is up to UE implementation to perform either Type 2B or Type 2C channel access procedures.

In fact, the proposal clearly defines the condition that the gap between the transmission(s) by a UE following transmission(s) by another UE is 16μs and the duration of the corresponding transmission is at most 584µs in a shared channel occupancy. That is, this proposal is adopted if and only if the transmission gap is 16μs. No matter whether Type 2B or Type 2C channel access procedures are performed, the gap will always be here. Therefore, it is unclear why the inter-UE blocking issue will be caused. The FL proposal 2-1 in RAN1#112b-e meeting should be supported.

Proposal 17: Support FL proposal 2-1 in RAN1 #112b-e meeting:

- When the gap between the transmission(s) by a UE following transmission(s) by another UE is 16μs and the duration of the corresponding transmission is at most 584µs in a shared channel occupancy, it is up to UE implementation to perform either Type 2B or Type 2C channel access procedures.

Sharing of channel occupancy time

In RAN1 #113 meeting, the following agreements were achieved regarding UE-to-UE COT sharing [6].

Agreement For the time-domain information to be included as part of COT sharing information, at least the following is included: - Remaining COT duration - FFS it is an absolute time length in ms or in number of slots, and payload size - FFS: how to determine the shared slots and the starting time of the shared slots, e.g. if some slots are only intended for the COT-initiating UE and not to be shared with other UEs Working assumption - For UE-to-UE COT sharing in SL-U, a parameter “ue-toUE-COT-SharingED-Threshold” is configured to be used in the energy detection threshold adaptation procedure (similar to ul-toDL-COT-SharingED-Threshold-r16 used for UL-to-DL COT sharing in NR-U) - FFS candidate value(s) (need to take into consideration of different UE power class) and the granularity for the configuration Working assumption The required UE processing time for decoding COT-SI is the same as SCI decoding, which is Tproc,0SL as defined by Table 8.1.4-1 in TS38.214. - The UE processing time starts from the end of slot of the SCI that carries the COT sharing information in a slot Working assumption For the case where a COT initiating UE uses Type 1 channel access procedure to initiate a SL transmission, - it is supported that the COT initiating UE can transmit transmission(s) within the same channel occupancy that follows a COT responding UE’s SL transmission(s) according to the channel access procedures. - FFS details of the SL transmission(s) from responding UE - FFS whether the above should be based on NR-U DL-UL-UL (Clause 4.2.1.0.3 of TS37.213) or DL-UL-DL (Clause 4.1.3 of TS37.213) COT sharing principle and its corresponding transmission gap requirements - FFS any other condition and restriction Agreement If a responding UE shares a channel occupancy initiated by a COT initiating UE using Type 1 SL channel access procedure on a channel, the responding UE may transmit a SL transmission that follows a SL transmission by the COT initiating UE after a gap as follows: - If the gap is at least 25μs, the responding UE can transmit the SL transmission on the shared channel after performing Type 2A SL channel access procedures. - If the gap is equal to 16μs, the responding UE can transmit the SL transmission on the shared channel after performing Type 2B SL channel access procedures. - If the gap is up to 16μs and the transmission is limited to 584μs, the responding UE can transmit the SL transmission on the channel after performing Type 2C SL channel access.

The following proposals were raised by the FL, but no consensus was achieved [7].

Proposal 5-1 (I): - UE forwarding/relaying information relating to a COT initiated by another UE is not supported in Rel-18. Proposal 5-2 (I): - A responding UE’s PSFCH transmission(s) within RB set(s) corresponding to a shared COT can be transmitted to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator. Proposal 5-3 (III): - Additional ID(s) can be included as part of COT sharing information from the COT initiator UE. - Additional ID for broadcast and groupcast is the L1 destination ID. - Additional ID(s) for a unicast are L1 source/destination ID(s) for unicast. - Maximum number of additional ID(s) is 2 Proposal 5-5 (I): For the agreed COT sharing information in RAN1#112bis-e, - CAPC used for initiating the COT is carried in the 1st stage SCI, the payload size is 4-bit - Existing / legacy R16/17 L1 source and destination IDs are carried in the 2nd stage SCI and their payload sizes are the same as per R16/R17

Based on the above agreements and FL proposals, the following issues should be further studied:

- COT sharing principle

- Further restrictions on responding UE’s transmission

- Contents of COT sharing information

- Container for COT sharing information

- Conditions for transmitting COT sharing information

- UE behavior after receiving multiple COT sharing information

Issue 1: COT sharing principle

In the last meeting, initial discussion regarding the COT sharing principle of SL-U was triggered. However, it was observed that companies may still have different views at this late stage and thus a working assumption was made for further consideration and discussion.

In NR-U, two kinds of COT sharing procedure are defined [9]. One is referring to DL channel access procedures in a shared channel occupancy, including the case where a gNB shares a channel occupancy initiated by a UE and the case where a gNB resumes its transmission after initiating a COT and sharing the COT with a UE. The former requires at most 25us gap between UL transmission and DL transmission, while the latter is applied when any gap between any two transmissions in the gNB channel occupancy is at most 25us.

Another COT sharing procedure is for UL channel access. UL transmissions can occur within the time interval starting at t0 and ending at t0+TCO, where TCO=Tmcot,p+Tg, t0 is the time instant when the eNB/gNB has started transmission, and Tg is the total duration of all gaps of duration greater than 25us that occur between the DL transmissions of the eNB/gNB and UL transmissions scheduled by the eNB/gNB, and between any two UL transmissions scheduled by the eNB/gNB starting from t0. It can be observed that transmission gap is not restricted to be at most 25us for NR-U UL transmissions in a shared channel occupancy.

Observation 2: The transmission gap is not restricted to be at most 25us for NR-U UL transmissions in a shared channel occupancy.

NR SL mode-2 resource allocation mechanism allows UE to perform autonomous resource selection based on SCI decoding and RSRP measurement. It may be impossible to guarantee that SL transmissions performed by different UEs are transmitted back-to-back. Even for the case that PSFCH transmission is performed by the COT responding UE, the transmission gap between COT initiating UE and COT responding UE is at least 10 symbols, which is much longer than 25us. If NR-U DL channel access principle in a shared channel occupancy is adopted, both COT responding UE and COT initiating UE may be hard to use COT once the COT is initiated, and the efficiency of COT sharing mechanism may be decreased.

Considering the nature of NR SL, NR-U UL channel access procedure in a shared channel occupancy is more suitable for SL-U. That is, a UE may transmit a SL transmission after performing Type 2A channel access procedures if the gap before the transmission is at least 25us, no matter whether it shares a channel occupancy initiated by another UE or it intend to resume its transmission within its COT. Furthermore, Type 2B or Type 2C channel access procedures can also be performed if the gap is equal to 16us or up to 16us, respectively.

Proposal 18: SL-U COT sharing should be based on NR-U DL-UL-UL (Clause 4.2.1.0.3 of TS37.213) COT sharing principle, that is, it is not required to guarantee the transmission gap to be at most 25us.

Another issue is whether UE forwarding/relaying information relating to a COT initiated by another UE is supported in SL-U. This issue remained for further discussion since RAN1#111 meeting, but no consensus was achieved due to objection from two companies. It seems really straightforward to support FL proposal 5-1. If UE forwarding/relaying a COT is supported, more issues may be caused while few benefits can be identified at this stage. Therefore, if there is strong willingness to support this kind of behavior, further consideration in future release is acceptable, but not now.

Proposal 19: Support FL proposal 5-1 in RAN1 #113 meeting:

- UE forwarding/relaying information relating to a COT initiated by another UE is not supported in Rel-18.

Issue 2: Further restrictions on responding UE’s transmission

Further restrictions on responding UE’s PSFCH transmission was discussed for several meetings, but companies’ views are still diverging. Majority company supports not to further restrict the PSFCH transmission, that is, a responding UE’s PSFCH transmission(s) within RB set(s) corresponding to a shared COT can be transmitted to UEs other than the COT initiator without requiring that at least one of PSFCH transmissions is intended for the COT initiator, while some companies think it is not aligned with the NR-U principle. In fact, there is no feedback channel in Uu interface as sidelink. The period of PSFCH resource in sidelink can be as long as four slots, which may be longer than the maximum COT sharing duration. That is, PSFCH transmission may not be transmitted within the COT that indicated by the COT initiating UE. In this case, allowing a responding UE to transmit PSFCH to the UE other than the initiator can provide more opportunity to the responding UE to transmit PSFCH. Otherwise, Type 1 channel access will be used which suffer even longer channel access duration.

Proposal 20: A responding UE is allowed to transmit PSFCH(s) to UE(s) other than the COT initiator.

For PSCCH/PSSCH transmission, allowing a responding UE to transmit PSSCH/PSCCH to destination other than the COT initiating UE is also beneficial to the whole COT sharing mechanism within the SL-U system. After a COT is initialized, it can be aware by any UE who has received the COT sharing information. There may be some UEs that have PSSCH/PSCCH transmissions towards the COT initiating UE, but the transmissions may not consecutive. Therefore, in order to maintain the COT, it is supported for a responding UE transmitting PSSCH/PSCCH to destination ID other than the source/destination ID of the COT initiating transmission.

Proposal 21: A responding UE can transmit PSSCH/PSCCH to destination ID other than the source/destination ID of the COT initiating transmission.

Issue 3: Contents of COT sharing information

In RAN1#112b-e meeting, it was agreed to include CAPC level and legacy L1 source/destination ID in the COT sharing information. In RAN1#113 meeting, remaining COT duration was considered as time-domain information to be included as part of COT sharing information, but issues such as the granularity and payload size still require further discussion.

Before discussing the remaining issues, how to interpret remaining COT duration should be first identified. In NR-U, COT sharing information is indicated by the gNB, while each UE sharing the COT can be distributed in the COT uniformly. UE can perform transmission within TCO=Tmcot,p+Tg, and the total channel occupancy duration is a relative time. The absolute duration of the COT depends on how many gaps of duration greater than 25μs within the COT. The restriction of the absolute duration of the COT can be up to the regulation of each country, such as 20ms in Chinese regulation.

Observation 3: In NR-U, the COT sharing duration is a relative time, and the absolute duration of the COT depends on how many gaps of duration greater than 25μs are within the COT.

For sidelink UE without the coordination by the gNB, it should first identify the availability of COT after receiving COT sharing information. If relative time is used to determine the remaining COT duration, the responding UE may not have sufficient information to determine the remaining available COT, due to the impact of distributed resource allocation mechanism. Therefore, using the absolute time to determine the remaining COT duration is a better choice. Considering the granularity of resource selection in NR SL is slot, it is proposed to indicate remaining COT duration also in number of slots. For 60kHz SCS, the maximum channel occupancy duration is 40 slots corresponding to 10ms, and thus the payload size of remaining COT duration is 6-bit.

Proposal 22: Remaining COT duration included as part of COT sharing information is indicated in number of slots and the payload size is 6-bit.

Another time-domain COT sharing information, i.e., COT starting offset, was also discussed during the last meeting, but no consensus was achieved. The pros think this information can inform the responder of where TX can be performed, while the cons do not think indicating this kind of offset is necessary. This divergence also comes from different understanding of COT sharing principle. The motivation of indicating the offset through COT sharing information is to prevent the responding UE from performing transmission at the same time as the COT initiating UE. However, FDMed and concurrent transmissions are always allowed in NR SL, which should not be violated in SL-U. Under this situation, once the responding UE successfully decode the COT sharing information, transmissions can be performed any time within the COT, no matter whether the COT initiating UE is performing transmission or not.

Based on the working assumption made in the last meeting about the COT-SI decoding time, the usage of shared COT is illustrated as Figure 1. A UE initiates a channel occupancy after finishing Type 1 channel access procedures and provides COT-SI with its PSCCH/PSSCH transmission (TX1). A responding UE can thus perform its transmission after successfully decoding the COT-SI taking processing time (Tproc,0) into account, even if the COT initiating UE hasn’t finished its transmissions.

Figure 1: The usage of shared channel occupancy time

Proposal 23: COT sharing offset is not considered as part of COT sharing information. The responding UE(s) determines the available channel occupancy duration only based on the remaining COT duration.

For the frequency domain information to be included as part of COT sharing information, the applicable RB set(s) of the COT is also valuable, which can be indicated implicitly by FRIVRBset according to the following working assumption:

Working assumption Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Use FRIVRBset to indicate used RB set index(s) - FRIVRBset is conveyed in 1st SCI - FRIVRBset is calculated as below - If sl-MaxNumPerReserve is 2 then - FRIVRBset=nRBset,1start+i=1LRBset-1NRBset+1-i - If sl-MaxNumPerReserve is 3 then - FRIVRBset=nRBset,1start+nRBset,2start⋅NRBset+1-LRBset+i=1LRBset-1NRBset+1-i2 - where - nRBset,1start denotes the starting RB set index for the second resource - nRBset,2start denotes the starting RB set index for the third resource - NRBset is the number of RB sets in a resource pool - LRBset is the number of used RB sets for each of the indicated resources

Therefore, it is proposed to further include applicable RB set(s) as part of COT sharing information.

Proposal 24: COT sharing information should also include applicable RB set(s) which can be indicated implicitly by FRIVRBset .

Issue 4: Container for COT sharing information

In the last meeting, the container for CAPC level and legacy L1 source/destination ID were discussed, but agreement was not made due to lack of time. In order to finalize the design of COT sharing information, the container and the payload size of each information should be clearly defined.

Legacy L1 source/destination ID are carried in the 2nd stage SCI, and the payload size can be the same as defined in R16/R17. The applicable RB set(s) can be indicated implicitly by FRIVRBset , whose payload size is determined by the number of RB sets in the corresponding resource pool and the maximum number of reserved PSCCH/PSSCH resources that can be indicated by an SCI. According to the above working assumption, this information is carried by the 1st stage SCI.

Regarding the CAPC level, four values were clearly defined, and thus 2-bit payload size is sufficient. Since the newly defined FRIVRBset will occupy additional fields in 1st stage SCI, carrying CAPC level by the 2nd stage SCI seems more reasonable. For the remaining COT duration, 6-bit payload size is required as analyzed before. The container should also be the 2nd stage SCI from the perspective of overhead.

Proposal 25: For the COT sharing information:

- The applicable RB set(s) can be indicated implicitly by FRIVRBset, conveyed in the 1st stage SCI, whose payload size is determined by the number of RB sets in the corresponding resource pool and the maximum number of reserved PSCCH/PSSCH resources that can be indicated by the SCI.

- Existing / legacy R16/17 L1 source and destination IDs are carried in the 2nd stage SCI and their payload sizes are the same as per R16/R17.

- CAPC level and the remaining COT duration are carried in the 2nd stage SCI, and the corresponding payload size are 2-bit and 6-bit, respectively.

Issue 5: Conditions for transmitting COT sharing information

This issue is to define the conditions under which UE can transmit COT sharing information, and the following principles should be considered:

- The first condition is that UE has data to transmit. Since COT sharing information should be carried by SCI, only when the UE has SCI to transmit, the COT sharing information can be transmitted. From the perspective of complexity reduction, standalone SCI should not be introduced in SL-U. That is, SCI can only be transmitted when UE has corresponding data information to transmit.

- Another condition is that the remaining COT is larger than a (pre-)configured threshold, so that more UEs can utilize COT to perform Type 2 channel access. Considering the maximum COT depends on the channel access priority, channel access priority could also be another metric, i.e., UE can perform COT sharing only when the channel access priority value is larger than a (pre-)configured value.

Proposal 26: The following conditions should be introduced under which UE can perform COT sharing:

- UE has data to transmit.

- The remaining COT is larger than a (pre-)configured threshold or the channel access priority value is larger than a (pre-)configured value.

Since standalone SCI is not supported in SL-U, when a UE performs Type 1 channel access for S-SSB or PSFCH transmission, a UE-to-UE COT sharing should not be started. Furthermore, S-SSB and PSFCH transmission occasions are determined by (pre-)configuration. If a certain transmission occasion is missed, the corresponding transmission may be dropped, leading to system performance decrease. Therefore, when there is no COT that can be used for S-SSB or PSFCH transmission, short control signaling is the preferred transmission method than Type 1 channel access. In a word, no matter whether Type 1 channel access is used or not for S-SSB or PSFCH transmission, UE-to-UE COT sharing started with S-SSB or PSFCH transmission is not supported.

Proposal 27: UE-to-UE COT sharing started with S-SSB or PSFCH transmission is not supported.

Issue 6: UE behavior after receiving multiple COT sharing information

Sidelink system can be considered as a distributive system when there is no coordination from the gNB. All the sidelink UE operating with mode-2 will perform autonomous resource selection as well as the channel access procedure. Therefore, when there is no available COT, multi-UEs may perform Type 1 channel access procedures at the same time, while multiple sets of COT sharing information may be received by a certain UE. Due to the fact that only one COT can be used by a UE, UE behavior after receiving multiple COT sharing information should be considered, such as the selection among the COT and the determination of the corresponding CPE starting position.

Proposal 28: UE behavior after receiving multiple COT sharing information should be considered, such as the selection among the COT and the determination of the corresponding CPE starting position.

Multiple channel access procedure

During the previous RAN1 meeting, the following agreement was achieved regarding multiple channel access procedure:

Agreement #109-e Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation as defined by TS37.213 for NR-U (wherever applicable) - FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and whether/how they are applied in SL mode 1 and mode 2 operation Agreement #110b-e For dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as baseline for transmission on multiple channels - FFS: whether transmission of PSFCH and/or S-SSB on a subset of RB sets is supported (using the NR-U DL channel access procedure as baseline) - FFS any necessary enhancement and modification for the SL-U operation Agreement #111 - For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets Agreement #112b-e For dynamic channel access mode with multi-channel case in SL-U, both NR-U DL Type A and Type B multi-channel access procedure are supported for multiple PSFCH transmissions on multiple channels. - FFS: It is up to UE implementation to perform either Type A or Type B multi-channel access procedure. - FFS: whether this can initiate a shared COT - FFS: whether there is any special handling needed for transmission in a shared COT on one or more of the channels Agreement #112b-e Channel access procedures for SL multi-channel transmission(s) include the following cases. - If a UE is scheduled to transmit on a set of channels C, and if the SL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C, or - If a UE intends to perform sidelink transmissions on configured resources on the set of channels C, and if the SL transmissions are configured to start transmissions at the same time on all channels in the set of channels C, or - If a UE intends to perform sidelink transmissions on selected resources on the set of channel C, and if SL transmissions are to start at the same time on all channels in the set of channels C.

However, during the draft CR discussion, it can be observed that many details are still requiring further discussion before finalizing this topic. According to the agreements and previous discussion, the following issues are discussed in this section:

- The applicability of different multi-channel access procedures

- Remaining issues on multi-channel access for multiple PSFCH transmissions

- Multiple channel access for S-SSB transmissions

- Multiple channel access for PSCCH/PSSCH transmissions

Issue 1: The applicability of different multi-channel access procedures

This issue comes from the draft CR discussion on TR 37.213. During the discussion, companies provide different views regarding the applicability of NR-U UL channel access procedure in SL-U. Based on previous agreements, our understanding is as follows:

- NR-U UL multi-channel access procedure is considered as baseline for PSCCH/PSSCH transmission

- NR-U DL multi-channel access procedure (including Type A and Type B) is considered as baseline for multiple PSFCH transmission on multiple channels

- No RAN1 consensus on whether multi-channel access is also applied to S-SSB transmission.

That is to say, NR-U UL multi-channel access procedure is inapplicable to PSFCH and S-SSB transmission on multiple channels.

Observation 4: NR-U UL multi-channel access procedure is inapplicable to PSFCH and S-SSB transmission on multiple channels according to the previous RAN1 agreements.

Another issue is how to capture the last agreement from RAN1#112b-e meeting. In NR-U, this kind of UE behavior, i.e., performing transmission on the set of channels at the same time, is only applicable to UL multi-channel access procedures. For DL multi-channel access procedures, it is not required to transmit at the same time, especially for the Type A multi-channel access procedures, where Type 1 channel access should be performed on each channel ci∈C.

Therefore, when a UE performs Type A or Type B multi-channel access procedures for PSFCH transmissions, the only prerequisite is that the UE intends to perform multiple PSFCH transmissions on multiple channels. The agreement made in RAN1 #112b-e meeting is applied to PSCCH/PSSCH transmission on multiple channels taking NR-U UL multi-channel access procedures as baseline.

Proposal 29: When a UE performs Type A or Type B multi-channel access procedures for PSFCH transmissions, the only prerequisite is that the UE intends to perform multiple PSFCH transmissions on multiple channels.

Issue 2: Remaining issues on multi-channel access for multiple PSFCH transmissions

In RAN1#112b-e meeting, it was agreed to support both NR-U DL Type A and Type B multi-channel access procedure for multiple PSFCH transmissions. Several remaining issues are left for further discussion. The first is how the UE select the multi-channel access procedure from Type A and Type B. In NR-U, gNB implementation may achieve this purpose while a similar approach should be reused in SL-U. That is, it is up to UE implementation to perform either Type A or Type B multi-channel access procedure.

Proposal 30: It is up to UE implementation to perform either Type A or Type B multi-channel access procedure.

Another issue also comes from the draft CR discussion, which is related to the CW adjustment for multi-channel access procedures. In NR-U, any PDSCH that fully or partially overlaps with channel  ci, is used to determine CWp for channel ci or for a set of channels C when performing Type A or Type B multi-channel access procedures. This is due to the fact that NR-U CW adjustment procedure refers to the PDSCH transmissions, which is also considered when multi-channel access procedure is performed. However, as discussed in section 2.2, if a UE transmits PSFCH transmission using Type 1 channel access procedures, only the latest CWp used for any PSFCH transmissions on the channel should be adopted for CW adjustment procedure. That is, when a UE intends to perform multiple PSFCH transmissions on multiple channels, only PSFCH that transmitted on channel ci is used to determine CWp in the CW adjustment procedures.

Proposal 31: For determining CWp for channel ci, only PSFCH that transmitted on channel ci is used in the CW adjustment procedures.

Regarding the COT-related issue, as pointed out in the last section, it is not preferred to support PSFCH initiating a shared COT since standalone SCI cannot be transmitted in SL-U. This principle should also be respected during multi-channel access procedure.

Proposal 32: It is not supported initiating a shared COT on any channel ci∈C by PSFCH transmissions.

Issue 3: Multiple channel access for S-SSB transmissions

Multiple channel access procedure for S-SSB transmissions is still pending a solution. According to the following agreement achieved in the last meeting under physical layer design agenda [6], S-SSB transmissions on multiple channels are supported, and thus multiple channel access for S-SSB transmissions should also be considered.

Agreement When the SL-BWP contains multiple RB sets, support the followings: - When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion) - UE may transmit S-SSB repetition in more than one RB set - Down-select one of the followings in RAN1#114: - Alt 1: At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets - FFS details, e.g., whether this can be satisfied by (pre-)configuration, whether the power for S-SSB transmission on other RB set(s) also does not change due to the number of used RB sets, etc. - Alt 2: The power for S-SSB transmission on each RB set does not change due to the number of used RB sets - FFS details, e.g., whether this can be satisfied by (pre-)configuration, etc. - FFS: Locations of S-SSB repetitions in each RB set are the same as the locations of S-SSB repetitions in the anchor RB set - FFS: how to (pre)configure resources for the S-SSB repetitions - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218, R1-2306198

No matter whether S-SSB repetitions are transmitted in R16/17 S-SSB occasions or R18 additional candidate S-SSB occasions, S-SSB transmissions will occur at the same time based on the (pre-)configuration. Therefore, the NR-U UL multi-channel access procedure is more suitable for S-SSB transmissions on multiple channels. For example, Type 1 is performed on the anchor RB set and Type 2 can be performed on the other RB sets which S-SSB repetitions are intended to be performed.

Proposal 33: NR-U UL multi-channel access procedure is considered as the baseline for transmitting S-SSB repetition on multiple channels.

Issue 4: Multiple channel access for PSCCH/PSSCH transmissions

Currently, the multi-channel access procedure for PSCCH/PSSCH transmission is somehow clear, which is based on NR-U UL multi-channel access procedure. However, the COT sharing mechanism may be neglected during the discussion in the previous meeting.

According to the ETSI regulation[10], COT sharing is allowed for transmissions on multiple channels:

The CAE may start transmissions belonging to the corresponding or higher priority classes on one or more operating channels. If the initiating device transmits in more than one operating channel, it shall conform to the requirements defined for multi-channel operation: a) The initiating device and its responding devices can have multiple transmissions without performing an additional CCA on the operating channel or combination of operating channels providing the gap in between such transmissions does not exceed 16 µs. Otherwise, if this gap exceeds 16 µs and does not exceed 25 µs, the initiating device may continue transmissions provided that for a duration of one observation slot the initiating device found the operating channel(s) to be unoccupied channel(s). b) The CAE may grant up to ten authorisations to transmit on the current operating channel to each of one or more responding devices. If the initiating device issues such a transmission grant to a responding device, the responding device shall operate according to the procedure described in clause 4.2.7.3.2.7. c) The initiating device may have simultaneous transmissions of priority classes lower than the priority class of the CAE, provided that the corresponding transmission duration (COT is not extended beyond the time that is needed for the transmission(s) corresponding to the priority class of the CAE.

While in NR-U spec [9], the following content can prove that COT sharing is also allowed after multi-channel access procedures:

##### 4.2.1.0.3 Conditions for indicating Type 2 channel access procedures An eNB/gNB may indicate Type 2 channel access procedures in the DCI of a UL grant or DL assignment scheduling transmission(s) including PUSCH on one or more channels or PUCCH on a channel, respectively, as follows: If the UL transmissions occur within the time interval starting at t0 and ending at t0+TCO, where - TCO=Tmcot,p+Tg, - t0 is the time instant when the eNB/gNB has started transmission on the carrier according to the channel access procedure described in clause 4.1.1, - Tmcot,p value is determined by the eNB/gNB as described in clause 4.1.1, - Tg is the total duration of all gaps of duration greater than 25us that occur between the DL transmissions of the eNB/gNB and UL transmissions scheduled by the eNB/gNB, and between any two UL transmissions scheduled by the eNB/gNB starting from t0, then, - the eNB/gNB may indicate Type 2 channel access procedures in the DCI if the eNB/gNB has transmitted on the channel(s) according to the channel access procedures described in clause 4.1.1 or the multi-channel access procedures in clause 4.1.6, or

Since there are indeed some discussions related to the multi-channel COT sharing in sidelink, such as the frequency domain information of the shared COT and the transmission of S-SSB, it is preferred to have clear definition and common understanding among the group. That is, COT sharing mechanism of multiple channel access for PSCCH/PSSCH transmissions should be supported.

Proposal 34: COT sharing mechanism of multiple channel access for PSCCH/PSSCH transmissions is supported.

The impact on resource allocation

In RAN1 #109-e meeting, the following agreement was achieved regarding resource allocation mechanisms for SL-U [12].

Agreement - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 1 resource allocation selection procedure needs to be updated / enhanced due to shared spectrum channel access - The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access - FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects - channel access, resource allocation and PHY channel design - FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access - RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2

In this section, the following aspects relating to the impact of channel access procedures on resource allocation mechanisms are discussed:

- Multi-consecutive slots transmission (MCSt)

- Mode-2 resource allocation enhancements

Multi-consecutive slots transmission (MCSt)

In RAN1 #110 meeting, it is agreed to support multi-consecutive slots transmission (MCSt) in SL-U for Mode 1 and Mode 2 resource allocation schemes, in order to maintain a COT from being occupied by other RATs [13]. In RAN1 #110b-e meeting, the following agreement was further achieved regarding the detailed design of multi-consecutive slots transmission (MCSt) for SL-U[3].

Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCH sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCH sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

However, after further discussion during RAN1#112b-e meeting, it seems that companies have different understanding about the options in the previous agreement and thus three detailed approaches are provided by the FL for further discussion. An LS was sent to RAN2 to seek RAN2’s view on the feasibility of the following approaches [15]:

Approach 1: “best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) - R16/17 behavior. - Step 2: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure - R16/17 behavior. - Step 3: Higher layer selects a set of resources either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Step 4: Repeat Step 1-3 for different TB if required. Approach 2: “guarantee MCSt for single TB and best effort for multiple TBs” - Step 1: Higher layer triggers L1 resource selection for one TB with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the logical channel/TB or other means. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior). - Step 4: Repeat Step 1-3 for different TB if required. Approach 3: “guarantee MCSt for multiple TBs” - Step 1: Higher layer triggers L1 resource (re-)selection one time for one or multiple TBs with one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) + “number of slots for MCSt” which could be derived based on CAPC of the multiple TBs. - Step 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure (FFS: RSRP calculation / threshold may need to change) - Step 3: Higher layer selects transmission resource for the one or multiple TB(s) from the reported set of candidate multi-slot resource (SA).

According to the agreement and previous discussion, there are still some remaining issues that need to be discussed as follows:

- Resource selection procedures for MCSt

- The impact of LBT failure to MCSt

- HARQ feedback for MCSt

- CAPC level of a MCSt

- Handling of the GP symbol(s) between the slots in MCSt

Issue 1: Resource selection procedures for MCSt

After RAN2#122 meeting, a reply LS was sent back to RAN1 regarding RAN2’s consideration on the above three approaches with the following content [16]:

RAN2 thanks RAN1 for the LS on MCSt resource (re-)selection in R1-2304257, RAN2 discussed the questions in the LS and agreed on the following answers: Question 1 (for Approach 1/ Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2) with the principle of “concatenating” across separate resource selection triggers (across TBs). RAN2’s answer to Question 1: It is feasible to (re-)select the resource for a single TB in MAC layer as per R16/R17 process and concatenate across separate resource (re-)selection triggers across TBs in a best-effort manner for MCSt. Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time. RAN2’s answer to Question 2: Approach 3 (i.e., triggering the resource (re-)selection procedures for multiple SL processes at the same time) is not compatible with the current MAC specification and it may bring big specification impacts to RAN2. Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt. RAN2’s answer to Question 3: It is feasible to provide a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt. RAN2 would like to ask RAN1 to take the above RAN2’s answers into consideration.

According to RAN2’s answers to Q1 and Q2, it can be observed that Approach 3 (i.e., triggering the resource (re-)selection procedures for multiple SL processes at the same time) is infeasible, while Approach 1 and Approach 2 (i.e., (re-)select the resource for a single TB in MAC layer and concatenate across separate resource (re-)selection triggers across TBs in a best-effort manner for MCSt) are feasible from RAN2’s perspective.

Observation 5: According to RAN2’s reply LS, Approach 3 (i.e., triggering the resource (re-)selection procedures for multiple SL processes at the same time) is infeasible, while Approach 1 and Approach 2 (i.e., (re-)select the resource for a single TB in MAC layer and concatenate across separate resource (re-)selection triggers across TBs in a best-effort manner for MCSt) are feasible.

Based on the above observation, down-selection should be considered between Approach 1 and Approach 2. The main difference between Approach 1 and Approach 2 is whether MCSt of single TB is achieved by MAC layer (selecting consecutive-slots from a set of candidate single-slot resource) or PHY layer (selecting from a set of candidate multi-slot resource).

According to the NR SL design, the single-slot candidate resources may be discrete after the resource exclusion procedure in mode 2 resource allocation. If Approach 1 is adopted, MAC layer may not find any consecutive transmission resources can be selected and thus decrease the probability of accessing the channel. While for Approach 2, MCSt can be guaranteed by reporting a set of candidate multi-slot resource to the MAC layer. It is only required to perform channel access procedure once and all the transmissions within a MCSt can be transmitted.

The following comparative analysis of Approach 1 and Approach 2 is provided with the system-level simulation results in the indoor broadcast scenarios. The detailed system-level simulation assumptions are summarized in Annex A.

Figure 2: UPT for Approach 1 and Approach 2 of MCSt

As shown in Figure 2, better UPT performance can be achieved when Approach 2 is adopted, i.e., MCSt is selected from a set of candidate multi-slot resource. Comparing to Approach 1, Approach 2 does not run the risk of not selecting multi-consecutive resources and less LBT failure is suffered by Approach 2. Therefore, in order to guarantee the selection of multi-consecutive recourses for MCSt and increase the UPT performance, it is proposed to report a set of candidate multi-slot resource to the MAC layer.

Proposal 35: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure, in order to guarantee MCSt for single TB.

Issue 2: The impact of LBT failure to MCSt

In RAN1#112b-e meeting, a LS was received from RAN2 regarding the impact of LBT failure to the SL resource (re)selection procedure with the following content [17]:

RAN2 discussed the impact of LBT failure to the SL resource (re)selection procedure. It is RAN2 understanding, that UE triggers resource (re)selection upon receiving an LBT failure indication from PHY for a PSSCH transmission. It is FFS whether such new resource (re)selection trigger is also applicable for the multiple consecutive slots transmission (MCSt) case.

It can be observed that the solution of MCSt case is still pending, and further discussion is required. For MCSt transmissions, if resource (re)selection is not triggered when channel access is failed, the corresponding transmission may be dropped, leading to performance decrease. Therefore, triggering resource reselection may be a better choice and the following schemes can be considered.

- Scheme 1: resource reselection is triggered per slot when channel access is failed and UE can continuing perform channel access procedure after resource selection, as shown in Figure 3.

- Scheme 2: resource reselection is triggered for the whole MCSt when channel access is failed, as shown in Figure 4.

Figure 3: Resource reselection scheme 1 for MCSt

Figure 4: Resource reselection scheme 2 for MCSt

Based on the above-mentioned resource selection approach 2, i.e., MCSt of single TB is achieved by PHY layer (selecting from a set of candidate multi-slot resource), Figure 5 shows the system-level simulation results of Scheme 1 and Scheme 2, considering approach 2 without resource reselection as baseline. The detailed system-level simulation assumptions are also summarized in Annex A.

Figure 5: UPT for Scheme 1 and Scheme 2 of resource reselection method of MCSt

When resource reselection is not performed, only packet dropping will be adopted if channel access is failed, and the corresponding UPT is the lowest. Scheme 1 shows performance gain comparing with Scheme 2, since selecting single-slot resource is much easier than selecting multi-consecutive slots when resource reselection is triggered. If a UE fails to access the channel or haven’t finished the Type 1 channel access procedure, Scheme 1 leaves the space for the UE to perform channel access again or continue the Type 1 channel access procedure, which performs better than dropping the whole MCSt and reselect other resources for the MCSt. Therefore, it is proposed to support triggering resource reselection even for MCSt case when LBT is failed.

Proposal 36: For the multiple consecutive slots transmission (MCSt) case, the UE triggers resource (re)selection upon receiving an LBT failure indication from PHY for a PSSCH transmission at a time.

Issue 3: HARQ feedback for MCSt

In the last meeting, the following FL proposal was raised but F2F discussion was not triggered due to lack of time [7].

Proposal 7-2 (I): When MCSt is used for transmitting a single TB in unicast and groupcast associated with SL HARQ feedback, HARQ feedback indicator is enabled in SCI only for the last PSCCH/PSSCH transmission in the MCSt.

The design of MCSt is to concatenate the transmissions of a TB as much as possible so that transmissions can not be interrupted by other RATs. However, there may still exist some cases that not all transmissions are contained in one MCSt, such as the number of transmissions is larger than the number of slots for MCSt. Therefore, it is worth considering how to support HARQ feedback transmission for MCSt, in order to guarantee the reliability of sidelink transmission for unicast and groupcast.

Considering that there is no HARQ RTT duration within a MCSt, receiving HARQ feedback may be hard to implement. In this case, it may be reasonable to perform HARQ feedback after the whole MCSt transmission is finished, as shown in Figure 6. While the mapping of PSFCH resource should be based on the last PSCCH/PSSCH transmission of the MCSt.

Figure 6: HARQ feedback based on the last PSCCH/PSSCH transmission of the MCSt

Proposal 37: For unicast and groupcast, HARQ feedback is transmitted after the whole MCSt transmission is finished and the mapping of PSFCH resource is based on the last PSCCH/PSSCH transmission of the MCSt.

Issue 4: CAPC level of a MCSt

In the last meeting, the following FL proposal was raised by email and all the feedback indicates supporting the proposal [7].

Proposal 7-1 (I): When UE performs Type 1 channel access for a MCSt carrying multiple TBs, the CAPC value to be used in Type 1 channel access is the largest CAPC value (lowest CAPC level) associated with the multiple TBs.

According to the analysis under issue 1, resource selection should be triggered for a single TB even for MCSt case and multiple TBs may be concatenated across separate resource (re-)selection triggers in a best-effort manner. When there is only one TB transmitting within one MCSt, Type 1 channel access procedures are clear which is derived based on CAPC level of the TB. But when multiple TBs are concatenated by separate resource (re-)selection triggers, which can also be considered as one MCSt, the corresponding CAPC levels may be various. If Type 1 channel access is performed based on the first transmission of the MCSt, it may be unfair for other transmissions when lower CAPC level is included in the MCSt. Therefore, higher layer should ensure that the CAPC level of a MCSt is a certain value, such as the lowest CAPC level among the transmissions within a MCSt.

Proposal 38: Support Proposal 7-1 (I) in RAN1#113 meeting:

- When UE performs Type 1 channel access for a MCSt carrying multiple TBs, the CAPC value to be used in Type 1 channel access is the largest CAPC value (lowest CAPC level) associated with the multiple TBs.

Issue 5: Handling of the GP symbol(s) between the slots in MCSt

This issue was raised in RAN1#112b-e meeting and the following agreement was achieved in RAN1#113 meeting [6]:

Agreement Specification supports that CPE can be transmitted between any two consecutive SL transmissions by the same UE to reduce the gap between the two transmissions so that it does not exceed 16 μs. - Note: for this case, the CPE length should not be longer than up to symbols, as per previous agreements - FFS: details if needed (e.g., considering outcome of discussion on PSFCH-like signal in PHY agenda) - FFS whether PSSCH can be transmitted instead of or in addition to CPE - FFS: how to determine the CPE starting position

According to the agreement, there are still some remaining issues requiring finalization. The first issue is whether PSSCH can be transmitted in the GP symbol. Actually, the motivation of filling the GP symbol is to guarantee the continuing transmission of MCSt. If PSSCH is transmitted in the GP symbol, that is, PSSCH symbol is extended from 12 to 13, critical complexity increase may be caused. Each RX UE performing PSSCH decoding should additionally identify whether the PSSCH symbol is 12-symbol length or 13-symbol length, leading to noticeable impact of original NR SL design. Furthermore, issues such as how to achieve FDM, how to consider the RX/TX switching gap and the physical layer structure related impact, should also be resolved. All of these issues require further consideration and discussion among the group, which is not preferred at this late stage of Rel-18.

Proposal 39: It is not supported to transmit PSSCH in the GP symbols.

Another issue is about how to determine the CPE starting position between any two consecutive SL transmissions. In the last meeting, candidate CPE starting positions were defined for each SCS and the smallest gap is 16us. Therefore, consecutive transmissions can be performed by leaving 16us transmission gap between each two slots, i.e., Type 2C can always be performed within a MCSt. Regarding the inter-UE blocking issue, as mentioned in the CPE section, a (pre-)configured default CPE starting position should be selected for transmissions within a COT. If MCSt is performed after Type 1 channel access, a channel occupancy duration is started when the first transmission of the MCSt is performed. For the rest transmissions within the MCSt, which can be considered as transmissions within a COT, a (pre-)configured default CPE starting position (16 us after the last PSCCH/PSSCH/PSFCH symbol) should be selected. An example is shown in Figure 7. The CPE starting position before the first transmission is selected based on the CPE staring position determination method for the case of initiating a COT. In the later transmissions within the COT, CPE can be transmitted from 16us after the last PSSCH symbol to the next AGC symbol.

Figure 7: CPE transmitted before/within a MCSt

Proposal 40: CPE can be transmitted after 16us from the starting position of the GP symbol.

Mode-2 Resource allocation enhancements

For sidelink mode-2 resource allocation, the existing mechanism is applied for the scenario that UEs are working on dedicated spectrum, that is, sidelink UE can ensure that transmissions can be completed on reserved resources without considering the channel being occupied by other technologies. However, for the unlicensed spectrum, UE needs to perform channel access before transmission and can only occupy the channel continuously within a maximum channel occupancy time (MCOT). If an interruption occurs between transmissions, the channel may be occupied by other technologies and subsequent transmission(s) may be delayed or dropped. Therefore, considering the impact of channel access mechanism on sidelink mode-2 resource allocation mechanism, the following potential enhancements should be considered for resource allocation mechanism:

- The relationship between resource selection and channel access

- The definition of T1 for the resource selection window

- Restrictions on the selected resources

Issue 1: The relationship between resource selection and channel access

This issue has been considered as one of the key issues of SL-U resource allocation procedures, but the solution is still vacant. From our understanding, UE should perform resource selection procedure to determine the corresponding PSCCH/PSSCH transmission resources and then perform LBT procedure. The reasons are provided as follows:

- Firstly, LBT should be carried out at identified resource(s). Otherwise, UE need to perform LBT for all LBT sub-bands, which will cause higher workload for channel access operation especially when multiple LBT sub-bands are configured, such as 100MHz bandwidth.

- Secondly, the sensing processing time (Tproc,0) and Tx processing time (Tproc,1) should also be considered. If UE starts to perform resource selection after the successful LBT procedure, other RAT can access and occupy the channel due to the gap of sensing and Tx processing time. Then, the previous LBT success will be useless.

Based on the above considerations, the starting time of channel access should be after the resource selection time of the PSCCH/PSSCH transmission.

Proposal 41: Considering the complexity and efficiency of SL-U channel access, it is preferred that UE should perform resource selection procedure first and then perform channel access procedure.

Issue 2: The definition of T1 for the resource selection window

This issue was raised in RAN1#112 meeting but failed to achieve an agreement. In the last meeting, related discussion was not triggered due to lack of time. Considering that LBT parameters and resource selection are RAN1 domain, RAN1 should continue these discussions.

According to the design of NR SL, T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL. If resource is selected immediately after the resource selection time, there may not have sufficient time for a UE to perform Type 1 channel access procedures. Therefore, T1  should be redefined as  TminLBT≤ T1, where TminLBT is the minimum duration of Type 1 channel access.

Proposal 42: For a UE performing Type 1 channel access procedures,  T1  is redefined as  TminLBT≤ T1 , where TminLBT is the minimum duration of Type 1 channel access.

Issue 3:  Further restrictions on resource selection

Comparing with legacy NR SL, additional channel access procedures should be performed by a SL-U UE, and thus requiring resource allocation enhancement in order to reduce the impact of channel access. Except for the issues discussed above, further restrictions on resource selection should also be considered from the perspective of both time domain and frequency domain.

Regarding time domain, it is proposed to consider the received COT when performing resource selection. For example, the received COT can be used to identify resource selection window or be considered as the restrictions for resource selection. UE can select resources within the received remaining COT so that only Type 2 channel access whose duration is determinate and much smaller than Type 1 channel access is required to perform.

Regarding frequency domain, prioritize the resource selection within one sub-band is preferred, in order to increase the probability of successful channel access. And if UE cannot find appropriate resources from any single sub-band, joint resource selection among multiple sub-bands can be adopted.

Proposal 43: The following enhancements for resource selection procedure can be further studied:

- Selecting resources with the limitation of COT in time domain

- Selecting resources within one sub-band in frequency domain

Conclusion

In this contribution, remaining issues of channel access mechanism for sidelink on unlicensed spectrum are discussed. Observations and proposals are given as follows:

Proposal 1: Regarding CAPC for SL, mp=1 can be additionally used with p=1 for such as S-SSB and PSFCH transmission.

Proposal 2: The ending time of the first transmission burst that contains PSSCH(s) with ACK/NACK HARQ-ACK enabled is transmitted, should also be considered as another ending time of SL reference duration.

Proposal 3: Groupcast option 1, i.e., NACK-based HARQ feedback, is not supported for SL-U.

Proposal 4: If the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit, CWp is updated for every priority class p∈1,2,3,4 to the next higher allowed value. X is selected by UE from the set of values {8, 9, …,15} for each priority class p∈1,2,3,4.

Proposal 5: If a UE transmits PSFCH transmission using Type 1 channel access procedures associated with the channel access priority class p on a channel, the following is adopted for the CW adjustment:

- Use the latest CWp used for any PSFCH transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p.

- If the corresponding channel access priority class p has not been used for any PSFCH transmission on the channel, CWp=CWmin,p  is used.

Proposal 6: If a UE transmits S-SSB transmission using Type 1 channel access procedures associated with the channel access priority class p on a channel, the following is adopted for the CW adjustment:

- Use the latest CWp used for any S-SSB transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p.

- If the corresponding channel access priority class p has not been used for any PSFCH transmission on the channel, CWp=CWmin,p  is used.

Proposal 7: When Type 2A channel access procedure is applied to S-SSB transmission as discovery burst in NR-U, the total S-SSB transmission duration per UE is at most 1ms and the duty cycle is at most 1/20 with the time interval of duty cycle defined as S-SSB period, i.e., 160ms.

Proposal 8: Within a shared channel occupancy, Type 2 channel access procedures are applicable to PSFCH transmission in SL-U.

Proposal 9: Type 2A channel access procedure is applicable for PSFCH transmissions for a UE without a shared channel occupancy, with the total PSFCH transmission duration per UE is at most 1ms and the duty cycle is at most 1/20 with the time interval of duty cycle defined as S-SSB period, i.e., 160ms.

Observation 1: The definition and limitations are both different between Type 2A channel access and SCSt.

Proposal 10: Type 2A channel access procedures and SCSt should be discussed separately for S-SSB and PSFCH transmission.

Proposal 11: UE may perform S-SSB or PSFCH transmission as SCSt without sensing the channel when the transmission meets the regulation for SCSt in each country.

Proposal 12: For the case of initiating a COT, one or multiple CPE starting candidate positions are (pre-)configured per CAPC level of the PSCCH/PSSCH transmission.

Proposal 13: When UE performs Type 2 channel access to start transmitting within a shared COT:

- Alt. 2: Use only the (pre-)configured default CPE starting position

Proposal 14: CPE starting candidate position(s) sets are (pre-)configured per resource pool as follows:

- A default CPE starting position is (pre-)configured and the candidate value can be {Tsym_1+16μs,  Tsym_1+25μs}.

- A set of one or multiple candidate CPE starting positions are (pre-)configured per CAPC level of the PSCCH/PSSCH transmission based on the pre-defined set of all candidate CPE starting positions.

Proposal 15: The CPE starting position for PSFCH should be (pre-)configured in each RP within the symbol just before the next PSFCH symbol.

Proposal 16: A single CPE starting position is adopted for S-SSB transmission. The CPE starting position should be (pre-)configured within the symbol just before the next AGC symbol.

Proposal 17: Support FL proposal 2-1 in RAN1 #112b-e meeting:

- When the gap between the transmission(s) by a UE following transmission(s) by another UE is 16μs and the duration of the corresponding transmission is at most 584µs in a shared channel occupancy, it is up to UE implementation to perform either Type 2B or Type 2C channel access procedures.

Observation 2: The transmission gap is not restricted to be at most 25us for NR-U UL transmissions in a shared channel occupancy.

Proposal 18: SL-U COT sharing should be based on NR-U DL-UL-UL (Clause 4.2.1.0.3 of TS37.213) COT sharing principle, that is, it is not required to guarantee the transmission gap to be at most 25us.

Proposal 19: Support FL proposal 5-1 in RAN1 #113 meeting:

- UE forwarding/relaying information relating to a COT initiated by another UE is not supported in Rel-18.

Proposal 20: A responding UE is allowed to transmit PSFCH(s) to UE(s) other than the initiator.

Proposal 21: A responding UE can transmit PSSCH/PSCCH to destination ID other than the source/destination ID of the COT initiating transmission.

Observation 3: In NR-U, the COT sharing duration is a relative time, and the absolute duration of the COT depends on how many gaps of duration greater than 25μs are within the COT.

Proposal 22: Remaining COT duration included as part of COT sharing information is indicated in number of slots and the payload size is 6-bit.

Proposal 23: COT sharing offset is not considered as part of COT sharing information. The responding UE(s) determines the available channel occupancy duration only based on the remaining COT duration.

Proposal 24: COT sharing information should also include applicable RB set(s) which can be indicated implicitly by FRIVRBset .

Proposal 25: For the COT sharing information:

- The applicable RB set(s) can be indicated implicitly by FRIVRBset, conveyed in the 1st stage SCI, whose payload size is determined by the number of RB sets in the corresponding resource pool and the maximum number of reserved PSCCH/PSSCH resources that can be indicated by the SCI.

- Existing / legacy R16/17 L1 source and destination IDs are carried in the 2nd stage SCI and their payload sizes are the same as per R16/R17.

- CAPC level and the remaining COT duration are carried in the 2nd stage SCI, and the corresponding payload size are 2-bit and 6-bit, respectively.

Proposal 26: The following conditions should be introduced under which UE can perform COT sharing:

- UE has data to transmit.

- The remaining COT is larger than a (pre-)configured threshold or the channel access priority value is larger than a (pre-)configured value.

Proposal 27: UE-to-UE COT sharing started with S-SSB or PSFCH transmission is not supported.

Proposal 28: UE behavior after receiving multiple COT sharing information should be considered, such as the selection among the COT and the determination of the corresponding CPE starting position.

Observation 4: NR-U UL multi-channel access procedure is inapplicable to PSFCH and S-SSB transmission on multiple channels according to the previous RAN1 agreements.

Proposal 29: When a UE performs Type A or Type B multi-channel access procedures for PSFCH transmissions, the only prerequisite is that the UE intends to perform multiple PSFCH transmissions on multiple channels.

Proposal 30: It is up to UE implementation to perform either Type A or Type B multi-channel access procedure.

Proposal 31: For determining CWp for channel ci, only PSFCH that transmitted on channel ci is used in the CW adjustment procedures.

Proposal 32: It is not supported initiating a shared COT on any channel ci∈C by PSFCH transmissions.

Proposal 33: NR-U UL multi-channel access procedure is considered as the baseline for transmitting S-SSB repetition on multiple channels.

Proposal 34: COT sharing mechanism of multiple channel access for PSCCH/PSSCH transmissions is supported.

Observation 5: According to RAN2’s reply LS, Approach 3 (i.e., triggering the resource (re-)selection procedures for multiple SL processes at the same time) is infeasible, while Approach 1 and Approach 2 (i.e., (re-)select the resource for a single TB in MAC layer and concatenate across separate resource (re-)selection triggers across TBs in a best-effort manner for MCSt) are feasible.

Proposal 35: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure, in order to guarantee MCSt for single TB.

Proposal 36: For the multiple consecutive slots transmission (MCSt) case, the UE triggers resource (re)selection upon receiving an LBT failure indication from PHY for a PSSCH transmission at a time.

Proposal 37: For unicast and groupcast, HARQ feedback is transmitted after the whole MCSt transmission is finished and the mapping of PSFCH resource is based on the last PSCCH/PSSCH transmission of the MCSt.

Proposal 38: Support Proposal 7-1 (I) in RAN1#113 meeting:

- When UE performs Type 1 channel access for a MCSt carrying multiple TBs, the CAPC value to be used in Type 1 channel access is the largest CAPC value (lowest CAPC level) associated with the multiple TBs.

Proposal 39: It is not supported to transmit PSSCH in the GP symbols.

Proposal 40: CPE can be transmitted after 16us from the starting position of the GP symbol.

Proposal 41: Considering the complexity and efficiency of SL-U channel access, it is preferred that UE should perform resource selection procedure first and then perform channel access procedure.

Proposal 42: For a UE performing Type 1 channel access procedures,  T1  is redefined as  TminLBT≤ T1 , where TminLBT is the minimum duration of Type 1 channel access.

Proposal 43: The following enhancements for resource selection procedure can be further studied:

- Selecting resources with the limitation of COT in time domain

- Selecting resources within one sub-band in frequency domain

References

  - RP-230077, “WID revision: NR sidelink evolution”, RAN #99 meeting.

  - R1-2307087, “Discussion on physical channel design framework for sidelink on unlicensed spectrum”, RAN1 #114 meeting.

  - 3GPP TSG RAN WG1 Meeting RAN1 #110bis-e Chairman's Notes, Oct. 2022.

  - 3GPP TSG RAN WG1 Meeting RAN1 #112bis-e Chairman's Notes, Apr. 2023.

  - 3GPP TSG RAN WG1 Meeting RAN1 #112 Chairman's Notes, Mar. 2023.

  - 3GPP TSG RAN WG1 Meeting RAN1 #113 Chairman's Notes, May 2023.

  - R1-2306029, “FL summary for AI 9.4.1.1: SL-U channel access mechanism (EOM)”, RAN1#113 meeting.

  - 3GPP TSG RAN WG1 Meeting RAN1 #111 Chairman's Notes, Nov. 2022.

  - 3GPP TS 37.213, “Physical layer procedures for shared spectrum channel access”, V17.6.0, 2023-06.

  - ETSI EN 301 893, “5 GHz WAS/RLAN; Harmonised Standard for access to radio spectrum”, V2.1.51, 2023-03.

  - MIIT〔2021〕No.129, “2400MHz, 5100MHz and 5800MHz bands technical requirements for interference avoidance of radio transmitting equipment”, 2021-09.

  - 3GPP TSG RAN WG1 Meeting RAN1 #109-e Chairman's Notes, May. 2022.

  - 3GPP TSG RAN WG1 Meeting RAN1 #110 Chairman's Notes, Aug. 2022.

  - R2-2301905, “Report from session on NR SL”, RAN2 #121 meeting.

  - R1-2304257, “LS on MCSt resource (re-)selection”, RAN1.

  - R1-2306389, “Reply LS on MCSt resource (re-)selection”, RAN2.

  - R1-2302278, “LS to RAN1 on SL resource (re)selection”, RAN2.

- Annex A: System Level Evaluation Assumptions for MCSt

| Parameter | Value |
| --- | --- |
| Deployment scenario | Indoor layout Option 1 in RAN1#110 [13] |
| Channel model | NR InH Mixed Office model used in NR-U |
| Simulated Bandwidth | 20 MHz |
| Subcarrier spacing | 30 kHz |
| Traffic model | FTP model 3 with arrival rate of 20% |
| Resource allocation schemes | - Approach 1: L1 report a set of candidate single-slot resource (SA) according to existing L1 resource allocation procedure. Higher layer selects a set of resources according to a consecutive-slots criterion (new behavior) to achieve MCSt. - Approach 2: L1 report a set of candidate multi-slot resource (SA) according to most of the existing L1 resource allocation procedure. Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior). |
| Resource reselection schemes | - Scheme 1: Resource reselection is triggered per slot when channel access is failed and UE can continuing perform channel access procedure after resource selection. - Scheme 2: Resource reselection is triggered for the whole MCSt when channel access is failed. |
| Transmission number per TB | - 3 |
| Number of slots for MCSt | - 3 |
| Performance metric | - UPT |