# 3GPP TSG-RAN WG1 Meeting #114																R1-2308288

Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda item:	9.2.1

Source: 	Moderator (Qualcomm)

Title: 	Summary#3 of General Aspects of AI/ML Framework

Document for:	Discussion/Decision

# GTW session (Wednesday)

###### Discussion 8-21d: (minor edit on latest status after offline session)

Assumption 1: RAN2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - For model inference of the UE-sided model, input data for model inference is available inside the UE. - For UE-side (real-time) monitoring of the UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW.

RAN1 confirms RAN2 assumption 1 with the following additional clarifications.

- For model inference of the UE-sided model, Assistance information from the NW is under RAN1 disussionmay also be needed.

- It is further clarified that input data available inside UE (coming from RS measurements) can still have some RAN1/2 specification effort depending on the enhancements/changes to the RS configuration and measurement framework.

For UE-side (real-time) monitoring of the UE-sided model,

- For some use cases, UE may require certain assistance information, input data, or configuration from NW. This is under discussion in RAN1. Some examples (not exhaustive) are provided below:

  - UE-sided monitoring of two-sided CSI compression may require reconstructed eigenvectors from the NW.

  - RS transmission for ground truth measurement for CSI prediction and beam prediction

  - Assistance information from NW to limit the number of beam measurements for Set A in beam prediction.

  - Labelling assistance from LMF to UE for label-based monitoring methods for AI/ML assisted positioning.

  - NW providing PRU-generated monitoring measurements to UE for UE-side positioning.

  - There are proposals in RAN1 where NW can send data samples to help UE to do the monitoring.

###### Discussion 8-22b: (agreement from offline session)

Assumption 2: For the latency requirement of data collection, RAN2 assumes: - For all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - For model inference, when required data comes from other entities, there is a latency requirement for data collection - For (real-time) model monitoring, when required monitoring data (e.g., performance metric) comes from other entities, there is a latency requirement for data collection.

RAN1 confirms Assumption 2, with the following clarification on the third bullet.

- Monitoring data includes both calculated performance metric and the input data for metric calculation.

###### Discussion 8-23a: (agreement from offline session)

Assumption 3: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement of the non-connected state can be revisited when needed.

RAN1 confirms RAN2’s Assumption 3 for CSI compression, CSI prediction, and beam prediction use cases.

For positioning, it is noted that existing specification supports DL PRS measurement and UE positioning in RRC_INACTIVE state. It is also noted that there are already some existing mechanisms to support measurements outside RRC connected state, e.g., logged MDT and positioning measurement in RRC inactive state. That being said, RAN1 confirms that RRC_CONNECTED can be a starting point and that non-connected state can be revisited when needed.

###### Discussion 8-24b:

Assumption 4: For the data generation entity and termination entity deployed at different entities, RAN2 made the following assumptions: - For CSI enhancement and beam management use cases: - For model training, training data can be generated by UE/gNB and terminated at gNB/OAM/OTT server. - For NW-sided model inference, input data can be generated by UE and terminated at gNB. - For UE-side model inference, input data/assistance information can be generated by gNB and terminated at UE. - For model monitoring at the NW side, performance metrics can be generated by UE and terminated at gNB. - For positioning enhancement use case: - For model training, training data can be generated by UE/gNB and terminated at LMF/OTT server. - For NW-sided model inference, input data can be generated by UE/gNB and terminated at LMF and/or gNB. - For UE-side model inference, input data/assistance information can be generated by LMF/gNB and terminated at the UE. - For model monitoring at the NW side, performance metrics can be generated by UE/gNB and terminated at LMF.

For the data generation entity and termination entity deployed at different entities, RAN1 revised the RAN2's assumptions as follows:

- For CSI compression enhancement and beam management use cases:

- For model training, training data can be generated by UE/gNB and terminated at NWgNB/OAM/OTT server.

- For NW-sided model inference and NW-part of two-sided model inference, input data and assistance information (if needed) can be generated by UE and terminated at gNB.

- For UE-side model inference and UE-part of two-sided model inference, input data/assistance information (if needed) can be generated by gNB and terminated at UE.

- For (real-time) model monitoring at the NW/OTT side, performance metrics data needed for performance metric calculation (if needed) can be generated 	by UE and terminated at gNB/OTT.

- For (real-time) model monitoring at the UE side, data needed for performance metric calculation (if needed) can be generated by gNB and terminated at UE.

- For positioning enhancement use case:

- For model training, training data can be generated by UE/PRU/gNB/LMF and terminated at NWLMF/OTT server.

- For LMFNW-sided model inference (Case 2b, Case 3b), input data and assistance information (if needed) can be generated by UE/gNB and terminated at LMF and/or gNB.

- For gNB-sided model inference (Case 3a), assistance information (if needed) can be generated by LMF and terminated at gNB.

- For UE-side model inference (Case 1, Case 2a), input data/assistance information (if needed) can be generated by LMF/gNB and terminated at the UE.

- For (real-time) model monitoring at the NWLMF/OTT side, performance metrics data needed for performance metric calculation (if needed) can be generated by UE/gNB and terminated at LMF/OTT.

- For (real-time) model monitoring at the UE side of the UE-side model, data needed for performance metric calculation (if needed) can be generated by gNB/LMF and terminated at UE.

- For (real-time) model monitoring at the gNB side of the gNB-side model, data needed for performance metric calculation (if needed) can be generated by LMF and terminated at gNB.

Note: For CSI prediction, the entities for data generation and termination are under RAN1 discussion.

Note: RAN1 notes that, regarding training, different NW entities for training (gNB/CN/LMF/OAM) is out of RAN1’s expertise that RAN1 cannot confirm. RAN1 simply denoted them as NW in the reply.

Note: RAN1 notes that, regarding model monitoring, performance metric is not a part of data collection but should rather be discussed as a procedure for performance monitoring. Instead, data needed for performance metric calculation (if needed) should be captured in the data collection requirement.

To facilitate the discussion on data collection in RAN2 for further progress, RAN2 would like RAN1 to provide feedback/inputs on the following essential aspects: - Data content - Typical data size (value or value range) of the identified data content - Reporting type (e.g., periodic, event triggered, other) of the identified data content - Typical latency requirement (value or value range) to transfer the identified data content RAN2 would require RAN1 feedback/inputs on the data collection requirements per LCM purpose (i.e., model training, inference and monitoring) for each (sub)use case, and the LCM sidedness should also be considered. Besides, RAN2 would also like to know to what extent the data would / should be specified (in detail).

###### Discussion 8-25b:

In answering latency requirements, RAN1 used the following descriptions:

- Relaxed (e.g., minutes, hours, days, or no latency requirement)

- Near-real-time (e.g., several tens 100s of msecs to a few seconds)

- Time-critical (e.g., a few msecs)

###### Discussion 8-26c:

General assumptions/remarks in RAN1’s answer:

- Model training is assumed to be offline training.

- Monitoring may be near-real-time or non-real-time. RAN1 provided reply for near-real-time monitoring only. RAN2 can consider the requirements for data collection for non-real-time monitoring to be similar to offline training requirements.

- In addition to contents listed in the tables, there may be RAN1 has discussed assistance information from NW/UE to UE, such as beam codebook, and beam information, timestamp, data quality, UE speed, SNR, and vendor-specific information that may be useful for the purpose of training, inference, and/or monitoring. Aspects related to such assistance information is not included in the following tables, as there has been no consensus on the necessity/feasibility/contents of such assistance information. If agreed, the assistance information or reference to the assistance information (from UE to the network, or vice versa) may need to be standardized.

- In addition, there may be other assistance/auxiliary information useful for training at the UE-side or third-party such as timestamp, data quality, UE speed, SNR, and vendor-specific information. Such information is not included in the following tables, as RAN1 has not discussed whether their standardization is required. For positioning enhancement, some information has been considered as Working Assumption in RAN1-112bis.

- For performance monitoring of UE-side AI/ML models, there could be different flavors, at least based on which side computes the monitoring KPIs (hence at which side the monitoring KPIs are created), and which side consumes this data. Given the fact that details of such monitoring KPIs and their reporting mechanisms have not been agreed yet, they are not included in the table.

- In the reply for Part B, RAN1 captured the typical data size per each data sample.

###### Discussion 8-28b:

| LCM purpose | Data content | Data transfer direction | Typical data size (per data sample) | Reporting type | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Offline model training | Target CSI (Precoding Matrix or channel matrix) | UE to NW/OTT | Depends on format: ~ 1000 bits (eType II), ~ A few 1000 bits (eType-like with enhance parameters); ~ 100K bits (float32) | No agreement | Relaxed |  |
|  | Compressed CSI (for sequential training) Target CSI (precoding matrix for Type 3 separate training) | NW-first: NW-side training entity to UE-side training entity; UE-first: UE-side training entity to NW-side training entity | ~ 1000 bits | offline | Relaxed |  |
| Inference | CSI Feedback | UE to NW | ~ 1000 bits | periodic, aperiodic | Time-critical | Can use L1 report similar to legacy CSI |
| UE-sided Monitoring | Reconstructed CSI | NW to UE | ~ 1000 bits (eType II format) | No agreement RAN1 to confirm the feasibility and necessity | Near-real-time |  |
| NW/OTT-sided monitoring | Target CSI (Precoding Matrix) | UE to NW/OTT | Depends on format: ~ 1000 bits (eType II) ~ A few 1000 bits (eType-like with enhanced parameters) ~ 100K bits (float32) | No agreement RAN1 to confirm the feasibility and necessity | Near-real-time |  |

###### Discussion 8-29a:

| LCM purpose | Data content | Data transfer direction | Typical data size (per data sample) | Reporting type | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Offline model training | Target CSI in observation and prediction window | UE to NW/OTT | Depends on format and window size (assumed 10 samples below): ~ 10K bits (eType II), ~ A few tens of Kbits (eType-like with enhance parameters); ~ 100K bits (float32) | No agreement | Relaxed |  |
| Inference | Predicted CSI feedback (AI/ML output) | UE to NW | ~ 1000 bits (eType II) | Periodic, aperiodic | Time-critical | Can use L1 report similar to legacy CSI |
| NW/OTT-sided monitoring | Ground truth corresponding to future CSI | UE to NW/OTT | ~ 1000 bits (eType II) | No agreement | Near-real-time |  |

###### Discussion 8-30a:

Table 12: Data collection aspects for AI/ML for beam management

| LCM purpose | UE-side/NW-side models | Data content | Data transfer direction | Typical data size (per sample) | Reporting type | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| offline model training | UE-side | L1-RSRP, Beam-ID for Set B, Set A | UE to NW/OTT | 10s of bits (Typical data size is based on existing L1-RSRP reporting methodology) | No agreement | Relaxed | No consensus on reporting type yet |
|  | NW-side | For Set B: L1-RSRPs [and beam-IDs] For Set A: L1-RSRPs and/or beam-IDs | UE to NW/OTT | 10s of bits | No agreement | Relaxed | Agreed options to be studied as data content. No consensus on reporting type yet |
| Inference | NW-side | L1-RSRP, Beam-ID for Set B | UE to NW | 10s of bits (Typical data size is based on existing L1-RSRP reporting methodology) | TBD (periodic/semi-persistent/aperiodic) | Time-critical |  |
| NW/OTT-sided monitoring | NW-side (model) | L1-RSRP, Beam-ID for Set A | UE to NW/OTT | 10s of bits (Typical data size is based on existing L1-RSRP reporting methodology) | No agreement | Near-real-time |  |

###### Discussion 8-31a:

| LCM purpose | Case | Data content | Data transfer direction | Typical data size (per data sample) | Reporting type | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Offline model training | 1, 2a, 3a, 3b | Measurements: timing, power, and/or phase info | 1,2a: UE/PRU to NW/OTT 3a,3b: gNB to NW/OTT | 26.66k bits (256 measurements/PRS; 4 PRS resources) 426.56k bits (256 measurements/PRS; 64 PRS resources) | No agreement | Relaxed |  |
|  | 2b | Measurements: Intermediate positioning measurement (timing, RSRP/RSRPP, LOS/NLOS indicator, and/or angle, etc.) | UE/PRU to NW/OTT | 752 bits (9 measurements/PRS; 4 PRS resources) 12.032 kbits (9 measurements/PRS; 64 PRS resources) | No agreement | Relaxed |  |
|  | 1, 2b, 3b | Label: Location information | UE/PRU/LMF to NW/OTT 3b: gNB to NW/OTT | 32 to 144 bits [See Note1 below for assumptions] | No agreement | Relaxed |  |
|  | 2a, 3a | Label: Intermediate positioning measurement (timing, RSRP/RSRPP, LOS/NLOS indicator, and/or angle, etc.) | 2a: UE/PRU/LMF to NW/OTT 3a: LMF to NW/OTT | 112 bits (one measurement/PRS; 4 PRS resources) 1792 bits (one measurement/PRS; 64 PRS resources) [See Note1 below for assumptions] | No agreement | Relaxed |  |
| Inference | 1 | Location information See note.2 for additional content | UE to LMF | 32 to 144 bits | Event triggered, semi-periodic, periodic (1 to 64 seconds) see IE Periodic Reporting [TS 37.355] | Response time (e.g., 10s millisecond to hundreds of seconds) [see IE ResponseTime in TS 37.355] |  |
|  | 2a, 3a | Intermediate positioning measurement of PRS (timing, RSRP/RSRPP, LOS/NLOS indicator, angle, etc.) See note.2 for additional content | 2a: UE to LMF 3a: gNB to LMF | 112 bits (one measurement/PRS; 4 PRS resources) 1792 bits (one measurement/PRS; 64 PRS resources) [See Note1 below for assumptions] | Same as case 1 | Same as case 1 |  |
|  | 2b | Intermediate positioning measurement of PRS (timing, RSRP/RSRPP, LOS/NLOS indicator, and angle, etc.) | UE to LMF | 752 bits (9 measurements/PRS; 4 PRS resources) 12.032 kbits (9 measurements/PRS; 64 PRS resources) [See Note1 below for assumptions] | Same as case 1 | Same as case 1 |  |
|  | 3b | Intermediate positioning measurement (timing, RSRP/RSRPP, LOS/NLOS indicator, and/or angle, etc.) | gNB to LMF | 26.66k bits (256 measurements/PRS; 4 PRS resources) 426.56k bits (256 measurements/PRS; 64 PRS resources) | Same as case 1 | Same as case 1 |  |
| Note1: Timing info is 21 bits for first arrival and 14 bits for relative timing; power/real info is 7 bits for first value and 6 bits for relative powers/real values (see TS 37.355) nr-DL-PRS-RSRP-Result-r16 7 bits nr-DL-PRS-RSRP-ResultDiff-r16 6 bits nr-RSTD-r16 16 to 21 bits nr-RelativeTimeDifference-r16 9 to 14 bits |  |  |  |  |  |  |  |

# Working list of terminologies

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

Working Assumption

Table 14: Working list of terminologies

| Terminology | Description |
| --- | --- |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model download | Model transfer from the network to UE |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Model parameter update | Process of updating the model parameters of a model |
| Model selection | The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model upload | Model transfer from UE to the network |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Unsupervised learning | A process of training a model without labelled data. |
| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

# Agreement from RAN#1 109-e

Agreement

- Use 3gpp channel models (TR 38.901) as the baseline for evaluations.

- Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

The description of the terminologies may be further refined as the study progresses.

New terminologies may be added as the study progresses.

It is FFS which subset of terminologies to capture into the TR.

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Conclusion

As indicated in SID, although specific AI/ML algorithms and models may be studied for evaluation purposes, AI/ML algorithms and models are implementation specific and are not expected to be specified.

Observation

Where AI/ML functionality resides depends on specific use cases and sub-use cases.

Conclusion

- RAN1 discussion should focus on network-UE interaction.

  - AI/ML functionality mapping within the network (such as gNB, LMF, or OAM) is up to RAN2/3 discussion.

Agreement

Take the following network-UE collaboration levels as one aspect for defining collaboration levels

1.	Level x: No collaboration

2.	Level y: Signaling-based collaboration without model transfer

3.	Level z: Signaling-based collaboration with model transfer

Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings

FFS: Clarification is needed for Level x-y boundary

# Agreement from RAN#1 110

Agreement

Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management

- Data collection

  - Note: This also includes associated assistance information, if applicable.

- Model training

- [Model registration]

- Model deployment

  - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device.

- [Model configuration]

- Model inference operation

- Model selection, activation, deactivation, switching, and fallback operation

  - Note: some of them to be refined

- Model monitoring

- Model update

  - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training.

- Model transfer

- UE capability

Note: Some aspects in the list may not have specification impact.

Note: Aspects with square brackets are tentative and pending terminology definition.

Note: More aspects may be added as study progresses.

Agreement

The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Performance

  - Intermediate KPIs

  - Link and system level performance

  - Generalization performance

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of data collection

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity of model inference: FLOPs

  - Computational complexity for pre- and post-processing

  - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

- Training complexity

- LCM related complexity and storage overhead

  - FFS: specific aspects

- FFS: Latency, e.g., Inference latency

Note: Other aspects may be added in the future, e.g. training related KPIs

Note: Use-case specific KPIs may be additionally considered for the given use-case.

Working Assumption

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |

Note: It is encouraged for the 3gpp discussion to proceed without waiting for online/offline training terminologies.

Working Assumption

Include the following into a working list of terminologies to be used for RAN1 AI/ML air interface SI discussion.

| Terminology | Description |
| --- | --- |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

Note: Companies are encouraged to bring discussions on various options and their views on how to define Level y/z boundary in the next RAN1 meeting.

# Agreement from RAN#1 110-bis-e

Summary#1 of General Aspects of AI/ML Framework	Moderator (Qualcomm Incorporated)	(rev of R1-2210375)

From Oct 11th GTW session

Working Assumption

- Define Level y-z boundary based on whether model delivery is transparent to 3gpp signalling over the air interface or not.

- Note: Other procedures than model transfer/delivery are decoupled with collaboration level y-z.

- Clarifying note: Level y includes cases without model delivery.

R1-2210472	Summary#2 of General Aspects of AI/ML Framework	Moderator (Qualcomm Incorporated)

From Oct 13th GTW session

Agreement

Clarify Level x/y boundary as:

- Level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement (e.g., LCM related signalling, RS) collaboration between network and UE.
(Note: The AI/ML operation may rely on future specification not related to AI/ML collaboration. The AI/ML approaches can be used as baseline for performance evaluation for future releases.)

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

- FFS: Detailed discussion of model ID with associated information and/or model functionality.

- FFS: usage of model ID with associated information and/or model functionality based LCM procedure

- FFS: whether support of model ID

- FFS: the detailed applicable AI/ML operations

Agreement

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

R1-2210661	Summary#3 of General Aspects of AI/ML Framework	Moderator (Qualcomm)

From Oct 18th GTW session

Conclusion

Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact.

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Agreement

Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects:

- Procedure and assistance signaling for the AI model switching and/or selection

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training).

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

Agreement

Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case:

- Monitoring based on inference accuracy, including metrics related to intermediate KPIs

- Monitoring based on system performance, including metrics related to system peformance KPIs

- Other monitoring solutions, at least following 2 options.

  - Monitoring based on data distribution

    - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc.

    - Output-based: e.g., drift detection of output data

  - Monitoring based on applicable condition

Note: Model monitoring metric calculation may be done at NW or UE

From Oct 19th GTW session

Agreement

Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance

- Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

- Overhead (e.g., signaling overhead associated with model monitoring)

- Complexity (e.g., computation and memory cost for model monitoring)

- Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

- FFS: Power consumption

- Other KPIs are not precluded.

Note: Relevant KPIs may vary across different model monitoring approaches.

FFS: Discussion of KPIs for other LCM procedures

Agreement

Study various approaches for achieving good performance across different scenarios/configurations/sites, including

- Model generalization, i.e., using one model that is generalizable to different scenarios/configurations/sites

- Model switching, i.e., switching among a group of models where each model is for a particular scenario/configuration/site

  - [Models in a group of models may have varying model structures, share a common model structure, or partially share a common sub-structure. Models in a group of models may have different input/output format and/or different pre-/post-processing.]

- Model update, i.e., using one model whose parameters are flexibly updated as the scenario/configuration/site that the device experiences changes over time. Fine-tuning is one example.

Agreement

The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML

- Clarification on inference complexity

  - Note: Inference complexity includes complexity for pre- and post-processing.

- LCM related complexity and storage overhead

  - Storage/computation for training data collection.

  - Storage/computation for training and model update

  - Storage/computation for model monitoring.

  - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation.

- FFS: Power consumption, latency (e.g., Inference latency)

Conclusion

This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP.

# Agreement from RAN#1 111

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Funtionality

- For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

Working Assumption

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion,

| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

# Agreement from RAN#1 112

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

      - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

      - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Agreement

For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

# Agreement from RAN#1 112-bis-e

Agreement

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

  - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

  - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

    - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

    - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

    - FFS: Other aspects that may constitute Functionality

  - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

- For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

  - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

  - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

  - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s).

  - FFS: relationship between functionality-based LCM and model-ID-based LCM

- Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

Conclusion

From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation.

- When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model.

Agreement

- Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities.

- Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

Working Assumption

The definition of ‘AI/ML model transfer’ is revised (marked in red) as follows:

| AI/ML model transfer | Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signaling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| --- | --- |

Working Assumption

| Model selection | The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation |
| --- | --- |

# Agreement from RAN#1 113

Agreement

Consider at least the following aspects and if applicable, the corresponding potential specification impact related to data collection:

- Measurement configuration and reporting

- Contents, type and format of data including:

  - Data related to model input

  - Data related to ground truth

  - Quality of the data

  - Other information

- Signaling of assistance information for categorizing the data

  - Note: The study should consider the feasibility of disclosure of proprietary information

- Signaling for data collection procedure

- Note 1: Use-case specific details can be studied in respective agenda items

- Note 2: Signaling mechanism details can be studied by appropriate working groups.

Agreement

For model identification of UE-side or UE-part of two-sided models, categorize model identification types as follows, and further study relevant aspects, necessity, and specification impact (if any).

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signaling

  - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signaling after model identification.

  - FFS: Spec impact to other WGs

- Type B: Model is identified via over-the-air signaling,

  - Type B1:

    - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification

      - the model may be assigned with a model ID during the model identification

    - FFS: details of steps

  - Type B2:

    - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification

      - the model may be assigned with a model ID during the model identification

    - FFS: details of steps

- Note: The support and applicability of each model identification Type is a separate discussion. This study does not imply that model identification is necessary.

Agreement

For functionality/model-ID based LCM,

- Once functionalities/models are identified, the same or similar procedures may be used for their activation, deactivation, switching, fallback, and monitoring.

Agreement

- Once models are identified, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point.

  - FFS: applicability to model identification, Type A, type B1 and type B2

    - FFS: Using a procedure other than UE capability report

  - Note: model identification using capability report is not precluded for type B1 and type B2

Agreement

Study how to handle the impact of UE’s internal conditions such as memory, battery, and other hardware limitations on functionality/model operations and AI/ML-enabled Feature.

Note: it does not preclude any existing solutions.

Agreement

- Revise the following terminologies for model activation, model deactivation, and model switching as follows

| Model activation | Enable an AI/ML model for a specific function AI/ML-enabled feature |
| --- | --- |
| Model deactivation | Disable an AI/ML model for a specific function AI/ML-enabled feature |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function AI/ML-enabled feature |

Agreement

In model delivery/transfer Case z4, the “known model structure” means an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.

In model delivery/transfer Case z5, the “unknown model structure” means any other model structure not covered in z4, including any model structure that is only partially known.

Agreement

For the purpose of activation/selection/switching of UE-side models/UE-part of two-sided models /functionalities (if applicable), study necessity, feasibility and potential specification impact for methods to assess/monitor the applicability and expected performance of an inactive model/functionality, including the following examples:

- Assessment/Monitoring based on the additional conditions associated with the model/functionality

- Assessment/Monitoring based on input/output data distribution

- Assessment/Monitoring using the inactive model/functionality for monitoring purpose and measuring the inference accuracy

- Assessment/Monitoring based on past knowledge of the performance of the same model/functionality (e.g., based on other UEs)

FFS: Requirements for the assessment/monitoring to be reliable (e.g., sufficient data coverage during evaluation)

FFS: Additional aspects specific to the case where the inactive model has never been activated before, if any.

# Agreement from RAN#1 114

- Agreement

- Conclude that applicable functionalities/models can be reported by UE.

Agreement

- Once models are identified via Type A, UE can indicate supported AI/ML model IDs for a given AI/ML-enabled Feature/FG in a UE capability report as starting point.

  - FFS: Using a procedure other than UE capability report

- Note: The support and applicability of model identification Type A is a separate discussion.

Agreement

- When a model of a known structure at UE (e.g., Case z4) is transferred from NW, the new model being identified (e.g., via Type B2) has the same structure as an previously identified model at the Network and UE

  - Note: the need of model transfer will be discussed separately

Agreement

- Model ID in RAN1 discussion may or may not be globally unique, and different types of model IDs may be created for a single model for various LCM purposes.

- Note: Details can be studied in the WI phase.