3GPP TSG-RAN WG1 Meeting #115                                                                                        R1-2310862

Chicago, USA, November 13 – 17, 2023

Agenda Item:	8.16.11

Source:	Huawei, HiSilicon

Title:	UE features for NR NCR

Document for:	Discussion and Decision

Introduction

In the previous meetings [1][2], most of NCR-Fwd and NCR-MT related features were agreed, as detailed in the Annex. This contribution presents our views on the remaining issues of features for NCR-Fwd.

Discussion

An NCR may optionally support simultaneous UL transmission of backhaul link and C-link. The total transmit power of backhaul link and C-link is subject to NCR capability and regulation as discussed in RAN4. For C-link, the UL transmit power follows normal UE power control procedure, while the backhaul link power control is not within the scope of Rel-18 WI. Instead of specifying a sophisticated power or amplifying gain control for NCR-Fwd, the power allocation between C-link and backhaul link to support simultaneous transmission is proposed in our companion paper [3]. In our understanding, this is different from the “power control” for NCR discussed in SI phase. Essentially, the goal is to determine the UL transmit power for the backhaul link and control link when the total power exceeds the maximum value supported by the NCR or regulation, especially when the NCR-MT and backhaul link share the same set of PAs/TXRUs. Given the different implementation choices, an optional capability can be added for NCR to support this scenario.

Proposal 1: A new FG 43-9 is added to support power allocation between backhaul link and control link for simultaneous UL transmission.

| 43. NR_netcon_repeater | 43-9 | Power allocation between backhaul link and C-link. | Support power allocation between backhaul link and C-link | 43-9 | Yes | N/A | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For semi-persistent and aperiodic beam indication FGs (i.e., FG 43-3 and FG 43-4) can be merged to FG 43-1 for providing better tradeoff between latency and signaling overhead for the forwarding control of different signals between gNB and UE. For forwarding the semi-persistent signals such as SP CSI-RS / SP SRS / SP DL scheduling / configured grant, semi-persistent beam indication is more flexible and of lower latency than periodic beam indication, and it is also with less signaling overhead than aperiodic beam indication. For control the forwarding of dynamic scheduling, aperiodic beam indication is of lower latency and it is also more energy efficient since the indication will only valid one time. If anyone of the FG 43-3 and FG 43-4 is not supported, the UE experience will be deteriorated and efficiency of gNB will be reduced.

Proposal 2: FG 43-3 and FG 43-4 are merged into FG 43-1.

Conclusion

In this contribution we discussed the Rel-18 NCR features and have the following proposals:

Proposal 1: A new FG 43-9 is added to support power allocation between backhaul link and control link for simultaneous UL transmission.

| 43. NR_netcon_repeater | 43-9 | Power allocation between backhaul link and C-link. | Support power allocation between backhaul link and C-link | 43-9 | Yes | N/A | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: FG 43-3 and FG 43-4 are merged into FG 43-1.

- References

- “RAN1 Chair’s Notes,” RAN1#114, Aug 21 – 25, 2023.

- “RAN1 Chair’s Notes,” RAN1#114bis, Aug 21 – 25, 2023.

- R1-2310860, “Maintenance of side control information and NCR behaviour”, Huawei, HiSilicon

- Annex: agreements till RAN1#114bis [2]

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. Support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd 7.Support periodic beam indication for access link 8. Priority flag for periodic indication 9. Support of simultaneous and TDMed DL reception of C-link and backhaul link |  | Yes | N/A | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link 2. Supported slot-offset k values for reference slot | 43-1 | Yes | N/A | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Component 2 candidate values: - 15 kHz: {[0,] 1} - 30 kHz: {[0,]1} - 60 kHz: {[0,]1, 2} - 120 kHz: {[0,]1, 2} Note: The value of slot offset k is selected based on the SCS of the PDCCH received by the NCR-MT Note 2: If k = 0 is reported, the NCR expects that the time resource in NCR-AperiodicFwdConfig of the aperiodic beam indication is at least after the end of time resource for PDCCH carrying the DCI for aperiodic beam indication. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3.Support of [temporary] MAC CE override of the RRC configuration configured of the beam index(es) in at activation of semi-persistent beam indication | 43-1 | N/A Yes | Yes N/A | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A Yes | Yes N/A | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication | 1. Support dedicated signalling for backhaul link beam indication | 43-1, 43-8 | Yes | N/A | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16 (non-unified TCI) only, Rel-17 (unified TCI) only, Rel-15/16 (non-unified TCI) and Rel-17} The reported value is the same as the reported value of FG 43-8 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A Yes | Yes N/A | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16 (non-unified TCI) only, Rel-17 (unified TCI) only, both} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Send LS to RAN2 informing them that

- dynamic DL/UL operation related UE features FG 3-6, FG 5-1a and Component 7 of FG 5-1 are not supported for NCR

- NCR-MT supports the following layer-1 mandatory UE features excluding CA/DC related statements (e.g., component 2 of 3-1 or component 4 of 6-1)

  - 0-1, 0-3, 0-4, 1-1, 2-1, 2-5, 2-6, 2-12, 2-16, 2-16a, 2-32 (only components 1-4 and 7), 2-50 (only components 1,2), 2-52 (only components 1, 2), 3-1 (only components 1,2,3,4,5), 4-1, 4-10, 5-1 (only components 1/2/3/4/5/6/9/10/12), 6-1, 7-1, 8-3