3GPP TSG RAN WG1 #115	R1-2310902

Chicago, IL, US, November 13 – 17, 2023

Agenda item:		8.16.14

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR NTN enhancements

WI code:	NR_NTN_enh-Core

Document for:		Discussion and Decision

# Introduction

UE features for Rel-18 NR NTN were first discussed in RAN1#112bis-e, and discussion was continued in RAN1#113, RAN1#114, and RAN1#114bis.  The rapporteur has compiled the UE feature groups and features for each WI and the outcome of previous discussion for these feature groups including Rel-18 NR NTN is captured in [1]. In this contribution, we provide our inputs to the UE features for Rel-18 NR NTN enhancements in RAN1#115

# Discussion

Inclusion of the note, “: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104,” for the basic Rel-18 NTN feature group has been discussed previously.  As already noted, Rel-18 NTN enhancements have not been studied for terrestrial networks and no agreement has been made on their use in terrestrial networks. For this reason, the note should be included.

Proposal 1:	Support of basic NTN features should be limited to bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 .

## Coverage Enhancement

In RAN 1#113 and RAN1#114 discussion was had on whether a UE can indicate support not to perform TA pre-compensation updated within a TDW if phase continuity limits may be exceeded; however, defining this component would indicate or assume a specific UE implementation and should therefore be removed. The intention of the UE capabilities should be providing what the UE is able to deliver towards the network and not how the UE reach this.

Proposal 2:	Remove component 3, “Support not to perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit” from FG 44-2

Additionally, the pre-requisite FG 30-4a/b for FG 44-2 includes notes that capture a number of use cases and scenarios not intended to be supported for NTN operation. For this reason, the scenarios under which FG 30-4 are a pre-requisite for FG 44-2 should be clarified.

Proposal 3:	Clarify the applicable scenarios and conditions for DM-RS bundling pre-requisite FG 26-1 in NTN case with FG 44-2.

The proposed changes have been reflected in Table 1, and should be captured for further discussion in RAN1#115.

Proposal 4:	Capture the proposed revisions in Table 1 for discussion of UE FGs in Rel-18 NR NTN.

Table 1: Proposed revisions to Rel-18 FGs for NR NTN Enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. NR_NTN_enh | 44-1 | PUCCH repetition on common PUCCH resource | 1. Support repetition transmission of PUCCH for Msg4 HARQ-ACK on common PUCCH resource (i.e., PUCCH resource before dedicated configuration is provided) 2. Support receiving repetition factor in system information 3. Support receiving repetition factor in DCI format 1_0 with CRC scrambled by TC-RNTI scheduling Msg4 PDSCH 4. Support Msg3 to transmit information for PUCCH Msg4 HARQ-ACK repetition 5. Extension of the repetition transmission of PUCCH before dedicated PUCCH resource configuration 6. Support of RSRP threshold for Msg4 HARQ-ACK repetition on common PUCCH resources |  | Yes | No | UE does not support PUCCH repetition for common PUCCH resources | Per Band | N/A | N/A | N/A | A UE that includes [RAN2 parameter name] in [RRC Setup Request] must support FG 44-1 Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 [and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional without capability signaling |
| 44. NR_NTN_enh | 44-2 | NTN DMRS bundling enhancement for PUSCH | 1. Support of DM-RS bundling for PUSCH over consecutive slots 2. Support of pre-compensation to keep phase rotation due to timing drift within the phase difference limit | At least one of {30-4a/b/c}, 26-1 | Yes | No | UE does not support DM-RS bundling enhancement for PUSCH in NTN | Per Band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signaling |
| 44. NR_NTN_enh | 44-3 | UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN | 1. Support UE Rx-Tx time difference and UE Rx-Tx time difference offset measurement and report for Multi-RTT positioning with single satellite in NTN 2. Support of reporting DL timing drift due to Doppler over the service link associated with the UE Rx-Tx time difference measurement period | 13-4, 13-8 | Yes | No | UE does not support Multi-RTT positioning with single satellite in NTN | Per Band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signaling |

# Summary

The following proposals are for discussion on UE features for Rel-18 non-terrestrial network enhancements:

Proposal 1:	Support of basic NTN features should be limited to bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 .

Proposal 2:	Remove component 3, “Support not to perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit” from FG 44-2

Proposal 3:	Clarify the applicable scenarios and conditions for DM-RS bundling pre-requisite FG 26-1 in NTN case with FG 44-2.

Proposal 4:	Capture the proposed revisions in Table 1 for discussion of UE FGs in Rel-18 NR NTN

# References

- R1-2310635	Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis 	Moderators (AT&T, NTT DOCOMO, INC.)