3GPP TSG RAN WG1 Meeting #115	R1-2310962

Chicago, USA, November 13th – 17th, 2023

Source:	ZTE

Title:	Discussion on SCell Type 1 virtual PHR reporting in UL CA

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #114bis, issues were raised and discussed [1]. Although no agreement was reached on the proposal, we found some related issues do exist and need to be addressed in RAN1 #115. In this contribution, we provide our analysis on the related issues.

Issue 1: The is obtained using PO_PRE (preambleReceivedTargetPower) and ΔPREAMBLE_Msg3 (msg3-DeltaPreamble). When deriving a vPHR report for an SCell, are the and the from PCell configuration where the random access procedure took place, or from the SCell configuration (if present)? Issue 2: The is obtained using p0-PUSCH-AlphaSetId = 0. When deriving a vPHR report for an SCell, is the p0-PUSCH-AlphaSet from PCell configuration where the random access procedure took place, or from the SCell configuration (if present)? The Monday comments also noted that there is a default value for ΔPREAMBLE_Msg3, hence the moderator suggests removing ΔPREAMBLE_Msg3 from the list of parameters and otherwise adopt the latest proposal from the proponent as a Release 17 correction as below: If an SCell is not provided with PO_PRE, , or p0-PUSCH-AlphaSet, then the corresponding parameter provided for the PCell is applied in the SCell. If the PCell is configured with two uplink carriers, then the parameters configured for the non-supplementary UL carrier are applied.

- Discussion

We analyzed the issues in three aspects as follows.

- Regarding the above issue 1, although PO_PRE (preambleReceivedTargetPower) is configured as a mandatory value, its parent IE, such as RACH-ConfigGeneric (in RACH-ConfigCommon and/or RACH-ConfigDedicated), does not need to be configured for each SCell, since not every SCell needs RACH configuration. As shown below, RACH-ConfigGeneric includes many mandatory RRC parameters, such as prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, etc. It is not reasonable to configure a whole set of RACH related parameters just for providing a single value for PO_PRE for each SCell. So the scenario for issue 1 does make sense. In this case, using PO_PRE from PCell is a straightforward method for determining the value for PO_PRE in SCell.

  - As for ΔPREAMBLE_Msg3, it is an power offset between msg3 or msgA-PUSCH and RACH preamble transmission, and a default value has been specified as in section 7.1.1 in TS 38.213 as: ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble and deltaPreamble are not provided, for carrier f of serving cell c. So there is no need to determine the value from PCell as for PO_PRE.

- Regarding the above issue 2, we tend to agree that p0-PUSCH-AlphaSet should be configured in the SCell, because it is also used for power control for PUSCH transmission. It is different from issue 1 which is related to RACH parameter configuration.

- Regarding selection between UL and SUL, we consider it also makes sense, and agree with moderator that non SUL, i.e., UL can be prioritized.

Except from TS 38.331 V17.4:

Therefore, the following TP is suggested, noting that the parameter of PO_PRE refers to preambleReceivedTargetPower.

Text Proposal 1: The following text should be adopted in section 7.7.1 of TS 38.213:

- If the parameter of PO_PRE is not provided for a serving cell c, it is determined by a parameter of PO_PRE on carrier of primary cell . If the primary cell  is configured with two uplink carriers, the parameters configured for the non-supplementary UL carrier are applied.

- Conclusion

In this contribution, we provide our views on SCell Type 1 virtual PHR reporting in UL CA. Proposals are listed as follows.

Text Proposal 1: The following text should be adopted in section 7.7.1 of TS 38.213:

- If the parameter of PO_PRE is not provided for a serving cell c, it is determined by a parameter of PO_PRE  on carrier of primary cell . If the primary cell  is configured with two uplink carriers, the parameters configured for the non-supplementary UL carrier are applied.

- References

R1-2310481 [114bis-R15-16-NR] Summary of SCell Type 1 virtual PHR reporting, Nokia