3GPP TSG RAN WG1 #115                                                      R1-2311087

Chicago, USA, November 13th – November 17th, 2023

Source: 	vivo

Title:	Maintenance on SRS enhancements

Agenda Item:	8.1.3.2

Document for:	Discussion and Decision

Introduction

In this contribution, some text proposals are provided for Rel-18 SRS enhancement, including the issues of TDMed SRS with comb offset hopping, and UE capability srs-AntennaSwitching2SP-1Periodic8T8R.

Text proposal

TDMed SRS with comb offset hopping

Conclusion For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when comb offset hopping is configured for the SRS resource, - Option B5: Do not support comb offset hopping for 8-port SRS with TDM.

Support the following TP.

Reason for change:

In the previous meeting, it has been agreed that comb offset hopping is not supported for an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ based on TDM. However, “The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously” is captured in the current spec, which is not very accurate. The key point is that these two parameters can’t be configured for the same SRS resource, instead of the same UE. In other words, if one SRS is configured with comb offset hopping while another SRS resource is configured with [tdm], it should be allowed. Therefore, the restriction should be applied to one SRS resource. Moverover, similar description has been agreed for cyclic shift hopping in the last meeting.

Summary of change:

To add “for an SRS resource” after “The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously”.

Consequences if not approved:

If not approved, the spec would mismatch with the meaning of the previous agreement, leading to unexpected restrictions for Rel-18 UE.

TS 38.214 6.2.1 UE sounding procedure < Unchanged parts are omitted > The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource. - srs-ResourceId or SRS-PosResourceId determines SRS resource configuration identity. - Number of SRS ports, as defined by the higher layer parameter nrofSRS-Ports and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1. - Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in clause 6.4.1.4 of [4, TS 38.211]. - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is slotOffset, can be configured by the higher layer parameter availableSlotOffsetList. The parameter availableSlotOffsetList can be configured up to 4 different values. For an SRS-PosResourceSet configured with higher layer parameter resourceType set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource. - Support of time division mapping subsets of ports of the SRS resource into S symbols (S=2), as defined by the higher layer parameter [tdm], where the SRS ports are evenly distributed in two symbols. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’. - Comb offset hopping pattern with repetition, as defined by the higher layer parameter [combOffsetHoppingWithRepetition], where the parameter can be set to either ‘[per-symbol]’ or ‘[per-R-repetition]’ subject to UE capability. When the parameter is set to ‘[per-symbol]’, the comb offset hopping pattern is determined by the symbol index, and the comb offset hopping pattern is determined by the symbol index of the first symbol of the repetition when the parameter is set to ‘[per-R-repetition]’. - Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource. - SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0. - Frequency hopping bandwidth , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0. - Defining partial frequency sounding factor and start RB index for partial frequency sounding as defined by the higher layer parameters FreqScalingFactor PF and StartRBIndex kF, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then PF = 1 and kF,= 0. - Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khop as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khop is fixed to be 0 for all SRS symbols. - Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero. - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching', or ‘codebook’ subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. For the cyclic shift hopping, a UE can be configured with finer hopping granularity by the higher layer parameter [hoppingFinerGranularity]. The UE is not expecting that [hoppingFinerGranularity] and [cyclicShiftHoppingSubset] are configured simultaneously for an SRS resource. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource. - Transmission comb value, as defined by the higher layer parameter transmissionComb described in clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When comb offset hopping is configured by the higher layer parameter [combOffsetHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching' or ‘codebook’, subject to UE capabilities, transmission comb offset(s) are updated as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the comb offset hopping, a UE can be configured with a subset of comb offsets by the higher layer parameter [combOffsetHoppingSubset], where the comb offset hopping is performed only across the comb offsets configured in the subset. The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource. - SRS sequence ID, as defined by the higher layer parameter sequenceId in clause 6.4.1.4 of [4, TS 38.211]. - SRS cyclic shift and/or comb offset hopping ID, as defined by the higher layer parameter [hoppingID] - The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP, and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When the target SRS is configured by the higher layer parameter SRS-PosResourceSet, the reference RS may also be a DL PRS configured on a serving cell or a non-serving cell indicated by the higher layer parameter dl-PRS, or an SS/PBCH block of a non-serving cell indicated by the higher layer parameter ssb-Ncell. If the UE is configured with dl-OrJointTCI-StateList or ul-TCI-StateList, the reference RS may additionally be an SS/PBCH block associated with a PCI different from the PCI of the serving cell. < Unchanged parts are omitted >

srs-AntennaSwitching2SP-1Periodic8T8R

Support the following TP.

Reason for change:

For 1T2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R/4T8R, the same UE capability srs-AntennaSwitching2SP-1Periodic is indicated by UE. Regarding SRS resource for 8T8R, though 8T8R would be a new UE capability, the UE capability srs-AntennaSwitching2SP-1Periodic can still be reused, since srs-AntennaSwitching2SP-1Periodic is for the maximum SRS resource set to be configured.

Summary of change:

To remove “[srs-AntennaSwitching2SP-1Periodic8T8R]”.

Consequences if not approved:

If not approved, the UE capability seems redundant.

TS 38.214 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > - For 1T=1R, 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, 4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching2SP-1Periodic8T8R], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, 4, or 8 or < Unchanged parts are omitted >

Conclusion

In summary, the following proposals are made for Rel-18 SRS enhancement.

- Support the following TP.

Reason for change:

In the previous meeting, it has been agreed that comb offset hopping is not supported for an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ based on TDM. However, “The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously” is captured in the current spec, which is not very accurate. The key point is that these two parameters can’t be configured for the same SRS resource, instead of the same UE. In other words, if one SRS is configured with comb offset hopping while another SRS resource is configured with [tdm], it should be allowed. Therefore, the restriction should be applied to one SRS resource. Moverover, similar description has been agreed for cyclic shift hopping in the last meeting.

Summary of change:

To add “for an SRS resource” after “The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously”.

Consequences if not approved:

If not approved, the spec would mismatch with the meaning of the previous agreement, leading to unexpected restrictions for Rel-18 UE.

TS 38.214 6.2.1 UE sounding procedure < Unchanged parts are omitted > The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource. - srs-ResourceId or SRS-PosResourceId determines SRS resource configuration identity. - Number of SRS ports, as defined by the higher layer parameter nrofSRS-Ports and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1. - Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in clause 6.4.1.4 of [4, TS 38.211]. - Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is slotOffset, can be configured by the higher layer parameter availableSlotOffsetList. The parameter availableSlotOffsetList can be configured up to 4 different values. For an SRS-PosResourceSet configured with higher layer parameter resourceType set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource. - Support of time division mapping subsets of ports of the SRS resource into S symbols (S=2), as defined by the higher layer parameter [tdm], where the SRS ports are evenly distributed in two symbols. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’. - Comb offset hopping pattern with repetition, as defined by the higher layer parameter [combOffsetHoppingWithRepetition], where the parameter can be set to either ‘[per-symbol]’ or ‘[per-R-repetition]’ subject to UE capability. When the parameter is set to ‘[per-symbol]’, the comb offset hopping pattern is determined by the symbol index, and the comb offset hopping pattern is determined by the symbol index of the first symbol of the repetition when the parameter is set to ‘[per-R-repetition]’. - Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource. - SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0. - Frequency hopping bandwidth , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0. - Defining partial frequency sounding factor and start RB index for partial frequency sounding as defined by the higher layer parameters FreqScalingFactor PF and StartRBIndex kF, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then PF = 1 and kF,= 0. - Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khop as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khop is fixed to be 0 for all SRS symbols. - Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift, respectively, and described in clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero. - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching', or ‘codebook’ subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. For the cyclic shift hopping, a UE can be configured with finer hopping granularity by the higher layer parameter [hoppingFinerGranularity]. The UE is not expecting that [hoppingFinerGranularity] and [cyclicShiftHoppingSubset] are configured simultaneously for an SRS resource. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource. - Transmission comb value, as defined by the higher layer parameter transmissionComb described in clause 6.4.1.4 of [4, TS 38.211]. - Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When comb offset hopping is configured by the higher layer parameter [combOffsetHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching' or ‘codebook’, subject to UE capabilities, transmission comb offset(s) are updated as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the comb offset hopping, a UE can be configured with a subset of comb offsets by the higher layer parameter [combOffsetHoppingSubset], where the comb offset hopping is performed only across the comb offsets configured in the subset. The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource. - SRS sequence ID, as defined by the higher layer parameter sequenceId in clause 6.4.1.4 of [4, TS 38.211]. - SRS cyclic shift and/or comb offset hopping ID, as defined by the higher layer parameter [hoppingID] - The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP, and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When the target SRS is configured by the higher layer parameter SRS-PosResourceSet, the reference RS may also be a DL PRS configured on a serving cell or a non-serving cell indicated by the higher layer parameter dl-PRS, or an SS/PBCH block of a non-serving cell indicated by the higher layer parameter ssb-Ncell. If the UE is configured with dl-OrJointTCI-StateList or ul-TCI-StateList, the reference RS may additionally be an SS/PBCH block associated with a PCI different from the PCI of the serving cell. < Unchanged parts are omitted >

- Support the following TP.

Reason for change:

For 1T2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R/4T8R, the same UE capability srs-AntennaSwitching2SP-1Periodic is indicated by UE. Regarding SRS resource for 8T8R, though 8T8R would be a new UE capability, the UE capability srs-AntennaSwitching2SP-1Periodic can still be reused, since srs-AntennaSwitching2SP-1Periodic is for the maximum SRS resource set to be configured.

Summary of change:

To remove “[srs-AntennaSwitching2SP-1Periodic8T8R]”.

Consequences if not approved:

If not approved, the UE capability seems redundant.

TS 38.214 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > - For 1T=1R, 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, 4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching2SP-1Periodic8T8R], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, 4, or 8 or < Unchanged parts are omitted >