3GPP TSG RAN WG1 #115                                                                                                                                          R1-2311185

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.16.3

Source:	Spreadtrum Communications

Title:	Discussion on UE features for expanded and improved NR positioning

Document for:	Discussion and decision

Introduction

Following are the objectives for WI on Expanded and Improved NR Positioning [1].

The specific objectives of this work item are: - Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify unicast session-based signalling and procedures to facilitate support of SL positioning for single target UE (it is not precluded to apply the procedures to multiple target UEs but no signaling optimizations will be considered for this case) [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and a single LMF for in coverage scenario only, including joint PC5-Uu scenarios. - NOTE: Assumes all involved UEs are served by same LMF. - For SL-TDOA, RAN2 will not work on procedures for synchronization of the anchor UEs. RAN2 can discuss and implement agreed RAN1 parameters related to synchronization. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4]. - Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3]. - Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between (e)DRX in RRC_INACTIVE and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4]. - Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4]. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4]. - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements with measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4]. - Define extensions of signalling, protocol, and procedure for NR positioning enhancement, as needed for the above objectives [RAN3].

In this contribution, we present our views on the UE feature list based on the current progress of Rel-18 positioning.

Discussion

UE features for Sidelink positioning

- Transmitting SL-PRS in a shared resource pool

In RAN1#114b, the follow agreements on multiple Rx-Tx measurements had been achieved [2].

Agreement Support to indicate to UE(s) with higher layer signaling to report multiple Rx-Tx measurements for the same SL PRS transmission (resp. reception) and up to N different SL PRS receptions (resp. transmissions) for the same pair of UE(s). - FFS: value range of N Agreement For the indicated number N of different SL PRS receptions (resp. transmissions) associated with the same SL PRS transmission (resp. reception), the value range of N is {2, 3, 4}.

Therefore, we have the following proposal.

Proposal 1: The FG 41-1-7 should be updated as following.

| 41. NR_pos_enh2 | 41-1-7a | SL PRS measurement for SL-RSTD | 1. Support SL RSTD measurement based on SL-PRS 2. Support SL RSTD measurement reporting 3. reporting N SL RSTD measurements for different SL-PRS reception for the same pair of UEs |
| --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-7b | SL PRS measurement for SL RTOA | 1. Support SL RTOA measurement based on SL-PRS 2. Support SL RTOA measurement reporting 3. reporting N SL RTOA measurements for different SL-PRS reception for the same pair of UEs |
| 41. NR_pos_enh2 | 41-1-7c | SL PRS measurement for UE Rx – Tx time difference without Tx time stamp | 1. Support UE Rx – Tx time difference measurement based on SL PRS 2. Support UE Rx – Tx time difference measurement reporting without Tx time stamp 3. reporting N Rx-Tx measurements for different SL-PRS reception for the same pair of UEs |
| 41. NR_pos_enh2 | 41-1-7d | SL PRS measurement for UE Rx – Tx time difference with Tx time stamp | 1. Support UE Rx – Tx time difference measurement based on SL PRS 2. Support UE Rx – Tx time difference measurement reporting with Tx time stamp 3. Reporting M Rx-Tx measurements for the same SL-PRS transmission (or reception) and different SL-PRS reception (or transmission) for the same pair of UEs 4. reporting N Rx-Tx measurements for different SL-PRS reception for the same pair of UEs |

UE features for RedCap positioning

The following UE FGs of redcap UE positioning had been achieved in previous RAN1 meeting [2].

| 41. NR_pos_enh2 | 41-5-1 | Support of PRS measurement with Rx frequency hopping within a MG [and measurement report] [within a MG] [in RRC_CONNECTED/RRC_INACTIVE/RRC_IDLE] for RedCap UEs | FFS: One component is Frequency domain overlap(s) between hops FFS: separate row for processing capability [1. Maximum DL PRS bandwidth across all hops ] [2. Maximum DL PRS bandwidth per hop 3. Maximum number of hops] [4. Duration of DL PRS symbols N in units of ms a UE can process every T ms ] [5. RF Rx retune times between consecutive hops] [6. Overlapping PRB(s) between adjacent hops ] |
| --- | --- | --- | --- |

In our view, we only need to define maximum DL PRS bandwidth across all hops. Therefore, we have the following proposal.

Proposal 2: The FG 41-5-1 should be updated as following.

| 41. NR_pos_enh2 | 41-5-1 | Support of PRS measurement with Rx frequency hopping within a MG [and measurement report] [within a MG] [in RRC_CONNECTED/RRC_INACTIVE/RRC_IDLE] for RedCap UEs | FFS: One component is Frequency domain overlap(s) between hops FFS: separate row for processing capability 1. Maximum DL PRS bandwidth across all hops 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms 3. RF Rx retune times between consecutive hops |
| --- | --- | --- | --- |

Conclusion

Based on the analyses, we have the following proposals.

Proposal 1: The FG 41-1-7 should be updated as following.

| 41. NR_pos_enh2 | 41-1-7a | SL PRS measurement for SL-RSTD | 1. Support SL RSTD measurement based on SL-PRS 2. Support SL RSTD measurement reporting 3. reporting N SL RSTD measurements for different SL-PRS reception for the same pair of UEs |
| --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-7b | SL PRS measurement for SL RTOA | 1. Support SL RTOA measurement based on SL-PRS 2. Support SL RTOA measurement reporting 3. reporting N SL RTOA measurements for different SL-PRS reception for the same pair of UEs |
| 41. NR_pos_enh2 | 41-1-7c | SL PRS measurement for UE Rx – Tx time difference without Tx time stamp | 1. Support UE Rx – Tx time difference measurement based on SL PRS 2. Support UE Rx – Tx time difference measurement reporting without Tx time stamp 3. reporting N Rx-Tx measurements for different SL-PRS reception for the same pair of UEs |
| 41. NR_pos_enh2 | 41-1-7d | SL PRS measurement for UE Rx – Tx time difference with Tx time stamp | 1. Support UE Rx – Tx time difference measurement based on SL PRS 2. Support UE Rx – Tx time difference measurement reporting with Tx time stamp 3. Reporting M Rx-Tx measurements for the same SL-PRS transmission (or reception) and different SL-PRS reception (or transmission) for the same pair of UEs 4. reporting N Rx-Tx measurements for different SL-PRS reception for the same pair of UEs |

Proposal 2: The FG 41-5-1 should be updated as following.

| 41. NR_pos_enh2 | 41-5-1 | Support of PRS measurement with Rx frequency hopping within a MG [and measurement report] [within a MG] [in RRC_CONNECTED/RRC_INACTIVE/RRC_IDLE] for RedCap UEs | FFS: One component is Frequency domain overlap(s) between hops FFS: separate row for processing capability 1. Maximum DL PRS bandwidth across all hops 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms 3. RF Rx retune times between consecutive hops |
| --- | --- | --- | --- |

Reference

- RP-232670, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation.

- R1-2310382, “Session Notes of AI 8.16.3”, Ad-Hoc Chair (NTT DOCOMO, INC.).