3GPP TSG RAN WG1 #115		R1-2311232

Chicago, USA, November 13th – 17th, 2023

Source:	OPPO

Title:	Discussion on UE features for NR mobility enhancements

Agenda Item:	8.16.7

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID on further NR mobility enhancement [1] includes the following RAN1-centric objectives:

- L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2]

In this contribution, we will present the initial RAN1 UE feature list for NR LTM from our perspective.

- UE Feature List for LTM

We propose to update the FG 45-1 as follows:

- We shall list support of inter-frequency L1-RSRP measurement as a separate item.

- Support the items 2, 3, 5 and 7.

| 45. NR_Mob_enh2 | 45-1 | L1 measurement and reports for L1-L2 Triggered Mobility (LTM) procedure[—processing capability] | 1. Support of intra-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) 1b. Support of inter-frequency L1-RSRP measurement and reporting based on SSBs of candidate cells 2. Maximum number of RRC configured candidate cells for L1-RSRP measurement 3. Maximum number of measured candidate cells for all periodic reports, activated semi-persistent reports, and triggered aperiodic reports at a given time 4. Support of up to L candidate cells and M beams in one report where a SSBRI-RSRP pair is used for each beam report. 5. The max number of SSB resources configured to measure L1-RSRP within a slot with candidate cells across all CC 6 Support of MAC-CE based cell switch operation 7. Support periodic reporting on PUCCH, semi-persistent reporting on PUCCH/PUSCH, and aperiodic report on PUSCH |  | Yes | No | UE does not support Rel-18 LTM operation | [Per band/BC] | No | No | n/a | [Component 2 candidate values: {1,2,3,4,5,6,7}] [Component 3 candidate values: FFS] Component 4 candidate values: L: {[1,] 2,3,4 } M: {1, 2,3,4} [Component 5 candidate values: {1,2,4,8}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In FG 45-2, we propose to delete the word of “always” because the gNB can configure the UE to include the SpCell or not if the UE report supporting this capability. Including the word “always” here would cause some unnecessary ambiguity.

| 45. NR_Mob_enh2 | 45-2 | Inclusion of current SpCell in the L1 measurement report | 1. Support of including the inclusion of current SpCell in the L1 measurement report | 45-1 | Yes | No | UE does not include measurement report for SpCell in the L1 measurement report | [Per band/BC] | No | No | n/a | FFS: whether to merge this FG with FG 45-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Regarding the FG 45-3, we propose to include the maximum number of TCI states per candidate cell and also across all the candidate cells.  Furthermore, in legacy spec, the SSB can not be used as QCL source in TCI states configured for PDCCH or PDSCH. Adding SSB to QCL source shall be a new UE capability. Regarding the MAC-CE TCI state activation, we shall consider the following two different aspects:

- The maximum number of activated TCI states of candidate cells. The UE is supposed to track all the activated TCI states. Therefore, the number of activated TCI states of candidate cell shall be UE capability.

- The maximum number of different candidate cells of all activated TCI states.

Therefore, we suggest to update the FG 45-3 and FG 45-3a as follows:

| 45. NR_Mob_enh2 | 45-3 | LTM beam indication Unified TCI with joint DL/UL TCI states indication for LTM procedure | 1. Support of unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure. 2. Maximum number of configured joint LTM TCI state(s) per candidate cell 2a. Maximum number of configured joint LTM TCI state(s) across all candidate cells 3. Support of indicating and activating a single joint LTM TCI state or a pair of UL/DL TCI-state in a cell switch command. 4. Support LTM TCI-state using SSB as QCL source RS for PDCCH and PDSCH reception [6 Support of MAC-CE based cell switch operation] |  | Yes | No | UE does not support Rel-18 LTM operation | [Per band/BC] | No | No | n/a | Component 2 candidate values: FFS FFS: how to count total number of joint TCI states | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. NR_Mob_enh2 | 45-3a | MAC-CE activated joint LTM TCI states | 1. Maximum number of MAC-CE activated joint LTM TCI states of candidate cells 2. Maximum number of different candidate cells in the MAC-CE activated jointed LTM TCI states. |  | Yes | No | UE does not support MAC-CE activated joint LTM TCI states | [Per band/BC] | No | No | n/a | Component 1 candidate values for K: FFS | Optional with capability signalling |

Regarding the FG 45-4, we shall support the following items:

- The maximum number of configured DL TCI states per cells and the maximum number of configured DL TCI states of all candidate cells.

- The maximum number of configured UL TCI states per cells and the maximum number of configured UL TCI states of all candidate cells.

- Support SSB or TRS as QCL source in DL and UL TCI state shall be UE capability.

Regarding the FG 45-4a, the following items shall be included:

- Maximum number of activated DL TCI states of candidate cell

- Maximum number of activated UL TCI states of candidate cell.

- The maximum number of candidate cells that can be included in TCI state activation MAC CE.

Therefore, we propose to update the FGs 45-4 and 45-4a as follows:

| 45. NR_Mob_enh2 | 45-4 | LTM beam indication Unified TCI with separate DL/UL TCI states indication for LTM procedure | 1. Support of unified TCI with separate DL/UL TCI-state indication for LTM procedure. 2. Maximum number of configured DL TCI state(s) percandidate cell 2a.Maximum number of configured DL TCI state(s) across all candidate cells. 3. Maximum number of configured UL TCI state(s) per candidate cell 3a. Maximum number of configured UL TCI state(s) across all candidate cells. 4. Support of indicating and activating a pair of UL/DL TCI-state in a cell switch command. 5. Support TCI-state using SSB as QCL source RS for PDCCH and PDSCH reception 6. Support TCI-state using TRS as QCL source RS for PDCCH and PDSCH reception |  | Yes | No | UE does not support Rel-18 LTM operation with separate DL/UL TCI states | [Per band/BC] | No | No | n/a | Component 2 candidate values: FFS Component 2 candidate values: FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. NR_Mob_enh2 | 45-4a | MAC-CE activated DL/UL TCI states | 1. Support Maximum number K1 of MAC-CE activated DL TCI states of candidate before cell-switch command 2. Support Maximum number K2 of MAC-CE activated UL TCI states of candidate cells before cell-switch command 3. Maximum number of different candidate cells of activated DL or UL TCI states before cell-switch command |  | Yes | No | UE does not support MAC-CE activated DL/UL TCI states | [Per band/BC] | No | No | n/a | Component 1 candidate values: FFS Component 2 candidate values: FFS | Optional with capability signalling |

For the function of RACH-based early TA acquisition, supporting overlap between UL transmission on serving cell and PRACH transmission to candidate cell shall be one UE capability. Therefore, the item 3 shall be included there. Furthermore, we prefer to include the features of intra-frequency PRACH and inter-frequency PRACH in two separate items because they correspond to different UE capability.

| 45. NR_Mob_enh2 | 45-5 | RACH-based early TA acquisition | 1. TA acquisition of candidate cell(s) based on PDCCH ordered CFRA procedure before receiving cell switch command MAC-CE 2. Power ramping for PRACH retransmission based on PDCCH order indication 3. Handling the overlap between UL transmission on serving cell and PRACH on intra- frequency candidate cell(s) 4. Support overlap between UL transmission on serving cell and PRACH on inter-frequency candidate cell. [4. Support signalling of TA value in cell switch command] |  | Yes | No | FFS | [Per band/BC] | No | No | n/a | [Component 1 candidate values {1, FFS}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Reference

RP-222332 Further NR Mobility Enhancements