3GPP TSG RAN WG1 #115		  R1-2311279

Chicago, USA, November 13th – November 17th, 2023

Source:	OPPO

Title:	   Discussion on UE features for multi-carrier enhancement

Agenda Item:	8.16.12

Document for:	Discussion and Decision

# Introduction

This contribution elaborates our views on UE features for Rel-18 multi-carrier enhancement, focusing on multi-cell scheduling with single DCI.

# Discussion

Updates of feature types in previous agreements in accordance to RAN2 guidance

RAN2 sends some guidance to RAN1 in [2] for UE feature/capability definitions. One of the guidance rules is to “Avoid defining capabilities with pre-requisite on a finer granularity”. Unfortunately the current MCE UE feature structure agreed in the past two RAN1 meetings violates this rule in that:

- FG49-1 can be the prerequisite of FG 49-4a, where the granularity of FG49-1 (per BC) is finer than that of FG49-4a (per UE)

- FG49-2 can be the prerequisite of FG 49-4b, where the granularity of FG49-2 (per BC) is finer than that of FG49-4b (per UE)

- One of {FG49-1, FG49-2} can be prerequisite of FG49-4c and FG49-4d, where the granularity of FG49-1/FG49-2 (per BC) is finer than that of FG49-4c/FG49-4d (per UE).

Proposal 1: Change the type granularity from “per UE” to “per BC” for FG49-4a/4b/4c/4d.

Scheduling cell not scheduled by different cell

Both of the following cross-carrier scheduling cases were not discussed in Rel-18 WI phase:

- Case-1: The scheduling cell not in any of its scheduled cell set is scheduled by another cell.

- Case-2: The scheduling cell that also appears in one of its scheduled cell set is scheduled by another cell.

The following RAN1 agreement made in maintenance phase excludes Case-2.

Agreement For a serving cell included in MC-DCI-SetofCells, a UE does not expect to be configured to monitor PDCCH candidates on more than one scheduling cell for detection of DCI formats scheduling the serving cell.

For the Case-1, first of all, this scheduling cell cannot be a SCell, because a SCell can never be the scheduling cell and scheduled cell (by another cell) at the same time. Further, because the size of scheduled cell set is no smaller than 2, Case-1 goes beyond Rel-17 DSS where PSCell (when scheduled by another SCell) can only schedule itself (instead of at least two scheduled cells). So, Case-1 cannot be considered as a combination of Rel-17 DSS and Rel-18 multi-cell scheduling via single DCI.

Proposal 2: Adopt the note in FG49-1 and FG49-2 as “[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]”.

- The same note is added to FG49-1b and FG49-2b.

FG49-1a/2a

FG49-1a reads “Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set”, while FG49-1 reads “Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set” with same carrier types being enumerated in its component 3. So FG49-1 is super set of FG49-1a, and it could cause RAN2 signaling confusion if UE reports both. Similar issue is applicable to FG49-2a.

Proposal 3: Drop FG49-1a and FG49-2a.

FG49-3

The monitoring of both legacy DCI and DCI 0_3/1_3 on the same scheduling cell is not a necessary function that multi-cell scheduling by single DCI requires. So it should be a separate optional feature. The current FG49-3 proposal left from RAN1 #113/#114 may need some editorial improvement in order to build clear correspondence between the prerequisite and feature components:

- The prerequisite of FG49-3 is “At least one of {49-1, [49-1a,] 49-1b} or at least one of {49-2, [49-2a,] 49-2b}”; and

- The feature component is described as “Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 1_3, or both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3, on the same scheduling cell”.

Proposal 4: Adopt FG49-3 for monitoring both legacy DCI and DCI 0_3/1_3 on the same scheduling cell.

- The prerequisite of FG49-3 is “At least one of {49-1, [49-1a,] 49-1b} or at least one of {49-2, [49-2a,] 49-2b}”; and

- The feature component is described as “Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 1_3, or both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3, on the same scheduling cell”.

# Conclusion

This contribution concludes with following proposals:

Proposal 1: Change the type granularity from “per UE” to “per BC” for FG49-4a/4b/4c/4d.

Proposal 2: Adopt the note in FG49-1 and FG49-2 as “[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]”.

- The same note is added to FG49-1b and FG49-2b.

Proposal 3: Drop FG49-1a and FG49-2a.

Proposal 4: Adopt FG49-3 for monitoring both legacy DCI and DCI 0_3/1_3 on the same scheduling cell.

- The prerequisite of FG49-3 is “At least one of {49-1, [49-1a,] 49-1b} or at least one of {49-2, [49-2a,] 49-2b}”; and

- The feature component is described as “Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 1_3, or both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3, on the same scheduling cell”.

# Reference

- TS 38.822 v17.1.0,  NR User Equipment (UE) feature list (Release 17)

- R1-2306381(R2-2306810), Further Guidelines on UE capability definitions, RAN2

- R1-2310635, Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis, Moderators (AT&T, NTT DOCOMO, INC.)