3GPP TSG RAN WG1 #115			   R1-2311651

Chicago, USA, November 13th – November 17th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for Multi-carrier enhancements

Agenda Item:	8.16.12

Document for: 	Discussion and Decision

- Introduction

At the RAN1#114 meeting, UE features for Rel-18 Multi-carrier enhancements including multi-cell PDSCH/PUSCH scheduling with a single DCI and multi-carrier UL Tx switching schemes were discussed [1], and the updated FGs for MC enhancements as shown in Appendix are captured in the Rel-18 RAN1 NR UE features list [2].

This contribution discusses remaining issues on UE features for Rel-18 MC enhancements.

- Discussion on FGs for multi-cell PDSCH/PUSCH scheduling with a single DCI

2.1	FG49-1/FG49-1b/FG49-2/FG49-2b: Multi-cell PDSCH scheduling by DCI format 1_3/0_3 on a scheduling cell with same/different SCS between scheduling cell and cells in the set

| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type: value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field 10) The number of unicast DL DCI to process [for a set of cells] configured for multi-cell PDSCH scheduling by a DCI format 1_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD/TDD scheduling cell - FFS whether to count DCI format 1_3 only or both legacy DCI formats and DCI format 1_3 [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-1b | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1b 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type:value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields 9) The number of unicast UL DCI to process [for a set of cells] configured for multi-cell PUSCH scheduling by a DCI format 0_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD scheduling cell - Two unicast DCI per slot of scheduling cell [for the set of cells] for TDD scheduling cell - FFS whether to count DCI format 0_3 only or both legacy DCI formats and DCI format 0_3 [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2b | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |

For component 6:

6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}

[Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b]

At the last RAN1 meeting, it was agreed not to support the additional component of max. total number of cells across different set of cells while the reported value of max number of DL/UL CC can be used instead. On the other hand, max. total number of cells across different sets of cells for the same scheduling cell cannot be assumed from other UE capability reporting values, then the under reporting is concerned for the values reported in the component 4 and component 6. More specifically, when a UE support up to 8 cells for the same scheduling cell, the UE can report {4, 4} for {component 4, component 6} and NW has a configuration flexibility between {2, 4} and {4, 2} for {co-scheduled cells, sets of cells for the same scheduling cell} if the total number of cells across sets of cells for the same scheduling cell is reported. Otherwise, the UE should report either {2, 4} or {4, 2} for {component 4, component 6} even if UE supports at most 4 co-scheduled cells and 4 sets of cells for the same scheduling cell. Therefore, it may be beneficial to support max. total number of cells across different sets for the same scheduling cell to avoid the under reporting of component 4 and 6.

However, in our view, if this is the only case which the under reporting is concerned, we don’t see the strong need of this component.

Proposal 1: In FG49-1/1b/2/2b, the following component can be removed;

Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {2, 3, …, 8}

Component 10 in FG49-1 and component 9 in FG49-2:

At the RAN1#114bis meeting, the following agreement was made regarding number of unicast DCI for DCI format 0_3/1_3.

Agreement For 49-1: - [At least for the case if UE does not support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 according to 49-1;] - If the scheduling cell is in the set and is the reference cell; - One unicast SC-DCI (for self scheduling) or one DCI 1_3 for the set of cells that schedules at least the scheduling cell per slot of scheduling cell, or; - One unicast SC-DCI (for self scheduling) and one DCI format 1_3 for the set of cells that schedules other than the scheduling cell per slot of scheduling cell - If the scheduling cell is NOT in the set; - One DCI 1_3 per slot of scheduling cell for the set of cells - FFS when the scheduling cell is in the set but is NOT the reference cell; - FFS one DCI 1_3 per slot of scheduling cell for the set of cells - FFS No SC-DCI is configured for the scheduling cell

Basically, FG3-1 (mandatory without capability signalling) which defines UE capabilities for number of unicast DCI for legacy DCI formats can be followed for multi-cell scheduling at least as default capability of multi-cell scheduling, i.e., component in FG49-1/1b/2/2b. More specifically, the number of unicast DCI should be counted per scheduled CC, i.e., one unicast DCI per scheduled CC for DL FDD/TDD or UL FDD and two unicast DCIs per scheduled CC for UL TDD.

In addition, based on the discussion at the last RAN1 meeting, in our understanding, the concerned case if it is supported by default is the case where monitoring legacy DCI which schedules the cell which is not a reference cell for MC DCI from BD/CCE budget perspective. This concern can be addressed if it is allowed to report legacy DCI monitoring capability from the candidate value of {reference cell only, any cell} which is under the discussion for the FFS in FG49-1/1b/2/2b. To address this concern, we suggest to discuss it together. More specifically, for the case when the scheduling cell in in the set but is NOT the reference cell which is captured in the above agreement, whether SC DCI which schedules can schedule the cell in a set of cells which are not the reference cell depends on the UE capability of legacy DCI and MC DCI monitoring on the same cell.

Based on the above, we propose to update the agreement at the last meeting as follows.

Proposal 2:

- Support simultaneous monitoring capability of DCI format 0_0/1_0/0_1/1_1 and 0_2/1_2 (if supported) and DCI format 0_3/1_3 as a component of FG49-1/1b/2/2b.

- The scheduled cell by DCI format 0_0/1_0/0_1/1_1 and 0_2/1_2 (if supported) is reported from {reference cell only, any cell} in a set

- For 49-1:

[At least for the case if UE does not support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 according to 49-1;]

  - If the scheduling cell is in the set and is the reference cell;

    - One unicast SC-DCI (for self scheduling) or one DCI 1_3 for the set of cells that schedules at least the scheduling cell per slot of scheduling cell, or;

    - One unicast SC-DCI (for self scheduling) and one DCI format 1_3 for the set of cells that schedules other than the scheduling cell per slot of scheduling cell

  - If the scheduling cell is NOT in the set;

    - One DCI 1_3 per slot of scheduling cell for the set of cells

  - FFS when the scheduling cell is in the set but is NOT the reference cell;

    - FFS one DCI 1_3 per slot of scheduling cell for the set of cells if UE report “reference cell only”

    - One SC DCI or one DCI 1_3 per slot of scheduling cell for the set of cells if UE report “any cell”

      - FFS No SC-DCI is configured for the scheduling cell

Based on the proposal 2, it can be generalized for all the three cases in the sub-bullets as “up to one unicast SC-DCI or DCI 1_3 for a given scheduled cell in the set of cells per slot of scheduling cell” in principle. Furthermore, assuming that the first main bullet in the proposal 2, i.e., scheduled cell by legacy DCI is reported from {reference cell only, any cell}, is supported, this principle can be applied regardless of whether UE supports FG6-10 or not.

Proposal 3: Update the component 10 in FG49-1 as follows;

10) The number of unicast DL DCI to process [for a set of cells] configured for multi-cell PDSCH scheduling by a DCI format 1_3

- One unicast DCI per slot of scheduling cell [for a given scheduled cell in the set of cells] for FDD/TDD scheduling cell

The same principle should be applied for FG49-2/1b/2b.

Proposal 4: Update the component 9 in FG49-2 as follows;

9) The number of unicast UL DCI to process [for a set of cells] configured for multi-cell PUSCH scheduling by a DCI format 0_3

- One unicast DCI per slot of scheduling cell [for a given scheduled cell in the set of cells] for FDD scheduling cell

- Two unicast DCI per slot of scheduling cell [for a given scheduled cell in the set of cells] for TDD scheduling cell

Proposal 5: Add the following component in FG49-1b as follows;

10) The number of unicast DL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3

- One unicast DCI per N consecutive slots of scheduling cell for a given scheduled cell in the set of cells for FDD/TDD scheduling cell

  - For low-to-high SCS, N = 1.

  - For high-to-low SCS,

    - N = 2 for (30, 15), (60, 30), (120, 30)

    - N = 4 for (60, 15), (120, 30)

    - N = 8 for (120, 15)

    - N = 16 for (240, 15), [(480, 30)]

    - [N = 32 for (480, 15)]

Proposal 6: Add the following component in FG49-2b as follows;

9) The number of unicast UL DCI to process for a set of cells configured for multi-cell PUSCH scheduling by a DCI format 0_3

- One unicast DCI per N consecutive slots of scheduling cell for a given scheduled cell in the set of cells for FDD scheduling cell

- Two unicast DCI per N consecutive slots of scheduling cell for a given scheduled cell in the set of cells for TDD scheduling cell

  - For low-to-high SCS, N = 1.

  - For high-to-low SCS,

    - N = 2 for (30, 15), (60, 30), (120, 30)

    - N = 4 for (60, 15), (120, 30)

    - N = 8 for (120, 15)

    - N = 16 for (240, 15), [(480, 30)]

    - [N = 32 for (480, 15)]

Furthermore, it was agreed to specify advanced feature FG49-3x/3y while details can be further discussed based on the discussion for basic feature on the number of unicast DCI.

According to the existing UE features for cross-carrier scheduling, advanced features to report increased number of unicast DCI are specified in FG18-5c/5d especially for scheduling from low SCS to high SCS and these features can be simply duplicated for multi-scheduling DCI. Therefore, we propose the following additional advanced feature as FG48-3x/3y.

Proposal 7: Introduce advanced UE capability for larger number of unicast DCI for DCI format 1_3 with scheduling from low SCS to high SCS

- The number of unicast DL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3

  - X unicast DCI per slot of scheduling cell for a given scheduled cell in the set of cells for FDD/TDD scheduling cell

    - Candidate value for X for pair of (scheduling CC SCS, scheduled CC SCS): {1,2,4} for (15,120), (15,60), (30,120), [(60, 480), (120, 960), (120, 480)] and {2} for (15,30), (30,60), (60,120 kHz), [(480, 960)]

Proposal 8: Introduce advanced UE capability for larger number of unicast DCI for DCI format 0_3 with scheduling from low SCS to high SCS

- The number of unicast UL DCI to process for a set of cells configured for multi-cell PUSCH scheduling by a DCI format 0_3

  - X unicast DCI per slot of scheduling cell for a given scheduled cell in the set of cells for FDD/TDD scheduling cell

    - Candidate value for X for pair of (scheduling CC SCS, scheduled CC SCS): {1,2,4} for (15,120), (15,60), (30,120), [(60, 480), (120, 960), (120, 480)] and {2} for (15,30), (30,60), (60,120 kHz), [(480, 960)]

Other FFSs in FG49-1/1b/2/2b

[Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]

This note clarifies that the scheduling cell for multi-cell scheduling cannot be scheduled by another cell. When the scheduling cell is inside of the set, this note is obvious from the previous agreement at the RAN plenary while there is no clear RAN1 agreement on this scheduling cell restrictions. At the RAN1#114 meeting, it was discussed in higher layer parameter discussion to capture this scheduling cell restriction based on the RAN plenary agreement but no consensus was achieved. In addition, it was discussed at the post-RAN1#114 e-mail discussion for CR for TS38.213 but not captured in the spec so far, and corresponding discussion would be expected in maintenance session at the RAN1#114bis meeting. We tend to agree with the arguments that the scheduling cell restriction is related to restriction or expected configuration of higher layer parameter rather than UE feature list. Given the situation, the scheduling cell restriction related to this note should be discussed in multi-carrier enhancement maintenance session and would be captured in RRC parameter list and/or TS38.213.

Proposal 9: The following note in FG49-1/2 should be removed and it should be discussed in multi-cell scheduling maintenance session.

- Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell

FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells

For the FFS whether FG49-1/2 is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, we believe it is not necessary to introduce separate FG for the whole components in FG49-1/2, however, some components can be reported separately.

As discussed above, max number of co-scheduled cells which is reported in component 4 can be reported separately for the case when scheduling cell is inside or outside of the set. While we don’t see the strong need, if under reporting of max number of co-scheduled cells is concerned, whether the same value can be applied for these cases or not can be reported.

In addition, it was argued at the previous RAN1 meetings that actual UE burden on DCI monitoring would be increased for the case when legacy DCI schedules non-reference cell of a set of cells or scheduling cell is not the reference cell while it was agreed that BD/CCE/DCI size for DCI format 0_3/1_3 is counted only on reference cell. Therefore, to address this concern, the simultaneous DCI monitoring capability for legacy and MC DCI can be separately reported between the case when scheduled cell by legacy DCI is not the reference cell of the set of cells or scheduled cell is not the reference cell. This point would be discussed further below in this section.

For the FFS whether FG49-1/2 is separated for the case when the same SCS but different carrier types between scheduling cell and set of cells, the relation of scheduling cell and set of cells in terms of carrier type should be reported in FG49-1. According to the current framework of FG49-1/2 and FG49-1b/2b, it is unclear whether the case where SCS is same but carrier type is different between scheduling cell and set of cells is included in FG49-1/2 or FG49-1b/2b.

Given that SCS is the same between scheduling cell and set of cells, this case can be considered as a part of FG49-1/2. In our view, the SCS is same between scheduling cell and set of cells, and hence the same reporting granularity as component 3b in FG49-1b/2b is not necessary to consider this cross-carrier type case. For example, whether cross-carrier type between scheduling cell and set of cells is supported or not can be reported with 1 bit. For another example, it can be reported whether cross-carrier type between scheduling cell and set of cells supported for FR1 and/or FR2 is reported, e.g., when a UE supports 15 and 120 kHz SCS band, then the UE can report whether TDD/FDD or license/unlicensed can be different between scheduling cell and set of cells for 15 kHz SCS and also can report whether FR2-1/FR2-2 is different between scheduling cell and set of cells for 120 kHz SCS. Otherwise, similar to component 3b in FG49-1b/2b, carrier type for scheduling cell and set of cells can be reported from the possible carrier type combination(s) for a given SCS, e.g., when a UE supports 15 kHz SCS band, indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD}.

Meanwhile, FG49-1b/2b includes the case for different carrier types between scheduling cell and set of cells, and hence this case can be a part of FG49-1b/2b. More specifically, the same SCS case is supported for reported carrier type combination(s) for component 3b in FG49-1b/2b regardless of what the UE reports for component 3a in FG49-1b/2b.

Proposal 10: For the case when the same SCS but different carrier types between scheduling cell and set of cells, discuss the supporting carrier type of scheduling cell and set of cells considering the following options.

- Opt.1: This case is included in FG49-1/2.

- Opt.1-1: No differentiation between the cases when carrier type is same or different between scheduling cell and set of cells.

- Opt.1-2: A UE can report whether cross-carrier type between scheduling cell and set of cells is supported or not with 1 bit.

- Opt.1-3: A UE can report whether cross-carrier type between scheduling cell and set of cells is supported or not for FR1 and FR2 separately.

- Opt.1-4: A UE can report whether cross-carrier type between scheduling cell and set of cells is supported or not for each of the possible carrier type combinations separately.

- Opt.2: This case is included in FG49-1b/2b.

  - A UE supports the case where SCS is the same between scheduling cell and set of cells for reported carrier type combination(s) for component 3b in FG49-1b/2b.

2.2	FG49-1a/FG49-2a: Multi-cell PDSCH scheduling by DCI format 1_3/0_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set

| 49. NR_MC_enh | 49-1a | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) HARQ feedback based on Type 1 HARQ codebook 7) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-2a | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |

As discussed in section 2.1, separate FG is not necessary for whole component in FG49-1/2 but component 4 in FG49-1/2 can be reported separately for the case where scheduling cell is out of the set of cells. The details are in section 2.1 but we made the following proposal for FG49-1a/2a.

Proposal 11: FG49-1a/2a is not necessary.

2.3	FG49-3: Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell

| 49. NR_MC_enh | 49-3 | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | At least one of {49-1, 49-1a, 49-1b, 49-2, 49-2a, 49-2b} | Yes |  | UE does not support monitoring both legacy DCI format(s) (0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | [Per UE] | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The default capability and advanced capability on monitoring legacy DCI and DCI format 0_3/1_3 on the same scheduling cell are discussed in section 2.1.

2.4	FG49-3x/3y: Advanced UE capability for larger number of unicast DL/UL DCI

| 49. NR_MC_enh | 49-3x | Advanced UE capability for larger number of unicast DL DCI | Details FFS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 49-3y | Advanced UE capability for larger number of unicast UL DCI | Details FFS |  |  |  |  |  |  |  |  |  |  |

The default capability and advanced capability on number of unicast DCI for legacy DCI and DCI format 0_3/1_3 are discussed in section 2.1.

2.5	FG49-5a: Trigger Type 3 HARQ CB based feedback using DCI format 1_3

| 49. NR_MC_enh | 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 10-16 (Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For triggering Type 3 HARQ CB based feedback using DCI format 1_3, UE behavior would not be exactly the same as the case for single cell scheduling. Even if a UE supports the feature by legacy DCI, the UE may not support the feature combined with multi-cell scheduling, then the UE cannot indicate the support of this feature even for legacy DCI format. Considering such case, it should be considered to introduce new FG for triggering Type 3 HARQ CB based feedback using DCI format 1_3. In our view, at least one of FG49-1/1b should be supported as a prerequisite feature and the reporting granularity of per BC may be necessary.

Proposal 12: FG49-5a to report a UE capability for triggering Type 3 HARQ CB based feedback using DCI format 1_3 should be introduced.

- Support at least one of FG49-1/1b should be supported as a prerequisite feature

- Reporting type should be per BC

2.6	FG49-5b: Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3

| 49. NR_MC_enh | 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 25-6 (Enhanced Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Similar to FG49-5a above, UE behavior would not be exactly the same as the case for single cell scheduling, and hence it should be considered to introduce new FG for triggering enhanced Type 3 HARQ CB based feedback using DCI format 1_3. In our view, at least one of FG49-1/1b should be supported as a prerequisite feature and the reporting granularity of per BC may be necessary.

Proposal 13: FG49-5b to report a UE capability for triggering enhanced Type 3 HARQ CB based feedback using DCI format 1_3 should be introduced.

- Support at least one of FG49-1/1b should be supported as a prerequisite feature

- Reporting type should be per BC

  - Other FGs for multi-cell scheduling

At the RAN1#113 meeting, the following proposal was discussed regarding other FGs which are supported for existing features.

Proposal 2-20: - Introduce following FGs - FFS whether to introduce UE features for DCI format 1_3/format 0_3 based BWP switching 49. NR_MC_enh; 49-6; Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3; 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. 2. Supports separate PUCCH configuration for different HARQ-ACK codebooks. 3. Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4. Supports a DCI format 1_3 scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_3/1_3 is configured per BWP. 5. Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook and UCI-OnPUSCH for different HARQ-ACK codebooks. 6. Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot Candidate values for the component 6 of FG49-6 is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration 7. Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels for HARQ-ACK; At least one of {49-1, 49-1b}; Yes; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-7; Two HARQ-ACK codebooks with two sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3; 1. Supports two subslot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2. Supports separate PUCCH configuration for different HARQ-ACK codebooks. 3. Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4. Supports a DCI format 1_3 scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_3/1_3 is configured in USS per BWP. 5. Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook and UCI-OnPUSCH for different HARQ-ACK codebooks. 6. Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot. 7. Candidate values for the component 6 of FG49-7 is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration.; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-8; DL priority indication in DCI with mixed DCI formats including DCI format 1_3; Support of priority indicator field configured in DCI formats 1_3 and 1_1/1_2 in a BWP when configured to monitor both DCI formats 1_3 and 1_1/1_2 in the BWP; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-9; UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer for DCI format 0_3; Support intra-UE multiplexing/prioritization of overlapping PUCCH/PUCCH and PUCCH/PUSCH with two priority levels in physical layer (PHY) for DCI format 0_3 1) Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format 2) Multiplexing/prioritization between UL channels/signals with the same PHY priority level 3) Prioritization between UL channels/signals with different PHY priority levels 4) Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. 5) Additional number of symbols (d2) of the preparation time needed for the high priority UL transmission that cancels a low priority UL transmission; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-10; UL priority indication in DCI with mixed DCI formats including DCI format 0_3; Support of priority indicator field configured in DCI formats 0_3 and 0_1/0_2 in a BWP when configured to monitor both DCI formats 0_3 and 0_1/0_2 in the BWP; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-11; PHY priority handling for one-shot HARQ-ACK feedback triggered by DCI format 1_3; Support transmission of type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI format 1_3; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-12; Triggered HARQ-ACK codebook re-transmission triggered by DCI format 1_3; 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_3 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 49-6) 3. Supported minimum value M for the HARQ re-tx offset 4. Supported maximum value N for the HARQ re-tx offset; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-13; SCell dormancy indication in DCI format 0_3; Support for Scell dormancy indication sent within the active time on Pcell with DCI format 0_3; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-14; Scell dormancy indication in DCI format 1_3; Support for Scell dormancy indication sent within the active time on Pcell with DCI format 1_3; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-15; Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3; 1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3 2) minimumSchedulingOffset K2 configuration for PUSCH 3) Support of extended value range for aperiodic CSI-RS triggering offset; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-16; Dynamic indication of applicable minimum scheduling restriction by DCI format 1_3; 1) Dynamic indication of applicable minimum scheduling restriction by DCI format 1_3 2) minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset; At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-17; Unified TCI with joint DL/UL TCI update by DCI format 1_3 for intra- and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC; - TCI state indication for update and activation b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 without DL assignment) - The minimum beam application time in Y symbols per SCS - The maximum number of MAC-CE activated joint TCI states per CC in a band; 23-1-1, At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling. 49. NR_MC_enh; 49-18; Unified TCI with separate DL/UL TCI update by DCI format 1_3 for intra-cell beam management with more than one MAC-CE activated separate TCI state per CC; - TCI state indication for update and activation b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 without DL assignment) - The minimum beam application time in Y symbols per SCS - The maximum number of MAC-CE activated DL TCI states per CC in a band - The maximum number of MAC-CE activated UL TCI states per CC in a band; 23-10-1, At least one of {49-1, 49-1b}; Per BC; N/A; N/A; N/A; Optional with capability signaling.

Regarding the existing FG corresponding to a field included in DCI format 0_3/1_3, the following two alternatives are considered.

  - Alt.1: Reuse Existing FG to indicate the support for DCI format 0_3/1_3

  - Alt.2: Introduce new FG to indicate the support for DCI format 0_3/1_3

Based on the discussion at the previous RAN1 meetings, it would be difficult to support unified solution of Alt.1/2 for all the features above, and hence which of Alt.1/2 is supported should be discussed one-by-one.

In our view, for cross-slot scheduling by DCI format 1_3 and DCI format 0_3, no specific operation to multi-cell scheduling is expected and there is no significant change from legacy operation which is indicated by legacy DCI format even if DCI format 0_3/1_3 is used. In that sense, we can simply reuse the existing UE capability even for the operation by DCI format 0_3/1_3.

On the other hand, for other UE features including DCI format 0_3/1_3 based BWP switching, DL priority indicator in a DCI format 1_3, UL priority indicator in a DCI format 0_3, PHY priority handling for one-shot HARQ-ACK feedback by DCI 1_3, HARQ-ACK re-transmission triggered by DCI format 1_3, SCell dormancy indication within active time by DCI format 1_3 and DCI format 0_3 and unified-TCI indication by DCI format 1_3, UE behavior for multi-cell scheduling would be changed from that for legacy DCI formats similar to FG49-5a/5b. Therefore, it should be considered to introduce new FG for these UE features and the reporting type can be per BC same as FG5a/5b.

Proposal 14:

- Introduce new FGs for the following UE features for DCI format 0_3/1_3;

- DL priority indicator in a DCI format 1_3, UL priority indicator in a DCI format 0_3, PHY priority handling for one-shot HARQ-ACK feedback by DCI 1_3, SCell dormancy indication within active time by DCI format 1_3, unified-TCI indication by DCI format 1_3, Triggered HARQ-ACK codebook re-transmission triggered by DCI format 1_3, DCI format 0_3 and cross-slot scheduling by DCI format 1_3 and DCI format 0_3, BWP switching by DCI format 0_3 and BWP switching by DCI format 1_3.

- Reporting type should be per BC

- Reuse the existing UE capability report for the following UE features for DCI format 0_3/1_3;

- cross-slot scheduling by DCI format 1_3 and DCI format 0_3

In addition to the above FGs, additional FGs can be discussed e.g., for Rel-17/18 CovEnh for DCI 0_3, depending on the discussion progress in maintenance session for multi-cell scheduling.

- Conclusion

In this contribution, we presented our views on UE features for Rel-18 Multi-carrier enhancements for multi-cell PDSCH/PUSCH scheduling with a single DCI. Based on the discussion, following proposals were made.

Proposal 1: In FG49-1/1b/2/2b, the following component can be removed;

Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {2, 3, …, 8}

Proposal 2:

- Support simultaneous monitoring capability of DCI format 0_0/1_0/0_1/1_1 and 0_2/1_2 (if supported) and DCI format 0_3/1_3 as a component of FG49-1/1b/2/2b.

- The scheduled cell by DCI format 0_0/1_0/0_1/1_1 and 0_2/1_2 (if supported) is reported from {reference cell only, any cell} in a set

- For 49-1:

[At least for the case if UE does not support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 according to 49-1;]

  - If the scheduling cell is in the set and is the reference cell;

    - One unicast SC-DCI (for self scheduling) or one DCI 1_3 for the set of cells that schedules at least the scheduling cell per slot of scheduling cell, or;

    - One unicast SC-DCI (for self scheduling) and one DCI format 1_3 for the set of cells that schedules other than the scheduling cell per slot of scheduling cell

  - If the scheduling cell is NOT in the set;

    - One DCI 1_3 per slot of scheduling cell for the set of cells

  - FFS when the scheduling cell is in the set but is NOT the reference cell;

    - FFS one DCI 1_3 per slot of scheduling cell for the set of cells if UE report “reference cell only”

    - One SC DCI or one DCI 1_3 per slot of scheduling cell for the set of cells if UE report “any cell”

      - FFS No SC-DCI is configured for the scheduling cell

Proposal 3: Update the component 10 in FG49-1 as follows;

10) The number of unicast DL DCI to process [for a set of cells] configured for multi-cell PDSCH scheduling by a DCI format 1_3

- One unicast DCI per slot of scheduling cell [for a given scheduled cell in the set of cells] for FDD/TDD scheduling cell

Proposal 4: Update the component 9 in FG49-2 as follows;

9) The number of unicast UL DCI to process [for a set of cells] configured for multi-cell PUSCH scheduling by a DCI format 0_3

- One unicast DCI per slot of scheduling cell [for a given scheduled cell in the set of cells] for FDD scheduling cell

- Two unicast DCI per slot of scheduling cell [for a given scheduled cell in the set of cells] for TDD scheduling cell

Proposal 5: Add the following component in FG49-1b as follows;

10) The number of unicast DL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3

- One unicast DCI per N consecutive slots of scheduling cell for a given scheduled cell in the set of cells for FDD/TDD scheduling cell

  - For low-to-high SCS, N = 1.

  - For high-to-low SCS,

    - N = 2 for (30, 15), (60, 30), (120, 30)

    - N = 4 for (60, 15), (120, 30)

    - N = 8 for (120, 15)

    - N = 16 for (240, 15), [(480, 30)]

    - [N = 32 for (480, 15)]

Proposal 6: Add the following component in FG49-2b as follows;

9) The number of unicast UL DCI to process for a set of cells configured for multi-cell PUSCH scheduling by a DCI format 0_3

- One unicast DCI per N consecutive slots of scheduling cell for a given scheduled cell in the set of cells for FDD scheduling cell

- Two unicast DCI per N consecutive slots of scheduling cell for a given scheduled cell in the set of cells for TDD scheduling cell

  - For low-to-high SCS, N = 1.

  - For high-to-low SCS,

    - N = 2 for (30, 15), (60, 30), (120, 30)

    - N = 4 for (60, 15), (120, 30)

    - N = 8 for (120, 15)

    - N = 16 for (240, 15), [(480, 30)]

    - [N = 32 for (480, 15)]

Proposal 7: Introduce advanced UE capability for larger number of unicast DCI for DCI format 1_3 with scheduling from low SCS to high SCS

- The number of unicast DL DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3

  - X unicast DCI per slot of scheduling cell for a given scheduled cell in the set of cells for FDD/TDD scheduling cell

    - Candidate value for X for pair of (scheduling CC SCS, scheduled CC SCS): {1,2,4} for (15,120), (15,60), (30,120), [(60, 480), (120, 960), (120, 480)] and {2} for (15,30), (30,60), (60,120 kHz), [(480, 960)]

Proposal 8: Introduce advanced UE capability for larger number of unicast DCI for DCI format 0_3 with scheduling from low SCS to high SCS

- The number of unicast UL DCI to process for a set of cells configured for multi-cell PUSCH scheduling by a DCI format 0_3

  - X unicast DCI per slot of scheduling cell for a given scheduled cell in the set of cells for FDD/TDD scheduling cell

    - Candidate value for X for pair of (scheduling CC SCS, scheduled CC SCS): {1,2,4} for (15,120), (15,60), (30,120), [(60, 480), (120, 960), (120, 480)] and {2} for (15,30), (30,60), (60,120 kHz), [(480, 960)]

Proposal 9: The following note in FG49-1/2 should be removed and it should be discussed in multi-cell scheduling maintenance session.

- Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell

Proposal 10: For the case when the same SCS but different carrier types between scheduling cell and set of cells, discuss the supporting carrier type of scheduling cell and set of cells considering the following options.

- Opt.1: This case is included in FG49-1/2.

- Opt.1-1: No differentiation between the cases when carrier type is same or different between scheduling cell and set of cells.

- Opt.1-2: A UE can report whether cross-carrier type between scheduling cell and set of cells is supported or not with 1 bit.

- Opt.1-3: A UE can report whether cross-carrier type between scheduling cell and set of cells is supported or not for FR1 and FR2 separately.

- Opt.1-4: A UE can report whether cross-carrier type between scheduling cell and set of cells is supported or not for each of the possible carrier type combinations separately.

- Opt.2: This case is included in FG49-1b/2b.

  - A UE supports the case where SCS is the same between scheduling cell and set of cells for reported carrier type combination(s) for component 3b in FG49-1b/2b.

Proposal 11: FG49-1a/2a is not necessary.

Proposal 12: FG49-5a to report a UE capability for triggering Type 3 HARQ CB based feedback using DCI format 1_3 should be introduced.

- Support at least one of FG49-1/1b should be supported as a prerequisite feature

- Reporting type should be per BC

Proposal 13: FG49-5b to report a UE capability for triggering enhanced Type 3 HARQ CB based feedback using DCI format 1_3 should be introduced.

- Support at least one of FG49-1/1b should be supported as a prerequisite feature

- Reporting type should be per BC

Proposal 14:

- Introduce new FGs for the following UE features for DCI format 0_3/1_3;

- DL priority indicator in a DCI format 1_3, UL priority indicator in a DCI format 0_3, PHY priority handling for one-shot HARQ-ACK feedback by DCI 1_3, SCell dormancy indication within active time by DCI format 1_3, unified-TCI indication by DCI format 1_3, Triggered HARQ-ACK codebook re-transmission triggered by DCI format 1_3, DCI format 0_3 and cross-slot scheduling by DCI format 1_3 and DCI format 0_3, BWP switching by DCI format 0_3 and BWP switching by DCI format 1_3.

- Reporting type should be per BC

- Reuse the existing UE capability report for the following UE features for DCI format 0_3/1_3;

- cross-slot scheduling by DCI format 1_3 and DCI format 0_3

References

[1]	R1-2310622	Summary on UE features for MC enhancements	Moderator (NTT DOCOMO, INC.)

[2]	R1-2310635	Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis	Moderators (AT&T, NTT DOCOMO, INC.)

Appendix: UE features list for MC enhancements in [2]

# 49.	NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-1 | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type: value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field 10) The number of unicast DL DCI to process [for a set of cells] configured for multi-cell PDSCH scheduling by a DCI format 1_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD/TDD scheduling cell - FFS whether to count DCI format 1_3 only or both legacy DCI formats and DCI format 1_3 [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-1a | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) HARQ feedback based on Type 1 HARQ codebook 7) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-1b | Multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 1_3 for DL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 49-1b 8) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 9) Support Type-2 for ‘Antenna port(s)’ field FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PDSCH scheduling by DCI format 1_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2 | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set 2) Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type:value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields 9) The number of unicast UL DCI to process [for a set of cells] configured for multi-cell PUSCH scheduling by a DCI format 0_3 - One unicast DCI per slot of scheduling cell [for the set of cells] for FDD scheduling cell - Two unicast DCI per slot of scheduling cell [for the set of cells] for TDD scheduling cell - FFS whether to count DCI format 0_3 only or both legacy DCI formats and DCI format 0_3 [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell] FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell with same SCS between scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2a | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with same SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3) Scheduling cell and co-scheduled cells have same SCS/carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2). 4) Max number of co-scheduled cells supported by UE is reported with candidate value set of {[2, 3, 4]} 5) UE can be configured with at least one set of cells. Maximum number of sets for a UE in total and maximum number of sets for a same scheduling cell are reported in FG49-4 6) FDRA field based co-scheduled cell indication | 6-10 (CCS with same SCS) | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with same SCS/carrier type scheduling cell and cells in the set | [Per BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-2b | Multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell not included in a set of cells with different SCS/carrier type between scheduling cell and cells in the set | 1) UE supports monitoring DCI format 0_3 for UL scheduling where scheduling cell is not included in a set of cells in same PUCCH group. 2) Scheduling cell is PCell or SCell, and a set of cells includes only SCells. 3a) Scheduling cell and co-scheduled cells have different SCS. The set of co-scheduled cells share the same SCS and carrier type Candidate value set for component 3a: - {Scheduling cell of lower SCS and scheduled cells of higher SCS, Scheduling cell of higher SCS and scheduled cells of lower SCS, both} 3b) Scheduling cell and co-scheduled cells have same or different carrier type (FR1 licensed FDD or FR1 licensed TDD or FR1 unlicensed TDD or FR2-1 or FR2-2). Candidate value set for component 3b: - Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations 4) Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4} 5) Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8} 6) Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4} [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b] 7) Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both} 8) Support Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields FFS: Number of unicast DCI(s) to process for a set of cells when monitoring DCI format 0_3 or 1_3 is configured FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default |  | Yes |  | UE does not support multi-cell PUSCH scheduling by DCI format 0_3 on a scheduling cell which is not included in a set of cells with different SCS/carrier type scheduling cell and cells in the set | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-3 | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | Monitoring both legacy DCI format(s) (0_0/1_0, 0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | At least one of {49-1, 49-1a, 49-1b, 49-2, 49-2a, 49-2b} | Yes |  | UE does not support monitoring both legacy DCI format(s) (0_1/1_1 and/or 0_2/1_2) and DCI format 0_3/1_3 on the same scheduling cell | [Per UE] | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-3x | Advanced UE capability for larger number of unicast DL DCI | Details FFS |  |  |  |  |  |  |  |  |  |  |
| 49. NR_MC_enh | 49-3y | Advanced UE capability for larger number of unicast UL DCI | Details FFS |  |  |  |  |  |  |  |  |  |  |
| 49. NR_MC_enh | 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | At least one of {49-1, 49-1b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | At least one of {49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4c | Configurable Type-1A fields for DCI format 0_3/1_3 | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 10-16 (Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 25-6 (Enhanced Type 3 HARQ CB), At least one of {49-1, 49-1a, 49-1b} | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-X | Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands | Indicate supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands Candidate value set is {switchedUL, dualUL, both} |  | Yes |  | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per band pair per band combination, details up to RAN2 | N/A | N/A | N/A | This FG is based on the following agreements. RAN1 will not discuss the detail of this FG and the detail is up to RAN2 Agreement Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination Agreement in RAN2#121 For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching. | Optional with capability signaling |
| 49. NR_MC_enh | 49-Y | Minimum separation time for two uplink switching on more than 2 bands within any two consecutive reference slots | If two uplink switchings are triggered and UL transmissions involved in the two uplink switchings are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching, and X us is reported with a candidate value set of {0us, 500us} - The reported value X is applied to both one TAG case and two-TAG case (if UE supports two-TAG case) Note: If the UE reports 0us, the minimum separation time is not applied | 49-X | Yes |  |  | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |