3GPP TSG RAN WG1 #115			R1-2311681

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.2.3

Source:	Apple

Title:	On Remaining Issue of Sidelink CA Operation

Document for:	Discussion/Decision

# Introduction

In RAN #101 meeting [1], [2] , the WI status report states that sidelink CA operation objective is completed.

In this contribution, we provide our views on the remaining issue on sidelink CA operation.

# Discussion

In RAN1 #114 meeting, we have the following agreement on

RAN1 #114 Agreement Rel-16/17 PSFCH power control and PSFCH TX/TX prioritization rule are performed across carriers for all PSFCH transmissions over all the aggregated SL carriers at the same time. - The UE does not expect to be provided with a (pre)configuration that would result in different transmit power per PSFCH on different carriers.

In the current specification of TS 38.213 [3], it is mentioned that PSFCH transmit power across multiple resource pools in one carrier is the same.

TS 38.213 Section 16.2.3 <Unchanged parts omitted> For resource pools configured with PSFCH resources overlapping in time, the UE either expects not to be provided with dl-P0-PSFCH or dl-Alpha-PSFCH in any of the resource pools, or expects to be provided with the same values of dl-P0-PSFCH and the same values of dl-Alpha-PSFCH for all the resource pools. <Unchanged parts omitted>

The above statement does not explicitly cover the case of multiple carriers. To capture RAN1 #114 agreement that UE does not expect to be provided with a (pre)configuration that would result in different transmit power per PSFCH on different carriers, we have the following proposal.

Proposal 1: To capture in TS 38.213 the agreement that UE does not expect to be provided with a (pre)configuration that would result in different transmit power per PSFCH on different carriers.

Following Proposal 1, the corresponding reference modification of the specification is as follows:

TS 38.213 Section 16.2.3 <Unchanged parts omitted> For resource pools configured with PSFCH resources overlapping in time, over multiple carriers if configured, the UE either expects not to be provided with dl-P0-PSFCH or dl-Alpha-PSFCH in any of the resource pools, or expects to be provided with the same values of dl-P0-PSFCH and the same values of dl-Alpha-PSFCH for all the resource pools. <Unchanged parts omitted>

# Conclusion

In this contribution, we provided our views on the remaining open issue on sidelink CA operation. Our proposal is as follows:

Proposal 1: To capture in TS 38.213 the agreement that UE does not expect to be provided with a (pre)configuration that would result in different transmit power per PSFCH on different carriers.

# References

- Chairman’s Notes, 3GPP TSG RAN #101 Meeting, Sep. 2023.

- RP-231583, “Status Report to WI NR sidelink evolution,” Sep. 2023.

- R1-2310739, “Maintenance of NR sidelink evolution	,” Oct. 2023.