# 3GPP TSG RAN WG1 #115																	R1-2312014

# Chicago, USA, November 13th – November 17th, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | DRAFT | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Koffset during random access procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current specification text for NTN IoT (eMTC and NB-IoT) states that, once a differential MAC CE is received, it is applied to all channels without exception. During contention-based random access in connected mode (in case of e.g. BSR transmission without SR resource, or contention-based PDCCH order) the eNB does not know the UE C-RNTI until after msg3 successful reception. Therefore, for (N)PUSCH associated with TC-RNTI (msg3 and its retransmissions), the UE shall use the cell-specific Koffset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that, for PUSCH associated with TC-RNTI, only the cell specific Koffset is applied. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eNB does not know the slots used by the UE for transmission of (N)PUSCH associated with TC-RNTI |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.2.3	Transmission timing adjustments

<Unchanged parts are omitted>

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers except if the KUE_offset is applied to a PUSCH transmission associated with the TC-RNTI, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

#### 5.1.1.1	UE behaviour

<Unchanged parts are omitted>

-	The value of Koffset for a BL/CE UE is

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers except if the KUE_offset is applied to a PUSCH transmission associated with the TC-RNTI, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

### 6.1.1	Timing

Throughout this clause, for a BL/CE UE, if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers except if the KUE_offset is applied to a PUSCH transmission associated with the TC-RNTI, otherwise KUE_offset=0

otherwise,

-	Koffset=0, Kcell_offset=0.

<Unchanged parts are omitted>

### 7.2.3	Channel Quality Indicator (CQI) definition

<Unchanged parts are omitted>

-	where Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers except if the KUE_offset is applied to a PUSCH transmission associated with the TC-RNTI, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0;

<Unchanged parts are omitted>

## 7.3	UE procedure for reporting HARQ-ACK

<Unchanged parts are omitted>

-	for a BL/CE UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers except if the KUE_offset is applied to a PUSCH transmission associated with the TC-RNTI, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

# 8	Physical uplink shared channel related procedures

<Unchanged parts are omitted>

Throughout this clause, for a BL/CE UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers except if the KUE_offset is applied to a PUSCH transmission associated with the TC-RNTI, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

# 10	Physical uplink control channel procedures

<Unchanged parts are omitted>

-	for a BL/CE UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers except if the KUE_offset is applied to a PUSCH transmission associated with the TC-RNTI, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

<Unchanged parts are omitted>

# 16	UE Procedures related to narrowband IoT

Throughout this clause,

-	for a NB-IoT UE, the value of Koffset is given by,

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers except if the KUE_offset is applied to an NPUSCH transmission associated with the TC-RNTI, otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.