3GPP TSG RAN WG1 #115			R1-2312021

Chicago, USA, November 13th – November 17th, 2023

Agenda item:	7.1

Source: 	Qualcomm Incorporated

Title: 	Clarification on CSI dropping with UCI multiplexing on PUSCH

Document for:	Discussion/Decision

# Introduction

In this contribution, we clarify UCI dropping in case UCI is multiplexed on PUSCH.

# UCI dropping in case UCI is multiplexed on PUSCH

In our understanding, the following is the overall framework defined TS 38.213 for UCI multiplexing procedure. A UE first determine a PUCCH resource to transmit UCI (either with or without PUCCH multiplexing on PUCCH). Then, if that determined PUCCH resource overlaps with PUSCHs, the UE multiplexing UCI on PUSCH and not transmit the PUCCH; otherwise, the UE transmit the PUCCH.

It is clear that, if the determined PUCCH resource does not overlap with any PUSCH, when UE transmit the PUCCH, if the number of REs of the PUCCH resource is not enough to transmit all UCI bits, certain UCI dropping rules are defined in TS 38.213 Section 9.2.5.1 and 9.2.5.2.

The question is that, when the determined PUCCH resource overlaps with at least a PUSCH, and the UE would eventually transmit UCI multiplexing on PUSCH, if the number of REs of the determined PUCCH resource is not enough for all UCI bits, does UE drop UCI, although later UCI will be multiplexed on PUSCH? In our understanding, a UE should not drop UCI in this case. It is hoped that this is RAN1’s common understanding.

Proposed conclusion: It is clarified that when UCI is multiplexed on a PUSCH, the UCI dropping rules defined for UCI transmission on PUCCH (in TS 38.213 section 9.2.5.1 and 9.2.5.2) do not apply.

# Conclusions

The following proposal is proposed to clarify UCI dropping in case UCI is multiplexed on PUSCH.

Proposed conclusion: It is clarified that when UCI is multiplexed on a PUSCH, the UCI dropping rules defined for UCI transmission on PUCCH (in TS 38.213 section 9.2.5.1 and 9.2.5.2) do not apply.