3GPP TSG RAN WG1 #115																		R1-2312063

Chicago, USA, November 13th – November 17th, 2023

Agenda item:	8.16.3

Source: 	Qualcomm Incorporated

Title: 	UE features on Expanded and improved NR Positioning

Document for:		Discussion and Decision

# Introduction

This contribution includes our suggestions for the UE features and components for the WI Expanded and improved NR Positioning.

# 2 SL Positioning

FG 41-1-1 has the largest impact on UE implementation complexity and a UE’s ability to process a given number of SL PRSs is dependent on how many are received and the bandwidth of each, which could vary from one band to another and is cumulative across bands. Further, since SL multi-carrier support is agreed to be introduced in Rel-18 and this processing scales with the number of carriers, we propose to make this FG reported per FSPC.

Proposal 1: FG 41-1-1 should be reported per FSPC

We propose to capture a subset of the sidelink positioning features as LTE features as was done for sidelink communications in previous releases.

Proposal 2: Capture the following FGs in the LTE UE feature list: 41-1-1, 41-1-2, 41-1-3, 41-1-4c, 41-1-4d, 41-1-5, 41-1-7x, 41-1-8, 41-1-10, 41-1-11, 41-1-12, 41-1-12, 41-1-13, 41-1-13b, 41-1-14, 41-1-18, 41-1-19, 41-1-20a, 41-1-20b.

# 3 Carrier Phase Positioning

##### FG 41-2-3 (Support of measurements within an indicated time window)

The feature of supporting measurements within an indicated time window should be applicable to both UE-based and UE-assisted positioning since the feature of UE-based carrier phase positioning with measurements forwarded from a PRU is supported. That is, an LMF will configure the PRU to perform measurements (UE-assisted) on the DL PRS resources within the indicated time window, while it will configure the UE to perform UE-based Positioning on the same DL PRS resources within the same indicated time window.

Proposal 3: The feature of supporting measurements within an indicated time window should be supported for both UE-based and UE-assisted Positioning.

##### New FGs for CPP

###### Support of a timestamp that includes a symbol index

According to the following agreement, a separate UE capability should be introduced for the support of a timestamp that includes a symbol index.

Agreement For the timestamp associated with a reported RSCP/RSCPD measurement, NR-TimeStamp, with the granularity of a slot, currently defined in TS 37.355, can be reused as the timestamp. - Subject to UE capability, a UE may optionally provide an OFDM symbol index in the timestamp. - Note: It is up to RAN2/RAN3 how to signal the timestamp

Proposal 4: Introduce a new FG for the support of a timestamp that includes a symbol index. It can be reported per UE.

###### Support of associating a single Rx-Tx or RSTD measurement with up to N_sample RSCP/RSCPD measurements

According to the following agreement, a separate UE capability should be introduced for the support of a associating a single Rx-Tx or RSTD measurement with up to N_sample RSCP/RSCPD measurement.

Agreement Subject to UE’s capability, if a UE Rx-Tx time difference/DL RSTD measurement is obtained with Nsample (=2, 4) samples, as defined in TS 38.133, the UE Rx-Tx time difference/DL RSTD measurement can be associated with (i.e., reported together with) up to Nsample RSCP/RSCPD measurements. - A single RSCP/RSCPD measurement is obtained within one sample - Each RSCP/RSCPD measurement has its own timestamp. - Note: It is up to RAN2 on how to define signalling support for the reporting of the timestamps of the RSCP/RSCPD measurements.

Proposal 5: Introduce a new FG for the Support of associating a single Rx-Tx or RSTD measurement with up to N_sample RSCP/RSCPD measurement. It should be reported per band.

###### Support of reporting time domain measurements with granularity using ReportingGranularityfactor k={-1, -2}

According to the following agreement, a new granularity for timing measurement reporting has been introduced. Such a feature requires a UE capability, otherwise the LMF would not know whether the UE supports it or not.

Agreement From RAN1’s perspective, the granularity with ReportingGranularityfactor k={-1, -2} for the reporting of DL/UL timing measurements is applicable to all positioning methods.

Proposal 6: Introduce a new FG for the Support of reporting time domain measurements with granularity using ReportingGranularityfactor k={-1, -2}. It can be reported per UE.

# 4 PRS / SRS BW Aggregation

##### FG 41-4-6 & 41-4-7 (SRS BW Aggregation in RRC_CONNECTED)

The basic feature for Positioning SRS bandwidth aggregation in RRC_CONNECTED, i.e. FG41-4-6 & 41-4-7, should be “per FS” following the same logic to the basic SRS for Positioning FG (13-8) which should also be a prerequisite. Furthermore, the 3 main components of this FG are still in brackets. We prefer to keep these components in the way they are currently written; we don’t see a strong motivation to update it.

Proposal 7: Regarding the FG 41-4-6 & FG 41-4-7 (Positioning SRS bandwidth aggregation in RRC_CONNECTED),

- They should be reported per FS.

- Support the components 1,2,3 that are shown in brackets with the values as written in the existing UE features list.

- Support keeping the “periodic/semi-persistent/aperiodic”

- Introduce a separate component the maximum number of periodic SRS resources:

  - Maximum number of P aggregated SRS resources for bandwidth aggregation

  - Maximum number of P aggregated SRS resources for bandwidth aggregation per slot

##### FG 41-4-8 (SRS BW aggregation in RR_INACTIVE)

In Re-17’s FG 27-15 (SRS transmission in RRC Inactive), we have the following 4 components:

2. Max number of P/SPSRS Resources for positioning 3. Max number of P/SPSRS Resources for positioning per slot 4. Max number of periodic SRS Resources for positioning 5. Max number of periodic SRS Resources for positioning per slot

The first 2 components set a bound on the overall number of SRS resources (periodic and semi-persistent), and the remaining 2 components set a bound on the maximum number of periodic SRS resources. A similar distinction is needed also for the SRS aggregation:

Proposal 8: For FG 41-4-8 (SRS BW aggregation in RRC Inactve) have a total 4 components for each FG related to the maximum number of periodic & semi-persistent SRS resources:

- Maximum number of P/SP aggregated SRS resources for bandwidth aggregation

- Maximum number of P/SP aggregated SRS resources for bandwidth aggregation per slot

- Maximum number of P aggregated SRS resources for bandwidth aggregation

- Maximum number of P aggregated SRS resources for bandwidth aggregation per slot

##### New FGs for BW Aggregation

###### Aperiodic SRS for Positioning BW Aggregation

With regards to aperiodic SRS for Positioning & BW aggregation, there currently 3 ways that a UE can support this feature according to the agreements below:

- 1st method: Existing Rel-17 DCI framework (i.e. use multiple DCIs schedule SRSs in multiple carriers) is being used

- 2nd method: use a DCI format 0_3 or 1_3 for multi-cell PDSCH/PUSCH scheduling to trigger SRS resources for bandwidth aggregation in multiple CCs

- 3rd method: Support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers, as a separate UE capability.

- Agreement With regard to support of aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, at least the existing Rel-17 DCI framework (i.e. use multiple DCIs schedule SRSs in multiple carriers) can be reused - FFS: whether Rel-18 DCI framework for multi-cell PDSCH/PUSCH scheduling with a single DCI (i.e. single DCI schedules SRSs in multiple carriers) can also be reused with or without specification work in RAN1. Agreement With regard to aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, support both Option 2 and Option1. - Option 2: Support to use a DCI format 0_3 or 1_3 for multi-cell PDSCH/PUSCH scheduling to trigger SRS resources for bandwidth aggregation in multiple CCs. - Option 1: Support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers, as a separate UE capability. - Reuse Rel-17 DCI framework without modification. - If a single DCI indicates transmission of an aperiodic positioning SRS resource set, UE transmits aperiodic positioning SRS resource sets across all linked carriers for bandwidth aggregation. Agreement For positioning SRS bandwidth aggregation, introduce a new RRC signaling to indicate whether to enable Rel-17 single DCI-triggering SRS resource sets across the linked carriers.

Observation 1: For Aperiodic SRS BW aggregation, it has been already agreed in the main session to introduce a separate capability for the following feature:

- Support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers

Proposal 9: For aperiodic SRS BW aggregation, introduce in the UE feature list a new FG dedicated for the aperiodic SRS for positioning BW aggregation, which has the following component that can be reported:

- Max number of aperiodic aggregated SRS Resources for positioning.

- Max number of aperiodic aggregated  SRS Resources for positioning per slot.

- Support receiving a single DCI scheduling positioning SRS resource sets across linked carriers

###### Support of processing multiple PFL combinations for DL PRS BW aggregation

The following agreement was achieved in the previous meeting:

Agreement Configuring up to two PFL combinations is supported (e.g. PFL1 aggregated with PFL2 and PFL3 aggregated with PFL4). - Send an LS to RAN4 (CC to RAN2 and RAN3) to inform them with the above agreement and specify corre-sponding requirements. - Note: more than one combinations are measured in TDMed manner

We believe the above feature should be a separate UE capability, as it is the case of multiple PFL processing in legacy NR Rel-16:

| 13. NR Positioning | 13-1a | Max number of positioning frequency layers UE supports across all positioning methods across all bands | Max number of positioning frequency layers UE supports across all positioning methods across all bands Values: {1, 2, 3, 4} |
| --- | --- | --- | --- |

Based on the above we make the following proposal:

Proposal 10: For DL PRS BW aggregation, introduce a new FG for the support of processing multiple PFL combinations for DL PRS BW Aggregation. This capability is reported per UE.

- Candidate values: {1, 2}

Furthermore, the following was agreed last meeting with regards to the interaction of Rel-17 features with the DL PRS BW aggregation.

Agreement When the LMF requests aggregated measurements, the following existing requested fields can also be applicable: - A request for reduced sample processing for aggregated measurement - Reuse the existing field: reducedDL-PRS-ProcessingSamples-r17 - A request for lower Rx beam sweeping factor for FR2 that is applicable for aggregated measurements - Reuse the existing field: lowerRxBeamSweepingFactor-FR2 - A request for the maximum number of aggregated UE-Rx-Tx / RSTD measurements for different DL-PRS Resources or DL-PRS Resource Sets per TRP - Reuse the existing field: maxDL-PRS-RSTD-MeasurementsPerTRPPair

From all these features, we believe that it should be a separate UE capability the UE to do fast processing (M<4) with PRS aggregated meausrements for the following reason:

- A device, provided a given processing and memory capability, it may be able to support fast processing of PRS, or PRS aggregated measurements, but it may not be able to support both of them concurrently. In other words, doing fast processing for PRS aggregated measurements is driving the processing envelope of a device and it should be considered as a separate UE capability.

Proposal 11: Introduce a separate FG for the support of reduced sample processing for aggregated measurements. This capability should be reported per band.

# 5 Redcap Positioning

##### FG 41-5-1 (PRS Frequency Hopping in RRC_CONNECTED)

For PRS frequency hopping, all the components are still in brackets and need to be discussed.

| Component | Views |
| --- | --- |
| Maximum DL PRS bandwidth across all hops | This component is needed to be reported because it directly corresponds to the amount of hopping a device can do, and the LMF needs to know in order to set correctly the response time, have expectations of potential accuracy, set the correct DL-PRS assistance data, etc. |
| Maximum DL PRS bandwidth per hop | Even though we support to keep this component, we are OK to keep it and add a note that: it follows component 1 of FG 13-1. In other words, we can reuse the value that the UE will be reporting either way for 13-1. |
| Maximum number of hops | This component is needed to be reported, so that the LMF can determine the measurement period of the UE and set the response time accordingly. |
| Duration of DL PRS symbols N in units of ms a UE can process every T ms for maximum bandwidth per hop | This is needed because for the same number of PRS symbols, the UE has to do the overlapped-based processing with larger FFT size, etc. |
| Max number of DL PRS resources that UE can processing per slot for a PFL | Even though we support to have this component to be explicitly reported, we are OK to add it and in addition include a note that says that it follows the corresponding component of FG 13-1. |
| RF Rx retune times between consecutive hops | This has been already agreed in the main session that it is a UE capability. |
| Overlapping PRB(s) between adjacent hops | We strongly support to keep this component, especially if the “Maximum number of hops” is not reported. We believe it is needed for multiple purposes: - Different UEs have different implementations and algorithms related to the amount of overlap they require. - In order to determine the amount of hops for a given BW-per-hop and max-BW-across-hops, the amount of overlap is needed to be known. |

Proposal 12: For the FG 41-5-1 (PRS Frequency hopping in RRC_CONNECTED), support the following components:

- Component 1: Maximum DL PRS bandwidth across all hops

- Component 2: Maximum DL PRS bandwidth per hop

  - No need to report a new value; reuse what is reported for component 1 of FG 13-1

- Component 3: Maximum number of hops

- Component 4: Duration of DL PRS symbols N in units of ms a UE can process every T ms

- Component 5: Max number of DL PRS resources that UE can processing per slot for a PFL

- Component 6: RF Rx retune times between consecutive hops

- Component 7: Overlapping PRB(s) between adjacent hops

  - Candidate Values: {0,1,2,4}

Proposal 13:  For the FG 41-5-1 (PRS Frequency hopping in RRC_CONNECTED), adding the FG 28-1 as prerequisite is unnecessary.

##### FG 41-5-2 (Support of positioning SRS with Tx frequency hopping)

For SRS frequency hopping, we support all the following components to be included in the FG 41-5-2:

- Maximum SRS BW per hop which is supported and reported by UE

- Maximum number of SRS hop(s) per SRS resource which is supported and reported by UE

- One or more Frequency domain overlap(s) between hops which are supported and reported by UE

- RF Tx retune times between consecutive hops which is supported and reported by the UE

- UE capability on the retune time between active BWP and the first/last SRS hop

In addition to the above components that are related to the mechanism of Tx Frequency hopping, the following components, related the maximum number of P/SP/AP SRS resources and sets should be introduced:

- Max number of SRS Resource Sets for positioning  with Tx Frequency Hopping supported by UE per CC.

- Max number of P/SP/AP SRS Resources for positioning with Tx Frequency Hopping per CC.

- Max number of periodic SRS Resources for positioning with Tx Frequency Hopping per CC.

- Max number of periodic SRS Resources for positioning with Tx Frequency Hopping per CC per slot

- Max number of aperiodic SRS Resources for positioning with Tx Frequency Hopping per CC

  - This component could start from “0” which would mean that the UE doesn’t support aperiodic SRS for Positioning with Tx hopping

- Max number of aperiodic SRS Resources for positioning with Tx Frequency Hopping per CC per slot

  - This component could start from “0” which would mean that the UE doesn’t support aperiodic SRS for Positioning with Tx hopping

- Max number of semi-persistent SRS Resources for positioning with Tx Frequency Hopping supported by UE per CC

  - This component could start from “0” which would mean that the UE doesn’t support semi-persistent SRS for Positioning with Tx hopping

- Max number of semi-persistent SRS Resources for positioning with Tx Frequency Hopping supported by UE per CC per slot

  - This component could start from “0” which would mean that the UE doesn’t support semi-persistent SRS for Positioning with Tx hopping

Proposal 14:  With regards to the support of positioning SRS with Tx frequency hopping, we support to introduce the following components:

- Maximum SRS BW per hop which is supported and reported by UE

- Maximum number of SRS hop(s) per SRS resource which is supported and reported by UE

- One or more Frequency domain overlap(s) between hops which are supported and reported by UE

- RF Tx retune times between consecutive hops which is supported and reported by the UE

- UE capability on the retune time between active BWP and the first/last SRS hop

- Max number of SRS Resource Sets for positioning  with Tx Frequency Hopping supported by UE per CC.

- Max number of P/SP/AP SRS Resources for positioning with Tx Frequency Hopping per CC.

- Max number of periodic SRS Resources for positioning with Tx Frequency Hopping per CC.

- Max number of periodic SRS Resources for positioning with Tx Frequency Hopping per CC per slot

- Max number of aperiodic SRS Resources for positioning with Tx Frequency Hopping per CC

- Max number of aperiodic SRS Resources for positioning with Tx Frequency Hopping per CC per slot

- Max number of semi-persistent SRS Resources for positioning with Tx Frequency Hopping supported by UE per CC

- Max number of semi-persistent SRS Resources for positioning with Tx Frequency Hopping supported by UE per CC per slot

##### New FGs for Redcap Positioning

###### Support of UL Time Window configuration and transmission of SRS for positioning with Tx Frequency hopping within the window.

According to the following agreement, a separate UE capability should be introduced for the support of UL Time Window configuration and transmission of SRS for positioning with Tx Frequency hopping within the window:

Agreement The agreement below is updated by removing the bracket on “or outside” and adding one note. Agreement For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to []transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without [or outside] UL time window (i.e. option 1) - FFS: details on the collision rules Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group. Note: UE is not expected to be configured with a SRS for positioning hopping cycle partially overlapping with UTW.

Proposal 15: Introduce a separate FG for the support of UL Time Window configuration and transmission of SRS for positioning with Tx Frequency hopping within the window, according to the above agreement. This capability should be reported per band.

# 6 Conclusions

We make the following observations and proposals:

Proposal 1: FG 41-1-1 should be reported per FSPC

Proposal 2: Capture the following FGs in the LTE UE feature list: 41-1-1, 41-1-2, 41-1-3, 41-1-4c, 41-1-4d, 41-1-5, 41-1-7x, 41-1-8, 41-1-10, 41-1-11, 41-1-12, 41-1-12, 41-1-13, 41-1-13b, 41-1-14, 41-1-18, 41-1-19, 41-1-20a, 41-1-20b.

Proposal 3: The feature of supporting measurements within an indicated time window should be supported for both UE-based and UE-assisted Positioning.

Proposal 4: Introduce a new FG for the support of a timestamp that includes a symbol index. It can be reported per UE.

Proposal 5: Introduce a new FG for the Support of associating a single Rx-Tx or RSTD measurement with up to N_sample RSCP/RSCPD measurement. It should be reported per band.

Proposal 6: Introduce a new FG for the Support of reporting time domain measurements with granularity using ReportingGranularityfactor k={-1, -2}. It can be reported per UE.

Proposal 7: Regarding the FG 41-4-6 & FG 41-4-7 (Positioning SRS bandwidth aggregation in RRC_CONNECTED),

- They should be reported per FS.

- Support the components 1,2,3 that are shown in brackets with the values as written in the existing UE features list.

- Support keeping the “periodic/semi-persistent/aperiodic”

- Introduce a separate component the maximum number of periodic SRS resources:

  - Maximum number of P aggregated SRS resources for bandwidth aggregation

  - Maximum number of P aggregated SRS resources for bandwidth aggregation per slot

Proposal 8: For FG 41-4-8 (SRS BW aggregation in RRC Inactve) have a total 4 components for each FG related to the maximum number of periodic & semi-persistent SRS resources:

- Maximum number of P/SP aggregated SRS resources for bandwidth aggregation

- Maximum number of P/SP aggregated SRS resources for bandwidth aggregation per slot

- Maximum number of P aggregated SRS resources for bandwidth aggregation

- Maximum number of P aggregated SRS resources for bandwidth aggregation per slot

Observation 1: For Aperiodic SRS BW aggregation, it has been already agreed in the main session to introduce a separate capability for the following feature:

- Support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers

Proposal 9: For aperiodic SRS BW aggregation, introduce in the UE feature list a new FG dedicated for the aperiodic SRS for positioning BW aggregation, which has the following component that can be reported:

- Max number of aperiodic aggregated SRS Resources for positioning.

- Max number of aperiodic aggregated  SRS Resources for positioning per slot.

- Support receiving a single DCI scheduling positioning SRS resource sets across linked carriers

Proposal 10: For DL PRS BW aggregation, introduce a new FG for the support of processing multiple PFL combinations for DL PRS BW Aggregation. This capability is reported per UE.

- Candidate values: {1, 2}

Proposal 11: Introduce a separate FG for the support of reduced sample processing for aggregated measurements. This capability should be reported per band.

Proposal 12: For the FG 41-5-1 (PRS Frequency hopping in RRC_CONNECTED), support the following components:

- Component 1: Maximum DL PRS bandwidth across all hops

- Component 2: Maximum DL PRS bandwidth per hop

  - No need to report a new value; reuse what is reported for component 1 of FG 13-1

- Component 3: Maximum number of hops

- Component 4: Duration of DL PRS symbols N in units of ms a UE can process every T ms

- Component 5: Max number of DL PRS resources that UE can processing per slot for a PFL

- Component 6: RF Rx retune times between consecutive hops

- Component 7: Overlapping PRB(s) between adjacent hops

  - Candidate Values: {0,1,2,4}

Proposal 13:  For the FG 41-5-1 (PRS Frequency hopping in RRC_CONNECTED), adding the FG 28-1 as prerequisite is unnecessary.

Proposal 14:  With regards to the support of positioning SRS with Tx frequency hopping, we support to introduce the following components:

- Maximum SRS BW per hop which is supported and reported by UE

- Maximum number of SRS hop(s) per SRS resource which is supported and reported by UE

- One or more Frequency domain overlap(s) between hops which are supported and reported by UE

- RF Tx retune times between consecutive hops which is supported and reported by the UE

- UE capability on the retune time between active BWP and the first/last SRS hop

- Max number of SRS Resource Sets for positioning  with Tx Frequency Hopping supported by UE per CC.

- Max number of P/SP/AP SRS Resources for positioning with Tx Frequency Hopping per CC.

- Max number of periodic SRS Resources for positioning with Tx Frequency Hopping per CC.

- Max number of periodic SRS Resources for positioning with Tx Frequency Hopping per CC per slot

- Max number of aperiodic SRS Resources for positioning with Tx Frequency Hopping per CC

- Max number of aperiodic SRS Resources for positioning with Tx Frequency Hopping per CC per slot

- Max number of semi-persistent SRS Resources for positioning with Tx Frequency Hopping supported by UE per CC

- Max number of semi-persistent SRS Resources for positioning with Tx Frequency Hopping supported by UE per CC per slot

Proposal 15: Introduce a separate FG for the support of UL Time Window configuration and transmission of SRS for positioning with Tx Frequency hopping within the window, according to the above agreement. This capability should be reported per band.

Proposal 16: Support the changes in the UE features shown in Section 7

# 7 Detailed Proposed Capabilities

#### 7.1 SL Positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | - Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-1 | Common SL PRS Processing Capability in a SL BWP | 1. Maximum SL PRS bandwidth in MHz in a resource pool for positioning, which is supported and reported by UE for SL-PRS measurement 2. Maximum number of active SL PRS resources across all configured RPs assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE 3. Maximum number of slots with active SL PRS resources across all configured RPs assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE 4. Minimum time after the end of a slot carrying the active SL-PRS resource(s) assuming maximum number of symbols and maximum bandwidth for a UE finish the SL-PRS resource processing which is supported and reported by UE 5. SL PRS buffering capability |  | Yes | No | The UE does not support the reception and processing of SL PRS | Per FCPC | n/a | n/a | n/a | Component 1 candidate values: FR1 bands: {5, 10, 20, 40, 50, 80, 100} FR2 bands: {50, 100, 200, 400} Component 2 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Component 3 candidate values: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} Component 4 candidate values: {20ms, 30ms, 40ms, 50ms, 100ms} Component 5 candidate values: {Type 1 – sub-slot/symbol level buffering, Type 2 – slot level buffering} Note: a SL PRS resource is considered as active starting at the end of the last symbol of the PSCCH carrying the SCI trigger and the occupancy is released at the end of timeline indicated in component 4 Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-2 | Receiving SL-PRS in a shared resource pool | 1. Support SL-PRS in shared resource pool 2. Support receiving SCI format 2D |  | Yes | No | Receiving SL-PRS in a shared resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-3 | Receiving SL-PRS in a dedicated resource pool | 1. Support SL-PRS in dedicated resource pool 2. Support receiving SCI format 1B |  | Yes | No | Receiving SL-PRS in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-4a | Transmitting SL-PRS scheme 1 in a shared resource pool | 1. Support of transmitting SL-PRS scheme 1 in a shared resource pool 2. Support transmitting SCI format 2D [3. Support downlink pathloss based open loop power control] |  | Yes | No | Transmitting SL-PRS scheme 1 in a shared resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported [Note: component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1] | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-4b | Transmitting SL-PRS scheme 1 in a dedicated resource pool | 1. UE can transmit SL-PRS and PSCCH within a slot without PSSCH in dedicated resource pool 2. UE can transmit SL-PRS according to the mapping rule between PSCCH and SL-PRS 3. Support transmitting SCI format 1B 4. Support receiving DCI format 3_0 [5. Support open loop power control of SL-PRS] |  | Yes | No | Transmitting SL-PRS scheme 1 in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported [Note: component 5 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1] | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-4c | Transmitting SL-PRS mode 2 in a dedicated resource pool | 1. UE can transmit SL-PRS and PSCCH within a slot without PSSCH in dedicated resource pool 2. UE can transmit SL-PRS according to the mapping rule between PSCCH and SL-PRS 3. Support transmitting SCI format 1B |  | Yes | No | Transmitting SL-PRS mode 2 in a dedicated resource pool is not supported | [Per band] | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-4d | Transmitting SL-PRS scheme 2 in a shared resource pool | 1. Support of transmitting SL-PRS scheme 2 in a shared resource pool 2. Support transmitting SCI format 2D |  | Yes | No | Transmitting SL-PRS scheme 2 in a shared resource pool is not supported | [Per band] | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-5 | SL-PRS congestion control in a dedicated resource pool | 1) UE can report CBR measurement to gNB when operating in Scheme 1 and Scheme 2 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. 3) UE can process CBR and CR within the time it indicates |  | Yes | No | SL-PRS congestion control in a dedicated resource pool is not supported | Per band | n/a | n/a | n/a | Component-3 candidate value set {Congestion process time 1, Congestion process time 2, Congestion process time 3} where Congestion process time 1: 2, 2, 4, 8 slots for 15, 30, 60, 120 kHz subcarrier spacing. Congestion process time 2: 2, 4, 8, 16 slots for 15, 30, 60, 120 kHz subcarrier spacing, Congestion process time 3: 3, 6, 12, 24 slots for 15, 30, 60, 120 kHz subcarrier spacing Note: component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | 41. NR_pos_enh2 |
| 41. NR_pos_enh2 | 41-1-7a | SL PRS measurement for SL-RSTD | 1. Support SL RSTD measurement based on SL-PRS 2. Support SL RSTD measurement reporting [FFS reporting N SL RSTD measurements for different SL-PRS reception for the same pair of UEs] |  | No | No | UE does not support SL PRS measurement for SL-RSTD | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported [FFS: candidate values of N={1,2,3,4}] | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-7b | SL PRS measurement for SL RTOA | 1. Support SL RTOA measurement based on SL-PRS 2. Support SL RTOA measurement reporting 3. Support reporting N SL RTOA measurements for different SL-PRS reception for the same pair of UEs |  | No | No | UE does not support SL PRS measurement for SL RTOA | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported candidate values of N={1,2,3,4} | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-7c | SL PRS measurement for UE Rx – Tx time difference without Tx time stamp | 1. Support UE Rx – Tx time difference measurement based on SL PRS 2. Support UE Rx – Tx time difference measurement reporting without Tx time stamp 3. Support reporting N Rx-Tx measurements for different SL-PRS reception for the same pair of UEs |  | No | No | UE does not support SL PRS measurement for Rx – Tx time difference without Tx time stamp | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported [FFS: candidate values of N={1,2,3,4}] | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-7d | SL PRS measurement for UE Rx – Tx time difference with Tx time stamp | 1. Support UE Rx – Tx time difference measurement based on SL PRS 2. Support UE Rx – Tx time difference measurement reporting with Tx time stamp 3. Reporting M Rx-Tx measurements for the same SL-PRS transmission (or reception) and different SL-PRS reception (or transmission) for the same pair of UEs [FFS reporting N Rx-Tx measurements for different SL-PRS reception for the same pair of UEs] |  | No | No | UE does not support SL PRS measurement for UE Rx – Tx time difference with Tx time stamp | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported Component 3 candidate values of M={1,2,3,4} [FFS: candidate values of N={1,2,3,4}] | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-7e | SL PRS measurement for SL PRS-RSRP | 1. Support SL PRS-RSRP measurement based on SL-PRS 2. Support SL PRS-RSRP measurement reporting |  | No | No | UE does not support SL PRS measurement for SL PRS-RSRP | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-7f | SL PRS measurement for SL PRS-RSRPP | 1. Support SL PRS-RSRPP measurement based on SL-PRS 2. Support SL PRS-RSRPP measurement reporting |  | No | No | UE does not support SL PRS measurement for SL PRS-RSRPP | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-7g | SL PRS measurement for SL AoA | 1. Support SL AoA measurement based on SL-PRS 2. Support SL AoA measurement reporting types. Candidate values: bitmap {GCS, LCS with translation, LCS without translation}. |  | No | No | UE does not support SL PRS measurement for SL AoA | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-8 | Support of random selection in a dedicated resource pool | 1. Support transmitting SL-PRS and associated PSCCH using random selection in a dedicated resource pool 2 Support DL pathloss based open loop power control when configured by NR Uu |  | Yes | No | UE cannot transmit SL-PRS using random selection in a dedicated resource pool | Per band | n/a | n/a | n/a | Note: Configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 2 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-10 | Support of full sensing in a dedicated resource pool | 1. UE can transmit SL-PRS and associated PSCCH using full sensing 2. Support DL pathloss based open loop power control when configured by NR Uu |  | Yes | No | UE cannot transmit SL-PRS using full sensing in a dedicated resource pool | Per FS | n/a | n/a | n/a | Note: Configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 2 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-11 | TDM-based multiplexing of SL-PRS reception from different UEs in the same slot in dedicated resource pool | - Support of TDM-based multiplexing of SL-PRS reception from different UEs in the same slot in dedicated resource pool |  | No | No | TDM-based multiplexing of SL-PRS reception from different UEs in the same slot is not supported in dedicated resource pool | [Per band] | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-12 | Comb-based multiplexing for SL-PRS reception from different UEs in the same slot in dedicated resource pool | - Support of comb-based multiplexing for SL-PRS reception from different UEs in the same slot in dedicated resource pool |  | No | No | Comb-based multiplexing for SL-PRS reception from different UEs in the same slot is not supported in dedicated resource pool | [Per band] | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-13 | Reporting the additional paths for SL positioning | 1. Maximum number of additional detected path timing reporting for K additional paths for SL positioning 2. Support of RSRPP reporting for additional paths |  | No | No | Reporting the additional paths for SL positioning is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 6, 8} Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-14 | LoS/NLoS indicator for SL positioning [per measurement] | Support of LoS/NLoS indicator for SL positioning [per measurement] |  | No | No | LoS/NLoS indicator for SL positioning [per measurement] is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {hard value, hard+soft value} Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-17 | Open loop SL pathloss based power control and SL RSRP report for dedicated resource pool | Support of open loop SL pathloss based power control and SL RSRP report for dedicated resource pool for unicast transmissions |  | Yes | Yes | Open loop SL power control and SL RSRP report [for dedicated resource pool] is not supported for unicast transmissions FFS: whether/how to address UEs that only support SL PRS reception but not sidelink RSRP reporting | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-19 | ARP location provision for sidelink as assistance data | Support of ARP location provision for sidelink as assistance data |  | No | No | UE cannot provide ARP location for sidelink as assistance data | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-19a | Report of Rx ARP-ID with SL positioning measurements | Support providing Rx ARP-ID with SL positioning measurements |  | No | No | UE cannot report Rx ARP-ID with SL positioning measurements | Per band | n/a | n/a | n/a | Need for location server/server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-20a | First path RSRPP reporting in SL for SL-TDoA measurement report | Support of SL-RSRPP reporting for first path for SL-TDOA | None | No | No | UE does not support first-path RSRPP reporting for SL TDoA | per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-1-20b | First path RSRPP reporting in SL for SL-RTT measurement report | Support of SL-RSRPP reporting for first path for SL-RTT | None | No | No | UE does not support first-path RSRPP reporting for SL RTT | per band | n/a | n/a | n/a | Need for location server / server UE to know if the feature is supported | Optional with capability signaling |

#### 7.2 Carrier Phase Positioning

| Features | Index |  | Feature group |  | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) |  | Need of FDD/TDD differentiation |  | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 |  | Note |  | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 |  | 41-2-1 |  | DL RSCP reporting based on DL PRS in RRC_CONNECTED |  |  | 1. Support of DL RSCP reporting based on DL PRS in RRC_CONNECTED 2. The maximum number of first path PRS RSCP measurements per TRP | 13-11 | No | n/a | DL RSCP reporting based on DL PRS in RRC_CONNECTED is not supported |  | Per band | n/a | n/a | n/a |  | Note: DL RSCP is reported together with UE Rx-Tx time difference measurement Note: The maximum number of RSCP measurements per TRP is less than or equal to FG 13-11 Need for location server to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 |  | 41-2-1a |  | DL RSCPD reporting based on DL PRS in RRC_CONNECTED |  |  | 1. Support of DL RSCPD reporting based on DL PRS in RRC_CONNECTED 2. The maximum number of first path PRS RSCPD measurements per pair of TRPs | 13-6 | No | n/a | DL RSCPD reporting based on DL PRS in RRC_CONNECTED is not supported |  | Per band | n/a | n/a | n/a |  | Note: DL RSCPD is reported along with measurement report for DL-RSTD Note: The maximum number of RSCP measurements per TRP is less than or equal to FG 13-6 [Need for location server to know if the feature is supported] | Optional with capability signaling |
| 41. NR_pos_enh2 |  | 41-2-2 |  | DL RSCP reporting based on DL PRS in RRC_INACTIVE |  |  | Support of DL RSCP reporting based on DL PRS measurement in RRC_INACTIVE | 27-18c | No | n/a | DL RSCP reporting based on DL PRS in RRC_INACTIVE is not supported |  | Per band | n/a | n/a | n/a |  | Note: DL RSCP is reported together with UE Rx-Tx time difference measurement [Note: The maximum number of RSCP measurements per TRP is less than or equal to FG 13-11] [Need for location server to know if the feature is supported] | Optional with capability signaling |
| 41. NR_pos_enh2 |  | 41-2-2a |  | DL RSCPD reporting based on DL PRS in RRC_INACTIVE |  |  | 1. Support of DL RSCPD reporting based on DL PRS measurement in RRC_INACTIVE | 27-18a | No | n/a | DL RSCPD reporting based on DL PRS in RRC_INACTIVE is not supported |  | Per band | n/a | n/a | n/a |  | Note: DL RSCPD is reported along with measurement report for DL-RSTD [Note: The maximum number of RSCP measurements per TRP is less than or equal to FG 13-6] [Need for location server to know if the feature is supported] | Optional with capability signaling |
| 41. NR_pos_enh2 |  | 41-2-3 |  | Measurement on indicated DL PRS resource sets within the indicated time window(s) for UE based and UE assisted |  |  | Support of Measurement on indicated DL PRS resource sets within the indicated time window(s) for UE based and UE assisted | 13-1 | No | N.A. | Measurement on indicated DL PRS resource sets within the indicated time window(s) for UE based and UE assisted is not supported |  | Per band | N.A. | N.A. | N.A. |  | Need for location server to know if the feature is supported | Optional with capability signaling |
| 41. NR_pos_enh2 |  | 41-2-4 |  | UE-based Carrier Phase Positioning |  |  | 1. Support of carrier phase measurement for UE-based positioning 2. Support of Assistance data for UE-based Carrier Phase Positioning |  | No | N.A. | UE-based Carrier Phase Positioning is not supported |  | Per band | N.A. | N.A. | N.A. |  | Need for location server to know if the feature is supported. | Optional with capability signaling. |

#### 7.3 LPHAP Positioning

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE |  | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) |  | Need of FDD/TDD differentiation |  | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-3-1 | SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state for initial UL BWP | 1. SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state for initial UL BWP | 27-15 | Yes | n/a |  | SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state for initial UL BWP is not supported |  | Per band |  | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signalling |
| 41. NR_pos_enh2 | 41-3-2 | SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state configured outside initial UL BWP | Support of SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state configured outside initial UL BWP | 27-15 | Yes | n/a |  | Support of SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state configured outside initial UL BWP is not supported |  | Per band |  | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signalling |
| 41. NR_pos_enh2 | 41-3-3 | Support of PRS measurement in RRC_IDLE | Support of DL PRS measurement in RRC_IDLE for Rel. 17 methods the UE supports in RRC_INACTIVE | FFS 27-17, 27-6, 27-18a and/or 27-18b | No | n/a |  | PRS processing in RRC_IDLE is not supported |  | Per band |  | n/a | n/a | n/a | Note: Same PRS processing capabilities as those described in FG 27-6 for RRC_INACTIVE PRS processing. Note: If both FG 27-18a and 41-3-3 is supported, then PRS measurement in RRC_IDLE for DL-TDOA is supported Note: If both FG 27-18b and 41-3-3 is supported, then PRS measurement in RRC_IDLE for DL-AoD is supported Note: Support of PRS processing measurement in RRC_IDLE state does not imply that LMF is aware of or controlling UE RRC state Need for location server to know if the feature is supported | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-3-4 | UE autonomous UL timing determinaton for SRS for positioning in multiple cells in RRC_INACTIVE | If configured by the network, a UE can autonomously adjust the TA, when cell-reselection happens. | 41-3-1 or 41-3-2 | Yes |  | UE is expected to maintain the TA from the last serving cell | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |

#### 7.4 PRS/SRS BW Aggregation

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-4-1 | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 13-1 | No | n/a | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTEDis not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG41-4-1 follows buffering capability type reported in FG13-1 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 13-1, or this value T should be equal or larger than the value T reported by FG 13-1 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 13-1 Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30% | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-4-1a | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 41-4-1 | No | n/a | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300}} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG41-4-1a follows buffering capability type reported in FG13-1 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 13-1 or this value T should be equal or larger than the value T reported by FG 13-1 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 13-1 Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30% | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-4-1b | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 27-6 | No | n/a | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG41-4-1b follows buffering capability type reported in FG13-1 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 27-6 or this value T should be equal or larger than the value T reported by FG 27-6 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 27-6 | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-4-1c | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot under it | 41-4-1b | No | n/a | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG41-4-1c follows buffering capability type reported in FG13-1 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 27-6 or this value T should be equal or larger than the value T reported by FG 27-6 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 27-6 | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-4-3 | PRS bandwidth aggregation in RRC_CONNECTED — DL-TDOA | Support of PRS bandwidth aggregation in RRC_CONNECTED for DL-TDOA | 13-3, 41-4-1 | No | n/a | PRS bandwidth aggregation in RRC_CONNECTED for DL-TDOA is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-3a | PRS bandwidth aggregation in RRC_CONNECTED —Multi-RTT | Support of PRS bandwidth aggregation in RRC_CONNECTED FOR Multi-RTT | 13-4, 41-4-1 | No | n/a | PRS bandwidth aggregation in RRC_CONNECTED FOR Multi-RTT is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-4 | PRS bandwidth aggregation in RRC_ INACTIVE — DL-TDOA | Support of PRS bandwidth aggregation in RRC_ INACTIVE for DL-TDOA | 27-18a, 41-4-1b | No | n/a | PRS bandwidth aggregation in RRC_ INACTIVE for DL-TDOA is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-4a | PRS bandwidth aggregation in RRC_ INACTIVE —Multi-RTT | Support of PRS bandwidth aggregation in RRC_ INACTIVE for Multi-RTT | 27-18a, 41-4-1b | No | n/a | PRS bandwidth aggregation in RRC_ INACTIVE for Multi-RTT is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-5 | PRS bandwidth aggregation in RRC_IDLE — DL-TDOA | Support of PRS bandwidth aggregation in RRC_IDLE for DL-TDOA | 41-3-3, 41-4-1b | No | n/a | PRS bandwidth aggregation in RRC_IDLE for DL-TDOA is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-6 | Positioning SRS bandwidth aggregation in RRC_CONNECTED | 1. The maximum number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 3. Support of periodic/semi-persistent/aperiodic UL SRS for UL bandwidth aggregation 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of periodic/semi-persistent/aperiodic aggregated SRS resources for bandwidth aggregation 7. Maximum number of [periodic/semi-persistent/aperiodicaggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers 9. Maximum number of P/SP aggregated SRS resources for bandwidth aggregation 10. Maximum number of P/SP aggregated SRS resources for bandwidth aggregation per slot | 13-86-6 | Yes | n/a | Positioning SRS bandwidth aggregation in RRC_CONNECTED is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {2,3} Component 2 candidate values: FR1 bands: {80, 100, 160, 200} FR2 bands: {50, 100, 200, 400, 600, 800} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: {1,2,4,8,16,32,64} Component 7 candidate values: {1,2,3,4,5,6,8,10,12,14} Note: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. Need for location server to know if the feature is supported. Note: each two or three linked SRS resources are counted as 1 resource | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-7 | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED | 1. The maximum number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE 3. Support of periodic/semi-persistent/aperiodic UL SRS for UL bandwidth aggregation 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of [periodic/semi-persistent/aperiodic] aggregated SRS resources for bandwidth aggregation 7. Maximum number of [periodic/semi-persistent/aperiodic] aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers 9. Guard period 10. Maximum number of P/SP aggregated SRS resources for bandwidth aggregation 11. Maximum number of P/SP aggregated SRS resources for bandwidth aggregation per slot | 13-8 | Yes | n/a | Positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: {2,3} Component 2 candidate values: FR1 bands: {80, 100, 160, 200} FR2 bands: {50, 100, 200, 400, 600, 800} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: {1,2,4,8,16,32,64} Component 7 candidate values: {1,2,3,4,5,6,8,10,12,14} Note: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. Need for location server to know if the feature is supported. Note: Guard period is needed before and after the aggregated SRS transmissions when SRS resource is configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC Note: each two or three linked SRS resources are counted as 1 resource | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-7b | Support of Aperiodic positioning SRS bandwidth aggregation | 1. Max number of aperiodic aggregated SRS Resources for positioning. 2. Max number of aperiodic aggregated SRS Resources for positioning per slot. 3.Support receiving a single DCI scheduling positioning SRS resource sets across linked carriers | 41-4-6 or 41-4-7 | Yes | n/a | Aperiodic Positioning SRS bandwidth aggregation is not supported | Per FS | n/a | n/a | n/a | Component 1 Values = {1,2,4,8,16,32,64} Component 2 Values = {1,2,3,4,5,6,8,10,12,14} Component 3 Values = {Support or not support} Note: If the UE supports component 3, then if a single DCI indicates transmission of an aperiodic positioning SRS resource set, UE transmits aperiodic positioning SRS resource sets across all linked carriers for bandwidth aggregation |  |
| 41. NR_pos_enh2 | 41-4-8 | Positioning SRS bandwidth aggregation in RRC_INACTIVE | 1. The maximum number of supported aggregated carriers in intra band contiguous carriers 2. Maximum aggregated UL SRS bandwidth in MHz, which is supported and reported by UE] 3. Support of periodic/semi-persistent UL SRS for UL bandwidth aggregation 5. Max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation 6. Maximum number of periodic aggregated SRS resources for bandwidth aggregation 7. Maximum number of periodic aggregated SRS resources for bandwidth aggregation per slot 8. Support the same SRS power reduction across aggregated carriers 9. Maximum number of P/SP aggregated SRS resources for bandwidth aggregation 10. Maximum number of P/SP aggregated SRS resources for bandwidth aggregation per slot | 27-15b | Yes | n/a | Positioning SRS bandwidth aggregation in RRC_INACTIVE is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {2,3} Component 2 candidate values: FR1 bands: {80, 100, 160, 200M} FR2 bands: {50, 100, 200, 400, 600, 800} Component 5 candidate values: {1, 2, 4, 8, 12, 16} Component 6 candidate values: {1,2,4,8,16,32,64} Component 7 candidate values: {1,2,3,4,5,6,8,10,12,14} Need for location server to know if the feature is supported. | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-5 | M-sample aggregated measurements in RRC_CONNECTED | The capability to support reporting an aggregated measurement based on measuring M=1 or 2 samples (instances) of aggregated DL PRS resources | 27-3-1 | No |  | If the UE does not provide the capability, the UE is assumed to support M=4 only for PRS BW aggregation | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |
| NR_pos_enh2 | X-Agg-6 | Max number of combinations of PFLs UE supports across all positioning methods across all bands | Max number of positioning frequency layers UE supports across all positioning methods across all bands Values: {1, 2} | 41-4-1 or 41-4-1b | No | N/A |  | Per UE | No | No | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |

#### 7.5 Redcap Frequency Hopping

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-5-1 | PRS measurement with Rx frequency hopping within a MG and measurement reportin RRC_CONNECTED for RedCap UEs | 1. Maximum DL PRS bandwidth across all hops 2. Maximum DL PRS bandwidth per hop 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms 5. Max number of PRS resources to be processed per slot 5. RF Rx retune times between consecutive hops 6. Overlapping PRB(s) between adjacent hops | 13-1, | Yes | n/a | PRS measurement with Rx frequency hopping within a MG and measurement report in RRC_CONNECTED for RedCap UEs is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: FR1: {40, 50, 80, 100} FR2: {100, 200, 400} Note 1: With regards to Component 2, it follows component 1 of FG 13-1 Component 3 candidate values: {2,3,4,5,6} Component 4 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note 1: With regards to Component 5, it follows component 4 of FG 13-1 Component 6 candidate values: FR1: {70us, 140us, 210us} FR2: {35us, 70us, 140us} Component 7 candidate values: {0, 1, 2, 4} Note 2: DL PRS buffering capability follows component 2 of FG 13-1 Need for location server to know if the feature is supported. | Optional with capability signalling |
| 41. NR_pos_enh2 | 41-5-1a | PRS measurement with Rx frequency hopping in RRC_INACTIVE for RedCap UEs | Support of PRS measurement with Rx frequency hopping in RRC_INACTIVE for RedCap UEs | 41-5-1, 27-6 | Yes | n/a | PRS measurement with Rx frequency hopping [and measurement report] in RRC_INACTIVE for RedCap UEs is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| 41. NR_pos_enh2 | 41-5-1b | PRS measurement with Rx frequency hopping in RRC_IDLE for RedCap UEs | Support of PRS measurement with Rx frequency hopping in RRC_IDLE for RedCap UEs | 41-5-1 | Yes | n/a | PRS measurement with Rx frequency hopping in RRC_IDLE for RedCap UEs is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| 41. NR_pos_enh2 | 41-5-2 | Support of positioning SRS with Tx frequency hopping in RRC_CONNECTEDfor RedCap UEs | 1. Maximum SRS BW per hop which is supported and reported by UE 2. Maximum number of SRS hop(s) per SRS resource which is supported and reported by UE 3. One or more Frequency domain overlap(s) between hops which are supported and reported by UE c) {0 PRB, 1 PRB, 2 PRBs, 4 PRBs} 4. RF Tx retune times between consecutive hops which is supported and reported by the UE FFS: UE capability on the retune time between active BWP and the first/last SRS hop 5. Max number of SRS Resource Sets for positioning with Tx Frequency Hopping supported by UE per CC. 6. Max number of P/SP/AP SRS Resources for positioning with Tx Frequency Hopping per CC. 7. Max number of periodic SRS Resources for positioning with Tx Frequency Hopping per CC. 8. Max number of periodic SRS Resources for positioning with Tx Frequency Hopping per CC per slot. 9. Max number of aperiodic SRS Resources for positioning with Tx Frequency Hopping per CC. 10. Max number of aperiodic SRS Resources for positioning with Tx Frequency Hopping per CC per slot. 11. Max number of semi-persistent SRS Resources for positioning with Tx Frequency Hopping supported by UE per CC. 13. Max number of semi-persistent SRS Resources for positioning with Tx Frequency Hopping supported by UE per CC per slot. Note: The UE is expected to support intra-slot SRS Tx frequency hopping |  |  |  |  |  |  |  |  | Component 1 candidate values: a) FR1 bands: {5, 10, 20} b) FR2 bands: {50, 100} Component 2 candidate values: {2, 3, 4, 5, 8, 10} Component 3 candidate values: {0 PRB, 1 PRB, 2 PRBs, 4 PRBs} Component 4 candidate values: a) FR1 bands: {70 usec, 140 usec, 210 usec} b) FR2 bands: {35 usec, 70 usec} Component 5 candidate values: Values = {1, 2, 4, 8, 12, 16}. Component 6 candidate values: Values = {1,2,4,8,16,32,64} Component 7 candidate values: Values = {1,2,4,8,16,32,64} Component 8 candidate values: Values = {1,2,3,4,5,6,8,10,12,14} Component 9 candidate values: Values = {0,1,2,4,8,16,32,64} Component 10 candidate values: Values = {0,1,2,3,4,5,6,8,10,12,14} Component 11 candidate values: Values = {0,1,2,4,8,16,32,64} Component 12 candidate values: Values = {0,1,2,3,4,5,6,8,10,12,14} Note: No additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-5-3 | Support of positioning SRS with Tx frequency hopping in RRC_INACTIVE for RedCap UEs | Support of positioning SRS with Tx frequency hopping in RRC_INACTIVE for RedCap UEs | 27-15 | Yes |  | Tx Frequency Hopping SRS for Positioning in RRC_INACTIVE is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-5-4 | Support of the configuration of an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit FH SRS for positioning. Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning | Support of the configuration of an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit | 41-5-2 |  |  | The UE does not require the configuration of an UL time window in order to perform SRS frequency hopping | per band | n/a | n/a | n/a |  | Optional with capability signaling |