3GPP TSG RAN WG1 Meeting #115	R1-2312317

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	8.1.3.2

Source:	Moderator (Futurewei)

Title:	FL Summary #2 on SRS enhancements

Document for:	Discussion and decision

# Introduction

In RAN#94-e, a Work Item for Rel-18 on “MIMO Evolution for Downlink and Uplink” was approved, and the motivations, scopes, and objectives were agreed and then updated in [1]. Among the objectives, the underlined in the following are related to SRS enhancements, mainly in the aspects of SRS for TDD Coherent Joint Transmission (CJT or C-JT) and 8 Tx operation:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

The Work Item has achieved 90% completion at RAN level by RAN#101 [2], and a set of Rel-18 specifications and CRs have been approved / endorsed.

19 contributions [3-21] have been submitted to Agenda Item 8.1.3.2 of RAN1#115 on SRS Enhancements targeting TDD CJT and 8 Tx operations. Main views and further discussion points based on these contributions are collected in this document. Any additional inputs from any company can also be provided in this document.

# SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT

## TS38.211 TPs

### Comb offset hopping symbol indexing notation

CATT, Futurewei, Xiaomi, and ZTE proposed to correct the comb offset hopping symbol indexing notation, to be aligned with the latest changes made in the last CR and to cover cases with per-symbol hopping and per-R-repetition hopping. This is a straightforward TP and seems reasonable.

Proposal 2.1.1: Adopt the text proposal for TS38.211 on comb offset hopping symbol indexing notation corrections:

6.4.1.4.3 Mapping to physical resources

< Unchanged text is omitted>

The frequency-domain starting position k0(pi) is defined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS

where

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+fcoh(nf,ns,fμ,l'l'') mod KTC

< Unchanged text is omitted>

The quantity fcohnf,ns,fμ,l'' is given by

-	if the higher-layer parameter combOffsetHopping is not configured:

fcohnf, ns,fμ,l''=0

-	if the higher-layer parameter combOffsetHopping is configured:

fcohnf ,ns,fμ,l'l''= 
scohSRSm=07c8nf mod 128Nslotframe,μNsymbslot+ns,fμNsymbslot+l0+l''+m2mmod ncohSRS

< Unchanged text is omitted>

- Additional information

  - Reason for change: Symbol indexing notation not updated in some places.

  - Summary for change: Update the indexing from fcohnf, ns,fμ,l' to fcohnf, ns,fμ,l''.

  - Consequences if not approved: Incorrect specification for comb offset hopping equation.

  - TP is for TS38.211 v18.0.0.

| Company | View |
| --- | --- |
| QC | - Ok. |
| Samsung | - Ok. |
| Google | - OK |
| Huawei, HiSilicon | - Support the proposal. |
| ZTE | - OK. |
| OPPO | - OK |
| Lenovo | - OK |
| New H3C | - OK |
| NTT DOCOMO | - OK |
| Nokia, NSB | - OK |
| InterDigital | - OK |
| Ruijie | - OK. |
| LGE | - Ok. |
| Fujitsu | - OK |
| CATT | - Ok |
| Xiaomi | - OK |
| vivo | - Support |
| Sharp | - OK |
| Apple | - OK |

### Other proposals for TS38.211

The following proposals are also for comb offset hopping / cyclic shift hopping. My initial assessment is that these changes are minor / non-technical / editorial, and do not require technical discussions. I still provide a proposal for the one that requires a correction. Companies’ views are appreciated.

- TP 1: Huawei, HiSilicon

6.4.1.4.3	Mapping to physical resources

< Unchanged parts are omitted >

The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDhop at the beginning of each radio frame for which nf mod 128=0, where the comb offset hopping identity nIDhop is contained in the higher-layer parameter combOffsetHopping.

< Unchanged parts are omitted >

- TP 2: InterDigital, Lenovo

It is proposed to change the parameter names hoppingFinerGranularity and hoppingWithRepetition to cyclicShiftHoppingFinerGranularity and combOffsetHoppingWithRepetition, respectively. However, according to the latest RAN2 TS38.331 running CR, the parameter names are already adopted, so it seems no change is needed.

nrofSRS-Ports-n8-r18                        ENUMERATED {ports8, ports8tdm}                                       OPTIONAL,   -- Need R

combOffsetHopping-r18                       SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                                  OPTIONAL,   -- Need R

hoppingSubset-r18                       CHOICE {

transmissionComb-n2                        BIT STRING (SIZE (2)),

transmissionComb-n4                        BIT STRING (SIZE (4)),

transmissionComb-n8                        BIT STRING (SIZE (8)),

}                                                                                                          OPTIONAL,   -- Need R

hoppingWithRepetition-r18               ENUMERATED {symbol, Rrepetition}                                   OPTIONAL  -- Need R

}                                                                                                              OPTIONAL,   -- Need R

cyclicShiftHopping-r18                      SEQUENCE {

hoppingId-r18                           INTEGER (0..1023)                                                  OPTIONAL,   -- Need R

hoppingSubset-r18                      CHOICE {

transmissionComb-n2                        BIT STRING (SIZE (8)),

transmissionComb-n4                        BIT STRING (SIZE (12)),

transmissionComb-n8                        BIT STRING (SIZE (6)),

}                                                                                                          OPTIONAL,   -- Need R

hoppingFinerGranularity-r18             ENUMERATED {enable}                                               OPTIONAL   -- Need R

}                                                                                                              OPTIONAL   -- Need R

So we have the following proposal based on TP1.

Proposal 2.1.2: Adopt the following text proposal for clause 6.4.1.4.3 in TS 38.211.

6.4.1.4.3	Mapping to physical resources

< Unchanged text is omitted>

The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDhop at the beginning of each radio frame for which nf mod 128=0, where the comb offset hopping identity nIDhop is contained in the higher-layer parameter combOffsetHopping.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Editorial error.

  - Summary for change: Editorial correction.

  - Consequences if not approved: Incorrectly referring to “comb offset hopping” as “comb hopping”.

  - TP is for TS38.211 v18.0.0.

| Company | View |
| --- | --- |
| QC | - Ok. |
| Samsung | - Ok. |
| Google | - OK |
| Huawei, HiSilicon | - Support the proposal. |
| ZTE | - OK. |
| OPPO | - OK |
| Lenovo | - OK |
| New H3C | - OK |
| NTT DOCOMO | - OK |
| Nokia, NSB | - OK |
| Interdigital | - OK |
| Ruijie | - OK. |
| LGE | - Ok. |
| Fujitsu | - OK |
| CATT | - Ok |
| Xiaomi | - OK |
| vivo | - Support |
| Sharp | - OK |
| Apple | - OK |

## TS38.214 TPs

### Adding “for an SRS resource” restriction

Vivo proposed to clarify that comb offset hopping and TDM cannot be configured simultaneously per SRS resource instead of per UE. This is relatively minor but still reasonable, and in parallel with the similar description for cyclic shift hopping.

Proposal 2.2.1: Adopt the text proposal for TS38.211 on comb offset hopping and TDM UE assumption:

6.2.1 UE sounding procedure

< Unchanged text is omitted>

-	Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When comb offset hopping is configured by the higher layer parameter [combOffsetHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching' or ‘codebook’, subject to UE capabilities, transmission comb offset(s) are updated as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the comb offset hopping, a UE can be configured with a subset of comb offsets by the higher layer parameter [combOffsetHoppingSubset], where the comb offset hopping is performed only across the comb offsets configured in the subset. The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Comb offset hopping and TDM cannot be configured simultaneously per SRS resource instead of per UE.

  - Summary for change: Add “for an SRS resource” restriction.

  - Consequences if not approved: Confusion on whether the restriction is per UE or per SRS resource.

  - TP is for TS38.211 v18.0.0.

| Company | View |
| --- | --- |
| QC | - Ok. |
| Samsung | - Ok. |
| Google | - OK |
| Huawei, HiSilicon | - Fine with the proposal. |
| ZTE | - OK. |
| OPPO | - OK |
| Lenovo | - OK |
| New H3C | - OK |
| NTT DOCOMO | - OK |
| Nokia, NSB | - OK |
| InterDigital | - OK |
| Ruijie | - OK. |
| LGE | - Ok. |
| Fujitsu | - OK |
| CATT | - Although we think it is not necessary since the whole paragraph is talking about which parameters are configured for an SRS resource, we can accept the change if the majority agree. |
| Xiaomi | - OK |
| vivo | - Support |
| Sharp | - OK |
| Apple | - OK |

## Other proposals for comb offset hopping / cyclic shift hopping:

- Issue 1: Lenovo proposed that the initial CS/Comb offset value (the configured value) may be out of the indicated CS/Comb hopping offset subset.

  - However, based on my understanding, this hopping offsets are additive to the configured initial values, so even if they are not in the hopping offset subset, the equations work fine. So no change is needed.

Any view or new issues can be discussed below.

| Company | View |
| --- | --- |
| QC | - Agree with FL. |
| Samsung | - We agree with FL’s assessment. |
| Google | - Agree with FL |
| Huawei, HiSilicon | - Agree with FL’s assessment. |
| ZTE | - Agree wit FL, no change is needed. |
| OPPO | - Agree with FL. |
| Lenovo | - Thanks for the discussion. We agree hopping offsets are additive to the configured initial values. Here, the discussion focuses on the CS and Comb offset value after hopping. We think there is still interference between SRS with CS and/or Comb offset and legacy SRS if there is no restriction for the initial value CS and/or Comb offset since SRS resource is determined based on CS value and Comb offset value after hopping and they are determined not only hopping value but also the initial configured value. To eliminate the interference between SRS with CS and/or Comb offset and legacy SRS, we propose the restriction with CS and/or Comb offset value for SRS port 0 after CS and/or Comb offset hopping in the restricted subset. |
| New H3C | - Agree with FL |
| Nokia, NSB | - Agree with FL. |
| InterDigital | - Agree with FL. |
| Ruijie | - Agree with FL. |
| LGE | - Agree with FL. |
| Fujitsu | - Agree with FL. |
| CATT | - Agree with FL’s assignment. |
| Xiaomi | - Agree with FL. |
| vivo | - Agree with FL. |
| Sharp | - Agree with FL. |
| Apple | - Agree with FL |

# SRS enhancements targeting 8 Tx operation

## 8T8R downgrading

CMCC and NTT DOCOMO proposed that downgrading configuration for 8T8R can be considered. Their proposal and TP are as follows:

- Support to report downgrading configuration(s) for antenna switching SRS, via FG for 8Tx SRS with antenna switching usage

  - Supported configuration(s) can be: {t1r1, t2r2, t4r4} on top of basic configuration t8r8

  - Details can be left to UE feature session

6.2.1.2	UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R, or 't1r1-t2r2-t4r4-t8r8' for 1T=1R/2T=2R/4T=4R/8T=8R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R):

In my view, this should be quite important and deserve discussion. However, before we propose the TP, technical agreement should be reached first. So the following proposal is suggested. If the proposal is agreed, it will significantly facilitate UE features discussion and the 214 TP discussion. The detailed TP may also be considered jointly with other 8T8R UE capabilities TPs; see Sec. 3.3.1.

Proposal 3.1: Support to report downgrading configuration(s) from {t1r1, t2r2, t4r4} for UE supporting 8T8R (t8r8) configuration.

| Company | View |
| --- | --- |
| Samsung | - Support to report downgrading configurations including 8T8R, but detailed possible reported values can be discussed in UE feature session, since bitmap based full flexible downgrading configuration reporting is adopted in Rel-17, including xTxR. |
| Google | - OK |
| Huawei, HiSilicon | - Share similar view with Samsung. |
| ZTE | - Support to report downgrading configuration(s) for 8T8R UE. This proposal is also related to 3.3.1 UE capability. We prefer to introduce a new UE capability supportedSRS-TxPortSwitchBeyond4Rx-r18 ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't8r8' for 8T8R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R), and UE can indicate supported downgrading configuration(s) via a bitmap. |
| OPPO | - Support the proposal and prefer a new UE capability. |
| Lenovo | - Support to report downgrading configurations including 8T8R and detail values for degraded configuration can be discussed in UE feature session. |
| New H3C | - Share the same view with Samsung. |
| NTT DOCOMO | - To clarify, we agree details of reporting should be left to UE feature session, which is what RAN1 did in previous releases. |
| Ruijie | - Support Proposal 3.1. |
| QC | - Same view as ZTE |
| Fujitsu | - Agree with other companies that this could be discussed in UE feature session. |
| CATT | - Similar view as Samsung, the issue can be discussed in UE feature session. |
| Xiaomi | - Support in principle, details can be further discussed in UE feature session |
| vivo | - Support to discuss in UE feature session. |
| Sharp | - OK |
| Apple | - We support the proposal |
| FL | - Most companies are supportive to the main principle of this proposal, and some prefer to work on the details in UE features. We can try to reach a high-level agreement in the SRS session and discuss further details in UE features session. Please check the following updated proposal. Proposal 3.1A: Support to report downgrading configuration(s) from {t1r1, t2r2, t4r4} for UE supporting 8T8R (t8r8) configuration. - Downgrading configuration(s) can be t1r1, t2r2, t4r4, and combination(s) of them. - Details to be decided in UE features design. |

## Equal power allocation on TDMed s symbols

Google, Huawei, HiSilicon, OPPO, and Qualcomm proposed to ensure that the s OFDM symbols used for TDMed 8 ports have equal power allocation via proper power scaling. Their proposals and TPs are as follows:

- When a TDMed 8-port SRS resource overlaps with higher priority uplink transmission(s) in different CC in time domain, align the SRS transmission power on s symbols of the SRS resource.

- Support to perform the transmission power scaling for the TDM based SRS per resource when it overlaps with another uplink signal.

- TP1 for clause 7.5 in TS 38.213:

5 Prioritizations for transmission power reductions

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i. If the UE transmits SRS on multiple SRS resources according the XYZ [6, TS 38.214], the UE allocates power so that all REs of the SRS transmission have same power. If the UE is provided ports8tdm for an SRS resource with 8 ports in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, the UE allocates power so that the SRS transmissions on s symbols of the SRS resource have same power.

- TP2 for clause 7.3 in TS 38.213:

7.3 	Sounding reference signals

For SRS,

-	if a UE is provided tdm for an SRS resource with 8 ports in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, the UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports on each symbol for SRS transmission, and the same transmit power is expected for the s symbols of an SRS resource.

This issue requires discussion. Generally this seems to be a reasonable requirement. In some sense it is a bit similar to TDMed partial dropping issue. It may also have the timeline issue as the partial dropping. That is, the current power scaling is on a per-symbol basis, but if it is to be changed to be across more than 1 symbol, the UE may not be able to change the power on a first symbol after the transmission parameters have been determined for the first symbol, when the power scaling is needed on the second symbol (and hence also needed on the first symbol according to the proposals). If this problem can be resolved, then the equal power requirement can be decided. Anyway this is related to some implementation issues. Please discuss.

- Question 1: Is equal power allocation on s TDMed symbols always feasible, e.g., is there a timeline issue in some cases? If yes, can it be resolved? If no, why not?

- Question 2: Should equal power allocation on s TDMed symbols be supported?

| Company | View |
| --- | --- |
| Samsung | - Q1. We agree with FL’s assessment that this issue is similar with TDMed partial dropping issue where timeline issue is involved. We are open to discuss, but one of possible way to go is having similar treatment with TDMed partial dropping issue. - Q2. Having equal power allocation on s TDMed symbols is always desirable, but considering some elaborations from other companies, there will be cases that partial symbol is overlapped with other UL channel/signal and power difference between different symbols can be occurred. |
| Google | - Q1: At least within s symbols should be feasible, since they are consecutive symbols. This does not require additional timeline. - Q2: Yes. |
| Huawei, HiSilicon | - Q1: FL raises a good point. While although we agree this case may face timeline issue similar to TDMed partial dropping case, given that now we only need to adjust the transmission power rather than cancel the transmission (which is relevant to waveform generation), we think the timeline issue can be handled by implementation. - Q2: Yes, the unguaranteed coherency and power difference among SRS ports led by unequal power allocation will severely impact the accuracy of UL/DL precoder and consequently incur disastrous UL/DL performance degradation. |
| ZTE | - Q1: We agree with Samsung that the timeline issue is involved if equal power allocation on the s TDMed symbol is required. We also prefer to handle this issue as we did with SRS dropping, and no spec change is needed. - Q2: We believe equal power allocation on the s TDMed symbols is beneficial. However, we think this issue can be handled by gNB scheduling. In other words, partially overlapping between the s TDMed SRS symbols and other UL channel/signal should be avoided. |
| OPPO | - Q1: Intend to agree with Google that no additional timeline is needed. The transmission power of a SRS resource is determined at the same time, not per symbol in our understanding. - Q2: Yes. |
| Lenovo | - Q1: Yes, similar way as TDMed partial dropping can be used and we prefer no spec. change. - Q2: Yes, it is desirable. |
| New H3C | - Q1: open to discuss about this issue - Q2:Yes |
| NTT DOCOMO | - Q1: Yes, with no specification effort. - Q2: Yes. |
| Nokia, NSB | - Q1: We agree with FL’s assessment. We are open to discuss. - Q2: Similar view as Samsung. |
| InterDigital | - Q1: Agree with OPPO and Google. - Q2: Yes. |
| Ruijie | - Q1: Yes. Can be resolved by implementation and no spec impact. - Q2: Yes. |
| QC | - Q1: We tend to agree power is determined per SRS occasion, not per SRS OFDM symbol. When gNB issue power control command, it should already guarantee timeline is not a problem for UE to determine the SRS transmission power. But we are open to discuss if that is not the case. - Q2: Yes, equal power is desirable. |
| LGE | - Q1: It seems that there is similar timeline issue as the partial dropping as FL mentioned. - Q2: Open to discuss. However, as Samsung mentioned, already there is different power allocation across different symbols for a UL channel/signal due to the partial overlapping in legacy. So, it seems that it can be up to gNB implementation if the legacy spec works well. Also, it should be firstly investigated whether there is performance loss of channel estimation or not if different power allocation on s TDMed symbols. |
| Fujitsu | - Q1: Same view as QC that the power is determined per SRS transmission occasion, not per symbol. - Q2: Yes. |
| CATT | - Q1: Yes. The potential time issue can be dealt with proper implementation. - Q2: Yes. |
| Xiaomi | - Q1: equal power allocation may not be feasible when collision with other UL channels/signals happen similar to partial overlapping, if it is determined per SRS occasion, then the timeline issue should be guaranteed for UE implementation. - Q2: Yes, we think it should be always ensured. |
| vivo | - Q1: Share the same view with Google that there is no timeline issue in this case. It is up to UE implementation and no spec impact. - Q2: Yes, for the performance of SRI determination and precoding, equal power allocation on s TDMed symbols is desired. |
| Sharp | - Q1: Yes. In our view, there is no timeline issue because the transmission power is determined per SRS resource. Therefore, no spec impact is needed. - Q2: Yes. |
| Apple | - Q1 We are open for further discussion. In legacy, we have more than 1 SRS symbol per transmission which seems to have the similar issue if there is transmission power problem. - Q2 Equal power allocation is preferred |
| FL | - All companies prefer to have equal power allocation on the s TDMed symbols, though some believe no additional spec impact is required and others prefer to ensure this via proper spec effort. We can try the following proposal. - Proposal 3.2: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s = 2, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2}, …, {1, 2}} (totally m/s groups of {1, 2}), the UE allocates the same transmission power on the 2 OFDM symbols within a group of {1, 2}. - If the UE reduces power on one OFDM symbol within a group of {1, 2}, it also reduces power on the other OFDM symbol within the group of {1, 2}. |

### ROUND2

We will continue to discuss the following proposal.

- Proposal 3.2A:

- For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s = 2, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2}, …, {1, 2}} (totally m/s groups of {1, 2}), the UE allocates the same transmission power on the 2 OFDM symbols within a group of {1, 2}.

- If the UE reduces power on one or both OFDM symbols within a group of {1, 2}, it also reduces allocates power equal to the lower power of the 2 OFDM symbols to both on the other OFDM symbols within the group of {1, 2}.

Some relevant information:

TS38.213:

7.3	Sounding reference signals

For SRS,

-	if a UE is provided tdm for an SRS resource with 8 ports in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, the UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports on each symbol for SRS transmission.

-	else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS.

7.3.1	UE behaviour

If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasion i as

[dBm]

where,

-	PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i

…

7.5 Prioritizations for transmission power reductions

For single cell operation with two uplink carriers or for operation with carrier aggregation, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cells in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cells in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i. If the UE transmits SRS on multiple SRS resources according the XYZ [6, TS 38.214], the UE allocates power so that all REs of the SRS transmission have same power.

-	PRACH transmission on a candidate cell, if any, as described in Clause 21

-	PRACH transmission on the PCell

-	PUCCH or PUSCH transmissions with larger priority index

-	For PUCCH or PUSCH transmissions with same priority index

-	PUCCH transmission with HARQ-ACK information, and/or SR, and/or LRR, or PUSCH transmission with HARQ-ACK information of the priority index

-	PUCCH transmission with CSI or PUSCH transmission with CSI

-	PUSCH transmission without HARQ-ACK information of the priority index or CSI and, for Type-2 random access procedure, PUSCH transmission on the PCell

-	If the UE is configured with prioSCellPRACH-OverSP-PeriodicSRS-r17

-	Aperiodic SRS transmission or PRACH transmission on a serving cell other than the PCell

-	Semi-persistent and/or periodic SRS transmission

-	otherwise,

-	SRS transmission, with aperiodic SRS having higher priority than semi-persistent and/or periodic SRS, or PRACH transmission on a serving cell other than the PCell

…

It seems that the companies are mostly aligned in terms of equal power allocation is preferable. How to interpret the existing specs, whether this is already enabled, or whether this can be achieved by gNB implementation, etc., can be further discussed.

Views can be provided and discussed in Tuesday offline.

| Company | View |
| --- | --- |
| Sharp | - We think no spec change is needed. - For the current spec, it is determined per SRS resource (i.e., per transmission occasion i) that whether the transmission power is reduced or not. Therefore, there is no timeline issue even if other UL channels are overlapped within the SRS resource. - Furthermore, the UE allocates the transmission power such that the total of transmission power is smaller than or equal to PCMAX per OFDM symbol. However, it is up to UE implementation that how much the UE reduces the transmission power for SRS. For this reason, the UE can implicitly allocate the same power within s OFDM symbols according to the legacy spec. Therefore, we think no spec change is needed. |
| FL | - Based on offline discussions, we can only conclude that there is no consensus on whether/how to ensure equal power allocation on the TDMed symbols. |

## TS38.214 TPs

### 8T8R UE capabilities

Ericsson, Futurewei, and Samsung provided TPs for the new 8T8R capability. A common part of these TPs is that the existing “'t8r8' for 8T8R” should be deleted as the current place for this phrase is for xRyR with x < y, but exactly how to describe the new capability is a bit different in these TPs:

TP1:

6.2.1.2	UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R) or the UE may be configured with the following configurations depending on the indicated UE capability newUECapabilitySupporting8T8R (‘t8r8’ for 8T=8R):

<unchanged text is omitted>

TP2:

6.2.1.2	UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't8r8' for 8T8R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R):

<unchanged text is omitted>

Exactly how to describe the new capability may be related to UE features outcome. And the antenna configuration downgrading, if agreed, can also be incorporated here. An initial proposal is provided below.

In addition, vivo and ZTE proposed to remove [srs-AntennaSwitching2SP-1Periodic8T8R] as this may be included in srs-AntennaSwitching2SP-1Periodic instead of having a new capability. This is related to UE features design outcome, and it can also be discussed together here.

Proposal 3.3.1: Adopt the text proposal for TS38.214 on 8T8R UE capability (for the two parts of blue texts, down select only one):

6.2.1.2 UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R[, or 't1r1-t2r2-t4r4-t8r8' for 1T=1R/2T=2R/4T=4R/8T=8R]) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't8r8' for 8T8R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R) or the UE may be configured with the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R] (‘t8r8’ for 8T=8R):

< Unchanged text is omitted>

-	For 1T=1R, 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, 4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching2SP-1Periodic8T8R], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, 4, or 8 or

< Unchanged text is omitted>

- Additional information

  - Reason for change: New UE capability is introduced for 8T8R support. The existing 8T8R description is out of place or may not apply. The parameter srs-AntennaSwitching2SP-1Periodic8T8R can be covered in srs-AntennaSwitching2SP-1Periodic. [Downgrading configuration for 8T8R is not supported.]

  - Summary for change: Remove 8T8R from existing description. Add a description of new UE capability for 8T8R. Remove srs-AntennaSwitching2SP-1Periodic8T8R. [Add 8T8R downgrading configuration.]

  - Consequences if not approved: Incorrect/incomplete specification for 8T8R.

  - TP is for TS38.214 v18.0.0.

| Company | View |
| --- | --- |
| Samsung | - Support the 1st red deleted part “'t8r8' for 8T8R,” and the last blue text “or the UE may be configured with the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R] (‘t8r8’ for 8T=8R):” only, since it is obvious that 8T8R will be reported by a new UE feature. - But other than that, the following three parts, - the 2nd red part “[, or 't1r1-t2r2-t4r4-t8r8' for 1T=1R/2T=2R/4T=4R/8T=8R]” for downgrading, - the 1st blue text “'t8r8' for 8T8R,”, - the last red part “or [srs-AntennaSwitching2SP-1Periodic8T8R]” for 2SP+1P, - the intention of above texts are to reuse legacy UE capability, then we prefer to define a new UE capability for corresponding feature (e.g., downgrading or 2SP+1P). |
| Google | - OK |
| Huawei, HiSilicon | - Share similar view with Samsung. |
| ZTE | - Firstly, we support to remove the first red part “t8r8' for 8T8R”. Moreover, we prefer to introduce a new UE capability supportedSRS-TxPortSwitchBeyond4Rx-r18 ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't8r8' for 8T8R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R). The new UE capability includes all the components of legacy UE capability supportedSRS-TxPortSwitchBeyond4Rx-r17 and a new component 't8r8' for 8T8R. It is equivalent to combine the legacy UE capability supportedSRS-TxPortSwitchBeyond4Rx-r17 with the new UE capability newUECapabilitySupporting8T8R proposed by Samsung. In this way, the new Rel-18 UE capability would support better backward compatibility. - Secondly, we support to remove “or [srs-AntennaSwitching2SP-1Periodic8T8R]”, and reuse the legacy UE capability srs-AntennaSwitching2SP-1Periodic to indicate the 2SP-1P capability for 8T8R. We do not see the necessity of introducing a separate capability srs-AntennaSwitching2SP-1Periodic8T8R, because supporting 2 SP and 1 P SRS resource set for 8T8R does not increase much UE complexity. |
| OPPO | - We have similar view as Samsung. New UE feature is expected for 8T8R. |
| Lenovo | - We are fine with the proposal. - For two parts of blue texts, we prefer the later one with new UE capability for ‘t8r8’ for 8T=8R. |
| New H3C | - Share this similar view with Samsung. |
| NTT DOCOMO | - Agree with Samsung. |
| Ruijie | - Agree with Samsung. |
| QC | - On 't8r8' for 8T8R, we support remove it. - On [, or 't1r1-t2r2-t4r4-t8r8' for 1T=1R/2T=2R/4T=4R/8T=8R], we don’t support. On downgrading, we have same view as ZTE. We prefer to introduce a new UE capability supportedSRS-TxPortSwitchBeyond4Rx-r18, duplicate all legacy 11 entries in Rel-17 and add 't8r8' for 8T8R. It is equivalent to combine the legacy UE capability supportedSRS-TxPortSwitchBeyond4Rx-r17 with the new UE capability newUECapabilitySupporting8T8R proposed by Samsung. But we fill ZTE’s proposal is cleaner. - On [srs-AntennaSwitching2SP-1Periodic8T8R], we don’t support it. Adding 8T8R to 2SP-1P would require some additional UE complexity. We agreed to add a new UE capability for this purpose in last RAN1 meeting. We don’t agree to revert previous agreement. |
| Fujitsu | - Should the change in the 1st paragraph be discussed together with Section 3.1? |
| CATT | - We support to remove the first red part “t8r8' for 8T8R” and the second red part “or [srs-AntennaSwitching2SP-1Periodic8T8R]”. Since legacy parameter srs-AntennaSwitching2SP-1Periodic can be reused for 8T8R, new parameter [srs-AntennaSwitching2SP-1Periodic8T8R] is not needed. - Besides, we support to add the second blue part “or the UE may be configured with the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R] (‘t8r8’ for 8T=8R)” since new UE capability is needed to indicate the supportive of 8T8R. |
| Xiaomi | - We share similar view with ZTE and QC on the approach to support the downgrading of 8T8R. |
| vivo | - Share the similar view with Samsung, excluding the second red part “or [srs-AntennaSwitching2SP-1Periodic8T8R]”. - For srs-AntennaSwitching2SP-1Periodic, it is used to determine the maximum number of SP and P-SRS resource sets. Even for SRS for 8T8R based on TDM which would occupy more time-domain resource, the maximum number of SP and P-SRS resource sets can still be reused up to UE capability. Therefore, we think srs-AntennaSwitching2SP-1Periodic8T8R should be removed. |
| Apple | - We can wait until we have UE feature discussion finished - For or [srs-AntennaSwitching2SP-1Periodic8T8R], we do not see the need to revert the agreement in the previous meeting. |
| FL | - Most companies are generally fine with the direction of this proposal, but whether to use a new or existing capability needs further discussion. Given these to-be-decided issues, I suggest we revisit after the relevant UE features outcome is out. |

#### ROUND2

We will continue to discuss how to best capture the UE capabilities related to 8T8R.

The first issue is whether to reuse srs-AntennaSwitching2SP-1Periodic or introduce a new srs-AntennaSwitching2SP-1Periodic8T8R.

Offline consensus

Proposal 3.3.1-1A: Introduce a new srs-AntennaSwitching2SP-1Periodic8T8R.

Another issue is about how to capture the new UE capability for 8T8R (including possible downgrading) based on agreed UE features design.

Offline consensus

Proposal 3.3.1-2A: Adopt the text proposal for TS38.214 on 8T8R UE capability:

6.2.1.2 UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R) or the UE may be configured with the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R]:

- Additional information

  - Reason for change: New UE capability is introduced for 8T8R support.

  - Summary for change: Remove 8T8R from existing description. Add a description of new UE capability for 8T8R.

  - Consequences if not approved: Incorrect/incomplete specification for 8T8R.

  - TP is for TS38.214 v18.0.0.

| Company | View |
| --- | --- |

### Divisibility of Ns by S*R

CATT proposed to capture that the number of SRS symbols should be divisible by S*R when TDM is configured:

When the higher layer parameter [tdm] is configured for the SRS resource, Ns should be divisible by sR, where the quantity s=2 and the quantity R is the repetition factor given by the field repetitionFactor.

This is a correct statement which was discussed in previous meetings; companies believe it is a sufficiently clear common understanding and no agreement is needed.

The divisibility of Ns by R, when TDM is not configured, is explicitly specified in TS38.214, clause 6.2.1.1. Contents similar to those in clause 6.2.1.1 and for TDM are only generally described in TS38.214 while referring to details in TS38.211 equations. So, to be clear, the divisibility of Ns by S*R can be included in TS38.214 clause 6.2.1.

Since the statement is true for both TDM and non-TDM, we can modify to cover both cases. A proposal is provided:

Proposal 3.3.2: Adopt the text proposal for TS38.214 on divisibility of Ns by S*R:

6.2.1 UE sounding procedure

< Unchanged text is omitted>

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet the higher layer parameter resourceMapping-r16 in SRS-PosResource indicates an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. When the SRS is configured with the higher layer parameter SRS-ResourceSet, the higher layer parameter resourceMapping-r17 in SRS-Resource indicates an SRS resource occupying NS∈1,2,4,8,10,12,14 adjacent symbols anywhere within the slot. Ns is divisible by S*R, where the quantity S=2 when [tdm] is configured and S=1 otherwise, and the quantity R is the repetition factor.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Current specification does not explicitly describe divisibility of Ns by S*R.

  - Summary for change: Add description of divisibility of Ns by S*R.

  - Consequences if not approved: Unclear specification.

  - TP is for TS38.214 v18.0.0.

| Company | View |
| --- | --- |
| Samsung | - We are fine. |
| Google | - OK |
| Huawei, HiSilicon | - Fine with the proposal. |
| ZTE | - OK. |
| OPPO | - Fine. |
| Lenovo | - Fine. |
| New H3C | - OK |
| NTT DOCOMO | - Agree. |
| Ruijie | - OK. |
| QC | - Fine with this clarification. |
| LGE | - Ok. |
| Fujitsu | - Ok |
| CATT | - Support. |
| Xiaomi | - ok |
| vivo | - Support |
| Sharp | - Support |
| Apple | - OK |

### TDM description

TP1:

CATT, Ruijie, and ZTE proposed TPs on the wording for TDM description. The main concern is that the current spec text may be misread as TDM allows only 2 symbols instead of {2, 4, …, 14} symbols.

6.2.1	UE sounding procedure

<Unrelated part omitted>

-  Support of time division mapping s (s = 2) subsets of ports of the SRS resource into the OFDM symbols occupied by the SRS resource, as defined by the higher layer parameter [tdm], where the SRS ports are distributed as defined in [4, TS 38.211] clause 6.4.1.4.2. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’.

<Unrelated part omitted>

TP2:

Samsung proposed a TP to add “consecutive” according to existing agreement and align the parameters according to TS38.331. The alignment will be separately discussed in Sec. 3.5.

6.2.1	UE sounding procedure

-  Support of time division mapping subsets of ports of the SRS resource into S consecutive symbols (S=2), as defined by the higher layer parameter [tdm]nrofSRS-Ports-n8, where the SRS ports are evenly distributed in two symbols. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’ nrofSRS-Ports-n8 is set to ‘ports8tdm’.

An initial proposal for the TDM description based on the above TPs is provided with some rewording:

Proposal 3.3.3: Adopt the text proposal for TS38.214 on TDM:

6.2.1	UE sounding procedure

< Unchanged text is omitted>

-  Support of time division mapping subsets of ports of the SRS resource into S symbols (S=2), as defined by the higher layer parameter [tdm], where the SRS ports are evenly distributed in each set of two consecutive symbols over the symbols in a slot for the SRS resource according to [4, TS 38.211] clause 6.4.1.4.2. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Insufficient description on TDM.

  - Summary for change: Add more description on TDM and reference to TS38.211.

  - Consequences if not approved: Unclear specification for SRS TDM.

  - TP is for TS38.214 v18.0.0.

| Company | View |
| --- | --- |
| Samsung | - TP1: We don’t have a strong view, but for us, it is unclear whether the original text causes ambiguity or not. - TP2: Support |
| Google | - OK |
| Huawei, HiSilicon | - Generally fine with the proposal. - But do we really need “in a slot”? |
| ZTE | - OK. |
| OPPO | - Fine. |
| Lenovo | - We are fine with both TP1 and TP2. |
| New H3C | - OK |
| NTT DOCOMO | - OK |
| Nokia, NSB | - OK |
| Ruijie | - OK. |
| QC | - TP1: we don’t see current spec broken. It looks clear enough to us. - TP2: support Samsung’s proposal to add “consecutive”. |
| LGE | - TP1: Seems the original text is fine. - TP2: Fine. |
| Fujitsu | - Ok with TP2. |
| CATT | - We prefer to adopt TP1 since it is much clearer. |
| Xiaomi | - Fine |
| vivo | - Support the FL proposal. |
| Sharp | - Support |
| Apple | - OK |

### Incorrect reference to clause number

Futurewei and ZTE pointed out an incorrect reference to a clause number. This is a straightforward correction.

Proposal 3.3.4: Adopt the text proposal for TS38.214:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

For a SRS resource, if the higher layer parameters [tdm] or [combOffsetHopping] are configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. If for a SRS resource the higher layer parameters [tdm] and [combOffsetHopping] are not configured, the UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211] and in clause 6.2.1.21.

<Unchanged text is omitted>

- Additional information

  - Reason for change: Incorrect reference to clause number.

  - Summary for change: Correct from “clause 6.2.1.2” to be “clause 6.2.1.1”.

  - Consequences if not approved: Incorrect specification for SRS.

  - TP is for TS38.214 v18.0.0.

| Company | View |
| --- | --- |
| Samsung | - Ok. |
| Google | - OK |
| Huawei, HiSilicon | - Support the proposal. |
| ZTE | - OK. |
| OPPO | - OK |
| Lenovo | - Fine. |
| New H3C | - OK |
| NTT DOCOMO | - OK |
| Nokia, NSB | - OK |
| Ruijie | - OK. |
| QC | - Fine with this editorial change. |
| LGE | - Ok. |
| Fujitsu | - Ok |
| Xiaomi | - ok |
| vivo | - Support |
| Sharp | - OK |
| Apple | - OK |

### Other proposals for TS38.214

The following proposals are also for 8-port SRS for 214. Companies’ views are appreciated.

- TP 1: Lenovo proposed to add description of the number of symbols for the resource mapping for 8-port SRS with and without TDM.

  - This should already be covered in the 214 TP agreed in the last meeting which refers to 211. So no change is needed.

| Company | View |
| --- | --- |
| Samsung | - We agree with FL’s assessment. |
| Google | - Agree with FL |
| Huawei, HiSilicon | - Agree with FL’s assessment. |
| ZTE | - Agree with FL. |
| OPPO | - Agree with FL. |
| Lenovo | - For this TP, we want to describe the number of symbols for the resource mapping for 8-port SRS with and without TDM. It is similar as the issue discussed in section 3.3.2 but described in a different way. Thus, we think it can be merged into the discussion in the section 3.3.2. And we are fine with the TP as the proposal in section 3.3.2. |
| New H3C | - Agree with FL |
| Ruijie | - Agree with FL and no change is needed. |
| QC | - Agree with FL. It seems no change is needed. |
| LGE | - Agree with FL. |
| Fujitsu | - Agree with FL |
| CATT | - Agree with FL’s assessment. |
| vivo | - Agree with FL |
| Apple | - Agree with FL |

## TS38.211 TPs

### Description of repetition factor with TDM

CATT and ZTE proposed to correct the description of repetition factor when TDM can be configured:

The quantity s is given by s=2 if the higher-layer parameter nrofSRS-Ports-n8 equals ‘ports8tdm’, otherwise s=1. The quantity R≤NsymbSRSsNsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

This seems reasonable, as when s = 2, R can be at most half of the number of SRS symbols. Note that the otherwise part “otherwise R=NsymbSRS” does not need to be changed, as in the case of “otherwise”, s is always 1. This is because per TS38.331, repetitionFactor shall always be configured for SRS resources (except for positioning SRS), and hence the otherwise part is only for positioning which has only 1 port and TDM does not apply.

Proposal 3.4.1: Adopt the text proposal for TS38.211 on repetition factor and TDM:

6.4.1.4.3	Mapping to physical resources

< Unchanged text is omitted>

If bhop<BSRS, frequency hopping is enabled and the frequency position indices nb are defined by

nb=4nRRCmSRS,b mod Nbb≤bhopFbnSRS+4nRRCmSRS,b mod Nbotherwise

where Nb is given by Table 6.4.1.4.3-1,

and where Nbhop=1 regardless of the value of Nb. The quantity nSRS counts the number of SRS transmissions. For the case of an SRS resource configured as aperiodic by the higher-layer parameter resourceType, it is given by nSRS=l'sR within the slot in which the NsymbSRS symbol SRS resource is transmitted. The quantity s is given by s=2 if the higher-layer parameter nrofSRS-Ports-n8 equals ‘ports8tdm’, otherwise s=1. The quantity R≤NsymbSRSsNsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Current description of repetition factor does not cover TDM case.

  - Summary for change: Replace ‘R≤NsymbSRS’ with ‘R≤NsymbSRS/s’.

  - Consequences if not approved: Incorrect specification of repetition factor for TDM.

  - TP is for TS38.211 v18.0.0.

| Company | View |
| --- | --- |
| Samsung | - Ok. |
| Google | - OK |
| Huawei, HiSilicon | - Support the proposal. |
| ZTE | - OK. |
| OPPO | - OK. |
| Lenovo | - Fine. |
| New H3C | - OK |
| NTT DOCOMO | - OK |
| Nokia, NSB | - OK |
| Ruijie | - OK. |
| QC | - Fine with the proposal. |
| LGE | - Ok. |
| Fujitsu | - Ok |
| CATT | - OK |
| Xiaomi | - OK |
| vivo | - Support |
| Sharp | - Support |
| Apple | - OK |

## Description related to nrofSRS-Ports-n8

Samsung pointed out that, based on the latest TS38.331 running CR, nrofSRS-Ports does not cover the cases of 8-port SRS, and nrofSRS-Ports-n8 should be referred to, and TDM is configured based on:

nrofSRS-Ports-n8-r18        ENUMERATED {ports8, ports8tdm}            OPTIONAL,   -- Need R

This will lead to corresponding changes in RAN1 specs.

The TPs proposed by Samsung is as follows:

Clause 6.4.1.4.1 in TS38.211

An SRS resource is configured by the SRS-Resource IE or the SRS-PosResource IE and consists of

-	NapSRS∈1,2,4,8 antenna ports pii=0NapSRS-1, where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 if configured, otherwise NapSRS=1, and pi=1000+i when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet not set to 'nonCodebook', or determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet set to 'nonCodebook'

Clause 7.3 in TS38.213

For SRS,

-	if a UE is provided nrofSRS-Ports-n8 set to ‘ports8tdm’tdm for an SRS resource with 8 ports in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, the UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports on each symbol for SRS transmission.

-	else, a UE splits a linear value PSRS,b,f,c(i,qs,l) of the transmit power PSRS,b,f,c(i,qs,l) on active UL BWP b of carrier f of serving cell c equally across the configured antenna ports for SRS.

Clause 6.2.1 in TS38.214

-  Support of time division mapping subsets of ports of the SRS resource into S consecutive symbols (S=2), as defined by the higher layer parameter [tdm]nrofSRS-Ports-n8, where the SRS ports are evenly distributed in two symbols. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’ nrofSRS-Ports-n8 is set to ‘ports8tdm’.

-	Number of SRS ports, as defined by the higher layer parameter nrofSRS-Ports or nrofSRS-Ports-n8 and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Portsthe number of SRS ports is 1.

CATT proposed to change the parameter name related to TDM in the current TS38.211. However, the current TS38.211 is aligned with the latest RAN2 running CR for TS38.331, so no change is needed.

In general, the discussions on the number of SRS ports may be seen as an alignment issue based on RAN2 inputs, which is necessary but can be largely left to the editors. In addition, there are multiple places in RAN1 specs (211/213/214, and more than those changed in the above TPs) using nrofSRS-Ports, so likely all of them need to be updated. Companies’ views can be provided.

| Company | View |
| --- | --- |
| Samsung | - Support |
| Google | - OK |
| Huawei, HiSilicon | - Fine with the proposal. |
| ZTE | - OK. |
| OPPO | - OK. |
| OPPO | - Fine. |
| New H3C | - OK |
| NTT DOCOMO | - OK |
| Nokia, NSB | - OK |
| Ruijie | - OK. |
| QC | - We are fine with these editorial changes, or leave them to editors to handle. |
| Fujitsu | - Ok |
| CATT | - Support FL’s assessment. The change is necessary but can be left to editors. |
| Xiaomi | - Fine with the corrections. |
| vivo | - Support |
| Apple | - OK |
| FL | - Based on the inputs, I suggest we leave it for the editors for now, and discuss further if needed. |

## Other proposals for 8Tx SRS

Several companies provided other discussion points. Again, some very minor editorial / RRC parameter name related TPs can be discussed later during the CR review phase and are not included here.

DP 1: Not support TDM if the UE cannot transmit at PCmax on a symbol

  - Supported by: OPPO

  - This was already ‘unofficially’ excluded per chairman’s request. May be discussed in UE feature/capability session.

DP 2: New PCMAX,f,cSRS(i) for TDMed 8-port SRS when the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports

  - Supported by: Qualcomm

  - This was already ‘unofficially’ excluded per chairman’s request.

DP 3: 2 or 4 SRS resource sets for NCB

  - Supported by: Xiaomi

  - Requires discussion and a new agreement. Companies can share their views.

DP 4: For Fully coherent UE, the SRS ports mapped onto non-consecutive OFDM symbols should not be utilized for the next PUSCH transmission if the SRS ports do not meet the requirements specified in clause 6.4D.4 of TS 38.101-1/2.

  - Supported by: KDDI

  - This is related to partial dropping which we had extensive discussions in the last meeting. It seems no further discussion is necessary.

DP 5: Partial dropping SRI issue:

  - For a UE’s latest TDMed SRS transmission is partially dropped, the UE shall assume that the indicated SRI, TPMI and TPC command for PUSCH transmission are corresponding to the latest SRS resource where all SRS ports are transmitted without partial dropping or with SRS repetition and partial dropping. The UE can skip the remaining SRS transmission if any of SRS ports is not transmitted at all in the transmission occasion.

  - If phase coherency cannot be kept between Subset#0 of SRS ports {1000,1001,1004,1005} and Subset#1 of SRS ports {1002,1003,1006,1007}, the identification of the SRS resource indicated by SRI should be modified.

  - Supported by: Nokia, Nokia Shanghai Bell, Sharp

  - We had extensive discussions in the last meeting. It seems no further discussion is necessary.

DP 6: Sharp: R value when TDM is configured but R is not configured

  - Supported by: KDDI

  - This was discussed in the last meeting and no change is needed. See also Sec. 3.4.1.

DP 7: Open to discuss further on the coherency issue of TDMed SRS dropping based on RAN4’s reply.

  - Supported by: Xiaomi

  - Should be fine but no action is needed for now.

Views can be provided for the above discussion points, and details / proposals can also be included.

| Company | View |
| --- | --- |
| Samsung | - We agree with FL’s assessments on all DP1~DP7. - For DP3, since spec is not broken and considering maintenance phase, we think it is not necessary. |
| Google | - We support to discuss DP1 and DP2. We failed to see the necessity for other DPs. |
| Huawei, HiSilicon | - DP1: Agree with FL’s assessment. - DP2: Agree with FL’s assessment. - DP3: Open to discuss. - DP4: Not support. As long as the SRS ports are mapped onto OFDM symbols within a SRS resource, the phase continuity can be achieved. - DP5: Support. - DP6: Agree with FL’s assessment. - DP7: Open to discuss. |
| ZTE | - We agree with FL’s assessments on DPs 1~7. We do not see the necessity of DPs 1~3, and 6. We can further discuss the dropping issue for TDMed SRS based on RAN4’s reply. |
| Ruijie | - Agree with FL’s assessments on all DP1~DP7. |
| QC | - For DP1/2: Our view is it is better to continue discuss this. If spec does not support TDMed SRS for a UE cannot transmit at PCmax on a symbol, it is fine from that UE perspective. But is this a good conclusion for NW? We don’t think so. Because NW would like to enable/disable TDMed SRS feature for all UEs in a cell (to facilitate SRS multiplexing across UEs), if just because some UE cannot support PCmax on a symbol, NW has to disable TDMed SRS feature for all UEs in the cell. This sounds like an unfortunate outcome. DP2 or other solutions can fix this issue. Anything, the above are just some thoughts for the group to consider. |
| CATT | - Agree with FL’s assessment. We can further discuss the dropping rule for TDMed SRS when RAN4’s reply is available. We don’t see the necessity of discuss other DPs. |
| Sharp | - DP4: - We think a discussion point is that whether the phase continuity between TDMed-SRS transmissions mapped on non-consecutive OFDM symbols within an SRS resource can be kept or not. - According to Rel-17 CovEnh, conditions to keep both phase continuity and power consistency were discussed. Furthermore, the conditions were defined in R1-2106423 and were captured in TS38.214: ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS … Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols for normal cyclic prefix or exceeds 11 symbols for extended cyclic prefix. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1.. - If the conditions for DMRS bundling are applicable to channel estimation of SRS, the gNB wants to estimate the combined channel within the SRS resource, and the gNB could not compensate the phase continuity and power consistency, DP4 is essential. |

# Conclusions

## For Monday Online

Proposals for new technical discussion/agreement:

Proposal 3.1A: Support to report downgrading configuration(s) from {t1r1, t2r2, t4r4} for UE supporting 8T8R (t8r8) configuration.

  - Downgrading configuration(s) can be t1r1, t2r2, t4r4, and combination(s) of them.

  - Details to be decided in UE features design.

- Proposal 3.2A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s = 2, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2}, …, {1, 2}} (totally m/s groups of {1, 2}), the UE allocates the same transmission power on the 2 OFDM symbols within a group of {1, 2}.

- If the UE reduces power on one or both OFDM symbols within a group of {1, 2}, it also reduces allocates power equal to the lower power of the 2 OFDM symbols to both on the other OFDM symbols within the group of {1, 2}.

Proposal for minor corrections that requires minor discussion:

Proposal 3.3.3: Adopt the text proposal for TS38.214 on TDM:

6.2.1	UE sounding procedure

< Unchanged text is omitted>

-  Support of time division mapping subsets of ports of the SRS resource into S symbols (S=2), as defined by the higher layer parameter [tdm], where the SRS ports are evenly distributed in each set of two consecutive symbols over the symbols in a slot for the SRS resource according to [4, TS 38.211] clause 6.4.1.4.2. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Insufficient description on TDM.

  - Summary for change: Add more description on TDM and reference to TS38.211.

  - Consequences if not approved: Unclear specification for SRS TDM.

  - TP is for TS38.214 v18.0.0.

Proposals for minor corrections that have received no concern:

Proposal 2.1.1: Adopt the text proposal for TS38.211 on comb offset hopping symbol indexing notation corrections:

6.4.1.4.3 Mapping to physical resources

< Unchanged text is omitted>

The frequency-domain starting position k0(pi) is defined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS

where

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+fcoh(nf,ns,fμ,l'l'') mod KTC

< Unchanged text is omitted>

The quantity fcohnf,ns,fμ,l'' is given by

-	if the higher-layer parameter combOffsetHopping is not configured:

fcohnf, ns,fμ,l''=0

-	if the higher-layer parameter combOffsetHopping is configured:

fcohnf ,ns,fμ,l'l''= 
scohSRSm=07c8nf mod 128Nslotframe,μNsymbslot+ns,fμNsymbslot+l0+l''+m2mmod ncohSRS

< Unchanged text is omitted>

- Additional information

  - Reason for change: Symbol indexing notation not updated in some places.

  - Summary for change: Update the indexing from fcohnf, ns,fμ,l' to fcohnf, ns,fμ,l''.

  - Consequences if not approved: Incorrect specification for comb offset hopping equation.

  - TP is for TS38.211 v18.0.0.

Proposal 2.1.2: Adopt the following text proposal for clause 6.4.1.4.3 in TS 38.211.

6.4.1.4.3	Mapping to physical resources

< Unchanged text is omitted>

The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDhop at the beginning of each radio frame for which nf mod 128=0, where the comb offset hopping identity nIDhop is contained in the higher-layer parameter combOffsetHopping.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Editorial error.

  - Summary for change: Editorial correction.

  - Consequences if not approved: Incorrectly referring to “comb offset hopping” as “comb hopping”.

  - TP is for TS38.211 v18.0.0.

Proposal 2.2.1: Adopt the text proposal for TS38.211 on comb offset hopping and TDM UE assumption:

6.2.1 UE sounding procedure

< Unchanged text is omitted>

-	Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When comb offset hopping is configured by the higher layer parameter [combOffsetHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching' or ‘codebook’, subject to UE capabilities, transmission comb offset(s) are updated as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the comb offset hopping, a UE can be configured with a subset of comb offsets by the higher layer parameter [combOffsetHoppingSubset], where the comb offset hopping is performed only across the comb offsets configured in the subset. The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Comb offset hopping and TDM cannot be configured simultaneously per SRS resource instead of per UE.

  - Summary for change: Add “for an SRS resource” restriction.

  - Consequences if not approved: Confusion on whether the restriction is per UE or per SRS resource.

  - TP is for TS38.211 v18.0.0.

Proposal 3.3.2: Adopt the text proposal for TS38.214 on divisibility of Ns by S*R:

6.2.1 UE sounding procedure

< Unchanged text is omitted>

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet the higher layer parameter resourceMapping-r16 in SRS-PosResource indicates an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. When the SRS is configured with the higher layer parameter SRS-ResourceSet, the higher layer parameter resourceMapping-r17 in SRS-Resource indicates an SRS resource occupying NS∈1,2,4,8,10,12,14 adjacent symbols anywhere within the slot. Ns is divisible by S*R, where the quantity S=2 when [tdm] is configured and S=1 otherwise, and the quantity R is the repetition factor.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Current specification does not explicitly describe divisibility of Ns by S*R.

  - Summary for change: Add description of divisibility of Ns by S*R.

  - Consequences if not approved: Unclear specification.

  - TP is for TS38.214 v18.0.0.

Proposal 3.3.4: Adopt the text proposal for TS38.214:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

For a SRS resource, if the higher layer parameters [tdm] or [combOffsetHopping] are configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. If for a SRS resource the higher layer parameters [tdm] and [combOffsetHopping] are not configured, the UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211] and in clause 6.2.1.21.

<Unchanged text is omitted>

- Additional information

  - Reason for change: Incorrect reference to clause number.

  - Summary for change: Correct from “clause 6.2.1.2” to be “clause 6.2.1.1”.

  - Consequences if not approved: Incorrect specification for SRS.

  - TP is for TS38.214 v18.0.0.

Proposal 3.4.1: Adopt the text proposal for TS38.211 on repetition factor and TDM:

6.4.1.4.3	Mapping to physical resources

< Unchanged text is omitted>

If bhop<BSRS, frequency hopping is enabled and the frequency position indices nb are defined by

nb=4nRRCmSRS,b mod Nbb≤bhopFbnSRS+4nRRCmSRS,b mod Nbotherwise

where Nb is given by Table 6.4.1.4.3-1,

and where Nbhop=1 regardless of the value of Nb. The quantity nSRS counts the number of SRS transmissions. For the case of an SRS resource configured as aperiodic by the higher-layer parameter resourceType, it is given by nSRS=l'sR within the slot in which the NsymbSRS symbol SRS resource is transmitted. The quantity s is given by s=2 if the higher-layer parameter nrofSRS-Ports-n8 equals ‘ports8tdm’, otherwise s=1. The quantity R≤NsymbSRSsNsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Current description of repetition factor does not cover TDM case.

  - Summary for change: Replace ‘R≤NsymbSRS’ with ‘R≤NsymbSRS/s’.

  - Consequences if not approved: Incorrect specification of repetition factor for TDM.

  - TP is for TS38.211 v18.0.0.

## Monday Online Agreements

Proposal 3.1A:

Support to report downgrading configuration(s) from {t1r1, t2r2, t4r4} for UE supporting 8T8R (t8r8) configuration.

  - Details to be decided in UE features design.

Proposal 3.3.3: Adopt the text proposal for TS38.214 on TDM:

6.2.1	UE sounding procedure

< Unchanged text is omitted>

-  Support of time division mapping subsets of ports of the SRS resource into S symbols (S=2), as defined by the higher layer parameter [tdm], where the SRS ports are evenly distributed in two consecutive symbols over the symbols in a slot for the SRS resource according to [4, TS 38.211] clause 6.4.1.4.2. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Insufficient description on TDM.

  - Summary for change: Add more description on TDM and reference to TS38.211.

  - Consequences if not approved: Unclear specification for SRS TDM.

  - TP is for TS38.214 v18.0.0.

Proposal 2.1.1:

Adopt the text proposal for TS38.211 on comb offset hopping symbol indexing notation corrections:

6.4.1.4.3 Mapping to physical resources

< Unchanged text is omitted>

The frequency-domain starting position k0(pi) is defined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS

where

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+fcoh(nf,ns,fμ,l'l'') mod KTC

< Unchanged text is omitted>

The quantity fcohnf,ns,fμ,l'' is given by

-	if the higher-layer parameter combOffsetHopping is not configured:

fcohnf, ns,fμ,l''=0

-	if the higher-layer parameter combOffsetHopping is configured:

fcohnf ,ns,fμ,l'l''= 
scohSRSm=07c8nf mod 128Nslotframe,μNsymbslot+ns,fμNsymbslot+l0+l''+m2mmod ncohSRS

< Unchanged text is omitted>

- Additional information

  - Reason for change: Symbol indexing notation not updated in some places.

  - Summary for change: Update the indexing from fcohnf, ns,fμ,l' to fcohnf, ns,fμ,l''.

  - Consequences if not approved: Incorrect specification for comb offset hopping equation.

  - TP is for TS38.211 v18.0.0.

Proposal 2.1.2:

Adopt the following text proposal for clause 6.4.1.4.3 in TS 38.211.

6.4.1.4.3	Mapping to physical resources

< Unchanged text is omitted>

The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDhop at the beginning of each radio frame for which nf mod 128=0, where the comb offset hopping identity nIDhop is contained in the higher-layer parameter combOffsetHopping.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Editorial error.

  - Summary for change: Editorial correction.

  - Consequences if not approved: Incorrectly referring to “comb offset hopping” as “comb hopping”.

  - TP is for TS38.211 v18.0.0.

Proposal 2.2.1:

Adopt the text proposal for TS38.211 on comb offset hopping and TDM UE assumption:

6.2.1 UE sounding procedure

< Unchanged text is omitted>

-	Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When comb offset hopping is configured by the higher layer parameter [combOffsetHopping] for an SRS resource in an SRS resource set with the usage configured as 'antennaSwitching' or ‘codebook’, subject to UE capabilities, transmission comb offset(s) are updated as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the comb offset hopping, a UE can be configured with a subset of comb offsets by the higher layer parameter [combOffsetHoppingSubset], where the comb offset hopping is performed only across the comb offsets configured in the subset. The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Comb offset hopping and TDM cannot be configured simultaneously per SRS resource instead of per UE.

  - Summary for change: Add “for an SRS resource” restriction.

  - Consequences if not approved: Confusion on whether the restriction is per UE or per SRS resource.

  - TP is for TS38.211 v18.0.0.

Proposal 3.3.2:

Adopt the text proposal for TS38.214 on divisibility of Ns by S*R:

6.2.1 UE sounding procedure

< Unchanged text is omitted>

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols within the last 6 symbols of the slot, or at any symbol location within the slot if resourceMapping-r16 is provided subject to UE capability, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet the higher layer parameter resourceMapping-r16 in SRS-PosResource indicates an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot. When the SRS is configured with the higher layer parameter SRS-ResourceSet, the higher layer parameter resourceMapping-r17 in SRS-Resource indicates an SRS resource occupying NS∈1,2,4,8,10,12,14 adjacent symbols anywhere within the slot. Ns is divisible by S*R, where the quantity S=2 when [tdm] is configured and S=1 otherwise, and the quantity R is the repetition factor.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Current specification does not explicitly describe divisibility of Ns by S*R.

  - Summary for change: Add description of divisibility of Ns by S*R.

  - Consequences if not approved: Unclear specification.

  - TP is for TS38.214 v18.0.0.

Proposal 3.3.4:

Adopt the text proposal for TS38.214:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

For a SRS resource, if the higher layer parameters [tdm] or [combOffsetHopping] are configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. If for a SRS resource the higher layer parameters [tdm] and [combOffsetHopping] are not configured, the UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211] and in clause 6.2.1.21.

<Unchanged text is omitted>

- Additional information

  - Reason for change: Incorrect reference to clause number.

  - Summary for change: Correct from “clause 6.2.1.2” to be “clause 6.2.1.1”.

  - Consequences if not approved: Incorrect specification for SRS.

  - TP is for TS38.214 v18.0.0.

Proposal 3.4.1:

Adopt the text proposal for TS38.211 on repetition factor and TDM:

6.4.1.4.3	Mapping to physical resources

< Unchanged text is omitted>

If bhop<BSRS, frequency hopping is enabled and the frequency position indices nb are defined by

nb=4nRRCmSRS,b mod Nbb≤bhopFbnSRS+4nRRCmSRS,b mod Nbotherwise

where Nb is given by Table 6.4.1.4.3-1,

and where Nbhop=1 regardless of the value of Nb. The quantity nSRS counts the number of SRS transmissions. For the case of an SRS resource configured as aperiodic by the higher-layer parameter resourceType, it is given by nSRS=l'sR within the slot in which the NsymbSRS symbol SRS resource is transmitted. The quantity s is given by s=2 if the higher-layer parameter nrofSRS-Ports-n8 equals ‘ports8tdm’, otherwise s=1. The quantity R≤NsymbSRSsNsymbSRS is the repetition factor given by the field repetitionFactor if configured, otherwise R=NsymbSRS.

< Unchanged text is omitted>

- Additional information

  - Reason for change: Current description of repetition factor does not cover TDM case.

  - Summary for change: Replace ‘R≤NsymbSRS’ with ‘R≤NsymbSRS/s’.

  - Consequences if not approved: Incorrect specification of repetition factor for TDM.

  - TP is for TS38.211 v18.0.0.

## Tuesday Offline Consensus

Offline consensus

Proposal 3.3.1-1A: Introduce a new srs-AntennaSwitching2SP-1Periodic8T8R.

Another issue is about how to capture the new UE capability for 8T8R (including possible downgrading) based on agreed UE features design.

Offline consensus

Proposal 3.3.1-2A: Adopt the text proposal for TS38.214 on 8T8R UE capability:

6.2.1.2 UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R) or the UE may be configured with the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R]:

- Additional information

  - Reason for change: New UE capability is introduced for 8T8R support.

  - Summary for change: Remove 8T8R from existing description. Add a description of new UE capability for 8T8R.

  - Consequences if not approved: Incorrect/incomplete specification for 8T8R.

  - TP is for TS38.214 v18.0.0.

## For Wednesday Online

Offline consensus

Proposal 3.3.1-1A: Introduce a new srs-AntennaSwitching2SP-1Periodic8T8R.

Another issue is about how to capture the new UE capability for 8T8R (including possible downgrading) based on agreed UE features design.

Offline consensus

Proposal 3.3.1-2A: Adopt the text proposal for TS38.214 on 8T8R UE capability:

6.2.1.2 UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R) or the UE may be configured with the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R]:

- Additional information

  - Reason for change: New UE capability is introduced for 8T8R support.

  - Summary for change: Remove 8T8R from existing description. Add a description of new UE capability for 8T8R.

  - Consequences if not approved: Incorrect/incomplete specification for 8T8R.

  - TP is for TS38.214 v18.0.0.

## Wednesday Online Agreements

Offline consensus

Introduce a new UE capability srs-AntennaSwitching2SP-1Periodic8T8R to extend srs-AntennaSwitching2SP-1Periodic to the case of 8T8R

Offline consensus

Proposal 3.3.1-2A: Adopt the text proposal for TS38.214 on 8T8R UE capability:

6.2.1.2 UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R) or the UE may be configured with the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R]:

- Additional information

  - Reason for change: New UE capability is introduced for 8T8R support.

  - Summary for change: Remove 8T8R from existing description. Add a description of new UE capability for 8T8R.

  - Consequences if not approved: Incorrect/incomplete specification for 8T8R.

  - TP is for TS38.214 v18.0.0.

# References

RP-223276, WID Update: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#98-e.

RP-231717, Status report for NR MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#101.

R1-2310829, SRS enhancements for TDD CJT and 8TX operation, FUTUREWEI.

R1-2310870, Maintenance of SRS enhancement for TDD CJT and UL 8Tx operation in Rel-18, Huawei, HiSilicon.

R1-2310923, Maintenance on Rel-18 SRS Enhancements, InterDigital, Inc.

R1-2310951, Maintenance on SRS enhancement targeting TDD CJT and 8 TX operation, ZTE.

R1-2311087, Maintenance on SRS enhancements, vivo.

R1-2311217, Remaining issues on SRS enhancement targeting TDD CJT and 8 TX operation, OPPO.

R1-2311318, Remaining issues on SRS enhancement, CATT.

R1-2311365, Maintenance on SRS enhancement, Lenovo.

R1-2311383, Maintenance on SRS enhancements, xiaomi.

R1-2311478, Remaining issues on SRS enhancement targeting TDD CJT and 8 TX operation, CMCC.

R1-2311569, On SRS Enhancement, Google.

R1-2311616, Remaining issues on SRS enhancement, NTT DOCOMO, INC.

R1-2311721, Maintenance on SRS enhancement targeting TDD CJT and 8 TX operation, Sharp.

R1-2311743, Maintenance of SRS enhancement for TDD CJT and 8Tx operation, Nokia, Nokia Shanghai Bell.

R1-2311834, Remaining details on SRS enhancements, Samsung.

R1-2311883, Remaining issues on SRS enhancement targeting TDD CJT and 8 TX operation, KDDI Corporation.

R1-2311929, Maintenance on SRS enhancement targeting TDD CJT and 8 TX operation, Ruijie Network Co. Ltd.

R1-2312029, SRS enhancement for TDD CJT and 8 Tx operation, Qualcomm Incorporated.

R1-2312180, Maintenance on SRS enhancements targeting TDD CJT and 8 TX operation, Ericsson.

# Appendix

## Appendix 1: Agreements from RAN1#109-e

Agreement

For SRS EVM, adopt combined relevant parts from Rel-17 SRS EVM and Rel-18 FDD CJT EVM as starting point

- Details are provided in Appendix 3 of R1-2205330 for system-level simulations

- Details are provided in Appendix 4 of R1-2205330 for link-level simulations.

Agreement

For 8 Tx SRS, a starting point of UE antenna configurations can be:

- (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or

- (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ.

- FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items.

Agreement

For SRS EVM, consider additional EVM as follows

- Realistic channel estimation based on sequence generation for SRS modelling, at least for TDD CJT SRS LLS and 8 Tx SRS LLS as baseline

- Evaluation metrics for 8 Tx SRS LLS can be MSE , BLER or throughput

- TDL-C for TDD CJT SRS LLS can be included as optional.

Agreement

Consider the scenario where there exists SRSs sent by a UE and utilized by multiple TRPs for channel estimation, and the pathlosses between the UE and the TRPs differ by at least x dB in Rel-18 SRS study

- x can be {3,6,10}, and other values can be used.

Agreement

Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement

- Randomized frequency-domain resource mapping for SRS transmission

  - E.g., further enhancements to frequency hopping, comb hopping

- Randomized code-domain resource mapping for SRS transmission

  - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence

- Randomized transmission of SRS

    - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

- Per-TRP power control and/or power control of one SRS towards to multiple TRPs

- SRS TD OCC

- Increasing the maximum number of cyclic shifts

  - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts

- Precoded SRS for DL CSI acquisition

- Enhanced signaling for flexible SRS transmission

  - E.g., dynamic update of SRS parameters

- Partial frequency sounding extensions

  - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS

- Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

  - E.g., configuration of v (sequence index within a group) per SRS resource

  - E.g., configuration of cyclic shift per SRS port per SRS resource.

- Resource mapping for SRS transmission based on network-provided parameters or system parameters

  - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index)

Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

Agreement

Study the potential enhancements for SRS of 8T8R with usage antennaSwitching.

Agreement

Study the potential enhancements for SRS for 8 Tx operation

- SRS resource(s) with 8 ports are configured for codebook-based PUSCH

- Up to 8 single-port SRS resources are configured for non-codebook-based PUSCH

Agreement

For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS,

- Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol

- For the next decision point, study

  - Whether to support 8 ports in one or multiple resources

  - Whether to support 8 ports in one or multiple OFDM symbols

  - The maximum number of SRS resource sets.

- Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

| Rel-18 SLS Assumptions for TDD CJT SRS |  |  |
| --- | --- | --- |
| Parameter |  | Value |
| Duplex, Waveform |  | TDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Outdoor OptA Companies can simulate from the following 2 layouts. 1) Outdoor (typical 57-sector, or 21-sector, SLS): OptionA: 1 TRP per sector, 3 sectors per site. N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). The N_TRP TRPs can be selected either only from the same site (intra-site - limited to 3 TRPs), or also from other sites (inter-site) - company should describe what is assumed OptionB: N_TRP co-located (at BS) panels per sector - companies describe how the panels are (azimuthally) oriented - Dense Urban (macro only) 200m ISD or Urban Macro 500m ISD 2) Indoor Hotspot: model in TS 38.802 - N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). |
| Frequency Range |  | FR1 only, 3.5GHz |
| Inter-BS (site) distance |  | Outdoor: 200m or 500m Indoor Hotspot: per TS 38.802 |
| Channel generation model |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| Antenna setup and port layouts at gNB |  | - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32,64} |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 |
| BS Tx power |  | Dense Urban or Urban Macro: - Per TRP: 44 dBm for 20MHz, 47dBm for 40MHz, 51dBm for 100MHz Indoor: per TRP 24dBm |
| BS antenna height |  | Depending on scenarios (cf. table A.2.1-1 of TS 38.802): DU (25m), UMa (25m), Indoor Hotspot (3m) |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 30kHz |
| Number of RBs |  | 52RB for 20MHz, 104RB for 40MHz, 272RB for 100MHz |
| Frame structure |  | DSUDD, or companies to state the used frame structure |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP 1 or FTP 3 with 20%, 50% or 70% traffic load |
| UE distribution |  | According to TS 38.802 - DU and UMa: 80% indoor (3km/h), 20% outdoor (30km/h) - Indoor Hotspot: 100% indoor (3km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| DL Channel estimation |  | Realistic |
| Evaluation Metric |  | DL throughput |
| Baseline for performance evaluation |  | R17 SRS design |
| SRS modeling for UL channel estimation |  | Companies to state the used SRS periodicity. Companies to state the SRS channel estimation modeling Number of ports = 2 or 4 Tx power = 23 dBm |

| Rel-18 LLS Assumptions for TDD CJT SRS |  |
| --- | --- |
| Parameter | Value |
| Scenario | N_TRP (#TRPs): 2, 3, 4 |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 40MHz, 100MHz |
| Channel model | CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| UE velocity | 3km/h |
| Antennas at UE | 1T4R, 2T4R, 4T4R |
| Antennas at gNB | 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Rank and MCS | Rank/MCS can be adaptive or fixed. |
| Evaluation metrics | MSE, BLER or throughput |
| Baseline | R17 SRS design |
| Precoding granularity | Fixed: 2, 4 or wideband for DL, wideband for UL. |
| SRS configurations | Companies to state the used SRS periodicity. Frequency hopping：Companies to state whether SRS frequency hopping is enabled and the hopping pattern if so. |
| DL SNR | Companies to state the used difference between DL SNR and UL SNR |

## Appendix 2: Agreements from RAN1#110

Agreement

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

RAN1 should strive to minimize the number of schemes supported in Rel-18

- SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

Target usage includes antenna switching, codebook/non-codebook based SRS

Agreement

For 8 Tx SRS, at least support

- 8 ports in 1 SRS resource for ‘antennaSwitching’;

  - FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

Agreement

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

Agreement

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

## Appendix 3: Agreements from RAN1#110bis-e

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Agreement

For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time-domain granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset / cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if CS hopping and/or comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

Other details are not excluded

Agreement

For SRS TD OCC for SRS enhancements for TDD CJT, study:

- Comparison against SRS on 1 OFDM symbol

- Comparison against SRS repeated on multiple OFDM symbols

- Study the following aspects: evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

Conclusion

The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT.

Conclusion

- No further discussion of increasing the maximum number of cyclic shifts for CJT SRS.

- No further discussion of partial frequency sounding extensions for CJT SRS.

Agreement

For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

Agreement

For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- n can be 8

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

## Appendix 4: Agreements from RAN1#111

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on:

Option 1: The hopping pattern is based on the pseudo-random sequence c(i), initialized with a network-configured ID.

FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID

FFS: The relation between the legacy group / sequence hopping and the new hopping

Agreement

For SRS interference randomization, support one from the following options  (to be decided in RAN1#112):

- Opt. 1: Cyclic shift hopping

- Opt. 2: Comb offset hopping

- Opt. 3: Both cyclic shift hopping and comb offset hopping

  - FFS: details including whether to support separate and/or combined hopping

  - FFS: details on UE capability and signaling

Conclusion

No consensus on enhanced signaling for flexible SRS transmission in Rel-18

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support:

- For comb 2, support 1 and 2 comb offsets

- For comb 4, support 2 and [4] comb offset

- For comb 8, support 4 comb offsets

Agreement

For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols

- Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM)

- FFS: m can be legacy values, i.e., 2,4,[8,10,12,14].

## Appendix 5: Agreements from RAN1#112

Agreement

For SRS interference randomization, support:

Opt. 3: Both cyclic shift hopping and comb offset hopping.

  - At least the two features can be separately configured

  - FFS: Combined cyclic shift hopping and comb offset hopping for a UE

  - FFS: Separate or combined with SRS sequence group hopping / sequence hopping

  - FFS: Associated UE capability

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs.

Option 1: Reuse the SRS sequence identity nIDSRS.

Option 2: Introduce new ID(s).

  - FFS: the value range, one new ID or two separate new IDs, default ID(s)

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

At least s = 2.

  - FFS: s = 4, s = 8.

m = 2,4,8, 10,12,14, and m is a multiple of s.

Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof),

- Option 2: For comb 4, do not support 4 comb offsets.

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

Option 2: Within a slot, hopping based on only the symbol index l'.

Option 3: No intra-slot hopping.

FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf.

  - FFS:  reinitialization periodicity of N radio frames or reinitialization based on system frame number.

FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

FFS: At least support reinitialization at the beginning of each radio frame.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern:

Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Conclusion

No consensus to support the following for TDD CJT SRS enhancement in Rel-18:

Further enhancements to frequency hopping

Sequence hopping/randomization, per-hop sequence from a long SRS sequence

Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

Precoded SRS for DL CSI acquisition

Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

Configuration of  (sequence index within a group) per SRS resource

Multiplying mask sequence to the legacy SRS sequence

Conclusion

No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18.

## Appendix 6: Agreements from RAN1#112bis-e

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for a SRS resource, the hopping pattern initialization ID determined by cinit=nIDhop, where nIDhop is a new ID for cyclic shift hopping and/or comb offset hopping.

- The range of the new ID is from 0 to 1023

Agreement

For a SRS resource configured with comb offset hopping and/or cyclic shift hopping,

- If the repetition factor R = 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol.

- If the repetition factor R > 1,

  - For cyclic shift hopping, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol.

  - For comb offset hopping, within a slot, the time-domain hopping behavior depends on one of the following alternatives:

    - Alt1: The OFDM symbol index l' of the first symbol across the R repetitions.

    - Alt2: The OFDM symbol index l' of each symbol.

    - Alt3: The OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on configuration, and FFS configuration details.

Agreement

For a SRS resource configured with comb offset hopping, if the repetition factor R > 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on RRC configuration, and FFS configuration details.

- UE can indicate whether it supports one or both the options. Details to be discussed in UE feature.

Agreement

For SRS comb offset hopping / cyclic shift hopping, support reinitialization at the beginning of every N radio frame(s), where N ≥ 1.

- FFS: N is fixed or configurable.

Agreement

Whether SRS comb offset hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

- FFS: Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

FFS: UE feature/capability design details.

Conclusion

No consensus on enhanced per-TRP power control and/or power control of one SRS towards to multiple TRPs in Rel-18.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with kTC>1 comb offsets, determine the mapping from the ports to comb offsets as follows:

- If kTC=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, and {1001, 1003, 1005, 1007} on the second comb offset

- If kTC=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} on the third comb offset, and {1003, 1007} on the fourth comb offset.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is configured with comb KTC=2 and with maximum nSRScs,max=8 cyclic shifts per comb offset, the number of comb offset(s) and the cyclic shift locations are determined based on the one RRC configured cyclic shift location nSRScs as follows:

- If nSRScs<nSRScs,max/2, then 1 comb offset is used, otherwise 2 comb offsets are used.

- The 8 cyclic shift locations for the 8 ports are {nSRScs, (nSRScs+1) mod nSRScs,max, …,(nSRScs+7) mod nSRScs,max}, reusing the existing equation nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,max in 38.211 6.4.1.4.2.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 or comb 8, decide one of the following options:

- Option 1: the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

  - For comb KTC=4, k=2. For comb KTC=8, k=4. For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kNapSRS/k mod nSRScs,max.

- Option 2: the cyclic shift positions are unaligned across the comb offsets on the same OFDM symbol for comb 4, and the cyclic shift positions are aligned on only 2 of the 4 comb offsets on the same OFDM symbol for comb 8.

  - For comb KTC=4, k=2. For comb KTC=8, k=4.  Example: For port pi, nSRScs,i=nSRScs+(pi-1000mod k)+nSRScs,maxpi-1000kNapSRS/k mod nSRScs,max. FFS equation details.

- FFS: potential impact on PAPR, if any.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, the s subsets of ports are mapped cyclically as {{1, 2, …, s}, …, {1, 2, …, s}} on the m OFDM symbols.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), the SRS transmissions within each of the m/s groups of {1, 2, …, s} use the same set of subcarriers. If consecutive groups of {1, 2, …, s} are configured as repetition, then the SRS transmissions of the consecutive groups use the same set of subcarriers.

- Note: applicable to the SRS resource with or without FH/RPFS.

- FFS the scenario where comb offset hopping is configured for the SRS resource.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, study at least the following solutions:

- Whether or not a UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, based on, for example, the usage, coherency, and/or repetition configuration.

- Whether or not a UE changes the transmission order of the subsets of ports.

## Appendix 7: Agreements from RAN1#113

Agreement

For SRS comb offset hopping / cyclic shift hopping reinitialization periodicity of N radio frame(s):

- N = 128

Agreement

Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured.

- The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol.

- This is a UE-optional feature.

Agreement

For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission.

- Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled.

If a subset for cyclic shifts is configured, this feature cannot be configured.

Above is a UE optional feature.

Agreement

SRS comb offset hopping / cyclic shift hopping can be configured for aperiodic SRS.

Agreement

Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

Agreement

SRS comb offset hopping and cyclic shift hopping can be configured for a SRS resource at the same time as a separate UE capability. No joint hopping scheme is supported.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 on 2 comb offsets (KTC=4, kTC=2) or comb 8 on 4 comb offsets (KTC=8, kTC=4), the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

- For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kTCNapSRS/kTC mod nSRScs,max.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, if the UE is capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications.

- Note: This may be captured in the specification in a few different but equivalent ways, and it is up to the editor to decide.

Conclusion

There is no consensus on the support of the following feature in RAN1:

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM, support TDM factor s = 4.

## Appendix 8: Agreements from RAN1#114

Agreement

When finer time-delay-domain granularity for SRS cyclic shift hopping is configured, K is 2

  - FFS (to be decided this week) Support of K=4

Agreement

For the SRS hopping formula in cyclic shift hopping or comb offset hopping except for SRS configured with TDM, let N=128 and M=7:

- For cyclic shift hopping: αi=2πnSRScs,inSRScs,max+2πfCS, hopKnSRScs,max, where fCS, hop=S{(m=0Mc8t+m∙2m) mod Y}, and

  - t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',

  - If cyclicShiftHoppingSubset is not configured, S{n}=n.

    - If cyclicShiftHoppingFinerGranularity is not configured, Y=nSRScs,max and K=1.

    - If cyclicShiftHoppingFinerGranularity is configured, Y=K.nSRScs,max, and K=2

  - If cyclicShiftHoppingSubset is configured, S{n} denotes the (n+1)th element of the configured subset, Y is the number of elements in the subset, and K=1.

- For comb offset hopping: k0(pi)=nshiftNscRB+kTC(pi)+fTC, hop+koffsetl' mod KTC, where fTC, hop=S{(m=0Mc8t+m∙2m) mod Y}, and

  - t=mod(SFN,N)Nslotframe,μNsymbslot+ns,fμNsymbslot+l'',

    - l''=l0+l' if R=1 or UE is provided with combOffsetHoppingWithRepetition=Per-symbol; otherwise, l'' is the OFDM symbol index of the first symbol across the R repetitions within the slot.

  - If combOffsetHoppingSubset is not configured, S{n}=n, and Y=KTC.

  - If combOffsetHoppingSubset is configured, S{n} denotes the (n+1)th element of the configured subset, and Y is the number of elements in the subset.

Conclusion

When finer time-delay-domain granularity for SRS cyclic shift hopping is configured, K = 4 is not supported.

Agreement

When a subset of comb offsets for comb offset hopping is configured, and when a subset of cyclic shifts for cyclic shift hopping is configured, support the following option for configuring the subset S={S(0), S(1), …, S(z-1)} with 1<z<Z, where Z=KTC for comb offset hopping and Z=nSRScs,max for cyclic shift hopping, and:

  - Option 1b: S(0), S(1), …, S(z-1) are configured via a Z-length bitmap with S(i-1) being the i-th bit set as 1.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s = 2, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot the port, down select from the following options:

  - Option 1: The first subset includes ports {1000, 1001, 1004, 1005}, and the second subset includes {1002, 1003, 1006, 1007}.

Conclusion

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when cyclic shift hopping is configured for the SRS resource,

- Option A4: Do not support cyclic shift hopping for 8-port SRS with TDM.

Conclusion

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when comb offset hopping is configured for the SRS resource,

  - Option B5: Do not support comb offset hopping for 8-port SRS with TDM.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when sequence/group hopping is configured for the SRS resource, the time-domain behavior of hopping depends only on the OFDM symbol index l’ of each symbol.

## Appendix 9: Agreements from RAN1#114bis

Agreement

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping ID notation:

6.4.1.4.2 Sequence generation <Unchanged text is omitted> The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDcshhop at the beginning of each radio frame for which nf mod 128=0, where the cyclic-shift hopping identity nIDcshhop is contained in the higher-layer parameter cyclicShiftHopping. <Unchanged text is omitted> 6.4.1.4.3 Mapping to physical resources <Unchanged text is omitted> The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDcohhop at the beginning of each radio frame for which nf mod 128=0, where the comb hopping identity nIDcohhop is contained in the higher-layer parameter combOffsetHopping. <Unchanged text is omitted>

- Additional information

  - Reason for change: It was agreed that the same ID is to be used for comb offset hopping and cyclic shift hopping, if both are configured for a same SRS resource. Current spec uses different notations for the ID in comb offset hopping and cyclic shift hopping.

  - Summary for change: Update with the same hopping ID notation.

  - Consequences if not approved: Confusion may arise regarding whether a same hopping ID shall be used.

  - TP is for TS38.211 v18.0.0.

Agreement

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset entry indexing:

6.4.1.4.2 Sequence generation <Unchanged text is omitted> where scshSRSn and ncshSRS is the (n+1)th entry and the cardinality of the set Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1} respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}. <Unchanged text is omitted> 6.4.1.4.3 Mapping to physical resources <Unchanged text is omitted> where scohSRSn and ncohSRS is the (n+1)th entry and the cardinality of the set Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1} respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}. <Unchanged text is omitted>

- Additional information

  - Reason for change: Array index of n should correspond to (n+1)th entry of the array, if the index starts from 0.

  - Summary for change: Correct the array index of n to (n+1).

  - Consequences if not approved: Incorrect entry may be used.

  - TP is for TS38.211 v18.0.0.

Agreement

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset bitmap:

6.4.1.4.2 Sequence generation <Unchanged text is omitted> where scshSRSn and ncshSRS is the (n+1)th entry and the cardinality of the set Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1} respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}. The higher-layer parameter [cyclicShiftHoppingSubset] includes a bitmap of nSRScs,max bits with 1<ncshSRS<nSRScs,max bits being set to 1, where the (n+1)th bit being set to 1 corresponds to scshSRSn. <Unchanged text is omitted> 6.4.1.4.3 Mapping to physical resources <Unchanged text is omitted> where scohSRSn and ncohSRS is the (n+1)th entry and the cardinality of the set Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1} respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}. The higher-layer parameter [combOffsetHoppingSubset] includes a bitmap of KTC bits with 1<ncohSRS<KTC bits being set to 1, where the (n+1)th bit being set to 1 corresponds to scohSRSn. <Unchanged text is omitted>

- Additional information

  - Reason for change: The hopping subset is not described.

  - Summary for change: Describe the hopping subset based on the configured RRC parameter. Also correct the array index of n to (n+1).

  - Consequences if not approved: RAN1 specs may be incomplete and it is unclear how to interpret/use the configured RRC parameter.

  - TP is for TS38.211 v18.0.0.

Agreement

SRS comb offset hopping / cyclic shift hopping can be configured for a SRS resource in a SRS resource set with usage ‘codebook’.

- SRS comb offset hopping / cyclic shift hopping are not supported for a SRS resource in a SRS resource set with usage ‘nonCodebook’ or ‘beamManagement’.

Agreement

Adopt the text proposal for TS38.214 on cyclic shift hopping subset and finer granularity configuration:

6.2.1 UE sounding procedure <Unchanged text is omitted> - Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. For the cyclic shift hopping, a UE can be configured with finer hopping granularity of K=2 by the higher layer parameter [hoppingFinerGranularity]. The UE is not expecting that [hoppingFinerGranularity] is configured when [cyclicShiftHoppingSubset] is configured for an SRS resource. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource. <Unchanged text is omitted>

- Additional information

  - Reason for change: Cyclic shift finer hopping granularity is not described in TS38.214, and the agreement that finer hopping granularity and hopping subset cannot be configured to the same SRS resource is not reflected.

  - Summary for change: Describe the cyclic shift finer hopping granularity based on the configured RRC parameter. Add that finer hopping granularity and hopping subset cannot be configured to the same SRS resource. Also clarify that ‘configured simultaneously’ means ‘configured simultaneously for an SRS resource’.

  - Consequences if not approved: RAN1 specs may be incomplete regarding cyclic shift finer hopping granularity.

  - TP is for TS38.214 v18.0.0.

Agreement

Adopt the text proposal for TS38.211 on cyclic shift hopping finer granularity:

6.4.1.4.2 Sequence generation <Unchanged text is omitted> If the higher-layer parameter hoppingFinerGranularity is configured and the higher-layer parameter cyclicShiftHoppingSubset is not configured, K=2, otherwise K=1. <Unchanged text is omitted>

- Additional information

  - Reason for change: Finer hopping granularity and hopping subset cannot be configured to the same SRS resource, and thus the second part of the If condition should be removed.

  - Summary for change: Remove ‘and the higher-layer parameter cyclicShiftHoppingSubset is not configured’.

  - Consequences if not approved: Confusion may arise regarding the ‘otherwise’ part.

  - TP is for TS38.211 v18.0.0.

Agreement

Adopt the text proposal for TS38.214 on the frequency hopping behavior when TDM or comb offset hopping is configured:

6.2.1 UE SRS frequency hopping procedure If for a SRS resource, the higher-layer parameter [tdm] is configured or higher-layer parameter [combOffsetHopping] is configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. If for a SRS resource, the higher-layer parameter [tdm] is not configured and higher-layer parameter [combOffsetHopping] is not configured, the UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211] and this clause. <Unchanged text is omitted>

- Additional information

  - Reason for change: TS38.214 Clause 6.2.1.1 describes the SRS frequency hopping procedure. However, the description covers only the cases without TDM and without comb offset hopping.

  - Summary for change: Add description of SRS frequency hopping procedure with TDM or with comb offset hopping by referring to TS38.211.

  - Consequences if not approved: TS38.214 Clause 6.2.1.1 is incomplete.

  - TP is for TS38.214 v18.0.0.

Agreement

Adopt the text proposal for TS38.214 on removing the limitation of no more than one SRS resource set to be triggered/configured:

6.2.1.2 UE sounding procedure for DL CSI acquisition <Unchanged text is omitted> For 1T2R, 1T4R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, or 4T8R, or 8T8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R 4T=4R, or 8T=8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol. <Unchanged text is omitted>

- Additional information

  - Reason for change: The per-slot restriction for antennaSwitching applies to xTyR where x<y, but 8T8R was included.

  - Summary for change: Remove 8T8R from the per-slot restriction.

  - Consequences if not approved: Unnecessary restriction for 8T8R.

  - TP is for TS38.214 v18.0.0.

Agreement

Adopt the text proposal for TS38.214 on allowing 2 SP SRS resource sets for 8T8R per UE capability:

6.2.1.2 UE sounding procedure for DL CSI acquisition <Unchanged text is omitted> - For 1T=1R, or 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2, 4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching2SP-1Periodic8T8R], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2, 4[, or 8] or <Unchanged text is omitted>

- Additional information

  - Reason for change: It was agreed that 8T8R can have the same maximum number of SRS resource sets configured.

  - Summary for change: Remove the brackets for 8 ports, and add the capability for 8T8R with 2 semi-persistent and 1 periodic SRS resource set.

  - Consequences if not approved: Incomplete support for 8T8R.

  - TP is for TS38.214 v18.0.0.

Conclusion

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ / ‘antennaSwitching’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE still transmits the SRS on the rest of OFDM symbols within the group of {1, 2, …, s}.

  - Above does not imply that UE can or cannot maintain phase coherence from partially dropped SRS resource to the next PUSCH transmission

  - (Agreement) Prepare a LS to inform RAN4 and ask if the UE can or cannot maintain phase coherence

No additional RAN1 spec change will be introduced to support this feature

LS to RAN4 is endorsed in R1-2310645