# 3GPP TSG RAN WG1 #115							R1-2312390

# Chicago, USA, November 13th – November 17th, 2023

Agenda item:	6

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Discussion on R1-2312308 - Correction on Koffset during random access procedure

Document for:	Discussion and Decision

# Issue #1: Applicability of UE-specific K-offset during random access in connected mode

<Unchanged parts are omitted>

-	if the UE is configured with the higher layer parameter k-Offset,

-	Koffset= Kcell_offset-KUE_offset where

Kcell_offset is the parameter k-Offset provided by higher layers, and

KUE_offset is the parameter Differential Koffset provided by higher layers , otherwise KUE_offset=0

-	otherwise,

-	Koffset=0.

If the UE is configured with higher layer parameter k-Offset, for a PUSCH (re)transmission associated with the TC-RNTI, Koffset= k-Offset.

The following change is proposed in several sections of TS 36.213 to address the issue of applicability of UE-specific kOffset during random access in connected mode:

During the online discussion, companies had some comments on the following points:

- Whether we should use “temporary C-RNTI” or “scheduled by RAR”.

- Whether the change was needed at all (which hopefully was resolved during the session).

Companies are encouraged to provide feedback on the draft CR above.

| Company | Comment |
| --- | --- |
| Ericsson | We are ok with the current wording of the draft CR above. |
| ZTE | Fine with the proposed CR. |
| Qualcomm | Based on some offline discussion, we would like to provide the spec information that indeed the (N)PUSCH associated with MSG3 RAR during CBRA is using TC-RNTI (36.213, NB-IoT) Note that the TC-RNTI is only set by higher layers if the UE initiates CBRA (36.321): |
| Skylo | We are OK with the proposed CR. The proposed CR eliminates ambiguity and simplifies implementation complexity for the base station by removing the need to attempt decoding across all possible UE specific K-offset values. |
| Nokia, NSB | It seems there will be several cases: 1) UE transmits Msg3 in initial access (CBRA) with RAR UL grant (TC-RNTI), 2) UE transmits Msg3 in CONNECTED mode with CFRA with RAR UL grant (C-RNTI) and 3) UE transmits Msg3 in CONNECTED mode with CBRA with RAR UL grant (TC-RNTI). We think for case 1) UE should use cell specific Koffset as UE is not configured with UE specific Koffset and for case 2)&3) UE has UE specific Koffset and it can use it. Then seems the previous Rel17 spec work well and no need to update. Please correct me if anything not correct. |
| Qualcomm | To Nokia: For case 3, how can the network know what Koffset the UE is using? Before successful reception of msg3, the network doesn’t know which UE transmitted the PRACH. If the CR is not agreed it would mean that, if you have a single UE in connected mode with UE-specific Koffset, the network would need to blindly detect between cell-specific and UE-specific Koffset for all msg3 transmissions (since you don’t know if this msg3 is transmitted by the UE with UE-specific Koffset, by another RRC_Connected UE with no UE-specific Koffset, or by a UE in RRC_IDLE mode). |
| Nordic | We are fine with the current proposal |
| Nokia, NSB | We agree with QC’s view. We can be fine for the proposed CR. |

## Conclusion

After the discussion, all companies seem to be OK with approving the draft CR.

Proposal:

The draft CR in R1-2302308 is endorsed. Final CRs in R1-2312490 (Rel-17, Cat-F) and R1-2312491 (Rel-18, Cat-A) are agreed.