3GPP TSG-RAN WG1 Meeting #115	  R1-2312401

Chicago, USA, November 13 - 17, 2023

Agenda Item:	8.14.3

Source:	Moderator (Huawei)

Title:	Summary#3 for CSI evaluation of [115-R18-AI/ML]

Document for:	Discussion and Decision

- Introduction

After the RAN1#114bis meeting, the remaining issues for the clean-up of “Evaluation on AI/ML for CSI feedback enhancement” for TR 38.843 [1] still include [2]:

- High level observations for the studied areas.

The FL summary handles the above remaining issues for CSI evaluations.

  - 1st round email discussions

Before going to the discussions in the following sections, companies are invited to type in the contact person information into the following table. Please update your contact information on top of the last meeting if needed.

Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| Samsung | Ameha | amehat.abebe@samsung.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Jianfeng Wang | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| ZTE | Lun Li Xingguang Wei | li.lun1@zte.com.cn wei.xingguang@zte.com.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Intel | Victor Sergeev | victor.sergeev@intel.com |
| Ericsson | Mattias Frenne | Mattias.frenne@ericsson.com |
| Futurewei | Baoling Sheen | bsheen@futurewei.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| AT&T | Isfar Tariq Salam Akoum | isfar.tariq@att.com salam.akoum@att.com |
| CMCC | Yuhua Cao | caoyuhua@chinamobile.com |
| Qualcomm | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Mavenir | Ali Fatih Demir Yuanlong Yang | ali.demir@mavenir.com yuanlong.yang@mavenir.com |
| NTT DOCOMO | Haruhi Echigo Liu Liu Xin Wang | haruhi.echigo.fw@nttdocomo.com liul@docomolabs-beijing.com.cn wangx@docomolabs-beijing.com.cn |
| Panasonic | Tetsuya Yamamoto | yamamoto.tetsuya001@jp.panasonic.com |
| IIT Kanpur | Abhishek Kumar Singh | Abhishekks@iitk.ac.in |
| Spreadtrum | Mimi Chen | Mimi.chen@unisoc.com |
| LG Electronics | Haewook Park | haewook.park@lge.com |
| vivo | Jianming Wu | jianming.wu@vivo.com |
| CATT | Yongqiang Fei Qianrui Li | feiyongqiang@catt.cn liqianrui@catt.cn |
| Mavenir | Fan Yang | fan.yang@mavenir.com |
| Fujitsu | Xin Wang Qun Zhang | wangxin@fujitsu.com zhangqun@fujitsu.com |
| ETRI | Anseok Lee | alee@etri.re.kr |
| Apple | Huaning Niu | huaning_niu@apple.com |
| MediaTek | Pedram Kheirkhah Sangdeh Yuching Huang | Pedram.kheirkhah@mediatek.com yuching.huang@mediatek.com |
| InterDigital | MoonIl Lee | MoonIl.Lee@InterDigital.com |
| Nokia | Tosato, Filippo | filippo.tosato@NOKIA.COM |
| Xiaomi | Min Liu | liumin10@xiaomi.com |
| China Telecom | Bei Yang | yangbei1@chinatelecom.cn |
| Fraunhofer | Ebrahim Amiri | ebrahim.amiri@iis.fraunhofer.de |
| CEWiT | Advaith | advaith22@cewit.org.in |

- High level observations to TR 38.843 for CSI compression

Moderator note: The high level observation on CSI compression has been discussed in the #114bis meeting, yet the consensus was not achieved for some of the contents due to limited time of discussion. In addition, in the last meeting, we mainly focused on the performance comparison between AI/ML solutions, yet the basic performance gain of AI/ML based CSI compression over non-AI/ML was not discussed.

  - 1st/2nd round email discussions

### Issue#2-1 High level observation - Basic performance gain over non-AI/ML

Moderator note: For the basic performance gain over the non-AI/ML benchmark, we have evaluated the 1-on-1 joint training case, where AI/ML has shown moderate UPT gains over Rel-16 eType II CB by a majority of companies. On the other hand, when considering other practical assumptions, e.g., generalization/scalability, multi-vendor joint training, separate training, etc., the performance gain is expected to be shrunk; in addition, the performance gain varies over RU, CSI overhead, and over companies. These are captured to the high level observation.

To illustrate the results of 1-on-1 joint training from companies, the results from Table 5.12 of R1-2308342 subject to RU>70% are summarized in Figure 1-Figure 3, where the x axis means the index of sources, while y axis means the gain in %-ile. Note that the numbers are based on the metric of FTP and mean UPT.

Figure 1 Mean UPT gain, Max Rank 1 (RU>70%)

Figure 2 Mean UPT gain, Max Rank 2 (RU>70%)

Figure 3 Mean UPT gain, Max Rank 4 (RU>70%)

Updates: It looks relatively more companies are in favor of Option 3. Let’s see if Option 3 can satisfy everybody.

@Qualcomm: CSI overhead reduction is not presented in the high level observation due to relatively smaller companies who submitted results and relatively large range among them – companies can refer to section 6.2.2.1 for the observation.

Upd Proposal 2.1.1: Capture the following high level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of basic performance gain over non-AI/ML benchmark, AI/ML based CSI compression outperforms Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where up to 9%/15%/17% gains0.23%~9%/-0.2%~15%/-1%~17% gains of mean UPT as shown in in Figure X1~Figure X3 are observed for Max rank 1/2/4, respectively, under certain RU and CSI overhead; on the other hand,

  - smaller gains or even performance loss are observed under other RU or CSI overhead

  - when considering other practical assumptions, e.g., generalization/scalability, multi-vendor joint training, separate training, etc., the performance gain may be lower

Figure X1 Mean UPT gain, Max Rank 1 (RU>70%)

Figure X2 Mean UPT gain, Max Rank 2 (RU>70%)

Figure X3 Mean UPT gain, Max Rank 4 (RU>70%)

| Company | View |
| --- | --- |
| Moderator | For the basic performance gain part, the intention is to provide typical and concise information to the outsiders, so we only provide the values/value ranges subject to RU>70%. In Moderator’s feeling, it captures the maximum values to show the potential of AI/ML, since the lower values may be due to inappropriate model design or training method; please provide your preference on how to capture this part if you have other ideas, e.g.: Opt 1: Only capture the maximum values with “up to” (i.e., current version) Opt 2: Capture the averaged values over companies (e.g., change the cyan part to “x%/y%/z% gains averged over sources”) Opt 3: Capture the range and Figure 1~Figure 3 (e.g., change the cyan part to “0.23%~9%/-0.2%~15%/-1%~17% gains as shown in Figure 1~Figure 3”) Opt 4: Other |
| NTT DOCOMO | We are fine with the current version (Opt 1). We also think that capturing Opt 2 in addition to Opt 1 (i.e., both maximum and averaged values) is also an option, which shows both the potential and typical gains. |
| Apple | Either opt 2 or opt 3 is OK. Opt 1 captures the max value gives wrong impression of the performance gain. |
| LG Electronics | Option 3 is preferred to fairly reflect the performance from sources. |
| Futurwei | We prefer Opt 3. Depending on the values in the range, average may also be misleading in some cases. |
| Qualcomm | We support Opt 1. We agree with the moderator that low values may be due to inadequacies in the model structure or training choices. Also, the two bullets can be removed, since “up to” already captures the meaning that the other cases will have lower gain. CSI feedback overhead reduction is another aspect of performance gain. This is an agreed evaluation metric in the EVM table. We propose to capture a similar high-level observation for overhead reduction also. |
| Fujitsu | Thanks FL for the efforts. For the performance of AI/ML based CSI compresseion, the reality is that most companies observed positive gain. Regarding the performance considering practical assumptions, the positive gains were observed by many companies. For example, as captured in the TR and copied below, many companies observed positive gains for generalization Case 3. - For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gain) for deployment scenario#B subject to any of UMa, UMi, and InH, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B, as observed by 15 sources. - Minor loss (0%~-1.6%) are observed by 15 sources. - Moderate loss (-1.69%~-4%) are observed by 8 sources. - Positive gains are observed by 10 sources. In addition, the AI/ML performance gain could be higher when considering the practical assumptions. Therefore, we have the following suggestion on this proposal. Proposal 2.1.1: Capture the following high level observations for CSI compression to section 6.2.2.8 of TR 38.843 - From the perspective of basic performance gain over non-AI/ML benchmark, AI/ML based CSI compression outperforms Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where up to 9%/15%/17% gains of mean UPT are observed by majority sources for Max rank 1/2/4, respectively, under certain RU and CSI overhead; on the other hand, - smaller gains or even performance loss are observed under other RU or CSI overhead - when considering other practical assumptions, e.g., generalization/scalability, multi-vendor joint training, separate training, etc., the performance gain may be lower Regarding the basic performance gain part, we prefer with Opt1 and can live with Opt2 from FL. |
| ZTE | Our preference is Opt.4 as added below. Opt 4. Capture the median values over companies For Option 1, if we only provide the max values of performance gain, it may mislead outsiders the actual performance gain of CSI compression, since many companies’ simulation results are significantly lower than the max value. Additionally, regarding the performance gain has a wide range from negative value to more than 15%, we think the average value may be also not appropriate, since the average value is largely impacted by the minimum/maximum value if the minimum/maximum value is far from the majority of values. Therefore, we suggest applying median values of performance gain, which is more appropriate since it is a general principle reflecting the distribution of the majority of values. Note that in the Rel-18 Duplex study item, similar discussion happened and the median value is finally selected in the TR. Furthermore, it is convincible that the results from more than 2 sources (i.e., >=3 sources) should be summarized. The performance summary based on median values are captured in our tdoc R1-2310988. The excerpt of CSI compression is copied below for reference. RU; Overhead A; Overhead B; Overhead C. Max Rank 1; RU<=39%; 1.25% (6 sources); 0.6% (6 sources); 0.65% (4 sources). RU 40%-69%; 2.4% (5 sources); 1.5% (4 sources); 2.5% (7 sources). RU>=70%; 3.6% (9 sources); 2.43% (8 sources); 1.85% (8 sources). Max Rank 2; RU<=39%; 2% (7 sources); 2% (7 sources); 2% (8 sources). RU 40%-69%; 4.3% (8 sources); 4.255% (8 sources); 4.275% (10 sources). RU>=70%; 11% (11 sources); 6.8% (11 sources); 5.5% (10 sources). Max Rank 4; RU<=39%; 6% (3 sources); N/A; N/A. RU 40%-69%; 11% (3 sources); N/A; 5.1% (3 sources). RU>=70%; 14.89% (3 sources); N/A; 6% (3 sources). Table 2. The median values of mean-UPT gain over benchmark under FTP traffic Table 3. The median values of 5%-UPT gain over benchmark under FTP traffic RU; Overhead A; Overhead B; Overhead C. Max Rank 1; RU<=39%; 1.9% (3 sources); 1% (3 sources); 1.68% (3 sources). RU 40%-69%; 5.8% (6 sources); 2% (3 sources); 3% (3 sources). RU>=70%; 11.1% (8 sources); 7% (7 sources); 3.24% (8 sources). Max Rank 2; RU<=39%; 4.86% (5 sources); 2.6% (6 sources); 2% (7 sources). RU 40%-69%; 7.55% (6 sources); 5.5% (7 sources); 3% (6 sources). RU>=70%; 17.96% (6 sources); 11.2% (6 sources); 7.635% (8 sources). Max Rank 4; RU<=39%; N/A; N/A; N/A. RU 40%-69%; 9.25% (3 sources); N/A; 8% (3 sources). RU>=70%; 23.27% (3 sources); N/A; 7.1% (3 sources). |
| Ericsson | Either option 3 or option 4 (mediam values) is ok. |
| Samsung | We support Option 3. Option 3 provides the full picture of the selected result, i.e., performance range and distribution, considered RU range, etc., as compared to Option 1. |

### Issue#2-2 High level observation - Complexity analysis for ground-truth CSI

Moderator note: For monitoring, one controversial issue in the last meeting was whether/how to address the complexity analysis in the observation for intermediate KPI based monitoring. It is straightforward that there is increased NW management complexity for Case 2 if the proxy model is not transparent to NW, and there is increased UE complexity on CSI calculation for Case 1; on the other hand, however, in the previous #114 observation on monitoring, there is no quantitative analysis on UE complexity, and it clearly captures “the complexity aspect for Case 1, Case 2-1 and Case 2-2  is not evaluated”, it seems no need to capture the complexity for either side in the high level observation. Thus, the complexity analysis is tentatively removed from the proposed observation, and a note is added to clarify that the complexity is not evaluated.

Similarly, for high-resolution quantization of the ground-truth CSI for the training, the complexity analysis is tentatively removed, and a note is added.

Updates:

@Apple @LG @Futurewei: “Benefitial” changed to “benefitial in performance” to clarify the point of benefits. @Futurewei: “data collection” added.

@QC: Changed as suggested.

@ZTE @Ericsson: “may be” changed to “would be”, to be consistent with the description on overhead increase.

@Fujitsu: “robustness” is supposed to be the same meaning as “realiability”?

Upd Proposal 2.1.2: Capture the following high level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of intermediate KPI based monitoring

  - For the monitoring at NW side, it is beneficial in performance to achieve increased monitoring accuracy by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for monitoring. On the other hand, the new/larger parameter(s) would lead to increased air-interface overhead and UE complexity compared to R16 eType II CB with legacy parameters

  - For the monitoring at UE side, it is beneficial in performance to achieve monitoring with smaller air-interface overhead by considering proxy model at UE. On the other hand, it may lead to increased NW management complexity if not transparent to NW and increased UE complexity for deploying and using the proxy model; in addition, the monitoring accuracy would may be impacted by the design/robustness of the proxy model

  - Note: the complexity aspect for Case 1, Case 2-1 and Case 2-2 is not evaluated

- From the perspective of high resolution ground-truth CSI for training, it is beneficial in performance to avoid severe performance degradation, and, achieve significant overhead reduction compared to unquantized ground-truth CSI (e.g., Float32) by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for training data colelction, or achieve moderate overhead reduction by considering scalar quantization format for training data colelction. On the other hand, the new/larger parameter(s) would may lead to increased overhead and UE complexity compared to R16 eType II CB with legacy parameters

  - For ground-truth CSI format, 5 sources observe R16 eType II CB with new/larger parameter(s) outperforms R16 eType II CB with legacy parameter, while one source observes R16 eType II CB with legacy parameter is already close to Float32 with particular dataset processing

  - Note: the complexity aspect is not evaluated

| Support/Can accept | NTT DOCOMO, Qualcomm (comment), |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| NTT DOCOMO | Fine to remove the discussions on the complexity. |
| Apple | Since complexity is not evaluated, we suggest to capture the fact only, instead saying it is benifical. - From the perspective of intermediate KPI based monitoring - For the monitoring at NW side, it is beneficial to achieve increased monitoring accuracy is acheived by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for monitoring. On the other hand, the new/larger parameter(s) would lead to increased air-interface overhead and UE complexity compared to R16 eType II CB with legacy parameters - For the monitoring at UE side, it is beneficial to achieve monitoring with smaller air-interface overhead is achieved by considering proxy model at UE. On the other hand, it may lead to increased NW management complexity if not transparent to NW and increased UE complexity for deploying and using the proxy model; in addition, the monitoring accuracy may be impacted by the design/robustness of the proxy model - Note: the complexity aspect for Case 1, Case 2-1 and Case 2-2 is not evaluated - From the perspective of high resolution ground-truth CSI for training, it is beneficial to avoid severe performance degradation, and, achieve significant overhead reduction compared to unquantized ground-truth CSI (e.g., Float32), by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for training avoid severe performance degradation and achieve significant overhead reduction. or achieve moderate overhead reduction by considering scalar quantization format for training achieve moderate overhead reduction. On the other hand, the new/larger parameter(s) may lead to increased overhead and UE complexity compared to R16 eType II CB with legacy parameters - For ground-truth CSI format, 5 sources observe R16 eType II CB with new/larger parameter(s) outperforms R16 eType II CB with legacy parameter, while one source observes R16 eType II CB with legacy parameter is already close to Float32 with particular dataset processing - Note: the complexity aspect is not evaluated |
| LG Electronics | Generally ok. We also fine with Apple’s modification. |
| Futurewei | Ok with Apple’s updates for the first bullet (“From the perspective of intermediate KPI based monitoring”). Some additional updates (in cyan) on top of Apple’s for the second bullet as follows. - From the perspective of high resolution ground-truth CSI for training, it is beneficial to avoid severe performance degradation, and, achieve significant overhead reduction compared to unquantized ground-truth CSI (e.g., Float32), by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for training avoids severe performance degradation and achieves significant overhead reduction in data colelction. or achieve moderate overhead reduction by considering Adopting scalar quantization format for training achieves moderate overhead reduction in data colelction. On the other hand, the new/larger parameter(s) may lead to increased overhead and UE complexity compared to R16 eType II CB with legacy parameters. - For ground-truth CSI format, 5 sources observe R16 eType II CB with new/larger parameter(s) outperforms R16 eType II CB with legacy parameters, while one source observes R16 eType II CB with legacy parameters is already close to Float32 with particular dataset processing technique. - Note: the complexity aspect is not evaluated |
| Qualcomm | To be consistent with the first paragraph, “may lead to” should be changed to “would lead to” in the second paragraph. |
| Fujitsu | Generally fine with the proposal. One comment is that the reliability of the proxy model is also an important aspect to be considered. Since the proxy model is also trained by data, it’s not clear how to monitor the performance of the proxy model, i.e., how to make sure that the proxy model works well. This will impact the monitoring accuracy. Therefore, we have the following suggestion. - For the monitoring at UE side, it is beneficial to achieve monitoring with smaller air-interface overhead by considering proxy model at UE. On the other hand, it may lead to increased NW management complexity if not transparent to NW and increased UE complexity for deploying and using the proxy model; in addition, the monitoring accuracy may be impacted by the design/robustness/reliability of the proxy model |
| ZTE | For the 2nd sub-bullet of the 1st bullet, there are some simulation results clearly showing a significant monitoring performance loss regarding the generalization of UE-side proxy model. So we suggest changing ‘may be’ to ‘is’ as blows - For the monitoring at UE side, it is beneficial to achieve monitoring with smaller air-interface overhead by considering proxy model at UE. On the other hand, it may lead to increased NW management complexity if not transparent to NW and increased UE complexity for deploying and using the proxy model; in addition, the monitoring accuracy may be is impacted by the design/robustness of the proxy model |
| Ericsson | Support the proposal, and ok with Fujitsu’s and ZTE’s changes. |

### Issue#2-3 High level observation – other aspects

Moderator note: Other aspects of the high level observation (model input/output type, quantization methods, generalization, and training collaboration types) seem relatively stable during the last meeting.

Updates:

@ NTT DOCOMO: “several” was removed in the last meeting since some companies think “several” implies “many”.

@ Futurewei: Comments addressed.

@Qualcomm: “VQ format achieves comparable performance with scalar quantization format (SQ)” is captured directly from the old observation. But to address your comments, your suggested words are added after. For “up to 3”, it is added by correcting to “up to 4” – scalability table has up to 4 mixed datasets.

@ZTE: if we do not provide channel matrix as comparison, then “precoding matrix” is benefitial over what is not clear. To address your comments, we can simply say it is “benefitial by considering predocing matrix” and remove “in performance” – precoding is deemed as the outcome after internally comparing input types by companies.

Upd Proposal 2.1.3: Capture the following high level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of model input/output type, it is more beneficial in performance by considering precoding matrix as the model input (for CSI generation part)/output (for CSI reconstruction part) than explicit channel matrix

- From the perspective of quantization methods for CSI feedback,

  - For the quantization awareness for training, it is beneficial to avoid severe performance degradation by considering quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) or jointly updated quantization method/parameters (Case 2-2). In particular, it is more beneficial in performance for Case 2-2 over Case 2-1 under vector quantization format (VQ)

  - For the quantization format, VQ format achieves comparable performance with scalar quantization format (SQ), where VQ achieves better performance than SQ in some cases while worse in some other cases

- From the perspective of generalization over scenarios, or scalability over configurations that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B,

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others

  - For generalization Case 3 where the training dataset is constructed with data samples subject to more than one scenario/configuration (evaluations studied up to four scenarios/configurations) including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved

  - In particular, appropriate scalability solution (e.g., truncation/padding, adaptive quantization granularities, adaptation layer in the AI/ML model) may need to be performed to scale the dimensions of the AI/ML model when the training dataset includes data samples subject to configuration#A which has different input/output dimension than configuration#B

- From the perspective of training collaboration types, compared to 1-on-1 joint training, both multi-vendor joint training and separate training with procedures given in Section 6.2.1 may suffer performance loss to some extent.

  - In particular, for multi-vendor joint training, minor or moderate degradation is observed

  - In particular, for separate training with procedure given in Section 6.2.1, the performance loss depends on the factors such as of backbone alignment, and multi-vendor training behavior:

    - For separate training of 1 NW part model and 1 UE part model, under both NW first training and UE first training, if backbones are aligned between the two sides, minor degradation is observed; otherwise, additional degradation is observed suffered, leading to minor or moderate performance degradation

    - For NW first training with 1 UE part model to N>1 NW part models, or UE first training with 1 NW part model to M>1 UE part models, additional degradation is observed suffered, leading to minor, moderate, or significant performance degradation, depending on the training approach

    - As a note, other procedures of separate training are not extensively evaluated

| Support/Can accept | Qualcomm (comments) |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| NTT DOCOMO | On the discussions about the generalization and scalability, we prefer to use the wording from the discussions results of last meeting, “From the perspective of generalization over several scenarios or scalability over several configurations that have been evaluated,” |
| Apple | Agree |
| LG Electronics | Generally ok. |
| Futurewei | Some minor updates (in cyan). - From the perspective of training collaboration types, compared to 1-on-1 joint training, both multi-vendor joint training and separate training with procedures given in Section 6.2.1 may suffer performance loss to some extent. - In particular, for multi-vendor joint training, minor or moderate degradation is observed. - In particular, for separate training with procedures given in Section 6.2.1, the performance loss depends on the factors like of backbone alignment, and multi-vendor training behavior: - For separate training of 1 NW part model and 1 UE part model, under both NW first training and UE first training, if backbones are aligned between the two sides, minor degradation is observed; otherwise, additional degradation is observed suffered, leading to minor or moderate performance degradation. - For NW first training with 1 UE part model to N>1 NW part models, or UE first training with 1 NW part model to M>1 UE part models, additional degradation is observed suffered, leading to minor, moderate, or significant performance degradation, depending on the training approach. - As a note, other procedures of separate training are not extensively evaluated. |
| Qualcomm | Regarding SQ vs. VQ, we propose the following wording for clarity: “Vector quantization (VQ) achieves better performance than scalar quantization (SQ) format in some cases while worse in some other casess, depending on the specific method” instead of saying “VQ format achieves comparable performance with scalar quantization format (SQ)” Regarding generalization case 3, after “more than one”, please add “(evaluations studied the case of up to three scenarios/configurations)”. |
| ZTE | For the 1st bullet, due to “no companies submitting explicit channel matrix as input” captured in TR, we suggest rewording it as - From the perspective of model input/output type, it is more beneficial in performance by considering precoding matrix as the model input (for CSI generation part)/output (for CSI reconstruction part) than explicit channel matrix |
| Ericsson | OK |

  - 3rd round email discussions

### Issue#2-1 High level observation - Basic performance gain over non-AI/ML

Moderator note: Updated based on Tue off-offline discussion.

Note: for Max rank 4, RU≤39%, “-4%~7.4%” in TR is changed to “-4%~6%” - In R1-2308342, the upper bound result in the observation seems wrongly captured from Table 5.12, to the observation text.

Proposal 2.2.1: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of basic performance gain over non-AI/ML benchmark, AI/ML based CSI compression outperforms Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where

  - 0.2%~2%/-0.3%~6%/-4%~6% 7.4% gains of mean UPT as shown in Figure X1~Figure X3 are observed for Max rank 1/2/4, respectively, under RU≤39%

  - 0.1%~4%/-0.5%~10%/-1.8%~12.22% gains of mean UPT as shown in Figure Y1~Figure Y3 are observed for Max rank 1/2/4, respectively, under RU40%-69%

  - 0.23%~9%/-0.2%~15%/-1%~17% gains of mean UPT as shown in Figure Z1~Figure Z3 are observed for Max rank 1/2/4, respectively, under RU>70%

Figure X1 Mean UPT gain, Max Rank 1 (RU≤39%), x-axis means index of source

Figure X2 Mean UPT gain, Max Rank 2 (RU≤39%), x-axis means index of source

Figure X3 Mean UPT gain, Max Rank 4 (RU≤39%), x-axis means index of source

Figure Y1 Mean UPT gain, Max Rank 1 (RU40%-69%), x-axis means index of source

Figure Y2 Mean UPT gain, Max Rank 2 (RU40%-69%), x-axis means index of source

Figure Y3 Mean UPT gain, Max Rank 4 (RU40%-69%), x-axis means index of source

Figure Z1 Mean UPT gain, Max Rank 1 (RU>70%), x-axis means index of source

Figure Z2 Mean UPT gain, Max Rank 2 (RU>70%), x-axis means index of source

Figure Z3 Mean UPT gain, Max Rank 4 (RU>70%), x-axis means index of source

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### Issue#2-1a (new) High level observation – CSI overhead reduction over non-AI/ML

Moderator note: Newly added based on Tue off-offline discussion. The observation merges the CSI feedback reduction over cases of CSI overhead A/B/C. Only FTP traffic is taken into account.

Proposal 2.2.2: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of CSI overhead reduction over non-AI/ML, AI/ML based CSI compression achieves CSI feedback reduction compared with Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where 5 sources observe the CSI feedback reduction of 10.24%~60%/10%~58.33%/8%~79% for Max rank 1/2/4, respectively, under FTP traffic.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ZTE | To clarify that the results of 4 sources (Huawei, ZTE, Futurewei, Qualcomm) are captured in the agreed observation in RAN1#114 meeting. - From the perspective of CSI overhead reduction over non-AI/ML, AI/ML based CSI compression achieves CSI feedback reduction compared with Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where 5 4 sources observe the CSI feedback reduction of 10.24%~60%/10%~58.33%/8%~79% for Max rank 1/2/4, respectively, under FTP traffic. |

### Issue#2-1b (new) High level observation – AI/ML complexity

Moderator note: Newly added based on Tue off-offline discussion from CSI other aspects.

Proposal 2.2.3: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of AI/ML complexity, a majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M; on the other hand, the actual model complexity may differ from the model complexity in the evaluation with respect to optimization implementations. In addition, the complexity between AI/ML and non-AI/ML benchmark is not compared.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | General OK. But it should be clear that what ‘ actual model complexity’ means. |

### Issue#2-2 High level observation - Complexity analysis for ground-truth CSI

Moderator note: Updated based on Tue off-offline discussion.

Proposal 2.2.4: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of intermediate KPI based monitoring

  - For the monitoring at NW side, increased monitoring accuracy can be achieved by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for monitoring. On the other hand, the new/larger parameter(s) would lead to increased air-interface overhead compared to R16 eType II CB with legacy parameters.

  - For the monitoring at UE side, performance can be monitored with smaller air-interface overhead by considering proxy model at UE compared with monitoring at NW side. On the other hand, the monitoring accuracy may be impacted by the design/robustness of the proxy model.

  - Note: the complexity aspect for Case 1, Case 2-1 and Case 2-2 is not evaluated

- From the perspective of high resolution ground-truth CSI for training, compared to unquantized ground-truth CSI (e.g., Float32), taking R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for training data collection can achieve significant overhead reduction without causing severe performance degradation; taking scalar quantization format for training data collection can achieve moderate overhead reduction without causing severe performance degradation. On the other hand, the R16 eType II CB with new/larger parameter(s) would lead to increased overhead compared to R16 eType II CB with legacy parameters

  - For ground-truth CSI format, 5 sources observe R16 eType II CB with new/larger parameter(s) outperforms R16 eType II CB with legacy parameter, while one source observes R16 eType II CB with legacy parameter is already close to Float32 with particular dataset processing technique

  - Note: the complexity aspect is not evaluated

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| ZTE | For the 2nd sub-bullet of the 1st bullet, after careful thinking, we suggest reverting to the previous wording, i.e., changing ‘may’ to ‘would’ as blows. The reason is that there are some simulation results (e.g., ZTE, Huawei) clearly showing a significant monitoring performance loss regarding the generalization of UE-side proxy model. In addition, the wording “would” can be aligned with other bullets. - For the monitoring at UE side, performance can be monitored with smaller air-interface overhead by considering proxy model at UE compared with monitoring at NW side. On the other hand, the monitoring accuracy may would be impacted by the design/robustness of the proxy model. |

### Issue#2-3 High level observation – other aspects

Moderator note: Updated based on Tue off-offline discussion.

Proposal 2.2.5: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of model input/output type, it is more beneficial by considering precoding matrix as the model input (for CSI generation part)/output (for CSI reconstruction part) than explicit channel matrix

- From the perspective of quantization methods for CSI feedback,

  - For the quantization awareness for training, it is beneficial to consider quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) or jointly updated quantization method/parameters (Case 2-2) to avoid severe performance degradation. In particular, it is more beneficial in performance for Case 2-2 over Case 2-1 under vector quantization format (VQ).

  - For the quantization format, VQ format achieves comparable performance with scalar quantization format (SQ) in general, where VQ achieves better performance than SQ in some cases while worse in some other cases

- From the perspective of generalization over scenarios, or scalability over configurations that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B,

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others

  - For generalization Case 3 where the training dataset is constructed with data samples subject to more than one scenario/configuration (evaluations studied up to four scenarios/configurations) including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved

  - In particular, appropriate scalability solution (e.g., truncation/padding, adaptive quantization granularities, adaptation layer in the AI/ML model) may need to be performed to scale the dimensions of the AI/ML model when the training dataset includes data samples subject to configuration#A which has different input/output dimension than configuration#B

- From the perspective of training collaboration types, compared to 1-on-1 joint training, both multi-vendor joint training and separate training with procedures given in Section 6.2.1 may suffer performance loss.

  - In particular, for multi-vendor joint training, minor or moderate degradation is observed

  - In particular, for separate training with procedure given in Section 6.2.1, the performance loss depends on the factors such as backbone alignment, and multi-vendor training behavior:

    - For separate training of 1 NW part model and 1 UE part model, under both NW first training and UE first training, if backbones are aligned between the two sides, minor degradation is observed; otherwise, additional degradation is observed, leading to minor or moderate performance degradation

    - For NW first training with 1 UE part model to N>1 NW part models, or UE first training with 1 NW part model to M>1 UE part models, additional degradation is observed, leading to minor, moderate, or significant performance degradation, depending on the training approach

    - As a note, other procedures of separate training are not extensively evaluated

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

  - 4th round email discussions

### Issue#2-1a High level observation – CSI overhead reduction over non-AI/ML

Moderator note: Outcome of Wed. offline discussion.

Proposal 2.3.1: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of CSI overhead reduction over non-AI/ML, AI/ML based CSI compression achieves CSI feedback reduction compared with Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where 4 sources observe the CSI feedback overhead reduction of 10.24%~60%/10%~58.33%/8%~79% for Max rank 1/2/4, respectively, under FTP traffic.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### Issue#2-1b High level observation – AI/ML complexity

Moderator note: Outcome of Wed. offline discussion.

Proposal 2.3.2: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of AI/ML complexity, a majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M; on the other hand, the actual model complexity may differ from the model complexity in the evaluation with respect to platform-dependent optimization on model implementations. In addition, the complexity between AI/ML and non-AI/ML benchmark is not compared.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

- High level observations to TR 38.843 for CSI prediction

  - 1st/2nd round email discussions

### Issue#3-1 High level observation for CSI prediction

Moderator note: In the RAN1#114bis meeting, the following high level observation on CSI prediction has been achieved, including the aspects of AI/ML solutions (input type, UE speed, prediction window, and observation window) and generalization.

Agreement Capture the following high level observations for CSI prediction to section 6.2.2.8 of TR 38.843: - From the perspective of model input/output type, it is more beneficial in performance by considering raw channel matrix as the model input than precoding matrix - The gain of AI/ML based CSI prediction over the benchmark of the nearest historical CSI is impacted by the length of the observation window length, prediction window length, and UE speed - From the perspective of generalization over various several UE speeds that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B, - For generalization Case 2 where the AI/ML model is trained with dataset from a different UE speed#A, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, generalized performance of the AI/ML model can be achieved in general

One remaining aspect is the basic performance gain over the non-AI/ML benchmark. For AI/ML based CSI prediction over the benchmark of the nearest historical CSI, most companies [Huawei, Spreadtrum, InterDigital, vivo, Apple, MediaTek] observe gains, including 6 sources observing up to 10.58% gain and 2 sources observe 15.1% ~23.5% gain; on the other hand, there is one source who observes loss of -1.3%~-13.8% [InterDigital]. After filtering out the biased results, we may say AI/ML based CSI prediction outperforms the benchmark of the nearest historical CSI in general, with majority achieving up to 10.58% gain.

For AI/ML based CSI prediction over the benchmark of non-AI/ML based CSI prediction, the numbers of sources that observe gain and loss are similar, and both sides are relatively small number: 3 sources observe performance gain [Huawei, vivo, MediaTek], while 2 sources observe performance loss [MediaTek, InterDigital]. Therefore, we only provide the number of sources but do not give judgement whether AI/ML outperforms non-AI/ML.

Updates:

@LG @Futurewei: Comments addressed.

@Qualcomm: Comments addressed.

@ZTE Thanks for providing the numbers; but I do worry companies have time to calculate and check the median values during the meeting week. Can you live with capturing the ranges as in the updated proposal?

Upd Proposal 3.1.1: Capture the following high level observation for CSI prediction to section 6.2.2.8 of TR 38.843

- From the perspective of basic performance gain over non-AI/ML benchmark, under the same UE speed for training and inference,

  - AI/ML based CSI prediction outperforms the benchmark of the nearest historical CSI in general, where the majority of sources observe up to 10.6% gain in terms of mean UPT.

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3 sources observe up to 0.7%~7% gain while 2 sources observe performance loss of -0.1%~-17% in terms of mean UPT.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| LG Electronics | To be consist with non-AI/ML based CSI prediction, we suggest to following changes on 2nd subbullet. - For AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3 sources observe up to 0.7%~7% gain while 2 sources observe performance loss of -0.1%~-17% in terms of mean UPT. |
| Futurewei | Minor updates on top of LG’s. - From the perspective of basic performance gain over non-AI/ML benchmark, - AI/ML based CSI prediction outperforms the benchmark of the nearest historical CSI in general, where the majority of sources observe up to 10.6% gain in terms of mean UPT. - For AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3 sources observe up to 0.7%~7% gain while 2 sources observe performance loss of -0.1%~-17% in terms of mean UPT. |
| Qualcomm | Please add the assumption behind the result as captured in the observation, i.e., “The same fixed UE speed of 30km/h or 60km/h is assumed for both training and inference” |
| ZTE | Similar view as Issue#2-1. We suggest capturing the median values of performance gain for CSI prediction, which are captured in our tdoc R1-2310988. Considering the simulation results of limited sources, we capture all the results despite from limited 1~2 sources. The excerpt of CSI prediction is copied below for reference. Table 7. The median values of mean-UPT gain over benchmark#1&2 under FTP&full buffer traffic Traffic model; Benchmark; Without spatial consistency; With spatial consistency. FTP; Benchmark#1; 4.9% (5 sources); 17.2% (3 sources). Benchmark#2; 2.3% (2 sources); 0.7% (3 sources). Full buffer; Benchmark#1; 7.6% (1 source); 8.7% (3 sources). Benchmark#2; 11.5% (1 source); 2.3% (2 sources). Traffic model; Benchmark; Without spatial consistency; With spatial consistency. FTP; Benchmark#1; 12.8% (5 sources); 8.6% (2 sources). Benchmark#2; 6.7% (3 sources); 1.44% (2 sources). Full buffer; Benchmark#1; 3.5% (1 source); 15% (3 sources). Benchmark#2; 15.4% (1 source); 6.7% (2 sources). Table 8. The median values of 5%-UPT gain over benchmark#1&2 under FTP&full buffer traffic |
| Ericsson | OK with LG and Futurwei’ modifications. |
| Samsung | Ok to capture the range to be consistent with compression. |

  - 3rd round email discussions

### Issue#3-1 High level observation for CSI prediction

Moderator note: Updated based on Tue off-offline discussion.

Proposal 3.2.1: Capture the following high-level observation for CSI prediction to section 6.2.2.8 of TR 38.843

- From the perspective of basic performance gain over non-AI/ML benchmark, under the same UE speed for training and inference,

  - AI/ML based CSI prediction outperforms the benchmark of the nearest historical CSI in general, where the majority of sources observe up to 10.6% gain in terms of mean UPT.

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3 sources observe up to 0.7%~7% gain while 2 sources observe performance loss of -0.1%~-17% in terms of mean UPT.

| Support/Can accept |  |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

- Other miscellaneous TPs for Section 6.2 of TR 38.843

  - 1st/2nd/3rd round email discussions

Moderator note: As pointed out by Ericsson in [3], there are some points related to the CSI evaluation sections which need to be corrected. Whether/how to capture them is discussed in this section

### Issue 4-1: Description of generalization for CSI enhancement

Motivation[3]: In the agreement to describe 3 generalization cases, the inference/test dataset is scenario#A/configuration#A, while the training dataset is scenario#A/configuration#A (Case 1), scenario#B/configuration#B (Case 2), and scenario#A/configuration#A + scenario#B/configuration#B (Case 3), i.e., Case 1, 2, 3 are “Train-on-#A/Test-on-#A”, “Train-on-#A/Test-on-#B”, and “Train-on-#A+B/Test-on-#B”, respectively.

Agreement The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two

However, for making templates and drawing observations, the 3 cases are changed to “Train-on-#B/Test-on-#B”, “Train-on-#A/Test-on-#B”, and “Train-on-#A+B/Test-on-#B”, respectively, with the intention to align the inference/test scenario for a better comparison.

Generalization Case 1; Train (setting#B, size/k). Test (setting#B, size/k). Generalization Case 2; Train (setting#A, size/k). Test (setting#B, size/k). Generalization Case 3; Train (setting#A+#B, size/k). Test (setting#B, size/k).

Therefore, it is proposed by Ericsson that the descritptions for generalization cases in the EVM sub-section should be changed to align with the observation sub-section.

Moderator assessment: It is good to align the order of #A and #B between the EVM part and the performance results part to avoid confusion, since in the observations related to generalization, they also capture with the notations of #A, #B.

Updates: updated based on Ericsson comments.

@LG The order of #A and #B for training/testing of current observations in 6.2.2.2 is inconsistent with the order defined in 6.2.1. The reason for the adjustment of order in the observations is that aligning the inference scenario #B over generalization cases make it much earsier for performance comparison. Therefore, correcting the 6.2.1 part will more benefit the understanding of outsiders.

Upd Proposal 4.1.1: Adopt the following TP related with changes to the generalization part to TR 38.843.

| Issue#4-1 Changes to the generalization part | Description of issue: The descritptions for generalization cases in the EVM sub-section (Case 1, 2, 3: “Train-on-#A/Test-on-#A”, “Train-on-#A/Test-on-#B”, and “Train-on-#A+B/Test-on-#B”) is inconsistent with the observation sub-section (Case 1, 2, 3: “Train-on-#B/Test-on-#B”, “Train-on-#A/Test-on-#B”, and “Train-on-#A+B/Test-on-#B”). |
| --- | --- |
| ------------------ Text Proposal for 38.843 v1.1.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** Model generalization: In order to study the verification of generalization, the following aspects are encouraged to be reported: - The configuration(s)/scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios - The configuration(s)/scenario(s) for testing/inference - The detailed list of configuration(s) and/or scenario(s) The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A Scenario#B/Configuration#B, and then - the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A Scenario#B/Configuration#B - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A a different dataset than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on Scenario#B/Configuration#B a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A Scenario#B/Configuration#B and a different dataset than Scenario#A/Configuration#A Scenario#B/Configuration#B, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from Scenario#B/Configuration#B a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two *** Unchanged text is omitted *** |  |

| Support/Can accept | ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| LG Electronics | We prefer original text as it comes from the agreement in RAN1#110. If some clarification is required, section 6.2.2.2. seems more suitable. |
| Ericsson | Support. Minor editorial comment: “Scenario#A/Configuration#B” is missing in Case 2 and Case 3 description, suggest updating the related text to - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A a different dataset than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B and then the AI/ML model performs inference/test on Scenario#B/Configuration#B a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A Scenario#B/Configuration#B and a different dataset than Scenario#A/Configuration#A Scenario#B/Configuration#B, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B and then the AI/ML model performs inference/test on a dataset from Scenario#B/Configuration#B a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. |

### Issue 4-2: Editorial change on aligning the term of “ground-truth CSI”

Motivation[3]: The TR is using both the terms “ground truth CSI” (10 places) and “ground-truth CSI” (18 places). As editorial changes, they are aligned to “ground-truth CSI”.

Moderator assessment: It is good to align the notations.

Proposal 4.1.2: Adopt the following TP related with editorial changes on “ground truth CSI” to TR 38.843.

| Issue#4-2 Changes to align the term of “ground-truth CSI” | Description of issue: The TR is using both the terms “ground truth CSI” (10 places) and “ground-truth CSI” (18 places), which are not aligned. |
| --- | --- |
| ------------------ Text Proposal for 38.843 v1.1.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** CSI compression sub use case specific aspects: *** Unchanged text is omitted *** - Considering performance impact of ground-truth quantization in the CSI compression - Studying high resolution quantization methods for ground-truth CSI, including at least the following options: - High resolution scalar quantization - High resolution codebook quantization, e.g., Rel-16 TypeII-like method with new parameters, in which case companies are to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc - Float32 adopted as the baseline/upper-bound for performance comparisons *** Unchanged text is omitted *** 6.2.2 Performance results Monitoring for intermediate KPI, NW side monitoring For the evaluation of intermediate KPI based monitoring mechanism for CSI compression, for monitoring Case 1, in terms of monitoring accuracy with Option 1, - For ground-truth CSI format of R16 eType II CB, monitoring accuracy is increased with the increase of the resolution for the ground-truth CSI (number of bits for each sample of ground-truth CSI) in general, with the impact of increased overhead, wherein - for ground-truth CSI format of R16 eType II CB with PC#6, 4 sources observe KPIDiff as 13.2%~71.6%/ 28.5%~100%/ 68.4%~100% for KPIth_1=0.02/0.05/0.1, respectively. - Note: two sources observed averaging on the test samples improves the monitoring accuracy. - for ground-truth CSI format of R16 eType II CB with PC#8, 5 sources observe KPIDiff as 21%~43.0%/ 48.1%~79.1%/ 79.8%~97.1% for KPIth_1=0.02/0.05/0.1, respectively. - for ground-truth CSI format of R16 eType II CB with new parameter of 580-750bits CSI payload size, 2 sources observe KPIDiff as 35.4%~63%/ 77.9%~93.0%/ 99.5%~99.9% for KPIth_1=0.02/0.05/0.1, respectively, which have 12.7%~20%/ 13.9%~29.8%/ 8%~31.1% gain over PC#8. - for ground-truth CSI format of R16 eType II CB with new parameter of around 1000bits CSI payload size, 4 sources observe KPIDiff as 34.9%~89%/ 82.9%~100%/ 99.9%~100% for KPIth_1=0.02/0.05/0.1, respectively, which have 12.2%~68%/ 18%~43.62%/ 2.9%~31% gain over PC#8 from 3 sources and 4.67%~10.6%/ 0%~5.88%/ 0%~0.49% gain over PC#6 from 1 source. - for ground-truth CSI format of R16 eType II CB with new parameter of around 1600bits CSI payload size, 2 sources observe KPIDiff as 89.1%~97%/ 99.9%~100%/ 100% for KPIth_1=0.02/0.05/0.1, respectively, which have 76%/33%/3% gain over PC#8 from 1 source. - for ground-truth CSI format of 4 bits scalar quantization, 2 sources observe KPIDiff as 9.4%~47%/ 96.3%~100%/ 100% for KPIth_1=0.02/0.05/0.1, respectively. *** Unchanged text is omitted *** Monitoring for intermediate KPI, UE side monitoring For the evaluation of intermediate KPI based monitoring mechanism for CSI compression, for Case 2, in terms of monitoring accuracy with Option 1, - For Case 2-1 subject to generalization Case 1 for the proxy model, 5 sources observe KPIDiff as 31%~84%/ 65.63%~99.8%/ 95%~100% for KPIth_1=0.02/0.05/0.1, respectively; - Compared with monitoring Case 1 with ground-truth CSI format of R16 eType II CB with new parameter of around 1000bits CSI payload size, - 2 sources observe +0.99%~+4.07% gain at KPIth_1=0.02; - 3 sources observe -6.03%~-58%/ -0.2%~-24%/ 0%~-5% degradation for KPIth_1=0.02/0.05/0.1, respectively; - Compared with monitoring Case 1 with ground-truth CSI format of R16 eType II CB with new parameter of around 1600bits CSI payload size, 2 sources observe -16.35%~-66%/ -0.4%~-24%/ 0%~-24% degradation for KPIth_1=0.02/0.05/0.1, respectively. - Note: For Case 2-1 subject to generalization Case 2 for the proxy model, 2 sources observe -1.77%~-37.42% / -1.07%~-23.93%/ -0.16%~-14% compared with generalization Case 1 with the same testing scenario. *** Unchanged text is omitted *** |  |

| Support/Can accept | LG Electronics, Futurewei, Qualcom, ZTE, Ericsson, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### Issue 4-3: Link-level simulations not evaluated

Motivation[3]: In TR38.843 v1.1.0, link level simulation assumption tables are provided for CSI. However, no link level simulation was provided so far. If no link level simulation results are provided at the end of the study item, it is proposed that the link level simulation assumption tables and related texts are deleted from the TR.

6.2.1 Evaluation assumptions, methodology and KPIs For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline. Link level simulations are optionally adopted. *** omitted *** Table 6.2.1-2 presents the baseline link level simulation assumptions for AI/ML based CSI feedback enhancement evaluations. Table 6.2.1-2: Baseline Link Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations Parameter; Value. Duplex, Waveform; FDD (TDD is not precluded), OFDM. Carrier frequency; 2GHz as baseline, optional for 4GHz. Bandwidth; 10MHz or 20MHz. Subcarrier spacing; 15kHz for 2GHz, 30kHz for 4GHz. Nt; 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ. Nr; 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ. Channel model; CDL-C as baseline, CDL-A as optional. UE speed; 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies. Delay spread; 30ns or 300ns. Channel estimation; Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline, FFS ideal channel estimation. Rank per UE; Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied. *** omitted ***

Moderator assessment: Though the LLS results are not captured to the observations/collected results, the EVM may be helpful for readers/future releases/other WGs such as RAN4, who can borrow the LLS EVM for AI/ML based CSI for futher simulation rather than discussing from scratch.

Question 4.1.1: Do you think it is needed to remove the LLS related EVM descritpions from Section 6.2.1 of TR 38.843?

| Yes/Remove LLS EVM |  |
| --- | --- |
| No /Keep LLS EVM | LG Electronics |

| Company | View |
| --- | --- |
| Qualcomm | The spreadsheet for Table 2 seems to include some results using CDL-C channel model. |
| Ericsson | Ok with either keep or remove LSS EVM. If keep the LLS EVM, then the LLS parameter setting shall be aligned with the SLS parameter settings, e.g., the Nt and Nr values. |
| Moderator | @Ericsson Not clear what is the action to align the parameter settings between LLS and SLS. The LLS assumption for Nt/Nr is already one typical value in SLS. |

  - 4th round email discussions

### Issue 4-1: Description of generalization for CSI enhancement

Proposal 4.2.1: Adopt the following TP related with changes to the generalization part to TR 38.843.

| Issue#4-1 Changes to the generalization part | Description of issue: The descritptions for generalization cases in the EVM sub-section (Case 1, 2, 3: “Train-on-#A/Test-on-#A”, “Train-on-#A/Test-on-#B”, and “Train-on-#A+B/Test-on-#B”) is inconsistent with the observation sub-section (Case 1, 2, 3: “Train-on-#B/Test-on-#B”, “Train-on-#A/Test-on-#B”, and “Train-on-#A+B/Test-on-#B”). |
| --- | --- |
| ------------------ Text Proposal for 38.843 v1.2.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** Model generalization: In order to study the verification of generalization, the following aspects are encouraged to be reported: - The configuration(s)/scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios - The configuration(s)/scenario(s) for testing/inference - The detailed list of configuration(s) and/or scenario(s) The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A Scenario#B/Configuration#B, and then - the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A Scenario#B/Configuration#B - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A a different dataset than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on Scenario#B/Configuration#B a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A Scenario#B/Configuration#B and a different dataset than Scenario#A/Configuration#A Scenario#B/Configuration#B, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from Scenario#B/Configuration#B a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two *** Unchanged text is omitted *** |  |

| Support/Can accept | ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |

### Issue 4-2: Editorial change on aligning the term of “ground-truth CSI”

Proposal 4.2.2: Adopt the following TP related with editorial changes on “ground truth CSI” to TR 38.843.

| Issue#4-2 Changes to align the term of “ground-truth CSI” | Description of issue: The TR is using both the terms “ground truth CSI” (10 places) and “ground-truth CSI” (18 places), which are not aligned. |
| --- | --- |
| ------------------ Text Proposal for 38.843 v1.2.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** CSI compression sub use case specific aspects: *** Unchanged text is omitted *** - Considering performance impact of ground-truth quantization in the CSI compression - Studying high resolution quantization methods for ground-truth CSI, including at least the following options: - High resolution scalar quantization - High resolution codebook quantization, e.g., Rel-16 TypeII-like method with new parameters, in which case companies are to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc - Float32 adopted as the baseline/upper-bound for performance comparisons *** Unchanged text is omitted *** 6.2.2 Performance results Monitoring for intermediate KPI, NW side monitoring For the evaluation of intermediate KPI based monitoring mechanism for CSI compression, for monitoring Case 1, in terms of monitoring accuracy with Option 1, - For ground-truth CSI format of R16 eType II CB, monitoring accuracy is increased with the increase of the resolution for the ground-truth CSI (number of bits for each sample of ground-truth CSI) in general, with the impact of increased overhead, wherein - for ground-truth CSI format of R16 eType II CB with PC#6, 4 sources observe KPIDiff as 13.2%~71.6%/ 28.5%~100%/ 68.4%~100% for KPIth_1=0.02/0.05/0.1, respectively. - Note: two sources observed averaging on the test samples improves the monitoring accuracy. - for ground-truth CSI format of R16 eType II CB with PC#8, 5 sources observe KPIDiff as 21%~43.0%/ 48.1%~79.1%/ 79.8%~97.1% for KPIth_1=0.02/0.05/0.1, respectively. - for ground-truth CSI format of R16 eType II CB with new parameter of 580-750bits CSI payload size, 2 sources observe KPIDiff as 35.4%~63%/ 77.9%~93.0%/ 99.5%~99.9% for KPIth_1=0.02/0.05/0.1, respectively, which have 12.7%~20%/ 13.9%~29.8%/ 8%~31.1% gain over PC#8. - for ground-truth CSI format of R16 eType II CB with new parameter of around 1000bits CSI payload size, 4 sources observe KPIDiff as 34.9%~89%/ 82.9%~100%/ 99.9%~100% for KPIth_1=0.02/0.05/0.1, respectively, which have 12.2%~68%/ 18%~43.62%/ 2.9%~31% gain over PC#8 from 3 sources and 4.67%~10.6%/ 0%~5.88%/ 0%~0.49% gain over PC#6 from 1 source. - for ground-truth CSI format of R16 eType II CB with new parameter of around 1600bits CSI payload size, 2 sources observe KPIDiff as 89.1%~97%/ 99.9%~100%/ 100% for KPIth_1=0.02/0.05/0.1, respectively, which have 76%/33%/3% gain over PC#8 from 1 source. - for ground-truth CSI format of 4 bits scalar quantization, 2 sources observe KPIDiff as 9.4%~47%/ 96.3%~100%/ 100% for KPIth_1=0.02/0.05/0.1, respectively. *** Unchanged text is omitted *** Monitoring for intermediate KPI, UE side monitoring For the evaluation of intermediate KPI based monitoring mechanism for CSI compression, for Case 2, in terms of monitoring accuracy with Option 1, - For Case 2-1 subject to generalization Case 1 for the proxy model, 5 sources observe KPIDiff as 31%~84%/ 65.63%~99.8%/ 95%~100% for KPIth_1=0.02/0.05/0.1, respectively; - Compared with monitoring Case 1 with ground-truth CSI format of R16 eType II CB with new parameter of around 1000bits CSI payload size, - 2 sources observe +0.99%~+4.07% gain at KPIth_1=0.02; - 3 sources observe -6.03%~-58%/ -0.2%~-24%/ 0%~-5% degradation for KPIth_1=0.02/0.05/0.1, respectively; - Compared with monitoring Case 1 with ground-truth CSI format of R16 eType II CB with new parameter of around 1600bits CSI payload size, 2 sources observe -16.35%~-66%/ -0.4%~-24%/ 0%~-24% degradation for KPIth_1=0.02/0.05/0.1, respectively. - Note: For Case 2-1 subject to generalization Case 2 for the proxy model, 2 sources observe -1.77%~-37.42% / -1.07%~-23.93%/ -0.16%~-14% compared with generalization Case 1 with the same testing scenario. *** Unchanged text is omitted *** |  |

### Issue 4-3 (Closed): Link-level simulations not evaluated

Moderator note: Little feedback in previous rounds. This issue is deemed as not critical and is closed.

### Issue 4-4 (new): Correction to the number of Mean UPT under RU≤39%, Max rank=4

Moderator note: As mentioned when drawing high-level observation for mean UPT, the results in the TR observation in section 6.2.2.1 for mean UPT, RU<=39% have some mistake. Now it is corrected based on table 5.12 of R1-2308432 (source table to derive the text of the observation) - Upper bound corrected to 6%, and 3 sources changed to 2.

Proposal 4.2.3: Adopt the following TP related with editorial changes on “ground truth CSI” to TR 38.843.

| Issue#4-4 Correction to the number of Mean UPT under RU≤39%, Max rank=4 | Description of issue: Simulation results from Table 5.12, R1-2308432 wrongly captured to the observation text. |
| --- | --- |
| ------------------ Text Proposal for 38.843 v1.2.0 ------------------ - 6.2.2 Performance results *** Unchanged text is omitted *** - 6.2.2.1 1-on-1 joint training for CSI compression *** Unchanged text is omitted *** Mean UPT for FTP traffic For the evaluation of AI/ML based CSI compression compared to the benchmark in terms of mean UPT under FTP traffic, more gains are achieved by Max rank 2 compared with Max rank 1 in general: *** Unchanged text is omitted *** - For Max rank 4: - For RU≤39%, 2 3 sources observe the performance gain of -4%~6% 7.4% - 2 3 sources observe the performance gain of 2.5%~6% 7.4% at CSI overhead A (small overhead); - 1 source observes the performance gain of 6% at CSI overhead B (medium overhead); - 2 sources observe the performance gain of -4%~0% at CSI overhead C (large overhead); - For RU 40%-69%, 3 sources observe the performance gain of -1.8%~12.22% - 3 sources observe the performance gain of 3%~12.22% at CSI overhead A (small overhead); - 2 sources observe the performance gain of 7.04%~11% at CSI overhead B (medium overhead); - 3 sources observe the performance gain of -1.8%~8.19% at CSI overhead C (large overhead); - For RU≥70%, 3 sources observe the performance gain of -1%~17% - 3 sources observe the performance gain of 3%~17% at CSI overhead A (small overhead); - 2 sources observe the performance gain of 6.64%~17% at CSI overhead B (medium overhead); - 3 sources observe the performance gain of -1%~8.40% at CSI overhead C (large overhead); - Note: 1 source observes significant gain or significant loss under Max rank 4 due to specific CQI/RI selection method (e.g., Option 1a/2a) for AI/ML and/or CQI/RI determination method for eType II benchmark. The above results are based on the following assumptions besides the assumptions of the agreed EVM table: - Precoding matrix of the current CSI is used as the model input. - Training data samples are not quantized, i.e., Float32 is used/represented. - 1-on-1 joint training is assumed. - The performance metric is mean UPT for Max rank 1, Max rank 2, or Max rank 4. - Benchmark is Rel-16 Type II codebook. - Note: Results refer to Table 5.12 of R1-2308342. *** Unchanged text is omitted *** |  |

### Issue 4-5 (new): Capturing high-level observations to TR

Moderator note: Capture all agreed high-level observations in #115 meeting to TR38.843 v1.2.0.

Note: for the following agreement achieved in Wed. GTW, “RU> 70%” should be corrected to “RU≥70%” based on offline comments. The correction is reflected to the TP. Sorry for the typo.

Agreement Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843 - From the perspective of basic performance gain over non-AI/ML benchmark, AI/ML based CSI compression outperforms Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where - 0.2%~2%/-0.3%~6%/-4%~6% 7.4% gains of mean UPT as shown in Figure X1~Figure X3 are observed for Max rank 1/2/4, respectively, under RU≤39%. - 0.1%~4%/-0.5%~10%/-1.8%~12.22% gains of mean UPT as shown in Figure Y1~Figure Y3 are observed for Max rank 1/2/4, respectively, under RU40%-69%. - 0.23%~9%/-0.2%~15%/-1%~17% gains of mean UPT as shown in Figure Z1~Figure Z3 are observed for Max rank 1/2/4, respectively, under RU≥ >70%.

Proposal 4.2.4: Adopt the following TP to capture high-level observations drawn in #115 meeting to TR 38.843.

| Issue#4-5 Capturing high-level observations to TR | Description of issue: Capture all high-level observations drawn in #115 meeting to TR38.843 v1.2.0. |
| --- | --- |
| ------------------ Text Proposal for 38.843 v1.2.0 ------------------ - 6.2.2.8 Summary of Performance Results for CSI feedback enhancement *** Unchanged text is omitted *** The following aspects have been studied for the evaluation on AI/ML based CSI prediction: - From the perspective of basic performance gain over non-AI/ML benchmark (without considering generalization), - It has been studied with corresponding observations on: - the metrics of SGCS, mean UPT, 5% UPT; - the benchmarks of nearest historical CSI and auto-regression/Kalman filter based CSI prediction. - Note: the benchmark of level x based CSI prediction is represented by generalization cases. - It has been studied but is lack of observations on: - the impact of modeling spatial consistency - the metrics of NMSE - It has been studied with corresponding observations on complexity but without comparison with non-AI/ML - From the perspective of AI/ML solutions (without considering generalization), - It has been studied with corresponding observations on (with the metric of SGCS and the benchmark of nearest historical CSI): impact of input type, impact of UE speed, impact of prediction window, impact of observation window - From the perspective of generalization over various scenarios, - It has been studied with corresponding observations on (with the metric of SGCS): - the scenario including various UE speeds - the approach of dataset mixing (generalization Case 3) - It has been studied but is lack of observations on: - various deployment scenarios, various carrier frequencies, and other aspects of scenarios. - the approach of fine-tuning - From the perspective of scalability over various configurations, it has been studied but is lack of observations. Based on the evaluation for CSI compression, the following high-level observations are provided: - From the perspective of basic performance gain over non-AI/ML benchmark, AI/ML based CSI compression outperforms Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where - 0.2%~2%/-0.3%~6%/-4%~6% gains of mean UPT as shown in Figure 6.2.2.8-1~ Figure 6.2.2.8-3 are observed for Max rank 1/2/4, respectively, under RU≤39%. - 0.1%~4%/-0.5%~10%/-1.8%~12.22% gains of mean UPT as shown in Figure 6.2.2.8-4~ Figure 6.2.2.8-5 are observed for Max rank 1/2/4, respectively, under RU40%-69%. - 0.23%~9%/-0.2%~15%/-1%~17% gains of mean UPT as shown in Figure 6.2.2.8-7~ Figure 6.2.2.8-8 are observed for Max rank 1/2/4, respectively, under RU≥70%. Figure 6.2.2.8-1: Mean UPT gain, Max Rank 1 (RU≤39%), x-axis means index of source Figure 6.2.2.8-2: Mean UPT gain, Max Rank 2 (RU≤39%), x-axis means index of source Figure 6.2.2.8-3: Mean UPT gain, Max Rank 4 (RU≤39%), x-axis means index of source Figure 6.2.2.8-4: Mean UPT gain, Max Rank 1 (RU40%-69%), x-axis means index of source Figure 6.2.2.8-5: Mean UPT gain, Max Rank 2 (RU40%-69%), x-axis means index of source Figure 6.2.2.8-6: Mean UPT gain, Max Rank 4 (RU40%-69%), x-axis means index of source Figure 6.2.2.8-7: Mean UPT gain, Max Rank 1 (RU>70%), x-axis means index of source Figure 6.2.2.8-8: Mean UPT gain, Max Rank 2 (RU>70%), x-axis means index of source Figure 6.2.2.8-9: Mean UPT gain, Max Rank 4 (RU>70%), x-axis means index of source - From the perspective of CSI overhead reduction over non-AI/ML, AI/ML based CSI compression achieves CSI feedback reduction compared with Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where 4 sources observe the CSI feedback overhead reduction of 10.24%~60%/10%~58.33%/8%~79% for Max rank 1/2/4, respectively, under FTP traffic. - From the perspective of AI/ML complexity, a majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M; on the other hand, the actual model complexity may differ from the model complexity in the evaluation with respect to platform-dependent optimization on model implementations. In addition, the complexity between AI/ML and non-AI/ML benchmark is not compared. - From the perspective of model input/output type, it is more beneficial by considering precoding matrix as the model input (for CSI generation part)/output (for CSI reconstruction part) than explicit channel matrix. - From the perspective of intermediate KPI based monitoring, - For the monitoring at NW side, increased monitoring accuracy can be achieved by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for monitoring. On the other hand, the new/larger parameter(s) would lead to increased air-interface overhead compared to R16 eType II CB with legacy parameters. - For the monitoring at UE side, performance can be monitored with smaller air-interface overhead by considering proxy model at UE compared with monitoring at NW side. On the other hand, the monitoring accuracy may be impacted by the design/robustness of the proxy model. - Note: the complexity aspect for Case 1, Case 2-1 and Case 2-2 is not evaluated. - From the perspective of quantization methods for CSI feedback, - For the quantization awareness for training, it is beneficial to consider quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) or jointly updated quantization method/parameters (Case 2-2) to avoid severe performance degradation. In particular, it is more beneficial in performance for Case 2-2 over Case 2-1 under vector quantization format (VQ). - For the quantization format, VQ format achieves comparable performance with scalar quantization format (SQ) in general, where VQ achieves better performance than SQ in some cases while worse in some other cases. - From the perspective of high resolution ground-truth CSI for training, compared to unquantized ground-truth CSI (e.g., Float32), taking R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for training data collection can achieve significant overhead reduction without causing severe performance degradation; taking scalar quantization format for training data collection can achieve moderate overhead reduction without causing severe performance degradation. On the other hand, the R16 eType II CB with new/larger parameter(s) would lead to increased overhead compared to R16 eType II CB with legacy parameters - For ground-truth CSI format, 5 sources observe R16 eType II CB with new/larger parameter(s) outperforms R16 eType II CB with legacy parameter, while one source observes R16 eType II CB with legacy parameter is already close to Float32 with particular dataset processing technique. - Note: the complexity aspect is not evaluated. - From the perspective of generalization over scenarios, or scalability over configurations that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B, - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others. - For generalization Case 3 where the training dataset is constructed with data samples subject to more than one scenario/configuration (evaluations studied up to four scenarios/configurations) including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved. - In particular, appropriate scalability solution (e.g., truncation/padding, adaptive quantization granularities, adaptation layer in the AI/ML model) may need to be performed to scale the dimensions of the AI/ML model when the training dataset includes data samples subject to configuration#A which has different input/output dimension than configuration#B. - From the perspective of training collaboration types, compared to 1-on-1 joint training, both multi-vendor joint training and separate training with procedures given in Section 6.2.1 may suffer performance loss. - In particular, for multi-vendor joint training, minor or moderate degradation is observed. - In particular, for separate training with procedure given in Section 6.2.1, the performance loss depends on the factors such as backbone alignment, and multi-vendor training behavior: - For separate training of 1 NW part model and 1 UE part model, under both NW first training and UE first training, if backbones are aligned between the two sides, minor degradation is observed; otherwise, additional degradation is observed, leading to minor or moderate performance degradation. - For NW first training with 1 UE part model to N>1 NW part models, or UE first training with 1 NW part model to M>1 UE part models, additional degradation is observed, leading to minor, moderate, or significant performance degradation, depending on the training approach. - As a note, other procedures of separate training are not extensively evaluated. Based on the evaluation for CSI prediction, the following high-level observations are provided: - From the perspective of basic performance gain over non-AI/ML benchmark, under the same UE speed for training and inference, - AI/ML based CSI prediction outperforms the benchmark of the nearest historical CSI in general, where the majority of sources observe up to 10.6% gain in terms of mean UPT. - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3 sources observe up to 0.7%~7% gain while 2 sources observe performance loss of -0.1%~-17% in terms of mean UPT. - From the perspective of model input/output type, it is more beneficial in performance by considering raw channel matrix as the model input than precoding matrix - The gain of AI/ML based CSI prediction over the benchmark of the nearest historical CSI is impacted by the observation window length, prediction window length, and UE speed - From the perspective of generalization over UE speeds that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B, - For generalization Case 2 where the AI/ML model is trained with dataset from a different UE speed#A, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, generalized performance of the AI/ML model can be achieved in general *** Unchanged text is omitted *** |  |

- Potential proposals for GTW/offline

  - 14 Nov (Tue.) GTW/Offline

### Issue#2-1 High level observation - Basic performance gain over non-AI/ML

Upd Proposal 2.1.1: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of basic performance gain over non-AI/ML benchmark, AI/ML based CSI compression outperforms Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where

  - 0.2%~2%/-0.3%~6%/-4%~7.4% gains of mean UPT as shown in in Figure X1~Figure X3 are observed for Max rank 1/2/4, respectively, under RU≤39%

  - 0.1%~4%/-0.5%~10%/-1.8%~12.22% gains of mean UPT as shown in in Figure Y1~Figure Y3 are observed for Max rank 1/2/4, respectively,  under RU40%-69%

  - 0.23%~9%/-0.2%~15%/-1%~17% gains of mean UPT as shown in in Figure Z1~Figure Z3 are observed for Max rank 1/2/4, respectively, under RU>70%

Figure Z1 Mean UPT gain, Max Rank 1 (RU>70%), x-axis means index of source

Figure Z2 Mean UPT gain, Max Rank 2 (RU>70%), x-axis means index of source

Figure Z3 Mean UPT gain, Max Rank 4 (RU>70%), x-axis means index of source

### Issue#2-2 High level observation - Complexity analysis for ground-truth CSI

Upd Proposal 2.1.2: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of intermediate KPI based monitoring

  - For the monitoring at NW side, increased monitoring accuracy can be achieved by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for monitoring. On the other hand, the new/larger parameter(s) would lead to increased air-interface overhead compared to R16 eType II CB with legacy parameters.

  - For the monitoring at UE side, performance can be monitored with smaller air-interface overhead by considering proxy model at UE compared with monitoring at NW side. On the other hand, the monitoring accuracy may be impacted by the design/robustness of the proxy model.

  - Note: the complexity aspect for Case 1, Case 2-1 and Case 2-2 is not evaluated

- From the perspective of high resolution ground-truth CSI for training, compared to unquantized ground-truth CSI (e.g., Float32), taking R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for training data collection can achieve significant overhead reduction without causing severe performance degradation; taking scalar quantization format for training data collection can achieve moderate overhead reduction without causing severe performance degradation. On the other hand, the R16 eType II CB with new/larger parameter(s) would lead to increased overhead compared to R16 eType II CB with legacy parameters

  - For ground-truth CSI format, 5 sources observe R16 eType II CB with new/larger parameter(s) outperforms R16 eType II CB with legacy parameter, while one source observes R16 eType II CB with legacy parameter is already close to Float32 with particular dataset processing technique

  - Note: the complexity aspect is not evaluated

### Issue#2-3 High level observation – other aspects

Upd Proposal 2.1.3: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of model input/output type, it is more beneficial by considering precoding matrix as the model input (for CSI generation part)/output (for CSI reconstruction part) than explicit channel matrix

- From the perspective of quantization methods for CSI feedback,

  - For the quantization awareness for training, it is beneficial to consider quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) or jointly updated quantization method/parameters (Case 2-2) to avoid severe performance degradation. In particular, it is more beneficial in performance for Case 2-2 over Case 2-1 under vector quantization format (VQ).

  - For the quantization format, VQ format achieves comparable performance with scalar quantization format (SQ) in general, where VQ achieves better performance than SQ in some cases while worse in some other cases

- From the perspective of generalization over scenarios, or scalability over configurations that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B,

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others

  - For generalization Case 3 where the training dataset is constructed with data samples subject to more than one scenario/configuration (evaluations studied up to four scenarios/configurations) including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved

  - In particular, appropriate scalability solution (e.g., truncation/padding, adaptive quantization granularities, adaptation layer in the AI/ML model) may need to be performed to scale the dimensions of the AI/ML model when the training dataset includes data samples subject to configuration#A which has different input/output dimension than configuration#B

- From the perspective of training collaboration types, compared to 1-on-1 joint training, both multi-vendor joint training and separate training with procedures given in Section 6.2.1 may suffer performance loss.

  - In particular, for multi-vendor joint training, minor or moderate degradation is observed

  - In particular, for separate training with procedure given in Section 6.2.1, the performance loss depends on the factors such as backbone alignment, and multi-vendor training behavior:

    - For separate training of 1 NW part model and 1 UE part model, under both NW first training and UE first training, if backbones are aligned between the two sides, minor degradation is observed; otherwise, additional degradation is observed, leading to minor or moderate performance degradation

    - For NW first training with 1 UE part model to N>1 NW part models, or UE first training with 1 NW part model to M>1 UE part models, additional degradation is observed, leading to minor, moderate, or significant performance degradation, depending on the training approach

    - As a note, other procedures of separate training are not extensively evaluated

### Issue#3-1 High level observation for CSI prediction

Upd Proposal 3.1.1: Capture the following high-level observation for CSI prediction to section 6.2.2.8 of TR 38.843

- From the perspective of basic performance gain over non-AI/ML benchmark, under the same UE speed for training and inference,

  - AI/ML based CSI prediction outperforms the benchmark of the nearest historical CSI in general, where the majority of sources observe up to 10.6% gain in terms of mean UPT.

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3 sources observe up to 0.7%~7% gain while 2 sources observe performance loss of -0.1%~-17% in terms of mean UPT.

  - 15 Nov (Wed.) GTW

### Issue#2-2 High level observation - Complexity analysis for ground-truth CSI

Proposal 2.2.4: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of intermediate KPI based monitoring

  - For the monitoring at NW side, increased monitoring accuracy can be achieved by considering R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for monitoring. On the other hand, the new/larger parameter(s) would lead to increased air-interface overhead compared to R16 eType II CB with legacy parameters.

  - For the monitoring at UE side, performance can be monitored with smaller air-interface overhead by considering proxy model at UE compared with monitoring at NW side. On the other hand, the monitoring accuracy may be impacted by the design/robustness of the proxy model.

  - Note: the complexity aspect for Case 1, Case 2-1 and Case 2-2 is not evaluated

- From the perspective of high resolution ground-truth CSI for training, compared to unquantized ground-truth CSI (e.g., Float32), taking R16 eType II CB with new/larger parameter(s) as the ground-truth CSI format for training data collection can achieve significant overhead reduction without causing severe performance degradation; taking scalar quantization format for training data collection can achieve moderate overhead reduction without causing severe performance degradation. On the other hand, the R16 eType II CB with new/larger parameter(s) would lead to increased overhead compared to R16 eType II CB with legacy parameters

  - For ground-truth CSI format, 5 sources observe R16 eType II CB with new/larger parameter(s) outperforms R16 eType II CB with legacy parameter, while one source observes R16 eType II CB with legacy parameter is already close to Float32 with particular dataset processing technique

  - Note: the complexity aspect is not evaluated

### Issue#2-3 High level observation – other aspects

Proposal 2.2.5: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of model input/output type, it is more beneficial by considering precoding matrix as the model input (for CSI generation part)/output (for CSI reconstruction part) than explicit channel matrix

- From the perspective of quantization methods for CSI feedback,

  - For the quantization awareness for training, it is beneficial to consider quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) or jointly updated quantization method/parameters (Case 2-2) to avoid severe performance degradation. In particular, it is more beneficial in performance for Case 2-2 over Case 2-1 under vector quantization format (VQ).

  - For the quantization format, VQ format achieves comparable performance with scalar quantization format (SQ) in general, where VQ achieves better performance than SQ in some cases while worse in some other cases

- From the perspective of generalization over scenarios, or scalability over configurations that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B,

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others

  - For generalization Case 3 where the training dataset is constructed with data samples subject to more than one scenario/configuration (evaluations studied up to four scenarios/configurations) including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved

  - In particular, appropriate scalability solution (e.g., truncation/padding, adaptive quantization granularities, adaptation layer in the AI/ML model) may need to be performed to scale the dimensions of the AI/ML model when the training dataset includes data samples subject to configuration#A which has different input/output dimension than configuration#B

- From the perspective of training collaboration types, compared to 1-on-1 joint training, both multi-vendor joint training and separate training with procedures given in Section 6.2.1 may suffer performance loss.

  - In particular, for multi-vendor joint training, minor or moderate degradation is observed

  - In particular, for separate training with procedure given in Section 6.2.1, the performance loss depends on the factors such as backbone alignment, and multi-vendor training behavior:

    - For separate training of 1 NW part model and 1 UE part model, under both NW first training and UE first training, if backbones are aligned between the two sides, minor degradation is observed; otherwise, additional degradation is observed, leading to minor or moderate performance degradation

    - For NW first training with 1 UE part model to N>1 NW part models, or UE first training with 1 NW part model to M>1 UE part models, additional degradation is observed, leading to minor, moderate, or significant performance degradation, depending on the training approach

    - As a note, other procedures of separate training are not extensively evaluated

### Issue#3-1 High level observation for CSI prediction

Proposal 3.2.1: Capture the following high-level observation for CSI prediction to section 6.2.2.8 of TR 38.843

- From the perspective of basic performance gain over non-AI/ML benchmark, under the same UE speed for training and inference,

  - AI/ML based CSI prediction outperforms the benchmark of the nearest historical CSI in general, where the majority of sources observe up to 10.6% gain in terms of mean UPT.

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3 sources observe up to 0.7%~7% gain while 2 sources observe performance loss of -0.1%~-17% in terms of mean UPT.

### Issue#2-1 High level observation - Basic performance gain over non-AI/ML

Moderator note: Updated based on Tue off-offline discussion.

Note: for Max rank 4, RU≤39%, “-4%~7.4%” in TR is changed to “-4%~6%” - In R1-2308342, the upper bound result in the observation seems wrongly captured from Table 5.12, to the observation text.

Proposal 2.2.1: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of basic performance gain over non-AI/ML benchmark, AI/ML based CSI compression outperforms Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where

  - 0.2%~2%/-0.3%~6%/-4%~6% 7.4% gains of mean UPT as shown in Figure X1~Figure X3 are observed for Max rank 1/2/4, respectively, under RU≤39%

  - 0.1%~4%/-0.5%~10%/-1.8%~12.22% gains of mean UPT as shown in Figure Y1~Figure Y3 are observed for Max rank 1/2/4, respectively, under RU40%-69%

  - 0.23%~9%/-0.2%~15%/-1%~17% gains of mean UPT as shown in Figure Z1~Figure Z3 are observed for Max rank 1/2/4, respectively, under RU>70%

Figure X1 Mean UPT gain, Max Rank 1 (RU≤39%), x-axis means index of source

Figure X2 Mean UPT gain, Max Rank 2 (RU≤39%), x-axis means index of source

Figure X3 Mean UPT gain, Max Rank 4 (RU≤39%), x-axis means index of source

Figure Y1 Mean UPT gain, Max Rank 1 (RU40%-69%), x-axis means index of source

Figure Y2 Mean UPT gain, Max Rank 2 (RU40%-69%), x-axis means index of source

Figure Y3 Mean UPT gain, Max Rank 4 (RU40%-69%), x-axis means index of source

Figure Z1 Mean UPT gain, Max Rank 1 (RU>70%), x-axis means index of source

Figure Z2 Mean UPT gain, Max Rank 2 (RU>70%), x-axis means index of source

Figure Z3 Mean UPT gain, Max Rank 4 (RU>70%), x-axis means index of source

  - 15 Nov (Wed.) Offline

### Issue#2-1a (new) High level observation – CSI overhead reduction over non-AI/ML

Moderator note: Newly added based on Tue off-offline discussion. The observation merges the CSI feedback reduction over cases of CSI overhead A/B/C. Only FTP traffic is taken into account.

Proposal 2.2.2: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of CSI overhead reduction over non-AI/ML, AI/ML based CSI compression achieves CSI feedback reduction compared with Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where 4 sources observe the CSI feedback overhead reduction of 10.24%~60%/10%~58.33%/8%~79% for Max rank 1/2/4, respectively, under FTP traffic.

### Issue#2-1b (new) High level observation – AI/ML complexity

Proposal 2.2.3: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of AI/ML complexity, a majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M; on the other hand, the actual model complexity may differ from the model complexity in the evaluation with respect to platform-dependent optimization on model implementations. In addition, the complexity between AI/ML and non-AI/ML benchmark is not compared.

  - 16 Nov (Thu.) GTW

### Issue#2-1a High level observation – CSI overhead reduction over non-AI/ML

Proposal 2.3.1: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of CSI overhead reduction over non-AI/ML, AI/ML based CSI compression achieves CSI feedback reduction compared with Rel-16 eType II CB in general under 1-on-1 joint training and generalization Case 1, where 4 sources observe the CSI feedback overhead reduction of 10.24%~60%/10%~58.33%/8%~79% for Max rank 1/2/4, respectively, under FTP traffic.

### Issue#2-1b High level observation – AI/ML complexity

Proposal 2.3.2: Capture the following high-level observations for CSI compression to section 6.2.2.8 of TR 38.843

- From the perspective of AI/ML complexity, a majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M; on the other hand, the actual model complexity may differ from the model complexity in the evaluation with respect to platform-dependent optimization on model implementations. In addition, the complexity between AI/ML and non-AI/ML benchmark is not compared.

- References

- R1-2310163, TR 38.843 v1.1.0, RAN1#114bis, Xiamen, October 9-13, 2023.

- R1-2310846, Highlights for the evaluation on AI/ML based CSI feedback enhancement, Huawei, HiSilicon, RAN1#115, Chicago, November 13-17, 2023.

- R1-2310933, Text Proposals to TR 38.843, Ericsson, RAN1#115, Chicago, November 13-17, 2023.

- Appendix I: Agreement list

### Agreements of the 109-e meeting

Agreement

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: How to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

Agreement (modified by May 23rd post)

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

Note: Above agreement is updated as follows

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - Additional assumptions from R17 TypeII EVM Same consideration with respect to utilizing angle-delay reciprocity should be considered taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all layers

  - Note:  is the eigenvector of the target CSI at resource unit i and K is the rank. is the  output vector of the output CSI of resource unit i.  is the total number of resource units.  denotes the average operation over multiple samples.

- Method 2: Weighted average over all layers

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

### Agreements of the 110 meeting

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

  - Note: Companies to report the ratio for dataset mixing

  - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted.

Agreement

For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point:

- Various deployment scenarios (e.g., UMa, UMi, InH)

- Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10)

- Various carrier frequencies (e.g., 2GHz, 3.5GHz)

- Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc.

- Companies to report the selected scenarios for generalization verification

Conclusion

If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point.

Agreement

For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point:

- Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband)

- Various sizes of CSI feedback payloads, FFS candidate payload number

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc.

- Companies to report the selected configurations for generalization verification

- Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the specific quantization/dequantization method, e.g., vector quantization, scalar quantization, etc.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, the capability/complexity related KPIs, including FLOPs as well as AI/ML model size and/or number of AI/ML parameters, are to be reported separately for the CSI generation part and the CSI reconstruction part.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation,

- 100% outdoor UE is assumed for UE distribution.

  - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

  - Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including:

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

### Agreements of the 110bis-e meeting

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

| Traffic model | At least, FTP model 1 with packet size 0.5 Mbytes is assumed Other options are not precluded. |
| --- | --- |

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, if realistic DL channel estimation is considered, regarding how to calculate the intermediate KPI of CSI accuracy,

- Use the target CSI from ideal channel and use output CSI from the realistic channel estimation

  - The target CSI from ideal channel equally applies to AI/ML based CSI feedback enhancement, and the baseline codebook

Note: there is no restriction on model training

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for “Baseline for performance evaluation” in the EVM table, Type I Codebook (if it outperforms Type II Codebook) can be optionally considered for comparing AI/ML schemes up to companies

- Note: Type II Codebook is baseline as agreed

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the outdoor UEs, add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, no explicit trajectory modeling is considered for evaluation

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, and if the AI/ML model outputs multiple predicted instances, the intermediate KPI is calculated for each prediction instance

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, both of the following types of AI/ML model input are considered for evaluations:

- Raw channel matrixes

- Eigenvector(s)

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for the evaluation of CSI prediction:

- Companies are encouraged to report the assumptions on the observation window, including number/time distance of historic CSI/channel measurements as the input of the AI/ML model, and

- Companies to report the assumptions on the prediction window, including number/time distance of predicted CSI/channel as the output of the AI/ML model

Conclusion

If ideal DL channel estimation is considered (which is optional) for the evaluations of CSI feedback enhancement, there is no consensus on how to use the ideal channel estimation for dataset construction, or performance evaluation/inference.

- It is up to companies to report whether/how ideal channel is used in the dataset construction as well as performance evaluation/inference.

Conclusion

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example:

- For each FP/BP loop,

  - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side

  - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side

  - Step 3: UE side trains the CSI generation part based on the BP information from NW side

- Note: the dataset between UE side and NW side is aligned.

- Other Type 2 training approaches are not precluded and reported by companies

Conclusion

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training):

- Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly

- Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part

- Step3: UE side trains the UE side CSI generation part based on the received set of information

- Other Type 3 NW-first training approaches are not precluded and reported by companies

Conclusion

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training):

- Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly

- Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part

- Step3: NW side trains the NW side CSI reconstruction part based on the received set of information

- Other Type 3 UE-first training approaches are not precluded and reported by companies

Working assumption

In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors

- Note: Eventual KPI can still be used to compare the performance

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, at least Method 3 is adopted, FFS whether additionally adopt a down-selected metric between Method 1 and Method 2.

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1Ni=1NjKλijk=1Kλik∨wijHwij∨∨wij∨∨wij∨2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

- Method 3: SGCS is separately calculated for each layer (e.g., for K layers, K SGCS values are derived respectively, and comparison is performed per layer)

Agreement

In CSI compression using two-sided model use case, evaluate and study quantization of CSI feedback, including at least the following aspects:

- Quantization non-aware training

- Quantization-aware training

- Quantization methods including uniform vs non-uniform quantization, scalar versus vector quantization, and associated parameters, e.g., quantization resolution, etc.

- How to use the quantization methods

Agreement

For evaluating the performance impact of ground-truth quantization in the CSI compression, study high resolution quantization methods for ground-truth CSI, e.g., including at least the following options

- High resolution scalar quantization, e.g., Float32, Float16, etc.

    - FFS select one of the scalar quantization resolutions as baseline

- High resolution codebook quantization, e.g., R16 Type II-like method with new parameters

    - FFS new parameters

- Other quantization methods are not precluded

Agreement

For the evaluation of the potential performance benefits of model fine-tuning of CSI feedback enhancement which is optionally considered by companies, the following case is taken

- The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Company to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations:

- Case 1 (baseline): Aligned AI/ML model structure between NW side and UE side

- Case 2: Not aligned AI/ML model structures between NW side and UE side

    - Companies to report the AI/ML structures for the UE part model and the NW part model, e.g., different backbone (e.g., CNN, Transformer, etc.), or same backbone but different structure (e.g., number of layers)

- FFS different sizes of datasets between NW side and UE side

- FFS aligned/different quantization/dequantization methods between NW side and UE side

- FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

Agreement

For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), the following evaluation cases are considered for multi-vendors,

- Case 1 (baseline): Type 2 training between one NW part model to one UE part model

- Case 2: Type 2 training between one NW part model and M>1 separate UE part models

    - Companies to report the AI/ML structures for the UE part model and the NW part model

    - FFS Companies to report the dataset used at UE part models, e.g., whether the same or different dataset(s) are used among M UE part models

- Case 3: Type 2 training between one UE part model and N>1 separate NW part models

    - Companies to report the AI/ML structures for the UE part model and the NW part model

    - FFS Companies to report the dataset used at NW part models, e.g., whether the same or different dataset(s) are used among N NW part models

- FFS N NW part models to M UE part models

- FFS different quantization/dequantization methods between NW and UE

- FFS: whether/how to evaluate the case where the input/output types and/or pre/post-processing are not aligned between NW part model and UE part model

- FFS: companies to report the training order of UE-NW pair(s) in case of M UE part models and/or N NW part models

- FFS: whether/how to report overhead

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, at least the following types of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) are considered for evaluations

- Raw channel matrix, e.g., channel matrix with the dimensions of Tx, Rx, and frequency unit

    - Companies to report the raw channel is in frequency domain or delay domain

- Precoding matrix

    - Companies to report the precoding matrix is a group of eigenvector(s) or an eType II-like reporting (i.e., eigenvectors with angular-delay domain representation)

- Other input/output types are not precluded

- Companies to report the combination of input (for CSI generation part) and output (for CSI reconstruction part),

    - Note: the input and output may be of different types

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for SLS, spatial consistency procedure A with 50m decorrelation distance from 38.901 is used (if not used, company should state this in their simulation assumptions)

- UE velocity vector is assumed as fixed over time in Procedure A modeling

Agreement

In the evaluation of the AI/ML based CSI feedback enhancement, for the calculation of intermediate KPI, the following is considered as the granularity of the frequency unit for averaging operation

- For 15kHz SCS: For 10MHz bandwidth: 4 RBs; for 20MHz bandwidth: 8 RBs

- For 30kHz SCS: For 10MHz bandwidth: 2 RBs; for 20MHz bandwidth: 4 RBs

- Note: Other frequency unit granularity is not precluded and reported by companies

### Agreements of the 111 meeting

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression without generalization/scalability verification

- FFS the description and results for generalization/scalability may need a separate table

- FFS the value or range of payload size X/Y/Z

- FFS the description and results for different training types/cases may need a separate table

- FFS: training related overhead

Table X. Evaluation results for CSI compression without model generalization/scalability, [traffic type], [Max rank value], [RU] [training type/case]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
| Dataset description | Train/k |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| Intermediate KPI I#1 of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Intermediate KPI I#1 of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI I#1, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI#1, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| Intermediate KPI I#2 of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Intermediate KPI I#2 of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI I#2, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for intermediate KPI#2, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| Gain for Mean UPT | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for 5% UPT | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors

- Case 1 (baseline): Type 3 training between one NW part model and one UE part model

  - Note 1: Case 1 can be naturally applied to the NW-first training case where 1 NW part model to M>1 separate UE part models

    - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training UE part model is the same or a subset of the dataset for training NW part model

  - Note 2: Case 1 can be naturally applied to the UE-first training case where 1 UE part model to N>1 separate NW part models

    - Companies to report the dataset used between the NW part model and the UE part model, e.g., whether dataset for training NW part model is the same or a subset of the dataset for training UE part model

  - Companies to report the AI/ML structures for the combination(s) of UE part model and NW part model, which can be the same or different

  - FFS: different quantization methods between NW side and UE side

- Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models

  - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models

  - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model

  - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models

- Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models

  - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models

  - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models

  - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models

- FFS: whether/how to report overhead of dataset

Working Assumption

For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies.

- Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example

  - It does not imply any restriction on future specification for CSI prediction

- FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), the generalization cases of are elaborated as follows

- Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, and/or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1.

- Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn.

- Note: For Case 2/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g., pre-processing to angle-delay domain, padding, additional adaptation layer in AI/ML model, etc.

- FFS the verification of fine-tuning

- FFS other additional cases

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows

- Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1.

- Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn.

- Note: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization.

- Note: For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc.

- FFS the verification of fine-tuning

- FFS other additional cases

Agreement

For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, Float32 is adopted as the baseline/upper-bound of performance comparison.

Agreement

For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies:

- Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training

  - Fixed/pre-configured quantization method/parameters is applied for the inference phase

    - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

- Case 2: Quantization aware training, where quantization/dequantization is involved in the training process

  - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase

    - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc.

  - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase

    - Companies to report how to update the quantization method/parameters during the training

- Note: the above cases apply for training Type 1/2/3

- Others are not precluded.

Agreement

For the evaluation of an example of Type 3 (Separate training at NW side and UE side) with sequential training, companies to report the set of information (e.g., dataset) shared in Step 2

- For NW-first training

  - Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable.

  - Quantization behavior, e.g., whether the shared output of the Network side CSI generation part is before or after quantization.

- For UE-first training

  - Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable.

  - Quantization behavior, e.g., whether the shared inputof the UE side CSI reconstruction part is before or after quantization.

Working Assumption

For the AI/ML based CSI prediction sub use case, the following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction for the case without generalization/scalability verification

- FFS the description and results for generalization/scalability may need a separate table

- FFS whether/how to capture the muliptle predicted CSI instances and their mapping to slots

Table X. Evaluation results for CSI prediction without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | UE speed |  |  |
|  | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Dataset size | Train/k |  |  |
|  | Test/k |  |  |
| Benchmark 1 |  |  |  |
| Intermediate KPI #1 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 1 |  |  |  |
| Intermediate KPI #2 of Benchmark 1 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 1 |  |  |  |
| Gain for eventual KPI (Benchmark 1) | Mean UPT |  |  |
|  | 5% UPT |  |  |
| Benchmark 2 |  |  |  |
| Intermediate KPI #1 of Benchmark 2 |  |  |  |
| Gain for intermediate KPI#1 over Benchmark 2 |  |  |  |
| Intermediate KPI #2 of Benchmark 2 |  |  |  |
| Gain for intermediate KPI#2 over Benchmark 2 |  |  |  |
| Gain for eventual KPI (Benchmark 2) | Mean UPT |  |  |
|  | 5% UPT |  |  |
| FFS others |  |  |  |

Agreement

For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, companies to report which case(s) in the following are evaluated

- Case 0 (benchmark for comparison): One CSI generation part with fixed input and output dimensions to 1 CSI reconstruction part with fixed input and output dimensions for each of the different input and/or output dimensions.

- Case 1: One CSI generation part with scalable input and/or output dimensions to N>1 separate CSI reconstruction parts each with fixed and different output and/or input dimensions

- Case 2: M>1 separate CSI generation parts each with fixed and different input and/or output dimensions to one CSI reconstruction part with scalable output and/or input dimensions

- Case 3: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions

Agreement

For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, if R16 Type II-like method is considered, companies to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L,pv, β, reference amplitude, differential amplitude, phase, etc.

### Agreements of the 112 meeting

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, except for Method 3 which has been supported, There is no consensus on whether to adopt an additional method.

Agreement

Confirm the following working assumption of RAN1#110bis-e:

Working assumption In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, companies to ensure the correct calculation of SGCS and to avoid disorder issue of the output eigenvectors - Note: Eventual KPI can still be used to compare the performance

Conclusion

For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which have been agreed as the baseline metrics, for whether/how to introduce an additional intermediate KPI, NO additional intermediate KPI is adopted as mandatory.

- It is up to companies to optionally report other intermediate KPIs, e.g., Relative achievable rate (RAR)

Agreement

For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison.

Agreement

- Confirm the following WA on the benchmark for CSI prediction achieved in RAN1#111:

Working Assumption For the AI/ML based CSI prediction sub use case, the nearest historical CSI w/o prediction as well as non-AI/ML/collaboration level x AI/ML based CSI prediction approach are both taken as baselines for the benchmark of performance comparison, and the specific non-AI/ML/collaboration level x AI/ML based CSI prediction is reported by companies. - Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example - It does not imply any restriction on future specification for CSI prediction - FFS how to model the simulation cases for collaboration level x CSI prediction and LCM for collaboration level y/z CSI prediction

Agreement

The CSI prediction-specific generalization scenario of various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) is added to the list of scenarios for performing the generalization verification.

- FFS various frequency PRBs (e.g., trained based on one set of PRBs, inference on the same/different set of PRBs)

Agreement

For how to separate the templates for different training types/cases for AI/ML-based CSI compression without generalization/scalability verification, the following is considered:

- The determined template in the RAN1#111 working assumption is entitled with “1-on-1 joint training”

- A second separate template is introduced to capture the evaluation results for “multi-vendor joint training”

  - Note: this table captures the results for the joint training cases of 1 NW part model to M>1 UE part models, N>1 NW part models to 1 UE part model, or N>1 NW part models to M>1 UE part models. An example is multi-vendor Type 2 training.

- A third separate template is introduced to capture the evaluation results for “separate training”

- FFS: additional KPIs for each template, e.g., overhead, latency, etc.

Agreement

For the evaluation of training Type 3 under CSI compression, besides the 3 cases considered for multi-vendors, add one new Case (1-on-1 training with joint training) as benchmark/upper bound for performance comparison.

- FFS the relationship between the pair(s) of models for Type 3 and the pair(s) of models for new Case

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies to report the specific option adopted for AI/ML model settings to adapt to ranks/layers.

- Option 1-1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference, any specific model operates on multi-layers jointly.

  - FFS on the reported complexity and storage

  - FFS: input/output type

- Option 1-2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference, the model operates on multi-layers jointly.

  - FFS: input/output type

- Option 2 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report the setting is

    - Option 2-1: layer specific and rank common (different models applied for different layers; for a specific layer, the same model is applied for all rank values), or

    - Option 2-2: layer specific and rank specific (different models applied for different layers; for a specific layer, different models are applied for different rank values)

- Option 3 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

  - FFS on the reported complexity and storage

  - Note: input/output type is Precoding matrix

  - Companies to report whether the setting is

    - Option 3-1: layer common and rank common (A unified AI/ML model is applied for each layer under any rank value to perform individual inference), or

    - Option 3-2: layer common and rank specific (different models applied for different rank values; for a specific rank, the same model is applied for all layers)

- Other options not precluded.

Agreement

The CSI feedback overhead is calculated as the weighted average of CSI payload per rank and the distribution of ranks reported by the UE.

- For AI/ML based solutions: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank.

- For legacy Type II CB: Option 2b is mandatorily reported by companies, while Option 2a can be optionally reported up to companies if partial NZC report is assumed for the legacy Type II CB

  - Option 2a: The above-mentioned “CSI feedback overhead” is calculated as each CSI reported payload with a given rank

  - Option 2b: The above-mentioned “CSI feedback overhead” is calculated as max allowed bits at the given rank

Working Assumption

For the initial template for AI/ML-based CSI compression without generalization/scalability verification achieved in the working assumption in the RAN1#111 meeting, X, Y and Z are determined as:

- X is <=80bits

- Y is 100bits-140bits

- Z is  >=230bits

Working Assumption

X, Y and Z are applicable for per layer

Working assumption

The following initial template is considered to replace the template achieved in the working assumption in the RAN1#111 meeting, for companies to report the evaluation results of AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS the description and results for generalization/scalability may need a separate table

- Note: the values of CSI feedback overhead for the mean UPT and 5% UPT may need to be revisited in the 112bis-e meeting

- FFS: training related overhead

- FFS: how to capture CSI overhead reduction to the template

  - Note: It is to be captured to the template after a way is found on how to derive the CSI overhead reduction.

Table X. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, [traffic type], [Max rank value], [RU]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AI/ML model backbone |  |  |  |
|  | Pre-processing |  |  |  |
|  | Post-processing |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| CSI reconstruction part | AI/ML model backbone |  |  |  |
|  | [Pre-processing] |  |  |  |
|  | [Post-processing] |  |  |  |
|  | FLOPs/M |  |  |  |
|  | Number of parameters/M |  |  |  |
|  | [Storage /Mbytes] |  |  |  |
| Common description | Input type |  |  |  |
|  | Output type |  |  |  |
|  | Quantization /dequantization method |  |  |  |
|  | Rank/layer adaptation settings for rank>1 |  |  |  |
| Dataset description | Train/k |  |  |  |
|  | Test/k |  |  |  |
|  | Ground-truth CSI quantization method (including scalar/codebook based quantization, and the parameters) |  |  |  |
|  | Overhead reduction compared to Float32 if high resolution quantization of ground-truth CSI is applied |  |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Benchmark |  |  |  |  |
| Benchmark assumptions, e.g., CSI overhead calculation method (Optional) |  |  |  |  |
| SGCS of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| SGCS of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for SGCS, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for SGCS, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … (other layers) |  |  |  |  |
| NMSE of benchmark, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| NMSE of benchmark, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for NMSE, [layer 1] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| Gain for NMSE, [layer 2] | CSI feedback payload X |  |  |  |
|  | CSI feedback payload Y |  |  |  |
|  | CSI feedback payload Z |  |  |  |
| … (other layers) |  |  |  |  |
| Other intermediate KPI (description/value) (optional) |  |  |  |  |
| Gain for other intermediate KPI (description/value) (optional) |  |  |  |  |
| Gain for Mean UPT (for a specific CSI feedback overhead) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for 5% UPT | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for upper bound without CSI compression over Benchmark –Mean UPT (Optional) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| Gain for upper bound without CSI compression over Benchmark –5% UPT (Optional) | [CSI feedback payload X*Max rank value] |  |  |  |
|  | [CSI feedback payload Y*Max rank value] |  |  |  |
|  | [CSI feedback payload Z*Max rank value] |  |  |  |
| [CSI feedback reduction (%)] |  |  |  |  |
| … |  |  |  |  |
| FFS others |  |  |  |  |

Note: “Benchmark” means the type of Legacy CB used for comparison.

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working assumption

A separate table to capture the evaluation results of generalization/scalability verification for AI/ML-based CSI compression is given in the following initial template

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI compression with model generalization/scalability, [Max rank value], [Scenario/configuration]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Number of parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Generalization/Scalability method description if applicable, e.g., truncation, adaptation layer, etc. |  |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) |  |  |
| Dataset description | Ground-truth CSI quantization method |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |
| Generalization Case 1 | Train (setting#A, size/k) |  |  |
|  | Test (setting#A, size/k) |  |  |
| SGCS, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| SGCS, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| NMSE, layer 1 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| NMSE, layer 2 | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (other settings for Case 1) |  |  |  |
| … |  |  |  |
| Generalization Case 2 | Train (setting#A, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
| … (results for Case 2) |  |  |  |
| … (other settings for Case 2) |  |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) |  |  |
|  | Test (setting#A/#B, size/k) |  |  |
| … (results for Case 3) |  |  |  |
| … (other settings for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting#A, size/k) |  |  |
|  | Fine-tune (setting#B, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
| … (results for Fine-tuning) |  |  |  |
| … (other settings for Fine-tuning) |  |  |  |
| FFS others |  |  |  |

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI prediction with generalization verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

Table X. Evaluation results for CSI prediction with model generalization, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| AI/ML model description | AL/ML model description (e.g., backbone, structure) |  |  |
|  | [Pre-processing] |  |  |
|  | [Post-processing] |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
|  | [Storage /Mbytes] |  |  |
|  | Input type |  |  |
|  | Output type |  |  |
| Assumption | CSI feedback periodicity |  |  |
|  | Observation window (number/distance) |  |  |
|  | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) |  |  |
|  | Whether/how to adopt spatial consistency |  |  |
| Generalization Case 1 | Train (setting#A, size/k) |  |  |
|  | Test (setting#A, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 1) |  |  |  |
| Generalization Case 2 | Train (setting#A, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 2) |  |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) |  |  |
|  | Test (setting#A/#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Case 3) |  |  |  |
| Fine-tuning case (optional) | Train (setting#A, size/k) |  |  |
|  | Fine-tune (setting#B, size/k) |  |  |
|  | Test (setting#B, size/k) |  |  |
|  | SGCS (1,…N, N is number of prediction instances) |  |  |
|  | NMSE (1,…N, N is number of prediction instances) |  |  |
| … (other settings and results for Fine-tuning) |  |  |  |
| FFS others |  |  |  |

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for multi-vendor joint training and without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

- FFS case of multiple layers

Table X. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | [Training method] |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset description | Train/k |  |  |
|  | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| Case 1 (baseline): NW#1-UE#1 | UE part AI/ML model backbone/structure |  |  |
|  | Network part AI/ML model backbone/structure |  |  |
| ... (other NW-UE combinations for Case 1) |  |  |  |
| Case 2 (1 NW part to M>1 UE parts) | NW part model backbone/structure |  |  |
|  | UE#1 part model backbone/structure |  |  |
|  | UE#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE#M part training dataset description and size |  |  |
| Case 3 (N>1 NW parts to 1 UE part) | UE part model backbone/structure |  |  |
|  | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset description and size |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| Case 1: NW#1-UE#1: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (results for other NW-UE combinations for Case 1) |  |  |  |
| Case 2: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

Working Assumption

The following initial template is considered for companies to report the evaluation results of AI/ML-based CSI compression for sequentially separate training and without generalization/scalability verification

- To be collected before 112bis-e meeting

- FFS whether the intermediate KPI results are gain over benchmark or absolute values

- FFS whether the intermediate KPI results are in forms of linear or dB

- FFS case of multiple layers

Table X. Evaluation results for CSI compression of separate training without model generalization/scalability, [Max rank value]

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
|  | Shared output of CSI generation part/input of reconstruction part is before or after quantization |  |  |
| Dataset description | Test/k |  |  |
|  | Ground-truth CSI quantization method |  |  |
| [Benchmark: NW#1-UE#1 joint training] | UE part AI/ML model backbone/structure |  |  |
|  | Network part AI/ML model backbone/structure |  |  |
|  | Training dataset size |  |  |
| ... (other NW-UE combinations for benchmark) |  |  |  |
| Case 1-NW first training | NW part AI/ML model backbone/structure |  |  |
|  | UE#1 part model backbone/structure |  |  |
|  | UE#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE#M part training dataset description and size |  |  |
|  | [air-interface overhead of information (e.g., dataset) sharing] |  |  |
| Case 1-UE first training | NW#1 part model backbone/structure |  |  |
|  | NW#1 part training dataset description and size |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | NW#N part training dataset description and size |  |  |
|  | UE part model backbone/structure |  |  |
|  | [air-interface overhead of information (e.g., dataset) sharing] |  |  |
| Case 2-UE first training | UE#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | UE#M part model backbone/structure |  |  |
|  | UE part AI/ML model backbone/structure |  |  |
|  | NW part training dataset description and size (e.g., description/size of dataset from M UEs and how to merge) |  |  |
| Case 3-NW first training | NW#1 part model backbone/structure |  |  |
|  | … |  |  |
|  | NW#N part model backbone/structure |  |  |
|  | UE part model backbone/structure |  |  |
|  | UE part training dataset description and size (e.g., description/size of dataset from N NWs and how to merge) |  |  |
| Intermediate KPI type (SGCS/NMSE) |  |  |  |
| FFS other cases |  |  |  |
| NW#1-UE#1 joint training: Intermediate KPI | CSI feedback payload X |  |  |
|  | CSI feedback payload Y |  |  |
|  | CSI feedback payload Z |  |  |
| … (results for other 1-on-1 NW-UE joint training combinations) |  |  |  |
| Case 1-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 1-UE first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 2-NW first training: Intermediate KPI | CSI feedback payload X, NW#1-UE |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW#N-UE |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| Case 3-NW first training: Intermediate KPI | CSI feedback payload X, NW-UE#1 |  |  |
|  | … |  |  |
|  | CSI feedback payload X, NW-UE#M |  |  |
|  | CSI feedback payload Y … |  |  |
|  | CSI feedback payload Z … |  |  |
| FFS other cases |  |  |  |
| FFS others |  |  |  |

Note: “Quantization/dequantization method” includes the description of training awareness (Case 1/2-1/2-2), type of quantization/dequantization (SQ/VQ), etc.

Note: “Input type” means the input of the CSI generation part. “output type” means the output of the CSI reconstruction part.

### Agreements of the 112bis-e meeting

Agreement

For the rank >1 options under AI/ML-based CSI compression, for a given configured Max rank=K, the complexity of FLOPs is reported as the maximum FLOPs over all ranks each includes the summation of FLOPs for inference per layer if applicable, e.g.,

- Option 1-1 (rank specific): Max FLOPs over K rank specific models.

- Option 1-2 (rank common): FLOPs of the rank common model.

- Option 2-1 (layer specific and rank common): Sum of the FLOPs of K models (for the rank=K).

- Option 2-2 (layer specific and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with a sum of k models.

- Option 3-1 (layer common and rank common): K * FLOPs of the common model.

- Option 3-2 (layer common and rank specific): Max of the FLOPs over K ranks, k=1,…K, each with k * FLOPs of the layer common model.

Agreement

For the rank >1 options under AI/ML-based CSI compression, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g.,

- Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models.

- Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model.

- Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models.

- Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank.

- Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model.

Working assumption

For the forms of the intermediate KPI results for the following templates:

Table 2. Evaluation results for CSI compression with model generalization Table 3. Evaluation results for CSI compression with model scalability, Table 4. Evaluation results for CSI compression of multi-vendor joint training without model generalization/scalability, Table 5. Evaluation results for CSI compression of separate training without model generalization/scalability, Table 7. Evaluation results for CSI prediction with model generalization

- The intermediate KPI results are in forms of absolute values and the gain over benchmark, e.g., in terms of “absolute value (gain over benchmark)”

- The intermediate KPI results are in forms of linear value for SGCS and dB value for NMSE

Working Assumption

For the per layer CSI payload size X/Y/Z in the templates of CSI compression, as a clarification, the X/Y/Z ranges in the working assumption achieved in RAN1#112 meeting is applicable to Max rank = 1/2. For Max rank (v) = 3/4, the per layer basis X/Y/Z ranges are re-determined as:

- X is <=⌈160v⌉bits

- Y is ⌈200v⌉bits-⌈280v⌉bits

- Z is >=⌈460v⌉bits

Working Assumption

For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback reduction is provided for 3 CSI feedback overhead ranges, where for each CSI feedback overhead range of the benchmark, it is calculated as the gap between the CSI feedback overhead of benchmark and the CSI feedback overhead of AI/ML corresponding to the same mean UPT.

- Note: the CSI feedback overhead reduction and gain for mean/5%tile UPT are determined at the same payload size for benchmark scheme

| CSI feedback reduction (%) (for a given CSI feedback overhead in the benchmark scheme) | [X*Max rank value], RU<=39% |
| --- | --- |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |

Note: for result collection for the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#112bis-e meeting,

- 15 sources show that compared to the case where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B, it has degraded performance if the model is trained with deployment scenario#A and applied for inference with a different deployment scenario#B.

  - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, deployment scenario#A is UMi, deployment scenario#B is UMa, or deployment scenario#A is InH, deployment scenario#B is UMa/UMi.

  - 6 sources observe that if deployment scenario#A and deployment scenario#B are subject to some certain combinations, the degradation is minor.

    - E.g., deployment scenario#A is UMa, deployment scenario#B is UMi, or deployment scenario#A is UMi, deployment scenario#B is UMa.

- 6 sources show that generalized performance of the AI/ML model can be achieved, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#A and deployment scenario#B, and the trained AI/ML model applies inference on either deployment scenario#A or deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa and/or UMi.

- 3 sources show that, compared to the case where the AI/ML model is trained on scenario#A and applied for inference on deployment scenario#B, the generalization performance can be improved, if the AI/ML model, after trained on deployment scenario#A, is updated based on a fine-tuned dataset subject to deployment scenario#B, and performs inference on deployment scenario#B.

  - E.g., deployment scenario#A is InH, deployment scenario#B is UMa or UMi.

Agreement

For the AI/ML based CSI prediction, add an entry for “Table 6. Evaluation results for CSI prediction without model generalization/scalability” to report the Codebook type for CSI report.

| Assumption | UE speed |
| --- | --- |
|  | CSI feedback periodicity |
|  | Observation window (number/distance) |
|  | Prediction window (number/distance [between prediction instances/distance from the last observation instance to the 1st prediction instance]) |
|  | Whether/how to adopt spatial consistency |
|  | Codebook type for CSI report |

Agreement

To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, the model monitoring methodology is considered as:

- Step1: Generate test dataset including K test samples

  - FFS how to obtain the K test samples

- Step2: For each of K test samples, a bias factor of monitored intermediate KPI (KPIDiff) is calculated as a function of KPIDiff=fKPIActual,KPIGenie, where KPIActual is the actual intermediate KPI, and KPIGenie is the genie-aided intermediate KPI.

- Step3: Calculate the statistical result of the KPIDiff over K test samples which represents the monitoring accuracy performance.

- Note: KPIGenie is introduced for the evaluation and comparison purpose; it may not be available in the real network.

- Note: the complexity, overhead and latency of the monitoring scheme are reported by companies. FFS how to evaluate latency.

Agreement

To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, for Step2 of the model monitoring methodology, the per sample KPIDiff is considered for

- Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar, R16 eType II-like method, etc.) or SRS measurements, where

  - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements.

  - KPIGenie is calculated with output CSI (as for KPIActual) and the ground-truth CSI of Float32.

  - Note: if Float32 is used for KPIActual, the monitoring accuracy is 100% if KPIActual and KPIGenie are based on the same CSI sample.

- Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE:

  - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI.

    - Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100%

  - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual)

  - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI.

  - FFS how to train the proxy model and the resulting monitoring performance, to be reported by companies.

  - FFS whether/how to evaluate the generalization performance of the proxy model.

- Case 3: others are not precluded

Conclusion

For the evaluation of CSI enhancements, when reporting the computational complexity including the pre-processing and post-processing, the complexity metric of FLOPs may be reported separately for the AI/ML model and the pre/post processing.

- How to calculate the FLOPs for pre/post processing is up to companies.

- While reporting the FLOPs of pre-processing and post-processing the following boundaries are considered.

  - Estimated raw channel matrix per each frequency unit as an input for pre-processing of the CSI generation part

  - Precoding vectors per each frequency unit as an output of post-processing of the CSI reconstruction part

Agreement

For the evaluation of CSI compression, companies are allowed to report (by introducing an additional field in the template to describe) the specific CQI determination method(s) for AI/ML, e.g.,

- Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment

  - Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW

  - Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW

- Option 2b: CQI is calculated using two stage approach, UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder

- Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation

- Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment

- Option 1c: CQI is calculated based on traditional codebook

- Other options if adopted, to be described by companies

Agreement

For the AI/ML based CSI prediction sub use case, if collaboration level x is reported as the benchmark, the EVM to distinguish level x and level y/z based AI/ML CSI prediction is considered from the generalization aspect.

- E.g., collaboration level y/z based CSI prediction is modeled as the fine-tuning case or generalization Case 1, while collaboration level x based CSI prediction is modeled as generalization Case 2 or Case 3.

Agreement

To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, KPIDiff is in forms of

- Option 1: Gap between KPIActual and KPIGenie, i.e. KPIDiff=KPIActual-KPIGenie;

  - Monitoring accuracy is the percentage of the samples for which KPIDiff<KPIth1, where KPIth1 is a threshold of the intermediate KPI gap.

- Option 2: Binary state where KPIActual and KPIGenie have different relationships to their threshold(s), i.e., KPIDiff=KPIActual>KPIth2,KPIGenie<KPIth3∨KPIActual<KPIth2,KPIGenie>KPIth3, where KPIth2 can be same or different from KPIth3.

  - Monitoring accuracy is the percentage of the samples for which KPIDiff=0.

- FFS other metrics: Misdetection, False alarm, etc.

- FFS the values of KPIth1, KPIth2, KPIth3.

- FFS whether/how to evaluate the monitoring metrics for Rank>1

Working Assumption

For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, the CSI feedback overhead for the metric of eventual KPI (e.g., mean/5% UPT) is re-determined as:

- CSI feedback overhead A: <=β* 80 bits.

- CSI feedback overhead B: β* (100bits – 140 bits).

- CSI feedback overhead C: >=β* 230 bits.

- Note: β=1 for max rank = 1, andβ=1.5 for max rank = 2/3/4.

- FFS for rank 2/3/4, whether to add an additional CSI feedback overhead D: >=γ* 230 bits, γ= [1.9], and limit the range of CSI feedback overhead C as:β* 230 bits-γ* 230 bits.

- Note: companies additionally report the exact CSI feedback overhead they considered

Observation

For the scalability verification of AI/ML based CSI compression over various CSI payload sizes, till the RAN1#112bis-e meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain CSI payload size#B and applied for inference with a same CSI payload size#B,

- Generalized performance of the AI/ML model can be achieved (0%~5.9% loss) under generalization Case 3 for the inference on either CSI payload size#A or CSI payload size#B, if the training dataset is constructed with data samples subject to multiple CSI payload sizes including CSI payload size#A and CSI payload size#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, shown by 7 sources (Note *) (6 sources (Note **) showing 0%~2.2% loss, 3 sources (Note ***) showing 2.35%~5.9% loss). The scalability solution is adopted as follows:

  - Pre/post-processing of truncation/padding, adopted by 3 sources (Note ****), showing 0.2%~5.9% loss.

  - Various quantization granularities, adopted by 1 source (Note *****), showing 1.8%~4.7% loss.

  - Adaptation layer in the AL/ML model, adopted by 3 sources (Note ******), showing 0%~4.05% loss.

- Note: the above results are based on the following assumptions

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - Input/output scalability dimension Case 3 is adopted: A pair of CSI generation part with scalable input/output dimensions and CSI reconstruction part with scalable output and/or input dimensions.

  - The performance metric is SGCS in linear value for layer 1/2.

- Note *: Huawei, HiSilicon (R1-2302358), Ericsson (R1-2302918), OPPO (R1-2302540), Fujitsu (R1-2302904), CMCC (R1-2303224), MediaTek (R1-2303336), NTT DOCOMO (R1-2303705).

- Note **: Huawei, HiSilicon (R1-2302358), Ericsson (R1-2302918), Fujitsu (R1-2302904), CMCC (R1-2303224), MediaTek (R1-2303336), NTT DOCOMO (R1-2303705).

- Note ***: Ericsson (R1-2302918), OPPO (R1-2302540), MediaTek (R1-2303336).

- Note ****: OPPO (R1-2302540), Fujitsu (R1-2302904), CMCC (R1-2303224).

- Note *****: Ericsson (R1-2302918).

- Note ******: Huawei, HiSilicon (R1-2302358), MediaTek (R1-2303336), NTT DOCOMO (R1-2303705).

Observation

For the AI/ML based CSI prediction, till the RAN1#112bis-e meeting,

- 11 sources (Note *) show that the AI/ML-based CSI prediction outperforms the benchmark of the nearest historical CSI, wherein

  - 5 sources (Note **) show the gain of 14% ~ 26.47% using raw channel matrix as input.

  - 2 sources (Note ***) show the gain of 5.64% ~ 9.49% using precoding matrix as input, which is in general worse than using raw channel matrix as input

- Note 1: spatial consistency is adopted in 1 source (Note ****) and not adopted in 5 sources (Note *****).

- Note 2: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - UE speed is 30km/h.

  - The performance metric is SGCS in linear value for layer 1.

- Note *: Huawei, HiSilicon (R1-2302358), ZTE (R1-2302437), Spreadtrum Communications, BUPT, (R1-2302593), Nokia, Nokia Shanghai Bell (R1-2302628), CATT (R1-2302695), Fujitsu (R1-2302904), Samsung (R1-2303120), ETRI (R1-2303194), CMCC (R1-2303224), NVIDIA (R1-2303435), Apple (R1-2303475).

- Note **: ZTE (R1-2302437), Nokia, Nokia Shanghai Bell (R1-2302628), Spreadtrum Communications, BUPT (R1-2302593), NVIDIA (R1-2303435), Apple (R1-2303475).

- Note ***: ZTE (R1-2302437), Fujitsu (R1-2302904).

- Note ****: Nokia, Nokia Shanghai Bell (R1-2302628).

- Note *****: Huawei, HiSilicon (R1-2302358), ZTE (R1-2302437), ETRI (R1-2303194), CMCC (R1-2303224), Apple (R1-2303475).

Agreement

For the AI/ML based CSI compression, for the submission of simulation results to the RAN1#113 meeting, for Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, companies are encouraged to take the following assumptions as baseline for the calibration purpose:

- Benchmark: R16 eType II CB;

  - Others can be additionally submitted, e.g., Type I CB.

- Input/Output type: Eigenvectors of the current CSI

  - Other can be additionally submitted, e.g., eigenvectors with additional past CSI, eType II-like input, raw channel matrix, etc.

- Ground-truth CSI quantization method: Float32, i.e., without quantization

  - Other high resolution CSI quantization methods can be additionally submitted for comparison, e.g., R16 Type II-like method with new parameters, scalar quantization, etc.

- Rank/layer adaptation settings for rank>1: Option 3-1, i.e., layer common and rank common

  - Other rank>1 options can be additionally submitted for comparison, e.g., Option 1-1/1-2/2-1/2-2/3-2.

- Quantization method: quantization-aware training (Case 2-1 or Case 2-2)

  - Quantization non-aware training can be additionally submitted for comparison

  - SQ and/or VQ is up to companies; companies are encouraged to provide results of various cases for comparison.

- Performance metric for intermediate KPI: SGCS

  - NMSE can be additionally submitted.

Agreement

For the AI/ML based CSI prediction, for the submission of simulation results to the RAN1#113 meeting,

- for Table 6. Evaluation results for CSI prediction without model generalization/scalability, companies are encouraged to take the following assumptions as baseline for the calibration purpose:

  - UE speed: 10km/h, 30km/h, 60km/h;

    - Others can be additionally submitted, e.g., 120km/h.

  - Input/Output type: Raw channel matrix

    - Other can be additionally submitted, e.g., eigenvectors.

  - Observation window: 5/5ms, 10/5ms

    - Other observation window configurations can be additionally submitted for comparison, e.g., 3/5ms, 4/5ms, 8/2.5ms, 10/4ms, etc.

  - Prediction window: 1/5ms/5ms

    - Other prediction window configurations can be additionally submitted for comparison, e.g., 3/5ms/5ms, 5/5ms/5ms, 4/2.5ms/2.5ms, 5/4ms/4ms, etc.

  - Performance metric for intermediate KPI: SGCS

    - NMSE can be additionally submitted.

  - Spatial consistency configuration (optional): procedure A with 50m decorrelation distance and channel updating periodicity of 1 ms.

- for Table 7. Evaluation results for CSI prediction with model generalization, companies are encouraged to take the following assumption as baseline for the calibration purpose:

  - Performance metric for intermediate KPI: SGCS

    - NMSE can be additionally submitted.

### Agreements of the 113 meeting

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 10km/h UE speed, 4 sources [Fujitsu, Samsung, Xiaomi, InterDigital] observe 1.03%~6% gain, 1 source [CMCC] observes 21.93% gain.

- For 30km/h UE speed, 2 sources [OPPO, ETRI] observes 6%~10.43% gain, 5 sources [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum] observe 18.72%~31.3% gain, and 3 sources [InterDigital, MediaTek, CMCC] observe 35%~ 41.75% gain, which are in general larger than 10km/h UE speed.

- For 60km/h UE speed, 2 sources [Fujitsu, InterDigital] observe -3%~5% gain, 4 sources [Huawei, Samsung, vivo, CMCC] observe 11.2%~19.98% gain, which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - No spatial consistency is considered by 11 sources [Fujitsu, Samsung, Xiaomi, InterDigital, CMCC, OPPO, ETRI, ZTE, Apple, Huawei, Spreadtrum]. 1 source [vivo] provides both results with spatial consistency and results w/o spatial consistency.

  - Note: Results refer to Table 5.1-1 of R1-2306059

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of SGCS,

- For Max rank 1, Layer 1,

  - 11 sources [Huawei, Nokia, ZTE, vivo, OPPO, ETRI, Fujitsu, CMCC, China Telecom, MediaTek, Apple] observe the performance gain of 2.6%~ 8.8% at CSI payload X (small payload);

  - 14 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, ETRI, Fujitsu, NTT DOCOMO, Xiaomi, China Telecom, MediaTek, BJTU, Apple] observe the performance gain of 0.9%~ 8.1% at CSI payload Y (medium payload);

  - 11 sources [Huawei, Nokia, Futurewei, Lenovo, ZTE, vivo, OPPO, ETRI, Fujitsu, BJTU, Apple] observe the performance gain of 0.9%~ 7% at CSI payload Z (large payload);

  - Note: 1 source [Futurewei] observes the performance gain of 11.6% at CSI payload X (small payload) which biases from the majority range.

- For Max rank 2, Layer 1,

  - 12 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 3.9%~ 11% at CSI payload X (small payload);

  - 11 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Qualcomm, Apple] observe the performance gain of 0.7%~ 4.5% at CSI payload Y (medium payload);

  - 9 sources [Huawei, Nokia, Futurewei, ZTE, vivo, OPPO, Fujitsu, Ericsson, Apple] observe the performance gain of -0.2%~ 6.5% at CSI payload Z (large payload);

- For Max rank 2, Layer 2, more gains are observed in general compared with Layer 1 of Max rank 2:

  - 12 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 5.92%~ 30.2% at CSI payload X (small payload);

  - 12 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Xiaomi, Qualcomm, Intel, Apple] observe the performance gain of 1.5%~ 23.08% at CSI payload Y (medium payload);

  - 10 sources [Huawei, Nokia, ZTE, vivo, OPPO, Fujitsu, NTT DOCOMO, Ericsson, Intel, Apple] observe the performance gain of 4.4%~ 12.99% at CSI payload Z (large payload);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1 of Max rank 1 or Layer 1/2 of Max rank 2.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-2 of R1-2306059

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under FTP traffic, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.2%~2%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.29%~2% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.2%~1% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.33%~1% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~4%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.09%~3% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.80%~2% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~4% at CSI overhead C (large overhead);

  - For RU>=70%, 6 sources [Huawei, Nokia, ZTE, vivo, OPPO, Spreadtrum] observe the performance gain of 0.23%~9%

    - 6 sources [Huawei, Nokia, ZTE, vivo, OPPO, Spreadtrum] observe the performance gain of 0.38%~9% at CSI overhead A (small overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.62%~5% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.23%~6% at CSI overhead C (large overhead);

  - Note: 2 sources [Spreadtrum, Futurewei] observe gain of 12.77%~21.21% at RU 40%-69%, 11.23%~21.5% at RU>=70%, which bias from the majority ranges.

- For Max rank 2, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 6 sources [Huawei, Nokia, Ericsson, Intel, Qualcomm, Fujitsu] observe the performance gain of -0.3%~6%

    - 5 sources [Huawei, Nokia, Ericsson, Intel, Qualcomm] observe the performance gain of 1%~6% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 0.5%~6% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, Ericsson, Intel, Fujitsu, Qualcomm] observe the performance gain of -0.3%~6% at CSI overhead C (large overhead);

  - For RU 40%-69%, 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm, InterDigital] observe the performance gain of -0.5%~10%

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of 3%~10% at CSI overhead A (small overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm] observe the performance gain of 1.2%~9% at CSI overhead B (medium overhead);

    - 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Qualcomm, InterDigital] observe the performance gain of -0.5%~9% at CSI overhead C (large overhead);

  - For RU>=70%, 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of -0.2%~15%

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of 5%~15% at CSI overhead A (small overhead);

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Qualcomm, Futurewei] observe the performance gain of 3%~9% at CSI overhead B (medium overhead);

    - 9 sources [Huawei, Nokia, ZTE, OPPO, Ericsson, Intel, InterDigital, Fujitsu, Qualcomm] observe the performance gain of -0.2%~12% at CSI overhead C (large overhead);

  - Note: 4 sources [Futurewei, NTT DOCOMO, InterDigital, Fujitsu] observe gain of 7%~30% at RU<=39%, 10%~23% at RU 40%-69%, 12.71%~26.8% at RU>=70%, which bias from the majority ranges.

- For Max rank 4:

  - For RU<=39%, 3 sources [CATT, Apple, Qualcomm] observe the performance gain of -4%~7.4%

    - 3 sources [CATT, Apple, Qualcomm] observe the performance gain of 2.5%~7.4% at CSI overhead A (small overhead);

    - 1 source [Qualcomm] observes the performance gain of 6% at CSI overhead B (medium overhead);

    - 2 sources [Apple, Qualcomm] observe the performance gain of -4%~0% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.8%~12.22%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 3%~12.22% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 7.04%~11% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.8%~8.19% at CSI overhead C (large overhead);

  - For RU>=70%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1%~17%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 3%~17% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 6.64%~17% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1%~8.40% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is mean UPT for Max rank 1, Max rank 2, or Max rank 4.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-3 of R1-2306059

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under FTP, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.8%~3%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.72%~3% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.80%~1.2% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.68%~3% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~7%

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 2.8%~7% at CSI overhead A (small overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 1.22%~2.7% at CSI overhead B (medium overhead);

    - 3 sources [Huawei, Nokia, vivo] observe the performance gain of 0.1%~3.25% at CSI overhead C (large overhead);

  - For RU>=70%, 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.85%~20.43%

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 4%~20.43% at CSI overhead A (small overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 1%~10.13% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, ZTE, vivo, OPPO] observe the performance gain of 0.85%~8% at CSI overhead C (large overhead);

  - Note: 2 sources [Spreadtrum, Futurewei] observe gain of 15.87%~21.04% at RU 40%-69%, 20.2%~50% at RU>=70%, which bias from the majority ranges.

- For Max rank 2, in general the performance gain increases with the increase of RU:

  - For RU<=39%, 6 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu, InterDigital] observe the performance gain of -2%~5%

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 1.1%~5% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of -2%~3% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu, InterDigital] observe the performance gain of -0.5%~5% at CSI overhead C (large overhead);

  - For RU 40%-69%, 7 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel, Fujitsu] observe the performance gain of -4%~13%

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel] observe the performance gain of 7%~13% at CSI overhead A (small overhead);

    - 4 sources [Huawei, Nokia, Ericsson, Qualcomm] observe the performance gain of 0.3%~8% at CSI overhead B (medium overhead);

    - 5 sources [Huawei, Nokia, Ericsson, Qualcomm, Fujitsu] observe the performance gain of -4%~8% at CSI overhead C (large overhead);

  - For RU>=70%, 9 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Fujitsu, NTT DOCOMO, Qualcomm, Futurewei] observe the performance gain of -1.3%~24%

    - 7 sources [Huawei, Nokia, ZTE, Ericsson, Intel, Fujitsu, NTT DOCOMO] observe the performance gain of 10.26%~24% at CSI overhead A (small overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Qualcomm, Intel] observe the performance gain of 9%~15.02% at CSI overhead B (medium overhead);

    - 6 sources [Huawei, Nokia, ZTE, Ericsson, Futurewei, Intel] observe the performance gain of -1.3%~13.67% at CSI overhead C (large overhead);

  - Note: 5 sources [Intel, NTT DOCOMO, InterDigital, Fujitsu, ZTE] observe gain of 7%~24% at RU<=39%, -8%~-2%, 13.4%~29.7% at RU 40%-69%, -5%~-10%, 18.1%~35.4% at RU>=70%, which bias from the majority ranges.

- For Max rank 4:

  - For RU<=39%, 2 sources [Apple, Qualcomm] observe the performance gain of -1.6%~10%

    - 2 sources [Apple, Qualcomm] observe the performance gain of 8%~10% at CSI overhead A (small overhead);

    - 1 source [Qualcomm] observes the performance gain of 5% at CSI overhead B (medium overhead);

    - 2 sources [Apple, Qualcomm] observe the performance gain of -1.6%~1% at CSI overhead C (large overhead);

  - For RU 40%-69%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.7%~23%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 5%~17% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 6.17%~23% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of -1.7%~9.47% at CSI overhead C (large overhead);

  - For RU>=70%, 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 2%~31%

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 5.8%~31% at CSI overhead A (small overhead);

    - 2 sources [ZTE, Qualcomm] observe the performance gain of 10.2%~30% at CSI overhead B (medium overhead);

    - 3 sources [Apple, ZTE, Qualcomm] observe the performance gain of 2%~15% at CSI overhead C (large overhead);

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is 5% UPT for Max rank 1, Max rank 2, or Max rank 4.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.1-4 of R1-2306059

Observation

For the generalization verification of AI/ML based CSI compression over various deployment scenarios, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B and applied for inference with a same deployment scenario#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - If deployment scenario#A is UMi & deployment scenario#B is UMa, deployment scenario#A is UMa & deployment scenario#B is UMi, or deployment scenario#A is UMa & deployment scenario#B is InH:

    - 9 sources [Xiaomi, InterDigital, MediaTek, vivo, Intel, ZTE, OPPO, Huawei, CATT] observe that generalized performance can be achieved:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 7 sources [Xiaomi, InterDigital, MediaTek, vivo, Intel, ZTE, CATT] observe less than -1.6% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 5 sources [vivo, OPPO, MediaTek, Intel, Xiaomi] observe less than -1.4% degradation or positive gain.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [Huawei, CATT] observe less than -0.6% degradation or positive gain

    - 10 sources [Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT, ZTE, Lenovo, MediaTek, Futurewei] observe that moderate/significant degradations are suffered under generalization Case 2:

      - For deployment scenario#A is UMi & deployment scenario#B is UMa, 8 sources [Futurewei, MediaTek, Intel, NTT DOCOMO, Xiaomi, Interdigital, OPPO, CATT] observe -1.69%~-14.2% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is UMi, 7 sources [Futurewei, NTT DOCOMO, ZTE, InterDigital, CATT, Xiaomi, Intel] observe -1.81%~-18.5% degradation.

      - For deployment scenario#A is UMa & deployment scenario#B is InH, 2 sources [ZTE, Lenovo] observe -1.74%~-3.6% degradation.

  - If deployment scenario#A is InH & deployment scenario#B is Uma/UMi, significant performance degradations are observed under generalization Case 2:

    - For deployment scenario#A is InH & deployment scenario#B is UMa, 4 sources [Huawei, CATT, Lenovo, ZTE] observe -5.55%~-21.76% degradation.

    - For deployment scenario#A is InH & deployment scenario#B is UMi, 2 sources [vivo, ZTE] observe -8.63%~-20% degradation.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gain) for deployment scenario#B subject to any of UMa, UMi, and InH, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B, as observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MediaTek, Futurewei, vivo, OPPO, Intel, Huawei, ZTE].

  - Minor loss (0%~-1.48%) are observed by 11 sources [CATT, Xiaomi, NTT DOCOMO, Interdigital, MediaTek, Futurewei, vivo, OPPO, Intel, Huawei, ZTE].

  - Moderate loss (-1.6%~-4%) are observed by 5 sources [Xiaomi, CATT, vivo, NTT DOCOMO, Intel].

  - Positive gains are observed by 8 sources [ZTE, Interdigital, MediaTek, vivo, Intel, Xiaomi, Futurewei, CATT].

  - Note: Significant degradations of up to -6.7% are still observed by 2 sources [Intel, Xiaomi] for deployment scenario#B subject to UMa, and by 2 sources [Intel, CATT] for deployment scenario#B subject to UMi.

- Note: For generalization Case 2, if deployment scenario#A is UMi & deployment scenario#B is InH, 2 sources [vivo, ZTE] observe different trends, where significant performance degradations of -27.8%~-29.9% are observed by [vivo], while moderate performance degradations of -1.44%~-2.41% are observed by [ZTE].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Note: Results refer to Table 5.1-5 of R1-2306059

Observation

For the generalization verification of AI/ML based CSI compression over various UE distributions, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE distribution#B and applied for inference with a same UE distribution#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE distribution#A and UE distribution#B but not for others

  - If UE distribution#A is Outdoor & UE distribution#B is Indoor, 3 sources [Nokia, Qualcomm, Huawei] observe that moderate/significant degradations of -2.9%~-11.5% degradation are suffered,

    - Note: 1 source [NTT DOCOMO] observes 0% degradation

  - If UE distribution#A is Indoor & UE distribution#B is Outdoor, 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei] observe minor loss of less than -0.7% degradation or positive gain

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-1% loss or positive gain) for UE distribution#B subject to any of Outdoor and Indoor, if the training dataset is constructed with data samples subject to multiple UE distributions including UE distribution#B, as observed by 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei].

  - Minor loss (0%~-1%) are observed by 3 sources [NTT DOCOMO, Nokia, Huawei].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, Qualcomm, Huawei].

  - Note: Moderate degradations of up to -3.9% are still observed by 1 source [Nokia] for deployment scenario#B subject to Indoor.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Note: Results refer to Table 5.1-6 of R1-2306059

Observation

For the scalability verification of AI/ML based CSI compression over various Tx port numbers, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain Tx port number#B and applied for inference with a same Tx port number#B,

- For generalization Case 2, significant performance degradations are observed in general, if Tx port number#A is 32 & Tx port number#B is 16, as -3.37%~-21.8% degradations are observed by 4 sources [OPPO, Fujitsu, ZTE, vivo]

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4% loss or positive gains) for Tx port number#B subject to any of 16 and 32, if the training dataset is constructed with data samples subject to multiple Tx port numbers including Tx port number#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, as observed by 7 sources [Huawei, OPPO, NTT DOCOMO, CATT, ZTE, Fujistu, Nokia].

  - Minor loss (0%~-1.75%) are observed by 6 sources [Huawei, OPPO, Fujistu, CATT, ZTE, NTT DOCOMO].

  - Moderate loss (-1.84%~-4%) are observed by 3 sources [Nokia, CATT, NTT DOCOMO].

  - Positive gains are observed by 3 sources [OPPO, ZTE, Fujistu].

  - Note: Significant degradations of up to -6.08% are still observed by 1 source [CATT] for deployment scenario#B subject to 32 ports, and for deployment scenario#B subject to 16 ports

  - Note: Pre/post-processing of truncation/padding is adopted by 6 sources [Huawei, OPPO, NTT DOCOMO, ZTE, Fujistu, Nokia], and adaptation layer in the AL/ML model is adopted by 1 source [CATT].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - Note: Results refer to Table 5.1-7 of R1-2306059

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 1 source [Huawei] observes 1.2%~4.2% gain;

    - 1 source [Apple] observes 7.6%~8.5% gain;

    - 1 source [vivo] observes 9.7%~17.2% gain.

    - 1 source [MediaTek] observes 22.6%~ 48.6% gain.

  - For full buffer traffic:

    - 1 source [Nokia] observes 2%~3% gain;

    - 1 source [vivo] observes 8.7% gain.

    - 1 source [MediaTek] observes 1.01% gain.

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 1 source [Huawei] observes 0.7%~3.1% gain;

    - 1 source [vivo] observes 3.4%~7.0% gain.

  - For full buffer traffic:

    - 1 source [vivo] observes 8.1% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-8 of R1-2306059

Observation

For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of 5% UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 2 sources [Huawei, vivo] observes 4.5%~9.3% gain;

    - 3 sources [Huawei, Apple, vivo] observes 11.3%~20.1% gain;

  - For full buffer traffic:

    - 2 sources [Nokia, vivo] observe 6%~17.5% gain;

- Compared to the benchmark of an auto-regression based CSI prediction:

  - For FTP traffic:

    - 2 sources [Huawei, vivo] observes 0.5%~16% gain;

  - For full buffer traffic:

    - 1 source [vivo] observes 11% gain.

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h or 60km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-9 of R1-2306059

Observation

For the generalization verification of AI/ML based CSI prediction over various UE speeds, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h & UE speed#A is 30 km/h, 4 sources [Xiaomi, CATT, Interdigital, Spreadtrum] observe a generalized performance of less than -2% degradation.

  - If UE speed#B is either 30 km/h or 60 km/h or 120 km/h, or if UE speed#B is 10km/h and UE speed#A is either 60km/h or 120km/h, 8 sources [Xiaomi, Samsung, Interdigital, Fujitsu, ZTE, ETRI, vivo, Huawei] observe that moderate/significant performance degradations are suffered:

    - For UE speed#B is 10 km/h & UE speed#A is either 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-2.7% loss), 1 source [Samsung] observes significant degradation (-53%~-61% loss).

    - For UE speed#B is 30 km/h & UE speed#A is either 10 km/h, 60 km/h or 120 km/h, 1 source [Xiaomi] observes moderate degradation (-3% loss), 8 sources [Xiaomi, Interdigital, Fujitsu, vivo, ZTE, Huawei, ETRI, Spreadtrum] observe significant degradation (-6%~-45.6% loss).

    - For UE speed#B is 60 km/h & UE speed#A is either 10 km/h, 30 km/h or 120 km/h, 1 source [ZTE] observes moderate degradation (-3% loss), 7 sources [Samsung, Xiaomi, Fujitsu, ETRI, ZTE, vivo, Spreadtrum] observe significant degradation (-7.8%~-52% loss).

    - For UE speed#B is 120 km/h & UE speed#A is either 30 km/h or 60 km/h, 1 source [ZTE] observes moderate degradation (-3.4% loss), 4 sources [ZTE, ETRI, vivo, Samsung] observe significant degradation (-7.55%~-32.3% loss).

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-4.45% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 9 sources [Xiaomi, Interdigital, Apple, Huawei, ZTE, Samsung, ETRI, vivo, Spreadtrum].

  - For UE speed#B is 10 km/h, minor loss (-0.6%~-1%) are observed by 3 sources [CATT, Xiaomi, Spreadtrum].

  - For UE speed#B is 30 km/h, minor loss (-0.08%~-1.34%) are observed by 3 sources [Xiaomi, Apple, Huawei], moderate loss (-2.2%~-4.07%) are observed by 3 sources [Interdigital, vivo, Spreadtrum].

  - For UE speed#B is 60 km/h, minor loss (-0.05%~-2%) are observed by 4 sources [ZTE, Apple, Xiaomi, Huawei], moderate loss (-2%~-3.76%) are observed by 2 sources [vivo, Spreadtrum].

  - For UE speed#B is 120 km/h, moderate loss (-2%~-4.45%) are observed by 4 sources [vivo, Samsung, ETRI, ZTE].

  - Note: For generalization Case 3, 5 sources [ETRI, ZTE, Samsung, Interdigital, Fujitsu] observe significant performance degradations (-5%~-26.5% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, but compared with generalization Case 2, in general the performance are still improved.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - No spatial consistency is considered

  - Note: Results refer to Table 5.1-10 of R1-2306059

Observation

For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, training non-aware quantization (Case 1) is in general inferior to the training aware quantization (Case 2-1/2-2), and may lead to lower performance than the benchmark.

- For scalar quantization, compared with benchmark,

  - -5.9%~-43.2% degradations are observed for training non-aware quantization (Case 1) from 4 sources [Source#1, Source#2, Source#3, Source#8].

  - 3.9%~8.64% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 5 sources [Source#1, Source#2, Source#3, Source#4, Source#5], which are 17.3%~83.2% gains over training non-aware quantization (Case 1) from 4 sources [Source#1, Source#2, Source#3, Source#8] and 0.9%~5.4% gains over training non-aware quantization (Case 1) from 2 sources [Source#6, Source#8].

    - Note: 0.72% gains are observed for Case 2-1 from 1 source [Source#1] due to SQ parameter chosen without matching latent distribution, which achieves 13.9% gains over Case 1.

  - 7.55% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 1 source [Source#1], which are 21.6% gains over training non-aware quantization (Case 1) from 1 source [Source#1].

- For vector quantization, compared with benchmark,

  - -2%~-10% degradations are observed for training non-aware quantization (Case 1) from 1 source [Source#7].

  - 6.0%~8.91% gains are observed for training aware quantization with fixed/pre-configured quantization method/parameters (Case 2-1) from 2 sources [Source#1, Source#2], which are 16.3%~23.1% gains over training non-aware quantization (Case 1) from 2 sources [Source#1, Source#2].

  - 4.67%~13.01% gains are observed for training aware quantization with jointly updated quantization method/parameters (Case 2-2) from 6 sources [Source#1, Source#2, Source#4, Source#5, Source#7, Source#8], which are 10.7%~27.8% gains over training non-aware quantization (Case 1) from 3 sources [Source#1, Source#2, Source#7] and 1.7%~7.5% gains over training non-aware quantization (Case 1) from 2 sources [Source#6, Source#8].

  - In general, Case 2-2 outperforms Case 2-1 with 0.7%~3.8% gains, as observed by 6 sources [Source#1, Source#2, Source#4, Source#5, Source#6, Source#8].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

  - Source#1: Qualcomm (R1-2305328); Source#2: vivo (R1-2304471); Source#3: Ericsson (R1-2304521); Source#4: ZTE (R1-2304534); Source#5: Xiaomi (R1-2304893); Source#6: Fujitsu (R1-2304764); Source#7: Huawei, HiSilicon (R1-2304653); Source#8: Apple (R1-2305234).

  - Note: Results refer to Table 5.4-1 of R1-2306060

Observation

For the comparison of quantization methods for CSI compression, till the RAN1#113 meeting, in general vector quantization (VQ) has comparable performance with scalar quantization (SQ):

- For SQ and VQ under the same training case, it is

  - observed by 1 source [Source#1] that VQ under Case 2-1 has -0.8% degradation over SQ under Case 2-1,

  - observed by 2 sources [Source#2, Source#3] that VQ under Case 2-1 has 0.3%~1.1% gain over SQ under Case 2-1, and

  - observed by 3 sources [Source#2, Source#3, Source#4] that VQ under Case 2-2 has 0.7%~5.1% gain over SQ under Case 2-2.

  - Note: VQ under Case 2-1 has 8% gains over SQ under Case 2-1 as observed from 1 source [Source#2] due to non-optimized SQ parameter chosen.

- For SQ and VQ across training cases, it is

  - observed by 5 sources [Source#1, Source#2, Source#3, Source#5, Source#6] that VQ under Case 2-2 has 0.5%~4% gain over SQ under Case 2-1, and

  - observed by 1 source [Source#5] that VQ under Case 2-2 has -1.3% degradation over SQ under Case 2-1.

- Note: in general, more companies (Source#1, Source#2, Source#3, Source#4, Source#5, Source#6) observing gain of VQ over SQ than companies observing loss (Source#1, Source#5).

- Note: it is observed by 1 source [Source#5] that combined SQ and VQ under Case 2-2 has minor gain of 0.2% over VQ only under Case 2-2.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Benchmark is Rel-16 Type II codebook.

  - Source#1: vivo (R1-2304471); Source#2: Qualcomm (R1-2305328); Source#3: Apple (R1-2305234); Source#4: Lenovo (R1-2305202); Source#5: ZTE (R1-2304534); Source#6: Xiaomi (R1-2304893);.

  - Note: Results refer to Table 5.4-2 of R1-2306060

Agreement

For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth1 KPIth_1 in Option 1, the candidate threshold values are set as 0.02, 0.05 and 0.1

Agreement

For the intermediate KPI monitoring of CSI compression, for the FFS issue on the value of threshold of KPIth_2 and KPIth_3  in Option 2, consider KPIth_2   = KPIth_3.

Agreement

For the evaluation of training Type 3 under CSI compression, for the benchmark case (1-on-1 joint training) for performance comparison, the structures for the pair of NW part model/UE part model for the new case are the same with the Type 3 case to be compared.

E.g., if the Type 3 is Transformer#1 for NW part model and CNN#1 for UE part model, then the benchmark case for performance comparison is also Transformer#1 for NW part model and CNN#1 for UE part model with joint training.

Agreement

For the intermediate KPI monitoring of CSI compression, between the two options to calculate KPIdiffKPIDiff achieved in the RAN1#112bis-e meeting, as baseline for calibration purpose, consider Option 1 (Gap between KPIActualKPIActual and KPIGenieKPIGenie).

- Option 2 (Binary state of KPIActual  and KPIGenie relationship) as optional and up to companies to report.

  - Results subject to Option 2, may be captured as a note in observation

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of mean UPT under full buffer, more gains are achieved by Max rank 2 compared with Max rank 1 in general:

- For Max rank 1, 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11%

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 6%~11% at CSI overhead A (small overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 3%~7% at CSI overhead B (medium overhead);

  - 5 sources [Huawei, Nokia, vivo, OPPO, Fujitsu] observe the performance gain of 1.1%~11% at CSI overhead C (large overhead);

- For Max rank 2, 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 0.2%~15%

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~15% at CSI overhead A (small overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of 4%~10% at CSI overhead B (medium overhead);

  - 7 sources [Huawei, Nokia, vivo, Fujitsu, Qualcomm, Intel, InterDigital] observe the performance gain of -0.2%~14% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 24.47%~28.24%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: For Max rank 4, 1 source [ZTE] observes gain of 7.44%~9.95% over CSI overhead A/B/C.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.5-1 of R1-2306061

Observation

For the evaluation of AI/ML based CSI compression, till the RAN1#113 meeting, compared to the benchmark, in terms of 5% UPT under full buffer,

- For Max rank 1, 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~20.9%

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.5%~20.9% at CSI overhead A (small overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 2.3%~17.4% at CSI overhead B (medium overhead);

  - 3 sources [Nokia, vivo, Fujitsu] observe the performance gain of 0%~6.62% at CSI overhead C (large overhead);

- For Max rank 2, 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~14.9%

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 4.1%~14.9% at CSI overhead A (small overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of 0.3%~4% at CSI overhead B (medium overhead);

  - 5 sources [Nokia, vivo, Fujitsu, Qualcomm, Intel] observe the performance gain of -7%~6.03% at CSI overhead C (large overhead);

  - Note: 1 source [Xiaomi] observe gain of 8.76%~30.17%, over CSI overhead A/B/C, which bias from the majority ranges.

- Note: For Max rank 4, 1 source [ZTE] observes gain of 3.59%~6.15% over CSI overhead A/B/C.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.5-2 of R1-2306061

Agreement

For the evaluation of the R16 eType II-like codebook based high resolution quantization of the ground-truth CSI in the CSI compression for AI/ML training, regarding the evaluation of new values of eType II parameters, consider the legacy values of PC6&PC8 as the baseline/lower-bound of performance comparison.

- Note: it has been agreed that Float32 is adopted as the baseline/upper-bound of performance comparison.

Observation

For the generalization verification of AI/ML based CSI compression over various carrier frequencies, till the RAN1#113 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B and applied for inference with a same carrier frequency#B,

- For generalization Case 2, generalized performance may be achieved in general

  - If carrier frequency#A is 3.5/4GHz & carrier frequency#B is 2GHz, 3 sources [NTT DOCOMO, Nokia, MediaTek] observe generalized performance of less than -1.72% degradation.

  - If carrier frequency#A is 2GHz & carrier frequency#B is 3.5/4GHz, 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek] observe generalized performance of less than -1% degradation or positive gain.

    - Note: 1 source [Nokia] observes significant degradations of -6.6%.

- For generalization Case 3, generalized performance of the AI/ML model may be achieved (0%~-0.8% loss or positive gain) for carrier frequency#B subject to any of 2GHz and 3.5/4GHz, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B, as observed by 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek].

  - Minor loss (0%~-0.8%) are observed by 3 sources [Nokia, vivo, MediaTek].

  - Positive gains are observed by 4 sources [NTT DOCOMO, Nokia, vivo, MediaTek].

  - Note: Significant degradations of up to -4.9% are still observed by 1 source [Nokia] for carrier frequency#B subject to 3.5/4GHz

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - Antenna layouts are assumed as the same over the different frequency carriers.

  - Note: Results refer to Table 5.5-3 of R1-2306061

Working Assumption

For the template of Table 1. Evaluation results for CSI compression of 1-on-1 joint training without model generalization/scalability, update the entry of CQI determination method(s) to include also the RI determination:

| Common description | Input type |
| --- | --- |
|  | Output type |
|  | Quantization /dequantization method |
|  | Rank/layer adaptation settings for rank>1 |
|  | CQI/RI determination method(s) for AI/ML (Option 1a/1b/1c/2a/2b, etc.) |

Observation

For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where the same backbone is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization:

  - For the case where the shared output of the Network side CSI generation part is after quantization, 5 sources [Ericsson, Nokia, ZTE，Fujitsu, Samsung] observe -0%~-0.5% degradation, 6 sources [Nokia, Qualcomm, ZTE, CATT, vivo, Samsung] observe -0.5%~-1% degradation, and 2 sources [Nokia, ZTE] observe -1%~-1.3% degradation.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 3 sources [Huawei, Apple, CMCC] observe -0%~-0.8% degradation.

- Note: For the NW first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the UE part model is less capable than the NW part model, 1 source [ZTE] observes -0%~-0.5% degradation, 2 sources [ZTE, CATT] observe -0.5%~-1% degradation, and 2 sources [Qualcomm, vivo] observe -2.1%~-5.2% degradation.

  - If the backbone of the UE part model is more capable than the NW part model, 1 source [ZTE] observes -0.08%~-0.64% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.6-1 of R1-2306062

Observation

For the evaluation of NW first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE part model is a subset of the dataset#1 applied for training the NW part model,

- If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.55% gap is observed from 2 sources [Huawei, CMCC].

- If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.55%~-8.41% gap is observed from 2 sources [CMCC, vivo].

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Note: Results refer to Table 5.6-2 of R1-2306062

Observation

For the evaluation of UE first separate training with dataset sharing manner for CSI compression, till the RAN1#113 meeting, for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where the same backbone is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization:

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 5 sources [Nokia, Fujitsu, CATT, vivo, Qualcomm] observe -0%~-0.5% degradation, and 1 source [ZTE] observes -1.05%~-1.75% degradation.

  - For the case where the shared output of the UE side CSI reconstruction part is before quantization, 1 source [Huawei] observes -0%~-1% degradation, and 1 source [Apple] observe -1%~-2.9% degradation.

- Note: For the UE first separate training case where different backbones are adopted for the NW part model and the UE part model, and

  - If the backbone of the NW part model is less capable than the UE part model, 1 source [Qualcomm] observes 0%~-0.5% degradation, 2 sources [CATT, ZTE] observes -0.5%~-1% degradation, and 2 sources [ZTE, vivo] observe -1%~-1.88% degradation.

  - If the backbone of the NW part model is more capable than the UE part model, 1 source [ZTE] observes -0.73%~-1.74% degradation.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.6-3 of R1-2306062

### Agreements of the 114 meeting

Agreement

For the evaluation of CSI enhancements, update the observations drawn in previous meetings to Updated Observation 2.1.8, Updated Observation 2.1.10, Updated Observation 2.1.12, Observation 2.1.15, and Updated Observation 2.1.20 in R1-2308340.

Agreement

For the evaluation of CSI enhancements, update the observations drawn in previous meetings to Updated Observation 2.1.1, Updated Observation 2.1.4, Updated Observation 2.1.5, Observation 2.1.9, and Updated Observation 2.1.11 in R1-2308340.

Note: for update observation 2.1.4, for Rank 2, 2 sources [xiaomi, MediaTek] observe the performance gain of 2% at CSI overhead B (medium overhead).

Note: for Updated Observation 2.1.11, Scalability of AI/ML based CSI compression over various CSI payload sizes can also be achieved by finetuning models on CSI payload size#B, showing loss [0%~-2.2%] by 2 sources [Ericsson, vivo].

Agreement

For the evaluation of CSI enhancements, update the observations drawn in previous meetings to Updated Observation 2.1.2, Updated Observation 2.1.3, Updated Observation 2.1.6, Updated Observation 2.1.7, Updated Observation 2.1.13, and Updated Observation 2.1.14 in R1-2308342.

Observation

For the evaluation of high resolution quantization of the ground-truth CSI for the training of CSI compression, compared to the upper-bound of Float32, quantized high resolution ground-truth CSI can achieve significant overhead reduction with minor performance loss if the parameters are appropriately selected.

- For high resolution scalar quantization,

  - Float16 achieves 50% overhead reduction and -0.6% or less performance loss from 2 sources [vivo, Apple]

  - 8 bits scalar quantization achieves 75% overhead reduction and -0.14%~-0.9% performance loss from 2 sources [Huawei, Apple]

- For high resolution R16 eType II-like quantization,

  - R16 eType II CB with legacy parameters can achieve significant overhead reduction while with performance loss compared to Float32, wherein

    - PC#6 achieves around 99% overhead reduction with -1.4% ~-1.7% performance loss from 2 sources [Huawei, Fujitsu], and -3%~-9.5% performance loss from 4 sources [Huawei, vivo, ZTE, Fujitsu].

    - PC#8 achieves around 98% overhead reduction with 0% ~-1.7% performance loss from 3 sources [Qualcomm, Huawei, Fujitsu], and -2.9%~-5.5% performance loss from 5 sources [Qualcomm, Huawei, vivo, ZTE, MediaTek].

  - For R16 eType II CB with new parameters:

    - R16 eType II CB with new parameter of 1000-1400bits CSI payload size achieves 95%~97.5% overhead reduction (3~4.1 times overhead compared to PC8) with performance gain of 0.7%~4.3% over PC#8 from 4 sources [Huawei, vivo, ZTE, Ericsson].

    - R16 eType II CB with new parameter of 1500-2100bits CSI payload size achieves 94%~96.2% overhead reduction (4.8~6.1 times overhead compared to PC8) with performance gain of 1.3%~5.4% over PC#8 from 3 sources [Huawei, ZTE, Fujitsu].

    - Note: it is observed by 1 source [Qualcomm] that using R16 eType II-like quantization with legacy PC may achieve close performance to Float32 by dataset dithering.

- Note: the new parameters include at least one from the follows:

  - L= 8, 10, 12;

  - pv = 0.8, 0.9, 0.95;

  - reference amplitude = 6 bits, 8 bits; differential amplitude = 4bits; phase = 5 bits, 6 bits;

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS for Layer 1.

  - Note: Results refer to Table 5.18 of R1-2308342

Observation

For the generalization verification of AI/ML based CSI compression over various TxRU mappings, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain TxRU mapping#B and applied for inference with a same TxRU mapping#B,

- For generalization Case 2, significant degradations are suffered in general from the perspective of the layouts of antenna ports, as observed by 2 sources [MediaTek, Nokia]:

  - For TxRU mapping#A is [2,8,2] & TxRU mapping#B is [4,4,2] or TxRU mapping#A is [8,2,2] & TxRU mapping#B is [4,4,2], 2 sources [MediaTek, Nokia] observe -13%~-36.1% degradation.

  - For TxRU mapping#A is [4,4,2] & TxRU mapping#B is [2,8,2] or TxRU mapping#A is [8,2,2] & TxRU mapping#B is [2,8,2], 2 sources [MediaTek, Nokia] observe -7%~-23.6% degradation.

  - For TxRU mapping#A is [4,4,2] & TxRU mapping#B is [8,2,2] or TxRU mapping#A is [2,8,2] & TxRU mapping#B is [8,2,2], 1 source [MediaTek] observes -19%~-27% degradation.

- For generalization Case 2, generalized performance may be achieved for some certain combinations of TxRU mapping#A and TxRU mapping#B but not for others, from the perspective of the layouts of antenna element mapping, as observed by 2 sources [Huawei, vivo]:

  - For TxRU mapping#A is 8x8x2 & TxRU mapping#B is 2x8x2, 2 sources [Huawei, vivo] observe minor/moderate degradation of -0.6%~-2.5%.

  - For TxRU mapping#A is 2x8x2 & TxRU mapping#B is 8x8x2, 1 source [Huawei] observes moderate degradation of -3%.

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (0%~-4.4% loss or positive gain) for TxRU mapping#B subject to any of [2,8,2], [4,4,2], and [8,2,2] from the perspective of the layouts of antenna ports, or subject to any of 8x8x2 and 2x8x2 from the perspective of the layouts of antenna element mapping, if the training dataset is constructed with data samples subject to TxRU mappings including TxRU mapping#B, as observed by 4 sources [MediaTek, Apple, Nokia, Huawei].

  - Minor loss (0%~-2%) are observed by 4 sources [MediaTek, Apple, Nokia, Huawei].

  - Moderate loss (-2.5%~-4.4%) are observed by 1 source [Nokia].

  - Positive gains are observed by 1 source [Apple].

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - [x,y,z] for TxRU mapping: Vertical port number, Horizontal port number, polarization

  - AxBxC for TxRU mapping: AxBxC antenna elements virtualized to [2,8,2]

  - Note: Results refer to Table 5.19 of R1-2308342

Observation

For the evaluation of NW first separate training with dataset sharing manner for CSI compression, for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the NW first separate training case where different backbones are adopted for the NW part model and the UE part model, more degradations are observed in general than the situation where the same backbone is adopted for the NW part model and the UE part model.

  - For the case where the shared output of the Network side CSI generation part is after quantization, 3 sources [ZTE, Xiaomi, CATT] observes minor degradation of -0%~-1.02%, and 3 sources [Qualcomm, vivo, Fujitsu] observe moderate degradation of -1.46%~-5.1%.

  - For the case where the shared output of the Network side CSI generation part is before quantization, 2 sources [Huawei, CMCC] observe minor degradation of -0%~-0.1%, 1 source [CMCC] observes moderate degradation of -2.03%.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

- Note: Results refer to Table 5.16 of R1-2308342.

Observation

For the evaluation of UE first separate training with dataset sharing manner for CSI compression, for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model,

- For the UE first separate training case where different backbones are adopted for the NW part model and the UE part model, more degradations are observed in general than the situation where the same backbone is adopted for the NW part model and the UE part model.

  - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 5 sources [Qualcomm, Xiaomi, CATT, ZTE, vivo] observes minor degradation of -0.23%~-1.07%, and 1 source [ZTE] observes moderate degradation of -1.74%~-1.88%.

  - For the case where the shared input of the UE side CSI reconstruction part is before quantization, 1 source [CMCC] observes moderate degradation of -1.58%~-2.73%.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1/2.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - Note: Results refer to Table 5.17 of R1-2308342

Observation

For the evaluation of NW first separate training with dataset sharing manner for CSI compression, for the pairing between 1 UE part model and N>1 separate NW part models (Case 3), when taking 1-on-1 joint training between the NW part model and the UE part model as benchmark, larger performance loss is observed in general than the case of NW first separate training with 1 UE part model and 1 NW part model pairing (Case 1):

- 6 sources [Huawei, Nokia, Qualcomm, Fujitsu, CATT, Xiaomi] observe minor loss of -0%~-1.6% compared to the 1-on-1 joint training.

- 3 sources [Nokia, CATT, Xiaomi] observe moderate loss of -1.9%~-6.64% compared to the 1-on-1 joint training.

- 5 sources [vivo, Samsung, OPPO, MediaTek, Apple] observe significant loss of -37.9%~-87% compared to the 1-on-1 joint training.

- Note: as opposed to companies which observe significant loss, the minor loss observed by other companies may due to the fact that special handling (e.g., adaptation layer) is performed to pair with N>1 NW part models during the training at the UE side.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and NW first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - N=2, 3, or 4 are considered.

  - Note: Results refer to Table 5.20 of R1-2308342

Observation

For the evaluation of intermediate KPI based monitoring mechanism for CSI compression, for monitoring Case 1, in terms of monitoring accuracy with Option 1,

- For ground truth CSI format of R16 eType II CB, monitoring accuracy is increased with the increase of the resolution for the ground-truth CSI (number of bits for each sample of ground-truth CSI) in general, with the impact of increased overhead, wherein

  - for ground truth CSI format of R16 eType II CB with PC#6, 4 sources [vivo, Ericsson, Intel, Qualcomm] observe KPIDiff as 13.2%~71.6%/ 28.5%~100%/ 68.4%~100% for KPIth_1=0.02/0.05/0.1, respectively.

    - Note: two sources [vivo, Qualcomm] observed averaging on the test samples improves the monitoring accuracy.

  - for ground truth CSI format of R16 eType II CB with PC#8, 5 sources observe [Apple, Huawei, ZTE, Ericsson, Intel] KPIDiff as 21%~43.0%/ 48.1%~79.1%/ 79.8%~97.1% for KPIth_1=0.02/0.05/0.1, respectively.

  - for ground truth CSI format of R16 eType II CB with new parameter of 580-750bits CSI payload size, 2 sources [Ericsson, Intel] observe KPIDiff as 35.4%~63%/ 77.9%~93.0%/ 99.5%~99.9% for KPIth_1=0.02/0.05/0.1, respectively, which have 12.7%~20%/ 13.9%~29.8%/ 8%~31.1% gain over PC#8.

  - for ground truth CSI format of R16 eType II CB with new parameter of around 1000bits CSI payload size, 4 sources [Huawei, ZTE, Ericsson, vivo] observe KPIDiff as 34.9%~89%/ 82.9%~100%/ 99.9%~100% for KPIth_1=0.02/0.05/0.1, respectively, which have 12.2%~68%/ 18%~43.62%/ 2.9%~31% gain over PC#8 from 3 sources [Huawei, ZTE, Ericsson] and 4.67%~10.6%/ 0%~5.88%/ 0%~0.49% gain over PC#6 from 1 source [vivo].

  - for ground truth CSI format of R16 eType II CB with new parameter of around 1600bits CSI payload size, 2 sources [Huawei, Fujitsu] observe KPIDiff as 89.1%~97%/ 99.9%~100%/ 100% for KPIth_1=0.02/0.05/0.1, respectively, which have 76%/33%/3% gain over PC#8 from 1 source [Huawei].

- for ground truth CSI format of 4 bits scalar quantization, 2 sources [Ericsson, NTT DOCOMO] observe KPIDiff as 9.4%~47%/ 96.3%~100%/ 100% for KPIth_1=0.02/0.05/0.1, respectively.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Time independency is assumed over the test samples for monitoring

  - Precoding matrix is used as the model input.

  - 1-on-1 joint training is assumed.

  - The performance metric is monitoring accuracy for Layer 1.

  - Note: Results refer to Table 5.21 of R1-2308343

Observation

For the evaluation of intermediate KPI based monitoring mechanism for CSI compression, for Case 2, in terms of monitoring accuracy with Option 1,

- For Case 2-1 subject to generalization Case 1 for the proxy model, 5 sources [Huawei, Lenovo, vivo, ZTE, Fujitsu] observe KPIDiff as 31%~84%/ 65.63%~99.8%/ 95%~100% for KPIth_1=0.02/0.05/0.1, respectively;

  - Compared with monitoring Case 1 with ground truth CSI format of R16 eType II CB with new parameter of around 1000bits CSI payload size,

    - 2 sources [vivo, ZTE] observe +0.99%~+4.07% gain at KPIth_1=0.02;

    - 3 sources [Huawei, vivo, ZTE] observe -6.03%~-58%/ -0.2%~-24%/ 0%~-5% degradation for KPIth_1=0.02/0.05/0.1, respectively;

  - Compared with monitoring Case 1 with ground truth CSI format of R16 eType II CB with new parameter of around 1600bits CSI payload size, 2 sources [Huawei, Fujitsu] observe -16.35%~-66%/ -0.4%~-24%/ 0%~-24% degradation for KPIth_1=0.02/0.05/0.1, respectively.

- Note: For Case 2-1 subject to generalization Case 2 for the proxy model, 2 sources [Huawei, ZTE] observe -1.77%~-37.42% / -1.07%~-23.93%/ -0.16%~-14% compared with generalization Case 1 with the same testing scenario.

- Note: For Case 2-2, 1 source [Qualcomm] observes KPIDiff as 61%~72.1%/ 91.2%~96.6%/ 99.2%~99.75% under generalization Case 1 for the proxy model, and 60%~71.3%/ 90.4%~99.3%/ 99%~100% under generalization Case 3 for the proxy model, for KPIth_1=0.02/0.05/0.1, respectively.

- Note: for Case 2-1, 1 source [Lenovo] observes that if different model backbone is adopted for proxy model as compared to the NW part model, it has negative impact to the monitoring performance.

- Note: for the complexity and overhead analysis:

  - Case 2-1/2-2 has smaller air-interface overhead for UE report for monitoring compared with Case 1. Overhead of proxy model from LCM perspective, if any, is not evaluated.

  - the complexity aspect for case1/2-1/2-2  is not evaluated.

- Note: “Generalization Case 1” means the proxy model is trained based on training dataset from one Scenario#A, and then tested for monitoring on a dataset from the same Scenario#A. “Generalization Case 2” means the proxy model is trained based on training dataset from one Scenario#B, and then tested for monitoring on a dataset from a different Scenario#A. “Generalization Case 3” means the proxy model is trained based on mixing datasets from multiple scenarios including Scenario#A, and then tested for monitoring on the dataset from Scenario#A.

- Note: two sources [vivo, Qualcomm] observed averaging on the test samples improves the monitoring accuracy.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Time independency is assumed over the test samples for monitoring

  - Precoding matrix is used as the model input.

  - 1-on-1 joint training is assumed.

  - The performance metric is monitoring accuracy for Layer 1.

  - Note: Results refer to Table 5.22 of R1-2308343

Observation

For the evaluation of Type 2 training between 1 NW part model and M>1 separate UE part models (Case 2), as compared to joint training between 1 NW part model and the 1 UE part model,

- 7 sources [Huawei, Ericsson, Qualcomm, vivo, Fujitsu, InterDigital, MediaTek] observe minor degradation of -0%~-1.67% or positive gain;

- 3 sources [Huawei, Fujitsu, MediaTek] observe moderate degradation of -2.5%~-6.5%.

- Note: among the above sources, 5 sources [Huawei, Ericsson, Qualcomm, vivo, Fujitsu] adopt simultaneous training, while 1 source [Qualcomm] adopts sequential training starting with NW side training.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1.

  - Same pair of NW part model and UE part model between 1-on-1 joint training and Type 2 training.

  - M=2, 3, or 4 are considered.

  - Note: Results refer to Table 5.23 of R1-2308343

Observation

For the evaluation of Type 2 training between 1 UE part model and N>1 separate NW part models (Case 3), as compared to joint training between 1 NW part model and the 1 UE part model,

- 2 sources [vivo, MediaTek] observe minor degradation of -0%~-0.8% or positive gain;

- 1 source [MediaTek] observe moderate degradation of -1.4%~-4.2%.

- Note: among the above sources, 1 source [vivo] adopts simultaneous training.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1.

  - Same pair of NW part model and UE part model between 1-on-1 joint training and Type 2 training.

  - N=2, 3, or 4 are considered.

  - Note: Results refer to Table 5.24 of R1-2308343

Observation

For the evaluation of UE first separate training with dataset sharing manner for CSI compression, for the pairing between M>1 separate UE part models and 1 NW part model (Case 2), when taking 1-on-1 joint training between the NW part model and the UE part model as benchmark, larger performance loss is observed in general than the case of UE first separate training with 1 UE part model and 1 NW part model pairing (Case 1):

- 8 sources [Nokia, Qualcomm, Fujitsu, Lenovo, Apple, CATT, vivo, Xiaomi] observe minor loss of -0%~-1.82% compared to 1-on-1 joint training.

- 4 sources [Nokia, Lenovo, CATT, CMCC] observe moderate loss of -2.17%~-4.96% compared to 1-on-1 joint training.

- 2 sources [OPPO, MediaTek] observe significant loss of -11.56%~-73.7% compared to 1-on-1 joint training.

- Note: 1 source [Lenovo] observes other UE first separate training implementations may achieve better performance.

- Note: the dataset sharing behavior from above sources follows the example of the agreement in the RAN1#111 meeting, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - The performance metric is SGCS for Layer 1.

  - Same size of training dataset for benchmark, NW part training and the UE part training

  - Same pair of NW part model and UE part model between 1-on-1 joint training and UE first separate training.

  - Quantization/dequantization method/parameters between NW side and UE side are aligned.

  - M=2, 3, or 4 are considered.

  - Note: Results refer to Table 5.25 of R1-2308343

Observation

For the AI/ML based CSI prediction, compared with the benchmark of the nearest historical CSI:

- spatial consistency is not adopted in 15 sources [Huawei, ZTE, ETRI, CMCC, Apple, Spreadtrum, Nokia, CATT, Fujitsu, Samsung, NVIDIA, vivo, InterDigital, Xiaomi, CEWiT], wherein

  - 15 sources [ZTE, Nokia, Spreadtrum, NVIDIA, Apple, Huawei, Samsung, Fujitsu, CATT, vivo, InterDigital, ETRI, Xiaomi, CMCC, CEWiT] observe the gain of 0.46% ~ 44.8% using raw channel matrix as input, wherein

    - 4 sources [Xiaomi, NVIDIA, InterDigital, Samsung] observe the gain of 0.46%~6.3%.

    - 14 sources [Huawei, Samsung, ZTE, Spreadtrum, Fujitsu, CATT, Apple, InteDigital, ETRI, Xiaomi, CMCC, CEWiT, NVIDIA, vivo] observe the gain of 7.57%~26.47%.

    - 5 sources [vivo, Fujitsu, CMCC, CEWiT, Nokia] observe the gain of 29.03%~44.8%.

  - 4 sources [ZTE, CATT, ETRI, OPPO] observe the gain of 2.24% ~ 19.4% using precoding matrix as input, which is in general worse than using raw channel matrix as input

- spatial consistency is adopted in 4 sources, all of which use raw channel matrix as input, wherein

  - 3 sources [Nokia, vivo, MediaTek] observe the gain of 1.7%~35.51%.

  - 1 source [MediaTek] observe the gain of 76.6%.

  - 1 source [InterDigital] observe the loss of -5.5%.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - UE speed includes 10km/h, 30km/h, and 60km/h. The same fixed UE speed is assumed for both training and inference.

  - The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 5.26 of R1-2308344

Observation

For the AI/ML based CSI prediction, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective, in general the gain of AI/ML based solution is related with the UE speed:

- For 10km/h UE speed, 6 sources [Samsung, Xiaomi, InterDigital, CEWiT, MediaTek, NVIDIA] observe 2.4%~12.5% gain (2.4%~12.5% gain for 5 sources [Samsung, Xiaomi, InterDigital, CEWiT, NVIDIA] who do not adopt spatial consistency, and 8.7% gain for 1 source [MediaTek] who adopts spatial consistency), 1 source [CMCC] observes 21.93% gain (who does not adopt spatial consistency).

- For 30km/h UE speed, 1 source [InterDigital] observes loss of -5.5% (who adopts spatial consistency), 3 sources [OPPO, ETRI, CATT] observe 6%~10.43% gain (who do not adopt spatial consistency), 8 sources [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum, InterDigital, NVIDIA, vivo] observe 12.65%~33% gain (14.65%~33% gain for 7 sources [ZTE, Fujitsu, Apple, Xiaomi, Spreadtrum, InterDigital, NVIDIA] who do not adopt spatial consistency, and 12.65% gain for 1 source [vivo] who adopts spatial consistency), and 3 sources [MediaTek, CMCC, CEWiT] observe 41.75%~ 76.6% gain (41.75%~ 44.8% gain for 2 sources [CMCC, CEWiT] who do not adopt spatial consistency, and 76.6% gain for 1 source [MediaTek] who adopts spatial consistency), which are in general larger than 10km/h UE speed.

- For 60km/h UE speed, 3 sources [Xiaomi, NVIDIA, MediaTek] observe 0.46%~2.6% gain (0.46%~2.3% gain for 2 sources [Xiaomi, NVIDIA] who do not adopt spatial consistency, and 1.7%~2.6% gain for 1 source [MediaTek] who adopts spatial consistency), 7 sources [Huawei, Samsung, vivo, CMCC, Fujitsu, CATT, Spreadtrum] observe 9.1%~20.6% gain (9.1%~20.6% gain for 6 sources [Huawei, Samsung, CMCC, Fujitsu, CATT, Spreadtrum] who do not adopt spatial consistency, and 13.8% gain for 1 source [vivo] who adopts spatial consistency), 1 source [vivo] observe 29.03% gain, which are in general smaller than 30km/h UE speed.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - The same fixed UE speed is assumed for both training and inference.

- Note: Results refer to Table 5.27 of R1-2308344

Observation

For the AI/ML based CSI prediction, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 4 sources [Huawei, Spreadtrum, InterDigital, vivo] observe 1.2%~4.9% gain;

    - 2 sources [Apple, vivo] observe 5.3%~10.58% gain;

    - 2 sources [vivo, MediaTek] observe 15.1% ~23.5% gain.

    - 1 source [InterDigital] observes loss of -1.3%~-13.8%.

  - For full buffer traffic:

    - 1 source [Nokia] observes 2%~3% gain;

    - 2 sources [vivo, MediaTek] observe 7.6%~15.6% gain.

- Compared to the benchmark of an auto-regression/Kalman filter based CSI prediction:

  - For FTP traffic:

    - 3 sources [Huawei, vivo, MediaTek] observe 0.7%~7.0% gain;

    - 2 sources [MediaTek, InterDigital] observe loss of -0.1%~-2.4%.

    - 1 source [MediaTek, InterDigital] observe loss of -3%~-17%.

  - For full buffer traffic:

    - 2 sources [vivo, MediaTek] observes 0.6%~2.78% gain.

    - 1 source [vivo] observes 8.1%~11.5% gain.

- Note: the above results are based on the following assumptions

  - The same fixed UE speed of 30km/h or 60km/h is assumed for both training and inference

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - Note: Results refer to Table 5.28 of R1-2308344

Observation

For the AI/ML based CSI prediction, in terms of 5% UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark of the nearest historical CSI:

  - For FTP traffic:

    - 4 sources [Huawei, vivo, Spreadtrum, InterDigital] observe 1% ~9.7% gain;

    - 5 sources [Huawei, Apple, vivo, InterDigital, Spreadtrum] observe 10%~26.4% gain;

    - 1 source [InterDigital] observes loss of -11.6%~-14%;

  - For full buffer traffic:

    - 3 sources [Nokia, vivo, MediaTek] observe 3.5%~35.3% gain;

- Compared to the benchmark of an auto-regression/Kalman filter based CSI prediction:

  - For FTP traffic:

    - 3 sources [Huawei, vivo, InterDigital] observe 0.18%~17.58% gain;

    - 1 source [InterDigital] observes -8.2%~-12.4% degradation;

  - For full buffer traffic:

    - 1 source [vivo] observes 6.7% ~15.4% gain.

    - 1 source [MediaTek] observes -2% degradation

- Note: the above results are based on the following assumptions

  - The same fixed UE speed of 30km/h or 60km/h is assumed for both training and inference

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - No post processing is considered.

  - Note: Results refer to Table 5.29 of R1-2308344

Observation

For the evaluation of AI/ML based CSI compression, compared to the benchmark, in terms of CSI feedback reduction,

- For Max rank = 1,

  - For CSI overhead A (small overhead), 1 source [ZTE] observes the CSI feedback reduction of 10.24% for FTP traffic;

  - For CSI overhead B (medium overhead), 3 sources [Huawei, ZTE, Futurewei] observe the CSI feedback reduction of 15.62%~60% for FTP traffic, and 2 sources [Huawei, Qualcomm] observes the CSI feedback reduction of 37%~66% for full buffer;

  - For CSI overhead C (large overhead), 2 sources [Huawei, ZTE] observe the CSI feedback reduction of 14.37%~55% for FTP traffic, and 2 sources [Huawei, Qualcomm] observes the CSI feedback reduction of 50%~53% for full buffer;

  - Note: For CSI overhead C (large overhead), 1 source [Futurewei] observes CSI feedback reduction of 75% for FTP traffic.

- For Max rank = 2,

  - For CSI overhead A (small overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 20.83%~54% for FTP traffic, and 1 source [Qualcomm] observes the CSI feedback reduction of 56% for full buffer;

  - For CSI overhead B (medium overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 22.22%~52% for FTP traffic, and 2 sources [Huawei, Qualcomm] observe the CSI feedback reduction of 52% for full buffer;

  - For CSI overhead C (large overhead), 3 sources [Futurewei, Qualcomm, ZTE] observe the CSI feedback reduction of 10%~58.33% for FTP traffic, and 2 sources [Huawei, Qualcomm] observe the CSI feedback reduction of 22%~54% for full buffer;

  - Note: For CSI overhead B (medium overhead), 1 source [Futurewei] observe CSI feedback reduction of up to ~83% for FTP traffic using particular VQ codebook solution.

- For Max rank = 4,

  - For CSI overhead A (small overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 50%~79% for FTP traffic, and 1 source [ZTE] observes the CSI feedback reduction of 70.53% for full buffer;

  - For CSI overhead B (medium overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 36.10%~78% for FTP traffic, and 1 source [ZTE] observes the CSI feedback reduction of 47.74% for full buffer;

  - For CSI overhead C (large overhead), 2 sources [Qualcomm, ZTE] observe the CSI feedback reduction of 8%~58% for FTP traffic, and 1 source [ZTE] observes the CSI feedback reduction of 42.59% for full buffer;

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix of the current CSI is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is CSI overhead reduction for Max rank 1/2/4.

  - Benchmark is Rel-16 Type II codebook.

  - Note: Results refer to Table 5.30 of R1-2308344

Observation

For the scalability verification of AI/ML based CSI compression over various bandwidths, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain bandwidth#B and applied for inference with a same bandwidth#B,

- For generalization Case 2, if bandwidth#A is 20MHz & bandwidth#B is 10MHz, or bandwidth#A is 10MHz & bandwidth#B is 20MHz, or bandwidth#A is 10MHz & bandwidth#B is 5MHz:

  - 2 sources [ZTE, Ericsson] observe that generalized performance can be achieved:

    - For bandwidth#A is 20MHz & bandwidth#B is 10MHz, 1 source [ZTE] observe less than -1.28% degradation.

    - For bandwidth#A is 10MHz & bandwidth#B is 20MHz, 2 sources [ZTE, Ericsson] observe less than -1.1% degradation.

  - 1 source [InterDigital] observe that moderate/significant degradations are suffered under generalization Case 2:

    - For bandwidth#A is 10MHz & bandwidth#B is 5MHz, 1 source [InterDigital] observe larger than -2.5% degradation.

- For generalization Case 3, 3 sources [NTT DOCOMO, Nokia, Qualcomm] observe that generalized performance of the AI/ML model can be achieved (0%~-2.97% loss) for bandwidth#B subject to each of 10MHz/52RB and 20MHz and 48RB, if the training dataset is constructed with data samples subject to multiple bandwidths including bandwidth#B.

  - Minor loss (0%~-1.7%) are observed by 2 source [NTT DOCOMO, Nokia].

  - Moderate loss (-1.91%~-2.97%) are observed by 2 sources [Nokia, NTT DOCOMO].

  - Positive gains are observed by 2 sources [Nokia, Qualcomm].

  - Note: Significant loss (-5.4%) is observed by 1 source [Qualcomm]

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Precoding matrix is used as the model input.

  - Training data samples are not quantized, i.e., Float32 is used/represented.

  - 1-on-1 joint training is assumed.

  - The performance metric is SGCS in linear value for layer 1/2.

  - Note: Results refer to Table 5.31 of R1-2308344

Observation

For the AI/ML based CSI prediction, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from observation window length perspective, in general the gain of AI/ML based solution is slightly increased with the increase of the length for the observation window:

- When the observation window is increased from 5/5ms to 8/5ms, the gain over benchmark is increased by 0.28%~2.19%, as observed by 2 sources [Xiaomi, CATT].

- When the observation window is increased from 5/5ms to 15/5ms, the gain over benchmark is increased by 5.59%~10.32%, as observed by 1 source [CMCC].

- When the observation window is increased from 4/5ms to 8/5ms and 10/5ms, the gain over benchmark is increased by 0.96%~4.23% and 1%~4.42%, respectively, as observed by 2 sources [ZTE, vivo].

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - Note: Results refer to Table 5.32 of R1-2308344

Observation

For the AI/ML based CSI prediction, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS/NMSE, from prediction window length perspective, in general the gain of AI/ML based solution is related with the prediction length in terms of the distance to the applicable time of the predicted CSI:

- When the prediction length is increased from 10ms to 15ms, the gain over benchmark is reduced (gap from -1.13%~-51%), as observed by 3 sources [ZTE, ETRI, MediaTek].

- When the prediction length is increased from 2.5ms/3ms to 5ms, the gain over benchmark is increased (gap from +5.85%~+13%), as observed by 2 sources [Apple, vivo].

- When the prediction length is increased from 5ms to 10ms, 5 sources [ZTE, Apple, ETRI, CMCC, OPPO] observe the gain over benchmark is reduced (gap from -1%~-12.1%) while 2 sources [MediaTek, vivo] observe the gain over benchmark is increased (+11.65%~+45.5%).

- Note: the above results are based on the following assumptions

  - The UE speed is 30km/h.

  - The observation window considers to start as early as 15ms~50ms.

  - Raw channel matrix is considered as model input

  - The performance metric is SGCS in linear value for layer 1.

  - No post processing is considered.

  - Note: Results refer to Table 5.33 of R1-2308344

Observation

For the evaluation of CSI compression, for the type of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part), a vast majority of companies adopt precoding matrix as model input/output.

- Note: For the evaluations of CSI compression with 1-on-1 joint training, 22 sources [Huawei, Nokia, Futurewei, Lenovo, ZTE, vivo, OPPO, Spreadtrum, Fujitsu, NTT DOCOMO, Xiaomi, Qualcomm, Intel, InterDigital, CATT, Apple, China Telecom, MediaTek, BJTU, ETRI, CMCC, Ericsson] take precoding matrix without angular-delay domain conversion as the model input/output; 2 sources [Ericsson, Samsung] takes precoding matrix with angular-delay domain representation as the model input/output. No company submitted explicit channel matrix as input.

### Agreements of the 114bis meeting

Agreement

- Adopt the following TP to TR 38.843 to describe the procedure of inference for CSI compression

------------------ Text Proposal for 38.843 v1.0.0 Clause 6.2.1 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** CSI compression sub use case specific aspects: The following figure provides an example for the inference procedure for CSI compression. For generating the input of CSI generation model, it may need some further pre-processing on the measured channel; for the output of the CSI reconstruction model, some further post-processing may also be applied. Besides CSI feedback of quantization output, there may also be other CSI/PMI related information transmitted. There may be other examples of merging quantization/dequantization into the inference for CSI generation/reconstruction, CSI generation model/CSI reconstruction model, respectively. Figure X An example of the CSI compression inference procedure. For the evaluation of the AI/ML based CSI compression sub use case, companies are encouraged to report details of their models, including: *** Unchanged text is omitted ***

Agreement

- Adopt the following TP to TR 38.843 to describe the procedure of inference for CSI prediction.

------------------ Text Proposal for 38.843 v1.0.0 Clause 6.2.1 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** CSI prediction sub use case specific aspects: The following figure provides an example for the inference procedure for CSI prediction. For generating the input of CSI prediction model, it may need some further pre-processing on the measured channel; for the output of the CSI prediction model, some further post-processing may also be applied. Figure X An example of the CSI prediction inference procedure. For the evaluation of the AI/ML based CSI prediction sub use case, companies are encouraged to report details of their models, including: *** Unchanged text is omitted ***

Agreement

- Adopt the following TP to TR 38.843 to capture the complexity results for CSI compression and CSI prediction.

------------------ Text Proposal for 38.843 v1.0.0 Clause 6.1------------------ 6.1 Common evaluation methodology and KPIs *** Unchanged text is omitted *** Common KPIs (if applicable): - Performance - Intermediate KPIs - Link and system level performance - Generalization performance - Over-the-air Overhead - Overhead of assistance information - Overhead of data collection - Overhead of model delivery/transfer - Overhead of other AI/ML-related signalling - Inference complexity, including complexity for pre- and post-processing - Computational complexity of model inference: TOPs, FLOPs, MACs - there may be a disconnection between actual complexity and the complexity evaluated as captured in Section 6 using these KPIs due to the platform-dependency and implementation (hardware and software) optimization solutions - Computational complexity for pre- and post-processing *** Unchanged text is omitted *** ------------------ Text Proposal for 38.843 v1.1.0 Clause 6.2.2 ------------------ 6.2.2 Performance results *** Unchanged text is omitted *** Observations: CSI compression For the evaluation of CSI compression, for the type of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part), a vast majority of companies adopt precoding matrix as model input/output. Note: For the evaluations of CSI compression with 1-on-1 joint training, 22 sources take precoding matrix without angular-delay domain conversion as the model input/output; 2 sources take precoding matrix with angular-delay domain representation as the model input/output. No company submitted explicit channel matrix as input. The complexity metric in terms of FLOPs and number of parameters of AI/ML models adopted in the evaluations of CSI compression with summarized in the following Max rank 1 are figure, where the complexity for the CSI generation part and the complexity for the CSI reconstruction part are illustrated separately. - A majority of 25 sources adopt the CSI generation model subject to the FLOPs from 10M to 800M, and 26 sources adopt the CSI reconstruction model subject to the FLOPs from 10M to 1100M. - A majority of 21 sources adopt the CSI generation model subject to the number of parameters from 1M to 13M, and 22 sources adopt the CSI reconstruction model subject to the FLOPs from 1M to 17M. - Results refer to Table 1 of Section 7.3, R1-2310450. Figure X Complexity of AI/ML models from evaluation results in terms of FLOPs and number of parameters for CSI compression. For the evaluation of the AI/ML based CSI compression sub use case, companies are encouraged to report details of their models, including: *** Unchanged text is omitted *** CSI Prediction The complexity values in terms of FLOPs and number of parameters of AI/ML models adopted in the evaluations of CSI prediction are summarized in the following figure. - Results refer to Table 2 of Section 7.3, R1-2310450. Figure X Complexity of AI/ML models from evaluation results in terms of FLOPs and number of parameters for CSI prediction. For the AI/ML based CSI prediction, compared with the benchmark of the nearest historical CSI: *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to the EVM table to TR 38.843.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** For calibration purposes on the dataset and/or AI/ML model across companies, companies were encouraged to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point. For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. *** Unchanged text is omitted *** Table 6.2.1-1: Baseline System Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations Parameter; Value. *** Unchanged text is omitted ***. CSI feedback; Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms. Overhead; Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation). Traffic model; At least, FTP model 1 with packet size 0.5 Mbytes is assumed. Other options are not precluded. Traffic load (Resource utilization); 20/50/70%. Companies are encouraged to report the MU-MIMO utilization.. UE distribution; CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) CSI prediction: 100% outdoor (10, 20, 30, 60, 120 km/h) including outdoor-to-indoor car penetration loss per TR 38.901 if the simulation assumes UEs inside vehicles. No explicit trajectory modeling considered for evaluations.. UE receiver; MMSE-IRC as the baseline receiver. Feedback assumption; Realistic. Channel estimation; Realistic as a baseline. Up to companies to choose the error modelling method for realistic channel estimation. Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation. FFS ideal channel estimation. *** Unchanged text is omitted ***. Baseline for performance evaluation; For CSI compression: Companies need to report which option is used between: - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. Additional assumptions from R17 TypeII EVM: Same consideration with respect to utilizing angle-delay reciprocity should be considered taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline. Optionally, Type I Codebook (if it outperforms Type II Codebook) can be considered for comparing AI/ML schemes. For CSI-prediction: Both of the followings are taken as baselines Companies need to report which option is used between: - The nearest historical CSI without prediction - Non-AI/ML or AI/ML with collaboration Level x based CSI prediction for which corresponding details would need to be reported Note: the specific non-AI/ML based CSI prediction is compatible with R18 MIMO; collaboration level x AI/ML based CSI prediction could be implementation based AI/ML compatible with R18 MIMO as an example. For the evaluation of CSI enhancements, companies can optionally provide the additional throughput baseline based on CSI without compression (e.g., eigenvector from measured channel), which is taken as an upper bound for performance comparison.. Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. Table 6.2.1-2 presents the baseline link level simulation assumptions for AI/ML based CSI feedback enhancement evaluations. Table 6.2.1-2: Baseline Link Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations Parameter; Value. Duplex, Waveform; FDD (TDD is not precluded), OFDM. Carrier frequency; 2GHz as baseline, optional for 4GHz. Bandwidth; 10MHz or 20MHz. Subcarrier spacing; 15kHz for 2GHz, 30kHz for 4GHz. Nt; 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ. Nr; 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ. Channel model; CDL-C as baseline, CDL-A as optional. UE speed; 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies. Delay spread; 30ns or 300ns. Channel estimation; Realistic channel estimation algorithms (e.g., LS or MMSE) as a baseline, FFS ideal channel estimation Ideal DL channel estimation is optionally taken into the baseline of evaluation methodology for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Up to companies to report whether/how ideal channel is used in the dataset construction and performance evaluation/inference. Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.. Rank per UE; Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied. Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions. The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations. *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to the KPI part to TR 38.843.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** KPIs and Evaluation metrics: - Capability/complexity: Floating point operations (FLOPs), AI/ML model size, number of AI/ML parameters AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters reported by companies who may select either or both - Reported separately for the CSI generation part and the CSI reconstruction part (for CSI compression sub-use case) - When reporting the computational complexity including the pre-processing and post-processing, the complexity metric of FLOPs may be reported separately for the AI/ML model and the pre/post processing. While reporting the FLOPs of pre-processing and post-processing the following boundaries are considered: - Estimated raw channel matrix per each frequency unit as an input for pre-processing of the CSI generation part. - Precoding vectors per each frequency unit as an output of post-processing of the CSI reconstruction part. - AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both. *** Unchanged text is omitted *** - CSI compression: Intermediate KPI: monitoring mechanism considered as: *** Unchanged text is omitted *** - Step 2: For each of the K test samples, a bias factor of monitored intermediate KPI (KPIDiff) is calculated as a function of KPIDiff = f ( KPIActual , KPIGenie ), where KPIActual is the actual intermediate KPI, and KPIGenie is the genie-aided intermediate KPI. *** Unchanged text is omitted *** - KPIDiff = f ( KPIActual , KPIGenie ) can take the following forms: - Option 1 (baseline for calibration): Gap between KPIActual and KPIGenie, i.e. KPIDiff = (KPIActual - KPIGenie); Monitoring accuracy is the percentage of samples for which ¦ KPIDiff¦ < KPIth 1, where KPIth 1 is a threshold of the intermediate KPI gap which can take the following values: 0.02, 0.05 and 0.1. - Option 2 (optional and up to companies to report): Binary state where KPIActual and KPIGenie, have different relationships to their threshold(s), i.e., KPIDiff = (KPIActual > KPIth 2, KPIGenie > < KPIth 3) OR (KPIActual < KPIth 2, KPIGenie < > KPIth 3), where KPIth 2 is considered to be the same as KPIth 3. Monitoring accuracy is the percentage of samples for which KPIDiff = 0. *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to the model generalization part to TR 38.843.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** Model generalization: In order to study the verification of generalization, the following aspects are encouraged to be reported: - The configuration(s)/scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios - The configuration(s)/scenario(s) for testing/inference - The detailed list of configuration(s) and/or scenario(s) The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations: *** Unchanged text is omitted *** To verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects: - Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband) - Various sizes of CSI feedback payloads, FFS candidate payload number - Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) *** Unchanged text is omitted *** For CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different generated CSI feedback dimensions), the generalization cases of are elaborated as follows - Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed CSI feedback dimension), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1. - Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2. - Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn. - Notes: For Case 1/2/3, companies to report whether the output of the CSI generation part is before quantization or after quantization. For Case 2/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g., truncation, additional adaptation layer in AI/ML model, etc. Model Fine-tuning: For the evaluation of the potential performance benefits of model fine-tuning of CSI feedback enhancement, which is optionally assessed, the following case is considered: - The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - In this case, the fine-tuning dataset setting (e.g., size of dataset) is to be reported along with the improvement of performance. Further details on evaluations including training collaboration types *** Unchanged text is omitted *** For SLS, spatial consistency Procedure A with 50m decorrelation distance from TR 38.901 is used (if not used, assumptions used need to be reported). UE velocity vector is assumed as fixed over time in Procedure A modelling. Model Fine-tuning: For the evaluation of the potential performance benefits of model fine-tuning of CSI feedback enhancement, which is optionally assessed, the following case is considered: - The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - In this case, the fine-tuning dataset setting (e.g., size of dataset) is to be reported along with the improvement of performance. *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to the training collaboration types part to TR 38.843.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** Further details on evaluations including training collaboration types For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side. For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example: *** Unchanged text is omitted *** For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training): - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly - Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part - Companies to report Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. Also report the Quantization behaviour, e.g., whether the shared output of the Network side CSI generation part is before or after quantization. - Step3: UE side trains the UE side CSI generation part based on the received set of information - Other Type 3 NW-first training approaches are not precluded For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training): - Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly - Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part - Companies to report Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable. Also, report the Quantization behaviour, e.g., whether the shared input of the UE side CSI reconstruction part is before or after quantization. - Step3: NW side trains the NW side CSI reconstruction part based on the received set of information - Other Type 3 UE-first training approaches are not precluded *** Unchanged text is omitted *** For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following evaluation cases for sequential training are considered for multi-vendors: *** Unchanged text is omitted *** - Case 2: For UE-first training, Type 3 training between one NW part model and M>1 separate UE part models - Note: Case 2 can be also applied to the M>1 UE part models to N>1 NW part models - Companies to report the AI/ML structures for the M>1 UE part models and the NW part model - Companies to report the dataset used at UE part models, e.g., same or different dataset(s) among M UE part models - Companies to report Dataset construction, e.g., the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only, or other information if applicable. Also, report the Quantization behaviour, e.g., whether the shared input of the UE side CSI reconstruction part is before or after quantization. - Case 3: For NW-first training, Type 3 training between one UE part model and N>1 separate NW part models - Note: Case 3 can be also applied to the N>1 NW part models to M>1 UE part models - Companies to report the AI/ML structures for the UE part model and the N>1 NW part models - Companies to report the dataset used at NW part models, e.g., same or different dataset(s) among N NW part models - Companies to report Dataset construction, e.g., the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only, or other information if applicable. Also report the Quantization behaviour, e.g., whether the shared output of the Network side CSI generation part is before or after quantization. - Case 4: 1-on-1 training with joint training: benchmark/upper bound for performance comparison. *** Unchanged text is omitted *** CSI compression sub use case specific aspects: For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side. For the evaluation of the AI/ML based CSI compression sub use case, companies are encouraged to report details of their models, including: *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to the CSI compression sub use case specific aspects to TR 38.843.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** For the evaluation of the AI/ML based CSI compression sub use case, companies are encouraged to report details of their models, including: - The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc. - AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) types for evaluations: - Raw channel matrix (in frequency or delay domain), e.g., channel matrix with dimensions of Tx, Rx, and frequency unit - Precoding matrix (as a group of eigenvectors or an eTypeII-like reporting) - Data pre-processing/post-processing - Loss function - Specific quantization/dequantization method, e.g., vector quantization, scalar quantization, etc, considering the following aspects: For the evaluation of the AI/ML based CSI compression sub use cases, at least the following types of AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) are considered for evaluations: - Raw channel matrix, e.g., channel matrix with the dimensions of Tx, Rx, and frequency unit. Companies to report the raw channel is in frequency domain or delay domain. - Precoding matrix. Companies to report the precoding matrix is a group of eigenvector(s) or an eType II-like reporting (i.e., eigenvectors with angular-delay domain representation). For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies - Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training - Fixed/pre-configured quantization method/parameters is applied for the inference phase. Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc - Case 2: Quantization-aware training, where quantization/dequantization is involved in the training process - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase. Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase. Companies to report how to update the quantization method/parameters during the training - Quantization methods including uniform vs non-uniform quantization, scalar versus vector quantization, and associated parameters, e.g., quantization resolution, etc. - How to use the quantization methods are reported by companies For evaluating the performance impact of ground-truth quantization in the CSI compression, - Considering performance impact of ground truth quantization in the CSI compression - Studying study high resolution quantization methods for ground truth CSI, including at least the following options: - High resolution scalar quantization - High resolution codebook quantization, e.g., Rel-16 TypeII-like method with new parameters, in which case companies are to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc - Float32 adopted as the baseline/upper-bound for performance comparisons - Consider the legacy values of PC6&PC8 for performance comparison *** Unchanged text is omitted *** 6.2.2 Performance results *** Unchanged text is omitted *** - Ground-truth CSI quantization method: Float32, i.e., without quantization (baseline/upper-bound for performance comparison) - Other high resolution CSI quantization methods can be additionally submitted for comparison, e.g., R16 eType II-like method with new parameters (consider the legacy values of PC6&PC8 as the baseline/lower-bound of performance comparison), scalar quantization, etc. *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to “Summary of Performance Results for CSI feedback enhancement” in TR 38.843, Section 6.2.2.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.2 Performance results *** Unchanged text is omitted *** 6.2.2.8 Summary of Performance Results for CSI feedback enhancement The following aspects have been studied for the evaluation on AI/ML based CSI compression in Rel-18: - From the perspective of basic performance gain over non-AI/ML benchmark (assuming 1 on 1 joint training without considering generalization), - It has been studied with corresponding observations on: - the metrics of SGCS, mean UPT, 5% UPT, CSI feedback overhead reduction - the benchmark of R16 Type II codebook - It has been studied but is lack of observations on: - the metric of NMSE - the benchmarks of Type I codebook and R17 Type II codebook - It has been studied with corresponding observations on complexity but without comparison with non-AI/ML. - From the perspective of AI/ML solutions (assuming 1 on 1 joint training without considering generalization), - It has been studied with corresponding observations on: model input/output type, monitoring for intermediate KPI (including NW side monitoring and UE side monitoring), quantization methods (including quantization awareness for training, and quantization format), and high resolution ground-truth CSI for training, with the metric of SGCS. - It has been studied but is lack of observations on: the options of CQI/RI calculation, and the options of rank>1 solution - From the perspective of generalization over various scenarios (assuming 1 on 1 joint training), - It has been studied with corresponding observations on (with the metric of SGCS): - the scenarios including various deployment scenarios, various outdoor/indoor UE distributions, various carrier frequencies, and various TxRU mappings - the approach of dataset mixing (generalization Case 3) - It has been studied but is lack of observations on: - other aspects of scenarios - the approach of fine-tuning - From the perspective of scalability over various configurations (assuming 1 on 1 joint training), - It has been studied with corresponding observations on (with the metric of SGCS): - the configurations including various bandwidths/frequency granularities, various CSI feedback payloads, and various antenna port numbers - the approach of dataset mixing (generalization Case 3), and the approach of fine-tuning for CSI feedback payloads - the scalability solutions - It has been studied but is lack of observations on: - other aspects of configurations - the approach of fine-tuning for configurations other than CSI feedback payloads - From the perspective of multi-vendor joint training (without considering generalization), - It has been studied with corresponding observations on (with the metric of SGCS): - joint training between 1 NW part model and M>1 UE part models, and joint training between 1 UE part model and N>1 NW part models - It has been studied but is lack of observations on: - joint training between N>1 NW part models and M>1 UE part models - performance comparison between simultaneous training and sequential training - From the perspective of separate training (without considering generalization), - It has been studied with corresponding observations on (with the metric of SGCS): - NW first training, including 1 NW part model to 1 UE part model with same backbone and with different backbones, and 1 UE part model to N>1 NW part models - UE first training, including 1 NW part model to 1 UE part model with same backbone and with different backbones, and 1 NW part model to M>1 UE part models - Impact of shared dataset under 1 NW part model to 1 UE part model for NW first training and UE first training - It has been studied but is lack of observations on: - the metric of air-interface overhead of information (e.g., dataset) sharing The following aspects have been studied for the evaluation on AI/ML based CSI prediction: - From the perspective of basic performance gain over non-AI/ML benchmark (without considering generalization), - It has been studied with corresponding observations on: - the metrics of SGCS, mean UPT, 5% UPT; - the benchmarks of nearest historical CSI and auto-regression/Kalman filter based CSI prediction. - Note: the benchmark of level x based CSI prediction is represented by generalization cases. - It has been studied but is lack of observations on: - the impact of modeling spatial consistency - the metrics of NMSE - It has been studied with corresponding observations on complexity but without comparison with non-AI/ML - From the perspective of AI/ML solutions (without considering generalization), - It has been studied with corresponding observations on (with the metric of SGCS and the benchmark of nearest historical CSI): impact of input type, impact of UE speed, impact of prediction window, impact of observation window - From the perspective of generalization over various scenarios, - It has been studied with corresponding observations on (with the metric of SGCS): - the scenario including various UE speeds - the approach of dataset mixing (generalization Case 3) - It has been studied but is lack of observations on: - various deployment scenarios, various carrier frequencies, and other aspects of scenarios. - the approach of fine-tuning - From the perspective of scalability over various configurations, it has been studied but is lack of observations. *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to the CSI prediction sub use case specific aspects to TR 38.843.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** CSI prediction sub use case specific aspects: For the evaluation of the AI/ML based CSI prediction sub use case, companies are encouraged to report details of their models, including: - The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc. - The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc. - Including assumptions on the observation window, i.e., number/time distance of historic CSI/channel measurements - The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc. - Including assumptions on the prediction window, i.e., number/time distance of predicted CSI/channel - Data pre-processing/post-processing - Loss function For the input CSI type, both of the following types are considered for evaluations: - Raw channel matrixes. - Eigenvector(s). For SLS, spatial consistency Procedure A with 50m decorrelation distance from TR 38.901 is used (if not used, assumptions used need to be reported). UE velocity vector is assumed as fixed over time in Procedure A modelling. *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to the results calibration part to TR 38.843.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.1 Evaluation assumptions, methodology and KPIs *** Unchanged text is omitted *** CSI compression sub use case specific aspects: *** Unchanged text is omitted *** For CSI compression sub use case with rank ≥ 1, AI/ML model setting to adapt to ranks/layers to be reported amongst the following options: *** Unchanged text is omitted *** - For CSI compression sub use case with rank >1, the storage of memory storage/number of parameters is reported as the summation of memory storage/number of parameters over all models potentially used for any layer/rank, e.g., - Option 1-1 (rank specific)/Option 3-2 (layer common and rank specific): Sum of memory storage/number of parameters over all rank specific models. - Option 1-2 (rank common): A single memory storage/number of parameters for the rank common model. - Option 2-1 (layer specific and rank common): Sum of memory storage/number of parameters over all layer specific models. - Option 2-2 (layer specific and rank specific): Sum of memory storage/number of parameters for the specific models over all ranks and all layers in per rank. - Option 3-1 (layer common and rank common): A single memory storage/number of parameters for the common model For the evaluation of CSI compression, the specific CQI determination method(s) for AI/ML can be reported by introducing an additional field in the template, e.g., - Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation. - Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment. - Option 1c: CQI is calculated based on traditional codebook. - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment. - Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW. - Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW. - Option 2b: CQI is calculated using two stage approach, UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder. *** Unchanged text is omitted *** 6.2.2 Performance results *** Unchanged text is omitted *** For the evaluation of CSI compression, the specific CQI determination method(s) for AI/ML can be reported by introducing an additional field in the template, e.g., - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment. - Option 2a-1: The CSI reconstruction part for CQI calculation at the UE same as the actual CSI reconstruction part at the NW. - Option 2a-2: The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the NW. - Option 2b: CQI is calculated using two stage approach, UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Option 1a: CQI is calculated based on the target CSI from the realistic channel estimation. - Option 1b: CQI is calculated based on the target CSI from the realistic channel estimation and potential adjustment. - Option 1c: CQI is calculated based on traditional codebook. For the evaluation of CSI compression of 1-on-1 joint training without model generalization/scalability, the The following baselines are recommended to facilitate calibration of results: - Benchmark: R16 eType II CB; - Others can be additionally submitted, e.g., Type I CB. - Input/Output type: Eigenvectors of the current CSI - Other can be additionally submitted, e.g., eigenvectors with additional past CSI, eType II-like input, raw channel matrix, etc. *** Unchanged text is omitted ***

Agreement

- Adopt the following TP related with changes to the observation part to TR 38.843.

------------------ Text Proposal for 38.843 v1.0.0 ------------------ 6.2.2 Performance results *** Unchanged text is omitted *** For the evaluation of AI/ML based CSI compression compared to the benchmark in terms of mean UPT under FTP traffic, more gains are achieved by Max rank 2 compared with Max rank 1 in general: *** Unchanged text is omitted *** - For Max rank 4: *** Unchanged text is omitted *** - For RU≥70%, 3 sources observe the performance gain of -1%~17% - 3 sources observe the performance gain of 3%~17% at CSI overhead A (small overhead); - 2 sources observe the performance gain of 6.64%~17% at CSI overhead B (medium overhead); - 3 sources observe the performance gain of -1%~8.40% at CSI overhead C (large overhead); - Note: 1 source observes significant gain or significant loss under Max rank 4 due to specific CQI/RI selection method (e.g., Option 1a/2a) for AI/ML and/or CQI/RI determination method for eType II benchmark. The above results are based on the following assumptions besides the assumptions of the agreed EVM table: - Precoding matrix of the current CSI is used as the model input. - Training data samples are not quantized, i.e., Float32 is used/represented. - 1-on-1 joint training is assumed. - The performance metric is mean UPT for Max rank 1, Max rank 2, or Max rank 4. - Benchmark is Rel-16 Type II codebook. - Note: Results refer to Table 5.12 of R1-2308342 R1-2308340. *** Unchanged text is omitted *** For the evaluation of AI/ML based CSI compression compared to the benchmark in terms of 5% UPT under FTP, more gains are achieved by Max rank 2 compared with Max rank 1 in general: *** Unchanged text is omitted *** - For Max rank 4: *** Unchanged text is omitted *** - For RU≥70%, 3 sources observe the performance gain of 2%~31% - 3 sources observe the performance gain of 5.8%~31% at CSI overhead A (small overhead); - 2 sources observe the performance gain of 10.2%~30% at CSI overhead B (medium overhead); - 3 sources observe the performance gain of 2%~15% at CSI overhead C (large overhead); - Note: 1 source observes significant gain or significant loss under Max rank 4 due to specific CQI/RI selection method (e.g., Option 1a/2a) for AI/ML and/or CQI/RI determination method for eType II benchmark. *** Unchanged text is omitted *** For the evaluation of intermediate KPI based monitoring mechanism for CSI compression, for monitoring Case 1, in terms of monitoring accuracy with Option 1, - For ground truth CSI format of R16 eType II CB, monitoring accuracy is increased with the increase of the resolution for the ground-truth CSI (number of bits for each sample of ground-truth CSI) in general, with the impact of increased overhead, wherein - for ground truth CSI format of R16 eType II CB with PC#6, 4 sources observe KPIDiff KPIDiff as 13.2%~71.6%/ 28.5%~100%/ 68.4%~100% for KPIth_1KPIth_1=0.02/0.05/0.1, respectively. - Note: two sources observed averaging on the test samples improves the monitoring accuracy. - for ground truth CSI format of R16 eType II CB with PC#8, 5 sources observe KPIDiff KPIDiff as 21%~43.0%/ 48.1%~79.1%/ 79.8%~97.1% for KPIth_1KPIth_1=0.02/0.05/0.1, respectively. - for ground truth CSI format of R16 eType II CB with new parameter of 580-750bits CSI payload size, 2 sources observe KPIDiff KPIDiff as 35.4%~63%/ 77.9%~93.0%/ 99.5%~99.9% for KPIth_1KPIth_1=0.02/0.05/0.1, respectively, which have 12.7%~20%/ 13.9%~29.8%/ 8%~31.1% gain over PC#8. - for ground truth CSI format of R16 eType II CB with new parameter of around 1000bits CSI payload size, 4 sources observe KPIDiff KPIDiff as 34.9%~89%/ 82.9%~100%/ 99.9%~100% for KPIth_1KPIth_1=0.02/0.05/0.1, respectively, which have 12.2%~68%/ 18%~43.62%/ 2.9%~31% gain over PC#8 from 3 sources and 4.67%~10.6%/ 0%~5.88%/ 0%~0.49% gain over PC#6 from 1 source. - for ground truth CSI format of R16 eType II CB with new parameter of around 1600bits CSI payload size, 2 sources observe KPIDiff KPIDiff as 89.1%~97%/ 99.9%~100%/ 100% for KPIth_1KPIth_1=0.02/0.05/0.1, respectively, which have 76%/33%/3% gain over PC#8 from 1 source. - for ground truth CSI format of 4 bits scalar quantization, 2 sources observe KPIDiff KPIDiff as 9.4%~47%/ 96.3%~100%/ 100% for KPIth_1KPIth_1=0.02/0.05/0.1, respectively. *** Unchanged text is omitted *** For the comparison of quantization methods for CSI compression, quantization non-aware training (Case 1) is in general inferior to the quantization aware training (Case 2-1/2-2), and may lead to lower performance than the benchmark: - For scalar quantization, compared with benchmark, - -2.4%~-43.2% degradations are observed for quantization non-aware training (Case 1) from 6 sources. - 3.9%~8.64% gains are observed for quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) from 5 sources, which are 17.3%~83.2% gains over quantization non-aware training (Case 1) from 5 sources and 7.56%~11.55% gains over quantization non-aware training (Case 1) from 1 source. - Note: 0.72% gains are observed for Case 2-1 from 1 source due to SQ parameter chosen without matching latent distribution, which achieves 13.9% gains over Case 1. - 8.91% 7.55% gains are observed for quantization aware training with jointly updated quantization method/parameters (Case 2-2) from 1 source, which are 23.1% gains over quantization non-aware training (Case 1) from 1 source. - For vector quantization, compared with benchmark, - -2%~-10% degradations are observed for quantization non-aware training (Case 1) from 1 source. - 5.64%~7.55% 8.91% gains are observed for quantization aware training with fixed/pre-configured quantization method/parameters (Case 2-1) from 3 sources, which are 3%~21.6% gains over quantization non-aware training (Case 1) from 3 sources. - 4.6%~13.01% gains are observed for quantization aware training with jointly updated quantization method/parameters (Case 2-2) from 7 sources, which are 10.7%~30% gains over quantization non-aware training (Case 1) from 4 sources and 3.66%~9.8% gains over quantization non-aware training (Case 1) from 2 sources. - In general, Case 2-2 outperforms Case 2-1 with 0.46%~5.1% 3.8% gains, as observed by 6 sources. *** Unchanged text is omitted *** For the comparison of quantization methods for CSI compression, in general vector quantization (VQ) has comparable performance with scalar quantization (SQ): - For SQ and VQ under the same training case, it is - observed by 3 sources that VQ under Case 2-1 has -1%~-4.5% degradation over SQ under Case 2-1, - observed by 1 source that VQ under Case 2-1 has 1.1% gain over SQ under Case 2-1, and - observed by 3 sources that VQ under Case 2-2 has 0.7%~3.8% 5.1% gain over SQ under Case 2-2. - Note: VQ under Case 2-1 has 8% gains over SQ under Case 2-1 as observed from 1 source due to SQ parameter chosen without matching latent distribution. *** Unchanged text is omitted *** For the evaluation of NW first separate training with dataset sharing manner for CSI compression for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model, - For the NW first separate training case where the same backbone is adopted for both the NW part model and the UE part model, minor degradation is observed for both the cases where the shared output of the Network side CSI generation part is before or after quantization: - For the case where the shared output of the Network side CSI generation part is after quantization, 9 sources observe -0%~-0.5% degradation, 10 sources observe -0.5%~-1% degradation, and 2 sources observe -1%~-1.3% degradation. - For the case where the shared output of the Network side CSI generation part is before quantization, 6 sources observe -0%~-0.8% degradation, and 1 source observes -1%~-1.5% degradation. - Note: the dataset sharing behaviour from above sources follows the example of the agreement “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”. *** Unchanged text is omitted *** For the evaluation of NW/UE first separate training with dataset sharing manner for CSI compression for the pairing of 1 NW to 1 UE (Case 1), as compared to the case where the same set of dataset is applied for training the NW part model and training the UE part model, if the dataset#2 applied for training the UE/NW part model is a subset of the dataset#1 applied for training the NW/UE part model, - If the dataset#2 is appropriately selected, minor additional performance degradation can be achieved, as -0%~-0.59% gap is observed from 3 sources. - If the dataset#2 has a significantly reduced size compared to dataset#1, moderate/significant additional performance degradation may occur, as -0.6%~-4.83% gap is observed from 4 sources. - Note: the dataset sharing behavior from above sources follows the example of the agreement where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”. *** Unchanged text is omitted *** For the evaluation of UE first separate training with dataset sharing manner for CSI compression for the pairing of 1 NW to 1 UE (Case 1), as compared to 1-on-1 joint training between the NW part model and the UE part model, - For the UE first separate training case where the same backbone is adopted for both the UE part model and the NW part model, minor degradation is observed in general for both the cases where the shared input of the UE side CSI reconstruction part is before or after quantization: - For the case where the shared input of the UE side CSI reconstruction part is after quantization, 9 sources observe -0%~-0.42% degradation, 2 sources observe -0.7%~-0.9% degradation, and 3 sources observe -1.05%~-1.8% degradation. - For the case where the shared input of the UE side CSI reconstruction part is before quantization, 3 sources observe -0%~-0.8% degradation, and 2 sources observe -1.3% -1.8%~-2.9% degradation. - Note: the dataset sharing behaviour from above sources follows the example of the agreement where “the set of information includes the input and label of the UE side CSI reconstruction part, or includes the input of the UE side CSI reconstruction part only”. *** Unchanged text is omitted *** For the scalability verification of AI/ML based CSI compression over various CSI payload sizes, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain CSI payload size#B and applied for inference with a same CSI payload size#B, - For generalization Case 2, significant performance degradations are observed in general, as -5.3%~-14.7% degradations are observed by 2 sources. - Generalized performance of the AI/ML model can be achieved (-0%~-5.9%loss) under generalization Case 3 for the inference on CSI payload size#B, if the training dataset is constructed with data samples subject to multiple CSI payload sizes including CSI payload size#B, and an appropriate scalability solution is performed to scale the dimension of the AI/ML model, shown by 13 sources (10 sources showing -0%~-2.2% loss, 7 sources showing -2.3%~-5.9% loss, 5 sources showing positive gain). The scalability solution is adopted as follows: - Pre/post-processing of truncation/padding, adopted by 6 sources, showing -0% ~-5.9% loss or positive gain. - Various quantization granularities, adopted by 1 source, showing -0.7% loss or positive gain. - Adaptation layer in the AL/ML model, adopted by 6 sources, showing -0%~-4.78% loss or positive gain. - Finetuning models on CSI payload size#B, showing loss [0%~-2.2%] by 2 sources - Note: Significant degradations of up to -14.22% are still observed by 2 sources for generalization Case 3. - Generalized performance of the AI/ML model can also be achieved by finetuning models on CSI payload size#B, showing loss [0%~-2.2%] by 2 sources The above results are based on the following assumptions: *** Unchanged text is omitted ***

Agreement

Capture the following high level observations for CSI prediction to section 6.2.2.8 of TR 38.843:

- From the perspective of model input/output type, it is more beneficial in performance by considering raw channel matrix as the model input than precoding matrix

- The gain of AI/ML based CSI prediction over the benchmark of the nearest historical CSI is impacted by the length of the observation window length, prediction window length, and UE speed

- From the perspective of generalization over various several UE speeds that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different UE speed#A, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others

  - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, generalized performance of the AI/ML model can be achieved in general