3GPP TSG-RAN WG1 Meeting #115	R1-2312781

Chicago, IL, USA, November 13 – 17, 2023

Agenda Item:	8

Source:	Ericsson

Title:	Editor’s summary on draft CR 38.211 for NR_MIMO_evo_DL_UL-Core

Document for:	Discussion, Decision

# 1	Introduction

This document is intended to facilitate the review process of the draft CR 38.211 for NR_MIMO_evo_DL_UL-Core.

# 2	Discussion – first round

Please provide your comments on the latest version of the draft CR on 38.211 available in this folder.

| Company | Comment |
| --- | --- |
| CATT | Thanks Stefan for your great efforts drafting the CR. Below please find one comment from us: Regarding the modifications to Table 6.3.1.5-9 to Table 6.3.1.5-24 and Table 6.3.1.5-47 (this one is somehow missed), we prefer to keep the wording “intermediate precoder matrix W`” in the tables, i.e., -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-9: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n4n1 and single-layer transmission using eight antenna ports. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-10: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n4n1 and two-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-11: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n4n1 and three-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-12: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n4n1 and four-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-13: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n4n1 and five-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-14: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n4n1 and six-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-15: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n4n1 and seven-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-16: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n4n1 and eight-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-17: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n2n2 and single-layer transmission using eight antenna ports. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-18: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n2n2 and two-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-19: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n2n2 and three-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-20: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n2n2 and four-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-21: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n2n2 and five-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-22: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n2n2 and six-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-23: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n2n2 and seven-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-24: Intermediate precoding matrix W'Precoding matrix W for codebook1=ng1n2n2 and eight-layer transmission using eight antenna ports with transform precoding disabled. TPMI index; Intermediate precoder matrix W'W (ordered from left to right in increasing order of TPMI index). -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-47: Intermediate precoding matrix W'Precoding matrix W for codebook4 and transmission using eight antenna ports. Up to 8 layers are supported with transform precoding disabled and up to one layer with transform precoding enabled. TPMI index; Intermediate precoder matrix W'W. -------------------------------------------Unchanged parts are omitted------------------------------------------- Which aligns the content in Table 6.3.1.5-29 to Table 6.3.1.5-36 and Table 6.3.1.5-39 to Table 6.3.1.5-46, e.g., Table 6.3.1.5-29: Intermediate precoding matrix W' for codebook2 and single-layer transmission using eight antenna ports. TPMI index i; Intermediate precoder matrix W'. 0 – 15; 12W1, i04×1. 16 – 31; 1204×1W1, (i-16). 32; 12211111111. To keep the consistency of the spec. |
| ZTE | Thank you Stefan for the great efforts. We have the following comments: Comment#1 The higher-layer parameter names “cyclicShiftHoppingSubset ” and “combOffsetHoppingSubset ” are not aligned with TS 38.331. SRS-Config (TS 38.331) combOffsetHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023) OPTIONAL, -- Need R hoppingSubset-r18 CHOICE { transmissionComb-n4 BIT STRING (SIZE (4)), transmissionComb-n8 BIT STRING (SIZE (8)), } OPTIONAL, -- Need R hoppingWithRepetition-r18 ENUMERATED {symbol, Rrepetition} OPTIONAL -- Need R } OPTIONAL, -- Need R cyclicShiftHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023) OPTIONAL, -- Need R hoppingSubset-r18 CHOICE { transmissionComb-n2 BIT STRING (SIZE (8)), transmissionComb-n4 BIT STRING (SIZE (12)), transmissionComb-n8 BIT STRING (SIZE (6)), } OPTIONAL, -- Need R. Proposed change (clauses 6.4.1.4.2 and 6.4.1.4.3) <unchanged part omitted> - If the higher-layer parameter cyclicShiftHopping is configured: fcshnf ,ns,fμ,l'= scshSRSm=07c8nf mod 128Nslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'+m2mmod ncshSRS where scshSRSn and ncshSRS is the (n+1)th entry and the cardinality of the set Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1} respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset hoppingSubset in cyclicShiftHopping if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}. The higher-layer parameter cyclicShiftHoppingSubset hoppingSubset in cyclicShiftHopping includes a bitmap of nSRScs,max bits with 1<ncshSRS<nSRScs,max bits being set to 1, where the (n+1)th bit being set to 1 corresponds to scshSRSn. <unchanged part omitted> 6.4.1.4.3 Mapping to physical resources <unchanged part omitted> - if the higher-layer parameter combOffsetHopping is configured: fcohnf ,ns,fμ,l''= scohSRSm=07c8nf mod 128Nslotframe,μNsymbslot+ns,fμNsymbslot+l0+l''+m2mmod ncohSRS where scohSRSn and ncohSRS is the n+1th entry and the cardinality of the set Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1} respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset hoppingSubset in combOffsetHopping if configured, otherwise Scoh={0, 1,…,KTC-1}. The higher-layer parameter combOffsetHoppingSubset hoppingSubset in combOffsetHopping includes a bitmap of KTC bits with 1<ncohSRS<KTC bits being set to 1, where the (n+1)th bit being set to 1 corresponds to scohSRSn. <unchanged part omitted>. Comment#2 According to TS 38.331, the higher-layer parameter hoppingWithRepetition can be set to symbol or Repetition. Therefore, corresponding change is needed in TS 38.211 to reach the alignment. SRS-Config (TS 38.331) combOffsetHopping-r18 SEQUENCE { hoppingId-r18 INTEGER (0..1023) OPTIONAL, -- Need R hoppingSubset-r18 CHOICE { transmissionComb-n4 BIT STRING (SIZE (4)), transmissionComb-n8 BIT STRING (SIZE (8)), } OPTIONAL, -- Need R hoppingWithRepetition-r18 ENUMERATED {symbol, Rrepetition} OPTIONAL -- Need R }. Proposed change (clause 6.4.1.4.3) 6.4.1.4.3 Mapping to physical resources <unchanged part omitted> If the higher-layer parameter hoppingWithRepetition is configured set to Repetition, l''=l'RR, otherwise l''=l'. <unchanged part omitted>. |

Editor’s response:

- The table header have in the “new” tables have been updated to “Intermediate precoding matrix W’” for consistency reasons (although the”old” tables only have “W” in the table header, which is slightly inconsistent)

- The higher-layer parameter names have been updates as suggested above.

# 2	Discussion – second round

Please provide your comments on the latest version of the draft CR on 38.211 available in this folder.

| Company | Comment |
| --- | --- |
| CATT | Thanks Stefan for your updates. Please find one comment from use below: The changes of header and the content in Table 6.3.1.5-47, which is shown below, are not captured yet. -------------------------------------------Unchanged parts are omitted------------------------------------------- Table 6.3.1.5-47: Intermediate precoding matrix W'Precoding matrix W for codebook4 and transmission using eight antenna ports. Up to 8 layers are supported with transform precoding disabled and up to one layer with transform precoding enabled. TPMI index; Intermediate precoder matrix W'W. -------------------------------------------Unchanged parts are omitted------------------------------------------- |

Editor’s response:

- For 8Tx (which is what Table 6.3.1.5-47 addresses), W’ and W are identical, but to make the description consistent the caption and table heading is changed as suggested, as well as replacing W with W’ in the first row of the table.

# 3	Discussion – third round

Please provide your comments on the latest version of the draft CR on 38.211 available in this folder.

| Company | Comment |
| --- | --- |

Editor’s response: No further comments have been received. The final CR is available in R1-2312749