3GPP TSG RAN WG1 #115			R1-231nnnn

Chicago, USA, November 13th – November 17th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.12

Agenda Item:		8.16.12

Document for: 	Endorsement

### 8.16.12	UE features for MC enhancements

Agreement

For DCI format 1_3 with same SCS between scheduling cell and cells in the set:

- [At least for the case if UE does not support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 for a set of cells;]

  - If the scheduling cell is in the set but is NOT the reference cell;

    - Same number of DCIs as the case when the scheduling cell is in the set and is the reference cell

  - FFS whether to include this case in FG 49-1 or advanced FG

Agreement

- Add following component in FG 49-1

  - Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases

    - 1) Search space set configuration for DCI format 1_3 for the set of cells is provided only on the scheduling cell, or;

    - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells

    - UE supporting FG 49-1 can additionally report whether the UE support following case

      - 3) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells

- Delete FGs 49-1a

- Update the sentence in FG 49-1 as follows:

  - FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells

Agreement

For DCI format 1_3 with same SCS between scheduling cell and cells in the set:

- [For the case if UE support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 for a set of cells;]

  - One DCI 1_3 for the set of cells and one unicast DL SC-DCI for each of the cells that are not scheduled by DCI 1_3 per slot of scheduling cell

Agreement

For 49-1:

- Component 10 is confirmed as:

  - The number of unicast DL DCIs to process per slot of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3

    - One DCI format 1_3 for the set of cells and,

    - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3

- Add following note in FG 49-1:

  - Support of CCS with DL DCI formats 1_1/1_2 is according to FG 6-10

Agreement

For 49-2:

- Component 9 is confirmed as:

  - The number of unicast UL DCIs to process per slot of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3

    - For FDD scheduling cell

      - [Up to] one DCI format 0_3 for the set of cells and,

      - [Up to] one unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells

      - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell

    - For TDD scheduling cell

      - [Up to] two DCI format 0_3 for the set of cells and,

      - [Up to] two unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells

      - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell

- Add following note in FG 49-2:

  - Support of CCS with UL DCI formats 0_1/0_2 is according to FG 6-10

Agreement

- Add following component in FG 49-2

  - Monitoring SS set(s) for DCI format 0_3 for a set of cells for the following cases

    - 1) Search space set configuration for DCI format 0_3 for the set of cells is provided only on the scheduling cell, or;

    - 2) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells

    - UE supporting FG 49-2 can additionally report whether the UE support following case

      - 3) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells

- Delete FGs 49-2a

- Update the sentence in FG 49-2 as follows:

  - FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells

Agreement

- Add following component in FG 49-1b

  - Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases

    - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells

Agreement

- Add following component in FG 49-2b

  - Monitoring SS set(s) for DCI format 0_3 for a set of cells for the following cases

    - 2) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells

Agreement

- Add following component in FG 49-1b:

  - The number of unicast DL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3

    - One DCI format 1_3 for the set of cells and,

    - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3

    - For low-to-high SCS, N = 1.

    - For high-to-low SCS, N is based on pair of (scheduling CC SCS, scheduled CC SCS): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15)

- Add following note in FG 49-1b:

  - Support of CCS with DL DCI formats 1_1/1_2 is according to FG 18-5

Agreement

- Add following component in FG 49-2b:

  - The number of unicast UL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3

    - For FDD scheduling cell

      - [Up to] one DCI format 0_3 for the set of cells and,

      - [Up to] one unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells

      - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell

    - For TDD scheduling cell

      - [Up to] two DCI format 0_3 for the set of cells and,

      - [Up to] two unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells

      - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell

    - For low-to-high SCS, N = 1.

    - For high-to-low SCS, N is based on pair of (scheduling CC SCS, scheduled CC SCS): N=2 for (30,15), (60,30), (120,60) and N=4 for (60,15), (120,30), N = 8 for (120,15)

- Add following note in FG 49-2b:

  - Support of CCS with UL DCI formats 0_1/0_2 is according to FG 18-5b

Agreement

- Delete following FFS from FG 49-1/1b/2/2b

  - FFS: whether to introduce new FG for Configuration/monitoring of DCI format 0_3 or 1_3 for a set of cells and legacy DCI format(s) for cell(s) in the set, or to support it by default

- Delete FG 49-3

Agreement

- Following is removed from FG 49-1/49-2

  - [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3]

- Following is removed from FG 49-1b/49-2b

  - [Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b]

Agreement

- Add component in FG49-1:

  - When multiple component 3 values are reported and if scheduling cell is not included in the set of cells, support multi-cell PDSCH scheduling by DCI format 1_3 from one carrier type, indicated in component 3, to another carrier type, indicated in component 3, for the following scheduling cases:

    - FR1 licensed TDD to FR1 unlicensed TDD

    - FR2-1 to FR2-2

    - UE can additionally report the support of {FR1 licensed FDD from/to FR1 licensed TDD}

Agreement

- Add component in FG49-2:

  - When multiple component 3 values are reported and if scheduling cell is not included in the set of cells, support multi-cell PUSCH scheduling by DCI format 0_3 from one carrier type, indicated in component 3, to another carrier type, indicated in component 3, for the following scheduling cases:

    - FR1 licensed TDD to FR1 unlicensed TDD

    - FR2-1 to FR2-2

    - UE can additionally report the support of {FR1 licensed FDD from/to FR1 licensed TDD}

Agreement

- Introduce following FGs

| 49. NR_MC_enh | 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 without scheduling a PDSCH using a reserved FDRA value | [10-16], At least one of {49-1, 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 3 HARQ codebook triggered by DCI format 1_3 | Per BC | N/A | N/A | N/A | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_3 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_3 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 11-4) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot | [25-6], At least one of {49-1, 49-1b} | Yes |  | UE does not support HARQ feedback based on enhanced Type 3 HARQ codebook triggered by DCI format 1_3 | Per BC | N/A | N/A | N/A | For component 2, the UE indicates its capability in the number of enhanced type 3 HARQ-ACK codebooks: {1, 2, 4, 8} For component 3, the dynamic indication is only supported if the UE for component 2 supports more than one enhanced type 3 HARQ-ACK codebook to be configured Candidate values for component 5 is: {1, 2, 3, 4, 5, 6, 7}. | Optional with capability signaling |

R1-2310886	Discussion on UE features for MC enhancements	Huawei, HiSilicon

R1-2310900	On UE features for MC enhancements	Nokia, Nokia Shanghai Bell

R1-2311028	Discussion on UE feature for MC enhancements	ZTE

R1-2311125	Discussion on Rel-18 Multi-carrier enhancements UE features	vivo

R1-2311191	Discussion on UE features for multi-carrier enhancement	Spreadtrum Communications

R1-2311279	Discussion on UE features for multi-carrier enhancement	OPPO

R1-2311393	Discussion on UE features for MC enhancements	xiaomi

R1-2311651	Discussion on UE features for Multi-carrier enhancements	NTT DOCOMO, INC.

R1-2311714	Views on UE features for Rel-18 MC enhancements	Apple

R1-2311877	UE features for multi-carrier enhancements	Samsung

R1-2311903	Discussion on UE features for MC enhancements	LG Electronics

R1-2311991	On UE feature discussion for Rel-18 multi-carrier enhancement	MediaTek Inc.

R1-2312070	UE features for MC enhancements	Qualcomm Incorporated

R1-2312104	UE features for Rel-18 multi-carrier enhancements	Ericsson