3GPP TSG RAN WG1 #115			R1-231nnnn

Chicago, USA, November 13th – November 17th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.2

Agenda Item:		8.16.2

Document for: 	Endorsement

### 8.16.2	UE features for NR sidelink evolution

Agreement

- FG 47-m8 is updated and FG 47-m9 is added as follows

| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1. UE supports transmitting S-SSB on additional S-SSB occasion(s) FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | [Optional without capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m9 | Receiving S-SSB on additional S-SSB occasion(s) | 1. UE supports receiving S-SSB on additional S-SSB occasion(s) |  | No | No | UE does not support receiving S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |

Agreement

- Introduce following FGs

| 47. NR_SL_enh2 | 47-k5 | Resource allocation for multi-consecutive slots transmission | UE supports resource (re-)selection for PSCCH/PSSCH transmission on multiple consecutive slots | TBD | [Yes] | [No] | UE does not support resource (re-)selection for multi-consecutive slots transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m10 | Contiguous RB-based PSCCH/PSSCH transmission | 1. UE supports contiguous RB-based PSCCH/PSSCH transmission 2. UE supports resource (re-)selection for contiguous RB-based PSCCH/PSSCH transmission FFS whether/how to define the capabilities for reception and/or PSFCH | TBD | [Yes] | [No] | UE does not support contiguous RB-based PSCCH/PSSCH transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11 | PSFCH transmissions in multiple contiguous RB sets | UE supports PSFCH transmissions in multiple contiguous RB sets | TBD | [No] | [No] | UE does not support PSFCH transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11a | PSFCH transmissions in multiple non-contiguous RB sets | UE supports PSFCH transmissions in multiple non-contiguous RB sets | TBD | [No] | [No] | UE does not support PSFCH transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12 | S-SSB transmissions in multiple contiguous RB sets | UE supports S-SSB transmissions in multiple contiguous RB sets | TBD | [No] | [No] | UE does not support S-SSB transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12a | S-SSB transmissions in multiple non-contiguous RB sets | UE supports S-SSB transmissions in multiple non-contiguous RB sets | TBD | [No] | [No] | UE does not support S-SSB transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k8 | CW autonomous update for SL transmission without HARQ feedback | UE support autonomous update of the to the next higher allowed value when the same value is consecutively used for X times for generation of for PSCCH/PSSCH transmission without HARQ feedback. FFS whether to merge with 47-k1 | TBD | No | No | UE does not update for PSCCH/PSSCH transmission without HARQ feedback. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k9 | Sidelink mode 1 resource allocation in shared spectrum | 1. UE can monitor DCI format 3_0 on a licensed band for NR sidelink dynamic scheduling and configured grant type 2 for transmitting interlaced RB-based PSCCH/PSSCH on a shared band] 2. UE supports reporting NACK to gNB when transmitting PSCCH/PSSCH on scheduled resource(s) is failed due to LBT failure | TBD | Yes | No | UE does not perform interlaced RB-based PSCCH/PSSCH based on mode 1 resource allocation. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m13 | Transmissions/receptions of multiple dedicated PRBs in interlace-based PSFCH | 1. UE can transmit PSFCH(s) on up to a total of K dedicated PRBs in a slot. 2. UE can receive PSFCH(s) on up to a total of L dedicated PRBs in a slot | TBD | No | No | UE does not support multiple transmissions/receptions of common interlace-based PSFCH. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Candidate values for K are FFS Candidate values for L are FFS | Optional with capability signalling |

Agreement

- FG 47-s1 is updated as follows

| 47. NR_SL_enh2 | 47-s1 | Transmission/Reception using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR sidelink TXs and RXs in a resource pool in 15kHz and 30kHz SCSs and uses the SCS that is (pre)configured for a SL BWP. | [15-3, 15-6, 15-11] | Yes | No | UE does not support transmission/reception using dynamic resource pool sharing | [Per band] | N/A | N/A |  | [UE supports EUTRA mode 4 fSL in the same band] Component 2 does not imply that two different SCSs can be (pre)configured simultaneously in a SL BWP | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 47-v1 is updated as follows

| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: FFS candidate values for X FFS whether to report following - Maximum number of simultaneous PSCCH/PSSCH TX/RX, equal to X and 1 per carrier - FFS Maximum number of simultaneous PSCCH decoding - FFS whether to report Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode 2) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | [15-3, 15-11] | Yes | No |  | [Per band] | N/A | N/A |  | Component 1: Candidate value of X = {2, 3, 4, 5, 6, 7, 8}, FFS on some BW restriction | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Add component 1-2 in FG 47-v1 as follows:

  - For the number of PSCCH decodes:

    - UE can receive Z* floor (NRB,i /10 RBs) PSCCH in a slot on carrier i of the X carriers.

- Add note in FG 47-v1: Component 1-2 candidate value set: Z={1, 2}

- Add note in FG 47-v1: NRB,i is the number of RBs defined per channel bandwidth of carrier i by RAN4 in 38.101-1 Table 5.3.2-1 for FR1

Agreement

- FG 47-v2 is updated as follows

| 47. NR_SL_enh2 | 47-v2 | Synchronization for SL CA with single or multiple carrier S-SSB transmission/reception | 1-1) UE supports transmitting/receiving S-SSB on [one selected or] all SL synchronization candidate synchronization carriers with the same sync reference from Set-B 1-2) UE supports receiving S-SSB from all candidate synchronization carriers with the same sync reference from Set-B FFS whether to report followings - Maximum number X of simultaneous S-SSB TX - Maximum number of simultaneous S-SSB RX 2) UE can adjust the transmission power of the S-SSB across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | 47-v1, [15-4] | Yes | No |  | [Per band] | N/A | N/A |  | Note: Option of UE selection of one selected SL synchronization carrier with the same sync reference from Set-B is not based on limited Tx capability Note: Component 1-2 does not require simultaneous reception of S-SSB on all candidate synchronization carriers with the same sync reference from Set-B | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 47-v3 is updated as follows

| 47. NR_SL_enh2 | 47-v3 | PSFCH for SL CA | 1) UE supports receiving up to X PSFCH resources in a slot over all aggregated multiple SL carriers - 1-1) UE is capable of receiving at least one PSFCH resource on each of the aggregated carriers in a slot 2) UE supports transmitting up to Y PSFCH resources in a slot over all aggregated multiple SL carriers according to PSFCH procedures - 2-1) UE is capable of transmitting at least one PSFCH resource on each of the aggregated carriers [3) UE can adjust the transmission number of PSFCH across aggregated carriers such that its total transmission number does not exceed the maximum transmission power.] FFS whether/how to merge FG for SL-CA | 47-v1, [15-11] | Yes | No |  | [Per band] | N/A | N/A |  | Candidate values for X are {FFS} Candidate values for Y are {FFS} Note: for component 1-1, it is up to UE implementation which PSFCH(s) to receive | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 47-k1 and 47-k2 are updated as follows

| 47. NR_SL_enh2 | 47-k1 | SL channel access for dynamic channel access mode | UE supports 1. SL Type 1 channel access and contention window size adjustment 2. SL Type 2A channel access 3. SL Type 2B channel access 4. SL Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol in 15kHz SCS if the UE supports 15 kHz SCS 7. CP extension up to 2 symbols in 30kHz SCS [8. CP extension up to 2 symbols if the UE supports 60kHz SCS] [9. Monitoring SCI to decode] FFS whether to merge with 47-k3 | At least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] At least one of {15-25, 15-3, [32-4, 32-4a]} | Yes | No | UE does not support channel access for NR sidelink operation in shared spectrum. | [Per band] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Component 8 is applicable in regions without OCB requirements. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling [For UE supports NR SL in unlicensed spectrum where and when shared spectrum channel access must be used, UE must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47- k2 | SL multi-channel access for dynamic channel access mode | 1. UE supports multi-channel access procedures for PSCCH/PSSCH/S-SSB/PSFCH transmission(s) in multiple RB sets in a slot 2. UE supports Type A and Type B multi-channel access procedures for concurrent PSFCH transmissions in multiple RB sets in a slot [3. UE supports Type A and Type B multi-channel access procedures for concurrent S-SSB transmissions in multiple RB sets] 4) UE supports multi-channel access procedure on N channel(s) with 20MHz LBT bandwidth for each channel. Candidate values of N: {2, 3, 4, 5} | 47-k1, 15-11, and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | [No] | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in shared spectrum. | [Per band / Per UE or Per FS (for 20MHz BW only UE)] | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Support of S-SSB/PSFCH transmission(s) in multiple RB-sets in a slot is according to the support of {47-m11, 47-m11a} and {47-m12, 47-m12a} | Optional with capability signalling |

Agreement

- FG 47-k3 and 47-k4 are updated as follows

| 47. NR_SL_enh2 | 47- k3 | Receiving UE to UE COT sharing information | [1. UE supports monitoring SCI to read COT sharing information] 2. UE supports transmitting NR SL based on COT sharing information subject to COT sharing conditions FFS whether to merge with 47-k1 | 47-k1, 15-1, and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support using UE-to-UE COT sharing information contained in SCI for sharing COT for NR sidelink operation in unlicensed shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR SL in unlicensed spectrum where shared spectrum channel access must be used, UE must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k4 | Transmitting UE to UE COT sharing information | 1. UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing 2. UE supports indicating in SCI COT sharing information in SCI | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | No | [No] | UE does not support transmitting UE-to-UE COT sharing information for sharing COT for NR sidelink operation in unlicensed shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |

Agreement

- FG 47-k6 and 47-k7 are updated as follows

| 47. NR_SL_enh2 | 47-k6 | Type1 LBT blocking Option 1 | UE supports 1. avoid selection of N consecutive resource(s) before a reserved resource when the L1 SL priority value for the transmission is higher than the L1 SL priority value of the reserved resource. It is up to UE whether to do it 2. avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. It is up to UE whether to do it | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support Type1 LBT blocking Option 1 | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k7 | Type1 LBT blocking Option 2 | UE supports 1. If transmission in slot(s) at least T_proc,0 before a reserved resource is able to share its initiated COT to the reservation [when the L1 SL priority value for the transmission is higher than the L1 SL priority value of the reserved resource], UE prioritize / select resource(s) in the slot(s) for transmission. It is up to UE whether to do it | 47-k1 and at least one of [15-25 except Component 3 and 4 in 15-2], [15-3 except Component 3] | Yes | No | UE does not support Type1 LBT blocking Option 2 | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |

R1-2310825	Sidelink unlicensed UE features	FUTUREWEI

R1-2310855	UE features for NR sidelink evolution	Huawei, HiSilicon

R1-2310890	On UE features for NR sidelink evolution	Nokia, Nokia Shanghai Bell

R1-2311118	Discussion on Rel-18 sidelink UE features	vivo

R1-2311184	Discussion on UE features for NR sidelink evolution	Spreadtrum Communications

R1-2311278	UE features list for Rel-18 NR sidelink evolution WI	OPPO, Huawei, HiSilicon, LG Electronics

R1-2311329	Further discussion on UE features for NR sidelink evolution	CATT, CICTCI

R1-2311394	Discussion on UE features for NR sidelink evolution	xiaomi

R1-2311519	Discussion on UE features for NR sidelink evolution	ZTE, Sanechips

R1-2311643	Discussion on UE features for NR SL evolution	NTT DOCOMO, INC.

R1-2311707	Views on UE Features for Rel-18 NR Sidelink Evolution	Apple

R1-2311868	UE features for NR sidelink evolution	Samsung

R1-2311977	Discussion on UE feature list for NR Sidelink Evolution	MediaTek Inc.

R1-2312062	UE feature for sidelink evolution	Qualcomm Incorporated