3GPP TSG RAN WG1 #116			R1-2400637

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	5

Source: 	ZTE, Sanechips

Title: 	Discussion on the LS from RAN4 on SL-U RSSI measurement

Document for:	Discussion and decision

Introduction

In RAN4 LS R1-2400019(R4-2321535), an agreement on SL-U RSSI measurement was reached by RAN4 when 2nd candidate starting symbol is configured [1].

Agreement: - For congestion control purpose, SL-RSSI measurements use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol.

In this contribution, some clarifications on the cases of SL-U RSSI measurement when 2nd candidate starting symbol is configured are discussed, and the suggestion on how to take RAN4’s agreement into RAN1’s work is provided from our side.

Discussions

Cases of SL-U RSSI measurement

When 2nd candidate starting symbol is configured, in a resource pool, there are two cases for a slot, as shown in Figure 1:

Case 1: A slot with PSFCH symbols;

Case 2: A slot without PSFCH symbols.

PSCCH/PSSCH

Case 1: Slot with PSFCH

Case 2: Slot w/o PSFCH

PSCCH/PSSCH

PSFCH

Figure 1: Two cases of a slot

In case 1 above, as RAN1 agreement below in [2], for the slot with PSFCH symbols, only 1 candidate starting for PSCCH/PSSCH is applied, i.e. PSCCH/PSSCH transmission on 1st starting symbol is allowed only. That means, SL transmission on the symbols from the 1st candidate starting symbol and end of the last usable symbol for PSSCH before PSFCH can be considered for SL-RSSI measurement.

Agreement (RAN1#109-e)

Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.

In case 2 above, as agreed in RAN1, the transmission on 1st starting symbol or 2nd starting symbols is allowed depend on the result of channel access. There may be no SL transmission on the symbols before the 2nd candidate starting symbol, as RAN4’s observation, while PSCCH/PSSCH transmission can be always assumed on the symbols starting from the symbol of the 2nd candidate starting symbol and end of the last usable symbol for PSCCH/PSSCH if SL transmission was performed on this slot.

Based on above analysis, the two cases of a slot should be distinguished for SL-U RSSI measurement when 2nd candidate starting symbol is configured in a resource pool.

For congestion control purpose, SL-RSSI measurements on a slot with or without PSFCH symbols is different when 2nd candidate starting symbol is configured.

Views on SL-RSSI measurements

Considering the cases of a slot with or without PSFCH symbols, for SL-RSSI measurements, for case 1(i.e. a slot with PSFCH symbols), SL-U RSSI measurement should use the OFDM symbols starting from the next symbol of the 1st candidate starting symbol and end of the last usable symbol for PSSCH. In case 2(i.e. a slot without PSFCH symbols), SL-U RSSI measurement should use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol, as RAN4’s agreement. The following proposal is given:

For a slot without PSFCH, SL-U RSSI measurement should use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol, as RAN4’s agreement, while for a slot with PSFCH, SL-U RSSI measurement should use the OFDM symbols starting from the next symbol of the 1st candidate starting symbol and end of the last usable symbol for PSSCH, same as Rel-16.

Conclusion

Based on the discussion on SL-U RSSI measurement, the following observation and proposal are given:

Observation 1:	For congestion control purpose, SL-RSSI measurements on a slot with or without PSFCH symbols is different when 2nd candidate starting symbol is configured.

Proposal 1:	For a slot without PSFCH, SL-U RSSI measurement should use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol, as RAN4’s agreement, while for a slot with PSFCH, SL-U RSSI measurement should use the OFDM symbols starting from the next symbol of the 1st candidate starting symbol and end of the last usable symbol for PSSCH, same as Rel-16.

References

R1-2400019(R4-2321535) LS on SL-U RSSI measurement, RAN4, RAN1 #116

Chair's notes RAN1#109-e, Chairman, RAN1 #109-e