3GPP TSG RAN WG1#116	        	R1-2400862

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:					    9.5.3

Source:	Sony

Title:	Considerations on adaptation of SSB in time domain

Document for:	Discussion

# Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial

    - This study is to be done in 2Q’2024 only

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

According to the WID for Rel-19 enhancements of NES [1], adaptation of common signal/channel transmissions will be specified in Rel-19.

In this contribution, our considerations on adaptation of SSB in time domain are made.

# Discussion

For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

It has been already defined according to the following text in TS 38.213[2] that legacy UE assume that SSB periodicity is 20 msec only for initial access.

If the periodicity of SSB used for initial access would be changed to more than 20 msec, it would impact on initial access for all legacy UEs since the legacy UEs are impossible to recognize that the periodicity is changed to more than 20 msec. More specifically, although the legacy UEs assume to receive SSB every 20 msec, SSB is not actually transmitted on some SSB occasions. Due to lack of SSB resource, it would make negative impact on SSB detection performance.

Observation 1: For initial cell selection, legacy UE assumes 20 msec of SSB periodicity. If SSB periodicity on initial bandwidth part would be changed to more than 20 msec, it would impact on initial access for legacy UEs.

On the other hand, the SSB periodicity could has been able to be configured on a range between 5 msec and 160 msec. Since UE assumes 20 msec of SSB periodicity for initial access, the SSB with coarse periodicity should be used for other purposes than initial access (e.g., cell synchronization, RRM/RLM/BM measurement, etc.). In other words, the SSB is typically used as non-cell defining (NCD) SSB which is not associated with SIB1 since the NCD SSB shouldn’t be used for initial access.

Observation 2: Adapting periodicity of non-cell defining (NCD) SSB has been able to be configured up to 160 msec since Rel-15 NR. There would be no negative impact to legacy UEs if the SSB with coarse periodicity up to 160 msec is used as NCD SSB.

It was described in the WID[1] that “Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown”. Therefore, it would be desired that SSB adapting 20 msec periodicity for CD SSB and more than 160 msec periodicity for NCD SSB should not be used to the legacy UE, otherwise negative impact to legacy UEs would be occurred.

On the other hand, Rel-19 NES UE should use the SSB for initial access to increase access opportunities.

Observation 3: SSB adapting more than 20 msec periodicity for initial access and more than 160 msec periodicity for other purposes is not recommended to be used to the legacy UE. The SSB could be used to Rel-19 NES UE for all purposes.

To avoid any negative impact to legacy UEs, preventing legacy UE for initially accessing by using SSB adapting coarse periodicity would be considered. If the legacy UE would not detect the SSB or recognize that the SSB is not suitable for access at the early stage, the negative UE impact in terms of access latency, RRM measurement, and performance could be reduced.

First option is to indicate “cell barred” on a MIB to the legacy UE. The MIB contains a field of “cellbarred” which indicates access barring to the legacy UE. To prevent accessing by the legacy UE, the field of “cellbarred” in MIB carried by the SSB with coarse periodicity could be always ON, then the legacy UE would try to search different SSB. Rel-19 NES UE can ignore the field of “cellbarred” then use the SSB for initial access.

Second option is to indicate as NCD SSB. NCD SSB is not used by the legacy UE for initial access. Like the cell barring, the legacy UE would try to search different SSB if the legacy UE is indicated as NCD SSB from the detected SSB. Rel-19 NES UE can ignore the indication of NCD SSB then use the SSB for initial access.

Third option is to locate the SSB on different frequency raster than existing frequency raster. The legacy UE search a SSB by using on frequency raster defined in TS 38.101[4] and would not search a SSB with coarse periodicity if the SSB is located on different frequency raster. Rel-19 NES UE can search SSB on both legacy frequency raster and new frequency raster.

Fourth option is to apply scramble code to a PBCH in the SSB with coarse periodicity. According to TS 38.212[5], no scramble code was applied to the PBCH. If the legacy UE would try to decode PBCH applying any scramble code, since PBCH decoding would be failed then UE determines cell access failure due to failure to failure of MIB acquisition. Rel-19 NES UE can successfully decode the PBCH because the Rel-19 NES UE know the scramble code.

The above four options would be helpful for the legacy UE to prevent accessing by using the SSB adapting coarse periodicity.

Proposal: One or more mechanisms should be considered to prevent legacy UE for initially accessing by using SSB adapting coarse periodicity. The following options would be candidate:

- Indicating “cell barred” in cellBarred on the MIB to the legacy UE

- Indicating as non-cell defining SSB

- Locating on different frequency raster than existing frequency raster

- Applying scramble code to PBCH

# Conclusions

The following proposals were made in this contribution.

Observation 1: For initial cell selection, legacy UE assumes 20 msec of SSB periodicity. If SSB periodicity on initial bandwidth part would be changed to more than 20 msec, it would impact on initial access for legacy UEs.

Observation 2: Adapting periodicity of non-cell defining (NCD) SSB has been able to be configured up to 160 msec since Rel-15 NR. There would be no negative impact to legacy UEs if the SSB with coarse periodicity up to 160 msec is used as NCD SSB.

Observation 3: SSB adapting more than 20 msec periodicity for initial access and more than 160 msec periodicity for other purposes is not recommended to be used to the legacy UE. The SSB could be used to Rel-19 NES UE for all purposes.

Proposal: One or more mechanisms should be considered to prevent legacy UE for initially accessing by using SSB adapting coarse periodicity. The following options would be candidate:

- Indicating “cell barred” in cellBarred on the MIB to the legacy UE

- Indicating as non-cell defining SSB

- Locating on different frequency raster than existing frequency raster

- Applying scramble code to PBCH

# References

[1]			 RP-234065	, “New WID: Enhancements of network energy savings for NR,” 3GPP TSG RAN Meeting #102, December 2024.

[2] TS 38.213,

[3] TR 38.864, “Study on network energy savings for NR”

[4] TS 38.101

[5] TS 38.212