3GPP TSG RAN WG1 #116			R1-2301712

Athens, Greece, February 26th – March 1st, 2024

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.12.2

Agenda Item:		8.12.2

Document for: 	Endorsement

### 8.12.2	UE features for NR sidelink evolution

R1-2401511	Summary of discussion on UE features for NR sidelink evolution	Moderator (NTT DOCOMO, INC.)

Agreement:

- Prerequisite FG of FG47-m1 is At least one of {15-25, 15-3, [32-4, 32-4a]}

- Add following notes for FG47-m1

  - Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2.

  - Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3.

  - Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs

Agreement:

- Prerequisite FG of FG47-m3 is At least one of {15-25, 15-3, [32-4, 32-4a]}

- Add following notes for FG47-m3

  - Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2.

  - Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3.

  - Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs

  - The FG is only expected for a band where shared spectrum channel access must be used.

Agreement:

- Prerequisite FG of FG47-m5 is 15-11

Agreement:

- “Need for the gNB to know if the feature is supported” for FG47-m6 is No

- FG47-m6 is Optional without capability signaling

- Reporting granularity of FG47-m6 is Per band

- Prerequisite FG of FG47-m6 is 15-4

- Remove “FFS whether to define the supported maximum number of repetitions in frequency domain within one RB set” from component of FG47-m6

- Remove “FFS whether/how to introduce Rx capability” from component of FG47-m6

- Note for FG47-m6 is kept, i.e., remove brackets

Agreement:

- Prerequisite FG of FG47-m8 is 15-4

Agreement:

- Prerequisite FG of FG47-m9 is 15-4

Agreement:

- Reporting granularity of FGs that are “Optional without capability signaling” is not described (i.e., remove text if any)

Agreement:

- Prerequisite FG of FG47-m10 is At least one of {15-25, 15-3, [32-4, 32-4a]}

- Add following notes for FG47-m10

  - Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2.

  - Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3.

  - Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs

- “Need for the gNB to know if the feature is supported” for FG47-m10 is Yes

- “Applicable to the capability signalling exchange between UEs” for FG47-m10 is No

- FG47-m10 is Optional with capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-m10 is kept as it is

- Reporting granularity of FG47-m10 is Per band

- Note for FG47-m10 is kept as it is

Proposal 2.8-2:

- “Applicable to the capability signalling exchange between UEs” for FG47-m4 is No

- FG47-m4 is Optional without capability signaling

  - FFS: [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.]

- Prerequisite FG of FG47-m4 is 15-1, and following notes are added

  - Note: If UE supports 15-1, the UE is not required to support Component 5

  - Note: It is up to RAN2 whether/how to implement the above Note and whether/how to update the prerequisite FGs

- Reporting granularity of FG47-m4 is Per band

Proposal 2.13-1:

- Prerequisite FG of FG47-m11 is 47-k2

- “Need for the gNB to know if the feature is supported” for FG47-m11 is No

- “Applicable to the capability signalling exchange between UEs” for FG47-m11 is No

- FG47-m11 is Optional without capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-m11 is kept as it is

- Reporting granularity of FG47-m11 is Per band

- Note for FG47-m11 is kept with updating “signaling” to “FG”

Proposal 2.13-2:

- Prerequisite FG of FG47-m11a is 47-k2

- “Need for the gNB to know if the feature is supported” for FG47-m11a is No

- “Applicable to the capability signalling exchange between UEs” for FG47-m11a is No

- FG47-m11a is Optional without capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-m11a is kept as it is

- Reporting granularity of FG47-m11a is Per band

- Note for FG47-m11a is kept with updating “signaling” to “FG”

Proposal 2.13-3:

- Prerequisite FG of FG47-m12 is 47-k2

- “Need for the gNB to know if the feature is supported” for FG47-m12 is No

- “Applicable to the capability signalling exchange between UEs” for FG47-m12 is No

- FG47-m12 is Optional without capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-m12 is kept as it is

- Reporting granularity of FG47-m12 is Per band

- Note for FG47-m12 is kept with updating “signaling” to “FG”

Proposal 2.13-4:

- Prerequisite FG of FG47-m12a is 47-k2

- “Need for the gNB to know if the feature is supported” for FG47-m12a is No

- “Applicable to the capability signalling exchange between UEs” for FG47-m12a is No

- FG47-m12a is Optional without capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-m12a is kept as it is

- Reporting granularity of FG47-m12a is Per band

- Note for FG47-m12a is kept with updating “signaling” to “FG”

Proposal 2.14-1:

- FG47-m13 is kept, i.e., remove yellow highlight

- Component for FG47-m13 is updated as follows

  - 1. UE can transmit up to K PSFCH(s) in a slot, where each PSFCH transmission occupy K3 dedicated PRBs.

  - 2. UE can receive up to L PSFCH(s) in a slot, where each PSFCH reception occupy K3 dedicated PRBs

- Prerequisite FG of FG47-m13 is 47-k1

- “Need for the gNB to know if the feature is supported” for FG47-m13 is No

- “Applicable to the capability signalling exchange between UEs” for FG47-m13 is No

- FG47-m13 is Optional without capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-m13 is kept as it is

- Reporting granularity of FG47-m13 is Per band

- Note for FG47-m13 is updated as follows

  - The signaling is only expected for a band where shared spectrum channel access must be used.

  - Candidate values for K are {4, 8, 16}

  - Candidate values for L are {5, 15, 25, 32, 35, 45, 50, 64}

Proposal 2.1-1:

- Remove “FFS whether to merge with 47-k3” from component of FG47-k1

- Remove “FFS whether to merge with 47-k1” from component of FG47-k3

- Remove bracket from “[For UE supports NR SL in unlicensed spectrum where shared spectrum channel access must be used, UE must indicate this FG is supported]” in note of FG47-k3

Proposal 2.1-2:

- Remove bracket from “[32-4, 32-4a]” in prerequisite FG of FG47-k1

- Remove “[9. Monitoring SCI to decode]” from component of FG47-k1

Proposal 2.1-3:

- “Applicable to the capability signalling exchange between UEs” for FG47-k2 is No

- Regarding “transmission of PSFCHs on a subset of the target set of RB sets based on channel access outcome”,

  - Alt.1: New FG is defined

  - Alt.2: it is covered by component of FG47-k2

Proposal 2.2-1:

- “Need for the gNB to know if the feature is supported” for FG47-k3 is No

- “Applicable to the capability signalling exchange between UEs” for FG47-k3 is No

- FG47-k3 is Optional without capability signaling

Proposal 2.2-2:

- “Need for the gNB to know if the feature is supported” for FG47-k4 is No

- “Applicable to the capability signalling exchange between UEs” for FG47-k4 is No

- FG47-k4 is Optional without capability signaling

Proposal 2.3-1:

- Prerequisite FG of FG47-k5 is 47-k1

- “Need for the gNB to know if the feature is supported” for FG47-k5 is No

- “Applicable to the capability signalling exchange between UEs” for FG47-k5 is No

- FG47-k5 is Optional without capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-k5 is kept as it is

- Reporting granularity of FG47-k5 is Per band

- Note for FG47-k5 is kept as it is

Proposal 2.5-1:

- FG47-k8 is kept, i.e., remove yellow highlight

- Remove “FFS whether to merge with 47-k1” from component of FG47-k8

- Component for FG47-k8 is kept as it is

- Prerequisite FG of FG47-k8 is 47-k1

- “Need for the gNB to know if the feature is supported” for FG47-k8 is No

- “Applicable to the capability signalling exchange between UEs” for FG47-k8 is No

- FG47-k8 is Optional without capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-k8 is kept as it is

- Reporting granularity of FG47-k8 is Per band

- Note for FG47-k8 is kept as it is

Proposal 2.6-1:

- FG47-k9 is kept, i.e., remove yellow highlight

- Component for FG47-k9 is kept as it is

- Prerequisite FG of FG47-k9 is 47-k1

- “Need for the gNB to know if the feature is supported” for FG47-k9 is Yes

- “Applicable to the capability signalling exchange between UEs” for FG47-k9 is No

- FG47-k9 is Optional with capability signaling

- “Consequence if the feature is not supported by the UE” for FG47-k9 is kept as it is

- Reporting granularity of FG47-k9 is Per band

- Note for FG47-k9 is kept as it is

Agreement:

- Prerequisite FG of FG47-s1 is kept as it is, i.e., remove bracket

Agreement:

- “FFS whether to report Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode” in component of FG47-v1 is updated as follows

  - 1-3) For the number of non-overlapped PRBs over aggregated SL carriers:

    - UE can attempt to decode NRB,i non-overlapping RBs in a slot on carrier i of the X carriers.

Proposal 4-1:

- Remove “FFS whether/how to merge FG for SL-CA” from component of FG47-v1/v2/v3

Proposal 4-2:

- Prerequisite FG of FG47-v1 is kept as it is, i.e., remove bracket

- “Applicable to the capability signalling exchange between UEs” for FG47-v1 is No

- “FFS on some BW restriction” in note of FG47-v1 is removed and following updates are made

  - Add “X. UE can aggregate up to total bandwidth B” in component

  - Add “Component X candidate value set: B = {20MHz, 40MHz, 70MHz, FFS other values}” in note

Proposal 4-3:

- Prerequisite FG of FG47-v2 is kept as it is, i.e., remove bracket

Proposal 4-4:

- Prerequisite FG of FG47-v3 is 47-v1 (i.e., [15-11] is removed)

- Component 3 in FG47-v3 is removed

- Candidate values for X in component 1 of FG47-v3 are {5, 15, 25, 32, 35, 45, 50, 64, 100, 128}

- Candidate values for Y in component 2 of FG47-v3 are {4, 8, 16, 32, 64}

R1-2400082	Remaining issues of NR sidelink evolution UE features	Nokia, Nokia Shanghai Bell

R1-2400131	UE features for NR sidelink evolution	Huawei, HiSilicon

R1-2400227	Discussion on Rel-18 sidelink UE features	vivo

R1-2400414	Remaining issues on UE features for NR sidelink evolution	CATT, CICTCI

R1-2400535	Maintenance on UE features for NR sidelink evolution	xiaomi

R1-2400584	UE features list for Rel-18 NR sidelink evolution WI	OPPO, Huawei, HiSilicon, LG Electronics

R1-2400639	UE features for NR sidelink evolution	ZTE, Sanechips

R1-2400715	UE features for NR sidelink evolution	Samsung

R1-2400997	On UE Features for Rel-18 NR Sidelink Evolution	Apple

R1-2401102	Discussion on UE features for NR SL evolution	NTT DOCOMO, INC.

R1-2401168	Discussion on UE features for Rel-18 NR sidelink evolution	Sharp

R1-2401321	UE feature for NR sidelink evolution	MediaTek Inc.

R1-2401425	UE Features for Sidelink Evolution	Qualcomm Incorporated