3GPP TSG RAN WG1 #116-bis	                                           R1-2402239

Changsha, Hunan Province, China, April 15th – 19th, 2024

Source:	vivo

Title:	Discussion on Rel-19 SBFD operation

Agenda Item:	9.3.1

Document for:	Discussion and Decision

- Introduction

In RAN#102, a WI on evolution of NR duplex operation in Rel-19 was approved, with the target of specifying SBFD [1].

It is common understanding that in agenda 9.3.1 the following is to be discussed.

- Semi-static indication of time & frequency locations of SBFD subbands to UEs in RRC_CONNECTED mode.

- UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE.

Regarding the above, during RAN1#116 meeting, a lot of agreements were achieved, which are listed in Annex for reference.

In this contribution, semi-static configuration of SBFD subbands in RRC_CONNECTED mode is discussed firstly, followed by discussion on transmission and reception behaviours on SBFD subbands in symbols configured as DL and/or flexible in TDD-UL-DL-ConfigCommon. Potential enhancements for SBFD operation are discussed lastly, including enhancements on resource allocation in frequency domain in SBFD symbols, UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, as well as collision handling between DL receptions in DL subband(s) and UL transmissions in UL subband in SBFD symbols.

- Semi-static configuration of SBFD subbands

  - Configuration granularity/level

Regarding configuration granularity/level, it was discussed preliminarily during the SI phase, i.e. whether SBFD subbands, including their frequency locations and time locations, are configured in cell common configuration or in UE dedicated configuration, as well as in carrier/serving cell level or in BWP level.

During RAN1#116 meeting, this issue was discussed further, with the following agreement achieved.

Agreement

For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier.

- FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

In our understanding, once an UL subband is planned in DL/flexible symbol(s) for SBFD operation, the part of frequency resources in the UL subband can be regarded as a sub-pool from the whole pool of frequency resources for the TDD carrier. In this regard, generally the UL subband, as well as symbol(s) planned for SBFD operation, is common to all SBFD aware UEs served by the TDD carrier, and can be configured in cell common configuration and in carrier/serving cell level. For a BWP pair configured for the TDD carrier/serving cell, actual UL subband to be applied within the BWP pair can be derived based on frequency location of the UL/DL BWP of the BWP pair, as well as that of the UL subband commonly configured for the TDD carrier/serving cell.

Based on the above analysis, the working assumption on cell-specific configuration on frequency locations of SBFD subbands can be confirmed.

Proposal 1: Confirm the working assumption that for SBFD aware UEs in RRC_CONNECTED mode, cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

  - Configuration of time locations for SBFD subbands

For semi-static configuration of subband time locations for SBFD operation, during RAN1#116 meeting, the following agreement was achieved.

Agreement:

For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options.

- Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- FFS: Further details

FFS: Details when two TDD-UL-DL patterns are configured

Regarding the above two options for configuring periodicity of SBFD subband time locations, it is common understanding that Option 2 can cover Option 1, and allows configuring a periodicity longer than TDD-UL-DL pattern period, with more flexibility at the cost of potentially more complexity, e.g. introducing corresponding parameter to configure integer multiple of TDD-UL-DL pattern period, differentiating TDD-UL-DL pattern periods with and without SBFD subbands.

Based on our understanding, the flexibility provided by Option 2 may be marginal, since when Option 1 is adopted, SBFD symbols within a period can be configured flexibly to meet the need of SBFD operation, which can afford sufficient flexibility already. Besides, when Option 2 would be adopted, and a periodicity longer than TDD-UL-DL pattern period would be configured, UL latency performance would be degraded accordingly. Therefore, Option 1 is slightly preferred for simplicity.

Proposal 2: For semi-static configuration of SBFD subband time locations within a period, at least when only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

Regarding the FFS point in the above agreement, when two TDD-UL-DL patterns are configured, i.e. both pattern1 and pattern2 are configured in TDD-UL-DL-ConfigCommon, it is straightforward that the periodicity is the sum of two periods, corresponding to two configured TDD-UL-DL patterns, respectively.

Proposal 3: For semi-static configuration of SBFD subband time locations within a period, when two TDD-UL-DL patterns are configured, the period is the sum of two periods corresponding to two configured TDD-UL-DL patterns, respectively.

  - Configuration of frequency locations for SBFD subbands

For semi-static configuration of subband frequency locations for SBFD operation, during RAN1#116 meeting, the following agreement was achieved.

Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.

The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

For semi-static indication of SBFD subband frequency location, down-select from the following options.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

The main difference between the above two options is whether frequency location(s) of DL subband(s) or the number of RBs for guardband(s), if any, is explicitly configured. From our perspective, Option 1 is more straightforward, since in any case of SBFD operation the UE should be aware of the frequency location(s) of DL subband(s) in SBFD symbols, however guardband(s) may not be needed in some cases. On the contrary, Option 2 will result in less configuration overhead; besides, it is consistent with configuration of RB sets in Rel-16 NR-U.

In our opinion, only minor differences exist between these two options. We are open to support Option 1 or Option 2.

Proposal 4: For semi-static configuration of SBFD subband frequency locations in SBFD symbols:

- Frequency location(s) of DL subband(s) or the number of RBs for guardband(s) is explicitly configured.

- Guardband(s)/DL subband(s) is implicitly derived as the RBs which are not within UL subband and DL subband(s)/guardband(s).

Besides, for cell-specific configuration of frequency locations of SBFD subbands, the following agreement was achieved in RAN1#116 meeting.

Agreement

The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

  - Subject to RAN4 guidance on the size of subband/guardband, if any

  - FFS reference starting RB and reference SCS

Regarding the FFS point on reference SCS, at least the following options can be identified.

- Option 1: Single reference SCS is assumed for configuration of SBFD subband frequency locations.

  - Option 1-1: referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as the reference SCS for frequency locations, i.e., the same reference SCS is used for configuration of both time locations and frequency locations of SBFD subbands.

  - Option 1-2: The reference SCS for frequency locations is configured separately from that for time locations, i.e., different reference SCSs are used for configuration of subband time locations and frequency locations, respectively. It may be beneficial to guarantee that the reference SCS is not larger than any SCS of configured UL BWPs for the serving cell.

- Option 2: Multiple reference SCSs are allowed for configuration of SBFD subband frequency locations. For example, for each SCS-SpecificCarrier configured in scs-SpecificCarrierList in FrequencyInfoUL-SIB or FrequencyInfoUL, frequency location(s) of cell-specific SBFD subband(s) is configured based on subcarrierSpacing in this SCS-SpecificCarrier. To determine UL usable PRBs for active UL BWP, the UL subband configured for the SCS-SpecificCarrier with a same subcarrierSpacing as that of the active UL BWP can be used.

Option 1 is simpler. On the contrary, Option 2 can afford more flexibility, and is more aligned with SCS specific carrier configuration mechanism adopted since Rel-15 NR.

Proposal 5: To determine reference SCS(s) for configuration of SBFD subband frequency locations, the following options can be considered:

- Option 1: Single reference SCS is assumed.

- Option 1-1: referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as the reference SCS for frequency locations.

- Option 1-2: The reference SCS for frequency locations is configured separately from that for time locations.

- Option 2: Multiple reference SCSs are allowed, e.g., using subcarrierSpacing in SCS-SpecificCarriers configured in scs-SpecificCarrierList in FrequencyInfoUL-SIB or FrequencyInfoUL.

Regarding the FFS point on reference starting RB, it is straightforward that for a reference SCS, only a single corresponding reference starting RB is assumed for configuration of SBFD subband frequency locations. Consequently, at least the following options can be identified to determine reference starting RB(s) for configuration of SBFD subband frequency locations.

- Option 1: Point A is used as the reference starting RB. In our understanding, Option 1 can be used in conjunction with any option discussed in Proposal 5, i.e. irrespective of whether single or multiple reference SCSs are used. In other words, Point A is common for all applicable SCSs, or independent of any SCS, within a TDD carrier.

- Option 2: Lowest usable RB on a TDD carrier is used as the reference starting RB corresponding to a reference SCS. When Option 1 listed in Proposal 5 is assumed, i.e., a single reference SCS is assumed, the lowest usable RB is determined by offsetToCarrier in a SCS-SpecificCarrier configured in scs-SpecificCarrierList in FrequencyInfoUL-SIB or FrequencyInfoUL corresponding to the TDD carrier, and with a same subcarrierSpacing as the single reference SCS. In contrast, when Option 2 listed in Proposal 5 is assumed, i.e., multiple reference SCSs are allowed, for each SCS-SpecificCarrier configured in scs-SpecificCarrierList in FrequencyInfoUL-SIB or FrequencyInfoUL corresponding to the TDD carrier, for which frequency location(s) of cell-specific SBFD subband(s) is configured based on subcarrierSpacing in it, corresponding lowest usable RB is determined by offsetToCarrier in this SCS-SpecificCarrier. In addition, subcarrierSpacing in this SCS-SpecificCarrier can be referred to as reference SCS corresponding to this SCS-SpecificCarrier as well.

Proposal 6: To determine reference starting RB(s) for configuration of SBFD subband frequency locations, the following options can be considered:

- Option 1: Point A is used as the reference starting RB.

- Option 2: Lowest usable RB on a TDD carrier is used as the reference starting RB corresponding to a reference SCS.

- Transmission and reception behaviors on SBFD subbands

  - Determination of usable PRBs

  - During RAN1#116 meeting, regarding usable PRBs, the following agreement was achieved.

Agreement

For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs.

For determining UL/DL usable PRBs, consider the following options.

- Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

- Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

  - For the above two options, based on discussion in section 2.1, since cell-specific configuration on frequency location of SBFD subbands is supported, Option 1 is straightforward and simple, and there seems no need to introduce Option 2.

Proposal 7: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

  - Determination of link direction

Regarding determination of actual link direction for a SBFD symbol, the following agreement was achieved during RAN1#116 meeting.

Agreement

For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Other options are not precluded.

In our opinion, Option 2 is straightforward, and can be realized by reusing legacy signaling. For example, if the UE is provided with TDD-UL-DL-ConfigDedicated, and/or configured to monitor dynamic SFI, TDD-UL-DL-ConfigDedicated and/or dynamic SFI can be used to indicate link direction explicitly for SBFD symbols, which is mentioned in section 3.3 as well.

On the contrary, if link direction is not indicated explicitly by the gNB, e.g. the UE is neither provided with TDD-UL-DL-ConfigDedicated, nor configured to monitor dynamic SFI, the UE can determine link direction based on configured/scheduled transmissions/receptions once collision is avoided by proper gNB configuration/scheduling. If there may be collision(s) between UL transmission(s) and DL reception(s) in SBFD symbol(s), collision handling discussed in section 4.3 can be used to determine the actual link direction in the SBFD symbol(s). Therefore, Option 1 is also reasonable in such cases.

Based on the above analysis, we are open to support both the above two options.

Proposal 8: For SBFD-aware UE transmission and reception in an SBFD symbol, link direction is indicated by the gNB explicitly, or the UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

  - Interaction with TDD-UL-DL-ConfigDedicated and/or dynamic SFI

  - Regarding transmission and reception behaviours in SBFD symbols, the following agreement was achieved during RAN1#116 meeting.

Agreement

For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL usable PRBs are allowed

  - FFS SSB symbols

- DL receptions within DL usable PRBs are allowed

- UL transmissions outside UL usable PRBs are not allowed

- DL receptions outside DL usable PRBs are not allowed

  - This restriction is not applicable for CLI measurement

CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3.

RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0

- DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

  - Based on the above agreement, in DL and/or flexible symbols indicated by TDD-UL-DL-ConfigCommon and configured as SBFD symbols, only UL transmissions are allowed within UL subband, and only DL receptions are allowed within DL subband(s), except for potential CLI measurement by the UE outside of the DL subband(s), if interaction with TDD-UL-DL-ConfigDedicated and/or dynamic SFI is not considered. In other words, same behaviours are expected for DL symbol(s) and flexible symbol(s) indicated by TDD-UL-DL-ConfigCommon.

  - Regarding interaction with TDD-UL-DL-ConfigDedicated and that with dynamic SFI, they will be discussed in the following separately.

- Interaction with TDD-UL-DL-ConfigDedicated

In Rel-15/16/17/18, a symbol indicated as flexible in TDD-UL-DL-ConfigCommon can be further overridden as DL/UL by TDD-UL-DL-ConfigDedicated with UE specific RRC signaling.

Regarding SBFD aware UE behaviours in SBFD symbols with interaction with TDD-UL-DL-ConfigDedicated, corresponding discussion for SBFD symbols indicated as flexible in TDD-UL-DL-ConfigCommon can be prioritized in our opinion. The outcome of discussion, if applicable, can be extended to SBFD symbols indicated as DL in TDD-UL-DL-ConfigCommon with minimum effort.

For SBFD aware UE behaviours in SBFD symbols indicated as flexible in TDD-UL-DL-ConfigCommon, the following alternatives can be identified based on our understanding.

- Alt. 1: The UE does not expect to be provided with TDD-UL-DL-ConfigDedicated and configured with SBFD symbols simultaneously for a TDD carrier. In other words, TDD-UL-DL-ConfigDedicated should not be provided to the UE for the TDD carrier since SBFD symbols are configured for the TDD carrier already.

- Alt. 2: TDD-UL-DL-ConfigDedicated shall only indicate link direction for non-SBFD symbols. In other words, TDD-UL-DL-ConfigDedicated should not cover SBFD symbols.

- Alt. 3: TDD-UL-DL-ConfigDedicated may indicate direction for SBFD symbols. In this case, regarding how to interpret the indicated direction, and related UE behavior, the following sub-alternatives can be identified further.

  - Alt. 3-1: TDD-UL-DL-ConfigDedicated shall indicate a same direction (i.e. flexible) for a SBFD symbol as that indicated for the SBFD symbol by TDD-UL-DL-ConfigCommon. At the same time, SBFD operation in the SBFD symbol is not affected by the indication of TDD-UL-DL-ConfigDedicated.

  - Alt. 3-2: The indication of TDD-UL-DL-ConfigDedicated for a SBFD symbol is invalid or inapplicable.

  - Alt. 3-3: TDD-UL-DL-ConfigDedicated indicates the usable SBFD subband(s) for a SBFD symbol. For example, if TDD-UL-DL-ConfigDedicated indicates DL for the SBFD symbol, only DL receptions are allowed within DL subband(s) in the SBFD symbol. In this regard, TDD-UL-DL-ConfigDedicated is used actually for indication of link direction for SBFD symbols, which is also discussed in section 3.2.

  - Alt. 3-4: TDD-UL-DL-ConfigDedicated overrides direction indicated by TDD-UL-DL-ConfigCommon. Specifically, if TDD-UL-DL-ConfigDedicated indicates the SBFD symbol as DL, the SBFD symbol can be changed to a full DL symbol; or the SBFD symbol remains a SBFD symbol, where SBFD operation for semi-static DL symbol will be performed for the SBFD symbol instead. Similarly, if TDD-UL-DL-ConfigDedicated indicates the SBFD symbol as UL, the SBFD symbol is changed to a full UL symbol. This sub-alternative may lead to different time locations for SBFD subbands for different SBFD aware UEs served by a same TDD carrier, and handling of potential interferences may be complicated. Therefore, it can be de-prioritized.

Based on the above analysis, the following proposal is made.

Proposal 9: For SBFD aware UE behaviours with interaction with TDD-UL-DL-ConfigDedicated in SBFD symbols indicated as flexible in TDD-UL-DL-ConfigCommon, the following alternatives can be considered:

- Alt. 1: The UE does not expect to be provided with TDD-UL-DL-ConfigDedicated and configured with SBFD symbols simultaneously for a TDD carrier.

- Alt. 2: TDD-UL-DL-ConfigDedicated shall only indicate link direction for non-SBFD symbols.

- Alt. 3: TDD-UL-DL-ConfigDedicated may indicate direction for SBFD symbols.

- Alt. 3-1: TDD-UL-DL-ConfigDedicated shall indicate a same direction for a SBFD symbol as that indicated for the SBFD symbol by TDD-UL-DL-ConfigCommon.

- Alt. 3-2: The indication of TDD-UL-DL-ConfigDedicated for a SBFD symbol is invalid or inapplicable.

- Alt. 3-3: TDD-UL-DL-ConfigDedicated indicates the usable SBFD subband(s) for a SBFD symbol.

Regarding the above alternatives, configurability of TDD-UL-DL-ConfigDedicated can be maintained in SBFD scenarios, thus Alt. 1 is not preferred. Among the remaining (sub-)alternatives, Alt. 3-3 is slightly preferred, as TDD-UL-DL-ConfigDedicated can be reused to indicate link direction for SBFD symbols.

- Interaction with dynamic SFI

In Rel-15/16/17/18, a UE may be configured to monitor PDCCH for DCI format 2_0, i.e. dynamic SFI. If a detected DCI format 2_0 indicates a semi-static flexible symbol as DL or UL, only DL reception(s) or UL transmission(s) can be performed in this symbol; otherwise, if the detected DCI format 2_0 indicates the semi-static flexible symbol as flexible, only dynamically scheduled DL reception(s)/UL transmission(s) can be performed in this symbol.

Similar to interaction with TDD-UL-DL-ConfigDedicated as discussed above, for SBFD aware UE behaviours in SBFD symbols with interaction with dynamic SFI, the following alternatives can be identified based on our understanding.

- Alt. 1: The UE does not expect to be configured to monitor dynamic SFI and configured with SBFD symbols simultaneously for a TDD carrier. In other words, the UE should not be configured to monitor dynamic SFI for the TDD carrier since SBFD symbols are configured for the TDD carrier already.

- Alt. 2: Dynamic SFI, if detected, shall only indicate link direction for non-SBFD symbols. In other words, a detected dynamic SFI should not cover SBFD symbols.

- Alt. 3: Dynamic SFI, if detected, may indicate direction for SBFD symbols. In this case, regarding how to interpret the indicated direction, and related UE behavior, the following sub-alternatives can be identified further.

  - Alt. 3-1: A detected dynamic SFI shall indicate a same direction for a SBFD symbol as that indicated for the SBFD symbol by TDD-UL-DL-ConfigCommon and/or TDD-UL-DL-ConfigDedicated. At the same time, SBFD operation in the SBFD symbol is not affected by the indication of the detected dynamic SFI.

  - Alt. 3-2: The indication of the detected dynamic SFI for a SBFD symbol is invalid or inapplicable.

  - Alt. 3-3: The detected dynamic SFI indicates the usable SBFD subband(s) for a SBFD symbol. For example, if the detected dynamic SFI indicates DL for the SBFD symbol, only DL receptions are allowed within DL subband(s) in the SBFD symbol. In this regard, dynamic SFI is used actually for indication of link direction for SBFD symbols, which is also discussed in section 3.2.

  - Alt. 3-4: The detected dynamic SFI overrides direction indicated by TDD-UL-DL-ConfigCommon and/or TDD-UL-DL-ConfigDedicated. This sub-alternative may be applicable only to a SBFD symbol indicated as flexible in TDD-UL-DL-ConfigCommon and/or TDD-UL-DL-ConfigDedicated. Based on the above agreement, dynamic SFI cannot be used to revert a SBFD symbol to non-SBFD symbol. Therefore, if the detected dynamic SFI indicates the SBFD symbol as DL, the SBFD symbol indicated as flexible can be changed to a SBFD symbol indicated as DL. This may be marginal or meaningless, since UE behaviors in SBFD symbol indicated as flexible or DL are the same, or at least quite similar.

Besides, if the UE is configured to monitor dynamic SFI, but does not detect a dynamic SFI for a SBFD symbol, it is also beneficial to clarify corresponding UE behavior in such cases.

Proposal 10: For SBFD aware UE behaviours in SBFD symbols with interaction with dynamic SFI, the following alternatives can be considered:

- Alt. 1: The UE does not expect to be configured to monitor dynamic SFI and configured with SBFD symbols simultaneously for a TDD carrier.

- Alt. 2: Dynamic SFI, if detected, shall only indicate link direction for non-SBFD symbols.

- Alt. 3: Dynamic SFI, if detected, may indicate direction for SBFD symbols.

- Alt. 3-1: A detected dynamic SFI shall indicate a same direction for a SBFD symbol as that indicated for the SBFD symbol by TDD-UL-DL-ConfigCommon and/or TDD-UL-DL-ConfigDedicated.

- Alt. 3-2: The indication of the detected dynamic SFI for a SBFD symbol is invalid or inapplicable.

- Alt. 3-3: The detected dynamic SFI indicates the usable SBFD subband(s) for a SBFD symbol.

- FFS: UE behavior in a case if the UE is configured to monitor dynamic SFI, but does not detect a dynamic SFI for a SBFD symbol.

Regarding the above alternatives, configurability of monitoring dynamic SFI can be maintained in SBFD scenarios, thus Alt. 1 is not preferred. Among the remaining (sub-)alternatives, Alt. 3-3 is slightly preferred, as dynamic SFI can be reused to indicate link direction for SBFD symbols.

- Enhancements for SBFD operation

  - Resource allocation in frequency domain

Regarding enhancements for resource allocation in frequency domain in SBFD symbols, the following two categories of enhancements were discussed extensively during the SI phase:

- Resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols.

- Handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource or PRG.

### 4.1.1. Resource allocation across two DL subbands

- PDSCH RA type 1

For PDSCH resource allocation across two DL subbands, generally PDSCH RA type 0 can be adopted to achieve non-contiguous resource allocation. Therefore, no enhancement for PDSCH RA type 1 across two DL subbands was captured in TR 38.858 [2].

Obviously if PDSCH RA type 1 can be enhanced to allocate non-contiguous frequency resources, more flexibility can be achieved. Besides, several companies provided related proposals on this issue during the SI phase.

In our opinion, enhancements for PDSCH RA type 1 across two DL subbands can be considered, and a unified solution for resource allocation across two DL subbands can be applied, e.g. rate matching is performed for a PDSCH, including its DM-RS, around the UL subband and guardband(s) if any automatically, when allocated frequency resources for the PDSCH overlap with the UL subband and guardband(s) if any.

Proposal 11: For a PDSCH with RA type 1, rate matching is performed for the PDSCH, including its DM-RS, around the UL subband and guardband(s) if any automatically, when allocated frequency resources for the PDSCH overlap with the UL subband and guardband(s) if any.

Based on the above proposal, it can be further discussed how to calculate TB size corresponding to the PDSCH, e.g. based on indicated frequency resources for the PDSCH, or resulting frequency resources after excluding RB(s) overlapping with the UL subband or guardband(s) if any.

- Wideband PRG for PDSCH

The issue of wideband PRG for PDSCH in case of non-contiguous DL subbands was discussed extensively during the SI phase, with the following description captured in TR 38.858 [2].

…

If PRG is determined as wideband, the following two options are studied.

-	Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated

-	Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated

It is agreed that Option 1 can achieve better scheduling flexibility and higher DL data rate. Compared with Option 2, Option 1 requires UE to handle two non-contiguous segments of contiguous RBs that may increase UE complexity for channel estimation.

…

Obviously, Option 1 can provide more flexibility, and make full use of frequency resources in two DL subbands, compared to Option 2. About UE complexity for Option 1, we are open to discuss it. If needed, corresponding UE capability can be introduced to resolve the UE implementation concern. Therefore, Option 1 is preferred.

Proposal 12: If PRG is determined as wideband for a PDSCH, support Option1, i.e., non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated for the PDSCH.

Regarding precoding assumption within and across the two DL subbands for Option 1, it is natural that at least frequency resources within a DL subband share the same precoding assumption, by reusing rules for wideband PRG in existing specification. To afford more flexibility, it is slightly preferred that precoding assumptions for frequency resources in two DL subbands can be different.

Proposal 13: If PRG is determined as wideband for a PDSCH, and non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband are allocated for the PDSCH, frequency resources within each DL subband share the same precoding assumption, and precoding assumptions for frequency resources in the two DL subbands can be different.

- CSI-RS

Regarding frequency resource allocation for CSI-RS across non-contiguous DL subbands, the following description is captured in TR 38.858 [2].

…

Frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs are studied considering the following options:

-	Option 1: Two contiguous CSI-RS resources that are linked

-	Option 2: One CSI-RS resource

-	Option 2-1: Non-contiguous CSI-RS resource allocation

-	Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

For all the options, there is no impact on CSI-RS sequence generation. Option 1 requires additional signalling to link two CSI-RS resources in two DL subbands. Option 2-1 requires new RRC structure to configure non-contiguous RBs for one CSI-RS resource, which may require additional signalling overhead. Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. Option 2-2 can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands. Further discussion is required on the UE complexity due to UE capability of maximum number of configured CSI-RS resources and/or processing non-contiguous CSI-RS.

…

In our understanding, Option 1 may have significant impacts on legacy signaling design where CSI-RS resources are involved, and CSI-RS resources are used in many scenarios, such as time-frequency tracking, beam management, RLM/RRM measurements, etc., in addition to CSI reporting. Therefore, much specification work is expected if Option 1 would be adopted.

Regarding Option 2, Option 2-1 is similar to Option 1 in terms of frequency resource allocation, i.e. for a CSI-RS resource, two clusters of contiguous frequency resources are signaled to the UE. On the contrary, for Option 2-2 the legacy signaling can be reused as much as possible, since for a CSI-RS resource, only a single contiguous CSI-RS resource allocation is involved. Therefore, less or even no specification impact is expected.

Based on the above analysis, Option 2-2 is preferred due to simplicity and back compatibility.

Proposal 14: For frequency resource allocation for CSI-RS across DL subbands for SBFD-aware UEs, support Option 2-2, i.e., one CSI-RS resource is configured, where one contiguous CSI-RS resource allocation for the CSI-RS resource is indicated, with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband(s).

### 4.1.2. Handling of unaligned boundaries in frequency domain

- RBG for PXSCH RA type 0

For resource allocation in frequency domain in case of unaligned boundaries between RBG and SBFD subbands, the following description is captured in TR 38.858 [2].

…

For resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands, RAN1 studied whether or not the part of the DL RBG inside/outside the DL subband and the part of the UL RBG inside/outside the UL subband can be used. It is agreed that for SBFD-aware UEs, the part of the DL RBG inside the DL subband can be used and the part of the UL RBG inside the UL subband can be used for better resource utilization. It is agreed that the part of the RBG outside the DL subband cannot be used for DL reception and the part of the RBG outside the UL subband cannot be used for UL transmission at least for semi-static SBFD.

…

Based on the above description, partial RBG(s) resulting from unaligned boundaries between indicated RBG(s) for a PXSCH with RA type 0 and SBFD subbands can be used for the PXSCH. However, in our opinion, partial RBG can also be avoided by proper gNB configuration or scheduling, thus the additional operation or complexity due to partial RBG may be unnecessary.

Similar to PDSCH RA type 1, it can also be further discussed how to calculate TB size corresponding to the PXSCH.

- PRG with size 2 or 4 for PDSCH

Regarding the issue of PRG with size 2 or 4 overlapping with subband boundary, the following description is captured in TR 38.858 [2].

…

For a PRG that overlaps with subband boundary, if the part of DL PRG inside the DL subband can be used, better scheduling flexibility and resource utilization can be achieved, however degraded channel estimation quality in the partial PRG is expected compared to a PRG due to limited RBs in the partial PRG. It is noted that UE complexity could increase if this feature is supported.

…

It is common understanding that partial PRG with size 2 or 4 is supported at boundaries of DL active BWP per existing specification. When DL subband boundaries are taken into account additionally, the same rules can be applied as well. From our perspective, no new rule is introduced, and only the number of partial PRGs may be increased, therefore additional UE complexity due to the increased number of partial PRGs should be acceptable.

Alternatively, additional partial RBGs due to overlapping with DL subband boundaries can be avoided by proper gNB configuration or scheduling.

Proposal 15: Partial PRG with size 2 or 4 at DL subband boundaries can be supported, or avoided by gNB implementation.

- CSI-RS and CSI reporting

Regarding issues of unaligned boundaries for CSI-RS resource and CSI reporting subband, the following description is captured in TR 38.858 [2].

…

For semi-static SBFD, for a CSI reporting subband which overlaps with SBFD subband boundaries, it is agreed that CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s) for SBFD-aware UE.

For semi-static SBFD, for a CSI-RS resource which overlaps with SBFD subband boundaries, it is agreed that only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE.

…

  - Alternatively, these issues can be handled by proper gNB configuration, e.g., the gNB can avoid overlapping with SBFD subband boundaries when configuring CSI-RS resource(s) and/or CSI reporting subband(s).

  - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols

The issue of UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols was extensively discussed for several meetings during the SI phase. Basically, the related discussion can be divided into two parts, of which the first one deals with the case where each transmission/reception occasion within a slot has either all SBFD symbols or all non-SBFD symbols (denoted as Case I), and the second one deals with the case where a transmission/reception occasion overlaps with both SBFD symbols and non-SBFD symbols in the same slot (denoted as Case II). These two cases will be discussed in the following separately.

### 4.2.1. Case I: Each transmission/reception occasion within a slot has either all SBFD symbols or all non-SBFD symbols

For Case I, for cases where a same time-frequency resource allocation applies to one or more UL transmission/DL reception occasions as per Rel-15/16/17/18 specification, the following description is captured in TR 38.858 [2].

…

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following options are studied for SBFD-aware UEs:

-	Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

-	Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

-	PDSCH/PUSCH/PUCCH repetitions

-	SPS PDSCH/CG PUSCH

-	TBoMS

-	Multi-PUSCH/PDSCH scheduled by a single DCI

-	Periodic/semi-persistent SRS/CSI-RS/PUCCH

-	PDCCH

Option 1 can be achieved by gNB configuration or scheduling to ensure that all transmission/reception occasions are confined to either SBFD symbols or non-SBFD symbols. Alternatively, Option 1 can be achieved by additional indication or rules to determine the transmission/reception occasions are valid within one symbol type and are invalid within the other symbol type. The frequency resources, power control and beam/spatial relation for all the transmission/reception occasions can be the same for Option 1 but may be different for Option 2. If different, it may require additional specification efforts. Option 1 may or may not increase the transmission/reception latency if the transmission/reception in the other symbol type is postponed and may degrade the performance if the transmission/reception in the other symbol type is dropped. Option 2 may or may not reduce the transmission/reception latency and improve coverage.

…

Obviously, at least available frequency domain resources for DL/UL within active DL/UL BWP are different between SBFD symbols and non-SBFD symbols. Furthermore, applied antenna configuration (as well as related spatial domain characteristics), allowed/required power, etc. may be different as well, to control or mitigate various types of interferences.

Regarding Option 1, it is simpler at least from UE perspective since there is no need to deal with the above differences when all transmissions/receptions are restricted to a single symbol type, where postponing or dropping can be performed for involved transmissions/receptions. However, it will potentially result in larger latency or degraded coverage, which contradicts with the objectives for introducing SBFD.

Regarding Option 2, clearly it can achieve better latency performance. To deal with the above differences between two symbol types, potential enhancements may be introduced accordingly. Taking PUCCH repetition as an example, if the time-frequency resource allocation for the involved PUCCH resource is configured to ensure that the PUCCH transmission is close to the boundary of the active UL BWP to avoid/mitigate segmentation in frequency domain, when Rel-15/16/17/18 specification would be followed directly, the PUCCH repetition(s) in SBFD symbols would be very likely overlapped with DL subband(s) when frequency domain pattern (D-U-D) is adopted in SBFD symbols (e.g., in Figure 1 when no frequency offset is introduced, and the PUCCH resource occupies PRB(s) at one side of the active UL BWP, obviously both the former two repetitions overlap with the upper DL subband), and the overlapped PUCCH repetition(s) may be dropped or postponed accordingly, resulting in worse transmission performance. On the contrary, if it is guaranteed by the gNB’s configuration that no overlapping occurs for any PUCCH repetition in SBFD symbols, the configuration will be rather restrictive, and obviously segmentation in frequency domain can hardly be avoided in full UL symbols, resulting in lower efficiency for UL resource usage of the whole cell. With respect to potential enhancements to address the above differences between two symbol types, introduction of frequency offset as shown in Figure 1 between SBFD and non-SBFD symbols to simply address only the difference of available UL frequency domain resources, as well as separate configurations for SBFD and non-SBFD symbols as shown in Figure 2 to address potentially all kinds of differences, etc., can be considered.

Figure 1: A frequency offset between PUCCH repetitions in different symbol types

Figure 2: Separate PUCCH Configs for different symbol types

To achieve better performance, Option 2 is preferred.

Proposal 16: For the following UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD symbols or all non-SBFD symbols), for SBFD-aware UEs, support Option 2 that the transmissions/receptions can be in SBFD symbols and non-SBFD symbols in different slots:

- PDSCH/PUSCH/PUCCH repetitions

- SPS PDSCH/CG PUSCH

- TBoMS

- Multi-PUSCH/PDSCH scheduled by a single DCI

- Periodic/semi-persistent SRS/CSI-RS/PUCCH

- PDCCH

In the above description, basically two solutions are identified to address differences between two symbol types for both UL and DL, and discussion on UL can be prioritized:

- Solution 1: introduction of frequency offset between SBFD symbols and non-SBFD symbols to simply address only the difference of available DL/UL frequency resources within active DL/UL BWP between SBFD symbols and non-SBFD symbols.

- Solution 2: separate configurations/indications of concerned parameters for SBFD symbols and non-SBFD symbols to address potentially all kinds of differences between SBFD symbols and non-SBFD symbols.

Obviously, Solution 2 can afford more flexibility, at the cost of potentially more signaling overhead. Based on Solution 2, the following description is captured in TR 38.858 [2].

…

For SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, it may be beneficial to have separate resources, FH parameters, UL power control parameters and/or beam/spatial relation.

…

It should be noted that these two solutions are also included or adopted in identified frequency resource allocation options to deal with different available DL/UL resources within active DL/UL BWP between SBFD symbols and non-SBFD symbols, which is discussed in details in the following section.

Proposal 17: To address differences between SBFD symbols and non-SBFD symbols for both UL and DL, the following two solutions can be considered:

- Solution 1: introduction of frequency offset between SBFD symbols and non-SBFD symbols to simply address only the difference of available DL/UL frequency resources within active DL/UL BWP between SBFD symbols and non-SBFD symbols, e.g. frequency resources in the UL subband and those in the active UL BWP.

- Solution 2: separate configurations/indications of concerned parameters for SBFD symbols and non-SBFD symbols to address potentially all kinds of differences between SBFD symbols and non-SBFD symbols, e.g. separate resources, FH parameters, UL power control parameters and/or beam/spatial relation.

- Frequency resource allocation

When the transmissions/receptions can be in SBFD symbols and non-SBFD symbols in different slots, several options for frequency resource allocation were identified during RAN1#112bis-e meeting, listed in TR 38.858 [2] as below.

…

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE are studied.

-	Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots.

-	Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots

-	Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication

-	Option 1-3: single FDRA configuration/indication and RB offset(s)

-	Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals.

-	Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed.

Note: Different options can be studied for different signals/channels.

…

As per Rel-15/16/17/18 specification, CSI-RS, SRS and PUCCH are configured in the form/granularity of resources, i.e. NZP-CSI-RS-Resource, SRS-Resource and PUCCH-Resource, respectively, where all related parameters including occupied time-frequency resources are configured by RRC signaling. CSI-RS/SRS/PUCCH resources can be further organized in the form/granularity of resource sets, i.e. NZP-CSI-RS-ResourceSet, SRS-ResourceSet and PUCCH-ResourceSet, respectively. L1 signaling is used to only select/trigger SRS/PUCCH transmission(s) or CSI-RS reception(s) corresponding to one or more resources or resource sets.

In contrast, the case for PDSCH/PUSCH is more flexible and complicated to support efficient and diverse usage of shared resources. For example, the time-frequency resources of PUSCH transmissions or PDSCH receptions, as well as other parameters that may be changed dynamically e.g. according to state of wireless channel, traffic data pending for delivery, etc., are indicated by L1 signaling, except that for CG Type 1.

For Option 1, to determine separate frequency resources for SBFD symbols and non-SBFD symbols, a simple way is to apply a frequency offset between SBFD symbols and non-SBFD symbols, i.e. Option 1-3. For example, a frequency offset can be configured by RRC signaling or indicated by a DCI if applicable, and the frequency offset will be applied between two neighboring transmissions corresponding to a same original time-frequency resource allocation before frequency offsetting, of which one is within SBFD symbols and the other is within non-SBFD symbols. As a result, all transmissions are confined within available frequency domain resources of corresponding symbol types. An example for PUCCH transmission with repetitions across SBFD symbols and non-SBFD symbols is illustrated in Figure 1.

If the number and location of frequency resources may be different for SBFD symbols and non-SBFD symbols, a more flexible way is to separately configure/indicate frequency resources for SBFD symbols and non-SBFD symbols, i.e. Option 1-1. Taking PUCCH as an example, in Figure 2 different PUCCH Configs are applied to different symbol types, i.e., a separate PUCCH Config is introduced for SBFD symbols, besides the existing PUCCH Config for legacy UL symbols. Each PUCCH Config can separately contain parameters better matching characteristics of the corresponding symbol type, e.g., time-frequency resources for each PUCCH resource of each PUCCH Config can be configured independently as required.

For Option 1-2, separate frequency resources are derived for SBFD symbols and non-SBFD symbols, respectively, based on single FDRA configuration/indication. One candidate solution for derivation of separate frequency resources is that different reference points in frequency domain are assumed for SBFD symbols and non-SBFD symbols, respectively, e.g. the lower boundary for DL/UL subband are assumed for SBFD symbols, while the lower boundary for active DL/UL BWP are assumed for non-SBFD symbols. Another candidate solution may be that for SBFD symbols, the valid frequency resources are those chosen from configured/indicated frequency resources and not overlapping with UL/DL subband(s) and/or guardband(s) if any for DL receptions/UL transmissions.

In our understanding, Option 1, including Option 1-1/1-2/1-3, is applicable to PDSCH, CSI-RS, PUSCH, PUCCH, SRS.

For Option 2, it is more suitable for DL receptions, including PDSCH and CSI-RS. For example, for PDSCH, rate matching mechanisms in Rel-15/16/17/18 specification can be extended to deal with RBs outside DL subband(s). For CSI-RS, puncturing can be adopted so that valid frequency resources for a CSI-RS resource can be derived by excluding frequency resources outside DL subband(s), similar to Option 2-2 for frequency resource allocation for CSI-RS across non-contiguous DL subbands.

For Option 3, it is more suitable for UL transmissions, including PUSCH, PUCCH and SRS. For example, for PUCCH repetition, TBoMS, or PUSCH repetition type A with available slot counting enabled, it is natural to adopt postponing by extending existing mechanism. For PUSCH repetition type A without available slot counting, dropping can be adopted since it is more consistent with legacy operation.

Proposal 18: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD symbols or all non-SBFD symbols), discuss and determine applicability of each frequency resource allocation option identified during the SI phase, and the following can be considered:

- Option 1, i.e. separate FDRA determination for SBFD slots and non-SBFD slots, is applicable to PDSCH, CSI-RS, PUSCH, PUCCH and SRS.

- Option 2, i.e. perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals, is at least applicable to DL receptions, including PDSCH and CSI-RS.

- Option3, i.e. a DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed, is at least applicable to UL transmissions, including PUSCH, PUCCH and SRS.

- CSI reporting

The issue on CSI reporting associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols was discussed for several meetings during the SI phase. Based on the discussion, it was well understood that in many SBFD scenarios, separate CSI information for SBFD symbols and non-SBFD symbols is beneficial, since antenna/RF settings, as well as involved interferences, etc., may be different between SBFD symbols and non-SBFD symbols.

  - Observation 1: In many SBFD scenarios, separate CSI information for SBFD symbols and non-SBFD symbols is beneficial, since antenna/RF settings, as well as involved interferences, etc., may be different between SBFD symbols and non-SBFD symbols.

In addition, based on discussion during the SI phase, the following description is captured in TR 38.858 [2].

…

For SBFD-aware UEs, the following options are studied for CSI report associated with periodic/semi-persistent CSI-RS, in case the periodicity is such that CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

-	Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols

-	Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only;

-	Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only.

-	Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols

-	Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively.

-	Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances.

Note that whether the CSI-RS resource can be used for SBFD and non-SBFD symbols may depend on, e.g., gNB implementation of same/different antenna configuration in both symbols.

Option 1-1 can be supported according to existing specification by gNB configuration of appropriate periodicities to ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. But it may restrict the gNB configuration flexibility and enhancements can be considered by additional indication or rules to determine the CSI-RS is valid within one symbol type and is invalid in the other symbol type.

Option 2-2 can be supported according to existing specification to configure measurement restriction so that UE would not average CSI measurements across SBFD and non-SBFD symbols.

…

Based on the above description, Option 1-1 and Option 2-2 can be supported according to existing specification, while Option 1-2 or Option 2-1 requires more specification efforts. Therefore, from our perspective, Option 1-1 and Option 2-2 should be prioritized.

Proposal 19: For SBFD-aware UEs and for CSI reporting associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD symbols or all non-SBFD symbols), Option 1-1 (two CSI-ReportConfigs, each associated with respective CSI-RS) and Option 2-2 (one CSI-ReportConfig associated with one CSI-RS) are prioritized.

For Option 1-1, separate CSI-ReportConfigs can be configured for SBFD symbols and non-SBFD symbols, respectively, and for each CSI-ReportConfig, the associated CSI-RS resources configured for channel and/or interference measurement are confined or valid only in the symbols corresponding to the CSI-ReportConfig. Furthermore, other parameters in CSI-ReportConfigs are also configured independently, such as reportQuantity, cqi-FormatIndicator, pmi-FormatIndicator, csi-ReportingBand, subbandSize, etc., to better match available frequency resources as well as antenna settings for corresponding symbol types. However, it may complicate UE’s implementation, as the UE should maintain more concurrent CSI-ReportConfigs compared to legacy deployment scenarios. In addition, since CSI measurement for a CSI-ReportConfig is only performed based on CSI-RS resources within symbols corresponding to the CSI-ReportConfig, larger measurement delay may be involved potentially.

In contrast, for Option 2-2, a same CSI-ReportConfig is used, with a single set of parameters commonly applied to SBFD symbols and non-SBFD symbols. In this regard, the signaling for CSI-ReportConfig, as well as the number of concurrent CSI-ReportConfigs to be maintained can be kept the same as those in existing specification as much as possible. However, since at least the available frequency resources are different for different symbol types, if NZP CSI-RS/CSI-IM resources associated to the CSI-ReportConfig are confined within only one DL subband in SBFD symbols, it will be too restrictive for non-SBFD symbols. On the contrary, if NZP CSI-RS/CSI-IM resources associated to the CSI-ReportConfig can span more than one subband, e.g. to facilitate frequency-selective scheduling across the whole active DL BWP for non-SBFD symbols, obviously frequent collisions between NZP CSI-RS/CSI-IM resources and UL subband/guardband(s) will be expected. To deal with these cases, it is feasible that, when performing CSI reporting, the UE can handle collision between NZP CSI-RS/CSI-IM resources and UL subband/guardband(s) based on predefined rules. For example, Option 2-2 for frequency resource allocation for CSI-RS across downlink subbands can be adopted to derive valid and non-contiguous CSI-RS resource based on a contiguous CSI-RS resource allocation, by excluding frequency resources outside DL subband(s), as discussed in section 4.1.1.

Furthermore, when Option 2-2 is adopted, for subband CSI reporting, in a CSI report the UE can skip the subband CSI corresponding to a CSI subband which is totally overlapped with UL subband/guardband(s). For a CSI subband overlapped with SBFD subband boundaries, it was agreed during the SI phase that subband CSI corresponding to the CSI subband is not skipped, and derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s). Similarly, for wideband CSI reporting, in a CSI report the UE may derive wideband CSI based on only valid CSI subband(s) for each of which a corresponding subband CSI is not skipped.

Based on the above analysis, some issues can be identified for Option 1-1 and Option 2-2, respectively.

Proposal 20: For SBFD-aware UEs and for CSI reporting associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD symbols or all non-SBFD symbols), discuss the details including the impact on UE CSI processing and reporting for Option 1-1 (two CSI-ReportConfigs, each associated with respective CSI-RS) and Option 2-2 (one CSI-ReportConfig associated with one CSI-RS).

### 4.2.2. Case II: A transmission/reception occasion overlaps with both SBFD symbols and non-SBFD symbols in the same slot

Regarding Case II, it is related to the discussion on whether or not a slot can consist of both SBFD symbols and non-SBFD symbols, which was concluded in RAN1#116 meeting with the agreement that a slot can consist of SBFD symbols and non-SBFD symbols.

Based on discussion during the SI phase, the following description is captured in TR 38.858 [2].

…

RAN1 studied whether the transmission/reception occasion of a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot for a UE, and whether a UE can transmit/receive in the occasion mapped to SBFD symbols and non-SBFD symbols including:

-	Use-case(s) including the locations and number of transition points of the SBFD and non-SBFD symbols in the slot.

-	Potential benefits if any

-	Phase continuity

-	Potential interruption of transmissions/receptions during transition

-	Required guard time if any

-	Potential impact on performance

-	Impact on link adaptation, channel estimation, and other procedures

-	UL transmission timing if any

-	Implementation complexity

-	Applicability for SBFD aware UE and non-SBFD aware UEs

-	NOTE: There are more than one scenario where a transmission overlaps SBFD and non-SBFD symbols and some may or may not face the aspects listed above

-	NOTE: This study doesn’t mean RAN1 agreement on a slot consisting of SBFD and non-SBFD symbols.

For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot if any, the following options for UE transmission/reception can be considered in the normative stage.

-	Option 1: UE does not transmit or receive the physical channel/signal within the slot.

-	Option 2: UE can transmit or receive the physical channel/signal within the slot only under certain conditions.

-	The conditions may depend on at least the following: whether or not phase continuity can be maintained across SBFD and non-SBFD symbols, whether or not there are same or different transmission/reception parameters e.g. power control, spatial/QCL, UL timing etc. applied in SBFD and non-SBFD symbols, and whether or not there is a guard period between the SBFD and non-SBFD symbols, etc.

-	Other options are not precluded.

…

Firstly, it is noted that Case II is not included in the WID [1] explicitly. Therefore, RAN1 should clarify whether Case II should be specified in Rel-19 or not.

Proposal 21: RAN1 clarifies whether the case where a transmission/reception occasion overlaps with both SBFD symbols and non-SBFD symbols in the same slot should be specified in Rel-19 or not.

Basically, not allowing a transmission/reception occasion to be mapped to both SBFD symbol(s) and non-SBFD symbol(s) can lead to simpler UE behaviours, but with some loss in flexibility.

In our understanding, for dynamically scheduled transmission/reception occasions, they can be allowed to be mapped to both SBFD symbol(s) and non-SBFD symbol(s) in the same slot, and the UE performs corresponding transmissions/receptions accordingly, since it is fully under gNB’s control. To avoid excessive complexity, in this case, the transmission/reception parameters such as frequency domain resources, powers, spatial relation/QCL assumptions should be the same across SBFD symbols and non-SBFD symbols. It should be noted that there are scenarios where the gNB does not need to switch between different antenna/RF settings, or change the connections between Tx/Rx chains and antennal panel groups, or switch between different sets of configurations, when switching from SBFD symbols to non-SBFD symbols, or vice versa, and it is up to gNB’s implementation. Therefore, at least for these scenarios, no issue in terms of phase continuity, interruption, guard time, etc. can be observed.

For semi-statically configured transmissions/reception occasions, flexibility is not so critical, therefore it can be considered separately. To avoid potential complexity, it is preferred to not allow a transmission/reception to be mapped to both SBFD symbol(s) and non-SBFD symbol(s) in the same slot, unless consistent configuration across all symbols mapped by the transmission/reception can be guaranteed, in terms of frequency domain resources, antenna/RF settings, power configurations, etc. In addition, since it may be difficult to avoid all such mapping cases by gNB’s configuration, different from dynamically scheduled transmission/reception occasions, when a semi-statically configured transmission/reception occasion is mapped to both SBFD symbol(s) and non-SBFD symbol(s) in the same slot, and consistent configuration across SBFD symbols and non-SBFD symbols cannot be guaranteed, the UE can simply drop the occasion, i.e., do not transmit/receive in the occasion.

Proposal 22: A dynamically scheduled transmission/reception occasion is allowed to be mapped to both SBFD symbol(s) and non-SBFD symbol(s) in the same slot.

- The UE expects transmission/reception parameters in terms of frequency domain resources, antenna/RF settings, spatial relation/QCL assumptions, powers, etc. are the same across SBFD symbols and non-SBFD symbols, and transmits/receives in the occasion.

Proposal 23: A semi-statically configured transmission/reception occasion can be mapped to both SBFD symbol(s) and non-SBFD symbol(s) in the same slot.

- If consistent configuration (in terms of frequency domain resources, antenna/RF settings, power configurations, etc.) across SBFD symbols and non-SBFD symbols are guaranteed, the UE transmits/receives in the occasion;

- Otherwise, the UE drops/postpones the occasion.

  - Collision handling in SBFD symbols

Regarding collision handling in SBFD symbols, the following agreement was achieved during RAN1#116 meeting, mainly identifying potential collision cases.

Agreement:

For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary:

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 6: Dynamic or semi-static DL vs. valid RO

Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

In our understanding, if link direction is indicated explicitly by the gNB for a SBFD symbol, i.e., Option 2 discussed in section 3.2 is adopted, no collision handling is needed, since even if both UL transmission(s) and DL reception(s) are configured/scheduled in the SBFD symbol, the UE shall only perform UL transmission(s) or DL reception(s) according to the indicated link direction, e.g., if the link direction is indicated as UL, the UE shall only perform UL transmission(s) in the SBFD symbol, and DL reception(s) in the SBFD symbol will be dropped.

On the contrary, if link direction is not indicated explicitly by the gNB for a SBFD symbol, it is beneficial to discuss prioritization rule(s) for collision handling, and for Case 1~4 in the above agreement, at least the following can be considered:

- Scheduled transmission(s)/reception(s) is prioritized compared to configured transmission(s)/reception(s).

- Transmission(s)/reception(s) scheduled later is prioritized compared to transmission(s)/reception(s) scheduled earlier.

- Transmission(s)/reception(s) with higher PHY priority is prioritized compared to that with lower PHY priority.

- A given link direction is always prioritized compared to the other link direction, and the given link direction may be determined in specification or by high-layer signaling, e.g. UL.

Proposal 24: Regarding prioritization rule(s) for collision handling, the following can be considered:

- Scheduled transmission(s)/reception(s) is prioritized compared to configured transmission(s)/reception(s).

- Transmission(s)/reception(s) scheduled later is prioritized compared to transmission(s)/reception(s) scheduled earlier.

- Transmission(s)/reception(s) with higher PHY priority is prioritized compared to that with lower PHY priority.

- A given link direction is always prioritized compared to the other link direction, and the given link direction may be determined in specification or by high-layer signaling, e.g. UL.

For Case 5 in the above agreement, it was agreed during the SI phase that an UL subband can be configured in an SSB symbol. Consequently, some issues can be identified when configuring the UL subband within an SSB symbol.

The first issue is whether an SBFD-aware UE is allowed to transmit within the UL subband configured in an SSB symbol or not.

On one hand, SSB detection/measurement is involved in many cases/functions/procedures, such as initial access, beam management, radio link monitoring, mobility related measurement and reporting, etc. Besides, SSB detection/measurement is performed by all UEs in the network, including legacy UEs, SBFD aware UEs, etc. Therefore, SSB performance should be protected as much as possible, especially for legacy UEs. On the other hand, if any transmission is not allowed within the UL subband configured in an SSB symbol, it may introduce excessive restriction on SBFD operation, and UL performance may be degraded.

Based on the above analysis, it is slightly preferred that UL transmissions that fulfilling specific condition(s) can be allowed within the UL subband configured in an SSB symbol, e.g. including UL transmissions with transmit powers below a predefined threshold, to avoid or mitigate potential impact on SSB performance. We are open to discuss details about the allowed UL transmissions within the UL subband configured in an SSB symbol.

Another issue is how to determine SSB symbols for a TDD carrier. For example, whether both CD-SSB and NCD-SSB, if configured, are considered to determine SSB symbols? Furthermore, if an SSB for the TDD carrier is not overlapped with, or not completely contained in the active DL BWP, whether the SSB is considered when determining SSB symbols?

In addition, since UL transmissions may be performed in the UL subband configured in an SSB symbol, it is beneficial to discuss the issue whether the UL subband is allowed to overlap with SSB(s) in the SSB symbol. For simplicity, it is straightforward that the UL subband should not overlap with SSB(s) in the SSB symbol. Therefore, all frequency resources within the UL subband in the SSB symbol can be usable if specific condition(s) is fulfilled.

Proposal 25: UL transmissions that fulfilling specific condition(s) are allowed within the UL subband configured in an SSB symbol.

- The UL subband does not overlap with SSB(s) in the SSB symbol.

- FFS: specific condition(s).

- FFS: determination of SSB symbols.

- Conclusion

In this contribution, SBFD related operations are discussed in details, including semi-static configuration of SBFD subbands, transmission and reception behaviours on SBFD subbands, and potential enhancements for SBFD operation, with the following observation and proposals:

  - Observation 1: In many SBFD scenarios, separate CSI information for SBFD symbols and non-SBFD symbols is beneficial, since antenna/RF settings, as well as involved interferences, etc., may be different between SBFD symbols and non-SBFD symbols.

Proposal 1: Confirm the working assumption that for SBFD aware UEs in RRC_CONNECTED mode, cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

Proposal 2: For semi-static configuration of SBFD subband time locations within a period, at least when only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

Proposal 3: For semi-static configuration of SBFD subband time locations within a period, when two TDD-UL-DL patterns are configured, the period is the sum of two periods corresponding to two configured TDD-UL-DL patterns, respectively.

Proposal 4: For semi-static configuration of SBFD subband frequency locations in SBFD symbols:

- Frequency location(s) of DL subband(s) or the number of RBs for guardband(s) is explicitly configured.

- Guardband(s)/DL subband(s) is implicitly derived as the RBs which are not within UL subband and DL subband(s)/guardband(s).

Proposal 5: To determine reference SCS(s) for configuration of SBFD subband frequency locations, the following options can be considered:

- Option 1: Single reference SCS is assumed.

- Option 1-1: referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as the reference SCS for frequency locations.

- Option 1-2: The reference SCS for frequency locations is configured separately from that for time locations.

- Option 2: Multiple reference SCSs are allowed, e.g., using subcarrierSpacing in SCS-SpecificCarriers configured in scs-SpecificCarrierList in FrequencyInfoUL-SIB or FrequencyInfoUL.

Proposal 6: To determine reference starting RB(s) for configuration of SBFD subband frequency locations, the following options can be considered:

- Option 1: Point A is used as the reference starting RB.

- Option 2: Lowest usable RB on a TDD carrier is used as the reference starting RB corresponding to a reference SCS.

Proposal 7: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

Proposal 8: For SBFD-aware UE transmission and reception in an SBFD symbol, link direction is indicated by the gNB explicitly, or the UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

Proposal 9: For SBFD aware UE behaviours with interaction with TDD-UL-DL-ConfigDedicated in SBFD symbols indicated as flexible in TDD-UL-DL-ConfigCommon, the following alternatives can be considered:

- Alt. 1: The UE does not expect to be provided with TDD-UL-DL-ConfigDedicated and configured with SBFD symbols simultaneously for a TDD carrier.

- Alt. 2: TDD-UL-DL-ConfigDedicated shall only indicate link direction for non-SBFD symbols.

- Alt. 3: TDD-UL-DL-ConfigDedicated may indicate direction for SBFD symbols.

- Alt. 3-1: TDD-UL-DL-ConfigDedicated shall indicate a same direction for a SBFD symbol as that indicated for the SBFD symbol by TDD-UL-DL-ConfigCommon.

- Alt. 3-2: The indication of TDD-UL-DL-ConfigDedicated for a SBFD symbol is invalid or inapplicable.

- Alt. 3-3: TDD-UL-DL-ConfigDedicated indicates the usable SBFD subband(s) for a SBFD symbol.

Proposal 10: For SBFD aware UE behaviours in SBFD symbols with interaction with dynamic SFI, the following alternatives can be considered:

- Alt. 1: The UE does not expect to be configured to monitor dynamic SFI and configured with SBFD symbols simultaneously for a TDD carrier.

- Alt. 2: Dynamic SFI, if detected, shall only indicate link direction for non-SBFD symbols.

- Alt. 3: Dynamic SFI, if detected, may indicate direction for SBFD symbols.

- Alt. 3-1: A detected dynamic SFI shall indicate a same direction for a SBFD symbol as that indicated for the SBFD symbol by TDD-UL-DL-ConfigCommon and/or TDD-UL-DL-ConfigDedicated.

- Alt. 3-2: The indication of the detected dynamic SFI for a SBFD symbol is invalid or inapplicable.

- Alt. 3-3: The detected dynamic SFI indicates the usable SBFD subband(s) for a SBFD symbol.

- FFS: UE behavior in a case if the UE is configured to monitor dynamic SFI, but does not detect a dynamic SFI for a SBFD symbol.

Proposal 11: For a PDSCH with RA type 1, rate matching is performed for the PDSCH, including its DM-RS, around the UL subband and guardband(s) if any automatically, when allocated frequency resources for the PDSCH overlap with the UL subband and guardband(s) if any.

Proposal 12: If PRG is determined as wideband for a PDSCH, support Option1, i.e., non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated for the PDSCH.

Proposal 13: If PRG is determined as wideband for a PDSCH, and non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband are allocated for the PDSCH, frequency resources within each DL subband share the same precoding assumption, and precoding assumptions for frequency resources in the two DL subbands can be different.

Proposal 14: For frequency resource allocation for CSI-RS across DL subbands for SBFD-aware UEs, support Option 2-2, i.e., one CSI-RS resource is configured, where one contiguous CSI-RS resource allocation for the CSI-RS resource is indicated, with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband(s).

Proposal 15: Partial PRG with size 2 or 4 at DL subband boundaries can be supported, or avoided by gNB implementation.

Proposal 16: For the following UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD symbols or all non-SBFD symbols), for SBFD-aware UEs, support Option 2 that the transmissions/receptions can be in SBFD symbols and non-SBFD symbols in different slots:

- PDSCH/PUSCH/PUCCH repetitions

- SPS PDSCH/CG PUSCH

- TBoMS

- Multi-PUSCH/PDSCH scheduled by a single DCI

- Periodic/semi-persistent SRS/CSI-RS/PUCCH

- PDCCH

Proposal 17: To address differences between SBFD symbols and non-SBFD symbols for both UL and DL, the following two solutions can be considered:

- Solution 1: introduction of frequency offset between SBFD symbols and non-SBFD symbols to simply address only the difference of available DL/UL frequency resources within active DL/UL BWP between SBFD symbols and non-SBFD symbols, e.g. frequency resources in the UL subband and those in the active UL BWP.

- Solution 2: separate configurations/indications of concerned parameters for SBFD symbols and non-SBFD symbols to address potentially all kinds of differences between SBFD symbols and non-SBFD symbols, e.g. separate resources, FH parameters, UL power control parameters and/or beam/spatial relation.

Proposal 18: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD symbols or all non-SBFD symbols), discuss and determine applicability of each frequency resource allocation option identified during the SI phase, and the following can be considered:

- Option 1, i.e. separate FDRA determination for SBFD slots and non-SBFD slots, is applicable to PDSCH, CSI-RS, PUSCH, PUCCH and SRS.

- Option 2, i.e. perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals, is at least applicable to DL receptions, including PDSCH and CSI-RS.

- Option3, i.e. a DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed, is at least applicable to UL transmissions, including PUSCH, PUCCH and SRS.

Proposal 19: For SBFD-aware UEs and for CSI reporting associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD symbols or all non-SBFD symbols), Option 1-1 (two CSI-ReportConfigs, each associated with respective CSI-RS) and Option 2-2 (one CSI-ReportConfig associated with one CSI-RS) are prioritized.

Proposal 20: For SBFD-aware UEs and for CSI reporting associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD symbols or all non-SBFD symbols), discuss the details including the impact on UE CSI processing and reporting for Option 1-1 (two CSI-ReportConfigs, each associated with respective CSI-RS) and Option 2-2 (one CSI-ReportConfig associated with one CSI-RS).

Proposal 21: RAN1 clarifies whether the case where a transmission/reception occasion overlaps with both SBFD symbols and non-SBFD symbols in the same slot should be specified in Rel-19 or not.

Proposal 22: A dynamically scheduled transmission/reception occasion is allowed to be mapped to both SBFD symbol(s) and non-SBFD symbol(s) in the same slot.

- The UE expects transmission/reception parameters in terms of frequency domain resources, antenna/RF settings, spatial relation/QCL assumptions, powers, etc. are the same across SBFD symbols and non-SBFD symbols, and transmits/receives in the occasion.

Proposal 23: A semi-statically configured transmission/reception occasion can be mapped to both SBFD symbol(s) and non-SBFD symbol(s) in the same slot.

- If consistent configuration (in terms of frequency domain resources, antenna/RF settings, power configurations, etc.) across SBFD symbols and non-SBFD symbols are guaranteed, the UE transmits/receives in the occasion;

- Otherwise, the UE drops/postpones the occasion.

Proposal 24: Regarding prioritization rule(s) for collision handling, the following can be considered:

- Scheduled transmission(s)/reception(s) is prioritized compared to configured transmission(s)/reception(s).

- Transmission(s)/reception(s) scheduled later is prioritized compared to transmission(s)/reception(s) scheduled earlier.

- Transmission(s)/reception(s) with higher PHY priority is prioritized compared to that with lower PHY priority.

- A given link direction is always prioritized compared to the other link direction, and the given link direction may be determined in specification or by high-layer signaling, e.g. UL.

Proposal 25: UL transmissions that fulfilling specific condition(s) are allowed within the UL subband configured in an SSB symbol.

- The UL subband does not overlap with SSB(s) in the SSB symbol.

- FFS: specific condition(s).

- FFS: determination of SSB symbols.

# References

- RP-234035, New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), RAN #102, Dec. 11 - 15, 2023.

- TR 38.858, Study on Evolution of NR Duplex Operation, V18.0.0, Dec., 2023.

# Annex

Agreements achieved during RAN1#116 meeting are listed as follows for reference.

- Semi-static configuration of SBFD subbands

- General

Agreement

For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier.

- FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

- Configuration of time location for SBFD subbands

Agreement:

For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options.

- Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- FFS: Further details

FFS: Details when two TDD-UL-DL patterns are configured

Agreement

A slot can consist of SBFD symbols and non-SBFD symbols.

Agreement

A slot can consist of SBFD symbols and non-SBFD symbols.

For semi-static indication of SBFD subband time location,

- When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period.

  - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon

  - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot.

  - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS.

  - FFS details

- Configuration of frequency location for SBFD subbands

Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.

The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

For semi-static indication of SBFD subband frequency location, down-select from the following options.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Agreement

The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

  - Subject to RAN4 guidance on the size of subband/guardband, if any

  - FFS reference starting RB and reference SCS

- Tx/Rx behaviour on SBFD subbands

Agreement

For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs.

For determining UL/DL usable PRBs, consider the following options.

- Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

- Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

Agreement

For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL usable PRBs are allowed

  - FFS SSB symbols

- DL receptions within DL usable PRBs are allowed

- UL transmissions outside UL usable PRBs are not allowed

- DL receptions outside DL usable PRBs are not allowed

  - This restriction is not applicable for CLI measurement

CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3.

RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0

- DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

Agreement

For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Other options are not precluded.

- Collision handling

Agreement:

For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary:

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 6: Dynamic or semi-static DL vs. valid RO

Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.