| 3GPP TSG RAN WG1 Meeting #116-bis | R1-2402539 |
| --- | --- |
| Changsha, Hunan Province, China, April 15th – 19th, 2024 |  |

| Agenda Item: | 9.6.1 |
| --- | --- |
| Source: | EURECOM |
| Title: | Discussion on LP-WUS and LP-SS Design |
| Document for: | Discussion and Decision |

# Introduction

In 3GPP wake-up signals (WUS) are specified in both LTE-M/NB-IoT as well as NR-Rel-17 under the name of paging early indication (PEI). The key difference between those WUS and a low-power (LP)-WUS is that the LP-WUS is received with a low-power wake-up receiver (WUR) independent of the main radio, i.e. the main radio can be turned off.
A SI [1] on low-power (LP) wake-up signal (WUS) and receiver for NR has been carried out in Rel-18 with the report available in [2]. In Rel-19, a WI [3] was agreed including the following objective related to LP-WUS and LP-SS design:

Objectives:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

This contribution focuses on LP-WUS design in RRC IDLE mode.

# OOK Waveform Designs

Regarding the LP-WUS waveform design the following has been agreed in RAN#116:

Agreement:

Support both OOK-1 and OOK-4 for LP-WUS.

- FFS how OOK-1 and OOK-4 are specified

- For OOK-4, M<=4, FFS supported values

- The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol

  - FFS different SCS.

In this section, we review the OOK modulation and the 2 agreed OOK schemes, OOK-1 and OOK-4.

We denote N PRBs as the bandwidth of the LP-WUS including potential guard bands (GB). The actual number of used PRBs for the WUS and GB are referred to as NWUS and NGB, respectively, such that N=NWUS+NGB.

Moreover, we refer to the ON-signal a as the sequence of complex symbols allocated to the OOK symbol if the corresponding bit is one.

The overall DL transmission block diagram is depicted in Figure 1. The B information bits b=[b0,b1,…,bB-1] are encoded and the resulting C coded bits c=[c0,c1,…,cC-1] are modulated onto L consecutive OFDM symbols each carrying M bits. That is, M is the number of coded bits per OFDM symbol or the number of OOK symbols. Subsequently, the signal in frequency-domain Sm of message m=0,1,…,2B-1 is mapped to the overall resources Xm of N sub-carriers and OFDM-modulated resulting in the time-domain signal xm(t).

Figure 1: Overall transmission block-diagram per OFDM symbol.

Subsequently, we review OOK modulation as well as the specific OOK-1 and OOK-4 modulations in the WI objectives.

## OOK Modulation

Denote the OOK modulated signal sm in time-domain for message m=0,1,…,2C-1 as

sm=[s0,s1,…,sC-1]

where C is the number of coded bits transmitted per OFDM symbol l and si=[si0,si1,…,siNB'-1] with NB'=N'/C the number of samples for sequence si, i=0,1,…,C-1. The OOK modulation consists of mapping an ON-signal or ON-sequence a=[a0,a1,…,aNB'-1] to si whenever the corresponding bit is one, i.e.

si=0,  &ci=0a,  &ci=1

Hereby, the ON-signal a can be any complex sequence and can be different for every si.

In case of multiple ON-sequence, consider Q ON-sequences aq able to encode B2= log2Q bits.

## OOK-1: Single-bit in 1 OFDM symbol

The simplest OOK scheme allocates the ON-signal a of length NWUS to the corresponding WUS SCs if the bit is one and zeros otherwise (in baseband).

A block-diagram is shown in Figure 2.

Figure 2: Block-diagram for OOK-1.

This transmission scheme can only transmit a single bit per OFDM symbol. Thus, the only means to increase the data rate is to shorten the OFDM symbol length by increasing the sub-carrier spacing (SCS).

## OOK-4: Transform M-bit OOK in time domain

This scheme uses DFT-precoding to convert M OOK symbols in time-domain to frequency domain for allocation to NWUS PRBs.

An example block-diagram is depicted in Figure 3. From the M (coded or physical) bits, a signal of length N' is generated where each bit is mapped to a sequence si of length NB', the sequence is 0 if the corresponding bit is zero and si=a otherwise. The resulting sequence is DFT precoded and the output is mapped to the overall transmission bandwidth.

Figure 3: OOK-4, Block-diagram with M=4.

## Discussion

One of the open aspects is how OOK-1 and OOK-4 are to be specified. It is desirable that the WUS generation of OOK-1 and OOK-4 follow the same procedure. Thus, we are in favor of specifying the WUS in time-domain because it is simpler for OOK-4, and OOK-1 can be viewed as a special case of OOK-4 with M=1.

Proposal 1: Specify LP-WUS in time-domain.

Regarding the possible values for M, i.e. the number of OOK symbols per OFDM symbol, we think that M=1,2,4, should be supported with M=1 being OOK-1.

Proposal 2: Support M=1,2,4 with M=1 being OOK-1.

# WUS Encoding

In this section, we discuss how the WUS payload is transmitted. The objectives of the WI state

Objectives:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

It is our understanding, that 2 distinct types of receivers are considered:

- Energy detector (ED) to receive the OOK waveform

- Sequence detector (Correlator) to receive the overlaid OFDM sequences

The ED detector (referred to as ED-WUR), does not require coherent detection (no phase information) of the received signal and only operates on the amplitude of the received signal. A sequence detector (referred to as COR-WUR) is able to carry out correlations, either in time or in frequency-domain, to decode the signal which requires coherent demodulation (i.e. phase information). Consequently, the ED-WUR is significantly simpler than the COR-WUR at the expense of inferior performance.

It is assumed that the base-station knows the supported WUR receiver type(s) of the UE via RRC signaling upon connection establishment. Moreover, we suppose that similar UE grouping techniques are used as for PEI or LTE-M/NB-IoT WUS, i.e. up to 8 groups can be signaled independently. Higher layers ensure that the grouping minimizes the false paging probability, that is, the likelihood that an entire group is woken up if only a single UE is paged regularly. For instance, UEs that are paged together periodically would be assigned to the same group. Note, that each group may contain UEs of either COR-WUR or ED-WUR.

Observation 1: The network knows the WUR-type(s) of the UEs.

## OOK Encoding

The study on LP-WUS [2] suggests to utilize Manchester Coding, where one input bit is mapped to two coded bits, i.e. 0->{0,1} and 1-> {1,0}. This coding technique is especially adapted to OOK with ED since it does not require threshold detection but a simple energy comparison of the two OOK symbols is sufficient for reliable decoding.

The detection performance of Manchester Coding can be improved by jointly encoding multiple bits. More precisely, B input bits b= b0,b1,…,bB-1 are encoded to C output bits c=c0,c1,…,cC-1 according to

c=2m

where m=0,1,…,2B-1 is the message and c is the decimal representation of codeword c. For B=1, we obtain the conventional Manchester code of rate R=1/2 presented in Table 1.

| Input Bits | Coded Bits |
| --- | --- |
| 0 | 01 |
| 1 | 10 |

Table 1: Encoding for B=1 input bits and C=2 coded bits, rate R = ½.

Encoding B=2 bits into C=4 coded bits is given by Table 2.

| Input Bits | Coded Bits |
| --- | --- |
| 00 | 0001 |
| 01 | 0010 |
| 10 | 0100 |
| 11 | 1000 |

Table 2: Encoding for B=2 input bits and C=4 coded bits, rate R = ½.

We can further decrease code rate to R=3/8 and obtain, cf. Table 3.

| Input Bits | Coded Bits |
| --- | --- |
| 000 | 00000001 |
| 001 | 00000010 |
| 010 | 00000100 |
| 011 | 00001000 |
| 100 | 00010000 |
| 101 | 00100000 |
| 110 | 01000000 |
| 111 | 10000000 |

Table 3: Encoding for B=3 input bits and C=8 coded bits, rate R = 3/8.

It can easily be seen that the rate of the code is given by R=B2B. Hence, increasing B will decrease the code rate.

The advantage of using B=2 compared to B=1, is that double the power can be used for B=2 per codeword because the power has to be divided among 2 codewords in case of B=1. In other words, the available power is concentrated on a single OOK symbol and thus increasing the SINR.

Proposal 3: Consider jointly encoding more than one bit via Manchester Coding.

### Discussion for OOK Encoding for M=4

For M=4 there are two possibilities of Manchester Encoding, (i) the 2 information bits can be encoded separately as shown in Table 4 or (ii) joint encoding as in Table 2.

| Input Bits | Coded Bits |
| --- | --- |
| 00 | 0101 |
| 01 | 0110 |
| 10 | 1001 |
| 11 | 1010 |

Table 4: 𝑀=4, 𝑆𝑒𝑝𝑒𝑟𝑎𝑡𝑒 𝑀𝑎𝑛𝑐ℎ𝑒𝑠𝑡𝑒𝑟 𝑒𝑛𝑐𝑜𝑑𝑖𝑛𝑔 𝑓𝑜𝑟 𝑖𝑛𝑔𝑜𝑟𝑚𝑎𝑡𝑖𝑜𝑛 𝑏𝑖𝑡𝑠.

Both approaches result in a coding rate R=12, however, given the same transmit power of the WUS, the joint approach concentrates the available transmit power in a single OOK symbol instead of two OOK symbols, hence increasing the SNR by 3dB.

It has been pointed out that this joint encoding approach increases the PAPR of the resulting OFDM time-domain waveform which impacts existing base-station implementations, e.g. they may fail RAN4 requirements. Table 5 provides simulation results for PAPR of various transmission schemes and system configurations.

| Scheme | 20MHz, SCS 30, 51PRBs, FFT=1024 | 20MHz, SCS 15, 106 PRBs, FFT=2048 | 10MHz, SCS 15, 52 PRBs, FFT=1024 | 100MHz, SCS 30, 273 PRBs, FFT=4096 | 20MHz, SCS 15, 106 PRBs, FFT=2048, WUS BW=2.5MHz |
| --- | --- | --- | --- | --- | --- |
| 64QAM | 8.74 | 9.13 | 8.74 | 9.49 | 9.13 |
| OOK-1, R=1/2 | 8.63 | 9.01 | 8.17 | 9.49 | 9.1 |
| OOK-4, M=1, R=1/2 | 8.6 | 9 | 8.15 | 9.48 | 9.09 |
| OOK-4, M=2, R=1/2 | 8.68 | 9.06 | 8.31 | 9.49 | 9.12 |
| OOK-4, M=4, R=1/2 | 8.69 | 9.08 | 8.35 | 9.49 | 9.12 |
| OOK-4, M=8, R=1/2 | 8.69 | 9.07 | 8.35 | 9.49 | 9.12 |
| OOK-4, M=4, R=2/4 | 9.1 | 9.45 | 9.27 | 9.51 | 9.26 |
| OOK-4, M=8, R=3/8 | 9.8 | 10.15 | 10.59 | 9.57 | 9.54 |

Table 5: PAPR [dB] simulation results for different encoding schemes and system configurations.

Unless stated otherwise, the simulation assumption in Table 6 are used. The transmission power of the WUS per OFDM symbol is kept constant. For an ON-sequence a=[a0a1…aNB'-1] of length NB', a cyclically extended Zadoff-Chu sequence is used which is generate as

an= e-jπun(n+1)NZC

where n=0,1,…,NB'-1 and NZC=NP(NB') with NP(x) denoting the next prime greater or equal to x. In the simulations we use root u=1.

The baseline “64QAM” assumes random 64-QAM symbols on all sub-carriers, i.e. no WUS. It can be observed that the impact on PAPR increases with larger WUS BW w.r.t. to the system BW. Moreover, using OOK-1 or OOK-4 does not increase PAPR w.r.t. to the baseline. Given Manchester coding R=1/2 , there is only a very small increase of PAPR of M=2/4/8 compared to M=1.

For M=4, the PAPR increase caused by joint Manchester Coding (R=2/4) is about 0.4 dB if the WUS BW constitutes about 25% of the overall BW (5MHz out of 20MHz). For a 10MHz channel and 5MHz WUS the PAPR increase is almost 1dB. On the other hand, for a 100MHz channel and 5MHz WUS there is no increase in PAPR. Similarly, for a 20MHz channel with a 2.5MHz WUS the PAPR increase is only 0.1dB.

Observation 2: PAPR increase of joint Manchester coding for M=4 compared to independent Manchester encoding depends on the ratio of channel BW to WUS BW and is minor (~0.1dB) for many system configurations.

Figure 4 shows the performance for M=4 (and for comparison M=2 with 4 bit payload) with independent Manchester coding (R=1/2) and joint Manchester Coding (R=2/4) for a 20MHz BW channel and a WUS BW of 5.04 MHz (24 PRB WUS + 2 PRB GB on each side) and 2.52 MHz (12 PRB WUS + 1 PRB GB on each side). It can be observed that there is a 3dB SNR gain of R=2/4 MC vs. R=1/2 MC in both system configurations. Moreover, in this setting (depending on RX filter design etc.), reducing the WUS BW by half, results in about 3dB SNR loss. The performance of M=2 is about the same as M=4 with R=2/4 but only delivering half the bit rate.

Figure 4: BLER vs. SNR, ED-WUR, 20 MHz (106 PRBs), SCS=15 kHz, M=4, 8 bit payload.

Therefore, with the same performance, a network operator can save 2.52 MHz of BW by configuring a WUS BW of 2.52 MHz with joint Manchester Coding (R=2/4) as opposed to a WUS BW of 5.04 MHz with independent Manchester coding (R=1/2). The difference in PAPR is only about 9.26dB - 9.08dB = 0.18dB which is only 0.13dB above the baseline of 9.13dB.

Alternatively, the operator can use M=2 but with half the bit rate.

Proposal 4: Allow configuration of joint Manchester Encoding for M=4.

The gains apply also to correlation-based detection as shown in the next sections.

## Single Sequence

If a single ON-sequence a is defined, the receiver can carry out correlations to decode the payload. No information is encoded in the sequences but only in the position of the ON-sequence (assuming Manchester coding). The performance of the three types of receivers (cf. Appendix) is shown in Figure 5. As expected, for Manchester Coding R=1/2, there is a ~2dB and ~6dB gain over the ED-WUR for COR-WUR-OOK and COR-WUR, respectively. The larger gain of the COR-WUR is due to the processing gain by carrying out correlations over the entire WUS which requires a more complex receiver.

Applying joint Manchester Coding R=2/4 yields a gain of 3dB, 2.8dB and 1.8dB over R=1/2 for ED-WUR, COR-WUR-OOK and COR-WUR, respectively. The gain for COR-WUR is smaller because the gain does not come from the increased SNR per OOK symbol, but from reduced cross-correlation among the possible transmit signals.

Figure 5: WUR performance in TDL-C with 𝑀=4, 𝐵=8 and single ON-sequence.

Observation 3: Correlation receiver achieves significant gain over energy detection.

Observation 4: For M=4 Joint Manchester Coding achieves significant performance gain for all receiver types.

## Multiple Sequences

The objective states that multiple ON-sequences aq can be defined, with q=1,2,…,Q, and that they can encode the same information b.

In RAN1#116 one issue was if the overlaid OFDM sequences carry only a part or all of the WUS payload of B bits.

- Option 1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK symbols.

- Option 2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s).

To better understand the issue, consider the example in Figure 6, where the payload b=[11011000] is transmitted with M=4 over L=4 OFDM symbols.

Figure 6: Example of OOK-4 WUS, Manchester coding and 8 groups, M=4, B=8 b = [11011000] and Q=2 sequences.

Denote bS the bit sequence carried by the overlaid OFDM sequences. If Q=2 sequences are available, one bit can be carried per ON-symbol. One option (Option A) is to transmit bS=bS0=b, i.e. the same bit sequence that is carried via the OOK modulation. This option would require the COR-WUR to receive all 4 OFDM symbols to decode the entire payload. Another option (Option B) is to transmit bS=bS1=[b4, b5,b6,b7,b0,b1,b2,b3], or any permutation in the first 4 bits and the associated last 4 bits. As with Option A, the COR-WUR needs to receive all OFDM symbols in order to decode the entire payload. However, it (i.e. a per-OOK symbol correlation receiver) can combine the correlation results of the first 2 OFDM symbols with the last 2 OFDM symbols to increase performance. For a COR-WUR that correlates over the entire WUS this mapping effectively reduces cross-correlation and improves performance.

The above options, i.e. Option 1 and 2, only make sense for multiple overlaid sequences. If there is only a single ON-sequence a correlation-based receiver obtains the information by correlating with potential ON-positions. In general, the COR-WUR has to implicitly decode the position of the ON-signal anyway. That is, for Manchester coding, the correlator has to correlate each OOK symbol with all possible overlaid OFDM sequences. In the example in Figure 6 with bS=bS1, the first 2 OFDM symbols carry all information if Option 1 is used and Option 2 is fulfilled for the 4 OFDM symbols.

Since it has been agreed that the same information is transmitted to both receivers, the base station has to transmit all 4 OFDM symbols anyway. Hence it can overlay the OFDM sequences for the entire transmission. On the receiver side, it makes a difference how the OFDM sequences are overlaid. With Option A, the COR-WUR has to receive the whole transmission. With Option B, it can turn off after 2 OFDM symbols or can receive the entire transmission with increased redundancy and performance.

Therefore, we support Option 2 with the following modification:

- Option 2: The overlaid OFDM sequences carry all information bits of LP-WUS.

What exact bit sequence is carried by the overlaid OFDM sequences can be further studied. One option was presented above as Option B and will further detailed in the next section.

Proposal 5: Further discuss how the information bits are mapped to multiple overlaid OFDM sequences.

## Joint OOK and Sequence Encoding

The objective states that the same information b is transmitted irrespectively of the WUR-type, i.e. the information encoded with Q available sequences is part of b.

For better understanding, consider the example in Figure 7, where groups 1,2,4 and 5 are signaled to wake up using an OOK-4 waveform with M=4 and Manchester coding, i.e. 2 groups can be signaled in a signaled OFDM symbol. It can be observed, that each group is encoded in OOK-symbols at a specific time. Thus, the UEs are not required to decode the entire OOK-waveform, but may only decode the portion containing their group ID. If repetition is required, it may be advantageous to repeat a group ID on consecutive OOK symbols so that the ED-WUR may go back to sleep faster and save power.

Figure 7: Example of OOK-4 WUS, with M=4, Manchester coding and 8 groups, b = [11011000]  .

Concerning the COR-WUR, the information is decoded by correlating with a known ON-sequence(s) aq. In case of a single ON-sequence, the information can be decoded by correlating each OOK symbol with the known sequence and comparing the absolute value of adjacent OOK symbol (in case of Manchester coding). With this strategy (COR-WUR-OOK), the OOK waveform of the sequence-based waveform are of the same duration because the same information is transmitted to both receiver types.

If multiple ON-sequences are used to encode information, the question is which groups should be signaled via the sequences because the groups encoded with OOK are fixed.

Figure 8 show a joint encoding with Q=2 sequences, the number in the ON-pulse denotes the group ID that is signaled with sequence encoding. Therefore, the COR-WUR can decode the payload B by only receiving the first 2 OFDM symbols instead of all 4 OFDM symbols. To improve detection performance of the COR-WUR, the payload can be partially repeated on the last 2 OFDM symbols. To allow for combining at the COR-WUR, the same groups have to be encoded in one coded OOK symbol (2 consecutive symbols in this example). More precisely, the first 2 OOK symbols encode group 1 through position of the ON pulse and group 5 through the ON-sequences. When group 5 is signaled with OOK encoding in the first two pulses in the second OFDM symbol, the ON-sequence encodes group ID 1. Hence, the correlation values of both cases can be combined since they signal the same groups.

Figure 8: Example of OOK-4 WUS, Manchester coding and 8 groups, M=4, B=8 b = [11011000] and Q=2 sequences.

Figure 9 shows an example with joint Manchester coding and otherwise the same parameters as in the previous example in Figure 7. It can be observed that the position of the ON-sequence within an OFDM symbol encodes 2 group IDs.

Figure 9: Example of OOK-4 WUS, Manchester coding and 8 groups 𝑀=4, 𝐵=8, b = [11011000], 𝐿=4 𝑂𝐹𝐷𝑀 𝑠𝑦𝑚𝑏𝑜𝑙𝑠 and 4 sequences.

Since the power is concentrated in a single OOK-symbol instead of 2 OOK symbols, there is a 3 dB gain for both ED-WUR and COR-WUR-OOK. However, in case of multiple ON-sequences, less bits can be encoded than in the example in Figure 8 because there are fewer ON-pulses. For instance, 2 ON-sequences can encode 4 bits over the entire OOK-4 waveform, as opposed to 8 bits in Figure 8. However, the power in the ON-pulse is larger which results in an overall performance gain.

Figure 10 provides link-level simulation results for multiple overlaid OFDM sequences in various settings. The results labeled “no encoding” use the same bit sequence for the overlaid OFDM sequences as carried by the OOK waveform, e.g. bS0 in Figure 6. On the other hand, the results labeled “encoding” use a different bit sequence for the overlaid OFDM sequences, e.g. bS1 in Figure 6. It can be observed that COR-WUR achieves a performance gain of ~4.4dB and ~3.3dB over COR-WUR-OOK without encoding and with encoding, respectively, due to the processing gain. For COR-WUR-OOK and COR-WUR encoding yields a 3.3dB and 2dB gain compared to no encoding, respectively.

Figure 10: WUR performance in TDL-C with 𝑀=4, 𝐵=8 and multiple ON-sequences.

With joint Manchester coding R=2/4 and Q=4 sequences, we observe a 2.8dB and 2.4dB gain for COR-WUR-OOK compared to R=1/2 Manchester coding without and with encoding, respectively. In case of COR-WUR those gains are 2dB and 1dB, respectively.

In summary the following observations are made:

Observation 5:

- COR-WUR performs better than COR-WUR-OOK due to the processing gain of carrying out longer correlations.

- Transmitting the same payload as the OOK waveform with the overlaid OFDM sequences but in a different bit sequence yields a significant performance gain.

- Using joint Manchester Coding and increasing the number of sequences results in a significant performance gain

Proposal 6: For multiple ON-Sequences, jointly encode the payload with OOK and sequence encoding.

## WUR-type Dependent Wake-Up

As explained earlier, the grouping criteria will likely result in mixed groups, i.e. groups that contain both ED-WUR and COR-WUR UEs.

If multiple ON-sequences are used to encode information, ED-WUR and COR-WUR can receive different information through OOK encoding and sequence encoding, respectively. For instance, if 2 sequences are available, one group ID can be signaled independently of the underlying OOK encoding.

If a group contains both COR-WUR and ED-WUR and the gNB wants to wake up a UE with an ED-WUR, it can signal 0 to the COR-WUR in that group and 1 to the ED-WUR. This avoids unnecessary wake ups of COR-WUR UEs. However, the WI objective states “…the same information is delivered irrespective of LP-WUR type…”. We suggest to consider the advantages, in terms of reduction of unnecessary paging, of waking up COR-WUR and ED-WUR independently and potentially revise the assumption in the WI objective.

Proposal 7: Consider WUR-type dependent wake-up to reduce unnecessary wake-ups.

## Repetition of Overlaid OFDM sequence

In general, there is no issue in using overlaid OFDM sequence for the entire WUS transmission assuming the OOK waveform detection is not impaired. Hence, repetition should be considered the base line.

In the encoding scheme Option B in Figure 6, repetition is already implicitly considered.

Proposal 8: OFDM sequence(s) are overlaid over the entire WUS duration.

## Sequence Design

In the previous meeting different design aspects have been discussed:

The following aspects should be considered for overlaid OFDM sequence design:

- The sequence should not compromise OOK detection performance.

- The sequence should have good cross-correlation property and/or auto-correlation property.

- The sequence based on existing NR OFDM sequence can be the starting point.

We agree with all of the above and find it reasonable to evaluate existing NR low-PAPR sequences of NR UL. If ZC-sequences are considered, they should be prime-length, e.g. 37 for M=4 and 5MHz WUS BW.

# Conclusion

In this contribution, the following proposals and observations have been made:

Proposal 1: Specify LP-WUS in time-domain.

Observation 1: The network knows the WUR-type(s) of the UEs.

Proposal 2: Support M=1,2,4 with M=1 being OOK-1.

Proposal 3: Consider jointly encoding more than one bit via Manchester Coding.

Observation 2: PAPR increase of joint Manchester coding for M=4 compared to independent Manchester encoding depends on the ratio of channel BW to WUS BW and is minor (~0.1dB) for many system configurations.

Proposal 4: Allow configuration of joint Manchester Encoding for M=4.

Observation 3: Correlation receiver achieves significant gain over energy detection.

Observation 4: For M=4 Joint Manchester Coding achieves significant performance gain for all receiver types.

Proposal 5: Further discuss how the information bits are mapped to multiple overlaid OFDM sequences.

Observation 5:

- COR-WUR performs better than COR-WUR-OOK due to the processing gain of carrying out longer correlations.

- Transmitting the same payload as the OOK waveform with the overlaid OFDM sequences but in a different bit sequence yields a significant performance gain.

- Using joint Manchester Coding and increasing the number of sequences results in a significant performance gain

Proposal 6: For multiple ON-Sequences, jointly encode the payload with OOK and sequence encoding.

Proposal 7: Consider WUR-type dependent wake-up to reduce unnecessary wake-ups.

Proposal 8: OFDM sequence(s) are overlaid over the entire WUS duration.

# References

[1] RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#94e

[2] TR-38.869, “Study on low-power Wake-up Signal and Receiver for NR”, V.18.0.0, 3GPP, 2023

[3] RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC (moderator, RAN VC), RAN#102, Edinburgh, GB, December 11-15, 2023

# Appendix

## Receiver Algorithms

### Envelop/Energy Detection in Base-band

The received signal y(t) is first passed through a band-pass filter to extract the WUS frequencies. Subsequently, y(t) is converted into base-band using a sampling frequency fS,WUS resulting in a sampling rate reduction of r=fS,OFDM/fS,WUS. Therefore, with K denoting the FFT size of the OFDM transmission, we obtain N=K/r WUS samples per OFDM symbol in time-domain at the receiver base-band. Denote yl∈CN the discrete received signal in base-band of OFDM symbol l. With M the number of OOK symbols per OFDM symbol we have yl=[yl,1, yl,2…yl,M] with yl,i∈CNM the NM=N/M samples per OOK symbol. The energy ei,l of OOK symbol i in OFDM symbol l is then given by

ei,l=n=1NMyl,i,n2=yl,iHyl,i

The energy is then compared to a threshold or if Manchester coding is used, the energy values are compared, e.g. for Manchester code R=1/2

bj,l=0,  &if ei,l<ei+1,l1,  &if ei,l≥ei+1,l

Where bj,l is bit j encoded in 2 consecutive OOK symbols i and i+1 of OFDM symbol l.

### Correlation detection in Base-band in Time-Domain

If the ON-sequence a is known the receiver can carry out correlations with all possible transmitted sequences sm=[sm1sm2…smL]∈CNL. After OFDM demodulation and the WUS signal (sub-carriers) are transformed back into time-domain via Inverse-DFT precoding. We assume that channel estimates are not available, hence a we use a RAKE demodulator for square-law combination of orthogonal signals [Proakis, Figure 14-5-7].

#### Correlation per OOK Symbol

If the correlation is carried out per OOK symbol (referred to as COR-WUR-OOK), the resulting time-domain signal yl,i of the ith OOK symbol is correlated with a and the NP peaks of the correlator output are combined to exploit frequency-diversity in fading channels i.e.

ei,l=n=1NPmaxNPyl,i⋆a[n]2

where maxNPfx2 returns the NP largest absolute values squared of f(x) and the linear cross-correlation function f⋆g[n]is defined as

f⋆gn=m=-NNf*[m]g[m+n]

#### Correlation per WUS

If the correlation is carried out over the entire WUS to achieve the largest processing gain (referred to as COR-WUR), the message  m, m=0,1,…,2B-1 , for payload of B bits WUS is decoded as

m=argmaxmn=1NPmaxNPy⋆sm[n]2

where y is the time-domain WUS and sm is the known WUS for message m.

## Simulation Assumptions

Unless otherwise stated, the link-level simulation assumption in Table 6 are used.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2.6 GHz (FDD) |
| Waveform | OOK-4 |
| Channel Structure | Option 3: Payload only (no CRC) |
| SCS | 30 kHz |
| WUS payload | 8 bits |
| Configuration of LP-WUS Signal | OOK-4: M=4 ON-Sequence = Zadoff-Chu Multiple Sequences are obtained via different cyclic shifts |
| WUS Duration | 4 OFDM symbols |
| Code Scheme | Manchester Code R=1/2 |
| Channel BW | 20MHz (51 PRBs @ 30kHz SCS) |
| LP-WUS BW | 5 MHz (14 PRBs = 168 SCs) 148 SCs for WUS (4.44 MHz) + 10 SCs GB on each side |
| Filter | 3rd order Butterworth with 4.32 MHz BW |
| Adjacent Sub-carrier Interference (ACI) | Random 64-QAM symbols with 0dB |
| WUS Sampling Rate | 7.68 MHz |
| ADC bit-width | inf |
| Channel Model | TDL-C, 300ns Delay Spread |
| Timing Error | 0 |
| Frequency Error | 0 |
| Antenna configuration | 1Tx, 1Rx |
| UE speed | 0 km/h |
| Receiver | Energy Detector, Correlation Detector (Energy of 5 highest peaks is combined) |

Table 6: Link-level simulation assumptions.