3GPP TSG RAN WG1 #116bis		R1-2403045

Changsha, China, April 15th – 19th, 2024

Source:	CATT

Title:	Maintenance on NR Network-Controlled Repeaters

Agenda Item:	8.4

Document for:	Discussion and Decision

Introduction

In this contribution, one remaining issue  regarding paired spectrum operation for NCR was discussed.

Discussion on control plane signaling and procedures

Access link beam indication for paired spectrum

In [1], the description of NCR’s operating bands is as follows:

5.2 Operating bands NR repeater is designed to operate in the operating bands in FR1 and FR2-1 defined in TS 38.104 [2] except the operating bands n46, n96 and n102.

In [2], the description of NR operating bands is as follows. The unsupported operating bands for NCR are marked in yellow:

5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. … Table 5.2-1: NR operating bands in FR1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high; Duplex mode. n1; 1920 MHz – 1980 MHz; 2110 MHz – 2170 MHz; FDD. n2; 1850 MHz – 1910 MHz; 1930 MHz – 1990 MHz; FDD. n3; 1710 MHz – 1785 MHz; 1805 MHz – 1880 MHz; FDD. n5; 824 MHz – 849 MHz; 869 MHz – 894 MHz; FDD. n7; 2500 MHz – 2570 MHz; 2620 MHz – 2690 MHz; FDD. n8; 880 MHz – 915 MHz; 925 MHz – 960 MHz; FDD. n12; 699 MHz – 716 MHz; 729 MHz – 746 MHz; FDD. n13; 777 MHz – 787 MHz; 746 MHz – 756 MHz; FDD. n14; 788 MHz – 798 MHz; 758 MHz – 768 MHz; FDD. n18; 815 MHz – 830 MHz; 860 MHz – 875 MHz; FDD. n20; 832 MHz – 862 MHz; 791 MHz – 821 MHz; FDD. n247; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n25; 1850 MHz – 1915 MHz; 1930 MHz – 1995 MHz; FDD. n26; 814 MHz – 849 MHz; 859 MHz – 894 MHz; FDD. n28; 703 MHz – 748 MHz; 758 MHz – 803 MHz; FDD. n29; N/A; 717 MHz – 728 MHz; SDL. n30; 2305 MHz – 2315 MHz; 2350 MHz – 2360 MHz; FDD. n34; 2010 MHz – 2025 MHz; 2010 MHz – 2025 MHz; TDD. n38; 2570 MHz – 2620 MHz; 2570 MHz – 2620 MHz; TDD. n39; 1880 MHz – 1920 MHz; 1880 MHz – 1920 MHz; TDD. n40; 2300 MHz – 2400 MHz; 2300 MHz – 2400 MHz; TDD. n41; 2496 MHz – 2690 MHz; 2496 MHz – 2690 MHz; TDD. n46; 5150 MHz – 5925 MHz; 5150 MHz – 5925 MHz; TDD (NOTE 3). n48; 3550 MHz – 3700 MHz; 3550 MHz – 3700 MHz; TDD. n50; 1432 MHz – 1517 MHz; 1432 MHz – 1517 MHz; TDD. n51; 1427 MHz – 1432 MHz; 1427 MHz – 1432 MHz; TDD. n53; 2483.5 MHz – 2495 MHz; 2483.5 MHz – 2495 MHz; TDD. n54; 1670 MHz – 1675 MHz; 1670 MHz – 1675 MHz; TDD. n65; 1920 MHz – 2010 MHz; 2110 MHz – 2200 MHz; FDD. n66; 1710 MHz – 1780 MHz; 2110 MHz – 2200 MHz; FDD. n67; N/A; 738 MHz – 758 MHz; SDL. n70; 1695 MHz – 1710 MHz; 1995 MHz – 2020 MHz; FDD. n71; 663 MHz – 698 MHz; 617 MHz – 652 MHz; FDD. n74; 1427 MHz – 1470 MHz; 1475 MHz – 1518 MHz; FDD. n75; N/A; 1432 MHz – 1517 MHz; SDL. n76; N/A; 1427 MHz – 1432 MHz; SDL. n77; 3300 MHz – 4200 MHz; 3300 MHz – 4200 MHz; TDD. n78; 3300 MHz – 3800 MHz; 3300 MHz – 3800 MHz; TDD. n79; 4400 MHz – 5000 MHz; 4400 MHz – 5000 MHz; TDD. n80; 1710 MHz – 1785 MHz; N/A; SUL. n81; 880 MHz – 915 MHz; N/A; SUL. n82; 832 MHz – 862 MHz; N/A; SUL. n83; 703 MHz – 748 MHz; N/A; SUL. n84; 1920 MHz – 1980 MHz; N/A; SUL. n85; 698 MHz – 716 MHz; 728 MHz – 746 MHz; FDD. n86; 1710 MHz – 1780 MHz; N/A; SUL. n89; 824 MHz – 849 MHz; N/A; SUL. n90; 2496 MHz – 2690 MHz; 2496 MHz – 2690 MHz; TDD. n91; 832 MHz – 862 MHz; 1427 MHz – 1432 MHz; FDD (NOTE 2). n92; 832 MHz – 862 MHz; 1432 MHz – 1517 MHz; FDD (NOTE 2). n93; 880 MHz – 915 MHz; 1427 MHz – 1432 MHz; FDD (NOTE 2). n94; 880 MHz – 915 MHz; 1432 MHz – 1517 MHz; FDD (NOTE 2). n95 (NOTE 1); 2010 MHz – 2025 MHz; N/A; SUL. n96 (NOTE 4); 5925 MHz – 7125 MHz; 5925 MHz – 7125 MHz; TDD (NOTE 3). n975; 2300 MHz – 2400 MHz; N/A; SUL. n985; 1880 MHz – 1920 MHz; N/A; SUL. n996; 1626.5 MHz -1660.5 MHz; N/A; SUL. n100; 874.4 MHz – 880 MHz; 919.4 MHz – 925 MHz; FDD. n101; 1900 MHz – 1910 MHz; 1900 MHz – 1910 MHz; TDD. n1024; 5925 MHz – 6425 MHz; 5925 MHz – 6425 MHz; TDD3. n1048; 6425 MHz – 7125 MHz; 6425 MHz – 7125 MHz; TDD. n105; 663 MHz – 703 MHz; 612 MHz – 652 MHz; FDD. Table 5.2-2: NR operating bands in FR2 NR operating band; Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high; Duplex mode. n257; 26500 MHz – 29500 MHz; TDD. n258; 24250 MHz – 27500 MHz; TDD. n259; 39500 MHz – 43500 MHz; TDD. n260; 37000 MHz – 40000 MHz; TDD. n261; 27500 MHz – 28350 MHz; TDD. n262; 47200 MHz – 48200 MHz; TDD. n263; 57000 MHz – 71000 MHz; TDD.

From the above description, the NCR supports both unpaired spectrum and paired spectrum.

- Both unpaired spectrum and paired spectrum are included for NCR operation.

According the definition of NR operating bands in FR1/FR2, only unpaired spectrum is included in FR2 and both unpaired and paired spectrum are included in FR1. For NCR operation in FR2, both beam information and ON-OFF indication are necessary for AC link. For NCR operation in FR1, only the ON-OFF is necessary and the beam information may be not always present. It is agreed that the “ON” state of NCR-Fwd is implicitly indicated by AC link beam information as following

Agreement (RAN1#112e) Update the agreements on ‘ON-OFF’ achieved in RAN1 #111 as follows: For FR2, the “ON” state of NCR-Fwd is indicated: - Implicit indication via the AC link beam indication (i.e., if there is an AC link beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) For FR1, the “ON” state of NCR-Fwd is indicated: - Indication via the AC link beam indication (i.e., if there is an AC link beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s)) - When there is only one beam, the sole purpose of the AC link beam indication is for indicating “ON” state of NCR-Fwd Note: Backhaul link is following the AC link in terms of ON/OFF

In current specification, time resources and a corresponding beam index are used for uplink slot(s)/symbol(s) or downlink slot(s)/symbol(s). For example, when NCR is provided time resources and a corresponding beam index by ncr-PeriodicFwdResourceSetToAddModList, the NCR determines behavior of transmissions or receptions based on TDD UL/DL configuration for unpaired spectrum. It is shown in figure 1 as following:

Figure 1: The explain of beam indication for unpaired spectrum

As shown in Figure 1, the symbols of #0#1#12#13 are not included in time resources indicated by the ncr-PeriodicFwdResourceSetToAddModList and regarded as OFF operation. The symbols of #2 to #4 are regarded as ON and transmitting operation, and the symbols of #5 to #11 are regarded as ON and receiving operation. Since those symbols are included in time resources by the ncr-PeriodicFwdResourceSetToAddModList and the beam index is 1.

For paired spectrum, the NCR can be provided by ncr-PeriodicFwdResourceSetToAddModList a list of sets of resources, according to the agreement that beam correspondence is assumed for the DL/UL of the access link of NCR-Fwd, it can be understood that the time resources and beam indication are used by uplink spectrum and downlink spectrum operation, that is for each symbol, transmission (DL) and reception (UL) are operated simultaneously and the beam indication is applied over the indicated time resources for DL and UL forwarding. It is shown in figure 2 as following:

Figure 2: The ON symbols are same for FDD-DL and FDD-UL.

As shown in Figure 2, the symbols of #0#1#12#13 are not included in time resources indicated by the ncr-PeriodicFwdResourceSetToAddModList and regarded as OFF operation. The OFF operation is applied on uplink spectrum and downlink spectrum. The symbols of #2 to #11 are included in time resources indicated by the ncr-PeriodicFwdResourceSetToAddModList and the beam index is 1. Those symbols are regarded as ON operation, and the ON operation is applied on uplink spectrum and downlink spectrum. Therefore, different from TDD mode, in FDD mode the NCR are both transmitting and receiving. It is noted this should not be viewed as an editorial changes , since the current wording will cause NCR to incorrectly turn off either uplink or downlink for FDD operation.

- Current wording in the specification is not correct for paired spectrum, and will cause NCR to incorrectly turn off either in uplink or downlink for FDD operation.

- For beam indication of paired spectrum operation, the time resources and a corresponding beam index are applied on uplink spectrum and downlink spectrum. Adopt the CR on NCR for clause 20 of TS 38.213.

Reason for change: The current description of usage of time resources and a corresponding beam index description is only suitable for unpaired spectrum as in TDD NCR is either transmitting or receiving when it is ON during a particular indicated duration. For paired spectrum, the description needs change because NCR is transmitting and receiving in the paired spectrum. Summary of change: Correct the NCR behavior that time resources and a corresponding beam index are used both on uplink spectrum and downlink spectrum. Consequences if not approved: NCR behaviour is not defined correctly for paired spectrum. ================================================================================ 20 Network controlled repeater < Unchanged parts are omitted> The NCR-Fwd uses a same beam for transmissions and receptions on the access link during respective time resources associated with the beam. For unpaired spectrum, the NCR can be provided by ncr-PeriodicFwdResourceSetToAddModList a list of sets of resources for transmissions or receptions on the access link. For paired spectrum, the NCR can be provided by ncr-PeriodicFwdResourceSetToAddModList a list of sets of resources for transmissions and receptions on the access link. A set of resources, from the list of sets of resources, is provided by NCR-PeriodicFwdResourceSet and occurs with a periodicity provided by ncr-periodicity. A resource from the set of resources is provided by NCR-PeriodicFwdResource and includes a pair of a time resource provided by ncr-PeriodicTimeResource and a beam [20, TS 38.106] with an index provided by ncr-beamIndex. The time resource starts at a slot that is offset by slotOffsetPeriodic slots from the start of the period for the set of resources and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS. For unpaired spectrum, the NCR can be provided by ncr-SemiPersistentFwdResourceSetToAddModList a list of sets of resources for transmissions or receptions on the access link. For paired spectrum, the NCR can be provided by ncr-PeriodicFwdResourceSetToAddModList a list of sets of resources for transmissions and receptions on the access link. And a MAC CE command can indicate a set of resources for the NCR to use or to stop using based on a corresponding identity provided by ncr-SemiPersistentFwdResourceSetId [11, TS 38.321]. The NCR uses or stops using the set of resources starting from the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the NCR-MT would transmit a PUCCH with HARQ-ACK information associated with the PDSCH providing the MAC CE command and μ is the SCS configuration for the PUCCH transmission. The set of resources is provided by NCR-SemiPersistentFwdResourceSet and occurs with a periodicity provided by ncr-periodicity. A resource from the set of resources is provided by NCR-SemiPersistentFwdResource and includes a pair of a time resource provided by ncr-SemiPersistentTimeResource and a beam with an index provided by ncr-beamIndex, where beamIndex can be updated by the MAC CE command. The time resource starts at a slot that is offset by slotOffsetSemiPersistent slots from the start of the period for the set of resources and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS. The NCR-MT can be configured to monitor PDCCH according to USS sets for detection of a DCI format 2_8 with CRC scrambled by an NCR-RNTI. For unpaired spectrum, a time resource and a corresponding beam index for transmissions or receptions on the access link are indicated by corresponding fields in DCI format 2_8 [4, TS 38.212]. For paired spectrum, a time resource and a corresponding beam index for transmissions and receptions on the access link are indicated by corresponding fields in DCI format 2_8 [4, TS 38.212]. When the NCR detects more than one DCI formats 2_8 that indicate beam indexes for time resources overlapping in a set of symbols, the NCR uses for the set of symbols a beam index that is indicated by a DCI format 2_8 that the NCR-MT detects in a most recent PDCCH monitoring occasion. The time resource starts at a slot that is offset by slotOffsetAperiodic slots from a reference slot and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS. The reference slot is a slot that is after a slot of a PDCCH reception that provides the DCI format 2_8 by a number of slots indicated by FG 43-3. < Unchanged parts are omitted >

Conclusion

In this contribution, we share our views on other aspects including control plane signaling and procedures relevant to RAN1 for NCR. The observations and proposals are summarized as follows:

- Both unpaired spectrum and paired spectrum are included for NCR operation.

- Current wording in the specification is not correct for paired spectrum,  and will cause NCR to incorrectly turn off either in uplink or downlink for FDD operation.

- For beam indication of paired spectrum operation, the time resources and a corresponding beam index are applied on uplink spectrum and downlink spectrum. Adopt the CR on NCR for clause 20 of TS 38.213.

Reason for change: The current description of usage of time resources and a corresponding beam index description is only suitable for unpaired spectrum as in TDD NCR is either transmitting or receiving when it is ON during a particular indicated duration. For paired spectrum, the description needs change because NCR is transmitting and receiving in the paired spectrum. Summary of change: Correct the NCR behavior that time resources and a corresponding beam index are used both on uplink spectrum and downlink spectrum. Consequences if not approved: NCR behaviour is not defined correctly for paired spectrum. ================================================================================ 20 Network controlled repeater < Unchanged parts are omitted> The NCR-Fwd uses a same beam for transmissions and receptions on the access link during respective time resources associated with the beam. For unpaired spectrum, the NCR can be provided by ncr-PeriodicFwdResourceSetToAddModList a list of sets of resources for transmissions or receptions on the access link. For paired spectrum, the NCR can be provided by ncr-PeriodicFwdResourceSetToAddModList a list of sets of resources for transmissions and receptions on the access link. A set of resources, from the list of sets of resources, is provided by NCR-PeriodicFwdResourceSet and occurs with a periodicity provided by ncr-periodicity. A resource from the set of resources is provided by NCR-PeriodicFwdResource and includes a pair of a time resource provided by ncr-PeriodicTimeResource and a beam [20, TS 38.106] with an index provided by ncr-beamIndex. The time resource starts at a slot that is offset by slotOffsetPeriodic slots from the start of the period for the set of resources and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS. For unpaired spectrum, the NCR can be provided by ncr-SemiPersistentFwdResourceSetToAddModList a list of sets of resources for transmissions or receptions on the access link. For paired spectrum, the NCR can be provided by ncr-PeriodicFwdResourceSetToAddModList a list of sets of resources for transmissions and receptions on the access link. And a MAC CE command can indicate a set of resources for the NCR to use or to stop using based on a corresponding identity provided by ncr-SemiPersistentFwdResourceSetId [11, TS 38.321]. The NCR uses or stops using the set of resources starting from the first slot that is after slot k+3Nslotsubframe,μ where k is the slot where the NCR-MT would transmit a PUCCH with HARQ-ACK information associated with the PDSCH providing the MAC CE command and μ is the SCS configuration for the PUCCH transmission. The set of resources is provided by NCR-SemiPersistentFwdResourceSet and occurs with a periodicity provided by ncr-periodicity. A resource from the set of resources is provided by NCR-SemiPersistentFwdResource and includes a pair of a time resource provided by ncr-SemiPersistentTimeResource and a beam with an index provided by ncr-beamIndex, where beamIndex can be updated by the MAC CE command. The time resource starts at a slot that is offset by slotOffsetSemiPersistent slots from the start of the period for the set of resources and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS. The NCR-MT can be configured to monitor PDCCH according to USS sets for detection of a DCI format 2_8 with CRC scrambled by an NCR-RNTI. For unpaired spectrum, a time resource and a corresponding beam index for transmissions or receptions on the access link are indicated by corresponding fields in DCI format 2_8 [4, TS 38.212]. For paired spectrum, a time resource and a corresponding beam index for transmissions and receptions on the access link are indicated by corresponding fields in DCI format 2_8 [4, TS 38.212]. When the NCR detects more than one DCI formats 2_8 that indicate beam indexes for time resources overlapping in a set of symbols, the NCR uses for the set of symbols a beam index that is indicated by a DCI format 2_8 that the NCR-MT detects in a most recent PDCCH monitoring occasion. The time resource starts at a slot that is offset by slotOffsetAperiodic slots from a reference slot and at a symbol that is offset by symbolOffset from the start of the slot, and has a duration provided by durationInSymbols for a SCS provided by ncr-referenceSCS. The reference slot is a slot that is after a slot of a PDCCH reception that provides the DCI format 2_8 by a number of slots indicated by FG 43-3. < Unchanged parts are omitted >

References

- 3GPP TS 38.106 v18.2.0: "NR; Base Station (BS) radio transmission and reception"

- 3GPP TS 38.104 v18.3.0: "NR; NR repeater radio transmission and reception"

- 3GPP TS 38.213 v18.0.0: "NR; Physical layer procedures for control"

- R2-2311173: "Introducing support for Network Controlled Repeaters to 38.321"