3GPP TSG RAN WG1 #117		                                                       R1-2404841

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	8.2.1

Source: 	OPPO, Huawei, HiSilicon, LG Electronics

Title: 	UE features list for Rel-18 NR sidelink evolution WI

Document for:	Discussion and decision

- Introduction

This contribution includes an updated RAN1 UE features list for the Rel-18 NR sidelink evolution WI based on the inputs received and discussion outcome of RAN1#116bis, and proposed changes compared with the endorsed version in RAN1#116bis (R1-2403703) are marked in red.

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k1 | SL channel access for dynamic channel access mode | UE supports 1. SL Type 1 channel access and contention window size adjustment 2. SL Type 2A channel access 3. SL Type 2B channel access 4. SL Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol in 15kHz SCS if the UE supports 15 kHz SCS 7. CP extension up to 2 symbols in 30kHz SCS 8. CP extension up to 2 symbols if the UE supports 60kHz SCS 9. SL Type 1 and Type 2 channel access for multiple starting positions in a slot | At least one of {15-25, 15-3, [32-4, 32-4a]} | Yes | No | UE does not support channel access for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. Note: Component 8 is applicable in regions without OCB requirements. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling For UE supports NR SL in shared spectrum and when where shared spectrum channel access must be used, UE must indicate support this FG is supported |
| 47. NR_SL_enh2 | 47- k2 | SL multi-channel access for dynamic channel access mode | 1. UE supports multi-channel access procedures for PSCCH/PSSCH/S-SSB/PSFCH transmission(s) in multiple RB sets in a slot 4) UE supports multi-channel access procedure on N channel(s) with 20MHz LBT bandwidth for each channel. Candidate values of N: {2, 3, 4, 5} | 47-k1 | Yes | No | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. Note: Support of S-SSB/PSFCH transmission(s) in multiple RB-sets in a slot is according to the support of {47-m11, 47-m11a} and {47-m12, 47-m12a} | Optional with capability signalling |
| 47. NR_SL_enh2 | 47- k2-1 | SL multi-channel access allowing Transmitting PSFCH/S-SSB transmission on a subset of the intended number of RB sets based on the outcome of channel access on individual RB sets | UE supports Type A and Type B multi-channel access procedures for PSFCH/S-SSB transmissions in multiple RB sets in a slot | 47-k2 | No | No |  |  |  |  |  |  | Optional without capability signaling |
| 47. NR_SL_enh2 | 47- k3 | Receiving UE to UE COT sharing information | 1. UE supports monitoring SCI to read COT sharing information 2. UE supports transmitting NR SL based on COT sharing information subject to COT sharing conditions | 47-k1 | No | No | UE does not support using UE-to-UE COT sharing information contained in SCI for sharing COT for NR sidelink operation in shared spectrum. |  |  |  |  | The FG is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling For UE supports NR SL in shared spectrum where shared spectrum channel access must be used, UE must support this FG |
| 47. NR_SL_enh2 | 47-k4 | Transmitting UE to UE COT sharing information | 1. UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing 2. UE supports indicating COT sharing information in SCI | 47-k1 | No | Yes | UE does not support transmitting UE-to-UE COT sharing information for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k5 | Resource allocation for multi-consecutive slots transmission | UE supports resource (re-)selection for PSCCH/PSSCH transmission on multiple consecutive slots | at least one of {15-3, 32-4} | No | No | UE does not support resource (re-)selection for multi-consecutive slots transmission |  |  |  |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-k6 | Type1 LBT blocking Option 1 | UE supports 1. avoid selection of N consecutive resource(s) before a reserved resource when the L1 SL priority value for the transmission is higher than the L1 SL priority value of the reserved resource. It is up to UE whether to do it 2. avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. It is up to UE whether to do it | 47-k1 | Yes | No | UE does not support Type1 LBT blocking Option 1 | Per band | n/a | n/a |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k7 | Type1 LBT blocking Option 2 | UE supports 1. If transmission in slot(s) at least T_proc,0 before a reserved resource is able to share its initiated COT to the reservation, UE prioritize / select resource(s) in the slot(s) for transmission. It is up to UE whether to do it | 47-k1 | Yes | No | UE does not support Type1 LBT blocking Option 2 | Per band | n/a | n/a |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k8 | CW autonomous update for SL transmission without HARQ feedback | UE support autonomous update of the CWp to the next higher allowed value when the same CWp ≠ CWmax,p value is consecutively used for X times for generation of Ninit for PSCCH/PSSCH transmission without HARQ feedback. | 47-k1 | No | No | UE does not update CWp for PSCCH/PSSCH transmission without HARQ feedback. |  |  |  |  | The FG is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-k9 | Sidelink mode 1 resource allocation in shared spectrum | 1. UE can monitor DCI format 3_0 on a licensed band for NR sidelink dynamic scheduling and configured grant type 2 for transmitting PSCCH/PSSCH on a shared spectrum 2. UE supports reporting NACK to gNB when transmitting PSCCH/PSSCH on scheduled resource(s) is failed due to LBT failure | 47-k1 | Yes | No | UE does not perform PSCCH/PSSCH based on mode 1 resource allocation in a shared spectrum. | Per band | N/A | N/A |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based SL transmission/reception | 1. UE supports interlace RB-based SL transmissions for the physical layer channels that it is capable of transmit 2. UE supports interlace RB-based SL receptions for the physical layer channels that it is capable of receive | At least one of {15-25, 15-3, [32-4, 32-4a]} | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling For UE supports NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate support this FG is supported. |
| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot in addition to the first starting symbol | At least one of {15-25, 15-3, [32-4, 32-4a]} | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot |  |  |  |  | Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs The FG is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol 2. UE can monitor a total up to X PSCCHs in a slot in the 1st and 2nd starting symbols | [15-1 except Component 5] | No | No | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot |  |  |  |  | The value X is the same as the reported value in FG 15-1 The FG is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling For UE supports NR sidelink in shared spectrum and when where shared spectrum channel access must be used, UE must support this FG.] |
| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH | 1. UE supports PSFCH transmission/reception on N PSFCH occasion(s) per PSCCH/PSSCH | 15-11 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | Per band | N/A | N/A |  | Candidate values for N are {1,2,3,4} The signaling FG is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | 1. UE supports transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set | 15-4 | No | No | UE does not support transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set |  |  |  |  | This is the basic FG for NR sidelink in shared spectrum where PSD and/or OCB requirements are defined by regulation. It is up to UE implementation whether S-SSB RX UE monitors more than one S-SSB repetition in frequency domain within one RB set as long as RAN4 requirements are satisfied The FG is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling For UE supports NR sidelink in shared spectrum where PSD and/or OCB requirements are defined by regulation, UE must support this FG. |
| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1. UE supports transmitting S-SSB on additional S-SSB occasion(s) | 15-4 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) |  |  |  |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m9 | Receiving S-SSB on additional S-SSB occasion(s) | 1. UE supports receiving S-SSB on additional S-SSB occasion(s) | 15-4 | No | No | UE does not support receiving S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) |  |  |  |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m10 | Contiguous RB-based PSCCH/PSSCH transmission/reception | 1. UE supports contiguous RB-based PSCCH/PSSCH transmission/reception 2. UE supports resource (re-)selection for contiguous RB-based PSCCH/PSSCH transmission | At least one of {15-25, 15-3, 32-4, 32-4a} | Yes | No | UE does not support contiguous RB-based PSCCH/PSSCH transmission/reception | Per band | N/A | N/A |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11 | PSFCH transmissions in multiple contiguous RB sets | UE supports PSFCH transmissions in multiple contiguous RB sets | at least one of {47-k2, 47-k2-1} | Yes | Yes | UE does not support PSFCH transmissions in multiple contiguous RB sets | Per band | N/A | N/A |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11a | PSFCH transmissions in multiple non-contiguous RB sets | UE supports PSFCH transmissions in multiple non-contiguous RB sets | 47-m11 | Yes | Yes | UE does not support PSFCH transmissions in multiple non-contiguous RB sets | Per band | N/A | N/A |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12 | S-SSB transmissions in multiple contiguous RB sets | UE supports S-SSB transmissions in multiple contiguous RB sets | at least one of {47-k2, 47-k2-1} | No | No | UE does not support S-SSB transmissions in multiple contiguous RB sets |  |  |  |  | The FG is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m12a | S-SSB transmissions in multiple non-contiguous RB sets | UE supports S-SSB transmissions in multiple non-contiguous RB sets | 47-m12 | No | No | UE does not support S-SSB transmissions in multiple non-contiguous RB sets |  |  |  |  | The FG is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m13 | Transmissions/receptions of multiple dedicated PRBs in interlace-based PSFCH | 1. UE can transmit PSFCH(s) on up to a total of K dedicated PRBs in a slot. 2. UE can receive PSFCH(s) on up to a total of L dedicated PRBs in a slot | TBD | No | No | UE does not support multiple transmissions/receptions of common interlace-based PSFCH. | Per band | N/A | N/A |  | The signaling FG is only expected for a band where shared spectrum channel access must be used. Candidate values for K are FFS Candidate values for L are FFS | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-s1 | Transmission/Reception using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR sidelink TXs and RXs in a resource pool in 15kHz and 30kHz SCSs and uses the SCS that is (pre)configured for a SL BWP. | 15-3, 15-6, 15-11 | Yes | No | UE does not support transmission/reception using dynamic resource pool sharing | Per band | N/A | N/A |  | Component 2 does not imply that two different SCSs can be (pre)configured simultaneously in a SL BWP | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: - 1-1) Maximum number of simultaneous PSCCH/PSSCH TX, equal to X and 1 per carrier - 1-2) For the number of PSCCH decodes: - UE can receive Z* floor (NRB,i /10 RBs) PSCCH in a slot on carrier i of the X carriers. - 1-3) For the number of non-overlapped PRBs over aggregated SL carriers: - UE can attempt to decode NRB,i non-overlapping RBs in a slot on carrier i of the X carriers. - 1-4) UE can aggregate up to total bandwidth Y MHz. 2) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. | 15-3, 15-11 | Yes | Yes |  | Per band | N/A | N/A |  | Component 1: Candidate value of X = {2, 3, 4, 5, 6, 7, 8} Component 1-2 candidate value set: Z={1, 2} NRB,i is the number of RBs defined per channel bandwidth of carrier i by RAN4 in 38.101-1 Table 5.3.2-1 for FR1 Component 1-4 candidate value set: Y={20, 30, 40, 50, 60, 70} Note: this feature is supported only in a band indicated with the PC5 interface in 38.101-1 Table 5.2E.1A-1 for FR1 | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v2 | Synchronization for SL CA | 1-1) UE supports transmitting S-SSB on one selected or all candidate synchronization carriers with the same sync reference from Set-B 1-2) UE supports receiving S-SSB from all candidate synchronization carriers with the same sync reference from Set-B 2) UE can adjust the transmission power of the S-SSB across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. | 47-v1, [15-4] | Yes | No |  | Per band | N/A | N/A |  | Note: Option of UE selection of one selected SL synchronization carrier with the same sync reference from Set-B is not based on limited Tx capability Note: Component 1-2 does not require simultaneous reception of S-SSB on all candidate synchronization carriers with the same sync reference from Set-B | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v3 | PSFCH for SL CA | 1) UE supports receiving X PSFCH resources in a slot over all aggregated SL carriers - 1-1) UE is capable of receiving at least one PSFCH resource on each of the aggregated carriers in a slot 2) UE supports transmitting Y PSFCH resources in a slot over all aggregated SL carriers according to PSFCH procedures - 2-1) UE is capable of transmitting at least one PSFCH resource on each of the aggregated carriers | 47-v1 | Yes | No |  | Per band | N/A | N/A |  | Candidate values for X are {5, 15, 25, 32, 35, 45, 50, 64, 128FFS} Candidate values for Y are {4, 8, 16, 32FFS} Note: for component 1-1, it is up to UE implementation which PSFCH(s) to receive | Optional with capability signalling |