3GPP TSG RAN WG1 Meeting #118	R1-2405804

Maastricht, NL, August 19 –23, 2024

Agenda Item:	9.4.2.3

Source:	Futurewei

Title:	D2R and R2D Channel/Signal Aspects for Ambient IoT

Document for:	Discussion and Decision

# Introduction

The objectives of the Ambient IoT study [1] are shown in Appendix A with the RAN1-related objectives being treated across six agenda items.

Appendix B captures the feature lead suggested topics for RAN1#118 [2]. Among the suggested topics are R2D control information for PRDCH and/or PDRCH, D2R control information, any enhancements of design details of PRDCH and PDRCH, further structure/design details of preamble for R2D and D2R, proximity determination, and intermediate UE aspects. We provide proposals on these topics as well as others.

# Discussion

## PRDCH

In RAN1#117, an agreement regarding the PRDCH was reached.

For R2D, the only physical channel is PRDCH.

- PRDCH carries any higher-layer payload

- PRDCH carries L1 R2D control information (if defined)

- FFS details of device behaviour(s) for receiving PRDCH

Coupled with the agreement in RAN1#116bis for the timing acquisition signal,

For the R2D timing acquisition signal immediately preceding the transmission of a physical channel, study a preamble with at least two parts which includes a start-indicator part and a clock-acquisition part, where the start-indicator part immediately precedes the clock-acquisition part:

- Start-indicator part provides the start of the R2D transmission

- FFS: Details of start-indicator part

- Clock-acquisition part provides at least the chip synchronization of the subsequent physical channel transmission

- FFS: Details of clock-acquisition part, e.g. structure, encoding, length, etc.

- FFS: Methods to determine chip duration of the subsequent physical channel transmission

- FFS: Other functionalities

- Note: the preamble is considered not to be part of a physical channel

- FFS: other part(s) of the preamble, if any

- FFS: whether the above clock acquisition is sufficient for all devices

- FFS: how to make the preamble compact

it is possible to draw a block diagram of the R2D transmission as shown in Fig. 1.

time

Timing acquisition signal

PRDCH

L1 R2D Control information

Start indicator part

Clock-acquisition part

Fig. 1. Fields of R2D transmission.

We first examine the control information and later the timing acquisition signal.

### L1 R2D Control information

The FL summary suggested additional discussion for the R2D control information [2].

Companies can consider a table like below to discuss what R2D control information is needed for PRDCH and PDRCH, respectively. And if needed, whether it is signaled via higher-layer or L1 R2D control signaling

The FL summary listed possible R2D control information.

- At least the following R2D control information for PRDCH and/or PDRCH are studied:

  - Time domain resource allocation

  - MCS/coding rate

  - TBS

  - Repetitions

  - Device ID and/or device group ID and/or device type

  - Cast type

  - Frequency domain resource allocation

  - Reader ID

Please note that the intent of above bullet is not consider all the listed control information for both PRDCH and PDRCH, only a subset of control information may be applicable for a channel

- For the study of each of the above R2D control information, at least following aspects are considered:

  - Whether the control information is needed/predefined or not

  - For control information that is not fixed, whether it is signaled as L1 control information and/or via higher-layer signaling (e.g. MAC CE, RRC)

  - Whether the control information is unicast, groupcast, broadcast

The FL proposal lists control information for both R2D and D2R transmissions. In our view, this complicates the design of the control information fields. We should discuss control information for R2D transmissions separately from the scheduling information for D2R transmission. For example, from the list above, some R2D control information can include {TBS, cast type, reader ID, device ID and/or group ID and/or device type} while the D2R scheduling information can include {TBS, time domain resource allocation, MCS/coding rate, Repetitions, Frequency domain resource allocation}.

A general design goal is to minimize the amount of L1 R2D control information so that the amount of R2D control information processing is reduced, the size of the R2D transmission can be shortened whenever possible, and to harmonize the design across the device types. In addition, as mentioned in our companion contribution [7], we should provide control information to support device availability.

There can be two types of PRDCH: the device obtains information, e.g., about the reader / system, configuration, broadcast. The other type, the device generates a response to the PRDCH, e.g., a command. A possible design approach is to identify information common to both types of PRDCH and consider that to be L1 R2D control information. The remaining information can be considered as part of a fields specific to a message, e.g., part of a MAC message. For example, the fields {TBS, time domain resource allocation, MCS/coding rate, Repetitions, Frequency domain resource allocation} are related to scheduling a D2R transmission but not necessary to a broadcast. It does not make sense for a L1 R2D control information to include those fields. By excluding those fields, the PRDCH can be smaller for a broadcast.

Proposal 1: For Ambient IoT, no scheduling information for a PDRCH transmission is carried in the L1 R2D control information of the PRDCH.

In Table 1, we provide views for some control information for the R2D link and how the information is conveyed, i.e. via L1 R2D control information or via higher layer signaling (e.g., MAC layer).

Table 1. Possible fields for L1 R2D control information for R2D reception

| Information | Purpose | Signaled via |  |
| --- | --- | --- | --- |
|  |  | L1 control | Higher layer |
| device ID | Indicates the intended recipients of R2D transmission. - Device ID is a network generated ID - Device ID can be a group ID | Y |  |
| Cast type | Similar to sidelink SCI format 2 to classify whether an ID is for groups or for broadcast. May not be present if some IDs are reserved for broadcast and groups | Y |  |
| TBS | PRDCH message length. If the R2D delimiter is agreed, message length may not be needed |  |  |
| Message type | Command, Configuration, etc. | Not supported | Y |
| [Reader ID] | May be needed if the device is able to cross the coverage of several readers. |  |  |
| D2R parameters | - Time domain resource allocation - TBS - Modulation / coding scheme - Repetition (if not combined with MCS) - Frequency domain resource allocation - [Line encoding scheme (e.g., Manchester, FM0/Miller)] - next decode | (no D2R parameters in L1 R2D control information) | Y |
| Next decode | - Indicates when device(s) or groups of devices can perform a full decoding of a R2D transmission [7] | Y |  |
| Reserved bits | - One or more reserved bits. Can provision a L1 control extension for future compatibility | Y |  |

Proposal 2: Study a design for the L1 R2D control information that:

- Carries the following: device ID; cast type, next decode field, Reserved bits

  - If device ID can indicate broadcast / groupcast, then cast type is not needed.

- Does not carry message type nor D2R parameters.

- FFS TBS, and Reader ID

The reserved bits provide a placeholder for additional fields for the L1 R2D control information, as shown in Fig. 2b. Because it is desirable to minimize the size of the L1 R2D control information, the number of reserved bits should be reduced. Having a systematic approach to add additional fields should be considered. A scalable approach [6], similar to extensible bit vectors in [8], is shown in Fig. 2c and Fig. 2d. For example, if the last reserved bit is ‘0’, there are no additional information fields. When the last reserved bit is ‘1’, there can be a daisy chain of additional fields. This approach allows the flexibility to increase the number of L1 R2D control bits as needed. In Fig. 2c and Fig. 2d, the last bit of the L1 R2D control information is ‘0’.

L1 R2D Control information

L1 R2D Control
 information

reserved

L1 R2D Control information

0

a)

b)

c)

Block 1

Block n

L1 R2D Control
information

Additional information

1

Additional information

1

0

d)

Fig. 2. Extending the L1 R2D control information. a) no additional bits; b) some reserved bits added; c) A last reserved bit set to ‘0’ indicating no additional information; d) Using ‘1’ to indicate an extensible number of fields

Proposal 3: For the L1 R2D control information format, study means of supporting flexible number of bits.

Another related topic for discussion is:

Further consider discussing details in terms PRDCH generation/mapping schemes/formats (if needed) to support carrying combination of one or more information types include R2D data, L1 R2D control (if any) and L1 system information (if any)

Additionally consider how coding/CRC attachment can be handled in above combinations, if needed

There are five possible formats of L1 R2D control information within a PRDCH, as shown in Table 2. In the diagrams below, it is assumed that the error detection capability of CRC1 is the same or better than the error detection capability of CRC2.

Table 2. Possible formats for PRDCH

|  | Description | Diagram | General observations |
| --- | --- | --- | --- |
| 1 | CRC2 spans L1 R2D control info. CRC1 starts after CRC2 until the end of the PRDCH transmission. | PRDCH L1 R2D Ctrl info CRC2 CRC1 | Arbitration rule if only one CRC passes. What should device do? |
| 2 | CRC2 spans L1 R2D control info. CRC1 covers the entire PRDCH, including CRC2 and L1 R2D control information | PRDCH L1 R2D Ctrl info CRC2 CRC1 | If CRC2 passes while CRC1 fails, what should device do? |
| 3 | No CRC over L1 R2D control info. CRC1 starts after L1 R2D control info until the end of the PRDCH transmission. | PRDCH L1 R2D Ctrl info CRC1 | No check over L1 R2D control information |
| 4 | No CRC over L1 R2D control info but CRC1 covers the entire PRDCH, including the L1 R2D control info. | PRDCH L1 R2D Ctrl info CRC1 | Device can process L1 R2D control information and uses CRC1 to validate the control information when device processes PRDCH |
| 5 | No L1 R2D control information. CRC1 covers the entire PRDCH | PRDCH CRC1 | A device must always process PRDCH |

For simplicity of design, L1 R2D control information, if any, should be placed at the start of the PRDCH.

Proposal 4: For the format of the PRDCH, if present, any L1 R2D control information is placed at the start of the PRDCH.

There are two formats with a CRC (CRC2) following the L1 R2D control information, as shown in rows 1 and 2 of Table 2. In addition to the extra overhead, having a CRC following the control information also leads to error scenarios. In row 1, the CRCs are independent – if one CRC fails and the other passes, it is unclear how to treat the PRDCH transmission. In row 2, it is unclear how to treat the PRDCH transmission if CRC2 passes but CRC1 fails (this situation most likely indicates that errors are present after CRC2). One possible option is when there is a mismatch, both CRCs be considered as failed. Any benefits of having a CRC after the L1 R2D control information are outweighed by the overhead and error scenarios.

Proposal 5: For the format of the PRDCH, do not support a CRC solely for L1 R2D control information.

Row 3 shows a format with no CRC covering the L1 R2D control information, implying that any errors present in the L1 R2D control information cannot be detected. We do not support this format.

Row 4 shows a format with PRDCH CRC covering the L1 R2D control information. This format is suitable for serial processing at the device. As the device starts receiving, it can begin processing the L1 R2D control information. The fields in L1 R2D control information, as we proposed, are informational. The CRC will detect whether any errors are present and allows the device to discard the PRDCH if necessary. This format enables one possible energy saving approach for device availability to stop receiving when there is an unexpected/invalid device ID [7]. This approach is still applicable even if errors are present in the device ID. If errors cause an invalid device ID to become valid, the CRC will eventually detect the error before the device executes any operation, such as performing an D2R transmission. Similarly, if errors cause a valid device ID to become invalid, the device stops receiving. The device eventually would have discarded the PRDCH due to CRC detection.

Observation 1. For the format of the PRDCH, L1 R2D control information covered by the PRDCH CRC has several performance benefits.

Row 5 shows a format without any L1 R2D control information. This format requires every device to decode the PRDCH. In addition, if this format is considered, it also implies that a R2D delimiter should be supported as the packet length is associated with the higher layer signaling.

Given the strong interest in device availability, we prefer having L1 R2D control information present and covered by the PRDCH.

Proposal 6: For the format of the PRDCH, L1 R2D control information covered by the PRDCH CRC is preferred.

### Timing acquisition signal

The FL summary suggested additional discussion on timing acquisition signal [2].

- Further discuss design details and potential down-selection for start-indicator part for R2D - Further discuss design details for clock-acquisition art including - sequence/pattern - Length - Line coding, if needed - additional functionalities if any - Whether it provides chip duration for PRDCH with R2D data, system information.

There were two agreements reached in RAN1#117 [9].

For the start-indicator part of the R2D time acquisition signal, study the two options below:

- Option 1: ON/OFF pattern i.e. high/low voltage transmission

- Option 2: OFF pattern, i.e. low voltage transmission

For R2D, the clock-acquisition part of the R2D time acquisition signal is used to determine the OOK chip duration

- FFS: Pattern design to support determination of chip duration

Our contribution for agenda item 9.4.1.1 provides some details on the design of the timing acquisition signal [10].

Option 1: ON/OFF pattern

For this option, the Start Indicator of the R2D Timing Acquisition preamble consists of a series of alternating “on” and “off” OOK states as shown in Fig. 3. The Start Indicator pattern is similar to the Clock Acquisition waveform. The CP length duration is set to the “off” OOK state, which can be used to indicate the beginning of the OFDM symbol. The OOK chip interval TR2D,oc can be derived from the R2D transmission bit interval TR2D,b and the OFDM symbol payload Top as follows:

TR2D,oc=TopM

where M is the number of OOK chips per OFDM symbol payload Top.

M can be expressed as

M=Moc×TopTR2D,b

where Moc is the number of chips used to represent one binary bit, which depends on the line encoding scheme employed. For Manchester encoding, Moc=2.

Fig. 3. OOK-based OFDM waveform for R2D Timing Acquisition preamble

Option 2: OFF pattern

For this option, the Start Indicator of the R2D Timing Acquisition preamble is in the “off” state for duration Tsi, where Tsi is a fixed value. The Clock Acquisition waveform pattern is a series of alternating “on” and “off” states as shown in Fig. 4.

The OOK chip interval TR2D,oc is provided in the Clock Acquisition waveform, which can be derived in the same way as in Option 1.

Fig. 4. OOK-based OFDM waveform for R2D Timing Acquisition preamble

## PDRCH

For RAN1#118, the feature lead requested some discussion on [2].

5. For D2R control information, please consider whether/what information might need to be signaled via L1 D2R control information 6. Further consider discussing details in terms PDRCH generation/mapping schemes/formats (if needed) to support carrying combination of one or more information types include D2R data, L1 D2R control (if any) a. Additionally consider how coding/CRC attachment can be handled in above combinations, if needed 7. Consideration on Msg1 design and whether/how it is carried by PDRCH or not 8. Further discuss design details for preamble preceding PDRCH a. sequence/pattern b. Length c. Any other details

### D2R scheduling information

A RAN1#117 agreement for the PDRCH is presented below [9].

Agreement For D2R - PDRCH carries any higher-layer payload - PDRCH carries L1 D2R control information (if defined) - Note: PDRCH carries the response agreed at RAN1#116

In Table 1, we indicated that L1 R2D control information does not carry D2R scheduling information. Because some R2D transmissions do not require a D2R transmission, it is not necessary to transmit D2R scheduling information fields. Another reason not to place D2R scheduling information within the L1 R2D control information is to simplify scheduling for FDMed D2R transmissions. Consider scheduling of NR Msg3. Multiple scheduling grants for Msg3 are contained in NR Msg2 as MAC CEs. A UE searches for its MAC CE and obtains Msg3 scheduling information. This approach can be used for Ambient IoT. A-IoT Msg2 can carry multiple “MAC CEs”, each carrying D2R scheduling information for a device that transmitted A-IoT Msg1. RAN2 provides a container while RAN1 specifies the D2R scheduling information.

In section 2.1.1, we indicated that the D2R scheduling information includes:

- Time domain resource allocation: note there is discussion in AI 9.4.2.2 regarding TR2D. This field indicates when a D2R transmission can occur.

- TBS – message size of the PDRCH payload

- Modulation / coding scheme

- Repetition (if not combined with MCS)

- Frequency domain resource allocation – frequency parameters for the D2R link (e.g., frequency, bandwidth). Note that since each device obtains its own scheduling information, FDM is from the reader’s perspective.

- [Line encoding scheme (e.g., Manchester, FM0/Miller)]

- next decode – This is for device availability. Indicates when a device is expected to receive a PRDCH to decode.

Proposal 7: The D2R scheduling information contains the following fields. - Time domain resource allocation; TBS; Modulation / coding scheme; Repetition; Frequency domain resource allocation; [line encoding]; next decode.

### Frame format

As shown below, the agreement shows that L1 D2R control information does not consider CSI or SR [9].

Agreement For L1 D2R control information (if defined), the following are not considered for further study: - CSI feedback - Autonomous SR - FFS: Whether any other L1 D2R control information is needed or not

Regarding the FFS in the agreement, we do not see a need for L1 D2R control information. Several companies proposed providing an upper layer message acknowledgement in the L1 D2R control information for latency purposes. However, considering that the entire PDRCH can be encoded and repeated, L1 D2R control information becomes available after decoding. Any latency benefits are small at best while complicating the signaling protocol.

Proposal 8: update the RAN1#117 agreement

For L1 D2R control information (if defined), the following are not considered for further study:

- CSI feedback

- Autonomous SR

- FFS: Whether any other L1 D2R control information is needed or not

With no L1 D2R control information considered at this moment, there are two possible formats for the PDRCH, as shown in Fig. 5. In Fig. 5a), a PDRCH just carries bits without error detection while the format in b) has error detection. This is based on the agreements in RAN1#116bis, as shown below.

a) PDRCH

b) PDRCH CRC1

Fig. 5. a) PDRCH with no CRC b) PDRCH with CRC

Agreement Study - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PRDCH - baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PDRCH - FFS: details when different CRC lengths or no CRC may be used - FFS: other 6 bits and 16 bits CRC with different polynomials than from TS 38.212 Agreement For PDRCH generation at the device, at least following blocks are studied as the baseline: - CRC bits are appended if there is non-zero length CRC - Note: CRC details discussed in agenda item 9.4.2.1 ...

Having one frame format with a CRC is simplifies design and specification efforts. Here, we consider whether there is any significant benefit for also supporting a 0-bit CRC given the RAN2#126 agreements below.

1. A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size 2. A-IoT Msg2: the reader echoes the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements 3. A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) FFS on contention resolution based on step 4 4. A-IoT Msg4: FFS on the need and presence for this

A-IoT Msg1 just consists of a random ID that is echoed back within an A-IoT Msg2. Considering the procedure without a CRC, if A-IoT Msg1 were received incorrectly, a device waiting for the echo in A-IoT Msg2 will not receive the echo and will not continue with A-IoT Msg3. If a CRC were used, a reader can determine when errors are present in PDRCH containing the random ID. In that case, a reader will not transmit an A-IoT Msg2 for that device, and consequently, a device will not transmit an A-IoT Msg3. The device behavior is the same regardless of a CRC being used.

We prefer having a common format for all PDRCH, which includes a CRC, to simplify the design. Unless there is a significant benefit by using no CRC for A-IoT Msg1, we would like to avoid exceptions to the PDRCH format.

Proposal 9: For the PDRCH, support a common design using a CRC.

## Proximity determination

The study item objective regarding proximity determination [1] was updated to:

Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects).

One agreement reached in RAN1#116bis was that:

Proximity determination based on device side measurements is not considered.

An agreement in RAN1#117 was reached [9]:

Study the following schemes for proximity determination:

- Option 1: If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

- FFS: Details on reception criteria (e.g. either successful or not) at reader and device

- Option 2: Device is determined to be near the reader based on measurements at the reader side

- FFS: Details on measurement methods

- FFS: Whether/how transmit power of R2D and/or D2R is considered for proximity determination

It should be noted that a D2R transmission from a device is in response a device receiving an R2D transmission. So, in examining the agreement in RAN1, Option 1 is normal behavior. Implicit for Option 2 is that measurements of D2R transmissions (or an absence of a D2R transmission) are due to reader sending a R2D transmission to trigger a D2R transmission. Hence, Option 2 is the same as Option 1 except the criteria for determining proximity are possibly different.

Observation 2: Proximity determination is based on a reader monitoring D2R transmissions from an A-IoT device in response to a R2D transmission from a reader.

There may be multiple ways of monitoring. For example, a reader can adjust the MCS levels / number of repetitions in the D2R scheduling parameters until a D2R transmission fails to decode. A reader can then associate an MCS level / number of repetitions to a proximity. Since the channel coding / repetition schemes are simple, even simple reader receive processing can be used. With n repetitions, a reader receives n+1 coded blocks. A reader can decode with fewer and fewer repetitions until decoding fails. Since repetition is a form of SNR combining, the number of repetitions can be associated with proximity. A similar procedure can be used on the coded bits when the coding rate for convolutional codes is 1/r, with r is an integer greater than 1. There may be several approaches at the reader that can be considered to determine proximity.

Based on our observation, we support option 1. To move forward from the last agreement, we propose to agree to the text of Option 1. In addition, we can add an FFS related to Option 2, such as adding another condition that measurements based on the response meet some criteria.

Proposal 10: For proximity determination, if reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

- FFS: Details on reception criteria (e.g. either successful or not) at reader and device

- FFS: Whether to further require additional criteria to be met on the received D2R transmission

## Intermediate UE

The FL asked companies to address the following for RAN1#118 [2]:

11)	Considerations on intermediate UE in case of topology 2

a.	How to handle control of intermediate UE?

b.	If higher-layer and/or L1-signaling needed? If yes, design and procedure considerations

In order to determine control of the intermediate UE, it is necessary to have an understanding how an intermediate UE operates in coverage of the gNB.

There are at least two possible operating modes: 1) intermediate UE acting as a “relay” between the gNB and device (as described in [6]); 2) intermediate UE acting autonomously on resources configured by the gNB.

The operation of the Ambient IoT links imposes some different requirements for gNB control of an intermediate UE. The transmission power of the intermediate UE can impact receive operations at a gNB. If an intermediate UE transmits at 23 dBm (one of the evaluation conditions), and if an intermediate UE is near a gNB, R2D transmissions from the intermediate UE can overwhelm reception at the gNB. A gNB can account for intermediate UE transmissions if the gNB knows when an intermediate UE is transmitting.

Observation 3: When an intermediate UE transmits an R2D transmission, the transmission power of the R2D transmission may impact gNB reception operation.

From the gNB perspective, the timing of R2D transmissions can vary depending on how the intermediate UE operates. For example, if a gNB schedules an intermediate UE when to transmit and the duration of an R2D transmission, the gNB can manage the interference from the R2D transmission.

If an intermediate UE operates autonomously, the gNB may not know exactly when the R2D transmission occurs nor its duration. The gNB may have to assume the intermediate UE is transmitting over the duration of the configured resources to account for the potential R2D interference.

A similar timing issue occurs for the intermediate UE receiving D2R transmissions. In this case, transmissions from other UEs may interfere with the intermediate UE as it receives an D2R transmission. Since D2R transmissions are expected at least TR2D_min after the R2D transmissions, it is possible to anticipate when and how long the D2R transmissions are (assuming the MCS / repetition / bit rate are known). When an intermediate UE operates autonomously, the gNB may be unaware of whether a D2R is scheduled by the R2D transmission or the duration of the D2R transmission. Regardless, an assumption is that the duration of the D2R transmission can be several slots (as an upper bound). Another consideration with the intermediate UE is that it may need to process one or more D2R transmissions at approximately the same time, especially if FDM of D2R transmissions is supported.

In addition, the timing of D2R and R2D transmissions can affect when the intermediate UE performs Uu operations. Since the Ambient IoT operations (i.e., transmitting R2D and receiving D2R transmissions) are often sequential and can last several time slots, an intermediate UE may not be able to disrupt its Ambient IoT operations to perform Uu operations.

Observation 4: An intermediate UE may be unable to perform Uu operations while performing A-IoT operations.

NR has approaches that allow a UE to communicate with other UEs under network control. These approaches can be considered for the operation of the intermediate UE. Specifically, the sidelink offers two modes: mode 1 and mode 2. With mode 1, the network schedules a UE to transmit in a resource pool on the sidelink while with mode 2, a UE autonomously determines when to transmit in a resource pool according to a resource selection procedure or using random resource selection. Mode 2 is used when a UE is out of coverage or can be used while a UE is in coverage. A sidelink UE is (pre-)configured with resource pool configuration, providing time and frequency dimensions of a resource pool as well as transmit power levels. The time dimension is determined by the slots supporting the resource pool within a period (a maximum of 160 slots as described by the field sl-TimeResource in the IE SL-ResourcePool). The following table captures some highlights of the modes.

Table 3. Sidelink modes

| Mode | Sidelink transmission control | Uplink transmission control | Observations |
| --- | --- | --- | --- |
| 1 | A sidelink UE transmits: - upon receiving a scheduling DCI (e.g., DCI format 3_0) which indicates which resource, resource pool, and when to transmit - receiving an activation DCI after the sidelink UE is configured with a type 2 configured grant (CG2) - receiving a type 1 configured grant (CG1) configuration | - Within a slot supporting a resource pool, priority rules apply - Within a slot not supporting a resource pool, Uu procedures apply | - Network determines when sidelink UE transmits. - Network can override sidelink operations |
| 2 | During a resource pool, when sidelink UE has data and can use - random resource selection - sensing / partial sensing coupled with a resource selection procedure | [in network coverage] Same as above | [in network coverage] - Network is aware of the opportunities for sidelink transmission but not exactly when - Network can override sidelink operations when necessary |

Some details how an intermediate UE can function with resource pools are provided.

A “Mode 1” operation may be suitable where the network manages the AIoT devices directly and uses an intermediate UE conceptually as a relay of information (as described in [6]). However, there are some limits in applying sidelink mode 1 to the intermediate UE. Typically, the mode 1 scheduling DCI is for one slot while a R2D transmission can span multiple slots and not even start on a slot boundary. Since the network knows the duration of R2D transmission and has a good understanding of the duration of the possible D2R transmission, the network can schedule any Uu traffic to the intermediate UE without disrupting Ambient IoT operations.

A “Mode 2” operation may be suitable when the intermediate UE manages the AIoT devices autonomously and sends the results of AIoT communications to the network. The gNB indicates parameters of the resource pool and possibly when the intermediate UE can start using the pool and the duration (if shorter than the period of the pool). The gNB still can configure the maximum power and enable the use of the pool. Since the intermediate UE performs A-IoT operations for the duration of the pool, the gNB knows when an intermediate UE is able to perform Uu operations.

Using the sidelink is just one example, specifically the resource pool, to illustrate how an intermediate UE can operate in a network.

Proposal 11: For Topology 2, support the operation of an intermediate UE where the intermediate UE is configured with a resource pool indicating frequency resources, power levels, and time when the resource pool is active.

    - FFS: whether/how the gNB can signal when the intermediate UE should perform R2D transmissions

    - FFS: whether/how the intermediate UE operates in autonomous mode

# Conclusion

This contribution has discussed channels, proximity determination, and intermediate UEs for Ambient

PRDCH

Proposal 1: For Ambient IoT, no scheduling information for a PDRCH transmission is carried in the L1 R2D control information of the PRDCH.

Proposal 2: Study a design for the L1 R2D control information that:

- Carries the following: device ID; cast type, next decode field, Reserved bits

  - If device ID can indicate broadcast / groupcast, then cast type is not needed.

- Does not carry message type nor D2R parameters.

- FFS TBS, and Reader ID

Proposal 3: For the L1 R2D control information format, study means of supporting flexible number of bits.

Proposal 4: For the format of the PRDCH, if present, any L1 R2D control information is placed at the start of the PRDCH.

Proposal 5: For the format of the PRDCH, do not support a CRC solely for L1 R2D control information.

Observation 1. For the format of the PRDCH, L1 R2D control information covered by the PRDCH CRC has several performance benefits.

Proposal 6: For the format of the PRDCH, L1 R2D control information covered by the PRDCH CRC is preferred.

PDRCH

Proposal 7: The D2R scheduling information contains the following fields. - Time domain resource allocation; TBS; Modulation / coding scheme; Repetition; Frequency domain resource allocation; [line encoding]; next decode.

Proposal 8: update the RAN1#117 agreement

For L1 D2R control information (if defined), the following are not considered for further study:

- CSI feedback

- Autonomous SR

- FFS: Whether any other L1 D2R control information is needed or not

Proposal 9: For the PDRCH, support a common design using a CRC.

Proximity determination

Observation 2: Proximity determination is based on a reader monitoring D2R transmissions from an A-IoT device in response to a R2D transmission from a reader.

Proposal 10: For proximity determination, if reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

- FFS: Details on reception criteria (e.g. either successful or not) at reader and device

- FFS: Whether to further require additional criteria to be met on the received D2R transmission

Intermediate UE

Observation 3: When an intermediate UE transmits an R2D transmission, the transmission power of the R2D transmission may impact gNB reception operation.

Observation 4: An intermediate UE may be unable to perform Uu operations while performing A-IoT operations.

Proposal 11: For Topology 2, support the operation of an intermediate UE where the intermediate UE is configured with a resource pool indicating frequency resources, power levels, and time when the resource pool is active.

    - FFS: whether/how the gNB can signal when the intermediate UE should perform R2D transmissions

    - FFS: whether/how the intermediate UE operates in autonomous mode

# References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Mar. 18-21, 2024.

R1-2404292, “FL summary#3 on downlink and uplink channel/signal aspects”, Apple, RAN1#117, May 20-24, 2024

RP-240854, “Moderator's summary on R19 Ambient IoT”, Huawei, RAN#103, Mar. 18-21, 2024.

RP-241688, “Moderator's summary for offline discussion about ambient IoT”, Huawei, RAN#104, Jun. 17-20, 2024

3GPP TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN (Release 18)”, v18.0.0 (2023-12)

R1-2403862, “D2R and R2D Channel/Signal Aspects for Ambient IoT”, Futurewei, RAN1#117, May 20-24, 2024

R1-2405803, “Frame Structure and Timing Aspects for Ambient IoT”, Futurewei, RAN1#118, Aug. 19-23, 2024

“EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Specification for RFID Air Interface Protocol for Communications at 860 MHz – 960 MHz Version 2.0.1 Ratified”

R1-2405696, “Session notes for 9.4 (Study on solutions for Ambient IoT in NR)”, Chairman, RAN1#117, May 20-24, 2024

R1-2405800, “Discussion on evaluation assumptions and results for Ambient IoT devices”, Futurewei, RAN1#118, Aug. 19-23, 2024

# Appendix

## Appendix A: Study item objectives [1]

This study targets a further assessment at RAN WG-level of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

Endorsed agreement following the SID [3].

Proposal 2 - Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19 - The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.” - No SID revision is necessary

Discussion from RAN#104 [4].

Moderator’s proposal - Regarding the proposal in RP-240929, RAN1 continues down-selection between option 1 and option 2 unless companies agree to down-select at RAN#104. - Check whether to limit the study on proximity determination only for mono-static scenario with the CW inside topology. - Clarify that the use case for proximity determination is to identify whether a device is less than [1] meter away from the reader, and that the determination should not be different for devices with different transmit powers. Offline conclusion: RAN1 to continue discussion on option 1 and option 2 for proximity determination. Continue discussion on the use case /design target clarification this week.

## Appendix B. Feature lead recommendations for RAN1#118 [2]

R2D - R2D Control information Companies can consider a like below to discuss what R2D control information is needed for PRDCH and PDRCH, respectively. And if needed, whether it is signaled via higher-layer or L1 R2D control signaling Information; Needs to be Signaled?; Via L1 Control; Via higher-layer. Information Type 1; Y/N; Y; N. - Further consider discussing details in terms PRDCH generation/mapping schemes/formats (if needed) to support carrying combination of one or more information types include R2D data, L1 R2D control (if any) and L1 system information (if any) - Additionally consider how coding/CRC attachment can be handled in above combinations, if needed - Further discuss design details and potential down-selection for start-indicator part for R2D - Further discuss design details for clock-acquisition art including - sequence/pattern - Length - Line coding, if needed - additional functionalities if any - Whether it provides chip duration for PRDCH with R2D data, system information. D2R - For D2R control information, please consider whether/what information might need to be signaled via L1 D2R control information - Further consider discussing details in terms PDRCH generation/mapping schemes/formats (if needed) to support carrying combination of one or more information types include D2R data, L1 D2R control (if any) - Additionally consider how coding/CRC attachment can be handled in above combinations, if needed - Consideration on Msg1 design and whether/how it is carried by PDRCH or not - Further discuss design details for preamble preceding PDRCH - sequence/pattern - Length - Any other details Proximity Determination - Further discuss details and potential preference for the two proximity determination options agreed in this meeting. - Consider aspects that may impact the determination of device as “near”, e.g. power transmission - Provide considerations on proximity determination in case when transmit reader and receive reader are different - Whether to study this scenario or not - If yes, how proximity determination is done relative to transmit and/or receive reader Intermediate UE in topology 2 - Considerations on intermediate UE in case of topology 2 - How to handle control of intermediate UE? - If higher-layer and/or L1-signaling needed? If yes, design and procedure considerations