3GPP TSG RAN WG1 #118			R1-2406086

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.2.4

Source:	China Telecom, ZTE

Title:	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion

- Introduction

The new Rel-19 WI of NR MIMO Phase 5 was approved in RAN#102 meeting, and the following objective is included in [1].

5. Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 a. Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

Asymmetric DL sTRP/UL mTRP deployment scenarios can be beneficial from the aspects of network energy saving, uplink coverage and throughput improvement. Further enhancements are needed to ensure the performance of such heterogeneous network, especially the SRS power control and UL pathloss calculation. In this contribution, we provide our views on the essential enhancements for asymmetric DL sTRP/UL mTRP network deployment.

- Considerations on TA enhancements for asymmetric DL sTRP/UL mTRP deployment

In last meeting [2], the following instruction was given for further discussion.

To fulfil the asymmetric DL sTRP/UL mTRP deployment scenarios, support two TAs for single DCI based multi-TRP/panel and single TRP. - Reuse Rel-18 specification of two TA for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured. Companies are encouraged to consider above for further discussion in RAN1#118

The average UPT gain with asymmetric DL sTRP/UL mTRP deployment can be more than 20% according to [3] in the typical scenario as shown in Fig 1, while for cell-edge UEs, the maximum UPT gain can be up to 31%. But for such UEs at cell-edge, the distance from them to macro and micro gNB can be quite different, which will further result in the non-negligible difference of TA. Thus, from our prospective, 2TAs for DL sTRP/UL mTRP should also be enhanced in asymmetric DL sTRP/UL mTRP scenarios using the legacy PRACH resources. We think above is also the common understanding of all the companies according to the process of last meeting.

Fig 1 illustration of asymmetric DL sTRP/UL mTRP deployment scenario

Though enhancement on TA is not included in WID, based on the simulation and calculation results in [4], in some cases, the uplink propagation difference between DL macro and UL micro nodes cannot meet the timing error limit Te as specified in TS 38.133. Thus, supporting 2TA for the asymmetric DL sTRP/UL mTRP scenario in specs is necessary to promote the deployment of asymmetric deployment in practical network.

Proposal 1: Support to specify 2TA for the asymmetric DL sTRP/UL mTRP scenario.

In current,.2TAs has been introduced in Rel-18 when mDCI are used for mTRP, but the ‘coresetPoolIndex’ needs to be configured.[5] But if 2TAs is supported for single DCI based mTRP, such restriction is not needed, the other rules can be reused.

Proposal 2: Support to reuse Rel-18 specification of two TA for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured to support two TAs for single DCI based multi-TRP/panel and single TRP.

- Considerations on PRACH PL offset indication

In RAN1 #117 meeting[2], the following agreement was reached on the offset indication of PDCCH-ordered PRACH transmission.

Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting: - Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS: the details of DCI field design.

From our perspective, both alternatives have their own cons and pros. For Alt1, it is the simplest and most straightforward alternative to configure multiple PL offset values in a list at one time and then indicating by DCI. And network has the flexibility to configure any values as needed. Network can always update the PL offset for PDCCH-ordered PRACH in the DCI 1_0 at any time. However, such since the possible value range of pathloss offset can be large, it is difficult for network to determine the length and values of the PL offset list. And if the values are not configured appropriately, MAC CE signaling for updating the list is still needed, which increase the complexity and burden of network.

Observation 1: For indicating the PL offset for PDCCH-ordered PRACH transmission, Alt1 is more straightforward and flexible to configure and update the PL offset value, but may be difficult and complex to determine the exact offset values.

For Alt3, the configuration is associated with the PL offset of joint/UL TCI for UL TRP, thus there is no extra payload and steps to update the PL offset for PDCCH-order PRACH specifically, and this can be beneficial since the PL offset for different channel and signals are likely to be the same between the same UE and TRP. But this alternative may not be effective since PL offset update occasion is limited, the actual PL offset can have been changed but the MAC CE for updating the value hasn’t been transmitted before the PDCCH-order PRACH, which will resulting in the PL offset for PDCCH-order PRACH is not accurate enough.

Observation 2: For indicating the PL offset for PDCCH-ordered PRACH transmission, Alt2 is simpler to configure and update the PL offset value, but may be not be effective enough.

- Considerations on PL offset configuration for UL TRP

In RAN1 #117 meeting, the following agreements were reached on PL offset configuration for UL TRP.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, study and decide the value range and candidate values of PL offset value Agreement For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS

3.1 value of PL offset

The PL offset is defined as the PL of DL TRP – PL of UL TRP, which means if the value is positive, the UE is closer to UL micro TRP, otherwise UE is closer to DL Macro TRP. In the reality network, even if UE is closer to the Macro TRP, extra gain on UL throughput can be acquired with UE access to both Macro and micro TRP. Thus, both negative and positive values should be supported for PL offset.

Observation 3: Both negative and positive values should be supported for PL offset configuration.

Fig 2. Distribution of Macro DL and micro UL TRPs used in simulation

We simulate the pathloss offset between UE to DL TRP and UL TRP, the deployment of TRPs is shown in figure 2, and the simulation parameters can be found in Appendix. The CDF curve of the PL offset is shown in figure 3. From which we can conclude that the PL offset varies from -60dB to 60dB, which should be the possible value for PL offset.

Fig 3 Simulation results of the PL offset

Proposal 4: Support to introduce the possible value of PL offset from -60 to 60dB.

3.2 PHR calculation based on pathloss offset

Regardless of PUSCH or SRS, the transmission power should always be monitored and be adjusted. For Type 1 PHR, it is natural to take the PL offset into consideration for adjusting the transmission power of PUSCH/SRS, regardless based on actual PUSCH transmission or reference PUSCH.

Proposal 5: Support to adopt the indicated PL offset for Type 1 PHR based on both actual PUSCH and reference PUSCH.

The simplest method to adopt the PL offset in calculation of PHR is UE updating the value of 𝑃𝐿𝑏,𝑓,𝑐(𝑞𝑑) in the current PHR calculation formulation[5] with (𝑃𝐿𝑏,𝑓,𝑐(𝑞𝑑) + PL offset) up to implement, but there is no need to modify the formulation in current specs.

Proposal 6: For the PHR where PL offset is taken into consideration, i.e., at least for Type 1 PHR based on actual and reference PUSCH, there is no need to update the calculation formulation in TS 38.213.

While for Type 3 PHR, in current specs it is not expected to be transmitted if Type 1 PHR is configured. Thus, since there must be Type 1 PHR transmitted in the asymmetric deployment, it seems that Type 3 PHR will never be reported for UE supporting the asymmetric DL sTRP/UL mTRP deployment. However, regardless whether the PL offset should be considered, Type 3 PHR is still needed since UE may need to report Type 3 PHR to DL TRP to acquire DL CSI-RS. Thus, Type 3 PHR should still be supported for such UEs, but whether the PL offset be taken into consideration may be further discussed.

Proposal 7: Type 3 PHR should be supported for UE who supports the asymmetric DL sTRP/UL mTRP scenario.

- FFS: whether to take PL offset into consideration for calculation of Type 3 PHR.

- Considerations on two separate CLPC for SRS

It has been agreed to use DCI format 2_3 for indicating the CL PC for SRS in the asymmetric DL sTRP and UL mTRP scenario. However, in legacy, DCI format 2_3 is introduced for TPC command indication for SRS carrier switching, and it is an optional UE feature (FG 8-6). Thus, for UE not supported SRS carrier switching, the DCI format 2_3 may not be needed supported, i.e., UE cannot decode DCI format 2_3 with scrambled by TPC-SRS-RNTI. Which means if only DCI format 2_3 is supported for indicating TPC command for two separate SRS CLPC, this feature will be bundled with TPC command for SRS carrier switching. Such mechanism is not flexible and unreasonable for UE.

Observation 4: Only support DCI format 2_3 to indicate the TPC command for SRS when two separate CLPC adjustment states are configured will results in the feature bundled with FG 8-6, which will decrease the flexibility of UE capability.

As proposed by companies [6], we think DCI format 0_1/1_1 can also be supported for indicating TRC command for SRS when two separate CLPC adjustment states are configured to decouple the two features, so that UE can support the separate CLPC for 2 SRS without supporting DCI format 2_3. Just as the enhancement on DCI format 2_3, 2-bit TPC command and 1-bit closed-loop indicator field should be introduced for the enhancement of DCI format 0_1/1_1.

Proposal 8: Support to additionally enhance DCI format 0_1/1_1 to indicate TPC command for SRS with two separate SRS CLPC adjustment states configured as an option.

- Introduce 1-bit closed-loop indicator and 2-bit TPC command field in DCI format 0_1/1_1.

- Conclusion

In this paper, we discuss the issues for asymmetric DL sTRP/UL mTRP scenarios and have the following observations and proposals.

Observation 1: For indicating the PL offset for PDCCH-ordered PRACH transmission, Alt1 is more straightforward and flexible to configure and update the PL offset value, but may be difficult and complex to determine the exact offset values.

Observation 2: For indicating the PL offset for PDCCH-ordered PRACH transmission, Alt2 is simpler to configure and update the PL offset value, but may be not be effective enough.

Observation 3: Both negative and positive values should be supported for PL offset configuration.

Observation 4: Only support DCI format 2_3 to indicate the TPC command for SRS when two separate CLPC adjustment states are configured will results in the feature bundled with FG 8-6, which will decrease the flexibility of UE capability.

Proposal 1: Support to specify 2TA for the asymmetric DL sTRP/UL mTRP scenario.

Proposal 2: Support to reuse Rel-18 specification of two TA for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured to support two TAs for single DCI based multi-TRP/panel and single TRP.

Proposal 3: Methods not supporting MAC CE to update the PL offset configuration, i.e., Alt1, Alt2a and Alt4 should not be further considered.

Proposal 4: Support to introduce the possible value of PL offset from -60 to 60dB.

Proposal 5: Support to adopt the indicated PL offset for Type 1 PHR based on both actual PUSCH and reference PUSCH.

Proposal 6: For the PHR where PL offset is taken into consideration, i.e., at least for Type 1 PHR based on actual and reference PUSCH, there is no need to update the calculation formulation in TS 38.213.

Proposal 7: Type 3 PHR should be supported for UE who supports the asymmetric DL sTRP/UL mTRP scenario.

- FFS: whether to take PL offset into consideration for calculation of Type 3 PHR.

Proposal 8: Support to additionally enhance DCI format 0_1/1_1 to indicate TPC command for SRS with two separate SRS CLPC adjustment states configured as an option.

- Introduce 1-bit closed-loop indicator and 2-bit TPC command field in DCI format 0_1/1_1.

- References

3GPP RP-234007, “New WID: NR MIMO Phase 5”, Samsung (Moderator), RAN#102, Dec 11th- 15th, 2024.

Chair's Notes RAN1#117, May, 2024, 3GPP RAN1

3GPP RP-233609, “Views on MIMO evaluation in Rel-19”, ZTE, RAN #102 meeting, Dec., 2023.

3GPP R1-2404242, “Discussion on enhancements for asymmetric DL sTRP UL mTRP scenarios”, ZTE, China Telecom, RAN1 #117 meeting, May. 2024

3GPP TS 38.213, “NR; Physical layer procedures for control (Release 18)”.

3GPP R1-2404532 “Enhancement for asymmetric DL sTRP UL mTRP scenarios”, Ericsson, RAN1#117, May, 2024

[7] ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”

Appendix. Simulation Assumptions for PL offset

| - Parameters | - value |
| --- | --- |
| - Carrier frequency | - 3.5GHz |
| - Height of DL TRP(m) | - 25 |
| - Height of UL TRP(m) | - 5 |
| - Height of UE(m) | - 1.5 |
| - ISD of DL TRP(m) | - 500 |
| - PL model | - Table A1-3 in [7], NLOS PL model for Uma_A scenario |