3GPP TSG RAN WG1 #118                                                                                          R1-2406146

Maastricht, NL, August 19th – 23rd, 2024

Source:	vivo

Title:	Discussion on LS on DL coverage enhancements

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

According to the LS from RAN2 [1], RAN2 has studied the DL coverage enhancement and identified some questions. RAN1 is asked to provide feedback on the following questions.

Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values? Question2 : Can RAN1 provide the information on whether/how the solution RAN1 is investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive? Question3 : Can RAN1 provide the feedback on whether UL beam hopping is also being studied in RAN1 (and whether this is separate from DL beam hopping)? Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE. Question5 : Can RAN1 provide the feedback on whether the beam status in different beams (visible to the UE) of one cell are the same or can be different in any given time, i.e.. the beam status is cell specific or beam specific?

In this contribution, we provide our views and considerations on the identified questions from the LS. A draft reply is proposed according to the discussions.

- Discussions

At the RAN1#117 meeting, some observations on the results of DL coverage ratio evaluation were achieved. RAN1 will further investigate the impact of SSB periodicity extension. As per the latest WID [2] as followed, SSB channel enhancement other than SSB periodicity extension is not considered. That means the existing SSB pattern for an NR cell is not expected to be changed, and SSB periodicity extension could be further discussed. Furthermore, in the next step, RAN1 would analysis impact of SSB periodicity extension on the legacy UE behavior, such as UE’s cell search complexity, and impact to initial cell selection, latency and success rate.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

Observation 1: As per the latest WID, the existing SSB pattern for an NR cell is not expected to be changed, and SSB periodicity extension is still under discussion.

Regarding the DL coverage enhancement, RAN1 has achieved some observations based on LLS results on common control signaling and system level results of DL coverage ratio evaluation. However, the results for one channel or signal were obtained independently from the performance of other channels or signals, and no targeted physical channel/signal for coverage was identified. Thus, RAN1 solution needs further investigation and discussion, and currently there is no consensus on whether/how to enhance the common control signalling for UEs in RRC idle/RRC inactive.

Observation 2: Till now there is no conclusion on whether/how to enhance the common control signaling for UEs in RRC idle/RRC inactive.

RAN1 has not investigated on UL beam hopping. There is no issue on UL beam hopping identified yet. The WID does not have a clear objective on any enhancements of UL beam hopping, either. In our understanding, UL beam hopping is not necessary from a UE transmitting perspective, while UL reception beam hopping at the satellite can be handled at least by a network implementation.

Observation 3: RAN1 has not investigated on UL beam hopping, and no issue on UL beam hopping is identified.

In RAN1#116 meeting, the following agreement for DL coverage enhancement evaluation at system level was achieved. It is worth noting that these beam status (i.e. "off", "common messages only" and "active traffic") are defined only for analytical evaluation at system level study. For example, based on these beam statuses, it would be easier to evaluate performance metrics, e.g. ratio of mean served cell throughput, ratio of common control channels and corresponding coverage ratio. There is no mean to introduce and to specify these beam statuses.

Agreement RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level: At least: - CDF of the received SINR - The dwell time and revisit time interval for each beam illumination across the coverage - Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio Other metrics may be reported such as - CDF of the cell throughput - CDF of user perceived throughput (UPT) - CDF of Latency - Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889) For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access - N1 + N2 + N3 = “Total number of beam footprints “ - N1, N2, N3, X are to be reported by companies. - Resource utilization obtained under the assumptions above is to be reported by companies. - Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Observation 4: RAN1 does not intend to define or specify the beam status to the UE.

According to WID, dynamic and flexible power sharing between satellite beams is allowed, and different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint is also allowed, which implicitly indicates the beam status in different beams (visible to the UE) of one cell can be different. Whether/how different beams with beam specific status at one satellite are divided into one cell or not can be up to network implementation.

Observation 5: Whether the beam status in different beams of one cell can be different or not is not expected to be specified, and can be handled up to network implementation.

Based on observations above, a draft reply LS is provided in our contribution [3].

Proposal 1: RAN1 sends an LS reply to RAN2 based on the draft reply provided in R1-2406147.

- Conclusion

In this contribution, we discuss the RAN2 LS on DL coverage enhancement. According to the discussions, we have the following observation and proposal.

Observation 1: As per the latest WID, the existing SSB pattern for an NR cell is not expected to be changed, and SSB periodicity extension is still under discussion.

Observation 2: Till now there is no conclusion on whether/how to enhance the common control signaling for UEs in RRC idle/RRC inactive.

Observation 3: RAN1 has not investigated on UL beam hopping, and no issue on UL beam hopping is identified.

Observation 4: RAN1 does not intend to define or specify the beam status to the UE.

Observation 5: Whether the beam status in different beams of one cell can be different or not is not expected to be specified, and can be handled up to network implementation.

Proposal 1: RAN1 sends an LS reply to RAN2 based on the draft reply provided in R1-2406147.

References

- R2-2405767, LS on DL coverage enhancements, 3GPP TSG RAN WG2#126, May 20th - 24th, 2024.

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, 3GPP TSG RAN Meeting#104, June 17-20, 2024.

- R1-2406147, Draft reply LS on DL coverage enhancements, vivo, August 19th – 23rd, 2024.