3GPP TSG RAN WG1 #118 	R1-2406157

Maastricht, NL, August 19th – 23rd, 2024

Source: 	vivo

Title:	Discussion on PHR report for mDCI-based STxMP

Agenda Item	8.1

Document for:	Discussion and Decision

Introduction

Agreement in RAN1#114 On unified TCI framework extension for S-DCI based MTRP, if twoPHRMode is configured, and two SRS resource sets for CB/NCB and multipanelScheme for SDM/SFN are configured: - If the UE determines that one or both Type 1 PHRs are based on an actual PUSCH transmission - If the actual PUSCH transmission applies both first and second indicated joint/UL TCI states, the UE provides the first {power headroom, configured maximum output power} associated with the first indicated joint/UL TCI state for the actual PUSCH transmission, and the second {power headroom, configured maximum output power} associated with the second indicated joint/UL TCI state for the actual PUSCH transmission - If the actual PUSCH transmission applies only the first indicated joint/UL TCI state, the UE provides the first {power headroom, configured maximum output power} associated with the first indicated joint/UL TCI state for the actual PUSCH transmission - FFS: How to provide the second report for a reference PUSCH transmission? - If the actual PUSCH transmission applies only the second indicated joint/UL TCI state, the UE provides the second {power headroom, configured maximum output power} associated with the second indicated joint/UL TCI state for the actual PUSCH transmission - FFS: How to provide the first report for a reference PUSCH transmission? - FFS: If the UE determines that both Type 1 PHRs are based on reference PUSCH transmissions, how to provide the first and second reports for reference PUSCH transmissions, respectively Agreement in RAN1 #114bis On unified TCI framework extension for S-DCI based MTRP, if twoPHRMode is configured, and two SRS resource sets for CB/NCB and multipanelScheme for SDM/SFN are configured: - If the UE determines that only one Type 1 PHR is based on an actual PUSCH transmission - If the actual PUSCH transmission applies only the first indicated joint/UL TCI state, the UE provides the second {power headroom, configured max output power} associated with the second indicated joint/UL TCI state for a reference PUSCH transmission - If the actual PUSCH transmission applies only the second indicated joint/UL TCI state, the UE provides the first {power headroom, configured max output power} associated with the first indicated joint/UL TCI state for a reference PUSCH transmission - If the UE determines that both Type 1 PHRs are based on reference PUSCH transmissions, the UE provides the first {power headroom, configured max output power} associated with the first indicated joint/UL TCI state for a reference PUSCH transmission, and the second {power headroom, configured max output power} associated with the second indicated joint/UL TCI state for another reference PUSCH transmission - FFS: Whether the configured max output power reported in above cases is per UE or per panel or both Conclusion in RAN1#115 There is no consensus in RAN1 to support the report of P-MPR for unified TCI framework extension for S-DCI based MTRP, if twoPHRMode is configured, and two SRS resource sets for CB/NCB and multipanelScheme for SDM/SFN are configured.

In Rel-18, two PHR reports associated with the first and second indicated TCI states for PUSCH are supported to report power headroom of two panels. Besides, maximum configured output power for each panel is also reported along with PH values to assist UL power control. However, due to time limitation, PHR report for mDCI-based sTXMP was not discussed even though it is also vital for gNB to adjust power of UL transmission. In this paper, we analyze potential solutions to support the scenario.

PHR report for mDCI based sTXMP

Two PH values together with two maximum configured output power are reported for sDCI-based sTXMP PUSCH transmission. For mDCI scheduled PUSCHs overlapping in time domain, following two options can be considered to enable PHR report.

- Option1: report one PHR for one PUSCH associated with one indicated TCI state

- Option2: support two PHRs based on two PUSCH associated with two indicated TCI states

Option 1 is simple from specification perspective. But further clarification is needed for the case that UE would transmit two PUSCHs in slot n that overlap in at least one symbol. In this case, UE behavior of which PUSCH is based on to report PHR should be clarified so that gNB can acquire power headroom for one panel.

In our view, Option2 is better than Option1 because Option2 provides more information to gNB for panel-specific power control. In Option1, a single, meaning that gNB cannot acquire power headroom information of the other panel with a single PHR reporting for one PUSCH transmission associated with one panel. Meanwhile, the maximum configured output power for the other panel is also unknown at gNB side. Consequently, power control signaling for the other panel would become invalid with high probability. On the contrary, with knowledge on maximum transmission power and power headroom for each panel through the reported two PHR regardless of they are based on actual or reference transmission in option2, panel-specific power control can be implemented efficiently. For progress, PHR MAC CE designed for sDCI-based sTXMP transmission can be reused as two PHR report for mDCI-based sTXMP.

Reporting two PHRs and two maximum configured power for mDCI-based STXMP in one MAC CE is supported.

If two PHR reporting is supported, whether these PHRs is actual or virtual can be determined similarly as two PHR report for sDCI-based mTRP PUSCH transmission. As shown in Figure 1, PHR report for three scheduling cases are illustrated and the corresponding description is as follows.

PHR report for mDCI scheduled PUSCHs for a serving cell c assuming PHR is reported in slot n

If UE is provided twoPHRMode, - if the UE provides a first Type 1 power headroom report and a first configured maximum output power for an actual PUSCH of a PUSCH transmission starting earliest in slot n that is associated with one TCI-State or TCI-UL-State, - if the UE transmits PUSCH associated with the other TCI-State or TCI-UL-State in slot n, the UE provides a second Type 1 power headroom report and a second configured maximum output power for an actual PUSCH associated with the other TCI-State or TCI-UL-State that overlaps with slot n - else, the UE provides a second Type 1 power headroom report and a second configured maximum output power for a reference PUSCH transmission associated with the other TCI-State or TCI-UL-State, where the second Type 1 power headroom report using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the TCI-State or TCI-UL-State. - else, UE provides a Type 1 power headroom report and a first configured maximum output power for a reference PUSCH transmission associated with the first TCI-State or TCI-UL-State and a Type 1 power headroom report and a configured maximum output power for a reference PUSCH transmission associated with the second TCI-State or TCI-UL-State, where - the UE provides the first and the second Type 1 power headroom reports using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State and with the second TCI-State or TCI-UL-State, respectively.

Whether these two PHRs are actual or virtual can be determined similarly as two PHR report for sDCI-based mTRP PUSCH repetition.

Similar as two PHR reporting in sDCI-based sTxMP, twoPHRmode should be configured optionally by RRC. If twoPHRmode is not provided, and if the Type 1 power headroom report is for an actual PUSCH transmission and UE would transmit two PUSCHs overlapping in time domain in slot n, or the Type 1 power headroom report is for a reference PUSCH transmission, clarification on which actual or reference PUSCH transmission should be based is required. One simple solution is that the PUSCH transmission associated with the first TCI state is used. Type 1 power headroom report for the actual PUSCH transmission or the reference PUSCH transmission is associated with the first TCI-State or TCI-UL-State.

If twoPHRmode is not provided and for the case UE cannot decide which PUSCH should be based, one Type 1 power headroom report should be based on the actual PUSCH transmission or the reference PUSCH transmission associated with the first TCI-State or TCI-UL-State.

In [1] we provide corresponding draft CR to capture the PHR report for mDCI-based PUSCH+PUSCH sTxMP.

Conclusion

In this contribution, following proposals are presented:

- Reporting two PHRs and two maximum configured power for mDCI-based STXMP in one MAC CE is supported.

- Whether these two PHRs are actual or virtual can be determined similarly as two PHR report for sDCI-based mTRP PUSCH repetition.

If twoPHRmode is not provided and for the case UE cannot decide which PUSCH should be based, one Type 1 power headroom report should be based on the actual PUSCH transmission or the reference PUSCH transmission associated with the first TCI-State or TCI-UL-State.

References

R1-2406156,” Draft CR on M-DCI based PUSCH+PUSCH STxMP”.