3GPP TSG RAN WG1 #118                                                                                  R1-2406363

Maastricht, NL, August 19th – 23rd, 2024

Source:	CATT

Title:	On UE-initiated/event-driven beam management

Agenda Item:	9.2.1

Document for:	Discussion and Decision

Introduction

In RAN1 #117 meeting, following agreements and working assumptions were achieved on UE-initiated/event-driven beam management [1].

Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case. Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured - FFS: Option-3b/3c - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH. - The UL-grant DCI format at least comprises DCI format 0_1/0_2. - FFS: DCI format 0_3 - FFS: How to trigger the UEI beam report by the UL-grant DCI format - FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH. - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report. - The DL-grant DCI format at least comprises DCI format 1_1/1_2. - FFS: DCI format 1_3 Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported) - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality. Agreement Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118: - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration; - FFS: The RS in the RS resource set can be updated by MAC-CE. - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. - Others are not precluded. - FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement (i.e., in such case, Scheme-2 is used) - Others are not precluded.

In this contribution, related issues on UE-initiated/event-driven beam management are discussed, including triggering events, RS resources for beam measurement, UL report contents and UL report container.

Discussion

  - Triggering events

In RAN1#117 meeting, multiple candidate events are precluded while the following three events are remained for further consideration [1]:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

It is essential to decide whether the surviving events should be supported or not. In the following, analysis and views with respect to the events are provided.

For event-1, beam reporting is beneficial for UE to request the gNB to pursue beam measurement before beam failure happens, while the process is highly similar to the BFR procedure, thus, the support on such an event is controversial.

Both event-7a and event-7b are beneficial to fast beam switching regarding the activated TCI state list. Based on event-7a or event-7b, gNB can acquire the knowledge that whether one or multiple activated TCI states can be replaced by a better beam. Event-7a may cause numerous reporting as the triggering condition is as long as the new beam is better than the worst beam within the activated TCI states. On the other hand, triggering event-7b is not frequently happened and is beneficial for fast beam acquisition for gNB to perform more effective beam management.

Based on the statements above, as valid beam information is beneficial for gNB performing effective beam management, it is beneficial to introduce more triggering events.

Proposal 1: On UE-initiated/event-driven beam management, support more trigger events, e.g., support event-7b in addition to event-2 as the triggering event.

  - RS resources for beam measurement

In RAN1 #117 meeting, it is agreed that two schemes are used for the measurement RS of current beam. One remaining issue is that whether the enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. Generally, different RS types have different beam width, number of ports, and transmission power. If the types are different, comparing the beam quality of SSB and CSI-RS is meaningless. Hence, UE should not expect that the measurement resource of the current beam and the measurement resources of the new beams are different in RS type, i.e. one is SSB and the other is CSI-RS.

Proposal 2: On UE-initiated/event-driven beam management, confirm the following working assumption:

(Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

Regarding RS measurement for the new beam for Event-2, one explicit manner (option-3a): The RS(s) for new beam(s) are explicitly configured was agreed as the baseline in RAN1#117 meeting. There are three options for down-selection:

- Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration;

  - FFS: The RS in the RS resource set can be updated by MAC-CE.

- Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

  - FFS: If a list size is small, MAC-CE activation is not needed

- Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state.

Option-1 aligns with the WID description mostly: Leverage legacy CSI measurement and reporting configuration frameworks as much as possible. For Event-2 with L1-RSRP as report quantity, a CSI-ReportConfig can be associated with a CSI-ResourceConfig, among which one resource set can be used for UE-initiated Beam Management. For the FFS in option-1, to avoid frequent RRC re-configuration of the RS resource set leading to unnecessary latency and overhead, the MAC-CE updating of the RS in the RS resource set can be supportive. Option-2 and Option-3 deviate from the legacy CSI configuration framework due to the fact that the list of RS resource sets should be associated with a CSI-ReportConfig instead of a TCI state or any other resource sets. Based on the statement above, option-1 is preferred.

Proposal 3: Regarding explicit RS configuration for new beam management for Event-2, Option-1 is supported. i.e., the RS(s) for new beam(s) are explicitly configured in one RS resource set associated with a CSI reporting configuration.

Regarding RS measurement for the new beam for Event-2, another issue to be discussed is whether support the implicit methods. Two implicit methods have been proposed for further discussion:

  - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s).

  - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

For option-3b, one possible use case is to update the indicated current serving beam among the activated beams, these QCL source RS signals of the activated TCI-state can be implicitly used as measurement RS resource rather than explicitly signaled. However, one concern for option-3b is that currently the QCL source RS for PDSCH in the unified TCI state can be only CSI-RS for beam management or TRS, if the RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s), it seems that only CSI-RS can be used as RSs for new beams for option-3b, which is contradictory with the agreement that support L1-RSRP as a measurement quantity on SSB.

For Option-3c, with the agreement that supporting L1-RSRP as a measurement quantity on SSB, applying the QCL RS(s) of the configured TCI-states as the measurement resource may lead to an unfair comparison between CSI-RS beam and SSB beam which have different beam width and transmission power, number of ports, etc.

As above, it is not necessary to support implicit methods for RS measurement for new beam for event-2.

Proposal 4：For RS measurement for new beam for event-2, do not support option-3b or option 3c.

Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. In RAN1#117 meeting, following options were proposed:

- Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state;

- Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP;

- Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement).

- Option-4: No further enhancement (i.e., in such case, Scheme-2 is used)

- Others are not precluded.

Currently, only CSI-RS for beam management and SSB can be used as the measurement RS while TRS can not be used. Several options are provided to resolve this issue. Option-1 introduces additional scheme that the TRS for current beam must be associated with a CSI-RS for beam management derived from the QCL RS in the indicated TCI state. However, TRS is mostly used for wide beam case and may not be used for narrow beam cases thus, such an association may not be able to achieve. Option-2 supports TRS as the measurement RS for current beam, which is not used in current spec. Additionally, the scope of UEIBM states that leveraging the legacy CSI framework, thus, it is not essential to support TRS as the measurement RS. Option-3 introduces additional scheme that the RS for current beam can be explicitly configured by RRC or MAC-CE, however, this follows a scheme (explicitly configuration for measurement RS for current beam) which is already being precluded in a previous agreement. It is not essential to re-discuss a precluded scheme. Option-4 proposes to introduce no further enhancement, while using scheme-2 in this case. It is not a complete solution. By contrast, UE can suspend the UEIBM report when only one TSS is configured in the indicated TCI state. As the RS of the current beam is from the indicated TCI state, UE can still perform UEIBM report as long as the indicated TCI state has changed to be one not configured with only one TRS.

Proposal 5：Regarding RS measurement for the current beam for event-2, when only one TRS is configured in the indicated TCI state, support one of the following schemes:

- Alt1: No further enhancement, i.e., UE suspends the measurement for UEIBM until the indicated TCI state has changed to be one not configured with only one TRS.

- Alt2: If there exists a CSI-RS QCLed with the TRS configured in the indicated TCI state, uses the CSI-RS as the measurement RS, otherwise, UE suspends the measurement for UEIBM until the indicated TCI state has changed to be one not configured with only one TRS or to be one configured with only one TRS but there exists a CSI-RS associated with the TRS.

  - UL report content

For the report content of UEIBM, one remaining issue is the detailed design on the reporting scheme regarding the L1-RSRP format. In this case, the differential reporting can be used. Specifically, UE always reports the absolute value of the best beam in the first place. then the differential value between the best beam and the current beam is reported in the second place, then report the differential values of other beams. By doing so, gNB could gain the knowledge of the best beam and the differential values between it and the current beam in the first places. In this case, the reporting can be achieved in an efficient manner.

Proposal 6: For the report content for event-2 in UE-initiated/event-driven beam management, support the differential report scheme.

Proposal 7: For the differential report, following reporting scheme can be used, e.g., the best beam is reported in the first place, the differential value between the best beam and the current beam is reported in the second place, and the differential values of other beams are reported.

In RAN1#117 meeting, it is agreed that whether the current beam is always reported or not is indicated by a new RRC parameter. When current beam is always reported is disabled by this RRC parameter, N beams are reported. However, whether to indicate the current beam and what should be indicated in this case needs to be discussed. When the RRC is disabled, gNB does not know whether the current beam is reported or not, to acquire more knowledge for beam management, it is necessary to indicate whether the current beam is reported or not. One possible solution is a 1-bit indication which would be sufficient for this case. If current beam is reported, it can be placed at a fixed position, e.g., the second/last beam is the current beam. By doing so there is no need to further indicate where the current beam is.

Proposal 8: For event-2 in UE-initiated/event-driven beam management, when current beam is always reported is disabled by RRC, a 1-bit indication is used to indicate whether the current beam is reported or not.

In RAN1#117 meeting, it is agreed that the UE initiated beam reporting can occur when a number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M. However, it is not mentioned which RSRP value is reported through the M satisfied values. As this condition is within a configured time window, the last satisfied RSRP values should be reported as it can show the latest current channel status.

Proposal 9: For event-2 in UE-initiated/event-driven beam management, when M event-2 instance(s) for one same new beam are used to trigger beam report within a configured time window, the latest RSRP value is reported for this new beam.

  - UL report container

Regarding UL report container, there was a working assumption that for the first channel of Mode A and Mode B, one-bit scheme could be used as the baseline for requesting resource a resource for a second channel/notifying a second channel for mode A and mode B, respectively. The 1-bit indication scheme for UEIBM can follow the legacy SR scheme. For mode A, the 1-bit indication is used to request a dynamic PUSCH resource for the second channel from gNB. For mode B, the 1-bit indication is used to notify gNB for a pre-configured PUSCH resource for the second channel. It is essential that confirming the working assumption as the starting point for the design of the first channel.

Proposal 10: On UE-initiated/event-driven beam management, confirm the following working assumption:

Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

For the design of the first channel for UEIBM, one remaining question is whether multi-bit scheme is used or not. If multiple trigger events are introduced, it is essential that gNB know which trigger event is used. In this case,  the triggering event can be reported to gNB in the first channel, e.g., by reporting an event-ID. In this case, a single bit indication is not sufficient. Also, different events may need different resource size in the second step, if the event-ID is reported in the first channel, the resource allocation from gNB can be more effective. Thus, it is essential to support multi-bit indication in the first channel.

Proposal 11： For UL report container of Mode A and Mode B in UEIBM, regarding to the first channel, support multi-bit scheme to carry more knowledge, e.g., event-ID.

For the design of the second channel for UEIBM, one issue to be discussed is the design of the second UL channel for carrying beam report for Mode A/Mode B in UEIBM. For Mode A, it is agreed that the second channel in step-3 is at least PUSCH. For Mode B, following alternatives were discussed in RAN1#117 meeting:

- Alt-1: The pre-configured resource(s) for the second channel in Step-2 can be either PUSCH or PUCCH;

- Alt-2: The pre-configured resource(s) for the second channel in Step-2 is at least PUCCH;

- Alt-3: the pre-configured resource(s) for the second channel in Step-2 is at least PUSCH.

Support both PUSCH and PUCCH will lead to much design efforts, thus, we prefer to support either PUCCH or PUSCH for the second channel. Mode B corresponds to a UCI in pre-configured resource(s) for second UL channel. The pre-configured resource(s) for the second channel can be used for other purposes when UEIBM is not reported if the second channel is PUSCH. It is beneficial for a higher resource usage efficiency compared with using PUCCH. Additionally, such resource can also be used for other users for MU-MUMO scenarios. Also, mode B can follow the same scheme as mode A to keep the design consistency.

Proposal 12: On Mode B (UCI in pre-configured resource(s) for second UL channel) in UE-initiated/event-driven beam reporting, support Alt-3. i.e., the second channel is at least PUSCH.

For the design of the second channel for UEIBM, one issue to be discussed is whether UE receives acknowledgement with response to each step for Mode A and Mode B.

For Mode A, first UE transmits a first PUCCH to request a resource for a second UL channel to carry beam report, then, UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. The DCI to schedule a UL grant can be regarded as an ACK feedback for the SR. No additional acknowledgement is needed.

For Mode B, UE sends UCI on the pre-defined PUSCH/PUCCH resource, since there is no response mechanism from gNB, UE will transmit beam reporting on the subsequent pre-configured resources. To avoid the potential collision between the first channel and the second channel, a timeline/gap should be introduced, e.g., N slots, between the first channel and the second channel for UEIBM. UE will transmit the second channel after N slots, where the second channel can be PUSCH or PUCCH. If the second channel is PUSCH, while the resource of it is not available at the N slot, UE will transmit the second channel in the next earliest PUSCH resource.

If the second channel is PUCCH, similar approach for PUSCH can be followed. Also, a PUCCH resource indicator can be applied in the first channel to indicate the resource used for the PUCCH of the second channel. The time gap between the first channel and second channel should ensure enough processing time for the network to re-schedule the resource for the second channel.

Proposal 13: For Mode A, additional acknowledgement is not needed.

Proposal 14: On Mode B (UCI in pre-configured resource(s) for second UL channel) in UE-initiated/event-driven beam reporting, introduce a timeline/gap between the first channel and the second channel, i.e., N slots. The second channel will use the PUSCH resource after N slot from the first channel. If the resource after the N slots from the first channel is not available, UE will transmit the UEIBM report at the next earliest PUSCH resource.

Meanwhile, suppose the UCI on the PUCCH resource is missing and gNB does not receive the request UCI, gNB will allocate the pre-configured resources to other UEs or the UE sending UCI for uplink transmission, leading to an un-negligible interference to the beam reporting of this UE. Thus, it might be beneficial to introduce ACK feedback for the notification transmission in the step-1 reporting. If so, only after receiving the ACK feedback, UE could transmit the beam report. Thus, blind detection on the network side will be avoided. For the indication of acknowledge information, the following options can be considered:

- Opt-1: UE transmits the beam report in the pre-configured resource (e.g., notification is a part of beam report, like two-part UCI, where Part-1 is to indicate the information of Part-2, Part-2 is to carry beam report), if trigger event occurs. No ACK/NACK feedback is needed.

- Opt-2: Introduce ACK/NACK feedback mechanism for the notification of the transmission in the 1st step reporting.

Proposal 15: For Mode B, following options for the indication of acknowledgement information are considered:

- Opt-1: UE transmits the beam report in the pre-configured resource (e.g., notification is a part of beam report, like two-part UCI, where Part-1 is to indicate the information of Part-2 and Part-2 is to carry beam report), if trigger event occurs. No ACK/NACK feedback is needed.

- Opt-2: Introduce ACK/NACK feedback mechanism for the notification of the transmission in the step-1 reporting.

Additionally, if one-bit scheme is used in the first channel while multiple trigger events are supported, gNB does not know the triggering event, it is essential to report such knowledge to gNB, which can be reported in the second channel along with the beam identifier and beam qualities.

Proposal 16: For Mode B, if multiple trigger events are supported and if one-bit indication is used in the first channel, event-ID should be reported in the second channel.

Regarding to cross-carrier UE initiated beam report, it was agreed in RAN1#116bis meeting [2] that cross-CC beam reporting is supported for both Mode A and Mode B. One issue to be discussed is whether the measurement DL RS is only from one CC or from more CCs. In the offline discussion, the following options were proposed:

- Opt-1: Measurement DL RS(s) associated with one report configuration is only from one CC.

- Opt-2: Measurement DL RS(s) associated with one report configuration can be from one or more CCs.

In the legacy framework, for L1-RSRP measurement, all measurement RSs associated with one report setting are from one CC, which is indicated by the high layer parameter carrier. Similar method as legacy can be considered here for UE-initiated beam management for its simplicity.

Proposal 17: Regarding to cross-carrier UE initiated beam reporting, support Opt-1, i.e., measurement DL RS(s) associated with one report configuration is only from one CC.

Conclusions

In this contribution, we provide our views on the enhancements for UE-initiated beam management, including triggering events, RS resources used for beam measurement, UL report contents and UL report container. We have the following proposals:

Proposal 1: On UE-initiated/event-driven beam management, support more trigger events, e.g., support event-7b in addition to event-2 as the triggering event.

Proposal 2: On UE-initiated/event-driven beam management, confirm the following working assumption:

(Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam.

Proposal 3: Regarding explicit RS configuration for new beam management for Event-2, Option-1 is supported. i.e., the RS(s) for new beam(s) are explicitly configured in one RS resource set associated with a CSI reporting configuration.

Proposal 4：For RS measurement for new beam for event-2, do not support option-3b or option 3c.

Proposal 5：Regarding RS measurement for the current beam for event-2, when only one TRS is configured in the indicated TCI state, support one of the following schemes:

- Alt1: No further enhancement, i.e., UE suspends the measurement for UEIBM until the indicated TCI state has changed to be one not configured with only one TRS.

- Alt2: If there exists a CSI-RS QCLed with the TRS configured in the indicated TCI state, uses the CSI-RS as the measurement RS, otherwise, UE suspends the measurement for UEIBM until the indicated TCI state has changed to be one not configured with only one TRS or to be one configured with only one TRS but there exists a CSI-RS associated with the TRS.

Proposal 6: For the report content for event-2 in UE-initiated/event-driven beam management, support the differential report scheme.

Proposal 7: For the differential report, following reporting scheme can be used, e.g., the best beam is reported in the first place, the differential value between the best beam and the current beam is reported in the second place, and the differential values of other beams are reported.

Proposal 8: For event-2 in UE-initiated/event-driven beam management, when current beam is always reported is disabled by RRC, a 1-bit indication is used to indicate whether the current beam is reported or not.

Proposal 9: For event-2 in UE-initiated/event-driven beam management, when M event-2 instance(s) for one same new beam are used to trigger beam report within a configured time window, the latest RSRP value is reported for this new beam.

Proposal 10: On UE-initiated/event-driven beam management, confirm the following working assumption:

Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Proposal 11： For UL report container of Mode A and Mode B in UEIBM, regarding to the first channel, support multi-bit scheme to carry more knowledge, e.g., event-ID.

Proposal 12: On Mode B (UCI in pre-configured resource(s) for second UL channel) in UE-initiated/event-driven beam reporting, support Alt-3. i.e., the second channel is at least PUSCH.

Proposal 13: For Mode A, additional acknowledgement is not needed.

Proposal 14: On Mode B (UCI in pre-configured resource(s) for second UL channel) in UE-initiated/event-driven beam reporting, introduce a timeline/gap between the first channel and the second channel, i.e., N slots. The second channel will use the PUSCH resource after N slot from the first channel. If the resource after the N slots from the first channel is not available, UE will transmit the UEIBM report at the next earliest PUSCH resource.

Proposal 15: For Mode B, following options for the indication of acknowledgement information are considered:

- Opt-1: UE transmits the beam report in the pre-configured resource (e.g., notification is a part of beam report, like two-part UCI, where Part-1 is to indicate the information of Part-2 and Part-2 is to carry beam report), if trigger event occurs. No ACK/NACK feedback is needed.

- Opt-2: Introduce ACK/NACK feedback mechanism for the notification of the transmission in the step-1 reporting.

Proposal 16: For Mode B, if multiple trigger events are supported and if one-bit indication is used in the first channel, event-ID should be reported in the second channel.

Proposal 17: Regarding to cross-carrier UE initiated beam reporting, support Opt-1, i.e., measurement DL RS(s) associated with one report configuration is only from one CC.

References

- 3GPP TSG RAN WG1 Meeting RAN1 #117 Chairman's Notes, May. 2024.

- 3GPP TSG RAN WG1 Meeting RAN1 #116bis Chairman's Notes, April. 2024.