3GPP TSG RAN WG1 #118						                R1-2406406

Maastricht, NL, August 19th – 23rd, 2024

# Title:                  Discussion on frame structure and physical layer procedure for Ambient IoT

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	9.4.2.2

# Document for:	Discussion

- Introduction

In RAN1#117 meeting, frame structure, random access, scheduling and timing were discussed by RAN1 for Ambient IoT. And the relevant agreements can be given as below [1]:

Agreement Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission. Agreement Scheduling information of PDRCH transmission is provided by a corresponding PRDCH. Conclusion RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2. Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

In this contribution, we further discuss frame structure, device availability, random access and timing aspect for Ambient IoT.

- Frame structure

- 2.1 Timing alignment

In previous RAN1 meeting, whether to align A-IoT transmission with NR symbol or slot boundary was discussed, and the following agreement is captured.

Agreement For R2D transmission, if OFDM-based waveform is used, the start of R2D transmission from reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation.

In RAN1#117 meeting, one discussion is about whether to align the end of R2D transmission with the OFDM symbol boundary. To address this issue, it is first necessary to clarify whether the R2D transmission includes the R2D postamble. Generally, R2D transmission should include all components within the frame. Thus, R2D transmission should include the R2D postamble, if postamble is used.

Clarify that the R2D transmission includes the R2D postamble, if postamble is used.

The purpose of aligning the end of R2D transmission with OFDM symbol boundary is to avoid the cases when the generated chips for the PRDCH do not fully occupy the last OFDM symbol, which is not friendly to the implementation of DFT-s-OFDM-based transmitter. Thus, the alignment is needed and the following options can be considered:

Option#1: Use a flexible postamble to pad the last OFDM symbol for the   alignment of the OFDM symbol boundary .

Option#2: Properly design the TBS and code rate to ensure that R2D transmission terminated at the end of OFDM symbol. With this option, the postamble needs to occupy an integer number of OFDM symbols.

Option#1 needs a flexible postamble design. For example, the postamble can consist of a continuous high voltage that spans more than two chips, and it should end at an OFDM symbol boundary.

Option#2 needs a proper TBS and code rate design and fixed postamble length. For different M values, the candidate TBS may be different.

Both options can be considered, and option#1 is preferred.

To align the end of R2D transmission(including postamble), the following options can be considered:

  - Option#1: Use a flexible postamble (preferred).

  - Option#2: A proper design of TBS and code rate to ensure that R2D transmission terminated  at the end of OFDM symbols. The postamble occupies an integer number of OFDM symbols.

Given that the the start-indicator part immediately precedes the clock-acquisition part and the R2D timing acquisition signal directly precedes the transmission of a physical channel, there is no gap in R2D transmission. Therefore, apart from the end of R2D transmission, there is no need to align the following components with the OFDM symbol boundary:

- the start of clock-acquisition part;

- the start of R2D data transmission.

- 2.2 R2D frame structure components

- Postamble

During the previous RAN1 meeting, there was a discussion regarding the necessity of a postamble, but consensus was not reached.

Agreement To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information.

Option 2 is using TBS indication within the R2D control information to specify the payload size. However , TBS indication is limited in its ability to represent a range of TBS values. The more TBS values need to be indicated, the larger payload is required. In contrast, employing a postamble enables greater flexibility in accommodating various payload sizes. Therefore, Option 1 is the preferred choice. It should be noted that, the overhead of postamble should not be large compared with Option 2.

One comment in the previous meetings is that for a small transport block containing only a few bits, having a lengthy postamble results in significant resource overhead. To reduce the resource overhead, whether to attach postamble after PRDCH transmission can be also determined by the payload size, which is similar with the discussion about the attachment of CRC that is influenced by the length of R2D data. Specifically, if CRC is attached for the R2D transmission, the postamble is appended at the end of the R2D transmission as well. Otherwise, the postamble is omitted.

To determine or derive the end of PRDCH transmission, Option 1: R2D postamble immediately following the PRDCH to indicate the end of the PRDCH is preferred.

  - If CRC is attached for the R2D transmission, the postamble is appended at the end of the R2D transmission; otherwise, the postamble is omitted.

- Control field

As discussed in our contribution [2], control information can include device ID and/or device group ID for R2D and D2R transmission, device type or cast type for R2D and D2R transmission, repetition times or code rate for R2D and D2R transmission, Reader ID for R2D, scheduling information(e.g., TDRA, FDRA, chip duration) for D2R transmission. Some control information needs to be decoded before the R2D data decoding, and some control information can be transmitted through higher layers. Thus, to reduce the overhead of L1 control information, the control field to convey the L1 control information can contain at least device type or cast type, repetition times and/or code rate.

Control field should be included in R2D transmission containing at least the following aspects

- device type or cast type

- Repetition times and/or code rate

- Frame structure

To sum up, for R2D transmission, the following components should be considered: preamble, control field, postamble, data. And the following R2D frame structure is proposed.

Figure 1 R2D frame structure with separate control field

For R2D transmission, the following components should be considered: preamble, control field, postamble, data.

It should be noted that, the control field may not be a separate field, it may be included in preamble or data. It is obvious that, the coding scheme of control field should be pre-defined to enable the device to decode it correctly. Since the control field encompasses the scheduling information for data that must be ascertained prior to data decoding, it should not be encoded together with data.

Control field should be encoded separately from R2D data.

Since D2R transmission is triggered by downlink command, uplink scheduling information can also be indicated in DL MAC layer. More details can be found in our accompanying contribution [2].

- 2.3 D2R frame structure components

- Midamble

For D2R transmission, the receiver is gNB/intermediate UE, which is equipped with higher data processing capabilities, making coherent receiving methods viable. However, the coherent receiving method is challenging to implement due to the influence of SFO. Since SFO induces variations in chip length, the total time offset accumulates with an increase in data chips. The use of a midamble can assist in mitigating the effects of SFO. Similar to a preamble, a midamble can be a predefined sequence. When an A-IoT reader detects the midamble, it can estimate the time accumulation offset caused by SFO. By combining it with the preamble, the A-IoT device can estimate the average time offset of one chip, aiding in data decoding. Moreover, upon detecting the midamble, the offset caused by SFO in the previous chips will no longer accumulate, thereby improving the reception performance of subsequent data. It should be noted that, if the SFO changes over time, the benefit of midamble will decrease.

Midamble can also be used for channel measurement. If the D2R transmission lasts for a long time, the channel may undergo changes. Channel estimation based solely on preamble may not be sufficient. Thus, utilizing the midamble for channel estimation can derive the channel state change and enhance data reception performance.

Midamble should be studied in D2R transmission.

- Postamble

Similar as R2D transmission, according to TR 38.848, the design target of maximum message size transmitted from A-IoT device is up to 1000 bit. The signal overhead of postamble can be smaller than the overhead of TBS indication. Moreover, if a postamble is utilized, when an A-IoT device is required to transmit data, such as sensor data to the reader, and the reader lacks knowledge of the packet size, the A-IoT device can forgo sending a Buffer Status Report (BSR) to the base station. This omission can consequently mitigate the scheduling latency that would otherwise be incurred by the transmission of a BSR. Similar as midamble, postamble in D2R transmission can assist the base station in estimating the SFO and performing channel measurements, which is benefit for decoding the last segmentation of data.

Thus, postamble can be used to indicate the end of data transmission. More discussion can be found in our contribution [2].

Postamble should be included in D2R transmission.

- Control field

In RAN1#117 meeting, the following was agreed.

- Agreement For L1 D2R control information (if defined), the following are not considered for further study: - CSI feedback - Autonomous SR - FFS: Whether any other L1 D2R control information is needed or not

If the A-IoT system allows the A-IoT device to autonomously select certain scheduling parameters (such as encoding method), it may need to carry corresponding information in the D2R control information. Otherwise, there is no need to have a L1 D2R control information.

The necessity of D2R control information in D2R transmission should be clarified firstly.

- Frame structure

To sum up, the following components can be considered in D2R transmission: Preamble, midamble, data, postamble. And the Figure 2 shows the D2R frame structure.

Figure 2 D2R frame structure

In some cases, readers may only require proximity determination without the need for additional data scheduling. Utilizing preamble as a reference signal for proximity determination can be considered.

Figure 3 Frame structure for proximity determination

For D2R transmission, the following frame structures can be considered:

- Frame structure includes preamble, data, postamble, and midamble, if needed.

- Frame structure includes only preamble.

- Device availability

- 3.1 Availability of device

The device availability can be assessed by investigating the available time and the proportion of available time. These two metrics are closely related to capacitance, power consumption, received RF power and RF energy harvesting efficiency, which follows the relationship below:

- The capacitance affects the available energy in storage.

- Available time = available energy / Power consumption.

- Charging time = available energy/(Received RF power*Energy harvesting efficiency)

- Proportion of available time = Available time /(Available time + Charging time )

- Based on the above methodology, Table 1 presents an initial assessment of device availability for different device capacitance and power consumption values.

- Table 1 Assessment for device availability

| Parameters | Values |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Capacitance (uF) | 1 | 2 | 10 | 10 | 20 | 50 |
| Available energy in storage (uJ) | 0.25 | 0.5 | 2.5 | 2.5 | 5 | 12.5 |
| Device power consumption (uW) | 2 |  |  | 200 |  |  |
| Received RF power (dBm) | -25 |  |  | -25 |  |  |
| RF energy harvesting efficiency | 10% |  |  | 10% |  |  |
| Available time | 125 ms | 250 ms | 1.25 s | 12.5 ms | 25 ms | 62.5 ms |
| Charging time | 833 ms | 1667 ms | 8.33 s | 8.33 s | 16.67 s | 41.67 s |
| Proportion of available time | 13% |  |  | 0.15% |  |  |

- The proportion of available time is around 13% for power consumption of 2 uW and around 0.15% for power consumption of 200 uW.

- 3.2 Duty-cycle

Duty-cycle based on energy storage status

- One approach of duty-cycle mechanism is to utilize the energy storage status to control ON and OFF states of the device. When the energy storage status is 100%, the device activates signal detection (ON state). When the energy storage status drops below a specified threshold, the device closes signal detection and enters a deep or light-sleep state (OFF state). During deep or light-sleep phase, the device is unavailable since it cannot receive and transmit signals. Then, the device availability time in duty-cycle mode is segmented and discrete while the device availability time in normal mode is continuous until the stored energy is depleted. From Table 2, it can be observed that deep-sleep based duty-cycle mode and normal mode have the same proportion of available time for a AIoT device. And the proportion of available time in light-sleep based duty-cycle mode is lower than that in normal mode since the device still consumes energy during the light-sleep phase.

- Table 2 Device availability for duty-cycle based on energy storage status

|  | Normal (w/o duty-cycle) | Deep-sleep based duty-cycle | Light-sleep based duty-cycle |
| --- | --- | --- | --- |
| Capacitance | 10 uF |  |  |
| Available energy in storage | 2.5 uJ |  |  |
| Received RF power | -25 dBm (~3 uW) |  |  |
| RF energy harvesting efficiency | 10% |  |  |
| Power consumption for ON state | ~2 uW |  |  |
| Power consumption for OFF state | N/A | N/A | 0.2 uW |
| Remaining available energy after ON state | 0 | 2 uJ (80%) | 2 uJ (80%) |
| Available time of ON state | 1.25 s | 0.25 s | 0.25 s |
| Unavailable time for full charging (including sleep duration) | 8.33 s | 1.67 s | 5 s |
| Proportion of available time | 13% | 13% | 4.8% |

- For duty-cycle based on energy storage status, deep-sleep based duty-cycle mode and normal mode have the same proportion of available time for a Ambient IoT device.

- For duty-cycle based on energy storage status, the proportion of available time in light-sleep based duty-cycle mode is lower than that in normal mode since the device still consumes energy during the light-sleep phase.

- Since the percentage of available time in duty-cycle mode is less than or equal to that in normal mode, the duty-cycle mechanism cannot increase the probability of the device receiving R2D signals. In contrast, the duty-cycle mechanism may decrease this probability. Therefore, duty cycle is not necessary before random access procedure.

Duty cycle is not considered before random access procedure.

Duty cycle based on periodicity

- In RFID technologies, the device is required to detect every slot decrement command until the RN16 is reported during sloted-ALOHA based random access procedure (i.e. inventory process). For Ambient IoT device, this will consume a considerable amount of energy. With limited energy storage and harvesting capabilities, the device's ability to continuously detect signals is constrained. In a random access process with a large number of devices, the prolonged signal detection time may prevent a device from completing the entire access process.

- To mitigate this issue, reducing the detection of slot decrement commands should be considered to shorten the signal detection time and conserve device energy. This would allow the device to handle a longer duration random access process. Consequently, the following duty-cycle mechanism based on periodic detection can be considered during sloted-ALOHA based random access procedure:

- The BS transmits decrement commands, where a part of these decrement commands are transmitted periodically, as shown in Figure 4. The period is equal to an absolute time length, e.g. x ms.

- The device initiates detection window periodically to detect decrement commands. Each time the device detects a decrement command, it reduces the stored slot value by P, where P is the number of decrement commands transmitted in a period.

- When detection window is closed, the device can enter a light-sleep state. In light-sleep state, the device maintains the operation of a low-speed clock and memory without signal reception or transmission, thereby reducing energy consumption and conserving battery power.

- If the stored slot value is less than P, the device sequentially detects the decrement commands sent by the BS without employing the duty cycle. Each time the device detects a decrement command, it reduces the stored slot value by 1.

- When the stored slot value decreases to 0, the device transmits the Step 2 signal during the access process.。

Figure 4 Duty cycle based on periodic detection

During random access procedure, duty cycle based on periodic detection can be considered for Ambient IoT device to reduce energy consumption.

- Random access

In RFID technologies, the inventory process usually uses slot-ALOHA, BTree or Q-selection methodology to avoid the collision of response signals sent by multiple devices. Whereas, the efficiency of existing algorithms is limited for inventory of a large number of devices. For Ambient IoT, considering the large coverage range and also the potential expansion of the future IoT applications, large number of devices and high connection density are expected. Then, more devices that have access requirements will response to the access command from BS, which greatly increases the probability of multi-device collision. As a result, higher access efficiency will be an important requirement for Ambient IoT. Therefore, the following methods can be considered to enhance access efficiency.

In RFID technologies, the inventory process usually uses slot-ALOHA, B-Tree, or Q-selection methodology to avoid the collision of response signals sent by multiple devices. However, the efficiency of existing algorithms is limited when inventorying a large number of devices. For Ambient IoT, considering its large coverage range and potential future expansion, high connection density and a large number of devices are expected. Thus, more devices with access requirements will respond to the access command from the BS, increasing the probability of multi-device collisions significantly. Consequently, achieving higher access efficiency is a crucial requirement for Ambient IoT. Then, the following methods can be considered to enhance access efficiency.

- Enhanced slot based anti-collision algorithm

In practice, the efficiency is highest for BTree or ALOHA algorithm when 2-4 devices are intended to be identified. For ALOHA or Q-slection algorithm, in a collision slot, the number of conflicting devices is usually small and there is a high probability that this number is less than or equal to 4. Hence, a combined algorithm of Q-selection and BTree can be considered to improve access efficiency. Specifically, Q-selection algorithm is performed for access of multiple devices. And for a collision slot of the Q-selection algorithm, BTree algorithm is used to complete the access process of conflicting devices in this slot and then the next slot of the Q-selection algorithm starts.

- Anti-collision algorithm orthogonal/non-orthogonal extended UL data

The orthogonal or non-orthogonal sequences are used for different devices to achieve parallel UL data transmission. Under the Ambient IoT coverage requirements, lower data rate (e.g., low coding rate and repetition etc.) may be configured to enhance coverage performance, which causes a decrease in access efficiency. In this case, orthogonal/non-orthogonal UL data will greatly improve the access efficiency compared with direct repeated transmission. Taking the extension sequence length of 4 as an example, the extended data is equivalent to four repeated transmissions. Compared with simple bit/TB repetition, N>=4 extension sequences can support the parallel transmission of a maximum of N devices. This means that access efficiency will be significantly improved.

For 4-step access procedure, code division multiplexing can be used either only for Step2 D2R signal or for both Step2 and Step4 D2R signals.

In a 4-step access process with CDM-based Step2 signal, the BS transmits Step1 signal to trigger the device(s) in the target slot to send CDM-based Step2 signal. If the BS correctly decode multiple Step2 signals, it transmit multiple Step3 signals where each Step3 signal contains a single ACK information. Furthermore, the BS receives the Step4 signal sent by the corresponding device after each step3 signal. Figure 5 shows the 4-step access process with CDM-based Step2 signal.

In a 4-step access process with CDM-based Step2 and Step4 signals, after the BS correctly decodes multiple CDM-based Step2 signals, it transmits one Step3 signal that contains multiple ACKs corresponding to those multiple Step2 signals. Then, the BS receives the CDM-based Step4 signals sent by the corresponding devices. Figure 6 illustrates the 4-step access process with CDM-based Step 2 and Step 4 signals.

Figure 5 4-step access process with CDM-based Step2 signal

Figure 6 4-step access process with CDM-based Step2 signal

Based on the above methods, Figure 7 shows the simulation results for access efficiency, which represents that the number of devices that can be accessed per second. The simulation assumptions are given in Table 3. In the simulation, Q + CDM-RN16 uses CDM for RN16 and Q + CDM-RN16-ID uses CDM for both RN16 and device ID.

Table 3 Simulation assumptions of random access for for multiple devices

| Parameters |  | Values |
| --- | --- | --- |
| Random access procedure |  | 4-step access |
| Number of Ambient IoT devices |  | 100, 500, 1000, 3750 |
| Initial number of slots for ALOHA |  | 1024 |
| Message size (Number of bits) | Query command | 8 |
|  | Decrement command | 4 |
|  | RN16 | 16 |
|  | Acknowledge of RN16 | 16 |
|  | Device ID | 96 |
| CRC length (Number of bits) |  | 16 for device ID report |
| Transmission time per bit |  | 25 us |
| Preamble length |  | 8×25 us for R2D and D2R |
| Interval between R2D and D2R signals |  | 10×25 us |
| Interval between D2R and R2D signals |  | 5×25 us |
| Length and number of extended codes |  | L = 4; N=4 |

Figure 7 Access efficiency for different anti-collision algorithms

From the figure, it can be observed that the algorithm of combined Q-selection and BTree can improve access efficiency by more than 10% compared to traditional Q-selection algorithm. The access efficiency of Q-selection based on CDM for RN16 exceeds 1.5 times that of Q-selection. And the access efficiency of Q-selection based on CDM for both RN16 and device ID is around 2 times that of Q-selection.

Due to the large coverage range, high connection density and limited access latency, the following approaches can be considered to enhance access efficiency for Ambient IoT.

- Enhanced slot based anti-collision algorithm, e.g. Q-selection + BTree

- Anti-collision algorithm orthogonal/non-orthogonal extended D2R data

- Timing aspect

According to the agreements, an approach with default delay range of [TR2D_min, TR2D_max] (Option 1) and an approach with delay indicator (Option 2) needs to be selected for time interval between a R2D transmission and the corresponding D2R transmission following it. Compared to Option 1, Option 2 can achieve more flexible scheduling, which is beneficial for time domain resource allocation and improving transmission efficiency. In Option 2, a value of the TR2D is indicated by the BS/intermediate UE from a scheduling delay set. Also, the indicated D2R transmission time TR2D is larger than or equal to the minimum time TR2D_min.

Considering device Rx signal processing time and Tx signal preparation time, TR2D_min should be greater than or equal to the sum of R2D signal processing time and D2R signal preparation time. The signal processing time of the device may be associated with command type or use case. The scheduling delay set can be applicable for the timing requirements of all use cases and command types.

On the other hand, the signal processing time may not be equal among device types due to different complexity and signal processing methods. Device 2b can generate independent UL signal internally, use IF/ZIF-ED receiver to detect signal and has more radio frequency modules, so it may require larger signal processing time than Device 1/2a. Thus, it can be considered that at least two types of devices among device 1, 2a and 2b have different scheduling delay sets for D2R transmission timing TR2D.

The corresponding D2R transmission timing TR2D following a R2D transmission is indicated from a scheduling delay set in the R2D transmission.

- Different scheduling delay sets can be defined for different device types.

Moreover, for the time between a D2R transmission and the corresponding R2D transmission following it, the minimum should not be shorter than the preparation time required for Tx-Rx switching. The maximum can be determined based on BS implementation and does not need to be specified.

For the time between two different consecutive R2D transmissions to the same A-IoT device, the minimum should not be less than the first R2D signal processing time of the device. And the maximum can be based on BS implementation and does not need to be specified.

- Conclusion

In this contribution, we discuss the frame structure, device availability, random access and timing aspects for Ambient IoT. And the relevant proposals are given as following:

- The proportion of available time is around 13% for power consumption of 2 uW and around 0.15% for power consumption of 200 uW.

- For duty-cycle based on energy storage status, deep-sleep based duty-cycle mode and normal mode have the same proportion of available time for a Ambient IoT device.

- For duty-cycle based on energy storage status, the proportion of available time in light-sleep based duty-cycle mode is lower than that in normal mode since the device still consumes energy during the light-sleep phase.

- Clarify that the R2D transmission includes the R2D postamble, if postamble is used.

To align the end of R2D transmission(including postamble), the following options can be considered:

  - Option#1: Use a flexible postamble (preferred).

  - Option#2: A proper design of TBS and code rate to ensure that R2D transmission terminated  at the end of OFDM symbols. The postamble occupies an integer number of OFDM symbols.

To determine or derive the end of PRDCH transmission, Option 1: R2D postamble immediately following the PRDCH to indicate the end of the PRDCH is preferred.

  - If CRC is attached for the R2D transmission, the postamble is appended at the end of the R2D transmission; otherwise, the postamble is omitted.

Control field should be included in R2D transmission containing at least the following aspects

- device type or cast type

- Repetition times and/or code rate

For R2D transmission, the following components should be considered: preamble, control field, postamble, data.

Control field should be encoded separately from R2D data.

Midamble should be studied in D2R transmission.

Postamble should be included in D2R transmission.

The necessity of D2R control information in D2R transmission should be clarified firstly.

For D2R transmission, the following frame structures can be considered:

- Frame structure includes preamble, data, postamble, and midamble, if needed.

- Frame structure includes only preamble.

Duty cycle is not considered before random access procedure.

During random access procedure, duty cycle based on periodic detection can be considered for Ambient IoT device to reduce energy consumption.

Due to the large coverage range, high connection density and limited access latency, the following approaches can be considered to enhance access efficiency for Ambient IoT.

- Enhanced slot based anti-collision algorithm, e.g. Q-selection + BTree

- Anti-collision algorithm orthogonal/non-orthogonal extended D2R data

The corresponding D2R transmission timing TR2D following a R2D transmission is indicated from a scheduling delay set in the R2D transmission.

- Different scheduling delay sets can be defined for different device types.

- References

- Draft Report of 3GPP TSG RAN WG1 #117 v0.2.0, RAN1#117

- R1-2406407, Discussion on channel and signal for Ambient IoT, ZTE Corporation, Sanechips, RAN1#118